3GPP TSG-RAN5 Meeting #98	R5-231867

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2693 | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to high range reference test frequency for n66 DL CA |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The definition for high range reference frequency of CBW combination 40/40 is incorrect in Table 4.3.1.1.1.66-1 and Table 4.3.1.1.1.66-2 that made overlapping in cells.e.g. PCELL=2130 BW=40MHz [2110,2150]MHz SCELL=2160 BW=40MHz [2140,2180]MHz then need add new row in Table 4.3.1.1.1.66-1/2 specified for n66 DL non-contiguous CA(40+40) to update high range frequency from 2160 to 2180 to solve the issue update Table 4.3.1.1.5.66-1/2 in order to pointing to the new row in 4.3.1.1.1.66-1/2 respectively |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correction to high range frequency in CBW combination 40/40 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case failed to execute caused by cell frequncy overlapping |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.1.1.1.66, 4.3.1.1.5.66 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: added new row in table 4.3.1.1.1.66-1/2 for DL non-contiguous CA and upate table Table 4.3.1.1.5.66-1/2 final version:revised from R5-230235r1 |  |  |  |  |  |  |  |  |

<Start of Changes>

4.3.1.1.1.66	Reference test frequencies for NR operating band n66

Table 4.3.1.1.1.66-1: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 15 kHz

| UL/DLCBW Combination (Asymmetric BCS) Note 3 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5/5 | 5 | 25 | Downlink | Low | 2112.5 | 422500 | 2110.25 | 422050 | 0 | 15 | 5279 | 422410 | 0 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2124.39 | 424878 | 102 |  | 5361 | 428910 | 0 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2177.5 | 435500 | 2084.53 | 416906 | 504 |  | 5443 | 435410 | 0 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2152.5 | 430500 | 2124.6 | 424920 | 102 |  | 5364 | 429150 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2084.74 | 416948 | 504 |  | 5446 | 435650 | 6 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2124.67 | 424934 | 102 |  | 5365 | 429170 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2187.5 | 437500 | 2084.81 | 416962 | 504 |  | 5444 | 435610 | 0 | 0 | 2 (4) | 508 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1652.03 | 330406 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1777.5 | 355500 | 1774.17 | 354834 | 6 |  | - | - | - | - | - | - |
| 5/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1742.5 | 348500 | 1649.53 | 329906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1769.17 | 353834 | 6 |  | - | - | - | - | - | - |
| 5/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 5 | 25 | Uplink | Low | 1712.5 | 342500 | 1710.25 | 342050 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1737.5 | 347500 | 1644.53 | 328906 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1762.5 | 352500 | 1759.17 | 351834 | 6 |  | - | - | - | - | - | - |
| 10/10 | 10 | 52 | Downlink | Low | 2115 | 423000 | 2110.32 | 422064 | 0 | 15 | 5280 | 422430 | 2 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2121.96 | 424392 | 102 |  | 5355 | 428430 | 2 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2079.6 | 415920 | 504 |  | 5430 | 434430 | 2 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2150 | 430000 | 2122.1 | 424420 | 102 |  | 5357 | 428650 | 6 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2180 | 436000 | 2079.74 | 415948 | 504 |  | 5432 | 434650 | 6 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2122.17 | 424434 | 102 |  | 5358 | 428670 | 8 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2182.5 | 436500 | 2079.81 | 415962 | 504 |  | 5433 | 434670 | 8 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2122.24 | 424448 | 102 |  | 5359 | 428690 | 10 | 1 | 2 (4) | 107 |
|  |  |  |  | High | 2185 | 437000 | 2079.88 | 415976 | 504 |  | 5434 | 434690 | 10 | 1 | 2 (4) | 509 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1649.6 | 329920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1769.24 | 353848 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 10 | 52 | Uplink | Low | 1715 | 343000 | 1710.32 | 342064 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1644.6 | 328920 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1759.24 | 351848 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 79 | Downlink | Low | 2117.5 | 423500 | 2110.39 | 422078 | 0 | 15 | 5281 | 422450 | 4 | 0 | 0 (0) | 0 |
|  |  |  |  | Mid | 2145 | 429000 | 2119.53 | 423906 | 102 |  | 5349 | 427950 | 4 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2172.5 | 434500 | 2074.67 | 414934 | 504 |  | 5417 | 433450 | 4 | 0 | 0 (0) | 504 |
|  | 15 | 79 | Uplink | Low | 1717.5 | 343500 | 1710.39 | 342078 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1647.17 | 329434 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1764.31 | 352862 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 106 | Downlink | Low | 2120 | 424000 | 2110.46 | 422092 | 0 | 15 | 5282 | 422650 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2117.1 | 423420 | 102 |  | 5343 | 427470 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2069.74 | 413948 | 504 |  | 5407 | 432530 | 2 | 0 | 1 (2) | 506 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2117.2 | 423440 | 102 |  | 5344 | 427490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
|  | 20 | 106 | Uplink | Low | 1720 | 344000 | 1710.46 | 342092 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1644.74 | 328948 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1759.38 | 351876 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 133 | Downlink | Low | 2122.5 | 424500 | 2110.53 | 422106 | 0 | 15 | 5283 | 422670 | 8 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2114.67 | 422934 | 102 |  | 5337 | 426990 | 8 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2167.5 | 433500 | 2064.81 | 412962 | 504 |  | 5394 | 431550 | 4 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2114.7 | 422940 | 102 |  | 5337 | 426990 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2175 | 435000 | 2064.84 | 412968 | 504 |  | 5394 | 431550 | 2 | 0 | 1 (2) | 506 |
|  | 25 | 133 | Uplink | Low | 1722.5 | 344500 | 1710.53 | 342106 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1642.31 | 328462 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1754.45 | 350890 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 160 | Downlink | Low | 2125 | 425000 | 2110.6 | 422120 | 0 | 15 | 5284 | 422690 | 10 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2112.24 | 422448 | 102 |  | 5331 | 426510 | 10 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2165 | 433000 | 2059.88 | 411976 | 504 |  | 5381 | 430570 | 6 | 0 | 1 (2) | 506 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2112.2 | 422440 | 102 |  | 5330 | 426490 | 6 | 0 | 0 (0) | 102 |
|  |  |  |  | High | 2170 | 434000 | 2059.84 | 411968 | 504 |  | 5383 | 430610 | 10 | 1 | 1 (2) | 507 |
|  | 30 | 160 | Uplink | Low | 1725 | 345000 | 1710.6 | 342120 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1639.88 | 327976 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1749.52 | 349904 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 216 | Downlink | Low | 2130 | 426000 | 2110.56 | 422112 | 0 | 15 | 5283 | 422670 | 6 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2107.2 | 421440 | 102 |  | 5319 | 425550 | 2 | 0 | 1 (2) | 104 |
|  |  |  |  | High | 2160 | 432000 | 2049.84 | 409968 | 504 |  | 5358 | 428670 | 6 | 1 | 2 (4) | 509 |
|  | 40 | 216 | Uplink | Low | 1730 | 346000 | 1710.56 | 342112 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1634.84 | 326968 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1739.48 | 347896 | 6 |  | - | - | - | - | - | - |
| N.A./40 (Note4) | 40 | 216 | Downlink | High | 2180 | 436000 | 2069.84 | 413968 | 504 |  | 5405 | 432490 | 6 | 0 | 0 (0) | 504 |
| 45/45 | 45 | 242 | Downlink | Low | 2132.5 | 426500 | 2110.72 | 422144 | 0 | 15 | 5284 | 422690 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2104.86 | 420972 | 102 | 15 | 5313 | 425070 | 10 | 1 | 0 (0) | 103 |
|  |  |  |  | High | 2157.5 | 431500 | 2045 | 409000 | 504 | 15 | 5345 | 427690 | 2 | 1 | 2 (4) | 509 |
|  | 45 | 242 | Uplink | Low | 1732.5 | 346500 | 1710.72 | 342144 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1632.5 | 326500 | 504 | - | - | - | - | - | - | - |
|  |  |  |  | High | 1757.5 | 351500 | 1734.64 | 346928 | 6 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-1 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3 Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. Note 4: Used only for non-contiguous CA x+40 combination with no UL configuration on the SB in Table 4.3.1.1.5.66-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 4.3.1.1.1.66-1A: Void

Table 4.3.1.1.1.66-2: Test frequencies for NR operating band n66, uplink and downlink channel bandwidth combinations and SCS 30 kHz

| UL/DLCBW Combination (Asymmetric BCS) Note 3 | CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10/10 | 10 | 24 | Downlink | Low | 2115 | 423000 | 2110.68 | 422136 | 0 | 15 | 5286 | 422910 | 18 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2103.96 | 420792 | 102 |  | 5361 | 428910 | 18 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1989.24 | 397848 | 504 |  | 5436 | 434910 | 18 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2150 | 430000 | 2104.1 | 420820 | 102 |  | 5360 | 428890 | 2 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1989.38 | 397876 | 504 |  | 5435 | 434890 | 2 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2104.08 | 420816 | 102 |  | 5361 | 428910 | 10 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2182.5 | 436500 | 1989.36 | 397872 | 504 |  | 5436 | 434910 | 10 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2155 | 431000 | 2104.24 | 420848 | 102 |  | 5362 | 428930 | 6 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2185 | 437000 | 1989.52 | 397904 | 504 |  | 5437 | 434930 | 6 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1559.24 | 311848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1775 | 355000 | 1768.52 | 353704 | 6 |  | - | - | - | - | - | - |
| 10/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 10 | 24 | Uplink | Low | 1715 | 343000 | 1710.68 | 342136 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1740 | 348000 | 1554.24 | 310848 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1758.52 | 351704 | 6 |  | - | - | - | - | - | - |
| 15/15 | 15 | 38 | Downlink | Low | 2117.5 | 423500 | 2110.66 | 422132 | 0 | 15 | 5287 | 422930 | 2 | 0 | 1 (6) | 12 |
|  |  |  |  | Mid | 2145 | 429000 | 2101.44 | 420288 | 102 |  | 5355 | 428430 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2172.5 | 434500 | 1984.22 | 396844 | 504 |  | 5423 | 433930 | 2 | 0 | 1 (6) | 1020 |
|  | 15 | 38 | Uplink | Low | 1717.5 | 343500 | 1710.66 | 342132 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1556.72 | 311344 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1772.5 | 354500 | 1763.5 | 352700 | 6 |  | - | - | - | - | - | - |
| 20/20 | 20 | 51 | Downlink | Low | 2120 | 424000 | 2110.82 | 422164 | 0 | 15 | 5285 | 422890 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2099.1 | 419820 | 102 |  | 5349 | 427950 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1979.38 | 395876 | 504 |  | 5413 | 433010 | 18 | 0 | 1 (6) | 1020 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 20/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (0,1) |  |  |  | Mid | 2155 | 431000 | 2099.2 | 419840 | 102 |  | 5350 | 427970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
|  | 20 | 51 | Uplink | Low | 1720 | 344000 | 1710.82 | 342164 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1554.38 | 310876 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1770 | 354000 | 1758.66 | 351732 | 6 |  | - | - | - | - | - | - |
| 25/25 | 25 | 65 | Downlink | Low | 2122.5 | 424500 | 2110.8 | 422160 | 0 | 15 | 5286 | 422910 | 10 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2096.58 | 419316 | 102 |  | 5343 | 427470 | 6 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2167.5 | 433500 | 1974.36 | 394872 | 504 |  | 5400 | 432030 | 2 | 0 | 2 (7) | 1022 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 25/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2152.5 | 430500 | 2096.7 | 419340 | 102 |  | 5343 | 427470 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2175 | 435000 | 1974.48 | 394896 | 504 |  | 5400 | 432030 | 18 | 0 | 1 (6) | 1020 |
|  | 25 | 65 | Uplink | Low | 1722.5 | 344500 | 1710.8 | 342160 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1551.86 | 310372 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1767.5 | 353500 | 1753.64 | 350728 | 6 |  | - | - | - | - | - | - |
| 30/30 | 30 | 78 | Downlink | Low | 2125 | 425000 | 2110.96 | 422192 | 0 | 15 | 5287 | 422930 | 6 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2094.24 | 418848 | 102 |  | 5337 | 426990 | 2 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2165 | 433000 | 1969.52 | 393904 | 504 |  | 5387 | 431050 | 22 | 0 | 1 (6) | 1020 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| 30/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
| (1) |  |  |  | Mid | 2150 | 430000 | 2094.2 | 418840 | 102 |  | 5336 | 426970 | 22 | 0 | 0 (5) | 214 |
|  |  |  |  | High | 2170 | 434000 | 1969.48 | 393896 | 504 |  | 5389 | 431090 | 14 | 0 | 2 (7) | 1022 |
|  | 30 | 78 | Uplink | Low | 1725 | 345000 | 1710.96 | 342192 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1549.52 | 309904 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1765 | 353000 | 1748.8 | 349760 | 6 |  | - | - | - | - | - | - |
| 40/40 | 40 | 106 | Downlink | Low | 2130 | 426000 | 2110.92 | 422184 | 0 | 15 | 5286 | 422910 | 2 | 0 | 0 (5) | 10 |
|  |  |  |  | Mid | 2145 | 429000 | 2089.2 | 417840 | 102 |  | 5325 | 426030 | 18 | 0 | 1 (6) | 216 |
|  |  |  |  | High | 2160 | 432000 | 1959.48 | 391896 | 504 |  | 5361 | 428910 | 2 | 0 | 0 (5) | 1018 |
|  | 40 | 106 | Uplink | Low | 1730 | 346000 | 1710.92 | 342184 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1544.48 | 308896 | 504 |  | - | - | - | - | - | - |
|  |  |  |  | High | 1760 | 352000 | 1738.76 | 347752 | 6 |  | - | - | - | - | - | - |
| N.A./40 (Note4) | 40 | 106 | Downlink | High | 2180 | 436000 | 1979.48 | 395896 | 504 |  | 5411 | 432970 | 22 | 0 | 0 (5) | 1018 |
| 45/45 | 45 | 242 | Downlink | Low | 2132.5 | 426500 | 2110.72 | 422144 | 0 | 15 | 5284 | 422690 | 2 | 1 | 2 (4) | 5 |
|  |  |  |  | Mid | 2145 | 429000 | 2104.86 | 420972 | 102 | 15 | 5313 | 425070 | 10 | 1 | 0 (0) | 103 |
|  |  |  |  | High | 2157.5 | 431500 | 2045 | 409000 | 504 | 15 | 5345 | 427690 | 2 | 1 | 2 (4) | 509 |
|  | 45 | 242 | Uplink | Low | 1732.5 | 346500 | 1710.72 | 342144 | 0 | - | - | - | - | - | - | - |
|  |  |  |  | Mid | 1745 | 349000 | 1632.5 | 326500 | 504 | - | - | - | - | - | - | - |
|  |  |  |  | High | 1757.5 | 351500 | 1734.64 | 346928 | 6 | - | - | - | - | - | - | - |
| Note 1: The CORESET#0 Index and the associated CORESET#0 Offset refers to Table 13-2 in TS 38.213 [22]. The value of CORESET#0 Index is signalled in controlResourceSetZero (pdcch-ConfigSIB1) in the MIB. The offsetToPointA IE is expressed in units of resource blocks assuming 15 kHz subcarrier spacing for FR1 and 60 kHz subcarrier spacing for FR2. Note 2: The parameter Offset Carrier CORESET#0 specifies the offset from the lowest subcarrier of the carrier and the lowest subcarrier of CORESET#0. It corresponds to the parameter ΔFOffsetCORESET-0-Carrier in Annex C expressed in number of common RBs. Note 3 Asymmetric CBW BCS refers to applicable asymmetric UL and DL channel bandwidth combination set as specified in TS 38.101-1 [7], Table 5.3.6-1 for NR band n66. Note 4: Used only for non-contiguous CA x+40 combination with no UL configuration on the SB in Table 4.3.1.1.5.66-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

4.3.1.1.5.66	CA_n66(2A)

Editor’s note:	Test frequencies for CA_n66(2A) with mixed numerology with SCS CC1=15kHz and SCS CC2=30 kHz or 60kHz; and SCS CC1=30kHz and SCS CC2=15 kHz or 60 kHz is FFS.

Table 4.3.1.1.5.66-1: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=15 kHz, Max Wgap

| CBW combination (BCS) | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range (Note 1) | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); n66A (5MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 5+5 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 5+10 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 5+15 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 5+20 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 5+25 (1) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 5+30 (1) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 5+40 (0,1,2) | SB1 | 5 | 25 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/5 |
|  | SB2 | 40 | 216 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-1: High range for N.A./40. Otherwise: Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (10MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 10+5 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 10+10 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 10+15 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 10+20 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 10+25 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 10+30 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 10+40 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 40 | 216 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-1: High range for N.A./40. Otherwise: Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (15MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 15+5 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/5 |
| 15+10 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 10/10 |
| 15+15 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 15/15 |
| 15+20 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 15+25 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 15+30 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 15+40 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 40 | 216 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-1: High range for N.A./40. Otherwise: Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (20MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 20+5 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 20+10 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 20+15 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 20+20 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 20+25 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 20+30 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 20+40 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-1: High range for N.A./40. Otherwise: Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (25MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 25+5 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 25+10 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 25+15 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 25+20 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 25+25 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 25+30 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 25+40 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-1: High range for N.A./40. Otherwise: Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (30MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 30+5 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 30+10 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 30+15 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 30+20 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 30+25 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 30+40 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-1: High range for N.A./40. Otherwise: Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (40MHz) + n66A(5-40MHz) |  |  |  |  |  |  |
| 40+5 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 5 | 25 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 5/5 |
| 40+10 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 10/10 |
| 40+15 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination 15/15 |
| 40+20 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | able 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 40+25 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 40+30 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 40+40 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-1: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 40 | 216 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-1: High range for N.A./40 Otherwise: Table 4.3.1.1.1.66-1: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

Table 4.3.1.1.5.66-2: NR Intra-Band non-contiguous CA configuration CA_n66(2A) without UL CA, SCS=30 kHz, Max Wgap

| CBW combination (BCS) | SB | Bandwidth [MHz] | carrierBandwidth [PRBs] | Range | Gap | Test frequencies and signalling parameters |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n66(2A); n66A (10MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 10+10 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/10 |
| 10+15 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 15/15 |
| 10+20 (0,1,2) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 10+25 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 10+30 (1) | SB1 | 10 | 52 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 10+40 (0,1,2) | SB1 | 10 | 24 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/10 |
|  | SB2 | 40 | 106 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-2: High range for N.A./40. Otherwise: Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (15MHz) +n66A(10-40MHz) |  |  |  |  |  |  |
| 15+10 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/10 |
| 15+15 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 15/15 |
| 15+20 (0,1,2) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/20 |
| 15+25 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 15+30 (1) | SB1 | 15 | 79 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 15+40 (0,1,2) | SB1 | 15 | 38 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 15/15 |
|  | SB2 | 40 | 106 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-2: High range for N.A./40 Otherwise: Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (20MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 20+10 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 20+15 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 20+20 (0,1,2) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 20+25 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 20+30 (1) | SB1 | 20 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 20+40 (0,1,2) | SB1 | 20 | 51 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
|  | SB2 | 40 | 106 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-2: High range for N.A./40 Otherwise: Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (25MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 25+10 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 25+15 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 25+20 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 25+25 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 25+30 (1) | SB1 | 25 | 133 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 25+40 (1) | SB1 | 25 | 65 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
|  | SB2 | 40 | 106 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-2: High range for N.A./40 Otherwise: Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (30MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 30+10 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 30+15 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 30+20 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for for UL/DL Bandwidth combination = 5/20, 10/20 or 20/20 depending on required UL bandwidth |
| 30+25 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 30+30 (1) | SB1 | 30 | 160 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 30+40 (1) | SB1 | 30 | 78 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
|  | SB2 | 40 | 106 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-2: High range for N.A./40 Otherwise: Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| CA_n66(2A); n66A (40MHz) + n66A(10-40MHz) |  |  |  |  |  |  |
| 40+10 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 10 | 52 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 10/10 |
| 40+15 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 10/40 or 20/40 depending on required UL bandwidth |
|  | SB2 | 15 | 79 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination 15/15 |
| 40+20 (1,2) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 20 | 106 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 10/20 or 20/20 depending on required UL bandwidth |
| 40+25 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 25 | 133 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/25, 10/25 or 25/25 depending on required UL bandwidth |
| 40+30 (1) | SB1 | 40 | 216 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 30 | 160 | Downlink & Uplink |  | Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/30, 10/30 or 30/30 depending on required UL bandwidth |
| 40+40 (1,2) | SB1 | 40 | 106 | Downlink & Uplink | Max Wgap | Table 4.3.1.1.1.66-2: Low range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
|  | SB2 | 40 | 106 | Downlink or Downlink & Uplink |  | If UL is not configured on SB2: Table 4.3.1.1.1.66-2: High range for N.A./40 Otherwise: Table 4.3.1.1.1.66-2: High range for UL/DL Bandwidth combination = 5/40, 10/40, 20/40 or 40/40 depending on required UL bandwidth |
| Note 1: CA_n66(2A) is specified in [7] 38.101-1 without uplink CA. PCC is configured on SB1 unless otherwise stated. The test frequencies for both SB1 and SB2 have been specified for both downlink and uplink to enable that the CC of either SB1 or SB2 can be used as PCC. |  |  |  |  |  |  |

<End of Changes>