3GPP TSG-RAN WG5 Meeting # 98	R5-231295

# Athens, Greece, 27 Feb – 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 2181 | rev | - | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on the requirements for UE MPR for intra-band contiguous CA in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The requirements for UE MOP reduction for intra-band contiguous CA are incorrect due to the misuse of MPR with contiguous RB allocation and non-contiguous RB allocation. Furthermore, the MPR is clarified to be set based on the used waveform type and modulation order for different component carriers of a CA configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the misuse of MPR with contiguous RB allocation and non-contiguous RB allocation. - Clarify the rules for MPR with waveform type and modulation order. - Some other editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirements for UE MOP reduction for intra-band contiguous CA will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.2.0.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.2A.2.0.4	Maximum Power Reduction for Intra-band contiguous CA

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.0.4-1 with contiguous RB allocation is specified in Table 6.2A.2.0.4-1 for UE power class 3 CA bandwidth classes B and C. The MPR with contiguous RB allocation is specified in Table 6.2A.2.0.4-1a for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth class C when the signalling is indicated for dualPA-Architecture IE. The MPR with contiguous RB allocation is specified in Table 6.2A.2.0.4-1b for power class 2 CA bandwidth classes B and C with TxD supported.

In case the modulation format or waveform type is different on different component carriers then the requirement is set by rules applied to the waveform type (DFT-s-OFDM or CP-OFDM) and modulation order used in the configuration with the largest MPR.

Unless otherwise specified, pi/2 BPSK in following MPR tables refers to both variants of pi/2 BPSK referenced in clause 6.2.2.3, Table 6.2.2.3-1.

Table 6.2A.2.0.4-1: Contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | outer | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | QPSK | 1.0 | 3.5 | 2.5 | 7 |
|  | 16QAM | 1.5 | 3.5 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.0 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.0 | 4.0 | 3.5 | 8 |
|  | 16QAM | 2.5 | 4.0 | 3.5 | 8 |
|  | 64QAM | 3.5 | 4.0 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |

Table 6.2A.2.0.4-1a: Contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | QPSK | 2.0 | 4.01 | 2.5 | 7 |
|  | 16QAM | 2.5 | 4.01 | 2.5 | 7 |
|  | 64QAM | 3.0 | 4.51 | 5 | 7 |
|  | 256QAM | 5.5 | 6.0 | 7 | 7.5 |
| CP-OFDM | QPSK | 2.5 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.0 | 5.01 | 3.5 | 8 |
|  | 64QAM | 3.5 | 5.01 | 5 | 8 |
|  | 256QAM | 6.5 | 6.5 | 7 | 8 |
| NOTE 1 When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. |  |  |  |  |  |

Table 6.2A.2.0.4-1b: Contiguous RB allocation for Power Class 2 with dual Tx2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | QPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.5 | 5.01 | 3.5 | 8 |
|  | 64QAM | 4.0 | 5.51 | 6 | 8 |
|  | 256QAM | 6.5 | 7.0 | 8 | 8.5 |
| CP-OFDM | QPSK | 3.0 | 5.51 | 4.0 | 8.5 |
|  | 16QAM | 3.5 | 5.51 | 4.0 | 8.5 |
|  | 64QAM | 4.0 | 5.51 | 5.5 | 8.5 |
|  | 256QAM | 7.0 | 7.0 | 7.5 | 8.5 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. NOTE 2: UE indicating TxD supported |  |  |  |  |  |

For CA bandwidth class B and bandwidth class C with contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner and Outer RB allocations:

An RB allocation is contiguous if LCRB1 = 0 or LCRB2 = 0 or (LCRB1  0 and LCRB2  0 and RBStart1 + LCRB1 = NRB1 and RBStart2 = 0), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a contiguous allocation is an inner allocation if

RBStart,Low ≤ RBStart_CA ≤ RBStart,High, and NRB_alloc ≤ ceil(NRB,agg /2),

where

RBStart,Low = max(1, floor(NRB_alloc /2)),

RBStart,High = NRB,agg – RBStart,Low – NRB,alloc,

with

NRB_alloc= (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2,

NRB,agg=NRB1∙2^µ1+ NRB2∙2^µ2.

If LCRB1 =0, RBStart_CA = NRB1∙2^µ1+ RBStart2∙2^µ2,

if LCRB1 > 0, RBStart_CA = RBStart1∙2^µ1.

A contiguous allocation that is not an Inner contiguous allocation is an Outer contiguous allocation

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2A.1.0.4-1 with non-contiguous RB allocation is specified in Table 6.2A.2.0.4-2 for UE power class 3 CA bandwidth classes B and C. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.0.4-3 for power class 2 CA bandwidth classes B and C when the signalling is absent for dualPA-Architecture IE, and for power class 2 CA bandwidth class C when the signalling is indicated for dualPA-Architecture IE. The MPR with non-contiguous RB allocation is specified in Table 6.2A.2.0.4-4 for power class 2 CA bandwidth classes B and C with TxD supported.

Table 6.2A.2.0.4-2: non-contiguous RB allocation for Power Class 3

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | Pi/2 BPSK | 2 | 5.5 | 11.5 | 2.5 | 6 | 13 |
|  | QPSK | 2 | 5.5 |  | 2.5 | 6 |  |
|  | 16QAM | 2.5 | 5.5 |  | 3 | 6 |  |
|  | 64QAM | 4.5 | 6 |  | 5 | 6 |  |
|  | 256QAM | 6 | 6.5 |  | 6.5 | 6.5 |  |
| CP-OFDM | QPSK | 2.5 | 6.5 | 12 | 3.5 | 7 | 14 |
|  | 16QAM | 3 | 7 |  | 3.5 | 7 |  |
|  | 64QAM | 5 | 7 |  | 5 | 7 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 7.5 |  |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.0.4-3: non-contiguous RB allocation for Power Class 2

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 31 | 6.5 | 13 | 31 | 7.5 | 13.5 |
|  | QPSK | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 16QAM | 31 | 6.5 |  | 31 | 7.5 |  |
|  | 64QAM | 5 | 6.5 |  | 5 | 7.5 |  |
|  | 256QAM | 6.5 | 7 |  | 6.5 | 7.5 |  |
| CP-OFDM | QPSK | 3.51 | 7 | 14 | 3.51 | 8 | 14.5 |
|  | 16QAM | 3.51 | 7 |  | 3.51 | 8 |  |
|  | 64QAM | 5 | 7 |  | 5 | 8 |  |
|  | 256QAM | 7.5 | 7.5 |  | 7.5 | 8 |  |
| NOTE 1: The allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz |  |  |  |  |  |  |  |

Table 6.2A.2.0.4-4: non-contiguous RB allocation for Power Class 2 with dual Tx4

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 41 | 7.5 | 14 | 41 | 8.5 | 14.5 |
|  | QPSK | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 16QAM | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 64QAM | 6 | 7.5 |  | 6 | 8.5 |  |
|  | 256QAM | 7.5 | 8 |  | 7.5 | 8.5 |  |
| CP-OFDM | QPSK | 4.51 | 8 | 15 | 4.51 | 9 | 15.5 |
|  | 16QAM | 4.51 | 8 |  | 4.51 | 9 |  |
|  | 64QAM | 6 | 8 |  | 6 | 9 |  |
|  | 256QAM | 8.5 | 8.5 |  | 8.5 | 9 |  |
| NOTE 1: The allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

For CA bandwidth classes B and C with non-contiguous RB allocation, the following parameters are defined to specify valid RB allocation ranges for Inner, Outer1 and Outer2 RB allocations:

Non-Contiguous RB allocation is defined as RBStart1 + LCRB1 < NRB1, or RBStart2 > 0 when all uplink CCs are activated and allocated with RB(s), where RBStart1, LCRB1, and NRB1 are for CC1, RBStart2, LCRB2, and NRB2 are for CC2, CC1 is the component carrier with lower frequency.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((BWChannel_CA / 3 – BWgap ) / 0.18MHz),

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2, RBStart_CA = RBStart1∙2^1

RBStart,Low = max(1, floor(NRB_alloc + (BWgap – BWGB,low)/0.18MHz))

RBStart,High = floor((BWChannel_CA – 2 ∙ BWgap – BWGB,low)/0.18MHz – 2 ∙ NRB_alloc)

BWGB,low =Foffset,low – (NRB1∙12+1)∙SCS1/2

BWgap is the bandwidth of the gap between NRB1 and NRB2 possible allocations of CC1 and CC2 respectively.

In contiguous CA, a non-contiguous RB allocation is a non-contiguous outer 1 RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart_CA ≤ RBStart,High and NRB_alloc ≤ ceil((3 BWChannel_CA / 5 – BWgap) / 0.18MHz)

where

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz)),

RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

NRB_alloc, RBStart_CA, BWgap and BWGB,low are as defined for the Inner region.

In contiguous CA, a non-contiguous allocation is an Outer 2 allocation if it is neither a non-contiguous Inner allocation nor an Outer 1 allocation.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2A.2.1.

<< End of changes >>