3GPP TSG-RAN5 Meeting #98	R5-230876

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1516 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS XCAP test case 15.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The test requirements in section 15.10.5 defines only the 1st rule in a ruleset in the simservs document. It is possible for some deivces that the supplementary service flavours can only be activated in some combinations, e.g. in this test case a device may activate rules for both CFNRc (not reachable) and CFNL (not logged in). In this case the condition to be checked may be in the 2nd rule in a ruleset. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a note in section 15.10.5 to specify the additional rule for CFNRc (not reachable) in the ruleset |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant device mail fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 15.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 15.10	Communication Forwarding on Not logged-in

### 15.10.1	Definition

Test to verify that the UE activates and deactivates IMS Multimedia Telephony Communication Forwarding for the case when user is not registered to IMS service. This process is described in 3GPP TS 24.604 [106].

### 15.10.2	Conformance requirement

Generic requirements for Communication Forwarding can be found from Annexes F.1 and F.4.

[TS 24.604, clause 4.2.1.7]:

The Communication Forwarding on Not Logged-in (CFNL) service enables a served user to redirect incoming communications which are addressed to the served user's address, to another user (forwarded-to address) in case the served user is not registered (logged-in). The CFNL service may operate on all communications, or just those associated with specified basic services.

As a service provider option, a subscription option can be provided to enable the served user to receive an indication that the CFNL service has been activated. This indication shall be provided when the served user logs out according to procedures described in RFC 3261

The maximum number of diversions permitted for each communication is a service provider option. The service provider shall define the upper limit of diversions. When counting the number of diversions, all types of diversion are included.

Reference(s)

3GPP TS 24.604 [106]

### 15.10.3	Test purpose

1)	To verify that the UE can request activation of Communication Forwarding (when the called user is not logged in) with a correctly composed HTTP PUT request; and

2)	To verify that the UE can request deactivation of Communication Forwarding; and

3)	To verify that the UE can authenticate its HTTP requests by including a correctly composed Authorization header with credentials of the user to the request. The UE may either include the Authorization header to its initial request or when sending the request again after receiving 401 response from SS.

### 15.10.4	Method of test

Initial conditions

UE contains either ISIM and USIM applications or only USIM application on UICC. UE is configured with the name of the XCAP root directory on the XCAP server and the user's directory name. If needed the UE is also configured with the HTTP Digest password to be used for XCAP. UE has activated an IPCAN bearer (e.g. PDP context or EPS bearer) with SS.

SS is configured with the HTTP Digest password for XCAP or shared secret key of IMS AKA algorithm, related to the IMS private user identity (IMPI) configured on the UICC card equipped into the UE.

If the UE uses GAA as XCAP authentication scheme, GAA bootstrapping exchange has been performed according to annex C.29.2.

Test procedure

The generic test procedure according to annex C.29.1 is applied:

At step 1 activation of Communication Forwarding on Not logged-in;

At step 5b, SS delivers a simservs document as specified in TS 24.604 [106] cl 4.9, including a non-empty rule set. Specifically, the SS includes the following:

<?xml version="1.0" encoding="UTF-8"?>

<simservs

xmlns="http://uri.etsi.org/ngn/params/xml/simservs/xcap"

xmlns:cp="urn:ietf:params:xml:ns:common-policy"

xmlns:ocp="urn:oma:xml:xdm:common-policy">

<communication-diversion active="true">

<cp:ruleset>

<cp:rule id="rule1">

<cp:conditions>

<not-registered/>

<rule-deactivated/>

</cp:conditions>

<cp:actions>

<forward-to>

<target>

px_XCAP_TargetUri

</target>

<notify-caller>true</notify-caller>

</forward-to>

</cp:actions>

</cp:rule>

</cp:ruleset>

</communication-diversion>

</simservs>

At step 7 deactivation of Communication Forwarding on Not logged-in is respectively triggered at the UE.

### 15.10.5	Test requirements

1. SS shall check that the UE can authenticate itself correctly with the authentication scheme that the UE supports:

-	HTTP Digest authentication (see Annex C.29.1 step 2 NOTE 1) or

-	GAA based authentication as specified in TS 33.222 [121] and TS 24.109 [119] (see Annex C.29.2).

2. SS shall check that after Annex C.29.1 step 6 the simservs document stored in the SS contains the following pieces of information supplied by the UE:

-	<communication-diversion> element with "active" attribute set as "true" or with “active” attribute not present.

-	within <cp:ruleset> one <cp:rule> element for communication forwarding as follows:

-	<cp:conditions> element containing a <not-registered> element and not containing a <rule-deactivated> element

-	<cp:actions> element containing <forward-to> element containing <target> element

-	value of target address to be px_XCAP_TargetUri

NOTE:	<cp:ruleset> may also contain one additional <cp:rule> element with <cp:conditions> element containing a <not-reachable> element and not containing a <rule-deactivated>, due to the UE activating CFNRc in conjunction with CFNL.

3. SS shall check that after step 9 the simservs document stored in the SS contains the following pieces of information supplied by the UE:

-	<communication-diversion> element with "active" attribute being set "false"

or

-	<communication-diversion> element with "active" attribute being set "true" or with “active” attribute not present.

-	within <cp:ruleset> one <cp:rule>  element found at step 2 for Communication Diversion as follows:

-	<cp:conditions> element containing a <rule-deactivated> element