3GPP TSG-RAN5 Meeting #98	R5-230997

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-1 | CR | 1517 | rev | - | Current version: | 16.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of A.2.11 - MT NOTIFY for refer package |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD, MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - The NOTIFY message for refer event package is sent in the same dialog as the correponding REFER message. There is no reason or use to change the the dialog state information by sending the NOTIFY; especially the Contact URI of the SS shall not be changed but kept as provided at establishment of the dialog. - According to RFC 3515 clause 2.4.6 in case of multiple REFER requests the NOTIFY messages need to have an id parameter in the Refer header field and on the other hand in case of a single REFER the id parameter does not harm either. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Contact URI of the NOTIFY corrected to be the Contact URI of the SS as provided by the SS at establishment of the dialog. - event-param added to Event header field with parameter id set to the CSeq header field value of the corresponding REFER |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail test cases (e.g. 15.21a) |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Associated with TTCN CR: R5s221272 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change> A.2.11	MT NOTIFY for refer package

| Header/param | Cond | Value/remark | Rel | Reference |
| --- | --- | --- | --- | --- |
| Request-Line |  |  |  | RFC 3261 [15] |
| Method |  | NOTIFY |  |  |
| Request-URI |  | same URI value which the UE sent in its Contact header within the REFER request |  |  |
| SIP-Version |  | SIP/2.0 |  |  |
| Via |  | order of the parameters in this header must be like in this table |  | RFC 3261 [15] |
| via-parm1: |  |  |  |  |
| sent-protocol |  | SIP/2.0/UDP when using UDP or SIP/2.0/TCP when using TCP |  |  |
| sent-by | A1 | IP address and protected server port of SS |  |  |
| sent-by | A2 | IP address and unprotected server port of SS (optional) |  |  |
| via-branch |  | value starting with ‘z9hG4bK’ (NOTE 1) |  |  |
| via-parm2: |  | In addition to the via-parm entry for the SS, the following via-parm entries are included: |  |  |
|  |  | SIP/2.0/UDP scscf1.3gpp.org;branch=z9hG4bK..., SIP/2.0/UDP scscf2.3gpp.org;branch=z9hG4bK..., SIP/2.0/UDP pcscf2.3gpp.org;branch=z9hG4bK..., SIP/2.0/UDP uas.3gpp.org:6543;branch=z9hG4bK... (NOTE 1) |  |  |
| From |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the SS which must be the same URI as used for the SS in the earlier requests within the dialog |  |  |
| tag |  | local tag of the dialog ID |  |  |
| To |  |  |  | RFC 3261 [15] |
| addr-spec |  | SIP URI of the UE which must be the same as used for the UE in the earlier requests within the dialog. |  |  |
| tag |  | remote tag of the dialog ID |  |  |
| Call-ID |  |  |  | RFC 3261 [15] |
| callid |  | same value as in the INVITE (and REFER) message |  |  |
| CSeq | A1,A2 |  |  | RFC 3261 [15] |
| value |  | value of CSeq sent by the SS within its previous request in the same dialog but increased by one |  |  |
| method |  | NOTIFY |  |  |
| Contact |  |  |  | RFC 3261 [15] |
| addr-spec |  | Contact URI of the SS as provided by the SS at establishment of the dialog (i.e. same address as the Request URI of the corresponding REFER) |  |  |
| Event | A1,A2 |  |  | RFC 6665 [140] RFC 3515 [72] |
| event-type event-param |  | refer id=(CSeq header field value of the corresponding REFER) |  |  |
| Max-Forwards |  |  |  | RFC 3261 [15] |
| value |  | 69 |  |  |
| Subscription-State |  |  |  | RFC 6665 [140] |
| substate-value |  | active |  |  |
| expires |  | 300 |  |  |
| Content-Type |  |  |  | RFC 3261 [15] RFC 3680 [22] |
| media-type |  | message/sipfrag |  |  |
| Content-Length |  |  |  | RFC 3261 [15] RFC 3680 [22] |
| value |  | length of message-body |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | IMS security (A.6a/2 3GPP TS 34.229-2 [5]) |
| A2 | GIBA (A.6a/1 3GPP TS 34.229-2 [5]) |

NOTE 1:	Branch parameter values sent by SS are different within a test case execution.

<End of Change>