3GPP TSG-RAN5 Meeting #98	R5-230123

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4714 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Routine maintenance for TS 36.523-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test |  |  |  |  |  | Date: |  |  | 2023/02/14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | 1. The number and the title of figure 7.7.3.1.1-1 are not accurate. 2. TDD is not supported by the NB-IoT test models. 3. Editorial issues present: a) Mix of EUTRA and E-UTRA is in use. 3GPP core specifications only use E-UTRA. b) Mix of upper case and lower in the clauses (except for the 1st word). c) Mix of 'subclause' and 'clause' is in use. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. The number and title of figure 7.7.3.1.1-1 have been updated. 2. Added a general statement to document non-support of TDD. 3. Editorial issues have been resolved: a) EUTRA has been replaced by E-UTRA wherever applicable. b) Upper cases has been replaced by lower cases wherever applicable (as according to the 3GPP drafting rules). c) 'subclause' has been replaced by 'clause' wherever applicable (as according to the 3GPP drafting rules). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Information will remain inaccurate and inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.3.1.1.2, 7A.0(new), many(editorials) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 7	E-UTRAN/SAE Test Methods and Design Considerations

## 7.1	Channel Mapping

Figure 7.1 shows the channel type mapping that is used for the configuration of the SS. In layer 2 test cases non default channel mapping can be applied on SS, as explained in clause 4.2.1.

Figure 7.1-1: Channel type mapping for the default configuration of the SS

### 7.1.1	PDCCH Candidate Selection

In this clause following abbreviations are used:

-	Common search Space Aggregation: CS_Agr.

-	UE-Specific Search Space Aggregation: UE_Agr.

-	Total number of CCEs available in a subframe: Max_CCE.

SS shall apply defined rules below in a DL subframe for PDCCH candidates selection.

-	Scheduled transmissions on SI-RNTI, P-RNTI, RA-RNTI, M-RNTI, G-RNTI, SC-RNTI, SC-N-RNTI and eIMTA-RNTI, use Common Search Space. UL and DL Scheduled transmissions on C-RNTI / SPS C-RNTI, DL Scheduled transmissions on Temp. C-RNTI and UL Scheduled transmission on SL-RNTI, SL-V-RNTI, SL-SPS-V-RNTI, UL-SPS-V-RNTI use UE-Specific Search Space. Transmissions on TPC-PUCCH-RNTI / TPC-PUSCH-RNTI and UL Scheduled transmissions on Temp. C-RNTI is not considered for default CCE management.

-	If a transmission on SI-RNTI is scheduled, PDCCH candidate corresponding to CCEs between 0 and (CS_Agr-1) is used. For FDD, this PDCCH candidate is reserved for SI-RNTI, and left vacant if no SI-RNTI transmission is scheduled. For TDD the default UL/DL configuration type 1, this PDCCH candidate is reserved for SI-RNTI in subframes 0 & 5 (i.e. subframes where PDCCH for UL grant for C-RNTI / SPS C-RNTI is not scheduled).

-	PDCCH candidates corresponding to CCEs between CS_Agr and (2*CS_Agr-1) can be used for the transmission on P-RNTI, RA-RNTI, M-RNTI, G-RNTI, SC-RNTI, SC-N-RNTI or eIMTA-RNTI. In conformance test cases with single UE, there is no requirement for transmissions scheduled for more than one of P-RNTI, RA-RNTI, M-RNTI, G-RNTI, SC-RNTI, SC-N-RNTI or eIMTA-RNTI in one DL subframe. As per TS 36.331 [19] clause 5.8.1.3, the MCCH change notification indicator shall be transmitted on MBSFN subframes only.

For FDD:

-	For DL transmission for C-RNTI / SPS C-RNTI / Temp C-RNTI the lowest value of m =m' which has a PDCCH available from CCEs between 2*CS_Agr and (Max_CCE-1) shall be used. 'm' is defined in TS 36.213 [30], clause 9.1.1.

-	For UL transmission for C-RNTI / SPS C-RNTI / SL-RNTI / SL-V-RNTI / SL-SPS-V-RNTI / UL-SPS-V-RNTI the lowest value of m =m">m' which has a PDCCH available from CCEs between 2*CS_Agr and (Max_CCE-1) shall be used, irrespective of PDCCH candidate corresponding to m' is used or not. In conformance test cases with single UE, there is no requirement for transmissions scheduled for more than one of C-RNTI / SPS C-RNTI or SL-RNTI / SL-V-RNTI / SL-SPS-V-RNTI / UL-SPS-V-RNTI in one DL subframe.

For TDD:

-	For DL transmission, for C-RNTI / SPS C-RNTI / Temp C-RNTI the lowest value of m =m' which has a PDCCH available from CCEs between 1*CS_Agr and (Max_CCE-1) shall be used. 'm' is defined in TS 36.213 [30] clause 9.1.1. In conformance test cases with single UE, there is no requirement for transmissions scheduled for more than one of C-RNTI / SPS C-RNTI / Temp C-RNTI, P-RNTI, RA-RNTI or eIMTA-RNTI in one DL sub frame.

-	For UL transmission, for C-RNTI / SPS C-RNTI / SL-RNTI / SL-V-RNTI / SL-SPS-V-RNTI / UL-SPS-V-RNTI the lowest value of m =m"and if in the sub frame m' is also used, m"!=m' has a PDCCH available from CCEs between 0 and (Max_CCE-1) shall be used. In conformance test cases with single UE, there is no requirement for transmissions scheduled for more than one of C-RNTI / SPS C-RNTI, SL-RNTI / SL-V-RNTI / SL-SPS-V-RNTI / UL-SPS-V-RNTI or eIMTA-RNTI in one DL subframe.

NOTE:	If m' or m'' cannot be allocated in any TTI, it is a TTCN error due to X-RNTI not properly allocated. The error shall be reported to TTCN. The TTCN will exit the test case assigning an inconclusive verdict.

#### 7.1.1.1	FDD candidates selection

Table 7.1.1.1-1 gives the CCE resources utilized for m' and m'' for default values of common search space aggregation level =4, UE-specific search space aggregation L=2 resulting in 6 PDCCH candidates m=0..5 and channel Bandwidth of 5 MHz. This give Max_CCE =20 for FDD. The table also gives the corresponding CCE start indices of PDCCH candidates for m' and m''.

Table 7.1.1.1-1: CCE Start indices(m' & m'') to be used for various C-RNTIs (5 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 0 | 1 | 0 | 0 | 0 | 3 | 4 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 12 | 8 | 14 | 8 | 12 | 8 | 8 | 8 | 14 | 10 |
|  |  | m'' | 1 | 2 | 1 | 1 | 1 | 4 | 5 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 14 | 10 | 16 | 10 | 14 | 10 | 10 | 10 | 16 | 12 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 0 | 2 | 0 | 0 | 4 | 4 | 1 | 0 | 0 |
|  |  | CCE_St_Ind' | 12 | 16 | 8 | 14 | 10 | 8 | 8 | 8 | 18 | 16 |
|  |  | m'' | 1 | 1 | 3 | 1 | 1 | 5 | 5 | 2 | 5 | 1 |
|  |  | CCE_St_Ind'' | 14 | 18 | 10 | 16 | 12 | 10 | 10 | 10 | 8 | 18 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 0 | 0 | 0 | 2 | 3 | 0 | 0 | 0 | 0 | 4 |
|  |  | CCE_St_Ind' | 16 | 10 | 14 | 8 | 8 | 10 | 14 | 8 | 18 | 8 |
|  |  | m'' | 1 | 1 | 1 | 3 | 4 | 1 | 1 | 1 | 5 | 5 |
|  |  | CCE_St_Ind'' | 18 | 12 | 16 | 10 | 10 | 12 | 16 | 10 | 8 | 10 |
| tsc_C_RNTI_Def4 | '1FF1'H 8177 | m' | 0 | 0 | 0 | 0 | 3 | 0 | 0 | 0 | 2 | 4 |
|  |  | CCE_St_Ind' | 12 | 12 | 18 | 16 | 8 | 18 | 18 | 18 | 8 | 8 |
|  |  | m'' | 1 | 1 | 5 | 1 | 4 | 5 | 5 | 5 | 3 | 5 |
|  |  | CCE_St_Ind'' | 14 | 14 | 8 | 18 | 10 | 8 | 8 | 8 | 10 | 10 |
| tsc_C_RNTI_Def5 | '04D2'H 1234 | m' | 0 | 2 | 0 | 4 | 0 | 2 | 3 | 0 | 1 | 0 |
|  |  | CCE_St_Ind' | 10 | 8 | 10 | 8 | 14 | 8 | 8 | 14 | 8 | 10 |
|  |  | m'' | 1 | 3 | 1 | 5 | 1 | 3 | 4 | 1 | 2 | 1 |
|  |  | CCE_St_Ind'' | 12 | 10 | 12 | 10 | 16 | 10 | 10 | 16 | 10 | 12 |
| tsc_C_RNTI_Def6 | '0929'H 2345 | m' | 4 | 0 | 4 | 0 | 0 | 1 | 3 | 3 | 4 | 2 |
|  |  | CCE_St_Ind' | 8 | 10 | 8 | 12 | 14 | 8 | 8 | 8 | 8 | 8 |
|  |  | m'' | 5 | 1 | 5 | 1 | 1 | 2 | 4 | 4 | 5 | 3 |
|  |  | CCE_St_Ind'' | 10 | 12 | 10 | 14 | 16 | 10 | 10 | 10 | 10 | 10 |
| tsc_C_RNTI_Def7 | '0D80'H 3456 | m' | 2 | 0 | 2 | 0 | 0 | 0 | 3 | 0 | 0 | 2 |
|  |  | CCE_St_Ind' | 8 | 16 | 8 | 18 | 14 | 14 | 8 | 16 | 14 | 8 |
|  |  | m'' | 3 | 1 | 3 | 5 | 1 | 1 | 4 | 1 | 1 | 3 |
|  |  | CCE_St_Ind'' | 10 | 18 | 10 | 8 | 16 | 16 | 10 | 18 | 16 | 10 |
| tsc_C_RNTI_Def8 | '11D7'H 4567 | m' | 0 | 0 | 0 | 2 | 0 | 0 | 3 | 2 | 0 | 2 |
|  |  | CCE_St_Ind' | 8 | 16 | 8 | 8 | 14 | 16 | 8 | 8 | 8 | 8 |
|  |  | m'' | 1 | 1 | 1 | 3 | 1 | 1 | 4 | 3 | 1 | 3 |
|  |  | CCE_St_Ind'' | 10 | 18 | 10 | 10 | 16 | 18 | 10 | 10 | 10 | 10 |
| tsc_C_RNTI_Def9 | '162E'H 5678 | m' | 0 | 3 | 0 | 0 | 0 | 2 | 0 | 0 | 3 | 2 |
|  |  | CCE_St_Ind' | 12 | 8 | 12 | 16 | 8 | 8 | 16 | 18 | 8 | 8 |
|  |  | m'' | 1 | 4 | 1 | 1 | 1 | 3 | 1 | 5 | 4 | 3 |
|  |  | CCE_St_Ind'' | 14 | 10 | 14 | 18 | 10 | 10 | 18 | 8 | 10 | 10 |
| tsc_C_RNTI_Def10 | '1A85'H 6789 | m' | 0 | 0 | 0 | 3 | 0 | 1 | 0 | 1 | 3 | 2 |
|  |  | CCE_St_Ind' | 16 | 8 | 16 | 8 | 8 | 8 | 16 | 8 | 8 | 8 |
|  |  | m'' | 1 | 1 | 1 | 4 | 1 | 2 | 1 | 2 | 4 | 3 |
|  |  | CCE_St_Ind'' | 18 | 10 | 18 | 10 | 10 | 10 | 18 | 10 | 10 | 10 |

Table 7.1.1.1-1a gives the CCE resources utilized for m' and m'' for default values of common search space aggregation level =4, UE-specific search space aggregation L=2 resulting in 6 PDCCH candidates m=0..5 and channel Bandwidth of 5 MHz and CFI=2, suitable for eMBMS test cases. This give Max_CCE =12 for FDD. The table also gives the corresponding CCE start indices of PDCCH candidates for m' and m''.

Table 7.1.1.1-1a: CCE Start indices(m' & m'') to be used for various C-RNTIs with CFI=2 (5 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 4 | 3 | 3 | 0 | 2 | 1 | 2 | 0 | 3 | 1 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 | 8 |
|  |  | m'' | 5 | 4 | 4 | 1 | 3 | 2 | 3 | 1 | 4 | 2 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 | 10 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 4 | 0 | 0 | 3 | 4 | 0 | 0 | 1 | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | 8 | 10 | 8 | 8 | 8 | 10 | 8 | 8 |
|  |  | m'' | 1 | 5 | 1 | 5 | 4 | 5 | 1 | 5 | 2 | 1 |
|  |  | CCE_St_Ind'' | 10 | 10 | 10 | 8 | 10 | 10 | 10 | 8 | 10 | 10 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 2 | 0 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 2 |
|  |  | CCE_St_Ind' | 8 | 10 | 10 | 8 | 10 | 10 | 10 | 8 | 10 | 8 |
|  |  | m'' | 3 | 5 | 5 | 5 | 5 | 5 | 5 | 3 | 5 | 3 |
|  |  | CCE_St_Ind'' | 10 | 8 | 8 | 10 | 8 | 8 | 8 | 10 | 8 | 10 |

Tables 7.1.1.1-2, 7.1.1.1-3 and 7.1.1.1-4 give the CCE resources utilized for m' and m'' for default values of common search space aggregation level =4, UE-specific search space aggregation L=2 resulting in 6 PDCCH candidates m=0..5 and bandwidths of 10/15/20 MHz respectively. This gives Max_CCE =25(10 MHz)/37(15 MHz)/50(20 MHz) for FDD. The tables also give the corresponding CCE start indices of PDCCH candidates for m' and m''. These are in general to be applied in MAC Transport block size test cases defined in clause 7.1.7 of TS 36.523-1 [1].

Table 7.1.1.1-2: CCE Start indices (m' & m'') to be used for default C-RNTI (10 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 0 | 3 | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 12 | 8 | 8 | 20 | 16 | 18 | 16 | 8 | 14 | 18 |
|  |  | m'' | 1 | 4 | 4 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 14 | 10 | 10 | 22 | 18 | 20 | 18 | 10 | 16 | 20 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 4 | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | 20 | 10 | 14 | 8 | 20 | 22 | 18 | 8 |
|  |  | m'' | 1 | 5 | 1 | 1 | 1 | 5 | 1 | 5 | 1 | 1 |
|  |  | CCE_St_Ind'' | 10 | 10 | 22 | 12 | 16 | 10 | 22 | 8 | 20 | 10 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 2 | 0 | 0 |
|  |  | CCE_St_Ind' | 16 | 10 | 10 | 8 | 22 | 22 | 22 | 8 | 10 | 16 |
|  |  | m'' | 1 | 1 | 1 | 5 | 5 | 5 | 5 | 3 | 1 | 1 |
|  |  | CCE_St_Ind'' | 18 | 12 | 12 | 10 | 8 | 8 | 8 | 10 | 12 | 18 |
| tsc_C_RNTI_Def4 | '1FF1'H 8177 | m' | 2 | 0 | 0 | 4 | 0 | 0 | 3 | 0 | 2 | 0 |
|  |  | CCE_St_Ind' | 8 | 20 | 14 | 8 | 10 | 18 | 8 | 22 | 8 | 12 |
|  |  | m'' | 3 | 1 | 1 | 5 | 1 | 1 | 4 | 5 | 3 | 1 |
|  |  | CCE_St_Ind'' | 10 | 22 | 16 | 10 | 12 | 20 | 10 | 8 | 10 | 14 |
| tsc_C_RNTI_Def5 | '04D2'H 1234 | m' | 3 | 0 | 0 | 0 | 0 | 2 | 3 | 3 | 1 | 0 |
|  |  | CCE_St_Ind' | 8 | 16 | 22 | 12 | 22 | 8 | 8 | 8 | 8 | 22 |
|  |  | m'' | 4 | 1 | 5 | 1 | 5 | 3 | 4 | 4 | 2 | 5 |
|  |  | CCE_St_Ind'' | 10 | 18 | 8 | 14 | 8 | 10 | 10 | 10 | 10 | 8 |
| tsc_C_RNTI_Def6 | '0929'H 2345 | m' | 0 | 0 | 2 | 2 | 0 | 1 | 0 | 0 | 0 | 2 |
|  |  | CCE_St_Ind' | 20 | 18 | 8 | 8 | 18 | 8 | 18 | 22 | 12 | 8 |
|  |  | m'' | 1 | 1 | 3 | 3 | 1 | 2 | 1 | 5 | 1 | 3 |
|  |  | CCE_St_Ind'' | 22 | 20 | 10 | 10 | 20 | 10 | 20 | 8 | 14 | 10 |
| tsc_C_RNTI_Def7 | '0D80'H 3456 | m' | 4 | 0 | 0 | 1 | 0 | 0 | 0 | 0 | 0 | 4 |
|  |  | CCE_St_Ind' | 8 | 20 | 20 | 8 | 14 | 22 | 10 | 8 | 18 | 8 |
|  |  | m'' | 5 | 1 | 1 | 2 | 1 | 5 | 1 | 1 | 1 | 5 |
|  |  | CCE_St_Ind'' | 10 | 22 | 22 | 10 | 16 | 8 | 12 | 10 | 20 | 10 |
| tsc_C_RNTI_Def8 | '11D7'H 4567 | m' | 2 | 0 | 0 | 0 | 0 | 4 | 3 | 2 | 4 | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | 12 | 8 | 10 | 8 | 8 | 8 | 8 | 20 |
|  |  | m'' | 3 | 1 | 1 | 1 | 1 | 5 | 4 | 3 | 5 | 1 |
|  |  | CCE_St_Ind'' | 10 | 10 | 14 | 10 | 12 | 10 | 10 | 10 | 10 | 22 |
| tsc_C_RNTI_Def9 | '162E'H 5678 | m' | 0 | 0 | 2 | 4 | 0 | 0 | 2 | 0 | 1 | 0 |
|  |  | CCE_St_Ind' | 8 | 10 | 8 | 8 | 16 | 16 | 8 | 14 | 8 | 16 |
|  |  | m'' | 1 | 1 | 3 | 5 | 1 | 1 | 3 | 1 | 2 | 1 |
|  |  | CCE_St_Ind'' | 10 | 12 | 10 | 10 | 18 | 18 | 10 | 16 | 10 | 18 |
| tsc_C_RNTI_Def10 | '1A85'H 6789 | m' | 0 | 0 | 0 | 3 | 0 | 0 | 0 | 0 | 3 | 0 |
|  |  | CCE_St_Ind' | 12 | 12 | 20 | 8 | 12 | 18 | 20 | 10 | 8 | 12 |
|  |  | m'' | 1 | 1 | 1 | 4 | 1 | 1 | 1 | 1 | 4 | 1 |
|  |  | CCE_St_Ind'' | 14 | 14 | 22 | 10 | 14 | 20 | 22 | 12 | 10 | 14 |

Table 7.1.1.1-3: CCE Start indices (m' & m'') to be used for default C-RNTI (15 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 4 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 8 | 14 | 14 | 20 | 16 | 18 | 28 | 20 | 26 | 30 |
|  |  | m'' | 5 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 10 | 16 | 16 | 22 | 18 | 20 | 30 | 22 | 28 | 32 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 0 | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 32 | 12 | 20 | 34 | 14 | 8 | 8 | 10 | 30 | 32 |
|  |  | m'' | 1 | 1 | 1 | 5 | 1 | 5 | 1 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 34 | 14 | 22 | 8 | 16 | 10 | 10 | 12 | 32 | 34 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 0 | 0 | 0 | 4 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 16 | 22 | 22 | 8 | 10 | 22 | 34 | 28 | 34 | 28 |
|  |  | m'' | 1 | 1 | 1 | 5 | 1 | 1 | 5 | 1 | 5 | 1 |
|  |  | CCE_St_Ind'' | 18 | 24 | 24 | 10 | 12 | 24 | 8 | 30 | 8 | 30 |

Table 7.1.1.1-4: CCE Start indices (m' & m'') to be used for default C-RNTI (20 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 3 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 2 | 0 |
|  |  | CCE_St_Ind' | 8 | 36 | 34 | 38 | 42 | 22 | 10 | 8 | 8 | 20 |
|  |  | m'' | 4 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 3 | 1 |
|  |  | CCE_St_Ind'' | 10 | 38 | 36 | 40 | 44 | 24 | 12 | 10 | 10 | 22 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 0 | 2 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |
|  |  | CCE_St_Ind' | 12 | 46 | 8 | 34 | 20 | 10 | 10 | 26 | 28 | 8 |
|  |  | m'' | 1 | 1 | 3 | 1 | 1 | 1 | 1 | 1 | 1 | 2 |
|  |  | CCE_St_Ind'' | 14 | 48 | 10 | 36 | 22 | 12 | 12 | 28 | 30 | 10 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 1 | 0 | 0 | 0 | 3 | 0 | 2 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 8 | 20 | 24 | 34 | 8 | 10 | 8 | 38 | 48 | 20 |
|  |  | m'' | 2 | 1 | 1 | 1 | 4 | 1 | 3 | 1 | 5 | 1 |
|  |  | CCE_St_Ind'' | 10 | 22 | 26 | 36 | 10 | 12 | 10 | 40 | 8 | 22 |

#### 7.1.1.2	TDD candidates selection

The default TDD subframe configuration 1 is applied to this clause. By default no DL data is scheduled in special subframes 1 and 6.

Considering that each TDD subframe having different PHICH group number, and only two symbols being present for PDCCH in the special subframes 1 and 6 for bandwidth of 5 MHz, two symbols for PDCCH in all subframes for bandwidth of 10/15/20 MHz (TS 36.508 [3]), each subframe has, therefore, different number of MAX_CCE.

Table 7.1.1.2-1 gives the PDCCH candidates of m' and m'' for default values of common search space aggregation level =8, UE-specific search space aggregation L=8 resulting in 2 PDCCH candidates m=0,1 and the corresponding CCE start indices for channel bandwidth of 5MHz. SF0 and SF5 cannot be used for UL grant. SF1 and SF6 are not used for DL assignment. SF2, SF3, SF7 and SF8 are not applicable to PDCCH CCE allocation since they are uplink subframes.

Table 7.1.1.2-1: CCE Start indices (m' & m'') to be used for various C-RNTIs (5 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max_CCE | 21 | 12 | - | - | 20 | 21 | 12 | - | - | 20 |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 1 | - | - | - | 1 | 0 | - | - | - | 0 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 0 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 1 | - | - | - | 0 | 1 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 1 | - | - | - | 0 | 0 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
| tsc_C_RNTI_Def4 | '1FF1'H 8177 | m' | 1 | - | - | - | 0 | 0 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
| tsc_C_RNTI_Def5 | '04D2'H 1234 | m' | 0 | - | - | - | 0 | 1 | - | - | - | 0 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
| tsc_C_RNTI_Def6 | '0929'H 2345 | m' | 1 | - | - | - | 0 | 0 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 10 | - | - | 0 |
| tsc_C_RNTI_Def7 | '0D80'H 3456 | m' | 1 | - | - | - | 0 | 0 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
| tsc_C_RNTI_Def8 | '11D7'H 4567 | m' | 1 | - | - | - | 0 | 0 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
| tsc_C_RNTI_Def9 | '162E'H 5678 | m' | 1 | - | - | - | 0 | 0 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
| tsc_C_RNTI_Def10 | '1A85'H 6789 | m' | 1 | - | - | - | 0 | 0 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |

Table 7.1.1.2-1a gives the PDCCH candidates of m' and m'' for default values of common search space aggregation level =4, UE-specific search space aggregation L=4 resulting in 2 PDCCH candidates m=0,1 and the corresponding CCE start indices for channel bandwidth of 5MHz and CFI=2, suitable for eMBMS test cases. SF0 and SF5 cannot be used for UL grant. SF1 and SF6 are not used for DL assignment. SF2, SF3, SF7 and SF8 are not applicable to PDCCH CCE allocation since they are uplink subframes.

Note: With Max-CCE=13 or 12, aggregation level of 8 results in only 1 PDCCH candidate. Hence aggregation level of 4 is used.

Table 7.1.1.2-1a: CCE Start indices (m' & m'') to be used for various C-RNTIs with CFI=2 (5 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max_CCE | 13 | 12 | - | - | 12 | 13 | 12 | - | - | 12 |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 1 | - | - | - | 0 | 1 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 4 | - | - | - | 8 | 4 | - | - | - | 4 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 4 | - | - | 0 | - | 8 | - | - | 0 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | - | - | - | 0 | 1 | - | - | - | 0 |
|  |  | CCE_St_Ind' | 4 | - | - | - | 4 | 4 | - | - | - | 4 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 8 | - | 4 | - | - | 8 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 0 | - | - | - | 0 | 0 | - | - | - | 0 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 8 | - | - | 0 | - | 8 | - | - | 0 |

Tables 7.1.1.2-2, 7.1.1.2-3 and 7.1.1.2-4 give the PDCCH candidates of m' and m'' for default values of common search space aggregation level =8, UE-specific search space aggregation L=8 resulting in 2 PDCCH candidates m=0,1 and the corresponding CCE start indices for bandwidths of 10/15/20 MHz respectively, with the different Max_CCE number for each subframe.

Table 7.1.1.2-2: CCE Start indices (m' & m'') to be used for default C-RNTI (10 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max_CCE | 27 | 25 | - | - | 25 | 27 | 25 | - | - | 25 |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 1 | - | - | - | 0 | 1 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 16 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 8 | - | - | 0 | - | 16 | - | - | 0 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | - | - | - | 0 | 1 | - | - | - | 0 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 16 | - | 8 | - | - | 16 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 0 | - | - | - | 0 | 0 | - | - | - | 0 |
|  |  | CCE_St_Ind' | 16 | - | - | - | 16 | 16 | - | - | - | 16 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 16 | - | - | 0 | - | 16 | - | - | 0 |

Table 7.1.1.2-3: CCE Start indices (m' & m'') to be used for default C-RNTI (15 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max_CCE | 41 | 37 | - | - | 37 | 41 | 37 | - | - | 37 |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 0 | - | - | - | 1 | 0 | - | - | - | 0 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 0 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 8 | - | - | 0 | - | 0 | - | - | 16 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | - | - | - | 0 | 1 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | - | - | - | 24 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 16 | - | - | 0 |
| tsc_C_RNTI_Def3 | '1111'H 4369 | m' | 0 | - | - | - | 0 | 1 | - | - | - | 1 |
|  |  | CCE_St_Ind' | 24 | - | - | - | 24 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 8 | - | - | 0 | - | 24 | - | - | 0 |

Table 7.1.1.2-4: CCE Start indices (m' & m'') to be used for default C-RNTI (20 MHz)

| C-RNTI | Value |  |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Max_CCE | 55 | 50 | - | - | 50 | 55 | 50 | - | - | 50 |
| tsc_C_RNTI_Def | '1001'H 4097 |  | m' | 1 | - | - | - | 0 | 0 |  | - | - | 0 |
|  |  |  | CCE_St_Ind' | 8 | - | - | - | 16 | 24 |  | - | - | 24 |
|  |  |  | m'' | - | 0 | - | - | 1 |  | 0 | - | - | 1 |
|  |  |  | CCE_St_Ind'' | - | 8 | - | - | 24 |  | 16 | - | - | 32 |
| tsc_C_RNTI_Def2 |  | ‘1034'H 4148 | m' | 0 | - | - | - | 0 | 1 | - | - | - | 0 |
|  |  |  | CCE_St_Ind' | 32 | - | - | - | 8 | 8 | - | - | - | 32 |
|  |  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  |  | CCE_St_Ind'' | - | 0 | - | - | 16 | - | 32 | - | - | 40 |
| tsc_C_RNTI_Def3 |  | ‘1111'H 4369 | m' | 0 | - | - | - | 0 | 0 | - | - | - | 0 |
|  |  |  | CCE_St_Ind' | 16 | - | - | - | 40 | 40 | - | - | - | 16 |
|  |  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  |  | CCE_St_Ind'' | - | 40 | - | - | 0 | - | 40 | - | - | 24 |
| tsc_C_RNTI_Def4 |  | ‘1FF1'H 8177 | m' | 0 | - | - | - | 0 | 0 | - | - | - | 1 |
|  |  |  | CCE_St_Ind' | 16 | - | - | - | 40 | 24 | - | - | - | 8 |
|  |  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  |  | CCE_St_Ind'' | - | 32 | - | - | 0 | - | 8 | - | - | 0 |
| tsc_C_RNTI_Def5 |  | ‘04D2'H 1234 | m' | 0 | - | - | - | 0 | 0 | - | - | - | 0 |
|  |  |  | CCE_St_Ind' | 8 | - | - | - | 40 | 16 | - | - | - | 40 |
|  |  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  |  | CCE_St_Ind'' | - | 16 | - | - | 0 | - | 8 | - | - | 0 |
| tsc_C_RNTI_Def6 |  | ‘0929'H 2345 | m' | 0 | - | - | - | 0 | 0 | - | - | - | 0 |
|  |  |  | CCE_St_Ind' | 32 | - | - | - | 24 | 24 | - | - | - | 16 |
|  |  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  |  | CCE_St_Ind'' | - | 24 | - | - | 32 | - | 24 | - | - | 24 |
| tsc_C_RNTI_Def7 |  | ‘0D80'H 3456 | m' | 1 | - | - | - | 0 | 0 | - | - | - | 1 |
|  |  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 40 | - | - | - | 8 |
|  |  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  |  | CCE_St_Ind'' | - | 32 | - | - | 16 | - | 40 | - | - | 0 |
| tsc_C_RNTI_Def8 |  | ‘11D7'H 4567 | m' | 0 | - | - | - | 0 | 1 | - | - | - | 0 |
|  |  |  | CCE_St_Ind' | 16 | - | - | - | 40 | 8 | - | - | - | 32 |
|  |  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  |  | CCE_St_Ind'' | - | 32 | - | - | 0 | - | 8 | - | - | 40 |
| tsc_C_RNTI_Def9 |  | ‘162E'H 5678 | m' | 0 | - | - | - | 0 | 0 | - | - | - | 0 |
|  |  |  | CCE_St_Ind' | 32 | - | - | - | 16 | 16 | - | - | - | 16 |
|  |  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  |  | CCE_St_Ind'' | - | 40 | - | - | 24 | - | 16 | - | - | 24 |
| tsc_C_RNTI_Def10 |  | ‘1A85'H 6789 | m' | 1 | - | - | - | 1 | 0 | - | - | - | 1 |
|  |  |  | CCE_St_Ind' | 8 | - | - | - | 8 | 24 | - | - | - | 8 |
|  |  |  | m'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
|  |  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 32 | - | - | 0 |

#### 7.1.1.2.1	TDD candidates selection in special subframes

In test cases (7.1.3.12/13) testing DL data transmission in special subframes following CCE tables will be used

Table 7.1.1.2.1-1: CCE Start indices (m' & m'') to be used for various default C-RNTI (5 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max_CCE | 21 | 12 | - | - | 20 | 21 | 12 | - | - | 20 |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 1 | 1 | - | - | 1 | 0 | 1 | - | - | 0 |
|  |  | CCE_St_Ind' | 8 | 0 | - | - | 8 | 8 | 0 | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 0 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |
| tsc_C_RNTI_Def2 | ‘1034'H 4148 | m' | 1 | 1 | - | - | 0 | 1 | 1 | - | - | 1 |
|  |  | CCE_St_Ind' | 8 | 0 | - | - | 8 | 8 | 0 | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 0 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 0 | - | 0 | - | - | 0 |

NOTE:	Special subframes with Max CCE =12 and aggregation of 8 result in only 1 PDCCH. Hence either only UL grant or DL allocation is possible.

Table 7.1.1.2.1-2: CCE Start indices (m' & m'') to be used for default C-RNTI (20 MHz)

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max_CCE | 55 | 50 | - | - | 50 | 55 | 50 | - | - | 50 |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 1 | 0 | - | - | 0 | 0 | 0 | - | - | 0 |
|  |  | CCE_St_Ind' | 8 | 8 | - | - | 16 | 24 | 16 | - | - | 24 |
|  |  | m'' | - | 1 | - | - | 1 | - | 1 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 16 | - | - | 24 | - | 24 | - | - | 32 |
| tsc_C_RNTI_Def2 | ‘1034'H 4148 | m' | 0 | 1 | - | - | 0 | 1 | 0 | - | - | 0 |
|  |  | CCE_St_Ind' | 32 | 8 | - | - | 8 | 8 | 32 | - | - | 32 |
|  |  | m'' | - | 0 | - | - | 1 | - | 1 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 16 | - | 40 | - | - | 40 |

### 7.1.2	ePDCCH Candidate Selection

Total number of eREGs available in a subframe: Max_eREG.

Total number of eCCEs available in a subframe: Max_eCCE.

The value of numberPRB-Pairs=8 to be used in ePDCCH signalling test cases result in MAX_eREG=128 as per TS 36.211 [35] clause 6.2.4A. As per TS 36.211 [35] clause 6.8A.1 results in MAX_eCCE=32.

#### 7.1.2.1	FDD candidates selection

The UE specific search space aggregation L=2 same as in clause 7.1.1.1 is used. For DL transmission for C-RNTI / SPS C-RNTI value of m=0 (m’) in set config ID 0 shall be used. 'm' is defined in TS 36.213 [30], clause 9.1.4. For UL transmission for C-RNTI / SPS C-RNTI the lowest value of m=1 (m”) shall be used.

Table 7.1.2.1-1: eCCE Start indices (m' & m'') to be used for various C-RNTIs

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 20 | 2 | 10 | 28 | 0 | 26 | 16 | 8 | 22 | 2 |
|  |  | m'' | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 4 | 18 | 26 | 12 | 16 | 10 | 0 | 24 | 6 | 18 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
|  |  | CCE_St_Ind' | 8 | 24 | 4 | 26 | 14 | 0 | 4 | 22 | 26 | 8 |
|  |  | m'' | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | CCE_St_Ind'' | 24 | 8 | 20 | 10 | 30 | 16 | 20 | 6 | 10 | 24 |

NOTE:	eCCE start indices are independent of bandwidth.

#### 7.1.2.2	TDD candidates selection

The search space aggregation L=8 same as in clause 7.1.1.2 is used. For DL transmission for C-RNTI / SPS C-RNTI value of m=0 (m’) in set config ID 0 shall be used. 'm' is defined in TS 36.213 [30], clause 9.1.4. For UL transmission for C-RNTI / SPS C-RNTI the lowest value of m=1(m”) shall be used.

Table 7.1.2.2-1: CCE Start indices (m' & m'') to be used for various C-RNTIs

| C-RNTI | Value |  | SF0 | SF1 | SF2 | SF3 | SF4 | SF5 | SF6 | SF7 | SF8 | SF9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Max_CCE | 21 | 12 | - | - | 20 | 21 | 12 | - | - | 20 |
| tsc_C_RNTI_Def | '1001'H 4097 | m' | 0 | - | - | - | 0 | 0 | - | - | - | 0 |
|  |  | CCE_St_Ind' | 16 | - | - | - | 0 | 8 | - | - | - | 8 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 8 | - | - | 16 | - | 0 | - | - | 24 |
| tsc_C_RNTI_Def2 | '1034'H 4148 | m' | 0 | - | - | - | 0 | 0 | - | - | - | 0 |
|  |  | CCE_St_Ind' | 0 | - | - | - | 24 | 0 | - | - | - | 0 |
|  |  | m'' | - | 0 | - | - | 1 | - | 0 | - | - | 1 |
|  |  | CCE_St_Ind'' | - | 0 | - | - | 8 | - | 0 | - | - | 16 |

### 7.1.3	MPDCCH Candidate Selection

The value of numberPRB-Pairs=2+4 (numberPRB-Pairs-v1310= n6) is to be used for MPDCCH in signalling test cases. The DCI format 6-2 used for P-RNTI will be sent in Type1-common search space. The DCI format 6-1A and 6-1B scrambled with RA-RNTI will be sent in Type2-common search space. The DCI format 6-1A and 1B scrambled by C-RNTI and DCI format 6-0A and 6-0B can be sent either in Type0-common (CEModeA only) or UE Specific by C-RNTI search space. For simplicity and to be applicable for both CE Modes, they will always be sent on UE Specific by C-RNTI search space.

It is assumed that transmissions based on P-RNTI, C-RNTI and RA-RNTI are mutually exclusive in a one UE test environment. In a subframe any one of following will happen:

-	1 DCI for P-RNTI

-	1 DCI for RA-RNTI

-	1 or 2 DCI(s) for C-RNTI

For TDD with default ‘specialSubframePatterns’ = ssp6, as per 36.213[30] clause 9.1.5, UE is not required to monitor MPDCCH in special subframes for both CE Mode’s A and B.

For Type1-common search space for P-RNTI, as per 36.213[30] clause 9.1.5 only one MPDCCH with Aggregation L’ = 24 is defined. This requires all 6 PRB’s to be used. For this eCCE’s with 0 to 23 (i.e. all eCCE’s) are used.

For Type2-common search space for RA-RNTI, Aggregation L’ of 24 results in only one MPDCCH utilising all 6 PRB’s. For this eCCE’s with 0 to 23 (i.e. all eCCE’s) are used.

For UE Specific by C-RNTI search space, if only one of DCI 6-1x and 6-0x is to be sent, then Aggregation L’ of 24 results in only one MPDCCH utilising all 6 PRB’s. For this eCCE’s with 0 to 23 (i.e. all eCCE’s) are used.

For UE Specific by C-RNTI search space, if both UL and DL DCI (6-1x and 6-0x) are to be sent then:

-	For CE Mode B with R=r1, aggregation level L’=8 on 2 PRB’s (the lowest 2 PRB’s in Narrow band), resulting in only one MPDCCH utilising all 8 eCCE’s with 0 to 7 are used. This is used for DCI 6-1x (36.213[30] table 9.1.5-2b).

-	For CE Mode B with R=r1, aggregation level L’=16 on 4 PRB’s (the highest 4 PRB’s in Narrow band), resulting in only one MPDCCH utilising all 16 eCCE’s with 0 to 15 are used. This is used for DCI 6-0x(36.213[30] table 9.1.5-2b).

-	For CE Mode A with R=r1, level L’=8 on 4 PRB’s (the highest 4 PRB’s in Narrow band), resulting in two MPDCCH, MPDCCH 1 with CCE’s 0-7are used for DCI 6- 1x and  MPDCCH 2 with CCE’s 8-15are used for DCI 6- 0x(36.213[30] table 9.1.5-2a).

MPDCCH is repeated based on semi-statically configured higher layer parameter mpdcch-NumRepetition and dynamically selected by network. Each value of mpdcch-NumRepetition = rmax maps into max 4 allowed values of repetitions R = r1, r2, r3, r4. UE is required to monitor all r1, r2, r3 & r4 as per 36.213[30] table 9.1.5-2a for CE Mode A and table 9.1.5-2b for CE mode B.

Following values shall be used for DCI subframe repetition number R based on rmax= mpdcch-NumRepetition default values:

-	Type1-common search space for P-RNTI, mpdcch-NumRepetition-Paging-r13 =r16, R=r1=1 for both CE-Modes A & B.

-	Type2-common search space for RA-RNTI, mpdcch-NumRepetition-RA-r13 =r1, R=r1=1, resulting in no retransmissions is used for both CE-Modes A & B.

-	Type2-common search space for RA-RNTI, mpdcch-NumRepetition-RA-r13 =r4, R=r1=1, resulting in no retransmissions is used for both CE-Modes A & B.

-	Type2-common search space for RA-RNTI, mpdcch-NumRepetition-RA-r13 =r16, R=r1=2, resulting in 2 transmissions is used for both CE-Modes A & B.

-	Type2-common search space for RA-RNTI, mpdcch-NumRepetition-RA-r13 =r64, R=r1=2, resulting in 8 transmissions is used for both CE-Modes A & B.

-	UE Specific by C-RNTI search space, mpdcch-NumRepetition-r13 =r16, R=r1=2 resulting in 2 transmissions.

-	UE Specific by C-RNTI search space, mpdcch-NumRepetition-r13 =r1, R=r1=1 resulting in 1 transmission.

### 7.1.4	LAA Considerations

In a LAA cell, the UE-Specific Search Space selection, as defined in above clauses for TDD, is applicable. As a UE cannot camp on a LAA cell, as there is no transmission of System Info, hence the CCE’s dedicated for SI-RNTI will be available for allocation. In a LAA Scell, the PDCCH scrambled with CC-RNTI PDCCH candidate at aggregation level L=8 with the CCEs corresponding to the PDCCH candidate given by CCEs numbered 0,1,2,3,4,5,6,7 shall be used, as per TS 36.213[30] clause 13A.

## 7.2	Uplink Grant

### 7.2.0	UL Sync and Grant Allocation Types

The Network/SS informs the UE if it is allowed to make Uplink Data transmission by transmitting 'DCI format 0' on PDCCH. The UE shall transmit (4 TTI later for FDD or variable for TDD) a Transport Block (TB) of exactly the same size as specified in DCI format 0. The UE has no control of its own on TB size, and has to merely follow the network, even if that means lots of MAC padding or resource starving.

The UE has the following means to communicate if it has UL data ready for transmission and subsequently the estimate of quantity of data to be transmitted.

-	RACH procedure: UE in idle mode, handed over to a new cell or connected mode but PUCCH is unsynchronized (sometimes referred to as PUCCH is not configured) will trigger RACH procedure on data ready for transmission in UL.

-	Scheduling Request: UE in connected mode, no grant configured, PUCCH is synchronized and has data ready for transmission in UL, will transmit a scheduling request on PUCCH.

-	Buffer Status Reports: UE in connected mode, PUCCH synchronized, has a configured grant for current TTI, but grant is not sufficient to transmit all the data will include MAC control element BSR in the UL MAC PDU.

RACH and SR indicate on data availability and BSR provides an estimate of data available for transmission.

CQI/PMI/RI feedback from the UE which indicates the channel conditions and recommended number of layers.

Hence to determine the exact need of the grant requirement of the UE a network/SS needs to act on all four of the above. This eventually complicates the SS implementation and hence the grant allocation procedure is simplified such that SS needs only to react on reception of SR and grant allocation configured from the TTCN.

The SS disables aperiodic CQI/PMI/RI feedback from the UE by setting the ‘CQI request field’ to 0 in DCI format 0/RAR grant.

When request for periodic CQI/PMI/RI feedback is requested due to TTCN configuration, the SS does not react on periodic CQI/PMI/RI feedback received and still allocates grants as configured from TTCN.

The SS, if configured for maintaining PUCCH synchronization at UE, shall periodically transmit automatically MAC PDUs containing the MAC control element 'Timing Advance'. The period as configured by the TTCN is set to 80 % of the 'Time Alignment Timer' default value (750 ms) configured at UE.

Additionally the SS can be configured to automatically transmit a 'configured' UL grant at every reception of a SR. This grant should be selected under the following restrictions:

-	All UE categories can handle this, i.e. (for UE Cat 1 and above TBS ≤ 5160 bits, for UE Cat 0/M1 TBS ≤ 1000 bits).

-	It is sufficiently large that most of UL signalling messages can be transmitted. In case the grant is not sufficient to fit the whole UL data, the UE will have to wait for the expiry of retxBSR-Timer and retransmit a SR. And hence the procedure is repeated.

The following 5 types of grant allocation configurations are possible. Grant allocation types 1 to 3 are applicable, when the UE is in connected state. Grant allocation type 4 is applicable when UE is establishing / re-establishing the RRC Connection, or during handover or in connected state but PUCCH is not synchronised. Grant allocation type 5 is applicable when the UE is in connected state and is used for lower category UEs when the UL grants greater than Max TBS supported by the UE is needed.

Grant Allocation Type 1:

-	SS is configured to maintain PUCCH Synch.

-	SS is configured to send an automatically 'configured Grant' (in terms of IMCS and NPRB) to the UE on every reception of a Scheduling Request, within 10 subframes.

-	defined in Table 7.2.0-1The default configured grant is, unless explicitly specified in test cases.

-	By default this type of grant allocation is applied. The majority of Idle mode, RRC and NAS test cases, the preambles of all tests and the postambles of those tests for which UE is still PUCCH synchronised at the end of test body. A few Layer 2 tests also use this type of grant.

Grant Allocation Type 2:

-	SS is configured to maintain PUCCH Synch.

-	SS is configured to periodically transmit a grant (IMCS and NPRB). Number of grants (1 or more) and period (in ms) is configured by TTCN.

-	The first grant transmitted is as specified in the explicit timing information. If timing information is "now" the SS selects the first suitable subframe for UL transmission.

-	The grant allocation period for TDD shall be assigned without conflict with the allowed UL subframes in the TDD subframe configurations. As example of allocation period, the TDD UL Grant allocation can be assigned as in multiples of 5 ms.

-	This type of grant allocation is applicable to the majority of RLC, PDCP and a few MAC test cases.

-	No additional grant is allocated on reception of any SRs.

Grant Allocation Type 3:

-	SS may or may not be configured to maintain PUCCH Synch.

-	SS is configured to transmit a one time grant (IMCS and NPRB) in the time requested by TTCN. The one time transmission is achieved by setting Number of grants=1 and period=Only once.

-	This type of grant allocation is suitable for MAC and DRB tests when UE is in UL Synchronised state.

Grant Allocation Type 4 (RACH configuration):

-	In addition to the 3 types of UL grant allocations, a fourth type of grant allocation during the RACH procedure is also possible, where the SS behaves as per the RACH procedure configured and allocates the configured grant during the RACH procedure.

-	This UL Grant type is used in the configuration for the preamble in many situations, basically in MAC test cases. This type of grant is further used when UE is establishing/re-establishing the RRC connection or during handover, or when the UE is not PUCCH synchronised.

Grant Allocation Type 5:

-	SS is configured to maintain PUCCH Synch.

-	SS is configured to periodically transmit a grant (IMCS and NPRB) on reception of SR.  Number of grants (1 or more) and period (in ms) are configured by TTCN.  The first grant is sent within 10 subframes of SR.  Reception of SR starts/restarts periodic grant transmission.

-	The default configured grant is defined in Table 7.2.0-1, unless explicitly specified in test cases.

-	This type of grant allocation is suitable when the UE category is lower than 2.  The majority of Idle mode, RRC and NAS test cases, the preambles of all tests and the postambles of those tests which UE is still PUCCH synchronised at the end of the test body.  A few Layer 2 tests also use this type of grant.

All the UL grant allocation types define grant allocation in terms of IMCS and NPRB to be used. The SS shall allocate RBs corresponding to PRB indices 0..(NPRB-1).

Table 7.2.0-1 specifies the default UL grant settings for grant allocation types 1 and 5 depending on the UE Category, whether the test case is applicable to CE mode A UEs, and whether the grant size is default or big.

Table 7.2.0-1: Default UL grant settings for grant allocation types 1 and 5

| UE Category | CE mode A test case | Grant size type | TBS (bits) {IMCS ; NPRB} | Grant allocation type | Total grant size (bits) |
| --- | --- | --- | --- | --- | --- |
| CatM1 | n/a | Default | 1000 {14 ; 4} | Type 1 | 1000 |
|  | n/a | Big | 1000 {14 ; 4} | Type 5 {20ms ; 10x} | 10000 |
| Cat0 | n/a | Default | 1000 {14 ; 4} | Type 1 | 1000 |
|  | n/a | Big | 1000 {14 ; 4} | Type 5 {20ms ; 10x} | 10000 |
| Cat1/1bis | yes | Default | 1000 {14 ; 4} | Type 1 | 1000 |
|  |  | Big | 1000 {14 ; 4} | Type 5 {20ms ; 10x} | 10000 |
|  | no | Default | 4008 {9 ; 25} | Type 1 | 4008 |
|  |  | Big | 4968 {12 ; 25} | Type 5 {20ms ; 2x} | 9936 |
| CatX with X≥2 | yes | Default | 1000 {14 ; 4} | Type 1 | 1000 |
|  |  | Big | 1000 {14 ; 4} | Type 5 {20ms ; 10x} | 10000 |
|  | no | Default | 4008 {9 ; 25} | Type 1 | 4008 |
|  |  | Big | 10680 {20 ; 25} | Type 1 | 10680 |
| NOTE: For CE mode B only test cases (e.g. 7.1.4.24c), other UL Grant settings are used. |  |  |  |  |  |

The type of test cases configured with big grant size is defined in Table 7.2.0-2:

Table 7.2.0-2: Test cases configured with big grant size

| UE Category | CE mode A test case | Condition to be fulfilled to use big grant in a test case |
| --- | --- | --- |
| CatM1/0 | n/a | if the UE supports IMS |
| CatX with X≥1 | yes | if the UE supports IMS |
|  | no | if the test case includes IMS signalling within the test body |

### 7.2.1	Exception test case list

This clause contains the exception test case list where the explicit UL grant allocation types other than UL grant types 1 and 5 are specified.

Table 7.2.1-1: Exception test case list with explicit UL grant types other than UL grant type 1 and 5

| Group | Test Case | UL Grant Type 2 | UL Grant Type 3 |
| --- | --- | --- | --- |
| RLC | 7.2.2.6 | X |  |
|  | 7.2.2.7 | X |  |
|  | 7.2.3.1 |  | X |
|  | 7.2.3.2 | X |  |
|  | 7.2.3.4 |  | X |
|  | 7.2.3.5 |  | X |
|  | 7.2.3.6 | X |  |
|  | 7.2.3.7 | X |  |
|  | 7.2.3.9 | X |  |
|  | 7.2.3.10 | X | X |
|  | 7.2.3.13 | X | X |
|  | 7.2.3.15 | X |  |
|  | 7.2.3.17 | X |  |
|  | 7.2.3.18 |  | X |
|  | 7.2.3.21 |  | X |
| MAC | 7.1.4.1 | X |  |
|  | 7.1.4.2 |  | X |
|  | 7.1.4.3 | X |  |
|  | 7.1.4.4 |  | X |
|  | 7.1.4.7 |  | X |
|  | 7.1.4.8 | X | X |
|  | 7.1.4.10 |  | X |
|  | 7.1.4.11 |  | X |
|  | 7.1.4.14 |  | X |
|  | 7.1.4.15 | X |  |
|  | 7.1.4.16 | X |  |
|  | 7.1.5.1 | X |  |
|  | 7.1.5.2 | X |  |
|  | 7.1.5.3 | X |  |
|  | 7.1.5.4 | X |  |
|  | 7.1.5.5 | X |  |
|  | 7.1.6.1 |  | X |
| PDCP | 7.3.5.4 |  | X |
| RRC | 8.2.1.5 | X |  |
| NAS | 9.2.1.1.24 |  | X |
| DRB | 12.1.1 |  | X |
|  | 12.1.2 |  | X |

## 7.3	Downlink Resource Allocation

The DL resource allocation is an SS emulation function. In order to ensure similar DL behaviours (within defined tolerances) on the different SS platforms in the timing stringent requirements, all downlink resource allocation schemes specified in the present clause shall be supported by the SS.

When the DL data is to be sent with a specific scheduling requirement, for instance, in a TTI in advance rather than “now”, the TTCN shall ensure that the data is scheduled at least 100 ms in advance. The 100 ms time in general covers all time delays, from the time DL data is sent by the TTCN at the EUTRA PTC to the completion of the transmission at the SS (TTCN delays, codec delays, adaptor delays and SS processing delays at various protocol Layers). In the case of more than one NAS PDU is piggy-backed in a scheduled RRC PDU, 20ms shall be added per additional NAS PDU: 100ms + ((NoOfNASPDUs – 1) * 20ms; this calculation is based on the assumption that there are not more than 7 piggy-backed NAS messages; this is valid for LTE.

NOTE:	The DL data means DL signalling and/or data in the present clause.

### 7.3.1	PDCCH DCI default formats

Two types of DCI combinations are identified as default formats for the signalling and protocol test.

DCI combination 1 uses:

-	DCI format 1A, resource allocation type 2 localised, for all DL scheduling types.

DCI combination 2 uses:

-	DCI format 1C, resource allocation type 2 distributed, for scheduling of PCCH/BCCH/RAR; and

-	DCI format 1 resource allocation type 0, for UE dedicated scheduling.

#### 7.3.1.1	Default DCI Format to be used in test cases configuring MIMO

Transmission mode 3 will be used in MIMO test cases configuring 2 Transmit antenna SS environment. As per 36.213 Table 7.1-5, in Transmission mode 3, UE is expected to decode only DCI formats 2A and 1A. Similarly for Transmission mode 4, UE is expected to decode only DCI formats 2 and 1A. Hence for all test cases configuring 2TX (2 antenna ports) at SS, DCI combination 1 is the default DCI combination to be applied. This allows DCI format 1A to be used as default DL scheduling scheme for test sequences not explicitly specified to use DCI formats 2A or 2(i.e. preamble, postamble etc.)

### 7.3.2	Radio parameters configured

The SS shall support DL QPSK, 16QAM and 64QAM modulation schemes. The configured radio parameters, including DCI format, resource allocation types, maximum allowed modulation scheme, first virtual / physical resource block to be used, maximum available resource blocks and redundancy version, are provided to the SS.

In the normal signalling test condition, DL RLC and HARQ retransmissions are rare. The redundancy version is provided to allow the occasional HARQ retransmissions. For those MAC, RLC tests contained in table 7.3.2-1 where timing requirements are involved the DL or UL HARQ retransmissions are not tolerable. Table 7.3.2-2 lists the RLC tests where timing requirements are involved, only one DL or UL HARQ retransmission per transport block is tolerable. Unless otherwise specified, if HARQ retransmissions occur in the test cases contained in table 7.3.2-1 or more than one HARQ retransmission occurs in the test cases of table 7.3.2-2, the test cases will be terminated with verdict inconclusive.

NOTE:	If the test is expecting the reporting of UL ACK/NACK for the DL MAC PDUs, or is configuring the PHICH in a certain mode, HARQ retransmissions other than those that are already specified in the prose will have an impact on the test sequence. If test cases perform scheduling of data transmissions and/or receptions, or the testing timers in the test cases are less than 900 ms (i.e. the tolerance for 90 ms), HARQ retransmissions will make it difficult to continue testing.

Table 7.3.2-1: TC list intolerable of HARQ retransmissions

| Test case | Comment |  |
| --- | --- | --- |
| MAC |  |  |
| 7.1.3.1, 7.1.3.2, 7.1.3.4, 7.1.3.5, 7.1.3.6, 7.1.3.9, 7.1.6.1, 7.1.6.2 |  | HARQ feedback reporting enabled or DL CRC errors introduced; DL HARQ un specified (re)transmissions will result in ‘Fail' in test body, UL HARQ retransmissions are allowed; |
| 7.1.4.8 |  | Strict relationship between grant and UL data |
| 7.1.4.3 |  | Up to 104 PDUs to be sent in DL every TTI; |
| 7.1.4.2, 7.1.4.11, 7.1.4.12, 7.1.4.14, 7.1.5.4 |  | HARQ feedback transmission specified or PHICH errors introduced |
| 7.1.4.15, 7.1.4.16 |  | Periodic UL grants |
| RLC |  |  |
| 7.2.2.6, 7.2.2.7, 7.2.2.8, 7.2.2.10, 7.2.3.1, 7.2.3.2, 7.2.3.4, 7.2.3.5, 7.2.3.10, 7.2.3.13, 7.2.3.14, 7.2.3.15, 7.2.3.18 |  | Tolerating HARQ retransmissions is not feasible due to rigid timing and scheduling conditions. Testing timer < 900 ms |

Table 7.3.2-2: TC list intolerable of more than one HARQ retransmission per transport block

| Test case | Comment |  |
| --- | --- | --- |
| RLC |  |  |
| 7.2.3.6, 7.2.3.7, 7.2.3.8, 7.2.3.9, 7.2.3.17 |  | Tolerating more than one HARQ retransmission is not feasible due to rigid timing and scheduling conditions. Testing timer < 900 ms |

### 7.3.2.1	HARQ Retransmission when MIMO is configured

For test cases configuring MIMO, if in a TTI more than one transport blocks are scheduled (DCI format 2/ 2A/2B), the HARQ retransmission is handled independently for each transport block by SS. In case UE ACKs one Transport block and NACKs the other and there is no fresh data scheduled for transmission, SS only schedules the NACKed transport block for retransmission, using same Imcs as used in initial transmission, mapped to codeword 0. Acked Transport block (and hence codeword 1) is disabled by setting corresponding and rvidx = 1. Resource allocation (Nprb) used in retransmission is same as in initial transmission.

It is assumed that retransmission and fresh data scheduled in one TTI will not happen.

### 7.3.3	General DL scheduling scheme

The rules in the present clause, unless particularly specified, are applied to both default DCI combinations.

The bandwidth of 5/10/15/20 MHz makes 25/50/75/100 available physical resource blocks respectively. The 25/50/75/100 resource blocks are divided into three distinct sets. Exact set sizes and the elements contained in the individual sets depend upon the DCI combination to be applied.

-	The first set is reserved for BCCH mapped to DL-SCH (SI-RNTI).

-	The second set is reserved for PCCH mapped to DL-SCH (P-RNTI).

-	The third set is used for one of mutually exclusive transmissions of:

-	'Random Access Response' mapped to DL-SCH (RA-RNTI); or

-	UE-dedicated scheduling mapped to DL-SCH (C-RNTI/ SPS C-RNTI/ Temp C-RNTI).

For each subframe for which data of one or more types is scheduled, the SS shall select a Transport Block Size (TBS), independently for each type of data scheduled, such that:

-	All the scheduled data is transmitted respecting the timing information. More details on the timing information can be found in clause 7.8.

-	Not more than MaxRbCnt resource blocks are used, for DCI format 1C, NPRB = MaxRbCnt.

-	Minimum MAC Padding is performed.

-	If all scheduled Data cannot be transmitted in the indicated subframe, for example due to TDD and half duplex configuration, it shall be transmitted in the next available subframe.

#### 7.3.3.1	Additional rules for BCCH scheduling scheme

This scheme is applicable for Data transmission on logical channel BCCH mapped to DL-SCH, PDCCH scrambled by SI-RNTI. For both DCI combinations 4 physical resource blocks are reserved for BCCH transmission. The maximum modulation scheme is restricted to QPSK.

Following additional rules are applied for TBS selection:

-	The Max TBS, the maximum TBS allowed for the scheduling scheme, is restricted to 600. (nearest value achievable for ITBS = 9 and NPRB = 4, as per table 7.1.7.2.1-1 of TS 36.213 [30]).

-	If the scheduled Data cannot fit into a TBS smaller or equal to Max TBS, SS generates an error (it's a TTCN error). TTCN should gracefully exit the test case as a fatal error, assigning inconclusive verdict.

-	Rules in clause 7.3.3.1.1 for DCI combination 1 and in clause 7.3.3.1.2 for DCI combination 2 shall be applied.

##### 7.3.3.1.1	BCCH with DCI combination 1

TS 36.213 [30], table 7.1.7.2.1-1, rows with ITBS =0..26 and columns with NPRB =2 (corresponding to TPC LSB =0) and NPRB =3 (corresponding to TPC LSB =1), TBS <=Max TBS are applicable.

Distinct TBSs and all (TPC LSB, ITBS) combinations for each distinct TBS are listed in the sheet.

If a TBS can have two (TPC LSB, ITBS) combinations, the combination with TPC LSB =0 is selected.

RIV indicates 4 PRBs with index 0..3 allocated.

##### 7.3.3.1.2	BCCH with DCI combination 2

TS 36.213 [30], table 7.1.7.2.3-1, ITBS =0..17 with TBS ≤Max TBS are applicable.

RIV indicates 4 virtual RBs with index 0..3 allocated. These virtual RBs correspond to the physical RBs

-	with index 0, 6, 12, 18 in even slots and 12, 18, 0, 6 in odd slots for 5 MHz bandwidth;

-	with index 0, 12, 27, 39 in even slots and 27, 39, 0, 12 in odd slots for 10 MHz bandwidth;

-	with index 0, 16, 48, 64 in even slots and 48, 64, 0, 16 in odd slots for 15 MHz bandwidth;

-	with index 0, 24, 48, 72 in even slots and 48, 72, 0, 24 in odd slots for 20 MHz bandwidth.

#### 7.3.3.2	Additional rules for PCCH specific scheduling scheme

This scheme is applicable for Data transmission on logical channel PCCH mapped to DL-SCH, PDCCH scrambled by P-RNTI. For DCI combination 1, one physical resource block is reserved. For DCI combination 2, two physical resource blocks are reserved for 5 MHz bandwidth, and four physical resource blocks are reserved for 10 MHz, 15 MHz or 20 MHz bandwidth. The maximum modulation scheme is restricted to QPSK.

Following additional rules are applied for TBS selection:

-	If the scheduled Data cannot fit into Max TBS, SS generates an error (it's a TTCN error). TTCN should gracefully exit the test case as a fatal error, assigning inconclusive verdict.

-	Rules in clause 7.3.3.2.1 for DCI combination 1 and clause 7.3.3.2.2 for DCI combination 2 shall be applied.

##### 7.3.3.2.1	PCCH with DCI combination 1

TS 36.213 [30], table 7.1.7.2.1-1, rows with ITBS =0..26 and columns with NPRB =2 (corresponding to TPC LSB =0) and NPRB =3 (corresponding to TPC LSB =1) TBS < =Max TBS are applicable.

The Max TBS is restricted to 120 (nearest value achievable for ITBS = 9 and NPRB =1, as per table 7.1.7.2.1-1 of TS 36.213 [30]).

Distinct TBSs and all (TPC LSB, ITBS) combinations for each distinct TBS are listed in the sheet.

If a TBS can have two (TPC LSB, ITBS) combinations, the combination with TPC LSB =0 is selected.

RIV indicates 1 PRBs with index 4 allocated.

##### 7.3.3.2.2	PCCH with DCI combination 2

TS 36.213 [30], table 7.1.7.2.3-1, ITBS =0..11 for 5 MHz/ ITBS =0..17 for 10, 15 or 20 MHz with TBS ≤ Max TBS are applicable.

The Max TBS is restricted to

296 bits (nearest value achievable for ITBS = 9 and NPRB =2) for 5 MHz bandwidth,

600 bits (nearest value achievable for ITBS = 9 and NPRB = 4) for 10, 15 or 20 MHz bandwidth.

RIV indicates either two virtual RBs with index 4 and 5 allocated, or four virtual RBs with index 4 to 7 allocated. These virtual RBs correspond to physical RBs:

with index 1 and 7 in even slots and 13 and 19 in odd slots for 5 MHz bandwidth;

with index 1, 13, 28, 40 in even slots and 28, 40, 1, 13in odd slots for 10 MHz bandwidth;

with index 1, 17, 49, 65 in even slots and 49, 65, 1, 17 in odd slots for 15 MHz bandwidth;

with index 1, 25, 49, 73 in even slots and 49, 73, 1, 25 in odd slots for 20 MHz bandwidth.

#### 7.3.3.3	Additional rules for RAR specific scheduling scheme

This scheme is applicable for transmission of Random Access Response mapped to DL-SCH, PDCCH scrambled by RA-RNTI. For both DCI combinations four physical resource blocks are reserved. The maximum modulation scheme is restricted to QPSK.

Following additional rules are applied for TBS selection:

-	The Max TBS is restricted to 600 bits (nearest value achievable for ITBS = 9 and NPRB =4, as per table 7.1.7.2.1-1 of TS 36.213 [30]).

-	If the scheduled Data cannot fit into Max TBS, SS generates an error (it's a TTCN error). TTCN should gracefully exit the test case as a fatal error, assigning inconclusive verdict.

-	Rules in clause 7.3.3.3.1 for DCI combination 1 and clause 7.3.3.3.2 for DCI combination 2 shall be applied.

##### 7.3.3.3.1	RAR with DCI combination 1

TS 36.213 [30], table 7.1.7.2.1-1, rows with ITBS = 0..26 and columns with NPRB = 2 (corresponding to TPC LSB = 0) and 3 (corresponding to TPC LSB = 1) TBS ≤ Max TBS are applicable

Distinct TBSs and all (TPC LSB, ITBS ) combinations for each distinct TBS are listed in the sheet.

If a TBS can have two (TPC LSB, ITBS ) combinations, the combination with TPC LSB = 0 is selected.

RIV indicates 4 PRBs with index 5..8 allocated.

##### 7.3.3.3.2	RAR with DCI combination 2

TS 36.213 [30], table 7.1.7.2.3-1, ITBS = 0..17 with TBS <= Max TBS are applicable.

RIV indicates 4 virtual RBs are allocated. These corresponds to physical RB

with index 13, 19, 2, 8 in even slots and 1, 7, 14, 20 in odd slots for 5 MHz bandwidth;

with index 2, 14, 29, 41 in even slots and 29, 41, 2, 14 in odd slots for 10 MHz bandwidth;

with index 2, 18, 50, 66 in even slots and 50, 66, 2, 18 in odd slots for 15 MHz bandwidth;

with index 2, 26, 50, 74 in even slots and 50, 74, 2, 26 in odd slots for 20 MHz bandwidth.

#### 7.3.3.4	Additional rules for UE-dedicated scheduling scheme in normal mode

The UE-dedicated DL scheduling can work in the normal mode or in the explicit mode. The two resource allocation schemes shall be reconfigurable from each other when the UE and SS are not sending and receiving data, for instance, at end of the test preamble and before the beginning of the test body.

The present clause is specified for the use of the normal mode. The explicit mode is referred to clause 7.3.3.6.

The scheme specified in the present clause is applicable for transmission of data dedicated to a UE in a DL subframe, mapped to DL-SCH, PDCCH scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI when spatial multiplexing MIMO mode is not configured. The maximum modulation scheme is restricted to 64QAM. For the DCI combination 1, 20 physical resource blocks (5 to 24), and for the DCI combination 2, 17 physical resource blocks are reserved. In the case when three intra frequency cells are applied to the test in the DCI combination 1, for the purpose of interference reduction, only 9 PRBs (16 to 24) are reserved.

In TDD normal TBS selection mode, no data is transmitted in DwPTS of the special subframe. For FDD, data can be transmitted in any subframe, except for half-duplex Type B UEs for which scheduling restrictions are specified in clause 7.26. In addition for BL/CE UEs, only valid subframes, according to fdd-DownlinkOrTddSubframeBitmapBR (when configured in the SS), shall be considered as suitable.

The following additional rules are applied for TBS selection:

-	Multiple ASPs can also carry same explicit timing information; indicating different ASP payloads, eventually needs to be transmitted in 1 TTI.

-	For UE Category ≠ 0 the Max TBS is restricted to 10296 bits (Max supported by UE category type 1).

-	For UE Category = 0 and M1, the Max TBS is restricted to 1000 bits. Hence in all TBS sheets only TB combinations with TBSize ≤ 1000 are used.

For 5 MHz bandwidth and the DCI combination 1 with 20 PRBs or DCI combination 2, the TBS 8248, 8760, and 9528 are blocked as they result in coding rates higher than 0.93.

For 5 MHz bandwidth and special DCI combination 1 with 9 PRBs, the TBS 2216, 5992 and 6712 are blocked as they result in coding rates higher than 0.93.

For 10 MHz, 15 MHz and 20 MHz bandwidths none of TBSs are blocked due to coding rates higher than 0.93.

TBS 280 is blocked for all bandwidths as it can be achieved by only Nprb =1 and Itbs=15, requiring 64 QAM modulation scheme. This TBS would normally be used for “RACH procedure MSG4+RRCconnectionSetup”.

The blocked TBS are considered to be not available for selection.

-	Data pending for transmission in a given sub-frame consists of (listed in transmission priority order):

-	MAC Control Elements that the SS needs to send.

-	AMD STATUS PDU(s) that the SS needs to send.

-	Data not sent in previous subframe(s).

-	Fresh Data scheduled for transmission in this subframe for all logical channels.

-	Distinct TBSs and all (NPRB, ITBS) combinations for each distinct TBS are listed in the sheet.

-	If a TBS size can be achieved with more than one combination of IMCS (ITBS) and NPRB:

-	Select combination with lowest delta between NPRB and IMCS.

-	If still more than one combination remain, select combination with highest NPRB.

-	Not more than one RLC Data PDU shall be placed in a MAC PDU per logical channel (i.e. minimize RLC segmentation).

-	In a subframe, in case there is data pending for transmission from more than one logical channel, for each type of data pending for transmission as defined above, priority shall be given to the logical channel with the lowest logical channel priority value. In case of more than one logical channel with the same logical channel priority value, these logical channels should be served equally. Data pending for transmission from more than one logical channel will rarely happen for the signalling and protocol test.

-	Data not transmitted within a subframe is scheduled as pending for transmission in the next available subframe according to the priorities given above. Pending data for transmission will rarely happen for the signalling and protocol test.

-	TBS selected in a context by various platforms shall be within an allowed deterministic tolerance of:

-	2 bytes for potential Timing Advance Command MAC Control Element (1 byte data + 1 byte MAC sub header).

-	4 bytes each for AMD STATUS PDU (2 bytes data + 2 bytes MAC subheader).

-	Therefore in the worst case the SS may add up to (2 + 4 x NAMRB ) bytes to the data scheduled for transmission in a certain subframe, where NAMRB is the number of AM radio bearers (SRB or DRB) actively sending DL data in the test, in any subframe.

-	For DCI combination 1 RIV is calculated based on physical resource blocks corresponding to NPRB of the selected TBS and (NPRB, ITBS) combination. The physical resource blocks that can be allocated are the first NPRB resources of index range

-	5..24 for 5 MHz bandwidth,

-	28..49 for 10 MHz bandwidth,

-	9..30 for 15 & 20 MHz bandwidth.

-	For DCI combination 2, RBG assignment is calculated based on physical resource blocks corresponding to NPRB of the selected TBS and (NPRB, ITBS) combination. The size of RBG is 2 for 5 MHz, 3 for 10 MHz and 4 for 15 & 20 MHz. The available physical resource blocks for allocation are:

-	For 5 MHz bandwidth, RBG1(2,3), RBG2(4,5), RBG4(8,9), RBG5(10,11), RBG7(14,15), RBG8(16,17), RBG10(20,21), RBG11(22,23) and RBG12(24). If NPRB is even, the first NPRB /2 available RBGs are allocated. If NPRB is odd, then first (NPRB -1)/2 RBGs and RBG 12 are allocated.

-	For 10 MHz bandwidth, RBG1(3,4,5), RBG2(6,7,8), RBG3(9,10,11), RBG5(15,16,17), RBG6(18,19,20), RBG10(30,31,32), RBG11(33,34,35), RBG12(36,37,38) and RBG16(48,49). If NPRB mod 3 is 0, the first NPRB /3 RBGs are allocated. If NPRB mod 3 is 2, then first (NPRB -2)/3 available RBGs and RBG 16 are allocated.

-	For 15 MHz bandwidth, RBG1(4,5,6,7), RBG2(8,9,10,11), RBG3(12,13,14,15), RBG5(20,21,22,23), RBG6(24,25,26,27), RBG7(28,29,30,31), RBG11(44,45,46,47), RBG13(52,53,54,55), RBG14(56,57,58,59), RBG15(60,61,62,63), RBG17(68,69,70,71), RBG18(72.73.74). If NPRB mod 4 is 0, the first NPRB /4 RBGs are allocated. If NPRB mod 4 is 3, then first (NPRB -3)/4 available RBGs and RBG 18 are allocated.

-	For 20 MHz bandwidth, RBG1(4,5,6,7), RBG2(8,9,10,11), RBG3(12,13,14,15), RBG4(16,17,18,19), RBG5(20,21,22,23), RBG7(28,29,30,31), RBG8(32,33,34,35), RBG9(36,37,38,39), RBG10(40,41,42,43), RBG14(56,57,58,59), RBG15(60,61,62,63), RBG16(64,65,66,67), RBG17(68,69,70,71), RBG19(76.77.78.79) and RBG20(80,81,82,83). The first NPRB /4 RBGs are allocated.

#### 7.3.3.5	DL Resource allocation bitmaps

##### 7.3.3.5.1	DCI combination 1

Table 7.3.3.5.1-1: Physical resource allocation bitmap
for DCI combination 1 (5 MHz) with 20 PRBs

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-2: Physical resource allocation bitmap
for DCI combination 1 (5 MHz) with 9 PRBs

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3 (columns 0–34): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9..22 | 23..27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3 (columns 35–49): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3A (columns 0–20): Physical resource allocation bitmap
for DCI combination 1 (15 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-3B (columns 21-74): Physical resource allocation bitmap
for DCI combination 1 (15 MHz)

| NPRB | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31..33 | 34..41 | 42..74 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals | Not Used |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-4 (columns 0–20): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.1-5 (columns 21-99): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31..46 | 47..52 | 53..99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals | Not Used |
| PCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### 7.3.3.5.2	DCI combination 2

Table 7.3.3.5.2-1: Physical resource allocation bitmap
for DCI combination 2 (5 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  | 1 |  |  |  |  |  | 2 |  |  |  |  |  | 3 |  |  |  |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  | 3 |  |  |  |  |  | 0 |  |  |  |  |  | 1 |  |  |  |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  | 5 |  |  |  |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  | 9 |  |  |  |  | 6 |  |  |  |  |  | 7 |  |  |  |  |  |
| RAR-Odd |  | 6 |  |  |  |  |  | 7 |  |  |  |  |  |  | 8 |  |  |  |  |  | 9 |  |  |  |  |
| UE-Dedicated |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.5.2-2 (columns 0-20): Physical resource allocation bitmap
for DCI combination 2 (10 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |  |
| PCCH-Odd |  | 6 |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |  |
| RAR-Odd |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  | 11 |  |  |  |  |  |  |
| UE-Specific | x | x |  |  |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |
| RBGs | 0 |  |  | 1 |  |  | 2 |  |  | 3 |  |  | 4 |  |  | 5 |  |  | 6 |  |  |

Table 7.3.3.5.2-2 (columns 21-41): Physical resource allocation bitmap
for DCI combination 2 (10 MHz)

| NPRB | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |
| BCCH-Odd |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |
| PCCH-Even |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |  | 7 |  |
| PCCH-Odd |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |  | 5 |  |
| RAR-Even |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  | 11 |
| RAR-Odd |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  | 9 |
| UE-Specific |  | x | x | x | x | x | x | x |  |  |  |  |  |  |  |  |  |  | x | x |  |
| RBGs | 7 |  |  | 8 |  |  | 9 |  |  | 10 |  |  | 11 |  |  | 12 |  |  | 13 |  |  |

Table 7.3.3.5.2-2 (columns 42-49): Physical resource allocation bitmap
for DCI combination 2 (10 MHz)

| NPRB | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  | Not Used |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |
| RBG's | 14 |  |  | 15 |  |  | 16 |  |

Table 7.3.3.5.2-2a (columns 0-19): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |
| PCCH-Odd |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |
| RAR-Odd |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11 |  |
| UE-Specific | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  | x | x |  |  |
| RBG's | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |

Table 7.3.3.5.2-2a (columns 20-39): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBG's | 5 |  |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  |  | 9 |  |  |  |

Table 7.3.3.5.2-2a (columns 40-59): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |  |  |  |  |
| RBG's | 10 |  |  |  | 11 |  |  |  | 12 |  |  |  | 13 |  |  |  | 14 |  |  |  |

Table 7.3.3.5.2-2a (columns 60-74): Physical resource allocation bitmap
for DCI combination 2 (15 MHz)

| nprb | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  | 11 |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  | x | x |  |  |  |  |  |  |  |  |  |
| RBG's | 15 |  |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  |

Table 7.3.3.5.2-3 (columns 0-19): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  | 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBGs | 0 |  |  |  | 1 |  |  |  | 2 |  |  |  | 3 |  |  |  | 4 |  |  |  |

Table 7.3.3.5.2-3 (columns 20-39): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  | 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  | 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  | 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  | 11 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  | x | x |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBGs | 5 |  |  |  | 6 |  |  |  | 7 |  |  |  | 8 |  |  |  | 9 |  |  |  |

Table 7.3.3.5.2-3 (columns 40-59): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  | 6 |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  | 4 |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  | 10 |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  | x | x | x |  |  |  |  |  |  |  |  |  |
| RBG's | 10 |  |  |  | 11 |  |  |  | 12 |  |  |  | 13 |  |  |  | 14 |  |  |  |

Table 7.3.3.5.2-3 (columns 60-79): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  | 7 |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  | 5 |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 11 |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 9 |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  | x | x |  |  |  |  |  |  |
| RBGs | 15 |  |  |  | 16 |  |  |  | 17 |  |  |  | 18 |  |  |  | 19 |  |  |  |

Table 7.3.3.5.2-3 (columns 80-99): Physical resource allocation bitmap
for DCI combination 2 (20 MHz)

| NPRB | 80 | 81 | 82 | 83 | 84 | 85 | 86 | 87 | 88 | 89 | 90 | 91 | 92 | 93 | 94 | 95 | 96 | 97 | 98 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Not Used |  |  |  |
| BCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| PCCH-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Even |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RAR-Odd |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RBGs | 20 |  |  |  | 21 |  |  |  | 22 |  |  |  | 23 |  |  |  | 24 |  |  |  |

NOTE:	Odd and even refer to slots.

#### 7.3.3.6	UE-dedicated scheduling scheme in explicit mode

This scheme applies to:

1.	spatial multiplexing MIMO configurations or

2.	transmit diversity MIMO configurations and non-MIMO configuration where the normal mode scheduling scheme is inappropriate.

SS is configured with an exact TBS (modulation and coding scheme, Imcs, and number of resource blocks, Nprb) to use. Other parameters, such as the HARQ process number and redundancy version to use for each transmission, are also configured by the TTCN. SS shall use TBS sheets with matching DCI format and Resource allocation Type. If the parameter 'FirstRbIndex' is configured different than specified in respective TBS sheet, the resource block bit maps in TBS sheet s are shifted by ' FirstRbIndex' and applied, with an exception for Resource allocation type 0 where only the full size 'Resource block groups' are shifted by ' FirstRbIndex'; if the last Resource block group is not full size, and is part of resource block bitmap, it is applied without any shift.

All data scheduled for a certain subframe shall be transmitted in the single indicated subframe, using configured parameters. The TTCN shall ensure that the configured parameters are consistent, in particular that the scheduled data size and the configured TBS match each other. Data scheduled by the prose, and hence also by the TTCN, provides possible space for the Timing Advance MAC control element and the RLC Status PDU. The SS shall include one of these if so triggered, else the bits reserved for these are filled by MAC padding.

Additionally, in the case of MIMO data scheduled for transmission in a given sub-frame, this consists of (listed in transmission priority order):

-	MAC Control Elements that the SS needs to send (if triggered).

-	AMD STATUS PDU(s) that the SS needs to send (if triggered).

-	Fresh data scheduled for transmission in this subframe for one or more logical channels, as per logical channel priority [lower value = higher priority]; if data is available for more than one logical channel with the same priority, then the logical channel corresponding to the DRB-ID with the lower value has the higher priority.

-	MAC padding.

The following additional rules need to be applied on data scheduled for transmission to be mapped on two transport blocks corresponding to two code words:

-	Higher priority data (as stated above) maps on to Transport Block 1 and lower priority data maps on Transport Block 2 (if Transport Block 1 gets full); and

-	Minimum MAC padding is performed in Transport Block 1; and

-	If data from one logical channel needs to be mapped on to two transport blocks, the PDCP PDUs with lower PDCP sequence numbers get mapped on to Transport Block 1.

By default no data is scheduled in TDD special subframes (i.e. subframes 1 and 6 for default TDD subframe configuration). For testing DL data reception in TDD special subframe, explicit mode TBS selection shall be used. The TTCN shall ensure that the configured parameters are consistent, in particular that the scheduled data size and the configured TBS match each other. As per TS36.213 [30] clause 7.1.7, for special subframe configuration 9 with normal cyclic prefix or special subframe configuration 7 with extended cyclic prefix, the actual Nprb used for TB size calculation will be a max( floor{ Nprb in DCI command *0.375},1). Tables 7.3.3.6-1/2 give the mapping for Nprb in DCI command and Nprb used for TB size determination.

Table 7.3.3.6-1: Nprb DCI to Nprb TBS mapping for 20 Mhz

| Nprb in DCI command (configured by TTCN) | 4 | 8 | 12 | 16 | 20 | 24 | 28 | 32 | 36 | 40 | 44 | 48 | 54 | 60 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nprb used for TBS determination | 1 | 3 | 4 | 6 | 7 | 9 | 10 | 12 | 13 | 15 | 16 | 18 | 20 | 22 |

Table 7.3.3.6-2: Nprb DCI to Nprb TBS mapping for 5 Mhz

| Nprb in DCI command (configured by TTCN) | 4 | 6 | 8 | 12 | 14 | 16 |
| --- | --- | --- | --- | --- | --- | --- |
| Nprb used for TBS determination | 1 | 2 | 3 | 4 | 5 | 6 |

##### 7.3.3.6.1	DL Scheduling in Transport Block Size Selection Test Cases

The MAC transport block size selection test cases defined in clause 7.1.7 of TS 36.523-1 [1], use bandwidths of 5/10/15/20MHz. For the preamble and post amble in these tests, the default scheduling rules defined in clauses 7.3.3.1 to 7.3.3.4 for 5/10/15/20 MHz and DCI combination 1A are applied respectively. During the test body, when the actual TB sizes with appropriate DCI and resource allocation formats needed are to be tested, the SS is configured in explicit mode for UE-dedicated scheduling.

#### 7.3.3.7	Resource allocation sheets

Attached with this Technical Specification, the DL resource allocation tables can be found, providing physical resource allocations for various transport block sizes, developed as per rules specified in clause 7.3.3, in Microsoft Excel format. Each individual sheet in the workbook represents various scheduling schemes as per table 7.3.3.7-1.

Table 7.3.3.7-1: DL resource allocation sheets

| S. No | Sheet Name | Description |
| --- | --- | --- |
| 1 | DCI-1A-PCCH | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by P-RNTI (5 MHz, 10 MHz, 15 MHz and 20 MHz) |
| 2 | DCI-1A-BCCH | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by SI-RNTI (5 MHz, 10 MHz, 15 MHz and 20 MHz) |
| 3 | DCI-1A-RAR | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by RA-RNTI (5 MHz, 10 MHz, 15 MHz and 20 MHz) |
| 4 | DCI-1A-UE-Specific | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (5 MHz) |
| 5 | DCI-1A-3-IntraFreq-UE-Specific | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI and three Intra Freq cells are configured (5 MHz) |
| 6 | DCI-1A-UE-Specific-10MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (10 MHz) |
| 6A | DCI-1A-UE-Specific-15MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (15 MHz) |
| 7 | DCI-1A-UE-Specific-20MHz | DL Resource scheduling for DCI format 1A and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (20 MHz) Also in preamble/postamble phase of MAC TBS test cases with 15 MHz bandwidth configured |
| 8 | DCI-1C-PCCH | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (5 MHz) |
| 9 | DCI-1C-BCCH | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (5 MHz) |
| 10 | DCI-1C-RAR | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (5 MHz) |
| 11 | DCI-1-UE-Specific | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (5 MHz) |
| 12 | DCI-1C-PCCH-10MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (10 MHz) |
| 13 | DCI-1C-BCCH-10MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (10 MHz) |
| 14 | DCI-1C-RAR-10MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (10 MHz) |
| 15 | DCI-1-UE-Specific-10MHz-Gap1 | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (10 MHz) |
| 15A | DCI-1C-PCCH-15MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (15 MHz) |
| 15B | DCI-1C-BCCH-15MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (15 MHz) |
| 15C | DCI-1C-RAR-15MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (15 MHz) |
| 15D | DCI-1-UE-Specific-15MHz-Gap1 | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (15 MHz) |
| 16 | DCI-1C-PCCH-20MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by P-RNTI (20 MHz) |
| 17 | DCI-1C-BCCH-20MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by SI-RNTI (20 MHz) |
| 18 | DCI-1C-RAR-20MHz-Gap1 | DL Resource scheduling for DCI format 1C and PDCCH is scrambled by RA-RNTI (20 MHz) |
| 19 | DCI-1-UE-Specific-20MHz-Gap1 | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI/ SPS C-RNTI/ Temp C-RNTI (20 MHz) |
| 20 | DCI-1-RA0-ExplicitConfig | DL Resource scheduling for DCI format 1, Resource allocation 0 and PDCCH is scrambled by C-RNTI |
| 21 | DCI-1-RA1-ExplicitConfig | DL Resource scheduling for DCI format 1, Resource allocation 1 and PDCCH is scrambled by C-RNTI |
| 22 | DCI1A-ExplicitConfig | DL Resource scheduling for DCI format 1A, Resource allocation 2(localised & distributed) and PDCCH is scrambled by C-RNTI |
| 23 | DCI-2A-RA0-ExplicitConfig | DL Resource scheduling for DCI format 2A, Resource allocation 0 and PDCCH is scrambled by C-RNTI |
| 24 | DCI-2A-RA1-ExplicitConfig | DL Resource scheduling for DCI format 2A, Resource allocation 1 and PDCCH is scrambled by C-RNTI |

#### 7.3.3.8	MPDCCH DL DCI formats

The PRB scheduling is for the PRB’s within a selected Narrow Band consisting of 6 resources. Hence all PRB selection is restricted to these 6 PRB’s in the ‘X’ selected narrow band.

##### 7.3.3.8.1	BCCH

There is no MPDCCH transmitted for BCCH hence no DCI format. All six contiguously allocated localized virtual resource blocks within a narrowband are used. The Itbs is configured by RRC.

##### 7.3.3.8.2	PCCH

DCI format 6-2 is used for PCCH scheduling. All six contiguously allocated localized virtual resource blocks within a narrowband are used. The Itbs to TB size mapping is in table 7.3.3.8.2-1 derived from TS 36.213 [30] table 7.1.7.2.3-1. The Itbs = Imcs in the DCI 6-2. The Imcs = Itbs shall be less than or equal to 7.

Table 7.3.3.8.2-1: Transport Block Size (TBS) table for PCCCH, DCI 6-2

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | 40 | 56 | 72 | 120 | 136 | 144 | 176 | 208 |

##### 7.3.3.8.3	RAR

7.3.3.8.3.1	RAR with DCI format 6-1A for CE ModeA UE

TS 36.213 [30], table 7.1.7.2.1-1, rows with ITBS = 0..15 and columns with NPRB = 2 (corresponding to TPC LSB = 0) and 3 (corresponding to TPC LSB = 1) TBS ≤ Max TBS are applicable

Distinct TBSs and all (TPC LSB, ITBS ) combinations for each distinct TBS are listed in the sheet.

If a TBS can have two (TPC LSB, ITBS ) combinations, the combination with TPC LSB = 0 is selected.

RIV =18, indicates 4 localised virtual RBs (0,1,2,3) are allocated. These correspond to physical RB (0,1,2,3) in the indicated narrow band. Resource allocation sheet ‘DCI-6-1A-RAR’ gives the transport block sizes.

RAR length is 7 bytes,1 byte header and 6 bytes Payload. Needing 1 byte for Back off indicator if included.

Tbsize of 600, allows max upto 10 RAR messages to be sent in one MAC PDU [8*7*10=560 bits]

7.3.3.8.3.2	RAR with DCI format 6-1B for CE ModeB UE

TS 36.213 [30], clause 7.1.7.2.6 Imcs=ITBS = 0..9.

Resource allocation bit is set to 0 in the indicated narrowband, indicates RBs with PRB index {0, 1, 2, 3} are allocated. The TB sizes are given by table 7.3.3.8.3.2-1.

Table 7.3.3.8.3.2-1: Transport Block Size (TBS) table for RAR, DCI 6-1B

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | 88 | 144 | 176 | 208 | 256 | 328 | 392 | 472 | 536 | 616 |

RAR length is 6 bytes,1 byte header and 5 bytes Payload. Needing 1 byte for Back off indicator if included.

Max Tbsize of 616, allows max up to 11RAR messages to be sent in one MAC PDU (8*7*11=616 bits).

##### 7.3.3.8.4	UE-dedicated scheduling

7.3.3.8.4.1	UE-dedicated scheduling with DCI format 6-1A for CE ModeA UE

TS 36.213 [30], table 7.1.7.2.3-1, ITBS = 0..15 with TBS <= Max TBS=1000 are applicable. The rules applied are same as clause 7.3.3.4. Resource allocation sheet ‘UE-Specific-DCI-6-1A’ gives the transport block sizes.

7.3.3.8.4.2	UE-dedicated scheduling with DCI format 6-1B for CE ModeB UE

TS 36.213 [30], clause 7.1.7.2.6 Imcs=ITBS = 0..9, Modulation is always QPSK.

Resource allocation bit is set to 0 in the indicated narrowband, indicates RBs with PRB index {0, 1, 2, 3} are allocated and 1 indicates all 6 PRB’s are allocated. If a TB size can be achieved by both nPRB=4 and 6, then the rules to select TBS combination are same as clause 7.3.3.4. Resource allocation sheet ‘UE-Specific-DCI-6-1A’ gives the transport block sizes.

#### 7.3.3.9	DL Resource allocation bitmaps for BL/CE UE

##### 7.3.3.9.1	DCI combination 1

For simplicity of scheduling, it is proposed to restrict DCI combination 1 for WB-E-UTRA operation in test cases also applicable for BL/CE UE and executed against BL/CE UE.

UE-specific below includes all other transmission C-RNTI/T-CRNTI/P-RNTI/RA-RNTI on PDSCH.

Table 7.3.3.9.1-1: Physical resource allocation bitmap
for DCI combination 1 (5 MHz)

| NPRB |  | 0 | 1 | 2 | 3 | 4 | 5 |  | 6 | 7 | 8 | 9-15 |  |  |  |  | 16 |  | 17 |  | 18 |  | 19-24 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  | Used for PBCH and other common signals |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB1-BR |  | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NB 3 |  |
| SIx-BR |  | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NB 3 |  |
| MPDCCH/P-RNTI |  | NB 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | NB 3 |  |
| MPDCCH |  |  |  |  |  |  | NB 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

NOTE:	The allocation assumes that SIB1-BR/SIx-BR/SIx are not transmitted in same subframe.

It is unavoidable to use NB 3 for both SIB1-BR and UE-specific. This means that UE-specific transmission cannot be used in (frame number mod 2=0 in even physical layer cell Id and frame number mod 2 =1 in odd physical layer cell 1d, subframe =4). NB 1 and 2 need to be avoided so as to avoid central 72 carriers carrying MIB and reduce interference between intra frequency cells.

Table 7.3.3.9.1-2: Narrow band allocation per cell (5 MHz)

| Physical cell ID mod 2 | SIB1-BR set | SIx-BR | UE-specific | MPDCCH | MPDCCH associated with the P-RNTI |
| --- | --- | --- | --- | --- | --- |
| 0 | NB 0,NB 3 | NB 0 | NB 3 | NB 1 | NB 0 |
| 1 | NB 3,NB 0 | NB 0 | NB 3 | NB 1 | NB 3 |

Table 7.3.3.9.1-3 (columns 0–30): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7-12 | 13-18 | 19..21 | 22..27 | 28-30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  | Not Used | Used for PBCH and other common signals | Not Used |
| SIB1-BR |  | NB 0 |  |  |  |  |  | NB 1 | NB 2 |  |  |  |
| SIx-BR |  | NB 0 |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |
| MPDCCH/P-RNTI |  | NB 0 |  |  |  |  |  | NB 1 | NB 2 |  |  |  |
| MPDCCH |  |  |  |  |  |  |  | NB1 | NB2 |  |  |  |

Table 7.3.3.9.1-3 (columns 31–49): Physical resource allocation bitmap
for DCI combination 1 (10 MHz)

| NPRB | 31-36 | 37-42 | 43-48 | 49 |
| --- | --- | --- | --- | --- |
| BCCH |  |  |  | Not Used |
| SIB1-BR | NB 5 | NB 6 | NB 7 |  |
| SIx-BR |  |  |  |  |
| UE-Specific |  | NB 6 | NB 7 |  |
| MPDCCH/P-RNTI | NB 5 | NB 6 | NB 7 |  |
| MPDCCH |  |  |  |  |

Table 7.3.3.9.1-4: Narrow band allocation per cell (10 MHz)

| Physical cell ID mod 6 | SIB1-BR set | SIx-BR | UE-specific | MPDCCH | MPDCCH associated with the P-RNTI |
| --- | --- | --- | --- | --- | --- |
| 0 | NB 0,NB 5 | NB 0 | NB 6 | NB 1 | NB 0 |
| 1 | NB 1,NB 6 | NB 0 | NB 7 | NB 2 | NB 1 |
| 2 | NB 2,NB 7 | NB 0 | NB 6 | NB 1 | NB 2 |
| 3 | NB 5,NB 0 | NB 0 | NB 6 | NB 1 | NB 5 |
| 4 | NB 6,NB 1 | NB 0 | NB 7 | NB 2 | NB 6 |
| 5 | NB 7,NB 2 | NB 0 | NB 6 | NB 1 | NB 7 |

NOTE 1:	The allocation assumes that SIB1-BR/SIx-BR and SIB1-BR/SIx are not transmitted in same subframe.

NOTE 2:	The setting of NBs does not consider the case of bandwidth 10MHz for TDD.

Table 7.3.3.9.1-5 (columns 0–43): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8-13 | 14-19 | 20-25 | 26-31 | 32-37 | 38-43 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SIB1-BR |  |  | NB 0 |  |  |  |  |  | NB 1 | NB 2 | NB 3 | NB 4 | NB 5 | NB 6 |
| SIx-BR |  |  |  |  |  |  |  |  | NB 1 |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| MPDCCH/P-RNTI |  |  | NB 0 |  |  |  |  |  | NB 1 | NB 2 | NB 3 | NB 4 | NB 5 | NB 6 |

Table 7.3.3.9.1-5 (columns 44-99): Physical resource allocation bitmap
for DCI combination 1 (20 MHz)

| NPRB | 44-46 | 47..52 |  |  |  | 53-54 | 56-61 | 62-67 | 68-73 | 64-79 | 80-85 | 86-91 | 92-97 | 98..99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BCCH | Not Used | Used for PBCH and other common signals |  |  |  | Not Used |  |  |  |  |  |  |  | Not Used |
| SIB1-BR |  |  |  |  |  |  | NB 9 | NB 10 | NB11 | NB 12 | NB 13 | NB14 | NB 15 |  |
| SIx-BR |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| UE-Specific |  |  |  |  |  |  |  |  |  |  |  | NB 14 |  |  |
| MPDCCH/P-RNTI |  |  |  |  |  |  | NB 9 | NB 10 | NB11 | NB 12 | NB 13 | NB 14 | NB 15 |  |
| MPDCCH | NB 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.3.9.1-6: Narrow band allocation per cell (20 MHz)

| Physical cell ID mod 14 | SIB1-BR set | SIx-BR | UE-specific | MPDCCH | MPDCCH associated with the P-RNTI |
| --- | --- | --- | --- | --- | --- |
| 0 | NB 0, NB 3, NB 6, NB 11 | NB 1 | NB 14 | NB 7 | NB 0 |
| 1 | NB 1, NB 4, NB 9, NB 12 | NB 1 | NB 14 | NB 7 | NB 1 |
| 2 | NB 2, NB 5, NB 10, NB 13 | NB 1 | NB 14 | NB 7 | NB 2 |
| 3 | NB 3, NB 6, NB 11, NB 14 | NB 1 | NB 15 | NB 7 | NB 3 |
| 4 | NB 4,NB 9, NB 12, NB 15 | NB 1 | NB 14 | NB 7 | NB 4 |
| 5 | NB 5, NB 10, NB 13, NB 0 | NB 1 | NB 14 | NB 7 | NB 5 |
| 6 | NB 6, NB 11, NB 14, NB 1 | NB 1 | NB 15 | NB 7 | NB 6 |
| 7 | NB 9, NB 12, NB 15, NB 2 | NB 1 | NB 14 | NB 7 | NB 9 |
| 8 | NB 10, NB13, NB 0, NB 3 | NB 1 | NB 14 | NB 7 | NB 10 |
| 9 | NB 11, NB 14, NB 1, NB 4 | NB 1 | NB 15 | NB 7 | NB 11 |
| 10 | NB 12, NB 15, NB 2, NB 5 | NB 1 | NB 14 | NB 7 | NB 12 |
| 11 | NB 13, NB 0, NB 3, NB 6 | NB 1 | NB 14 | NB 7 | NB 13 |
| 12 | NB 14, NB 1, NB 4, NB 9 | NB 1 | NB 15 | NB 7 | NB 14 |
| 13 | NB 15, NB 2, NB 5, NB 10 | NB 1 | NB 14 | NB 7 | NB 15 |

NOTE:	The allocation assumes that SIB1-BR/SIx and SIB1-BR/SIx-BR are not transmitted in same subframe.

## 7.4	Cell Configurations

### 7.4.1	Cell Configuration Types

Three cell configurations are defined in TS 36.508 [3] clause 6.3.3: Full Cell, Minimum Uplink Cell and Broadcast Only Cell; however the TTCN always considers all cells as Full Cells, and thus always provides the complete cell configuration parameters.

The SS may:

-	always configure a cell as a 'Full Cell' based on the complete information; or

-	configure the cell based on the 'CellConfig_Type' flag taking only the required configuration parameters and ignoring the others.

For a given value of the 'CellConfig_Type' flag, the TTCN shall:

-	For Full Cell Configuration:

-	expect normal SS behaviour.

-	For Minimum Uplink Cell Configuration:

-	Configure the SS to report Preamble detection.

-	Assign verdicts based on the PRACH Preamble Indications.

-	Consume any uplink SRB0 messages (if the SS is configured as a Full Cell).

-	For Broadcast Only Cell Configuration:

-	Not configure the SS to report Preamble detection.

-	Consume any uplink SRB0 messages (if the SS is configured as a Full Cell).

### 7.4.2	Cell Power Change

To set and adjust the cell power at the two test ports, Reference Power and Attenuation, are provided in the record Reference Power.

The field Reference Power is only set when the cell is created and is not updated during the test case execution. The SS applies the Reference Power when the cell is fully configured.

To adjust the power level in the test case, the field Attenuation is used. After initial configuration of a cell the attenuation corresponds to the value "off". When the power is changed for more than one cell, the power changes shall happen at the same time for all the cells according to the time instances for power level changes specified in TS 36.523-1 [1]. The time it takes to complete the power change for all the cells shall be done:

-	within a maximum of 700 ms when changing the power of a cell from "off" to a certain value; or

-	within a maximum of 100 ms (10 frames) otherwise.

The SS shall ensure the power level at the test ports conform to the required downlink signal levels specified in clause 6.2.2.1 of TS 36.508 [3].

### 7.4.3	E-UTRAN cell identity

#### 7.4.3.1	Timing parameters of cells

For RRC and Idle mode test, the timing parameters in table 7.4.3.1-1 are applied. The specification of Cell 1 - Cell 30 can be found in TS 36.508 [3].

Table 7.4.3.1-1: Timing parameters of simulated cells

| cell ID | SFN offset | FDD Tcell (Ts) | TDD Tcell (Ts) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Synchronous | Non synchronous |
| Cell 1 | 0 | 0 | 0 | 0 |
| Cell 2 | 124 | 30720 | 154 | 30720 |
| Cell 3 | 257 | 150897 | 77 | 150897 |
| Cell 4 | 1000 | 61440 | 307 | 61440 |
| Cell 6 | 657 | 524 | 77 | 524 |
| Cell 10 | 129 | 43658 | 77 | 43658 |
| Cell 11 | 957 | 92160 | 154 | 92160 |
| Cell 12 | 1015 | 181617 | 154 | 181617 |
| Cell 13 | 890 | 31244 | 154 | 31244 |
| Cell 14 | 680 | 300501 | 77 | 300501 |
| Cell 23 | 383 | 212337 | 154 | 212337 |
| Cell 28 | 890 | 31244 | 154 | 31244 |
| Cell 29 | 680 | 300501 | 77 | 300501 |
| Cell 30 | 1015 | 181617 | 154 | 181617 |
| NOTE: For TDD, synchronous Tcell values are applied unless specified otherwise in the test cases. |  |  |  |  |

Table 7.4.3.1-2 is applied to the NAS test when more than one PLMN exists in a test case. Further cell parameters can be found in TS 36.508 [3], table 6.3.2.2-3.

Table 7.4.3.1-2: Timing parameters of simulated cells for NAS TCs in different PLMNs

| cell ID | SFN offset | FDD Tcell (Ts) | TDD Tcell (Ts) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Synchronous | Non synchronous |
| Cell A | 0 | 0 | 0 | 0 |
| Cell B | 124 | 30720 | 154 | 30720 |
| Cell C | 257 | 61440 | 307 | 61440 |
| Cell D | 1000 | 92160 | 154 | 92160 |
| Cell E | 752 | 32047 | 77 | 32047 |
| Cell F | NA | NA | NA | NA |
| Cell G | 957 | 631 | 77 | 631 |
| Cell H | 1015 | 31351 | 154 | 31351 |
| Cell I | 890 | 127200 | 77 | 127200 |
| Cell J | 680 | 1327 | 77 | 1327 |
| Cell K | 383 | 157920 | 154 | 157920 |
| Cell L | 562 | 188640 | 307 | 188640 |
| Cell M | 471 | 122880 | 307 | 122880 |
| NOTE: For TDD, synchronous Tcell values are applied unless specified otherwise in the test cases. |  |  |  |  |

Figure 7.4.3.1-3 illustrates shifting DL transmission timing offset by Tcell = 1 subframe, between multiple NAS FDD cells on the same frequency (table 7.4.3.1-4) in the same PLMN.

Figure 7.4.3.1-3: Timing offset between FDD cells on the same frequency

Table 7.4.3.1-4 is applied to the NAS test when all NAS cells in a test case belong to the same PLMN. Further cell parameters can be found in TS 36.508 [3], table 6.3.2.2-2.

Table 7.4.3.1-4: Timing parameters of simulated cells for NAS TCs in same PLMN

| cell ID | SFN offset | FDD Tcell (Ts) | TDD Tcell (Ts) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Synchronous | Non synchronous |
| Cell A | 0 | 0 | 0 | 0 |
| Cell B | 124 | 30720 | 154 | 30720 |
| Cell C | 257 | 150897 | 77 | 150897 |
| Cell D | 1000 | 61440 | 307 | 61440 |
| Cell E | NA | NA | NA | NA |
| Cell F | NA | NA | NA | NA |
| Cell G | NA | NA | NA | NA |
| Cell H | NA | NA | NA | NA |
| Cell I | NA | NA | NA | NA |
| Cell J | NA | NA | NA | NA |
| Cell K | NA | NA | NA | NA |
| Cell L | NA | NA | NA | NA |
| Cell M | 471 | 31244 | 154 | 31244 |
| NOTE: For TDD, synchronous Tcell values are applied unless specified otherwise in the test cases. |  |  |  |  |

Shifting radio frame transmission timing can eliminate the following interference between intra frequency cells:

-	P-SS/S-SS to P-SS/S-SS, RS, PBCH, PCFICH, PDCCH and PHICH.

-	PBCH to PBCH.

-	PBCH to PCFICH, PDCCH and PHICH.

-	PDSCH to PCFICH, PDCCH, PHICH.

As TDD UL and DL are on same frequency, to avoid interference between DL and UL, the Random Access Response Timing Advance (RAR TA) is related to the Tcell:

For TDD cells:

RAR TA = [FLOOR ((Tcell) mod 30720 / 16) ] mod 1282

For FDD, the Random Access Response Timing Advance is set to 0.

In MBMS test cases, cells belonging to the same MBSFN Area have a synchronized radio frame timing: the SFN offset and Tcell values are set to the values of the cell having the lower cellId value.

In Carrier Aggregation signalling test cases, the SFN offset and Tcell of configured cells has to be same.

The H-SFN Offset shall be set to 0 for all cells.

### 7.4.4	Cell configurations for NAS test cases

The default cell identifiers for NAS cells are defined in 36.508 [3] clause 6.3.2.2.

The allocation of Physical layer cell identifiers to the individual cells is according to (PCI mode 6) being differential for the cells working on the same radio frequency. The way of PCI allocation can reduce the interference between the intra-frequency cells for reference signal to reference signal, PCFICH to PCFICH and PHICH to PHICH. The definition of Cell A - Cell M can be found in TS 36.508 [3].

### 7.4.5	Configuration of Multi-Cell Environment

When there is more than one E-UTRA cell in a test case the following rules are applied in TTCN:

-	At the beginning of the preamble, before initial attachment of the UE, all E-UTRA cells are configured but switched off.

-	In the preamble only the serving cell is switched on; all other cells remain switched off.

-	At the end of the preamble the cells are configured according to the initial power level settings (T0) of the test case.

The mapping of cells to physical resources and management of the physical resources are out of TTCN scope. The following principles can be applied to the system simulator:

-	Cells being switched off need not to be mapped to physical resources.

-	When a cell is switched off mapping to a physical resource may be kept and reused when the cell is switched on again.

-	When a cell is switched on it can either already been mapped to a physical resource or it needs to be mapped to a free resource.

-	When there are less physical resources than cells it is up to SS implementation to find strategies to dynamically map the cells to the resources.

Independent from the strategies being used the system simulator shall obey timing restrictions for changing power-levels of one or several cells as stated in clause 7.4.2.

## 7.5	TDD Considerations

LTE options of FDD and TDD will be contained in the same common FDD and TDD test cases, similar to the prose in TS 36.523-1 [1].

The TDD Uplink-downlink configuration 1 in 3GPP TS 36.211 [35], table 4.2-2 is applied.

### 7.5.1	FDD vs. TDD implementation

FDD/TDD differences are introduced in the common FDD and TDD test cases using branches at a low level in the test case. The branches are used either:

-	to assign a variable;

-	to implement a different behaviour;

-	to change an FDD or TDD parameter in a template sent to the UE or SS.

The mode under test (FDD or TDD) is based on the value of the bands under test.

### 7.5.2	Guideline for FDD vs. TDD verification

With respect to E-UTRA FDD vs. TDD technologies, it is recommended that separate verifications for FDD and TDD are required for the TCs in TS 36.523-1 [1]:

-	clause 6, 7, 8, 12, 13;

-	with MultiRAT involved.

## 7.6	Special RLC Modes

### 7.6.1	Suppression of RLC Acknowledgements

Two different modes, both applicable per radio bearer, are defined as:

-	General suppression:

-	If this mode is activated, no RLC acknowledgements will be generated by the SS. This mode can be switched on and will persist until it is switched off. Afterwards the SS will continue handling the RLC acknowledgements as normal.

-	One time suppression:

-	If this mode is activated, no RLC acknowledgement will be generated by SS for the next RLC message data PDU received. Once this has been done, the SS continues handling RLC acknowledgements as normal.

While any of these test modes is active, the SS shall continue updating all state variables of the bearer’s RLC, and in particular the receive state variable (VR(R) for E-UTRA, RX_Next for NR).

If a split radio bearer is configured in one of these RLC test modes then each of the involved RLC entities can be configured individually for the chosen RLC test mode.

In case of a handover the modes continue to be active.

### 7.6.2	Modification of VT(S)

This mode allows to manipulate the RLC state variable VT(S) so that the SS can generate an RLC sequence number as needed during a test. The input to the special test mode is an integer (0..1023) as value of ModifyVTS, The SS shall set variable VT(S) as follows:

VT(S):= ModifyVTS.

The purpose of this special test mode is to force an incorrect RLC sequence number to be used by the SS. Once VT(S) has been modified in the RLC entity at the SS side, this RLC entity will be inconsistent. One possibility to bring the RLC entity back to normal is to re-establish the RLC peer connection. This is done in the only use case of this special RLC test mode by performing an RRC Connection reconfiguration immediately after the test mode has been applied.

Users of this test mode should ensure that the RLC AM PDU carrying the incorrect sequence number will reach the peer RLC entity. It is therefore recommended to activate the RRC Connection reconfiguration only after some delay. This delay shall be short enough to ensure that the UE will not yet request the retransmission of the RLC PDU corresponding to the skipped sequence numbers.

## 7.7	System information

### 7.7.1	System information broadcasting

The rules for the transmission of BCCH messages are specified in TS 36.331 [19], clause 5.2. The current clause provides the implementation guidelines.

The ASPs SYSTEM_CTRL_REQ and SYSTEM_CTRL_CNF are used as interface to SS; the following rules apply:

-	The complete system information is provided to SS by using a single ASP.

-	SS starts scheduling all system information from the same SFN.

-	The scheduling information sent to SS is the same as the scheduling information sent to the UE. For each SI message, the subframeOffset in SYSTEM_CTRL_REQ indicates the exact point in time in the SI window at which SS shall start the transmission of the related SI.

-	SS shall set the systemFrameNumber in the MIB to the 8 most significant bits of the SFN. A dummy value is provided by TTCN.

-	If the H-SFN value is included in SIB1 sent from the TTCN, the SS shall set the hyperSFN in this message to the current H-SFN. In this case, a dummy value is provided by TTCN.

-	If the H-SFN is not included in the SIB1 sent from the TTCN, it shall not be added by the SS.

-	The system information is sent to SS using the asn.1 types, SS shall encode in unaligned PER and add the necessary padding bits as specified in TS 36.331 [19], clause 9.1.1.1.

-	In the E-UTRAN-CDMA2000 Inter RAT configuration, SS shall set the CDMA2000 synchronousSystemTime in SystemInformationBlockType8 to the SFN boundary at or after the ending boundary of the SI-window in which SystemInformationBlockType8 is transmitted (see TS 36.331 [19], clause 6.3.4). The changes of synchronousSystemTime will not result in system information change notification, nor in a modification of systemInfoValueTag in SIB1 in TTCN as specified in TS 36.331 [19], clause 6.3.1. If 1xRTT is being tested, then SS shall overwrite the longCodeState1XRTT in SystemInformationBlockType8 to the state of long code generation registers in CDMA2000 1xRTT system as defined in C.S0002-A [12, Section 1.3] at ms, where t equals to the cdma-SystemTime. The changes of longCodeState1XRTT will not result in system information change notification, nor in a modification of systemInfoValueTag in SIB1 in TTCN as specified in TS 36.331 [19], clause 6.3.1.

### 7.7.2	Scheduling information

The maximum number of resource blocks as defined in table 7.7.2-1 are used to broadcast the system information.

Table 7.7.2-1: Maximum number of resource blocks

|  | Maximum number of resource blocks assigned |
| --- | --- |
| SIB1 | 4 |
| for all SIs | 4 |

The subframe offset values used for SI messages are according to table 7.7.2-2.

Table 7.7.2-2: SubframeOffset values

| Scheduling Information No. Acc to TS 36.508 [3], clause 4.4.3.1.2 | subframeOffset (FDD) | subframeOffset (TDD) |
| --- | --- | --- |
| SI1 | 1 | 0 |
| SI2 | 1 | 0 |
| SI3 | 3 | 15 |
| SI4 | 7 | 15 |
| SI5 | 7 | 15 |

All System Information messages are sent only once within the SI-window.

Table 7.7.2-3 (FDD) and 7.7.2-4(TDD) give the SFN's and subframe numbers in which the MIB, SI1, SI2, SI3, SI4 & SI5 are actually scheduled as per default parameters for si-WindowLength(20sf), periodicity for SI1(16), SI2(32), SI3(64) , SI4(64) and SI5(64) for bandwidths 5/10/15/20 MHz defined in TS 36.508 [3]:

Table 7.7.2-3: System Information Scheduling (FDD)

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB | SI1 |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB |  |  |  |  |  |  |  |  |  |
| 2 | MIB | SI2 |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB |  |  |  |  |  |  |  |  |  |
| 4 | MIB |  |  | SI3 |  | SIB1 |  |  |  |  |
| 5 | MIB |  |  |  |  |  |  |  |  |  |
| 6 | MIB |  |  |  |  | SIB1 |  | SI4 |  |  |
| 7 | MIB |  |  |  |  |  |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  | SI5 |  |  |
| 9 | MIB |  |  |  |  |  |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  |  |  |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  |  |  |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  |  |  |  |  |  |  |
| 16 | MIB | SI1 |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB |  |  |  |  |  |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  |  |  |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  |  |  |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  |  |  |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  |  |  |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  |  |  |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  |  |  |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  |  |  |  |  |  |  |
| 32 | MIB | SI1 |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB |  |  |  |  |  |  |  |  |  |
| 34 | MIB | SI2 |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB |  |  |  |  |  |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  |  |  |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  |  |  |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  |  |  |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  |  |  |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  |  |  |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  |  |  |  |  |  |  |
| 48 | MIB | SI1 |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB |  |  |  |  |  |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  |  |  |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  |  |  |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  |  |  |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  |  |  |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  |  |  |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  |  |  |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  |  |  |  |  |  |  |
| 64 | MIB | SI1 |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB |  |  |  |  |  |  |  |  |  |
| 66 | MIB | SI2 |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB |  |  |  |  |  |  |  |  |  |
| 68 | MIB |  |  | SI3 |  | SIB1 |  |  |  |  |
| 69 | MIB |  |  |  |  |  |  |  |  |  |
| 70 | MIB |  |  |  |  | SIB1 |  | SI4 |  |  |
| 71 | MIB |  |  |  |  |  |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  | SI5 |  |  |

Table 7.7.2-4: System Information Scheduling (TDD)

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB, SI1 |  |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB |  |  |  |  |  |  |  |  |  |
| 2 | MIB, SI2 |  |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB |  |  |  |  |  |  |  |  |  |
| 4 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 5 | MIB |  |  |  |  | SI3 |  |  |  |  |
| 6 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 7 | MIB |  |  |  |  | SI4 |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  |  | SI5 |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  |  |  |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  |  |  |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  |  |  |  |  |  |  |
| 16 | MIB, SI1 |  |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB |  |  |  |  |  |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  |  |  |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  |  |  |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  |  |  |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  |  |  |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  |  |  |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  |  |  |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  |  |  |  |  |  |  |
| 32 | MIB, SI1 |  |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB |  |  |  |  |  |  |  |  |  |
| 34 | MIB, SI2 |  |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB |  |  |  |  |  |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  |  |  |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  |  |  |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  |  |  |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  |  |  |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  |  |  |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  |  |  |  |  |  |  |
| 48 | MIB, SI1 |  |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB |  |  |  |  |  |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  |  |  |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  |  |  |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  |  |  |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  |  |  |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  |  |  |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  |  |  |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  |  |  |  |  |  |  |
| 64 | MIB, SI1 |  |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB |  |  |  |  |  |  |  |  |  |
| 66 | MIB, SI2 |  |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB |  |  |  |  |  |  |  |  |  |
| 68 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 69 | MIB |  |  |  |  | SI3 |  |  |  |  |
| 70 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 71 | MIB |  |  |  |  | SI4 |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 73 | MIB |  |  |  |  | SI5 |  |  |  |  |

NOTE: Subframes 4 and 9 are avoided so as to facilitate availability of PDCCHs in the UE specific search space for transmission of PDCCH for both UL C-RNTI/SPS-RNTI and DL C-RNTI/SPS-RNTI/Temp C-RNTI.

### 7.7.2a	Scheduling information for BR System information

The scheduling of BR System Information is based upon the following conditions ( TS 36.211 [35] clause 6.4.1):

-	 =4 i.e. the schedulingInfoSIB1-BR-r13 values are selected from {1,4,7,10,13,16} which as per TS 36.213 [30] table 7.1.6-1 would result in 4 repetitions.

-	si-WindowLength-BR-r13 =20 ms (same value as si-WindowLength)

-	si-RepetitionPattern-r13 = every2ndRF (hence results eventually in only transmission in first frame of window)

-	Except for SIB1-BR (mandated for SIB1-BR by TS 36.211 [35]) hopping is assumed to be not enabled for anything else.

-	The current test plan excludes inter frequency and inter rat test cases due to limitations in RAN4. Inter frequency, inter RAT system information combinations are not considered.

-	The typical size of SIB1/2/3/4 is less than 700 bits, the Max TB size for SIx-BR is 936 bits, TS 36.213 [30] table 7.1.7.2.7-1, hence no segmentation is assumed. For SIB1-BR schedulingInfoSIB1-BR-r13 =10, and for SIB 2,3,4 the si-TBS-r13 is set as b712.

-	TTCN may configure the SS to repeat the MIB. In that case the SS shall repeat the MIB as per TS 36.331[19] clause 5.2.1.2 in subframe#9 of the previous radio frame for FDD and subframe #5 of the same radio frame for TDD, the same can be achieved in the below schedule.

-	System information broadcasting rules defined in clause 7.7.1 apply with the exception that SIB1 is replaced with SIB1 & SIB1-BR and SI is replaced with SI & SI-BR.

The subframe offset values used for SI messages are according to table 7.7.2a-1.

Table 7.7.2a-1: SubframeOffset values

| Scheduling Information No. Acc to TS 36.508 [3], clause 4.4.3.1.2 | subframeOffset (FDD) | subframeOffset (TDD DL bandwidth > 5 MHz) | subframeOffset (TDD DL bandwidth = 5 MHz) |
| --- | --- | --- | --- |
| SI1 | 1 | 0 | 10 |
| SI2 | 1 | 0 | 10 |
| SI3 | 3 | 0 | 10 |
| SI4 | 3 | 0 | 10 |
| SI1-BR | 0 | 0 | 0 |
| SI2-BR | 0 | 0 | 0 |
| SI3-BR | 0 | 0 | 0 |
| SI4-BR | 0 | 0 | 0 |
| NOTE: The subframe offsets for SIx/SIx-BR of 0 and 10 are suitable for all TDD uplink-downlink configurations. |  |  |  |

The following tables give the SFN and subframe numbers in which MIB, SIs and SI-BRs are actually scheduled as per default parameters defined in TS 36.508 [3] for:

Table 7.7.2a-2 for FDD with even physical layer cell IDs

Table 7.7.2a-4 for FDD with odd physical layer cell IDs

Table 7.7.2a -3 for TDD for DL bandwidth > 5 MHz and even physical layer cell Id

Table 7.7.2a-5 for TDD for DL bandwidth > 5 MHz and odd physical layer cell Id

Table 7.7.2a-6 for TDD for DL bandwidth = 5 MHz.

Table 7.7.2a-2: BR System Information Scheduling (FDD) for even physical layer cell IDs[=0]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 1 | MIB |  |  |  |  |  |  |  |  |  |
| 2 | MIB SI2-BR | SI2 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 3 | MIB |  |  |  |  |  |  |  |  |  |
| 4 | MIB SI3-BR |  |  | SI3 | SIB1-BR | SIB1 |  |  |  |  |
| 5 | MIB |  |  |  |  |  |  |  |  |  |
| 6 | MIB SI4-BR |  |  | SI4 | SIB1-BR | SIB1 |  |  |  |  |
| 7 | MIB |  |  |  |  |  |  |  |  |  |
| 8 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  |  |  |  |  |  |  |
| 10 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  |  |  |  |  |  |  |
| 12 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  |  |  |  |  |  |  |
| 14 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  |  |  |  |  |  |  |
| 16 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 17 | MIB |  |  |  |  |  |  |  |  |  |
| 18 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  |  |  |  |  |  |  |
| 20 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  |  |  |  |  |  |  |
| 22 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  |  |  |  |  |  |  |
| 24 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  |  |  |  |  |  |  |
| 26 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  |  |  |  |  |  |  |
| 28 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  |  |  |  |  |  |  |
| 30 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  |  |  |  |  |  |  |
| 32 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 33 | MIB |  |  |  |  |  |  |  |  |  |
| 34 | MIB SI2-BR | SI2 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 35 | MIB |  |  |  |  |  |  |  |  |  |
| 36 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  |  |  |  |  |  |  |
| 38 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  |  |  |  |  |  |  |
| 40 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  |  |  |  |  |  |  |
| 42 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  |  |  |  |  |  |  |
| 44 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  |  |  |  |  |  |  |
| 46 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  |  |  |  |  |  |  |
| 48 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 49 | MIB |  |  |  |  |  |  |  |  |  |
| 50 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  |  |  |  |  |  |  |
| 52 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  |  |  |  |  |  |  |
| 54 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  |  |  |  |  |  |  |
| 56 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  |  |  |  |  |  |  |
| 58 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  |  |  |  |  |  |  |
| 60 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  |  |  |  |  |  |  |
| 62 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  |  |  |  |  |  |  |
| 64 | MIB SI1-BR | SI1 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 65 | MIB |  |  |  |  |  |  |  |  |  |
| 66 | MIB SI2-BR | SI2 |  |  | SIB1-BR | SIB1 |  |  |  |  |
| 67 | MIB |  |  |  |  |  |  |  |  |  |
| 68 | MIB SI3-BR |  |  | SI3 | SIB1-BR | SIB1 |  |  |  |  |
| 69 | MIB |  |  |  |  |  |  |  |  |  |
| 70 | MIB SI4-BR |  |  | SI4 | SIB1-BR | SIB1 |  |  |  |  |
| 71 | MIB |  |  |  |  |  |  |  |  |  |
| 72 | MIB |  |  |  | SIB1-BR | SIB1 |  |  |  |  |

Table 7.7.2a-3: BR System Information Scheduling (TDD) for DL bandwidth > 5 MHz [] and even physical layer cell Id [=0]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 2 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 4 | MIB, SI3, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 5 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 6 | MIB, SI4, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 7 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 16 | MIB, SI1 SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 32 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 34 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 48 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 64 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 66 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 68 | MIB, SI3, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 69 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 70 | MIB, SI4, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 71 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 73 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |

Table 7.7.2a-4: BR System Information Scheduling (FDD) for odd physical layer cell IDs[=1]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 2 | MIB SI2-BR | SI2 |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 4 | MIB SI3-BR |  |  | SI3 |  | SIB1 |  |  |  |  |
| 5 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 6 | MIB SI4-BR |  |  | SI4 |  | SIB1 |  |  |  |  |
| 7 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 16 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 32 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 34 | MIB SI2-BR | SI2 |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 48 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 64 | MIB SI1-BR | SI1 |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 66 | MIB SI2-BR | SI2 |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 68 | MIB SI3-BR |  |  | SI3 |  | SIB1 |  |  |  |  |
| 69 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 70 | MIB SI4-BR |  |  | SI4 |  | SIB1 |  |  |  |  |
| 71 | MIB |  |  |  | SIB1-BR |  |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |

Table 7.7.2a-5: BR System Information Scheduling (TDD) for DL bandwidth > 5 MHz [] and odd physical layer cell Id [=1]

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 2 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 4 | MIB, SI3, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 5 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 6 | MIB, SI4, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 7 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 16 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 32 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 34 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 48 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 64 | MIB, SI1, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 66 | MIB, SI2, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 68 | MIB, SI3, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 69 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 70 | MIB, SI4, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 71 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 73 | MIB, SIB1-BR |  |  |  |  |  |  |  |  |  |

Table 7.7.2a-6: BR System Information Scheduling (TDD) for DL bandwidth = 5 MHz

| SFN\SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 1 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 2 | MIB, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 3 | MIB, SI2 |  |  |  |  | SIB1-BR |  |  |  |  |
| 4 | MIB, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 5 | MIB, SI3 |  |  |  |  | SIB1-BR |  |  |  |  |
| 6 | MIB, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 7 | MIB, SI4 |  |  |  |  | SIB1-BR |  |  |  |  |
| 8 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 9 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 10 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 11 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 12 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 13 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 14 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 15 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 16 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 17 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 18 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 19 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 20 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 21 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 22 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 23 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 24 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 25 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 26 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 27 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 28 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 29 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 30 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 31 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 32 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 33 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 34 | MIB, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 35 | MIB, SI2 |  |  |  |  | SIB1-BR |  |  |  |  |
| 36 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 37 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 38 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 39 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 40 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 41 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 42 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 43 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 44 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 45 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 46 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 47 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 48 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 49 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 50 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 51 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 52 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 53 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 54 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 55 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 56 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 57 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 58 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 59 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 60 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 61 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 62 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 63 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |
| 64 | MIB, SI1-BR |  |  |  |  | SIB1 |  |  |  |  |
| 65 | MIB, SI1 |  |  |  |  | SIB1-BR |  |  |  |  |
| 66 | MIB, SI2-BR |  |  |  |  | SIB1 |  |  |  |  |
| 67 | MIB, SI2 |  |  |  |  | SIB1-BR |  |  |  |  |
| 68 | MIB, SI3-BR |  |  |  |  | SIB1 |  |  |  |  |
| 69 | MIB, SI3 |  |  |  |  | SIB1-BR |  |  |  |  |
| 70 | MIB, SI4-BR |  |  |  |  | SIB1 |  |  |  |  |
| 71 | MIB, SI4 |  |  |  |  | SIB1-BR |  |  |  |  |
| 72 | MIB |  |  |  |  | SIB1 |  |  |  |  |
| 73 | MIB |  |  |  |  | SIB1-BR |  |  |  |  |

### 7.7.3	System information modification

For system information modification, the same rules as defined in clause 7.7.1 are applied.

The SFN and the H-SFN for the start of modification period are calculated by TTCN. The modified system information and the calculated SFN, H-SFN are provided in the ASP SYSTEM_CTRL_REQ.

#### 7.7.3.1	Non-PWS System Information modification

The modification of system information is notified by paging messages containing the systemInfoModification (normal DRX) or systemInfoModification-eDRX (extended DRX). The paging messages are sent during one modification period before broadcasting the modified system information for normal DRX, and within a Paging Time Window (PTW) for extended DRX. The paging messages are sent on paging occasions (PO) within the paging frames (PF). With the default paging and sysinfo parameters provided in TS 36.508 [3] PO is set to 9 for FDD and 0 for TDD.

In case the UE is configured by the SS with an extended DRX cycle and the cell indicates support for eDRX in System Information, eDRX is performed. See to TS 36.304 [14], clause 7.3

##### 7.7.3.1.1	UE in Idle_mode

7.7.3.1.1.1	Paging in DRX when P-RNTI transmitted on PDCCH

In the case of normal DRX, the paging frames calculation is based on the UE identity (see to TS 36.304 [14], clause 7). With:

defaultPagingCycle=128

nB=oneT

modificationPeriodCoeff=n4

it results in 4 paging messages to be sent on the paging occasions during the modification period in the frames of:

SFN mod 128 = (UE_ID) mod 128.

7.7.3.1.1.2	Paging in extended DRX

In the case of eDRX, the DRX cycle is longer than the modification period and the parameter is the Paging Hyperframe (PH) and refers to the hyper SFN (H-SFN) in which the UE starts monitoring POs during a Paging Time Window (PTW). The H-SFN is broadcasted by the cell and increments by one when the SFN wraps around. When a change notification is received, the UE acquires the updated system information at the next H-SFN boundary defined by:

H-SFN mod 256 = 0

If the UE is configured with a TeDRX cycle of 512 radio frames, it monitors POs as defined on normal DRX with parameter T = 512, otherwise it will monitor POs calculated as normal DRX only (as per clause 7.7.3.1.1.1 or 7.7.3.1.1.3) within the periodic PTW configured for the UE. The calculation of the bounds for the PTW are UE-specific and described on TS 36.304 [14] clause 7.3.

Figure 7.7.3.1.1.2-1: POs in RRC_IDLE state when eDRX cycle > 512 radio frames

7.7.3.1.1.3	Paging in DRX when P-RNTI transmitted on MPDCCH

BL/CE UEs can be notified using Direct Indication information or paging message.

For BL/CE UEs, the paging frames calculation is based on the UE identity (see to TS 36.304 [14], clause 7.1). With:

defaultPagingCycle=128

nB=oneT

modificationPeriodCoeff=n4

number of paging narrowbands=1

SystemInformationBlockType1-BR modification period = 512 frames

SIs-BR modification period = 512 frames

The paging occasion calculation is resulting in the frames of: SFN mod 128 = (UE_ID) mod 128. The Paging Narrowband (PNB) is resulting in PNB=0 and then resulting in the Narrow Band used for MPDCCH defined in clause 7.3.3.9.1.

One paging message is sent on the calculated paging occasion before the start of SystemInformationBlockType1-BR modification period. When both SystemInformationBlockType1-BR and SIs-BR are modified, their respective modification period starts may not match:

-	When they match, SS is configured with the modified SystemInformationBlockType1-BR and SIs-BR in one ASP

-	When they do not match, SS is configured with the modified SystemInformationBlockType1-BR and SIs-BR in two different ASPs with activation time set to the start of the respective modification periods.

##### 7.7.3.1.2	UE in connected mode

When the UE is in connected mode, paging messages are sent on the paging occasions of each frame within the paging cycle throughout a modification period. This results in 128*4 consecutive paging messages to be sent during the modification period.

Figure 7.7.3.1.2-1: Paging notification UE in connected mode

For ETWS and/or CMAS capable UEs in connected mode, paging messages are sent on the paging occasions of each frame within the last paging cycle of the modification period. This results in 128 consecutive paging messages to be sent during the modification period.

Figure 7.7.3.1.2-2: Paging notification for ETWS and/or CMAS capable UE in connected mode

#### 7.7.3.2	PWS System Information modification

The modification of system information for ETWS and CMAS notification is notified by paging messages. The transmission of system information notification is not necessarily at the beginning of a modification period.

When the UE is in idle mode, the paging frames calculation is the same as defined in clause 7.7.3.1.1.

When the UE is in connected mode, paging messages are sent on the paging occasions of each frame during a paging cycle. This results in 128 consecutive paging messages to be sent. The transmission of the first paging message and the first system information notification are simultaneous and are sent at the beginning of a paging cycle.

## 7.8	Timers and Timing Considerations

A timer is set at the beginning of each test case to guard against system failure. Behaviour on expiry of this guard timer shall be consistent for all test cases.

A watchdog timer can be specified for receive statements in order to reduce blocking time when a test case has already failed. Watchdog timers are a kind of TTCN auxiliary timer. When a watchdog timer is used to control a receive event, its expiry does not need to be handled explicitly in the test case, but will lead to a fail or inconclusive verdict due to handling in the default behaviour.

In idle mode operations, an idle mode generic timer is specified for receive statements if the test case specification does not explicitly specify a wait time for the specific test step or test purpose. The expiry of this idle mode generic timer is at least 6 minutes to safely cover most test scenarios.

The watchdog timer and the idle mode generic timer are only to be used inside the test case test body; if the timer expires a fail verdict is applied.

It is the TTCN responsibility to ensure that appropriate timer values are being used.

Tolerances (as described in TS 36.508 [3]) are not applicable to guard timers, idle mode generic timers and watchdog timers.

In general timers of less than 500 ms shall not be implemented by TTCN timers but controlled by usage of the timing information provided by the SS (This is based on an estimate of the system delay). To achieve this, there will be cases when a DL message is scheduled at a specific point in time. This shall be done by adding at least 100 ms to the current time.

If Timing is ‘now' the SS shall schedule the data transmission or the (re)configuration in the next available sub-frame, but will ensure that this period is less than 80 ms.

### 7.8.1	Auxiliary timers

For practical reasons, the TTCN can include timers that are not specified as part of the expected sequence. These timers are documented below.

RLC and PDCP watchdog timer.

### 7.8.2	RRC timers reconfiguration

Considering the allowed UE accuracy for the RRC timer T3xx being between 100 ms and 2.5 % of T3xx (see TS 36.133 [37]), the TTCN applies the RRC net timers tolerance as MAX (10% of T3xx, (100 ms + 5 RTT)), whereby:

FDD: 10 % of T3xx or 140 ms whichever is higher.

TDD: 10 % of T3xx or 155 ms whichever is higher.

### 7.8.3	MAC TA timer reconfiguration

Considering that the UE applies new values for MAC timers not before restart of the timer (see TS 36.321 [16], clause 5.8), when the TA timer is changed at the UE, a delay in TTCN will be added so as to allow SS to transmit Timing Advance Command MAC CE (based on current periodic Timing Advance configuration) and hence resulting in restart of TA timer at UE with new value.

### 7.8.4	Non-protocol timers

Time durations or periods in the test specification without corresponding references in the core specifications are considered as non-protocol timers for which no timer tolerances are applied in the TTCN.

### 7.8.5	Timing information

For scheduling of DL data or UL Grant, when timing information is explicit, the timing provided by TTCN corresponds to the time at which the SS shall transmit to the UE the (M)PDCCH carrying the DCI message.

## 7.9	Error Indication

There are several situations on lower layer in which SS shall raise an error rather than trying to resolve the problem. This is done by sending a SystemIndicationError to the test case. SS shall raise an error in the following cases:

-	HARQ retransmissions (applicable when SS is configured to indicate HARQ retransmissions as errors):

-	HARQ CRC error for UL data;

-	HARQ NACK from the UE unless SS is configured to report HARQ ACK/NACK.

-	Paging, System information exceeds max. number of resource blocks.

-	Configuration: max. number of resource blocks specified for a channel exceeds system bandwidth.

-	When in User-Plane a DL PDCP PDU or SDU not fitting into one TTI is sent with Harq Process being explicitly specified.

-	SS gets invalid TimingInfo for TDD from the test case.

-	SS detects contradiction of UL grant(s) and TDD configuration.

-	Data scheduled for the same TTI does not fit into an available transport block.

Further error conditions are specified in annex D.

## 7.10	Race Conditions

When two uplink messages are sent from the UE within a very small amount of time, they may be received in either order in the TTCN if they are received on different ports. This may cause a race condition which is due to the snapshot mechanism in TTCN. In these cases, the TTCN will accept the messages in either order and then compare the timestamps of both messages to ensure they were sent in the correct order.

For UL messages received at a single port, there are normally no race conditions, with the exception of the SRB port where the following rules shall be fulfilled, in order to achieve an ordered UL message queue:

-	UL messages are queued according to the timing information.

-	UL messages with the same timing information are queued according to the logical channel priority with the "higher-first-in" principle.

## 7.11	Radio Link Failure

A radio link failure shall be triggered by switching the downlink power level of the source cell to the value for non-suitable "Off" for the time period of least T310 + time it takes to receive N310 consecutive out-of-sync indications from lower layers (non-suitable "Off" is defined in TS 36.508 [3], whereas T310 and N310 are defined in TS 36.331 [19]).

If the RRC re-establishment procedure is used in a radio link failure context, it shall be realised by using two cells.

## 7.12	Test method for RRC signalling latency

Test cases testing RRC signalling latency will need special test method. The PUCCH synchronisation state of UE influences the test method. Following 2 different ways in which the UE's completeness of procedure can be probed are considered:

1.	UE is still PUCCH synchronized and can respond to uplink grants.

2.	UE needs a RACH procedure and hence RACH procedural delays add upon the actual procedure delay.

### 7.12.1	Procedure delays in PUCCH synchronized state

For latency tests there may be up to 4 HARQ retransmissions in DL (corresponding to the default configuration of the SS) but HARQ retransmissions in UL cannot be compensated , i.e. any HARQ error in UL shall result in an inconclusive verdict for the test case (otherwise a UE may get fail due to a HARQ error).

Figure 7.12.1-1 demonstrates the latency check procedure that will be applied when UE is in PUCCH synchronized state and can respond to uplink grants.

SS is configured to report ACK/NACK received from UE, to TTCN.

NOTE:	Due to L2 signalling (e.g. RLC STATUS PDUs) it is necessary to limit the reporting of UL HARQ ACK/NACK to the time between sending of the RRC message and receiving the ACK.

By default SS is configured to retransmit any DL MAC PDU max 4 times.

To avoid unexpected side effects the Time Alignment timer needs to be set to infinity and the SS shall be configured to not send any Timing Advance MAC control elements during the latency tests (since this may result in additional ACK/NACK)

The SS shall be configured to report HARQ errors and in the case of an UL HARQ error, an inconclusive verdict is assigned.

In the case of HARQ retransmissions in DL the HARQ RTT Timer according to TS 36.321 clause 7.7 [16] is

-	8 for FDD

-	10 for TDD configuration 1 in case the DL PDU is sent in subframe 4 (as per default; see Table 7.12.1-1).

For type B FDD half-duplex, the restrictions provided in clause 7.26 shall be respected.  This results in an RTT of 10.

For Cat M1, explicit timing is applied with T0 pointing to PDSCH.

The SS shall schedule DL retransmission at 4th TTI for FDD and TDD since reception of the NACK.

Let N be the max allowed delay for procedure.

TTCN schedules at time T1 a DL message to the UE.

TTCN schedules UL grants at

T2(k) = T1 + N + 1 + ∆1 + k * RTT;

with 
k = 0..4; number of HARQ retransmission in DL
RTT = 8 (FDD)
RTT = 10 (TDD) 
∆1 = 0 (FDD)
∆1 = 0..3 (TDD; possible UL subframe uncertainty since not all subframes can be used for UL)

Example: 
given TDD; DL PDU sent at subframe 4; N=10
 ∆1 = 1 since UL grant cannot be scheduled for subframe 5 but needs subframe 6 (36.213 [30] cl.8.0)

The UL data is sent by the UE at

T3(K) = T2(K) + 4 + ∆2 with ∆2 = 0 for FDD and ∆2 = 0..3 for TDD and K is the value of k corresponding to which a HARQ Ack is received

The latency requirements are fulfilled when

T3(K) - T1 = N + 5 + ∆1 + ∆2 + K * RTT

Looking at TDD configuration 1 in detail it can be shown that ∆ = ∆1 + ∆2 = 0 .. 3

T3(K) - T1 = N + 5 + ∆ + k * RTT; with ∆ = 0 .. 3

NOTE:
as long as N is a multiple of 5ms, given T1 is at 4th TTI for TDD configuration 1 we get ∆ = 3 (∆1 = 1 and ∆2 = 2, 36.213 [30] cl.8.0)

Figure 7.12.1-1: Delays in PUCCH synchronized state

Table 7.12.1-1: TDD configuration 1

| Subframe | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Configuration 1 | D | S | U | U | D | D | S | U | U | D |
| Delay from DL to Ack/Nack [TTIs] |  |  | 6,7 | 4 |  |  |  | 6,7 | 4 |  |
| Delay from NCK to re tx [TTIs] |  |  | 4 | 6 |  |  |  | 4 | 6 |  |
| RTT | 11 | 10 |  |  | 10 | 11 | 10 |  |  | 10 |

### 7.12.2	Procedure delays when RACH procedure required

Figure 7.12.2-1 demonstrates the latency check procedure that will be applied when UE is not PUCCH synchronized state needs RACH procedure.

PRACH configuration index is set as 14 for FDD, 12 for TDD which allows UE to send Preamble in any frame at any subframe.

SS is configured to report ACK/NACK, PRACH preambles received from UE.

By default SS is configured to retransmit any DL MAC PDU max 4 times [1 Transmission and 4 Retransmission].

Let N be the max allowed delay for procedure.

TTCN schedules at time T1, DL message to the UE. This is achieved using Time stamps in send ASP's.

The time difference between the ACK and the reception of PRACH preamble will be checked against N plus any Interruption time (TS 36.133 [37]) and verdict is assigned, when (Y-X) <= N + Tinterrupt + ∆:

∆ = 0 for FDD;

∆ = 3TTI for TDD, where 3TTI is UL subframe uncertainty.

If cell change occurs, cell timing differences, Frame number offsets need to be included for procedural delay evaluations.

Figure 7.12.2-1: Delays when RACH procedure needed

## 7.13	RLC test method for scheduled data

The test loop mode is applied to the RLC tests. The allowed SS delay for sending data (< 80 ms) is comparable to the default values of the RLC timers. In order to ensure a unique TTCN implementation of the RLC test cases and the deterministic test result, independent from the SS platforms and UEs, scheduled data method can be applied to the test.

The scheduled data method is suitable to the RLC test if:

Receiving multiple UL RLC SDUs is expected in the test; the UE may send a STATUS PDU in addition.

Time measurement is required for the looped back RLC SDUs.

DL RLC PDUs are sent on consecutive TTIs; the subframe numbers to be applied are relevant in TDD.

Table 7.13-1 illustrates the data scheduling in the RLC test.

Table 7.13-1: Scheduled RLC test events

| Scheduled timing |  | t0 (see note 1) | t1 (see note 1) | t2 |
| --- | --- | --- | --- | --- |
| Test event descriptions | Multiple SDUs | Obtain the reference time | Send DL data | Provide UL grant (see note 2) |
|  | Time measurement |  | Send DL data | Receive UL data |
|  | DL data in TDD |  | Send 1st DL data | Send subsequent data (see note 3) |
| NOTE 1: (t1-t0) ≥ 100 ms which is greater than the allowed SS max. delay time, 80 ms. NOTE 2: (t2-t1) = 60 ms, this duration will allow the UE transmitting max. 3 scheduling requests (every 20 ms once) after the UL data to be looped back being available at the UE without going onto PRACH. NOTE 3: The applied TDD subframe numbers 4, 5, 9, 10, 14, 15, 19, 20, 24, 25,... |  |  |  |  |

If the test case prose does not indicate timely restrictions for the scheduling, sequential sending events are scheduled in consecutive TTIs.

NOTE 1:	 For TDD configuration 1, the subframes 0, 4, 5 and 9 are considered as consecutive.

NOTE 2:	Scheduling may imply to execute the test steps in the TTCN in an order different from the order given in the test case prose. However, the sequence of the events over the air follows the prose description.

## 7.14	IP packets for Loopback Mode

### 7.14.1	IP packets used for Loopback Mode A

It is irrelevant which kind of data is used in loopback mode A. Some PDCP test cases however specify to use IP packets. In these cases, an ICMPv4 ECHO REPLY shall be used with a valid IP header checksum and valid ICMP checksum.

### 7.14.2	IP packets used for Loopback Mode B

According to TS 36.509 [4], the UE performs loopback mode B above the UL TFT entity. Therefore IP packets need to match the packet filters signalled to the UE according to TS 36.508 [3], clause 6.6.2:

a)	IP packets used when a dedicated bearer is configured with packet filters #1 and #2

When the UE gets configured via NAS signalling with packet filters #1 and #2 according to TS 36.508 [3] clause 6.6.2 the IP packets shall fulfil the following requirements:

Protocol:

UDP referred to packet filter #1 and #2

IP addresses:

Referred to TS 36.508 [3], table 6.6.2-3, note 1 source and destination IP address are the same.

Ports:

packet filter #1 specifies DL filter  IP packet's source port shall match remote port of packet filter #1.

packet filter #2 specifies UL filter  IP packet's destination port shall match remote port of packet filter #2.

To summarize, on dedicated bearers for loopback mode B, UDP packets used shall match the packet filters configured at the UE side. The UDP packets, having no specific content, shall have the correct header checksum and UDP checksum. On the default bearer, any other packets can be used, as an example, ICMPv4 ECHO REPLY similar as for loopback mode A.

b)	IP packets used when a dedicated bearer is configured with packet filters #6 and #7

When the UE gets configured via NAS signalling with packet filters #6 and #7 according to TS 36.508 [3] clause 6.6.2 the IP packets used on the dedicated bearer shall fulfil the following requirements:

Protocol:

ICMP or ICMPv6 depending on the packet filters being for IPv4 or IPv6

IP addresses:

The same network address shall be used for the source and destination address of the IP packet as used as remote address for the packet filters.

If there are IP packets to be used on the default bearer too, they can be the same as for the dedicated bearer but with the UE’s address being used for the source and destination address.

c)	IP packets used for default bearer when no associated dedicated bearer is configured

When there is no dedicated bearer and therefore no packet filters being configured any valid IP packet may be used for loopback mode B on a default bearer.

## 7.15	Connected Mode DRX

The SS shall support connected mode DRX according to TS 36.321 [16], i.e. the SS shall not send any data to the UE while the UE is not monitoring the PDCCH. To achieve this, the SS needs to estimate the UE's Active Time by considering the on-duration as well as the drx-inactivity timer:

on-duration:

The on-duration can be derived from the SS' DRX configuration.

drx-inactivity timer:

According to TS 36.321 [16], clause 5.7 at the UE the drx-inactivity timer is started or restarted during the Active Time whenever PDCCH indicates a new transmission (DL or UL).

There is no activation time for the configuration of DRX at the UE and it is not acceptable just to consider the on-duration after re-configuration of the UE (for DRX_L according to TS 36.508 [3] the DRX cycle is 1.28 s); instead the drx-inactivity timer needs to be taken in account after DRX reconfiguration as well.

The following rules shall be applied to achieve synchronisation of SS and UE:

1.	SS shall consider drx-inactivity timer as restarted at the UE whenever the UE is addressed on the PDCCH (DL data or UL grant).

2.	When there is a scheduling request sent by the UE, SS assigns a grant independent of DRX;
when sending out that grant on PDCCH SS considers drx-inactivity timer as (re-)started (as per 1. above).

3.	For all DL messages scheduled with specific timing information SS shall send the data at the given time irrespective of current DRX configuration.

4.	DRX (re-)configuration:

a)	when DRX has not been configured at the UE yet:

a1)	TTCN will configure the SS just before the sending out the RRCConnectionReconfiguration message configuring DRX at the UE; no other send-events between the reconfiguration of the SS and sending the RRC message shall be scheduled in TTCN.

a2)	TTCN will schedule sending of the RRCConnectionReconfiguration message configuring DRX with specific timing information.

b)	Reconfiguration of DRX at the UE: Same as a) but:

b1)	TTCN shall schedule sending of the RRCConnectionReconfiguration according to the old DRX configuration (i.e. the SS does not need to cache the new configuration).

c)	RRC connection release:

c1)	TTCN will release DRX at the SS just after the RRC connection release procedure.

5.	There shall be no parallel data on any DRBs during DRX reconfiguration.

NOTE:	Timing requirements in the DRX test cases:

a)	The drx-Inactivity Timer shall be long compared to the duration between sending RRCConnectionReconfiguration and receiving RRCConnectionReconfigurationComplete (> 50 ms). It ensures the SS in-time sending of the RLC STATUS PDU.

or

b)	The drx-cycle shall be short compared to the RLC timers applied for SRB1.

Figure 7.15-1 illustrates DRX (re)configuration at the SS and the UE.

Figure 7.15-1: DRX (Re)configuration

NOTE 1:	Between RRCConnectionReconfiguration and RRCConnectionReconfigurationComplete the UE may send a separate RLC STATUS PDU to acknowledge the RRCConnectionReconfiguration, but that does not affect the principle as long as SS applies rule 2.

NOTE 2:	During the "greyzone" SS does not know about DRX configuration at the UE; during that period according to rule 4a1 and rule 5 there is no data to be sent by SS.

The TTCN (re)configures the connected mode DRX in SS for the test cases if DRX_S is applied (Ref. TS 36.508 [3]. The (re)configuration of DRX_L in SS is FFS.

Together with DRX, the SS shall implement eDRX for which an extended longDRX-Cycle can be configured. The values of eDRX cycle can be of 5.12 and 10.24s for connected mode. When TTCN configures the eDRX cycle on the SS alongside the DRX configuration, the SS shall apply it.

For test case 7.1.6.1 and 7.1.6.2, DRX will not be activated at the SS. Periodic UL grants every 5ms (suitable for both FDD and TDD and less than drx-InactivityTimer 6ms) will be allocated to the UE during the steps configuring test case specific DRX parameters of the test case to prevent UE from activating DRX; These grants may result in padding MAC PDU's transmitted by UE, which will be received by SS MAC and discarded.

## 7.16	Handover Sequences

### 7.16.1	Sequence of inter-cell handover

In general, the Inter-Cell handover is done without activation time, i.e. the timing information for configuration of the SS and sending of the RRCConnectionReconfiguration is ‘Now'.

The sequence may be interrupted if other events need to be handled. E.g. when a TAU procedure is performed in the target cell and there are procedures left to be executed on the source cell.

1.	Transfer of the PDCP Count for AM DRBs from source to target cell:

a)	Source Cell: Get PDCP COUNT.

b)	Target Cell: Set PDCP COUNT.

NOTE 1:	There shall be no further sending/receiving of AM DRB data before the HO has been done.

2.	Target Cell:	Inform the SS about the HO and about the source cell id.

3.	Target Cell:	Configure RACH procedure either dedicated or C-RNTI based.

4.	Target Cell:	Activate security.

NOTE 2:	For AM DRBs the PDCP count is maintained (for SRBs and UM DRBs the PDCP count is reset).

5.	Target Cell:	configure DRX (if necessary).

NOTE 3:	As long as the DRX configuration is not modified by the RRCConnectionReconfiguration the target cell gets the same DRX configuration as the source cell.

NOTE 3A:	According to TS 36.331 clause 5.5.6.1 the measurement gap configuration is released at the UE due to the handover, therefore nothing needs to be configured at the target cell regarding measurement gaps.

6.	Source Cell:	Stop periodic TA.

NOTE 4:	Unless explicitly specified UL grant configuration keeps configured as per default at the source cell.

7.	Target Cell:	Configure UL grant configuration ("OnSR", periodic TA is not started).

8.	Source Cell:	Send RRCConnectionReconfiguration.

9.	Target Cell:	Receive RRCConnectionReconfigurationComplete.

10.	Target Cell:	Start periodic TA.

11.	Target Cell:	Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS PDU).

12.	Target Cell:	Re-configure RACH procedure as for initial access.

12a.	Target Cell:	Configure measurement gap configuration (if necessary).

13.	Source Cell:	Reset SRBs and release DRBs.

14.	Source Cell:	Release DRX and MeasGapConfig configuration.

### 7.16.1a	Sequence of inter-cell CA handover (more than one CC before and after handover)

The Inter-Cell handover is done with activation time, i.e. the timing information for configuration of the SS and sending of the RRCConnectionReconfiguration is explicit. Time ‘T’ is set to 700 ms in advance of the handover, time T1 = T + 10 ms and time T2 = T +20 ms.

At Time T, steps 1-3:

1.	Source Pcell: Configure source primary cell for stop of automatic Time alignment MCE transmission by SS

2.	Target Pcell: Configure target pcell for no RACH response transmission

3.	Source Pcell: Schedule the transmission of RRC Connection Reconfiguration message to UE requesting Handover to target Pcell and Scell

At time T2, steps 4-5:

4.	Target Pcell: If target Pcell is same as source Scell, configure SS for target Pcell to be converted from a Scell to Pcell

4a.	Configuration of DRBs at the target PCell

5.	Transfer of the PDCP Count for AM DRBs from source to target Pcell:

a)	Source PCell: Get PDCP COUNT.

b)	Target PCell: Set PDCP COUNT.

NOTE 1:	There shall be no further sending/receiving of AM DRB data before the HO has been done.

At time T1, steps 6-12:

6.	Target PCell:	Inform the SS about the HO and about the source Pcell id.

7.	Target PCell:	Configure RACH procedure either dedicated or C-RNTI based.

8.	Target PCell:	Activate security.

NOTE 2:	For AM DRBs the PDCP count is maintained (for SRBs and UM DRBs the PDCP count is reset).

9.	Target PCell & Target Scell:	configure DRX.

NOTE 3:	As long as the DRX configuration is not modified by the RRCConnectionReconfiguration the target cell gets the same DRX configuration as the source cell.

NOTE 4:	According to TS 36.331 clause 5.5.6.1 the measurement gap configuration is released at the UE due to the handover, therefore nothing needs to be configured at the target cell regarding measurement gaps.

10.	Target PCell:	Configure UL grant configuration ("OnSR", periodic TA is not started).

11.	Target PCell:	Configure Target Pcell as Pcell.

12.	Target SCell:	Configure Target Scell for

12.1	Configure RACH procedure C-RNTI based.

12.2	Configure UL grant configuration ("OnSR", periodic TA is not started).

12.3	Configure target SCell as Scell with new Pcell association.

After time T2 (without activation time):

13.	Target PCell:	Receive RRCConnectionReconfigurationComplete.

14.Target PCell:	Start periodic TA.

15.	Target PCell:	Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS PDU).

16.	Target PCell:	Re-configure RACH procedure as for initial access.

16a.	Target Pcell & TargetSCell: Configure measurement gap configuration (if necessary).

17.	Source PCell:	If source Pcell is not target Scell, reset SRBs and release DRBs.

18.	Source Pcell:	If source Pcell is not target Scell, configure from Pcell to normal cell.

19.	Source Pcell:	If source Pcell is not target Scell, Release DRX and MeasGap configuration

20.	Source SCell:	If source Scell is neither target Pcell or Scell, Release DRX and MeasGapConfig configuration.

21.	Source Scell:	If source Scell is neither target Pcell or Scell, configure from Scell to normal cell.

22.	Source Scell:	If source Scell is neither target Pcell or Scell, Configure UL grant configuration ("OnSR", periodic TA is not started).

### 7.16.2	Sequence of intra-cell handover

For Intra-Cell handover dedicated timing information is used: the sequence starts at time T with sending of the RRCConnectionReconfiguration. T is set to 300 ms in advance of the handover.

0.	Before T:	Get PDCP count for AM DRBs.

1.	At T:	Send RRCConnectionReconfiguration.

2.	At T + 5ms:	Release SRBs and DRBs.

3.	At T + 5ms:	Configure RACH procedure either dedicated or C-RNTI based.

NOTE 1:	Since the RACH procedure may require a new C-RNTI to be used it cannot be configured before sending out the RRCConnectionReconfiguration.

3A	At T + 5ms:	Release MeasGapConfig configuration.

NOTE 2:	According to TS 36.331, clause 5.5.6.1 the measurement gap configuration is released at the UE due to the handover, therefore MeasGapConfig is released unless a new measurement gap configuration is explicitly given in the RRCConnectionReconfiguration.

4.	At T + 10ms:	(Re-) configure SRBs and DRBs.

5.	At T + 10ms:	Re-establish security, disable TA transmission.

NOTE 3:	For AM DRBs the PDCP count is maintained while for SRBs and UM DRBs the PDCP count is reset.

6.	(after step 5)	Receive RRCConnectionReconfigurationComplete.

7.	(after step 6)	Re-configure RACH procedure as for initial access, enable TA transmissions.

8.	(after step 7) 	Restore the PDCP count for AM DRBs.

### 7.16.3	UL Grants used in RA procedure during handover

In the Random Access Procedure a grant is assigned to the UE by the Random Access Response and another grant, as initial grant, is assigned for contention resolution.

When UL data is pending, the UE will try to put as much data into given grants as possible, i.e. it will segment the user data and send it e.g. with the initial grant if possible. To avoid this segmentation of user data, the grants assigned during handover will be set in TTCN to:

Grant assigned by Random Access Response:	56 bits.

Initial grant:	104 bits.

NOTE 1:	According to TS 36.321 [16], clause 5.1.4, 56 bits are the minimum grant which can be assigned by the Random Access Response. That is sufficient to convey C-RNTI (3 bytes) and short BSR (2 bytes) or long BSR (4 bytes) but even with short BSR the remaining 2 bytes are not sufficient to convey any segment of the RRCConnectionReconfigurationComplete (at least 4 bytes).

NOTE 2:	The RRCConnectionReconfigurationComplete (9 bits) shall completely be conveyed in the initial grant of RA procedure. This requires a minimum of 10 bytes (1 byte MAC header + 2 bytes RLC header + 5 bytes PDCP header + 2 bytes payload). Additionally an optional PHR MAC element (2 bytes) needs to be considered since the PHR has higher priority than the MAC SDU. Any further user data would require a minimum of 5 additional bytes (2 bytes MAC header + 2 bytes RLC header + 1 byte payload).

## 7.17	Simulation of PDCP MAC-I Failure in UE

PDCP integrity protection test cases 7.3.4.x have the requirement to trigger MAC-I failures in UE for downlink messages; to achieve the MAC-I failure in UE two methods are specified in the subsequent sub clauses.

### 7.17.1	Integrity and ciphering not yet activated

UE has not yet started Integrity protection and it is required to trigger MAC-I failure for the PDCP PDU carrying RRC SecurityModeCommand starting integrity with one of integrity protection algorithms. Further a conformant UE will respond with SecurityModeFailure without any integrity protection.

This is achieved by:

Not configuring SS PDCP to start integrity and ciphering with selected algorithm.

RRC SecurityModeCommand is sent indicating Integrity protection through the desired algorithm.

Normal behaviour of PDCP layer in SS will include all zeros in MAC-I.

This results in MAC-I failure as UE will calculate the XMAC-I with indicated algorithm.

### 7.17.2	Integrity and/or ciphering already activated

UE has started Integrity protection (ciphering configured with possibly non null algorithm) and it is required to trigger MAC-I failure for the PDCP PDU carrying an RRC UECapabilityEnquiry message. A conformant UE will trigger an RRCConnectionReestablishment procedure.

This is achieved by:

Configuring SS PDCP to use a different Integrity algorithm other than used by UE (i.e. if UE is configured to use AES, SS is configured to use SNOW3G and vice versa).

Ciphering is configured at SS side same as in UE side.

The MAC-I included by SS PDCP will be as per new algorithm.

UE will calculate XMAC-I based on its own algorithm which is different from the algorithm SS has used and will result in MAC-I failure.

## 7.18	RRC Connection Release Sequence

According to TS 36.331 [19], clause 5.3.8.3, after reception of the RRCConnectionRelease the UE may either wait 60 ms or for indication of acknowledgement from lower layer. After the RRC connection release there are cases where the UE may immediately come up with an RRC connection request. This requires scheduled release of resources at the SS:

1.	At T:	Send RRCConnectionRelease, stop UL grants.

2.	At T + 5ms:	Release security.

3.	At T + 10ms:	Release DRX configuration at the SS.

3A	At T + 15ms:	Release measurement gap configuration at the SS.

4.	At T + 50ms:	no action.

5.	At T + 55ms:		Release SRBs and DRBs.

6.	At T + 60ms:	(Re-)configure SRBs.

7.	Delay of 840ms (NOTE)

T is set to 300ms in advance of RRC connection release.

NOTE: The delay ensures that the UE is camping on the serving cell again to avoid side effects e.g. due to subsequent power level changes. It does not affect any sending of messages by the UE. The delay 840ms is chosen to ensure the UE is re-camping on the cell and has read relevant system information, MIB, SIB1, SIB2 and all other SIs.

## 7.18a	RRC Connection Release Sequence for BL/CE UE

According to TS 36.331 [19], clause 5.3.8.3, after reception of the RRCConnectionRelease the UE may either wait 1250 ms for a BL/CE UE or for indication of acknowledgement from lower layer. After the RRC connection release there are cases where the UE may immediately come up with an RRC connection request. This requires scheduled release of resources at the SS:

1.	At T:	Send RRCConnectionRelease,.

2.	At T + 5ms:	Release security.

3.	At T + 10ms:	Release DRX configuration at the SS.

3A	At T + 15ms:	Release measurement gap configuration at the SS.

4.	At T + 200ms:	stop UL grants (NOTE 1).

5.	At T + 205ms:		Release SRBs and DRBs.

6.	At T + 210ms:	(Re-)configure SRBs.

7. 	Delay of 1880ms(1040 +840) (NOTE 2)

T is set to 300ms in advance of RRC connection release.

NOTE 1: The delay of 200ms added for release of UL grant is to allow UE to send an UL RLC Ack for RRCConnectionRelease. It is selected to be small enough so as to not impact successive cell selection and long enough that both CE mode A and B type UE’s can be catered.

NOTE 2: The delay ensures that the UE is camping on the serving cell again to avoid side effects e.g. due to subsequent power level changes. It does not affect any sending of messages by the UE. The delay 840ms is chosen to ensure the UE is re-camping on the cell and has read relevant system information, MIB, SIB1, SIB2 and all other SIs. The 1880 ms is 1.25s - 210ms.

## 7.19	DL CCCH Message and Contention Resolution MAC Control Element transmission in one MAC PDU or in separate MAC PDUs

When the contention based RACH procedure is being executed (RRC Connection Establishment or RRC Connection Reconfiguration), in general the contention resolution MAC control element and the DL RRC PDU (RRC Connection Setup/RRC Connection Reject/RRC Connection Re-establishment/RRC Connection Re-establishment Reject) are sent in one MAC PDU. This is achieved by pre-configuring the SS (before the start of the RRC procedure) to send the encoded DL message and contention resolution MCE in one MAC PDU.

Nevertheless, due to specific test purposes there are still many cases where it is necessary to send the DL CCCH message separately, this is implemented in TTCN using the test case attribute EarlyContentionResolution:

-	RRC connection establishment:

-	When RRC connection establishment is part of the test purpose

-	Special cases: , e.g. when no contention resolution shall be sent according to the test purpose

-	RRC Connection Reestablishment is part of the test purpose

-	RRC Connection Reject is part of the test purpose

-	RRC Connection Reestablishment Reject is part of the test purpose

The way contention resolution is applied has impact on the DCI format being used in a test case: when the DL CCCH message is sent separately DCI combination 1 according to clause 7.3.1 shall be used.

## 7.20	RRC Connection Reconfiguration Sequence (Measurement Control)

When an RRC Connection Reconfiguration message contains information to configure measurement gaps at the UE according to TS 36.331 [19] clause 5.5.2.9, the SS needs to be configured accordingly:

IF MeasConfig contains measGapConfig:

1.	At T:	Send RRC Connection Reconfiguration.

2.	At T + 5ms:	Configure measurement gaps at the SS.

3.	(after step 2)	Receive RRC Connection Reconfiguration Complete

ELSE

1.	Send RRC Connection Reconfiguration (without scheduling)

2.	Receive RRC Connection Reconfiguration Complete.

T in general is set to 100ms in advance of the RRC connection reconfiguration.

## 7.21	Inter-RAT - GERAN special issues

### 7.21.1	Timeslot assigned for GERAN CS traffic

Timeslot 3 shall be used as the timeslot assigned for GERAN CS traffic, in order to avoid conflicts with timeslots reserved for other purposes (e.g. the GPRS channel which is assigned to timeslot 4).

### 7.21.2	Subchannel used in GERAN L2 access message

The subchannel is valid only for the following logical channel types: FACCH/H, SDCCH/8, SDCCH/4. For other logical channel types this field is not applicable and shall be coded as 15 for compatibility with TTCN2 test cases. The SS shall ignore it if this field is coded as 15.

### 7.21.3	Paging in GERAN

The system paging parameter BS_PA_MFRMS is set to a value of 2 multi frames, in order to ensure the next paging occasion is as soon as possible.

## 7.22	E-UTRAN RSRQ Calculations

### 7.22.1	Assumptions

-	As per 36.214 [53] clause 5.1.1 and 5.12, the RSRP and RSSI shall be averaged over the same set of resource blocks. It is assumed that the power calculations made over one symbol are good enough for RSRQ calculations. This is based on the assumption that the power levels remain the same across the symbols on which the UE is calculating the average. The contribution of Nprb and the OFDM symbols carrying cell specific reference symbols per PRB contribute equally in numerator and denominator, hence RSRQ calculations in dB with aggregation over cell DL bandwidth and without aggregation result in the same output.

-	As per table 7.4.3.1.1-1, for FDD the timing offset between the intra frequency cells is always a multiple of subframe duration i.e. 3072 Ts, hence in the symbols carrying cell-specific reference signals in one cell, the other cell (interference) is also carrying a cell-specific reference signal, even though in a different subframe. Hence from an interference calculation perspective, we can safely assume that the cells contributing an interference shall also transmit the same cell specific reference signal in the symbol.

-	The noise source is treated as a dummy cell transmitting on all resource elements with equal EPRE. Hence to switch off the noise source, a value of non-suitable "Off" cell, as per 36.508 [3] table 6.2.2.1-1 shall be used (<-145), and the signal level uncertainties similar to configured cells in 36.508 [3] clause 6.2.2.1 will also be applied to the noise source.

-	When the Noc value remains 'off' during the test case execution, the Virtual Noise Generator is not configured in the SS.

### 7.22.2	The Ideal Calculation

The test case specifies the RS-EPRE (dBm/15kHz) which is the cell absolute power-cell attenuation in dBm per carrier of 15 kHz; the linear average over few measurements will give the RSRP value.

It is assumed that the power levels of all contributors is fluctuating hence in the typical test environment RS-EPRE will be the RSRP over a single carrier frequency.

The RS-EPREmW(mW/15kHz) = 10 power (RSRP/10).

The energy in noise source [Noc] for the frequency is also specified in the same units of dBm/15kHz.

The RSSImW calculation(mW over a resource block frequency of 180 kHz) = 2 (the sum over all intra frequency configured cells RS-EPREmW) + 10 power ((Noc/10)*12.

The cell inference is multiplied by 2 as in a RB, cell specific reference is transmitted in only 2 carriers. But the AWGN transmits on all 12 carriers in the resource block.

The RSRQ in dB will be the 10*log (RS-EPREmW/RSSImW).

### 7.22.3	Additional RSRQ Calculations For Fixing Boundary Values

In addition to the ideal calculation, various RSRQ calculations take into consideration the combinations of the SS signal uncertainties and possibly all 12 carriers being used for DL transmission.

The 12 carriers being used can happen when the UE makes the measurement in a subframe when a DL PDSCH is scheduled or measurement is in subframe zero and the UE measures in OFDM symbol carrying PBCH i.e. slot 1, symbol 0.

The Min and Max RSRQ values are identified from 6 different RSRQ calculations:

RSRQ Ideal: RSRQ calculated in ideal conditions as given above

RSRQ Min: RSRQ calculation applying –ve signal uncertainty to all configured cells and noise source

RSRQ Max: RSRQ calculation applying +ve signal uncertainty to all configured cells and noise source

RSRQ Max Worst Case: RSRQ calculation applying +ve signal uncertainty for measured cell and –ve signal uncertainty to all remaining configured cells and noise source. This shall be the Max RSRQ possible

RSRQ Max And 12 Carriers: RSRQ calculation applying +ve signal uncertainty to all configured cells and noise source and cell interference considered over all 12 carriers

RSRQ Min Worst Case And 12 Carriers: RSRQ calculation applying -ve signal uncertainty for measured cell and +ve signal uncertainty to all others; and cell interference considered over all 12 carriers. This shall be the Min RSRQ possible

Applying UE measurement accuracy 36.133 [37], Table 9.1.6.2-1, +/-4dB normal conditions and side conditions of RSRQ when RSRP Ês/Iot ≥ -6 dB & RSRP ≥ -124 dBm the final boundary value for RSRQ is

Min RSRQ With UE Meas Acc := Min RSRQ -4 dB

Max RSRQ With UE Meas Acc := Max RSRQ + 4 dB

For conditions to fulfil desired cell selection or reselection, the respective conditions shall be satisfied for both the boundary values Min RSRQ With UE Measurement Accuracy and Max RSRQ With UE Measurement Accuracy.

As the Boundary conditions also consider the worst scenario of all 12 carriers being transmitted, the requirement for OCNG is removed. There is no need to restrict measurement bandwidth and will be applicable for both DCI formats.

## 7.23	Test method for eICIC and feICIC

When periodic CQI feedback is requested due to TTCN configuration, the SS reports the periodic CQI to the TTCN. SS does not react on periodic CQI received and still allocates grants as configured from TTCN.

In the selected ABS no DL/UL user data (SRB/DRB) is scheduled, nor paging is transmitted; this is controlled by TTCN.

For feICIC testing, the SS may be configured to temporarily stop the transmission of SIB1 and its associated DCI.

## 7.24	Carrier Aggregation Signalling Sequences

### 7.24.1	Initial configuration of Pcell

Cell is configured as a normal cell by using function f_EUTRA_CellConfig_Def. The missing CA default parameters (e.g. UL power control Common) are configured by using additional ASP call(s).

Figure 7.24.1-1: CA sequence of an initial cell configuration for a cell to be used as Pcell

### 7.24.2	Initial configuration of SCell

Cell is configured as a normal cell except for SRB/DRB configuration by function f_EUTRA_SS_ConfigureActiveCell.

SRB0 not yet configured in step 1 is then configured.

The missing CA default parameters (e.g. UL power control (Common + Dedicated), PUSCH config Dedicated, SRS UL Dedicated) are configured by using additional ASP call(s).

Figure 7.24.2-1: CA sequence of an initial cell configuration for a cell to be used as Scell

### 7.24.3	Scell Addition and/or release

Figure 7.24.3-1: CA sequence of Scell Addition and/or release

## 7.25	Test method for MBMS

### 7.25.1	Schedule transmission of MCCH messages

The rules for the transmission of MCCH messages are specified in TS 36.331 [19], clause 5.8.1.2. The ASPs SYSTEM_CTRL_REQ and SYSTEM_CTRL_CNF are used as interface to SS; the following rules apply:

The complete MCCH information is provided to SS by using a single ASP. The MCCH carries the MBSFNAreaConfiguration and may also carry the MBMSCountingRequest message.

The scheduling information sent to SS is the same as the scheduling information sent to the UE.

The MCCH information is sent to SS using the asn.1 types, SS shall encode in unaligned PER and add the necessary padding bits as specified in TS 36.331 [19], clause 9.1.1.1.

Segmentation occurs when the MCCH message size is larger than the TBS of the MCH. SS starts scheduling all MCCH information blocks from the same SFN.

NOTE: With the default values NPRB=NRBDL=25 for 5 MHz and default signalling Imcs =2; then Itbs=2 and the TB size will be 1096 bits, hence segmentation may never happen.

### 7.25.2	MCCH change notification

A change notification is used to indicate the modification of MCCH information. The notification is transmitted on PDCCH, it includes the notification indicator provided in the ASP and is transmitted using the M-RNTI (defined in 3GPP TS 36.321 [16] Table 7.1-1).

The notification messages are sent during one modification period before transmitting the modified MCCH information. The SFN for the start of modification period is calculated by TTCN. The modified MCCH information and the calculated SFN are provided in the ASP SYSTEM_CTRL_REQ. The notification messages are sent on SFN/subframes calculated by TTCN according to 36.331 [19] clause 6.3.7.

### 7.25.3	MTCH data scheduling

The SS is configured with configuration parameters for CSA, PMCH, MRBs, MSI and scheduling information for MTCH data transmission.

The TTCN shall ensure that:

-	For each configured MCH, the routing & timing information of the scheduled MRB data is consistent with the configured MSI.

-	The size of all MRB data scheduled in the same subframe shall fit within the resource allocation of that subframe.

The SS shall ensure that:

-	In subframes belonging to the CSA and for which no MCCH/MTCH/MSI data is scheduled by the TTCN, no MCH data shall be sent in MBMS subframes not used as per MSI.

If an MTCH packet is scheduled from TTCN in a subframe in which MCCH will be automatically transmitted by SS and/or MSI is configured and needs to be included, the MTCH packet and MCCH data along with possible MSI are included in one MAC PDU and signallingMCS is used.

When data is sent on MTCH the SS shall set the MSI in the MAC header according to TS 36.321 [16] clause 6.1.3.7. The periodicity of the MSI is defined by the MCH scheduling period i.e. the MSI is not necessarily included in the subframe where the RLC SDU is transmitted.

## 7.26	Type B FDD Half-Duplex Considerations

Type B FDD half-duplex UE has limitations in receiving a DL subframe preceding or following a UL subframe for the same UE hence a blank subframe is needed. In [non TTI bundling considerations] max 3 contiguous subframes UL or DL can be scheduled. And after and before any contiguous DL/UL subframes block a blank subframe is needed.

If the UE under test is a Type B FDD half-duplex UE:

-	SS will be configured per cell by providing UE category info and FDD half-duplex operation.

-	When explicit timing info is provided in any DL transmission, the SS shall ignore the UE capability and shall follow the explicit scheduling. It will be TTCN responsibility to schedule UL/DL transmissions based on UE capability. Paging is always explicitly scheduled and hence SS follows explicit scheduling for paging.

-	For UE Category 0, SI scheduling will be configured as per TDD SI scheduling (Table 7.7.2-4) restricting system info transmissions in sub frames 0 and 5. For non-explicitly scheduled new DL transmissions/ new UL grant allocations, the SS shall restrict UL/DL operations as per table below:

Table 7.26-1: Scheduling pattern for type B half-duplex FDD Cat.0 UE (non-explicitly scheduled)

| SFN\ SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X (any) | MIB/SI | Blank | New UL Data/ HARQ | Blank | Blank | SIB1/SI | DL HARQ | Blank | New DL Data/ UL Grant | Blank |

- For BL/CE UE, SI scheduling is specified in clause 7.7.2a. BL/CE UE acquires sysinfo in RRC_IDLE. In RRC_CONNECTED BL/CE UE acquires MIB/SIB1-BR only when T311 is running. For non-explicitly scheduled new DL transmissions/ new UL grant allocations:

-	No subframe restriction using fdd-DownlinkOrTddSubframeBitmapBR nor fdd-UplinkSubframeBitmapBR are provided by TTCN to SS,

-	The SS shall restrict UL/DL operations as per table below:

Table 7.26-2: Scheduling pattern for type B half-duplex FDD BL/CE UE (non-explicitly scheduled)

| SFN\ SubFrame | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| X (any) | MIB/SI-BR/New DL Data | Blank | New UL Data/ HARQ | Blank | SIB1-BR | Blank | DL HARQ | Blank | DL Grant/ UL Grant | Blank |

NOTE 1:	As per TS 36.213[30] clause 7.1.11, the DL data will be 2 subframes after the subframe containing DL Grant on MPDCCH. As per TS 36.213[30] clause 8.0, the UL data will be 4 subframes after the subframe containing UL Grant on MPDCCH.

NOTE 2:	For new DL data transmission, SS transmits DL DCI in subframe 8 and PDSCH data in subframe 0 of the next subframe.

-	If HARQ retransmissions are required:

-	DL retransmissions being asynchronous in nature shall be scheduled by the SS as per above schedule (i.e. subframe 8).

-	UL retransmissions being synchronous will not be limited to subframe 2. i.e. the first UL retransmission will be in frame X+1, subframe 0, second retransmission in frame X+1, subframe 8 and so on.

-	HARQ retransmissions shall be prioritised over new transmissions.

## 7.27	Test method for Device-to-Device Proximity Services

### 7.27.1	Direct Discovery test method

SS-UE is configured in coverage, i.e. it is associated with an E-UTRA-cell (as specified in clause 7.27.2.1).

SS-UE can be configured for sidelink data transmission and/or reception:

-	When UE under test is configured in sidelink discovery type 1 (i.e. ue-Selected), the SS-UE is configured in reception with the same pool(s) of resource.

-	When UE under test is configured in sidelink discovery type 2B (i.e. scheduled), the SS-UE is configured in reception with the same pool of resource and same indexes.

-	For transmission, TTCN configures the SS-UE with a pool of resources and provides the SL_DATA_REQ ASP including:

-	the SL-DCH message to be transmitted,

-	the timing info indicating the SFN/Subframe of the first discovery period,

-	the number of discovery periods in which the message shall be transmitted,

-	the selected PSDCH resource value (n_PSDCH), this value is incremented by TTCN after each transmission cycle. To avoid clashes when multiple PC5_DISCOVERY messages are transmitted within the same discovery period, TTCN will provide different n_PSDCH values.

### 7.27.2	Direct Communication test method

#### 7.27.2.1	Synchronisation and SBCCH transmission

When configured in coverage, the SS-UE is associated with an E-UTRA-cell, it is configured with the same configuration parameters i.e. Band, Bandwidth, UL frequency, Cell timing, Frame structure of the associated E-UTRA cell.

When out of coverage, SS-UE is configured explicitly by TTCN, e.g. Band, Bandwidth, Frame structure. For timing synchronisation SS-UE can be configured either:

-	To synchronise from UE under test transmitted SLSS/MIB-SL: UE under test is the synchronisation source. The SS-UE shall use the received SLSS and MIB-SL including the DirectFrameNumber/DirectSubframeNumber as synchronisation source as specified in TS 36.331 [19], clause 5.10.9.2

-	To be the synchronisation source: SS-UE shall initiate autonomously the DirectFrameNumber/DirectSubframeNumber and transmit SLSS/MIB-SL.

When the SS-UE is configured by TTCN over sidelink system control port to receive/transmit SLSS and MIB-SL, the following rules apply:

-	For transmission, the complete MIB-SL message is provided to SS-UE. SS-UE shall set the DirectFrameNumber and DirectSubframeNumber in the MIB-SL, a dummy value is provided by TTCN.

-	The MIB-SL is sent to SS-UE using asn.1 types. The rules for encoding and transmission of SBCCH messages (MIB-SL) are specified in TS 36.331 [19].

-	For reception, SS-UE shall report SLSS or MIB-SL over the sidelink system indication port.

#### 7.27.2.2	Sidelink data transmission/reception

When UE is in coverage and configured in transmission mode 1 (i.e. scheduled), the sidelink Scheduling Grant is configured in SS by TTCN over system control port. On PDCCH, the SS informs the UE under test if it is allowed to make sidelink data transmission by transmitting 'DCI format 5' including ‘SCI format 0’ with the CRC scrambled by SL-RNTI (acc. to TS 36.213 [30] clause 14.2). The UE under test will transmit on PSCCH/PSSCH as specified in the received DCI format 5 and ‘SCI format 0’. SS shall only react on reception of SR and grant allocation configured from the TTCN. The following sidelink grant allocation is configured:

-	SS is configured to maintain PUCCH Synch.

-	SS is configured to send automatically a 'configured Grant' (in terms of NPSCCH, ITRP and NPRB) to the UE on every reception of a Scheduling Request, within 10 subframes.

-	The default resource for PSCCH is NPSCCH = 11,

-	The default configured grant for PSSCH in SCI format 0 is ITRP = 36 and NPRB = 25. The SS shall allocate resource allocation RBs corresponding to PRB indices 0..(NPRB-1).

-	SS-UE is configured with the same pool(s) of resources as UE under test.

When UE is out of coverage or configured in transmission mode 2 (i.e. ue-Selected), UE selects autonomously the resources on PSCCH/PSSCH. SS-UE is configured with the same pool(s) of resources as UE under test.

The SS-UE, when triggered to transmit data, is configured with IMCS=0 and the same NPSCCH , ITRP and NPRB as specified above.

SS-UE is configured with a ProSe layer 2 identity and with the UE ProSe layer 2 identity. When configured in transmission, the SS-UE shall construct the MAC packets using these identities. In reception, SS-UE shall send to TTCN only the PDCP data packets received with the correct layer 2 identities in PSCCH and in MAC headers.

## 7.28	Test method for SC-PTM

### 7.28.1	Schedule transmission of SC-MCCH messages

The rules for the transmission of SC-MCCH messages are specified in TS 36.331 [19], clause 5.8a.1.2. The ASPs SYSTEM_CTRL_REQ and SYSTEM_CTRL_CNF are used as interface to SS; the following rules apply:

-	The complete SC-MCCH information is provided to SS by using a single ASP. The SC-MCCH carries the SPCTMConfiguration message.

-	The scheduling information sent to SS is the same as the scheduling information sent to the UE.

-	The SC-MCCH information is sent to SS using the asn.1 types, SS shall encode in unaligned PER and add the necessary padding bits as specified in TS 36.331 [19], clause 9.1.1.7.

-	Segmentation occurs when the SC-MCCH message size is larger than the TBS of the DL-SCH. SS starts scheduling all SC-MCCH information blocks from the same SFN.

NOTE:	With the default values NPRB=NRBDL=25 for 5 MHz and default signalling Imcs =2; then Itbs=2 and the TB size will be 1096 bits, hence segmentation may never happen.

### 7.28.2	SC-MCCH information change

There are two possibilities to update SC-MCCH information. One is to transmit modified SC-MCCH information at the beginning of the modification period. Alternatively, notification mechanism can be used.  The notification is transmitted using the DCI format 1C with SC-N-RNTI (defined in 3GPP TS 36.321 [16] Table 7.1-1) and one bit within the 8-bit bitmap. The notification is sent in the first subframe, which can be used for SC-MCCH transmission in a repetition period. Modified SC-MCCH message is to be transmitted starting from the same subframe, as defined in TS 36.300 clause 15.3.5a.

The SFN for the start of modification to SC-MCCH is calculated by TTCN according to TS 36.331 [19] clause 6.3.1 as defined for SystemInformationBlockType20. The modified SC-MCCH information, the calculated SFN and a flag, pointing on if a notification mechanism has to be used, are provided in the ASP SYSTEM_CTRL_REQ.

### 7.28.3	SC-MTCH data scheduling

The SS may be configured with scheduling information for SC-MTCH data transmission. The TTCN shall ensure that SC-MTCH packets will be scheduled for the transmission only during the Active Time for a PDCCH subframe as defined in TS 36.321 [16] clause 5.7a.

## 7.29	Test method for V2X Services

### 7.29.1	Synchronisation and SBCCH transmission

In coverage and out of coverage configurations defined in clause 7.27.2.1 apply with the exception that MIB-SL is replaced by MIB-SL-V2X. In addition, the SS-UE can be configured in GNSS/UTC time synchronised mode, in which case GNSS data and the SS-UE shall be UTC time synchronised. SS-UE shall calculate the DFN and Subframe in accordance with TS 36.331[19] clause 5.10.14.

### 7.29.2	Sidelink data transmission/reception

UE and SS-UE are configured with the same pool(s) of resources.

When configured in transmission, the SS-UE is configured with a layer 2 identity, corresponding to the source layer 2 identity and with the UE layer 2 identity corresponding to the service destination layer 2 identity; the SS-UE shall construct the MAC packets using these identities. The SS-UEs layer 2 identity is arbitrarily set to ‘0000BB’H for SS-E1 and ‘0B00EC’H for SS-UE2.

When configured in reception, the SS-UE is configured with a layer 2 identity, corresponding to the service destination layer 2 identity; SS-UE shall send to TTCN only the PDCP data packets received with the correct destination layer 2 identities in the MAC header. The source layer 2 identity, randomly and uniquely self-selected by UE, can be any value. SS-UE can be configured to enable/disable SCI format 1 in SL-DATA-IND.

UE can be configured in Transmit mode 3 or 4:

-	When UE is configured in transmission mode 3 (i.e. scheduled), the sidelink Scheduling Grant is configured in SS by TTCN over system control port. On PDCCH, the SS informs the UE under test if it is allowed to make sidelink data transmission by transmitting 'DCI format 5A' including ‘SCI format 1’ parameters with the CRC scrambled by SL-V-RNTI. The UE under test will transmit on PSCCH/PSSCH as specified in the received DCI format 5A and ‘SCI format 1’. Sidelink SPS grant can be activated/deactivated in SS by TTCN with SL-SPS-V-RNTI scrambling. SS shall only react on reception of SR and grant allocation configured from the TTCN. The following sidelink grant allocation is configured:

-	SS is configured to maintain PUCCH Synch.

-	SS is configured to send automatically a 'configured Grant' to the UE on every reception of a Scheduling Request, exactly 10 subframes after reception of SL-BSR:

-	the Carrier Indication and SL-Index are set as per test case prose

-	the default Lowest index of the subchannel allocation to the initial transmission is set to 0

-	the default configured grant for PSSCH in SCI format 1 is:

-	Frequency resource location of initial transmission and retransmission is set to 30 for Bandwidth 10 and 10 for Bandwidth 20.

-	Time gap = 0, no retransmission is applied

-	SPS configuration index and Activation/Release are provided when SL-SPS is configured

-	When UE is configured in transmission mode 4 (i.e. ue-Selected), UE selects autonomously the resources on PSCCH/PSSCH.

-	When specified in the test case prose, when UE retransmits data on PSSCH, SS-UE sends to TTCN all PDCP data packets received on PSSCH independently whether this is a first transmission or a retransmission on PSSCH.

The SS-UE, when triggered to transmit data, is provided with timing either ‘timing now’ or ‘SFN/Subframe’ or ‘DFN/DSubframe’. Multiple STCH packets are transmitted every 20ms. Sensing is not applied to SS-UE. SS-UE is configured with a fixed SCI format 1 grant:

Priority = ‘100’B (arbitrary value)

Resource Reservation = 0

Frequency resource location of initial transmission and retransmission  is set to 20 for Bandwidth 10 and 10 for Bandwidth 20

Time gap = 0, no retransmission is applied

MCS index = 8

### 7.29.3	Congestion

SS-UE is configured to transmit, over PSCCH/PSSCH, QPSK modulated uncorrelated pseudo random data on all RBs on a list of specified subchannels and subframes.

The congestion configuration is provided for one frame, it is repeated continuously or for a specific number of consecutive frames. When continuous transmission is configured the SS-UE shall transmit the congestion until a Deactivate command is received.

SS-UE is previously configured in transmission mode so that the necessary timing synchronisation, power level, frequency, bandwidth and subchannels are configured. When configured in transmission mode, only the resource pool is provided with all bits of the transmission subframe bitmap set to ‘1’B.

### 7.29.4	UE transmission/reception over Uu

When UE is receiving V2X data over Uu, SS-UE is not configured. SS can be configured for transmission of data either with:

-	SC-PTM broadcasting with sc-mcch-RepetitionPeriod-v1430 and sc-mcch-ModificationPeriod-v1430

-	MBMS broadcasting with mcch-RepetitionPeriod-v1430, mcch-ModificationPeriod-v1430 and mch-SchedulingPeriod-v1430.

When UE is transmitting V2X data over Uu, SS-UE is not configured. SS can be configured with multiple UL SPS.

## 7.30	Test method for UL Data Compression (UDC)

### 7.30.1	General

UDC testing uses the PDCP test model (Non ROHC) specified in clause 4.2.1.3.2. The PDCP is configured in transparent mode. ROHC is not configured. Data sent in DL is not compressed, data sent in UL may be compressed. This is indicated in the UL Data PDUs.

UDC works on the condition that the compression buffer and decompression buffer are synchronized. The SS shall support a decompressor and a decompression buffer, which are accessed and controlled by TTCN via a set of external functions.

### 7.30.2	Initialization

When the PDCP DRB is configured, a predefined dictionary can be provided to the UE. This dictionary will be used by the UE’s compression function. If no dictionary is provided then the compression buffer will be initialized with all 0s.

At the SS side, the external function fx_UDC_Initialise is used to initialize the decompression buffer consistently with the setting of the UE’s compression buffer.  If the compression buffer is reset at the UE side, then the same needs to be done at the SS side. This is normally achieved by passing the reset flag FR, received in the Data PDU, to the decompressor.

### 7.30.3	Compression and decompression

With each PDCP Data PDU the UE provides information if the contained SDU is compressed, if the compression buffer has been reset, as well as the Checksum calculated during compression.

This information allows the TTCN to request the SS to decompress the data using external function fx_UDC_Decompress. The compressed SDU, as well as the information if the compression was reset or not before compression, are passed. If decompression succeeds then the decompressed SDU, the calculated Checksum, as well as a success indication are returned. The TTCN can then compare the Checksum received in the PDU to the Checksum returned by fx_UDC_Decompress. If decompression does not succeed failure is reported.

## 7.31	Test method for QoE Measurement Collection

Application layer measurement configuration message is wrapped into the measConfigAppLayerContainer that is an octet string with a maximum length of 1000 bytes. It is delivered to UE via RRCConnectionReconfiguration message. UE is expected to transmit a corresponding MeasReportAppLayer message back to the network over the SRB4 that has been configured by the network in advance. It contains measReportAppLayerContainer that is also an octet string which size is bound to be maximum of 8000 bytes.

The test model does not include an Application Layer, it is limited only to RRC signalling. Thus contents of the measConfigAppLayerContainer and measReportAppLayerContainer are dummy bytes. Sending of the Application Layer Measurement report is triggered at the UE using the +CAPPLEVMR AT command.

## 7.32	MTC Wake-Up Signal

The SS shall transmit MWUS signal according to TS 36.211 [35] clause 6.11B. For MWUS generation TTCN provides SS with:

-	the time offset between the end of MWUS and start of the first PO in frames, used to calculate  needed for generation of ,

-	the actual duration of the MWUS signal in subframes,

-	the frequency location of MWUS within paging narrowband,

-	and optionally with the power offset of WUS relative to CRS in dB.

Actual transmission of the MWUS signal is triggered by the system request at the activation time calculated by TTCN.

# 7A	NB-IoT test methods and design considerations

## 7A.0	General

Unless explicitly stated otherwise, the present clause and its subclauses apply to the NB-IoT anchor carrier.

In this version of the specification, NB-IoT operation in TDD is not supported.

## 7A.1	Physical signals and channels

For Signalling testing, single Tx antenna shall be used by the SS, unless specified otherwise in a test case.

The SS shall transmit all NB-IoT physical signals and channels according to TS 36.211 [35], with the following additional requirements, unless specified otherwise in a test case:

-	The NRS shall be transmitted on one antenna port.

-	The NPSS and NSSS shall be transmitted on one and the same antenna port.

-	The NWUS shall be transmitted on one and the same antenna port, which is different from the antenna port where NPSS and NSSS shall be transmitted.

No (WB-E-UTRA) LTE signals and channels shall be transmitted by the SS, except for the case of ‘in-band same PCI’ operation mode where the SS shall transmit the LTE CRS. For the in-band operation modes:

-	TTCN will provide the necessary LTE CRS configuration to the SS ; cyclic prefix (CP) is normal.

-	For the case of ‘in-band different PCI’ operation mode:

-	 The SS shall use this information to skip the resource elements, which should be occupied by the LTE CRS, when transmitting NB-IoT downlink physical channels;

-	The LTE physical cell identity  is calculated from the NB-IoT physical cell identity  as follows:

= ( + 6) MOD 504

For NWUS generation as per TS 36.211 [35] clause 10.2.6B, TTCN provides SS with:

-	time offset between the end of NWUS and start of the first PO in frames, used to calculate  needed for generation of ,

-	with the actual duration of the NWUS signal in subframes,

-	and optionally with the power offset of NWUS relative to NRS in dB.

Actual transmission of the NWUS signal is triggered by the system request at the activation time calculated by TTCN.

## 7A.2	System information

### 7A.2.1	System information broadcasting in general

TTCN provides separately the BCCH (MIB-NB, SIB-NB’s) messages to transmit, and the scheduling of how and when to transmit them. However the complete information is provided to the SS by using a single ASP.

The SS shall transmit all BCCH messages according to TS 36.331 [19], with the following additional requirements:

-	SS shall start scheduling all BCCH messages from the same timing (H-SFN, SFN).

-	SS shall set and maintain the SFN-MSB and the HSFN-LSB in the MIB-NB message provided by TTCN, according to its internal timing. A dummy value is provided by TTCN.

-	SS shall set and maintain the HSFN-MSB in the SIB1-NB message provided by TTCN, according to its internal timing. A dummy value is provided by TTCN.

-	TTCN provides the BCCH messages to the SS using the ASN.1 types. The SS shall encode them in unaligned PER and add the necessary padding bits as specified in TS 36.331 [19], clause 8.5.

### 7A.2.2	System information scheduling and synchronisation signals

For NB-IoT the scheduling of UL and DL data strongly depends on the scheduling of system information and synchronisation signals: The SS can schedule DL assignments and UL grants only in subframes which are not used by system information or synchronisation signals.

This clause shows the allocation of system information and synchronisation signals according to core specifications and for cases of common system information scheduling according to TS 36.508 [3] clause 8.1.4.3.

NOTE:	Unless explicitly stated otherwise, the present clause and its subclauses apply to downlinkBitmap-r13 not being specified in SIB1-NB.

#### 7A.2.2.1	MIB-NB, NPSS, NSSS

NPSS is transmitted in subframe 5 of every radio frame (TS 36.211 [35] clause 10.2.7.1.2).

NSSS is transmitted in subframe 9 of every radio frame with even frame number (TS 36.211 [35] clause 10.2.7.2.2).

MIB-NB is sent on NPBCH: It is split into 8 blocks, each of which is repeated 8 times in the first subframe of a frame (TS 36.331 [19] clause 5.2.1.2a)  MIB uses the first subframe of every frame, periodicity is 640ms.

Table 7A.2.2.1-1: MIB-NB, NPSS, NSSS scheduling

| ... | SFN mod 2 = 0 |  |  |  |  |  |  |  |  |  | SFN mod 2 = 1 |  |  |  |  |  |  |  |  |  | ... |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | ... |
| ... | N P B C H |  |  |  |  | N P S S |  |  |  | N S S S | N P B C H |  |  |  |  | N P S S |  |  |  |  | ... |

#### 7A.2.2.2	SIB1-NB

SIB1-NB scheduling depends on:

-	schedulingInfoSIB1-r13 contained in MasterInformationBlock-NB:
TBS of 208, 328, 440 or 680 and 4, 8 or 16 repetitions (TS 36.213 [30] Tables 16.4.1.3-3 & 16.4.1.5.2-1)

-	Physical Cell ID (PCID) derived by the UE from the NSSS:
Starting radio frame number for SIB1-NB repetitions depends on the PCID and the number of repetitions (TS 36.213 [30] Table 16.4.1.3-4)

According to TS 36.331 [19] clause 5.2.1.2a, independent from the TBS, a single SIB1-NB transmission takes every second of 16 sub-sequent frames and the parts of the SIB1-NB are sent at subframe #4.

Table 7A.2.2.2-1 shows SIB1-NB scheduling for the 16 repetitions for the case that the radio frame of the first transmission is 0 (mod 4 = 0 in Table 16.4.1.3-4 of TS 36.213 [30]); frames with SIB1-NB are shown in green.

Table 7A.2.2.2-1: SIB1-NB scheduling with 16 repetitions

| frame 0 | frame 1 | frame 2 | frame 3 | frame 4 | frame 5 | frame 6 | frame 7 | frame 8 | frame 9 | frame 10 | frame 11 | frame 12 | frame 13 | frame 14 | frame 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| frame 0 | frame 1 | frame 2 | frame 3 | frame 4 | frame 5 | frame 6 | frame 7 | frame 8 | frame 9 | frame 10 | frame 11 | frame 12 | frame 13 | frame 14 | frame 15 |
| frame 16 | frame 17 | frame 18 | frame 19 | frame 20 | frame 21 | frame 22 | frame 23 | frame 24 | frame 25 | frame 26 | frame 27 | frame 28 | frame 29 | frame 30 | frame 31 |
| frame 32 | frame 33 | frame 34 | frame 35 | frame 36 | frame 37 | frame 38 | frame 39 | frame 40 | frame 41 | frame 42 | frame 43 | frame 44 | frame 45 | frame 46 | frame 47 |
| frame 48 | frame 49 | frame 50 | frame 51 | frame 52 | frame 53 | frame 54 | frame 55 | frame 56 | frame 57 | frame 58 | frame 59 | frame 60 | frame 61 | frame 62 | frame 63 |
| frame 64 | frame 65 | frame 66 | frame 67 | frame 68 | frame 69 | frame 70 | frame 71 | frame 72 | frame 73 | frame 74 | frame 75 | frame 76 | frame 77 | frame 78 | frame 79 |
| frame 80 | frame 81 | frame 82 | frame 83 | frame 84 | frame 85 | frame 86 | frame 87 | frame 88 | frame 89 | frame 90 | frame 91 | frame 92 | frame 93 | frame 94 | frame 95 |
| frame 96 | frame 97 | frame 98 | frame 99 | frame 100 | frame 101 | frame 102 | frame 103 | frame 104 | frame 105 | frame 106 | frame 107 | frame 108 | frame 109 | frame 110 | frame 111 |
| frame 112 | frame 113 | frame 114 | frame 115 | frame 116 | frame 117 | frame 118 | frame 119 | frame 120 | frame 121 | frame 122 | frame 123 | frame 124 | frame 125 | frame 126 | frame 127 |
| frame 128 | frame 129 | frame 130 | frame 131 | frame 132 | frame 133 | frame 134 | frame 135 | frame 136 | frame 137 | frame 138 | frame 139 | frame 140 | frame 141 | frame 142 | frame 143 |
| frame 144 | frame 145 | frame 146 | frame 147 | frame 148 | frame 149 | frame 150 | frame 151 | frame 152 | frame 153 | frame 154 | frame 155 | frame 156 | frame 157 | frame 158 | frame 159 |
| frame 160 | frame 161 | frame 162 | frame 163 | frame 164 | frame 165 | frame 166 | frame 167 | frame 168 | frame 169 | frame 170 | frame 171 | frame 172 | frame 173 | frame 174 | frame 175 |
| frame 176 | frame 177 | frame 178 | frame 179 | frame 180 | frame 181 | frame 182 | frame 183 | frame 184 | frame 185 | frame 186 | frame 187 | frame 188 | frame 189 | frame 190 | frame 191 |
| frame 192 | frame 193 | frame 194 | frame 195 | frame 196 | frame 197 | frame 198 | frame 199 | frame 200 | frame 201 | frame 202 | frame 203 | frame 204 | frame 205 | frame 206 | frame 207 |
| frame 208 | frame 209 | frame 210 | frame 211 | frame 212 | frame 213 | frame 214 | frame 215 | frame 216 | frame 217 | frame 218 | frame 219 | frame 220 | frame 221 | frame 222 | frame 223 |
| frame 224 | frame 225 | frame 226 | frame 227 | frame 228 | frame 229 | frame 230 | frame 231 | frame 232 | frame 233 | frame 234 | frame 235 | frame 236 | frame 237 | frame 238 | frame 239 |
| frame 240 | frame 241 | frame 242 | frame 243 | frame 244 | frame 245 | frame 246 | frame 247 | frame 248 | frame 249 | frame 250 | frame 251 | frame 252 | frame 253 | frame 254 | frame 255 |

#### 7A.2.2.3	SI-messages containing SIB3-NB, SIB4-NB, SIB5-NB, SIB14-NB, SIB16-NB

Scheduling of SI messages is specified in TS 36.331 [19] clauses 5.2.1.2a and 5.2.3a.

As SI messages are sent on NPDSCH, only "NB-IoT DL subframes" can be used according to TS 36.213 [30] clause 16.4: In case of SIB1-NB not specifying downlinkBitmap-r13 NB-IoT DL subframes are subframes not containing NPSS/NSSS/NPBCH/SIB1-NB transmissions.

The parameters for SI scheduling are contained in SIB1-NB:

Configuration common for all SI messages:

-	Window size,

-	Frame offset for scheduling.

Configuration per SI message:

-	SI window:
According to TS 36.331 [19] clause 5.2.3a the position of an SI-window depends on the order of the corresponding SI-message in the list, the window size and the periodicity.

-	Repetitions of the SI-message:
Within an SI-window repetitions are specified by the repetition-pattern.

-	SIBs mapped to the SI-message.

-	Transport block size (TBS) and number of subframes (NSF) used for transmission: 
According to clause 6.7.2 of TS 36.331 [19] NSF is either 2 (for TBS = 56, 120) or NSF is 8 (for TBS = 208, 256, 328, 440, 552, 680).

##### 7A.2.2.3.1	SI combination 1 scheduling

TS 36.508 [3] clause 8.1.4.3 specifies configuration of SI combination 1 scheduling as shown in table 7A.2.1.3.1-1 with

-	16 repetitions of SIB1-NB  SIB1-NB in every second frame at subframe 4

-	si-WindowLength-r13: 			ms160

-	si-RadioFrameOffset-r13: 		0 (not present in SIB1-NB)

-	1 SI-message:
-	SI-Window 1
	si-Periodicity-r13				rf64
	si-RepetitionPattern-r13		every8thRF
	si-TB-r13						552  NSF = 8

-	According to TS 36.331 [19] clause 5.2.3a the SI-windows start at subframe 0 of frame SI1:
			(H-SFN * 1024 + SFN) mod TSI1 = 0; 		with TSI1 = 64

Table 7A.2.2.3-1: SI combination 1 scheduling (SIB2-NB only)

| SI Window | K | SFN = K |  |  |  |  |  |  |  |  |  | SFN = K + 1 |  |  |  |  |  |  |  |  |  | SFN = K + 2 |  |  |  |  |  |  |  |  |  | SFN = K + 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
|  | ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-1 | 64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

With

|  | NPBCH |  | NPSS |  | NSSS |  | SIB1-NB |  | SI-1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 7A.2.2.3.2	SI combination 2/3/4 scheduling

TS 36.508 [3] clause 8.1.4.3 specifies configuration of SI combination 2/3/4 scheduling as shown in table 7A.2.1.3.1-2 with

-	16 repetitions of SIB1-NB  SIB1-NB in every second frame at subframe 4

-	si-WindowLength-r13: 			ms160

-	si-RadioFrameOffset-r13: 		0 (not present in SIB1-NB)

-	3 SI-messages:
-	SI-Window 1
	si-Periodicity-r13				rf64
	si-RepetitionPattern-r13		every8thRF
	si-TB-r13						552  NSF = 8
-	SI-Window 2
	si-Periodicity-r13				rf64
	si-RepetitionPattern-r13		every16thRF
	si-TB-r13						256  NSF = 8
-	SI-Window 3
	si-Periodicity-r13				rf64
	si-RepetitionPattern-r13		every16thRF
	si-TB-r13						256  NSF = 8

-	According to TS 36.331 [19] clause 5.2.3a the SI-windows start at subframe 0 of frame
			(H-SFN * 1024 + SFN) mod TSI1 = 0; 			with TSI1 = 64
			(H-SFN * 1024 + SFN) mod TSI2 = 16; 		with TSI2 = 64
			(H-SFN * 1024 + SFN) mod TSI3 = 32; 		with TSI3 = 64

Table 7A.2.2.3-2: SI combination 2/3/4 scheduling (SIB2-NB + SIB3-NB + SIB4/5/14-NB)

| SI Window | K | SFN = K |  |  |  |  |  |  |  |  |  | SFN = K + 1 |  |  |  |  |  |  |  |  |  | SFN = K + 2 |  |  |  |  |  |  |  |  |  | SFN = K + 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
|  | ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-1 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-2 | 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-3 | 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-1 | 64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-2 | 80 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-3 | 96 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 104 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 112 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 116 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 124 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SI-1 | 128 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 132 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 136 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 140 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

With

|  | NPBCH |  | NPSS |  | NSSS |  | SIB1-NB |  | SI-1 |  | SI-2 |  | SI-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

### 7A.2.3	System information modification

According to TS 36.331 [19] for normal DRX the modification period for change of system information is

ModificationPeriod = modificationPeriodCoeff * defaultPagingCycle

(modificationPeriodCoeff, defaultPagingCycle are given by SIB signalling).

The SFN and the H-SFN for the start of modification period are calculated by TTCN. The modified system information and the calculated SFN, H-SFN are provided in the ASP NB_SYSTEM_CTRL_REQ.

TTCN implementation is responsible to wait long enough to ensure that the UE has acquired the new system information. With modificationPeriodCoeff=n32 and defaultPagingCycle=rf128 this means that TTCN needs to wait more than 40s to be sure that the UE has got the change.

In case UE is configured in eDRX, the boundaries of the eDRX acquisition period are determined by H-SFN values for which H-SFN mod 1024 =0 and calculated by TTCN accordingly. Duration of one eDRX cycle is 10.24s.

As working assumption it is sufficient to schedule just one paging message or one direct indication in a modification period to indicate change of system information in the next modification period. The same working assumption is applied when UE is configured in eDRX.

## 7A.3	Search space configurations

There are 3 kinds of search spaces in NB-IoT (TS 36.213 [30] clause 16.6):

-	Type1-NPDCCH common search space (Type1CSS): Paging

-	Type2-NPDCCH common search space (Type2CSS): Random access

-	NPDCCH UE-specific search space (UESS): UL grants and DL assignments for UE in connected mode

The search spaces define in which time windows the network may schedule DL assignments and UL grants.

Scheduling of DL assignments and UL grants is not possible in subframes occupied by system information or synchronisation signals or in subframes outside the respective search space.

-	Timing of NB-IoT conformance test cases strongly depends on the system information scheduling (see clause 7A.2.2) and the search space configurations described in this clause.

In addition it is the responsibility of the SS to determine the exact timing for scheduling of DL assignments and UL grants.

Table 7A.3-1 shows the relevant parameters for search space calculations.

Table 7A.3-1: Parameters for search space calculations

| Rmax | Type1CSS | configured in SIB2-NB (PCCH-Config-NB-r13) |
| --- | --- | --- |
|  | Type2CSS | configured in SIB2-NB (NPRACH-Parameters-NB-r13) |
|  | UESS | configured in RadioResourceConfigDedicated-NB-r13 (NPDCCH-ConfigDedicated-NB-r13) |
| G | Type1CSS | (not applicable) |
|  | Type2CSS | configured in SIB2-NB (NPRACH-Parameters-NB-r13) |
|  | UESS | configured in RadioResourceConfigDedicated-NB-r13 (NPDCCH-ConfigDedicated-NB-r13) |
| offset | Type1CSS | (not applicable) |
|  | Type2CSS | configured in SIB2-NB (NPRACH-Parameters-NB-r13) |
|  | UESS | configured in RadioResourceConfigDedicated-NB-r13 (NPDCCH-ConfigDedicated-NB-r13) |
| T | Type1CSS | DRX cycle according to PCCH-Config-NB-r13.defaultPagingCycle-r13 |
|  | Type2CSS | search space cycle: Rmax * G |
|  | UESS | search space cycle: Rmax * G |
| R | Type1CSS | depends on Rmax and the "DCI subframe repetition number" in the DCI format N2 as shown in TS 36.213 [30] table 16.6-2 |
|  | Type2CSS | depends on Rmax and the "DCI subframe repetition number" in the DCI format N1 as shown in TS 36.213 [30] table 16.6-3 |
|  | UESS | depends on Rmax and the "DCI subframe repetition number" in the DCI format N0/N1 as shown in TS 36.213 [30] table 16.6-1 |

### 7A.3.1	Type1CSS - Paging

For NB-IoT the same calculations for paging are applied as for WB-E-UTRA (TS 36.304 [14] clause 7), but in contrast to WB-E-UTRA the paging does not happen at the paging occasion itself, but the Type1-NPDCCH common search space starts at the paging occasion according to TS 36.213 [30] clause 16.6: DCI with format N2 is sent out on NPDCCH from the next suitable subframe onward (starting at the paging occasion).

As for WB-E-UTRA the paging occasion is calculated by TTCN and provided to SS as timing information of the system request. The SS shall schedule the DCI and the corresponding paging message (if any) according to the given configuration of the Type1CSS, PCCH configuration and according to the rules provided by core specifications (mainly TS 36.213 [30] clause 16).

### 7A.3.2	Type2CSS – Random access

In principle the same considerations can be applied as for the UESS (see below).

### 7A.3.3	UESS

Taking into account the SIB scheduling as described in clause 7A.2 and applying parameters for the UESS as per TS 36.508 [3], UESS configuration results in:

Table 7A.3.3-1: Parameters for UESS

| Rmax | 16 | according to TS 36.508 [3] table 8.1.6.3-3 |
| --- | --- | --- |
| G | 4 | according to TS 36.508 [3] table 8.1.6.3-3 |
| offset | 0 | according to TS 36.508 [3] table 8.1.6.3-3 |
| T | 64 | Rmax * G |
| R | 2 | according to DCI subframe repetition number in TS 36.508 [3] Table 8.1.3.6.1.2-2 and parameters as above |

-	according to TS 36.213 [30] clause 16.6

the search spaces start at k0 = 0, 64, 128, 192, 256, ... 
the candidates within a search space start at b = R * u = 0, 2, 4, 6, 8, 10, 12, 14.

Table 7A.3.3-2: Scheduling of UESS

| K | SFN = K |  |  |  |  |  |  |  |  |  | SFN = K + 1 |  |  |  |  |  |  |  |  |  | SFN = K + 2 |  |  |  |  |  |  |  |  |  | SFN = K + 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 |
| ... ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 48 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 64 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 68 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 76 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

With

|  |  | NPBCH |  | NPSS |  | NSSS |  |  |  | SIB1-NB |  | SI-1 |  |  | SI-2 |  | SI-3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | search space candidate 0 |  |  |  |  |  |  | search space candidates 1..7 |  |  |  |  |  |

NOTE 1:	Depending on SS configuration in general the first search space candidate can be used for the transmission of the DCI. Therefore the first search space candidate is highlighted in table 7A.3.3-2.

## 7A.4	Timing considerations

Based on SIB scheduling and search space configuration as described in the previous clauses timing considerations are made as below.

### 7A.4.1	Random access procedure

Details regarding random access procedure in NB-IoT are specified in clause 16.3 of TS 36.213 [30] and clause 5.1 of TS 36.321 [16]. The timing of a random access procedure in NB-IoT is illustrated in table 7A.4.1-1.

Table 7A.4.1-1: Timing of random access procedure

| ... | ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... |  | ... |  |  | ... |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ... | Type2 CSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... |  |  | Type2 CSS |  |  | ... |
| RACH preamble |  |  |  | DCI format N1 |  |  |  |  |  | RAR with UL grant |  |  |  |  |  |  |  |  | Msg3 | ... | DCI format N0/1 |  |  |  |
| ... | tdelay1 |  |  | tDCI |  | tdelay2 |  |  |  | tRAR |  | tdelay3 |  |  |  |  |  |  | tMsg3 | tdelay4 | tDCI |  | tdelay5 |  |
| step 1 |  |  |  | step 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | step 3 |  | step 4 |  |  | ... |

with

| steps 1-4 | Random access procedure according to TS 36.300 [69] clause 10.1.5.1: step 2: RAR is addressed to RA-RNTI step 3: Msg3 contains either a CCCH SDU (a) or the C-RNTI (b) step 4: a) temporary C-RNTI based contention resolution (DL assignment for Msg4) b) C-RNTI based contention resolution (UL grant) NOTE: According to TS 36.213 [30] clause 16.5.1, Table 16.5.1-4 and clause 16.4.1, Table 16.4.1-5 the DCI for step 4 is sent on NPDCCH in the Type2 common search space |  |
| --- | --- | --- |
| tdelay1 | 3ms (but up to 50ms) | depends on Type2-CSS configuration, start of RA response window according to TS 36.321 [16] clause 5.1.4 and when the RACH preamble happens |
| tDCI | 2ms | for Rmax = 16 and DCI subframe repetition number = 0 (TS 36.213 [30] clause 16.6) |
| tdelay2 | 4ms | scheduling delay according to DCI (TS 36.213 [30] clause 16.4.1 and table 16.4.1-1) |
| tRAR | 1ms | depending on TBS and number of repetitions (TS 36.213 [30] clause 16.4.1) |
| tdelay3 | 12ms | scheduling delay according to RAR grant (TS 36.213 [30] clause 16.3.3) |
| tMsg3 | 1ms | depending on MSC index and number of repetitions according to RAR grant; the TBS is 88 bits in all cases (TS 36.213 [30] clause 16.3.3) |
| tdelay4 | 3ms | delay before the UE monitors NPDCCH again (TS 36.213 [30] clause 16.6) |
| tdelay5 | 13ms | depending on whether UL or DL transmission follows, see clause 7A.4.2 and 7A.4.3 |

NOTE 1:	In addition to the delay in table 7A.4.1-1 there is further delay caused by PBCH, NPSS, NSSS, SI transmissions.

NOTE 2:	Assuming a search space configuration as per clause 7A.3 in general a random access procedure needs more than two search space cycles.

### 7A.4.2	Uplink transmissions

Table 7A.4.2-1: Timing of uplink transmissions

| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... | DCI format N0 |  |  |  |  |  |  |  |  |  | UL-SCH |  |  |  |  | ACK/NACK |  | ... |
|  | tDCI1 |  | tdelay1 |  |  |  |  |  |  |  | tData |  | tdelay2 |  |  | tDCI2 |  |  |

with

| tDCI1 | 2ms | for Rmax = 16 and DCI subframe repetition number = 0 (TS 36.213 [30] clause 16.6) |
| --- | --- | --- |
| tdelay1 | 8ms | according to TS 36.213 [30] table 16.5.1-1 |
| tData | 1ms | depending on TBS and number of repetitions (TS 36.213 [30] clause 16.5.1 and TS 36.211 Table 10.1.2.3-1) (NOTE 4) |
| tdelay2 | 3ms | according to TS 36.213 [30] clause 16.5.2 |
| tDCI2 | 2ms | (same as tDCI1) |

NOTE 1:	In addition to the delay in table 7A.4.2-1 there is further delay caused by PBCH, NPSS, NSSS, SI transmissions.

NOTE 2:	Assuming a search space configuration as per clause 7A.3 in general an uplink transmission takes one search space cycle.

NOTE 3:	Even when in case of necessary UL re-transmission the grant for the re-transmission is sent immediately using adaptive HARQ NACK, it is hardly predictable how many search space cycles the complete successful transmission takes. Nevertheless one re-transmission may still be finished in the same search space cycle in which the initial UL has been scheduled.

NOTE 4:	For single tone () according to TS 36.213 [30] Table 16.5.1.2-1 the maximum value of ITBS is 10. Therefore according to table 16.5.1.2-2 the maximum grant of 1000 bits requires at least IRU = 5 corresponding to 6 resource units.
Furthermore for single tone according to TS 36.211 [35] Table 10.1.2.3-1 a resource unit takes 16 slots i.e. 8ms.
 tData  48ms (6 * 8) for maximum grant (single-tone).

### 7A.4.3	Downlink transmissions

Table 7A.4.3-1: Timing of downlink transmissions

| ... |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | ... |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ... | DCI format N1 |  |  |  |  |  | DL-SCH |  |  |  |  |  |  |  |  |  |  |  |  |  | ACK/NACK |  |  |  | ... |
|  | tDCI |  | tdelay1 |  |  |  | tData |  | tdelay2 |  |  |  |  |  |  |  |  |  |  |  | tACK/NACK |  |  |  |  |

with

| tDCI | 2ms | for Rmax = 16 and DCI subframe repetition number = 0 (TS 36.213 [30] clause 16.6) |
| --- | --- | --- |
| tdelay1 | 4ms | scheduling delay according to DCI (TS 36.213 [30] clause 16.4.1 and table 16.4.1-1) |
| tData | 1ms | depending on TBS and number of repetitions (TS 36.213 [30] clause 16.4.1) |
| tdelay2 | 12ms | depending HARQ-ACK resource allocation according to TS 36.213 [30] clause 16.4.2 |
| tACK/NACK | 2ms | multiples of 2ms according to TS 36.211 [35] Table 10.1.2.3-1; depending on number of repetitions configured by RRC signalling (NPUSCH-ConfigCommon-NB-r13, NPUSCH-ConfigDedicated-NB-r13; NRep 1) |

NOTE 1:	In addition to the delay in table 7A.4.3-1 there is further delay caused by PBCH, NPSS, NSSS, SI transmissions.

NOTE 2:	Assuming a search space configuration as per clause 7A.3 in general a downlink transmission takes one search space cycle.

NOTE 3:	Assuming a search space configuration as per clause 7A.3 in case of a HARQ NACK from the UE, a re-transmission needs to be scheduled in the next search space cycle (as even when the DL transmission has been scheduled using the first search space candidate, at the point in time the SS detects the NACK there is no search space candidate in the current search space cycle anymore).

### 7A.4.4	Half-duplex mode

According to TS 36.211 [35] for NB-IoT only type-B half-duplex FDD operation is supported (clause 10.2.2.3) in which according to clause 4.1 the UE cannot transmit and receive at the same time.

In addition in clause 16.6 of TS 36.213 [30] there are further rules further restricting the timing of UL/DL transmissions which are summarised in table 7A.4.4-1 and table 7A.4.4-2 for Rel-13 and if the UE is not configured with higher layer parameter twoHARQ-ProcessesConfig.

Table 7A.4.4-1: Uplink transmissions

|  | n | n+1 | ... | n+k-1 | n+k |  |  | n1 | n1+1 | n1+2 | n1+3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCI Format N0 |  |  |  |  | NPUSCH transmission |  |  |  |  |  |  | ACK/NACK (NPDCCH) |  |
|  |  | UL-SCH transmission: NPDCCH not monitored by the UE |  |  |  |  |  |  |  |  |  |  |  |

Table 7A.4.4-2: Downlink transmissions

|  | n | n+1 |  |  |  |  |  |  |  |  |  |  |  |  | n+k-1 | n+k |  |  | n1 | n1+1 | n1+2 | n1+3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DCI Format N1 |  |  |  |  |  |  | NPDSCH transmission |  |  |  |  |  |  |  |  | NPUSCH Format 2: ACK/NACK |  |  |  |  |  |  |
|  |  | DL-SCH transmission: NPDCCH not monitored by the UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

If the UE is configured with higher layer parameter twoHARQ-ProcessesConfig (Rel-14 and later), the UE can detect another PDCCH candidate before NPDSCH or NPUSCH transmission starts (see TS 36.213 [30] clause 16.6 for further details). Nevertheless there needs to be an appropriate timing for the transmissions being associated with two consecutive PDCCH candidates.

NOTE:	So far there are no test requirements in any test case to schedule more than one PDCCH candidate in the same search space.

### 7A.4.5	Conclusions

Based on the system information scheduling as in clause 7A.2 and search space configuration as in clause 7A.3 the following conclusions are made:

-	Random access procedure:
After detection of the random access preamble steps 2-4 of the random access procedure take at least two search space cycles.

-	Uplink and downlink transmissions:
In general an UL or DL transmission takes one search space cycle and UL/DL transmissions are mutually exclusive, i.e. a search space cycle can be either for UL or for DL.

## 7A.5	Scheduling requests and scheduling of UL grants

As there is no NPUCCH in NB-IoT the UE cannot directly indicate scheduling requests to the network. Instead according to TS 36.321 [16] clause 5.4.4 the UE starts with a RACH procedure, unless an SR-ProhibitTimer is configured by RRC signalling.

NOTE 1:	It seems not to be reliable that, when no SR-ProhibitTimer is configured, the UE will use an UL grant being scheduled in advance.

NOTE 2:	It seems not to be reliable that a UE uses an UL grant addressed with C-RNTI when it has already started a RACH procedure and is waiting for the RA response being addressed by the RA-RNTI. Furthermore there is no core spec reference about the UE to abort the RACH procedure in this case.

In terms of UL scheduling there are two modes for the UE in RRC connected mode: RACH procedure mode and polling mode.

### 7A.5.1	RACH procedure mode

UL grant scheduling is triggered by a RACH procedure and the subsequent evaluation of BSRs. There is no explicit scheduling of UL grants in advance.

The SS assigns UL grants according to RACH procedure configuration and common MAC configuration (DCI format N0 configuration of UL grants).

NOTE:	In case of uncorrelated UL and DL transmissions there may be race conditions resulting in the UE having started a random access procedure already when the SS schedules the DL transmission. In this case the UE may not read the DL assignment anymore and therefore not receive the DL data.

### 7A.5.2	Polling mode

SR-ProhibitTimer (logicalChannelSR-Prohibit-r13 according to Rel-13 ASN.1, TS 36.331 [19]) is configured at the UE (for each logical channel) and the SS polls the UE with UL grants: When the UE has UL data pending it will use a given grant; the UE in connected mode will not start any RACH procedure while SR-ProhibitTimer is running.

The SS gets configured with periodic UL grants to be assigned to the UE: grant size, periodicity, total number of grants.

## 7A.6	Scheduling requirements

As conclusion of the above timing and scheduling considerations this clause defines SS requirements about how to schedule UL grants and DL assignments.

### 7A.6.1	Random access procedure

The SS gets configured with:

-	configuration of the Type2 common search space (Rmax, G, offset)

-	Type 2 common search space candidate to schedule RAR

-	Type 2 common search space candidate to schedule DL assignment or UL grant at step 4 of the random access procedure

-	UL grant to be provided in RAR

-	DCI for DL assignment of Msg4 in case of temporary C-RNTI based contention resolution

-	For C-RNTI based contention resolution (RACH procedure mode):
DCI for UL grants to be used at step 4 of the random access procedure and for any subsequent UL transmissions (if any)

The SS shall:

-	when it has detected a RACH preamble, wait until start of the next Type2 common search space within the RA response window and schedule the Random Access Response containing the UL grant for Msg3 by using the Type2 common search space candidate as configured

-	after it has received Msg3, schedule a DL assignment or UL grant in the next Type2 common search space by using the search space candidate as configured

-	in case of RACH procedure with C-RNTI based contention resolution (i.e. UE in connected mode), check whether the UL MAC PDU sent at step 4 of the random access procedure contains a BSR element indicating that the UE needs more UL grant. If so, the SS shall use the ACK/NACK response corresponding to step 4's UL transmission as specified in clause 16.5.2 of TS 36.213 [30] to schedule another UL grant as configured at the SS 
( the SS shall not do any dynamic calculation for the UL grant based on the details of the BSR);
this shall be repeated until the UE does not indicate need for further UL grant anymore.
Pending DL transmissions shall be postponed until after all UL data has been received.
NOTE: This rule applies for RACH procedure mode according to clause 7A.5.1.

-	in case of RACH procedure with C-RNTI based contention resolution (i.e. UE in connected mode), not use any adaptive HARQ ACK to schedule a DL transmission.

### 7A.6.2	Downlink transmissions

The SS gets configured with:

-	configuration of the UE specific search space (Rmax, G, offset)

-	search space candidate to be used for DL assignments within the search space

-	DCI to be used for DL assignments:
normal mode: 	transport block size, ITBS and ISF to be automatically determined by the SS
explicit mode:	DCI with ITBS, ISF provided by TTCN

When the SS gets a message from TTCN to be sent in DL it shall:

-	when the timing info is "now", schedule the DL assignment in the next search space not being used for HARQ re-transmissions.

-	when a specific timing info is given, schedule the DL assignment by using the configured search space candidate in the next NPDCCH period starting at or after the given point in time. If for any reason there is an ongoing UL or DL transmission so that the UE may not monitor the NPDCCH, the SS shall raise an error. 
NOTE: The search space candidate is given as index (0..7) in the configuration of the UE specific search space corresponding to u in the calculations of clause 16.6 in TS 36.213 [30].

-	when timing info is "now" and a previous DL transmission is still pending, schedule the DL assignment of the 2nd message in the search space starting after the HARQ ACK of the 1st message.

-	when e.g. an RRC message containing user data needs to be segmented by the RLC layer, transmission of the resulting transport blocks shall be scheduled in consecutive search space cycles and the given timing info is applied for the first transmission.

-	when TBS, ITBS and ISF shall be determined by the SS, use the mapping according to table 7A.6.2-1, otherwise use ITBS and ISF as provided by TTCN.

Table 7A.6.2-1: DL Resource allocation in normal mode

| amount of pending DL data (bits): NData | TBS to be chosen by the SS | ITBS (NOTE) | ISF (NOTE) | Comment |
| --- | --- | --- | --- | --- |
| NData ≤ 16 | 16 | 0 | 0 |  |
| 16 < NData ≤ 24 | 24 | 1 | 0 |  |
| 24 < NData ≤ 32 | 32 | 2 | 0 |  |
| 32 < NData ≤ 40 | 40 | 3 | 0 |  |
| 40 < NData ≤ 56 | 56 | 4 | 0 |  |
| 56 < NData ≤ 72 | 72 | 5 | 0 |  |
| 72 < NData ≤ 88 | 88 | 6 | 0 |  |
| 88 < NData ≤ 104 | 104 | 7 | 0 |  |
| 104 < NData ≤ 120 | 120 | 8 | 0 |  |
| 120 < NData ≤ 136 | 136 | 9 | 0 |  |
| 136 < NData ≤ 144 | 144 | 10 | 0 | TBS=152 skipped as it would require ISF=5 (NOTE 1) |
| 144 < NData ≤ 176 | 176 | 6 | 1 | NOTE 2 |
| 176 < NData ≤ 208 | 208 | 4 | 2 | NOTE 2 |
| 208 < NData ≤ 224 | 224 | 7 | 1 |  |
| 224 < NData ≤ 256 | 256 | 8 | 1 |  |
| 256 < NData ≤ 296 | 296 | 9 | 1 |  |
| 296 < NData ≤ 328 | 328 | 10 | 1 | TBS=344 skipped as it would require ISF=7 (NOTE 1) |
| 328 < NData ≤ 392 | 392 | 8 | 2 | TBS=376 skipped as it would require ITBS=11 (NOTE 2) TBS=408 skipped as it would require ISF=5 (NOTE 1) TBS=424 skipped as it would require ISF=4 (NOTE 1) |
| 392 < NData ≤ 456 | 456 | 9 | 2 | TBS=440 skipped as it would require either ITBS=12 (NOTE 2) or ISF=6 (NOTE 1) |
| 456 < NData ≤ 472 | 472 | 7 | 3 |  |
| 472 < NData ≤ 504 | 504 | 10 | 2 |  |
| 504 < NData ≤ 536 | 536 | 8 | 3 | TBS=552 skipped as it would require ISF=6 (NOTE 1) TBS=568 skipped as it would require ISF=7 (NOTE 1) |
| 536 < NData ≤ 616 | 616 | 9 | 3 | TBS=584 skipped as it would require either ITBS=11 (NOTE 2) or ISF=4 (NOTE 1) TBS=600 skipped as it would require ISF=5 (NOTE 1) |
| 616 < NData | 680 | 10 | 3 | NOTE 2 |
| NOTE 1: According to TS 36.213 [30] Table 16.4.1.5.1-1 NOTE 2: According to TS 36.213 [30] clause 16.4.1.5.1 for operationModeInfo set to ‘00’ or ‘01’ (Inband Operation Mode) ITBS is 0 ≤ ITBS ≤ 10 |  |  |  |  |

-	ISF ≤ 3 for all DL transmission (i.e. a DL transmission needs not more than 4 NB-IoT DL subframes, which is needed for maximum TBS of 680).

### 7A.6.3	Periodic uplink grants

Periodic scheduling of UL grants applies for polling mode according to clause 7A.5.2, i.e. in general the UE is configured with SR-ProhibitTimer(s) to be prevented from triggering RACH procedures in connected mode.

The SS gets configured with:

-	configuration of the UE specific search space (Rmax, G, offset)

-	search space candidate to be used for UL grants within the search space

-	configuration of grant scheduling:

-	DCI of the UL grants

-	number of UL grants to be scheduled: one grant only, several, continuous

-	periodicity: every search space, every 2nd search space, ...

When the SS gets configured for periodic scheduling of UL grants, the SS shall:

-	start UL grant scheduling with the next available search space candidate as configured for the UE specific search space after the given timing information (which can be "now" or a specific point in time).

-	not give any additional grants based on buffer status reporting by the UE.

-	not use the adaptive HARQ ACK to schedule another transmission.

-	when DL transmission is scheduled, shift any UL grant which would be scheduled for the same search space to the next search space cycle not being needed for DL transmission, i.e. DL transmissions have higher priority than UL grants.
NOTE: the periodicity is related to beginning of the UL grant scheduling i.e. independent from whether or not a single grant is shifted due to DL transmissions.

Table 7A.6.3-1 gives an example for periodic UL grant scheduling for a periodicity of every 3rd search space cycle: at UESS cycle 7 the UL grant is shifted to cycle 8 due to DL transmission.

Table 7A.6.3-1: Periodic UL grant scheduling – Example for UL and DL transmissions

| ... | UESS cycle 1 | UESS cycle 2 | UESS cycle 3 | UESS cycle 4 | UESS cycle 5 | UESS cycle 6 | UESS cycle 7 | UESS cycle 8 | UESS cycle 9 | UESS cycle 10 | ... |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UL grant configuration | UL |  | DL | UL |  |  | DL | UL |  | UL |  |

### 7A.6.4	HARQ re-transmissions

NOTE:	HARQ re-transmission are considered being rare in a RF cabled test environment. Nevertheless depending on test requirements HARQ re-transmissions may be tolerated at least for some test cases.

The SS shall report HARQ errors to TTCN if HARQ error reporting is configured by TTCN.

#### 7A.6.4.1	Uplink

When the SS needs to request a re-transmission in uplink, it shall use adaptive HARQ ACK/NACK to indicate the NACK to the UE.  The re-transmission shall be scheduled immediately by using the ACK/NACK response corresponding to the UL transmission as specified in clause 16.5.2 of TS 36.213 [30] independent from any UL grant scheduling.

If the re-transmission is not successfully finished within the search cycle in which the transmission has been started, i.e. a second search space cycle is needed to complete the UL transmission, the SS shall:

-	if a DL transmission is pending with timing info "now":
postpone the DL transmission

-	if DL transmission is scheduled explicitly for the second search space cycle:
raise an error

-	if an UL grant is scheduled for the second search space cycle:
postpone this UL grant for one cycle.

#### 7A.6.4.2	Downlink

When the UE indicates a HARQ NACK to the SS for a DL transmission in search space cycle N, the SS shall re-schedule the DL assignment in search space cycle N+1.

The SS shall:

-	if another DL transmission is pending with timing info "now":
postpone the DL transmission

-	if DL transmission is scheduled explicitly for the second search space cycle:
raise an error

-	if an UL grant is scheduled for the second search space cycle:
postpone this UL grant for one cycle.

Depending on test requirements, the SS may get configured to generate a CRC error for a DL transmission corresponding to a configured DCI. In this case the DCI may be reconfigured before the retransmission happens. Therefore the SS shall not re-use the DCI of the initial transmission but shall consider a possible reconfiguration of the DCI.

### 7A.6.5	Timing info for UL messages

The SS shall provide the H-SFN, SFN and subframe number of the subframe corresponding to the first RU of the NPUSCH transmission. It is up to TTCN implementation to derive other timing information if needed (e.g. the start of the corresponding search space).

NOTE:	E.g. in case of IP data a layer 3 message carrying the user data on the control plane may be segmented by the RLC layer. In this case the timing info shall be related to the first segment.

### 7A.6.6	Uplink time alignment

As for legacy LTE the SS can be configured to periodically send Timing Advance Commands to the UE to prevent the UE from starting a RACH procedure due to expiry of the Time Alignment Timer. When the Time Alignment Timer at the UE is set to infinity, the SS is configured not to do any automatic transmission of Timing Advance Commands.

## 7A.7	RRC connection release sequence

According to TS 36.331 [19] clause 5.3.8.3, after reception of the RRCConnectionRelease the UE may either wait 10s or for indication of acknowledgement from lower layer. Furthermore according to TS 36.331 [19] clause 5.3.12, upon leaving RRC_CONNECTED state the UE resets layer 2.

Assuming search space cycles of 64ms as described in clause 7A.3 and with a given search space cycle N starting at time T being at least 100ms in advance, TTCN schedules the RRC connection release sequence as:

1.	At T:	Start of search space cycle N:
	Send RRCConnectionRelease, stop UL grants.

2.	At T + 64ms:	Start of search space cycle N+1:
	Release security (optional; only when security has been configured).
	Scheduling of UL grants for search space cycle N+1 and four subsequent cycles
	(NOTE 1).

3.	At T + 80ms:	Release SRBs and DRBs (if configured).

4.	At T + 96ms:	(Re-)configure SRBs.

5.	Delay of 1100ms from T onward (NOTE 2)

NOTE 1:	The UL grants are provided to allow the UE to send an RLC ACK to acknowledge the RRCConnectionRelease; as RLC is not configured anymore or reset at the SS already, the ACK is discarded by the SS  It is assumed that once the UE has sent the RLC ACK it stops the 10s timer.

NOTE 2:	The delay of 1100ms is to ensure the UE is re-camping on the cell and has read relevant system information, MIB-NB, SIB1-NB, SIB2-NB and all other SIs.

## 7A.8	DL RRC message and UE Contention Resolution Identity MAC CE transmission in one MAC PDU or in separate MAC PDUs

### 7A.8.1	DL CCCH message

In Rel-13 or if the UE does not support early contention resolution, in case of temporary C-RNTI based contention resolution, Msg4 of the random access procedure shall contain the DL CCCH message together with the UE Contention Resolution Identity MAC Control Element transmission, i.e. the DL CCCH cannot be sent in a separate DL message after contention resolution.

If the UE supports early contention resolution, Msg4 of the random access procedure may contain the UE Contention Resolution Identity MAC Control Element transmission only, i.e. the DL CCCH message can be scheduled in a separate DL transmission.

### 7A.8.2	DL DCCH message

In Rel-13 or if the UE does not support early contention resolution, in case of RRC connection resume, it is necessary to allow even a DL DCCH message to be contained in Msg4 of the random access procedure together with the UE Contention Resolution Identity MAC Control Element.

In this case the SS gets configured by TTCN with the PER encoded RRC message (PDCP SDU). Then the SS shall:

-	apply integrity protection according to security configuration at the point in time when the Msg3 is received,

-	add a PDCP header accordingly,

-	add an AM RLC header,

-	send the given RRC message in Msg4.

The SS shall not segment the RLC SDU, i.e. the RLC PDU sent within Msg4 shall contain the complete DL DCCH RRC message. The logical channel id to be used for the MAC DL-SCH SDU shall be ‘00001’B corresponding to SRB1.

## 7A.9	Cell configuration

### 7A.9.1	Cell power change

Please refer to clause 7.4.2

### 7A.9.2	Timing parameters of cells

To reduce interference for cells with same frequency the timing for these cells is shifted to avoid that NPSS and NSSS are transmitted at the same time. This is done by TS values in multiples of 30720 (one subframe).

Table 7A.9.2-1 shows timing parameters for cells with frequencies according to Table 8.1.4.2-1 in TS 36.508 [3], Table 7A.9.2-2 shows timing parameters for cells with frequencies according to Table 8.1.4.2-4 in TS 36.508 [3], Table 7A.9.2-3 shows timing parameters for cells with frequencies according to Table 8.1.4.2-5 in TS 36.508 [3]; 
cells with same frequency are marked with the same colour.

Table 7A.9.2-1: Timing parameters of simulated NB-IoT cells – Batch1

| Cell ID | SFN offset | Tcell (Ts) | Frequency (Table 8.1.4.2-1 in TS 36.508 [3]) |
| --- | --- | --- | --- |
| Ncell 1 | 0 | 0 | f1 |
| Ncell 2 | 124 | 184320 | f1 |
| Ncell 3 | 257 | 150897 | f2 |
| Ncell 4 | 1000 | 61440 | f1 |
| Ncell5 | 1020 | 30720 | f1 |
| Ncell 6 | 657 | 524 | f3 |
| Ncell 10 | 129 | 43658 | f5 |
| Ncell 11 | 957 | 92160 | f1 |
| Ncell 12 | 1015 | 181617 | f2 |
| Ncell 13 | 890 | 31244 | f3 |
| Ncell 14 | 680 | 300501 | f4 |
| Ncell18 | 280 | 122880 | f1 |
| Ncell 23 | 383 | 212337 | f2 |

Table 7A.9.2-2: Timing parameters of simulated NB-IoT cells – Batch2 in same PLMN

| Cell ID | SFN offset | Tcell (Ts) | Frequency (Table 8.1.4.2-4 in TS 36.508 [3]) |
| --- | --- | --- | --- |
| Ncell 50 | 0 | 0 | f1 |
| Ncell 51 | 124 | 30720 | f1 |
| Ncell 52 | 257 | 150897 | f2 |
| Ncell 53 | 1000 | 61440 | f1 |
| Ncell 61 | 471 | 31244 | f2 |

Table 7A.9.2-2: Timing parameters of simulated NB-IoT cells – Batch2 in different PLMNs

| Cell ID | SFN offset | Tcell (Ts) | Frequency (Table 8.1.4.2-5 in TS 36.508 [3]) |
| --- | --- | --- | --- |
| Ncell 50 | 0 | 0 | f1 |
| Ncell 51 | 124 | 30720 | f1 |
| Ncell 52 | 257 | 61440 | f1 |
| Ncell 53 | 1000 | 92160 | f1 |
| Ncell 54 | 752 | 32047 | f2 |
| Ncell 55 | 957 | 631 | f2 |
| Ncell 56 | 1015 | 31351 | f2 |
| Ncell 57 | 890 | 127200 | f3/f1 |
| Ncell 58 | 680 | 1327 | f4/f2 |
| Ncell 59 | 383 | 157920 | f3/f1 |
| Ncell 60 | 562 | 188640 | f3/f1 |
| Ncell 61 | 471 | 122880 | f1 |
| Ncell 62 | 595 | 153600 | f1 |
| Ncell 63 | 728 | 184320 | f1 |

The Random Access Response Timing Advance is set to 0.

### 7A.9.3	Configuration of multi-cell environment

Please refer to clause 7.4.5

## 7A.10	Timers and timing restrictions

Please refer to clause 7.8

## 7A.11	Error indication

The SS shall raise an error indication in all cases of system errors or other unforeseen events at lower layers.

In addition the SS may get configured for HARQ error reporting, e.g. in cases when HARQ retransmissions would have unacceptable impact on the test behaviour.

Any error indication results in an "inconc" verdict in TTCN.

## 7A.12	RSRQ calculations

Please refer to clause 7.22.

## 7A.13	RRC connection suspend and resume

General sequence of RRC connection suspend and resume for NB-IoT:

1.	RRC Connection Release Sequence according to clause 7A.7.

2.	Reestablishment of the AS security at the SS (for the cell at which subsequent RRC Connection Resume is expected).

3.	SS configuration with DL DCCH message to be contained in Msg4 of the RACH procedure according to clause 7A.8.2.

4.	Regular RACH procedure but with Msg4 containing pre-configured DL DCCH message:

-	The final encoding of DL DCCH message (AS security, PDCP header, RLC header) shall not be done before start of the RRC connection resume procedure.

# 8	Other SS Requirements with TTCN-3 impact

## 8.1	Codec Requirements

For UE conformance tests several encoding rules need to be applied by the TTCN-3 codec. In principle, how the codec is implemented in the SS is out of scope of the present document. However there are aspects with impact on TTCN-3 implementation depending on different type definitions, in particular for protocol messages exchanged with the UE under test. Table 8.1-1 lists the codec requirements that the SS shall comply to.

Table 8.1-1: Codec requirements

| Type definitions | Codec requirements | Encoding rule in TTCN-3 |
| --- | --- | --- |
| E-UTRA ASN.1 types used for RRC signalling | shall comply to TS 36.331 [19] clause 8.3 | UNALIGNED_PER_OctetAligned |
| UTRA ASN.1 types used for RRC signalling | shall comply to TS 25.331 [36] clause 12.1 | PER_Unaligned |
| LPP(e) ASN.1 types used for LPP(e) signalling | shall comply to TS 37.355 [71] clause 6.1 | UNALIGNED_PER_OctetAligned |
| NAS types | shall comply to Tabular Notated (see note) | NAS Types |
| SMS Types | shall comply to Tabular Notated (see note) | SMS Types |
| DRB types | shall comply to Tabular Notated (see note) | DRB Types |
| GERAN types | shall comply to Tabular Notated (see note) | GERAN Types |
| GPRS Padding | shall comply to TS 34.123-3 [7], clause 6.10.2.9.1 | GPRSPadding Rule1 |
| GSM Spare Padding | shall comply to TS 34.123-3 [7], clause 6.10.2.9.2 | GSM Spare Padding Rule2 |
| LowHigh Rule | shall comply to TS 34.123-3 [7], clause 6.10.2.9.3 | LowHighRule3 |
| SACCHSysInfo Spare Padding | shall comply to TS 34.123-3 [7], clause 6.10.2.9.5 | SACCHSysInfoSparePadding Rule4 |
| ASN.1 & TTCN-3 types not used at the air interface: - Configuration of system simulator - Coordination between components - Types used internally in TTCN-3 | no specific encoding requirement | - |
| NOTE: Tabular Notated is performed by concatenation of all the present fields in the TTCN-3 template. |  |  |

# 8.2	External Function Definitions

The following external functions shall be implemented by the SS.

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_KeyDerivationFunction |  |
| Description | Hashing function for Hashing algorithms as defined in TS 33.401 [24] SHA-256 encoding algorithm is used as KEY Description Function |  |
| Parameters | KDF | KDF_HMAC_SHA_256 (no other KDF defined yet) |
|  | Key | bit key |
|  | String | string being constructed acc. to TS 33.401 [24], annex A |
| Return Value | 256 bit derived key |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_NasIntegrityAlgorithm |  |
| Description | Apply integrity protection algorithm on a given octetstring |  |
| Parameters | NAS PDU | octetstring according to TS 24.301 [21], clause 4.4.3.3 this shall include octet 6 to n of the security protected NAS message, i.e. the sequence number IE and the NAS message IE |
|  | Integrity Algorithm | 3 bits as defined in TS 24.301 [21], clause 9.9.3.23 |
|  | KNASint | Integrity key |
|  | NAS COUNT | as documented in TS 24.301 |
|  | BEARER Id | fix value ('00000'B) acc. TS 33.401 [24], clause 8.1 |
|  | Direction | UL: 0 DL: 1 (acc. to TS 33.401 [24], clause B.1) |
| Return Value | Message Authentication Code (4 octets) |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_NasIntegrityAlgorithmBitstring |  |
| Description | Apply integrity protection algorithm on a given bitstring |  |
| Parameters | Message | bitstring according to TS 33.401 [24], clause 7.4.4 this shall contain the target cell identity |
|  | Integrity Algorithm | 3 bits as defined in TS 24.301 [21], clause 9.9.3.23 |
|  | KNASint | Integrity key |
|  | NAS COUNT | as documented in TS 24.301 |
|  | BEARER Id | fix value ('00000'B) acc. TS 33.401 [24], clause 8.1 |
|  | Direction | UL: 0 DL: 1 (acc. to TS 33.401 [24], clause B.1) |
| Return Value | Message Authentication Code (4 octets) |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_NasCiphering |  |
| Description | Apply ciphering on a given octetstring |  |
| Parameters | NAS PDU | octetstring |
|  | Ciphering Algorithm | 3 bits as defined in TS 24.301 [21], clause 9.9.3.23 |
|  | KNASenc | Ciphering Key |
|  | NAS COUNT | as documented in TS 24.301 |
|  | BEARER Id | fixed value ('00000'B) acc. TS 33.401 [24], clause 8.1 |
| Return Value | ciphered octet string |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_NasDeciphering |  |
| Description | Apply deciphering on a given octetstring |  |
| Parameters | ciphered NAS PDU | octetstring |
|  | Ciphering Algorithm | 3 bits as defined in TS 24.301 [21], clause 9.9.3.23 |
|  | KNASenc | Ciphering Key |
|  | NAS COUNT | as documented in TS 24.301 [21] |
|  | BEARER Id | fixed value ('00000'B) acc. TS 33.401 [24], clause 8.1 |
| Return Value | deciphered octet string |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_AsIntegrityAlgorithm |  |
| Description | Apply integrity protection algorithm on a given octetstring |  |
| Parameters | PDCP PDU | octetstring |
|  | Integrity Algorithm | 3 bits as defined in TS 33.401 [24] |
|  | KRRCint | Integrity key |
|  | PDCP COUNT | octetstring, length 4 |
|  | BEARER Id | the value of the DRB identity minus one |
|  | Direction | UL: 0 DL: 1 (acc. to TS 33.401 [24], clause B.2) |
| Return Value | Message Authentication Code (4 octets) |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_AsCiphering |  |
| Description | Apply ciphering on a given octetstring |  |
| Parameters | SDU | octetstring |
|  | Ciphering Algorithm | 3 bits as defined in TS 33.401 [24] |
|  | KRRCenc | Ciphering Key |
|  | PDCP COUNT | octetstring, length 4 |
|  | BEARER Id | the value of the DRB identity minus one |
| Return Value | ciphered octet string |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_AsDeciphering |  |
| Description | Apply deciphering on a given octetstring |  |
| Parameters | ciphered SDU | octetstring |
|  | Ciphering Algorithm | 3 bits as defined in TS 33.401 [24] |
|  | KRRCenc | Ciphering Key |
|  | PDCP COUNT | octetstring, length 4 |
|  | BEARER Id | the value of the DRB identity minus one |
| Return Value | deciphered octet string |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_GetSystemTime |  |
| Description | Function to get the system time: Implementation is based on C standard library (time.h) |  |
| Parameters | p_Struct_tm (out) | p_Struct_tm returns local system time equivalent to "struct tm" as defined for C standard library (time.h or ctime): type record Struct_tm_Type { integer tm_sec, // seconds after the minute // (0..61; see NOTE) integer tm_min, // minutes after the hour (0..59) integer tm_hour, // hours since midnight (0..23) integer tm_mday, // day of the month (1..31) integer tm_mon, // months since January (0..11) integer tm_year, // years since 1900 integer tm_wday, // days since Sunday (0..6) integer tm_yday, // days since January 1 (0..365) integer tm_isdst // Daylight Saving Time flag }; NOTE: tm_sec is generally 0-59. Extra range to accommodate for leap seconds in certain systems C implementation: time_t v_Now = time(NULL); struct tm *v_Tm = localtime(&v_Now); |
|  | p_TimezoneInfo (out) | p_TimezoneInfo returns the difference (in seconds) between the UTC time (GMT) and the local time (integer value); C implementation: int timezone = (int)difftime(mktime(gmtime(&v_Now)), v_Now); NOTE: p_TimezoneInfo does not consider daylight saving e.g. it is always 3600 for CET independent of summer/winter |
| Return Value | None |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_MD5_Hex |  |
| Description | external function to calculate the MD5 Message-Digest Algorithm according to RFC 1321 |  |
| Parameters | Data | octetstring |
| Return Value | octetstring |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_CalculateFCS32 |  |
| Description | 32-bit frame check sequence (FCS) calculation according to ITU-T Recommendation V.42 [68] section 8.1.1.6.2 and TS36.304 [14] section 7.3. |  |
| Parameters | p_Input | bitstring |
| Return Value | bitstring, length 32 |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_ UDC_Initialise |  |
| Description | Initialise decompressor and decompression buffer in the SS |  |
| Parameters | bufferSize | 2 bits 00 - kbyte2 (2048 bytes) 01 - kbyte4 (4096 bytes) 10 - kbyte8 (8192 bytes) 11 - spare1 |
|  | Dictionary | 1 bit 0 - Decompression buffer is set to all zeros 1 - Decompression buffer is set to sip-SDP |
| Return Value | error_indicator: 1 bit 0 - Initialization is success 1 - Initialization is not success |  |

| TTCN-3 External Function |  |  |
| --- | --- | --- |
| Name | fx_UDC_Decompress |  |
| Description | Applies decompression of a compressed PDCP SDU. The decompression function makes use of IE FR and the content of the decompression buffer. Checksum: Implementation is calculated by the content of decompression buffer before decompression. In the case of FR is set to 1, the value of checksum is after decompression buffer reset. SS shall update the content of the decompression buffer after the decompression. |  |
| Parameters | compressed SDU | octetstring |
|  | FR | 1 bit 0 - Decompression buffer is not reset 1 - Decompression buffer is reset to all zeros |
| Return Value | bitstring, error_indicator(1 bit) + checksum(4 bits) + decompressed SDU(octetstring), error_indicator: 1 bit 0 - Decompression is success 1 - Decompression is not success decompressed SDU : octetstring The decompressed SDU is valid only when error_indicator is set to 0 |  |

# 9	IXIT Proforma

This partial IXIT proforma contained in the present document is provided for completion, when the related Abstract Test Suite is to be used against the Implementation Under Test (IUT).

Text in italics is a comment for guidance for the production of an IXIT, and is not to be included in the actual IXIT.

The completed partial IXIT will normally be used in conjunction with the completed ICS, as it adds precision to the information provided by the ICS.

## 9.1	E-UTRAN PIXIT

Table 9.1-1: Common PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_AccessPointName | octetstring |  |  | Access Point Name, as defined in TS 23.003 [48] and used in TS 24.008 [20], clause 10.5.6.1 |
| px_AttachTypeTested | EUTRA_ATTACH_TESTED_Type | EPS_ATTACH_ONLY | EPS_ATTACH_ONLY, COMBINED_ATTACH | Attach Type to be tested, if UE supports both pc_Attach and pc_Combined_Attach |
| px_eAuthRAND | B128_Type | oct2bit(‘A3DE0C6D363E30C364A4078F1BF8D577’O) |  | Random Challenge |
| px_EllipsoidPointWithAltitude | O8_Type |  |  | Ellipsoid Point With Altitude value to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_HorizontalVelocity | O3_Type |  |  | Horizontal Velocity value to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_eJapanMCC_Band6 | NAS_Mcc | ‘442’H |  | Japan MCC code to be used for Band 6. The same value will be used for E-UTRA and Inter-RAT cells. Type is different to that defined in TS 34.123-3 [7] |
| px_PWS_CB_DataPage1 | charstring |  |  | ETWS or CMAS Page 1 warning data message |
| px_PWS_CB_DataPage2 | charstring |  |  | ETWS or CMAS Page 2 warning data message |
| px_PWS_CB_DataPage3 | charstring |  |  | ETWS or CMAS Page 3 warning data message |
| px_PWS_CB_DataPage4 | charstring |  |  | ETWS or CMAS Page 4 warning data message |
| px_PWS_CB_DataCodingScheme | bitstring |  |  | ETWS or CMAS data coding scheme of the alphabet/coding and the applied language [see TS 23.041] |
| px_IPv4_Address1_UE | charstring |  |  | IPv4 Address connected to PDN1 |
| px_IPv4_Address2_UE | charstring |  |  | IPv4 Address connected to PDN2 |
| px_IPv4_Address2a_UE | charstring |  |  | IPv4 Address connected to PDN2a |
| px_IPv4_Address3_UE | charstring |  |  | IPv4 Address connected to PDN3 |
| px_IPv4_Address1_NW | charstring |  |  | IPv4 Gateway Address in PDN1 |
| px_IPv4_Address2_NW | charstring |  |  | IPv4 Gateway Address in PDN2 |
| px_IPv4_Address3_NW | charstring |  |  | IPv4 Gateway Address in PDN3 |
| px_IPv4_Address1_RemoteUE | charstring |  |  | IPv4 Address of remote UE connected to PDN1 |
| px_IPv4_Address2_RemoteUE | charstring |  |  | IPv4 Address of remote UE connected to PDN2 |
| px_IPv4_Address3_RemoteUE | charstring |  |  | IPv4 Address of remote UE connected to PDN3 |
| px_IPv4_Address_HomeAgent | charstring |  |  | IPv4 Home Agent Address |
| px_IPv6_Address1_UE | charstring |  |  | IPv6 Address connected to PDN1 |
| px_IPv6_Address2_UE | charstring |  |  | IPv6 Address connected to PDN2 |
| px_IPv6_Address2a_UE | charstring |  |  | IPv6 Address connected to PDN2a |
| px_IPv6_Address3_UE | charstring |  |  | IPv6 Address connected to PDN3 |
| px_IPv6_Address1_NW | charstring |  |  | IPv6 Gateway Address in PDN1 |
| px_IPv6_Address2_NW | charstring |  |  | IPv6 Gateway Address in PDN2 |
| px_IPv6_Address3_NW | charstring |  |  | IPv6 Gateway Address in PDN3 |
| px_IPv6_Address1_RemoteUE | charstring |  |  | IPv6 Address of remote UE connected to PDN1 |
| px_IPv6_Address2_RemoteUE | charstring |  |  | IPv6 Address of remote UE connected to PDN2 |
| px_IPv6_Address3_RemoteUE | charstring |  |  | IPv6 Address of remote UE connected to PDN3 |
| px_IPv6_Address_HomeAgent | charstring |  |  | IPv6 Home Agent Address |
| px_ePrimaryFrequencyBand | integer | 1 |  | E-UTRA primary frequency band |
| px_SMS_ChkMsgReceived | boolean | true |  | Whether the operator can check an MT Short Message received |
| px_RATComb_Tested | RATComb_Tested_Type | EUTRA_UTRA | EUTRA_UTRA, EUTRA_GERAN, EUTRA_Only | This parameter represents the network RAT capability / preference and indicates which, if any is supported, RAT combination is to be tested. |
| px_SinglePLMN_Tested | SinglePLMN_Tested_Type | MultiPLMN | SinglePLMN, MultiPLMN, MultiPLMNinPrimaryBand, MultiPLMNinSecondaryBand | This parameter represents the network capability/preference to support multi PLMNs on the same test Band and indicates the preference of multi PLMNs or single PLMN test environment. This PIXIT shall be set to SinglePLMN when only one frequency is defined for the test band. |
| px_UE_CS_PS_UsageSetting_Tested | CS_PS_MODE | VOICE_CENTRIC | VOICE_CENTRIC, DATA_CENTRIC | Specifies which CS/PS mode is under test |
| px_UE_PS_UsageSetting_Tested | PS_MODE | VOICE_CENTRIC | VOICE_CENTRIC, DATA_CENTRIC | Specifies which PS mode is under test |
| px_UTRAN_ModeUnderTest | UTRAN_FDD_TDD | UTRAN_FDD | UTRAN_FDD, UTRAN_TDD | Specifies which radio access technology is being tested in UTRAN |
| px_TestLoopModeB_Delay | O1_Type | ‘5A’ |  | This parameter represents the IP_PDU_delay to be used for UE test loop mode B in test cases, where long delay may be needed e.g. because of user interaction. |
| px_IP_MTU_Size | integer | 65535 |  | MTU Size. This value is specific to the SS. |
| px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI | integer |  |  | A not supported UTRAN frequency band that is overlapping with a supported band (px_MFBI_UTRAN_FrequencyBand). This PIXIT shall be set also in accordance with the values of the 3 PIXIT items: px_UARFCN_D_High, px_UARFCN_D_Mid,px_UARFCN_L_Mid. Applied to MFBI test case scenario. |
| px_MFBI_UTRAN_FrequencyBand | integer |  |  | A supported UTRAN frequency band overlapping with px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI. Applied to MFBI test case scenario. |
| px_WLAN_PDN1_ToOffload | boolean | true |  | If set to true, the first PDN connection is offloaded to WLAN. If set to false, the second PDN connection is offloaded to WLAN. |
| px_AquireGNSS_MaxTime | integer | 60 |  | The maximum time (in seconds) taken for the UE to aquire GNSS and derive UTC time (and normally position) and then start to transmit or receive as appropriate |
| px_MobileDataOn | boolean | true |  | Specifes whether mobile data, associated with Internet PDN, is enabled on the DUT. |

Table 9.1-2: E-UTRAN PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_eTDDsubframeConfig | TDD_SubframeAssignment_Type | 1 |  | TDD uplink-downlink subframe configuration |
| px_ePrimaryBandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | Channel bandwidth used on px_ePrimaryFrequencyBand |
| px_eSecondaryBandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | Channel bandwidth used on px_eSecondaryFrequencyBand |
| px_EUTRA_CA_BandCombination | CA_BandCombination_Type | CA_1C |  | Band combination for CA test cases |
| px_EUTRA_DC_BandCombination | DC_BandCombination_Type | DC_1A-3A |  | Band combination for DC test cases |
| px_eMaxNumberROHC_ContextSessions | MaxNumberROHC_ContextSessions_Type | Cs16 |  | Maximum number of ROHC context sessions |
| px_MFBI_FrequencyBand | integer | 26 |  | A supported E-UTRA frequency band overlapping with px_OverlappingNotSupportedFrequencyBand_MFBI. Applied to MFBI test case scenario. |
| px_MFBI_BandChannelBandwidth | Dl_Bandwidth_Type | n25 |  | E-UTRAN channel bandwidth used on px_MFBI_FrequencyBand. Applied to MFBI test case scenario. |
| px_OverlappingNotSupportedFrequencyBand_MFBI | integer | 27 |  | A not supported E-UTRA frequency band that is overlapping with a supported band (px_MFBI_FrequencyBand). Applied to MFBI test case scenario. |
| px_OverlappingSupportedFrequencyBand_MFBI | integer | 26 |  | A supported E-UTRA frequency band overlapping with px_OverlappingNotSupportedFrequencyBand_MFBI. Applied to MFBI enhancement test case scenario. |
| px_NoTDD_LCR_onLTE | boolean | false |  | Only applicable to UE supporting (at least) LTE and LCR TDD. If set to true, the UE is configured not to report support of LCR TDD (IE utraTDD128 in UECapabilityInformation) when being camped on an LTE cell with no detectable LCR TDD cell in the environment |

Table 9.1-3: D2D PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_EllipsoidPointWithAltitude_ProSe_Area1 | O8_Type |  |  | Ellipsoid Point With Altitude value corresponding to geographical area 1 provided in EFPROSE_RADIO_COM to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_EllipsoidPointWithAltitude_ProSe_Area2 | O8_Type |  |  | Ellipsoid Point With Altitude value corresponding to geographical area 2 provided in EFPROSE_RADIO_COM to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_EllipsoidPointWithAltitude_NotProSe | O8_Type |  |  | Ellipsoid Point With Altitude value not corresponding to geographical area provided in EFPROSE_RADIO_COM to be provided in Update UE location information (see 36.509 cl. 6.12) |
| px_ProSeAnnApplicationIdentity1 | charstring | mcc345.mnc012.ProSeApp.Food.Restaurants.Italian |  | A ProSe Application ID with which data structure the UE is preconfigured and for which the UE can request and perform Direct Discovery Announcing. TS 23.003, section 24.2. |
| px_ProSeAnnApplicationIdentity2 | charstring | mcc345.mnc012.ProSeApp.mcptt |  | An MCPTT relevant ProSe Application ID with which data structure the UE is preconfigured and for which the UE can request and perform Group Member Discovery Announcement. TS 23.003, section 24.2. |
| px_ProSeMonApplicationIdentity1 | charstring | mcc300.mnc165.ProSeApp.*.Sports.Surfing |  | A ProSe Application ID with which data structure the UE is preconfigured and for which the UE can request and perform Direct Discovery Monitoring. TS 23.003, section 24.2 |
| px_ProSeMonApplicationIdentityNot1 | charstring | mcc208.mnc*.ProSeApp.Shops.Food.Wine |  | A ProSe Application ID with which data structure the UE is NOT preconfigured to request and perform Direct Discovery Monitoring. TS 23.003, section 24.2 |
| px_ProSe_TLS_CipherSuite | TLS_CIPHER_Type | TLS_PSK_WITH_3DES_EDE_CBC_SHA |  | TLS cipher suite to be used for ProSe communication |
| px_SL_AdditionalSupportedBandwidth | Ul_Bandwidth_Type | n25 |  | Supported sidelink bandwidth different from px_ePrimaryBandChannelBandwidth |

Table 9.1-4: EUTRA_NB PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_eSecondaryFrequencyBand | integer | 2 |  | E-UTRA secondary frequency band |
| px_NAS_CipheringAlgorithm | B3_Type | 001’B |  | NAS Ciphering Algorithm (eea1, see NOTE) |
| px_NAS_IntegrityProtAlgorithm | B3_Type | 001’B |  | NAS Integrity Algorithm (eia1, see NOTE) |
| px_RRC_CipheringAlgorithm | CipheringAlgorithm_r12 | eea1 |  | Ciphering Algorithm (see NOTE) |
| px_RRC_IntegrityProtAlgorithm | IntegrityProtAlgorithm | eia1 |  | Integrity Algorithm (see NOTE) |
| px_SMSTransport_CP_CIoT | boolean | false |  | UE configured to utilise the SMS service as transport mechanism for user data for Control Plane CIoT Optimisation |
| px_nonSMSTransport_CP_CIoT | boolean | false |  | UE configured to utilise non-SMS services as transport mechanism for user data for Control Plane CIoT Optimisation |
| NOTE: Unless specified otherwise in the test case prose, the null algorithm shall not be used for verification. |  |  |  |  |

## 9.2	MultiRAT PIXIT

Table 9.2-1: GERAN PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_GERAN_BandUnderTest | GERAN_BandUnderTestType | GSM_P900 |  | Indicates which band is under test |

Table 9.2-2: UTRAN PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_UTRAN_CipheringAlgorithm | CipheringAlgorithm_r7 | uea2 | uea0, uea1, uea2 | UTRAN Ciphering algorithm |
| px_UARFCN_Low_TDD | integer |  |  | Low Range downlink UARFCN value for LCR TDD |
| px_UARFCN_Mid_TDD | integer |  |  | Mid Range downlink UARFCN value for LCR TDD |
| px_UARFCN_High_TDD | integer |  |  | High Range downlink UARFCN value for LCR TDD |
| px_TDD_OperationBandCharstring | charstring | a | px_UARFCN_Mid_TDD, px_UARFCN_Low_TDD and px_UARFCN_High_TDD shall take the values according to the value of px_TDD_OperationBandCharstring | Applicable for TDD The operation band under test as defined in 34.108 clause 5.1.2 |
| px_ReportUTRA_RAT | UTRA_ToReport_Type | Both | FDD, TDD, Both | Only applicable to UE supporting (at least) LTE and UTRA. If set to FDD, the UE is configured to report support UTRA FDD only. If set to TDD, the UE is configured to report support UTRA TDD only. If set to Both, the UE is configured to report both UTRA RATs. |

Table 9.2-3: CDMA2000 HRPD PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_HRPD_BandClass | BandclassCDMA2000_Type | 1 |  | Band Class; Table 1.5-1 of C.S0057-E v1.0 Default value corresponds to 1.8 to 2.0 GHz PCS band |
| px_HRPD_SectorID_Cell15 | SectorID_HRPD_Type | oct2bit('FEA00000000000000000000000000001'O) |  | Sector ID of Cell 15; Clause 13.9 of C.S0024-C v2.0 |
| px_HRPD_SectorID_Cell16 | SectorID_HRPD_Type | oct2bit('FEA00000000000000000000000000002'O) |  | Sector ID of Cell 16; Clause 13.9 of C.S0024-C v2.0 |
| px_HRPD_SectorID_Cell17 | SectorID_HRPD_Type | oct2bit('FEA00000000000000000000000000003'O) |  | Sector ID of Cell 17; Clause 13.9 of C.S0024-C v2.0 |
| px_HRPD_SectorID_Cell18 | SectorID_HRPD_Type | oct2bit('FEA00000000000000000000000000004'O) |  | Sector ID of Cell 18; Clause 13.9 of C.S0024-C v2.0 |
| px_ColorCode | ColorCode_Type | 64 |  | Colour code of the subnet to which the sectors belong; Same for all HRPD cells |
| px_ColorCodeDiff | B32_Type | 128 |  | Colour code of the subnet to which the sectors belong; Adifferent colour code than default |
| px_OpenLoopAdjust | OpenLoopAdjust_Type | 10 |  | The value of open loop adjust to be used by access terminals in the open loop power estimate, expressed as an unsigned value in units of 1 dB. The value used by the access terminal is -1 times the value of this field |
| px_UATI24 | O3_Type | '123456'O |  | UATI to be allocated to the UE, clause 6.3.7.2.2 of C.S0024-C v2.0 |
| px_MACIndex | integer | 15 |  | ReverseLinkMACIndex to be used. Allowed values 0...383 C.S0024-C v2.0 clause 12.4.1.3.2.2 |

Table 9.2-4: CDMA2000 1xRTT PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_1XRTT_BaseId_Cell19 | B16_Type | int2bit (39,16) |  | Base ID of Cell 19 |
| px_1XRTT_BaseId_Cell20 | B16_Type | int2bit (40,16) |  | Base ID of Cell 20 |
| px_1XRTT_BaseId_Cell21 | B16_Type | int2bit (41,16) |  | Base ID of Cell 21 |
| px_1XRTT_BaseId_Cell22 | B16_Type | int2bit (42,16) |  | Base ID of Cell 22 |
| px_1XRTT_NID | B16_Type | int2bit (100,16) |  | default Network ID of 1xRTT Cells |
| px_1XRTT_SID | B15_Type | int2bit (200,15) |  | default SystemID of 1xRTT Cells |
| px_1XRTT_TMSI_Def | O4_Type | '1234ABCD'O |  | TMSI to be used in 1XRTT |
| px_1XRTT_MinProtRev | ProtRev_Type | 0 |  | Minimum Protocol revision supported by Base Station |
| px_1XRTT_UserInfo_EncMode | EncryptionMode_Type | 2 |  | Encryption Mode Rijndael algorithm |
| px_1XRTT_Sig_EncMode | EncryptionMode_Type | 2 |  | Encryption Mode Rijndael algorithm |
| px_1XRTT_BandClass | BandclassCDMA2000_Type | 1 |  | Band Class; Table 1.5-1 of C.S0057 E v1.0. Default value corresponds to 1.8 GHz to 2.0 GHz PCS band |
| px_PowerDownRegEnabled | boolean | true |  | Parameter for power down reg in 1xRTT |
| px_1XRTT_Zone_Timer | B3_Type | ‘000’B |  | Zone timer sent in ‘System Parameters Message’ overhead message |
| px_RAND | B32_Type | '00001111000011110000111100001111'B |  | Random Challenge Data to be included along with mobility parameters in CSFBParametersResponseCDMA2000 or HandoverFromEUTRAPreparationRequest |
| px_RAND2 | B32_Type | '00001111000011110000111100001111'B |  | Random Challenge Data to be included along with mobility parameters in CSFBParametersResponseCDMA2000 or HandoverFromEUTRAPreparationRequest |

Table 9.2-5: WLAN PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_ePDG_Ipsec_CipheringAlgorithm | IPsec_CipheringAlgorithm_Type | des_ede3_cbc | des_ede3_cbc, aes_cbc, nociph | Ipsec Ciphering Algorithm used in ePDG NOTE: Unless specified otherwise in the test prose „nociph“ shall not be used for verification |
| px_ePDG_Ipsec_IntegrityAlgorithm | IPsec_IntegrityAlgorithm_Type | hmac_sha_1_96 | hmac_md5_96, hmac_sha_1_96 | Ipsec Integrity Algorithm used in ePDG |
| px_WLANBand | WLAN_Band_Type | Band2_4Ghz | Band2_4Ghz, Band5Ghz, Band3_6Ghz | WLAN ISM Band |
| px_WLAN_RulesToBeTested | WLAN_RulesToBeTested | "ANDSF_Rules" | "ANDSF_Rules", "RAN_Rules" | Rules to be tested in WLAN test cases |
| px_WLAN_UE_AddrIPv4 | charstring |  |  | IPv4 WLAN UE Address |
| px_WLAN_UE_AddrIPv6 | charstring |  |  | IPv6 WLAN UE Address |
| px_WLAN_NetworkAddrIPv4 | charstring |  |  | IPv4 WLAN Network Address |
| px_WLAN_NetworkAddrIPv6 | charstring |  |  | IPv6 WLAN Network Address |
| px_ePDG_OperatorProvisionedFQDN | charstring |  |  | domain name of the ePDG to be considered to assign the IP addresses of the ePDG in responses to DNS queries |
| px_WLAN_BeaconRSSI | WLAN_BeaconRSSI_Type | 78 (-50dBm) | 0-255 | Beacon RSSI value for IMS over WLAN test cases |

## 9.3	NB-IoT PIXIT

Table 9.3-1: NB-IoT PIXIT

| Parameter Name | Parameter Type | Default Value | Supported Values | Description |
| --- | --- | --- | --- | --- |
| px_NB_OperationMode | NB_OperationMode_Type | standalone | inband_samePCI, inband_differentPCI, guardband, standalone | NB-IoT Operation Mode |
| px_DoAttachWithoutPDN | boolean | false |  | UE is configured to do Attach without PDN; if the value is FALSE it is assumed that the UE is configured to do Attach with PDN |
| px_ModifyBearerResources | boolean | false |  | NB-IoT UE configured to handle modification of bearer resources including dedicated bearers (PDN of type "IP") |
| px_NS_Value | integer | 1 |  | Network Signalling value as defined in TS 36.101 [34] clause 6.2.4F and used in TS 36.508 [3] clause 8.1.3.1 to select the NB-IoT frequencies. |

# 10	Postambles

The purpose of this clause is to specify postambles to bring the UE to a well defined state regardless of the UE state at the termination of main test body or of the SS conditions and values of the system information inherited from the test.

## 10.1	Postambles for E-UTRA to UTRA tests

This clause describes UE postamble procedures which are used at the end of inter-RAT test cases specified in TS 36.508 [3] so as to switch off the UE.

UE LTE and UTRAN postamble conditions are specified in table 10.1-1.

Table 10.1-1: UE postamble conditions

| LTE UE attach type | UE UTRA CS/PS domain | Postamble condition |
| --- | --- | --- |
| attach | pc_CS AND pc_PS | C1 |
|  | pc_PS AND NOT (pc_CS) | C2 |
| combined_attach | pc_CS AND pc_PS | C3 |
|  | pc_CS AND NOT (pc_PS) | C4 |

### 10.1.1	UE postamble states and procedures for E-UTRA to UTRA

In order to bring the UE to the switched/powered off state, a number of procedures need to be executed in a hierarchical sequence, according to the reference end state specified in each test procedure sequence. The sequences and the identified procedures are shown in figure 10.1.1-1.

Figure 10.1.1-1: UE postamble procedures for E-UTRA / UTRA test cases

NOTE 1:	Depending on the test case specifications the termination of a test case can be in any state of figure 10.1.1-1.

NOTE 2:	NAS and AS security procedures during routing area update and handover are performed according to TS 33.401 [24], clauses 9.1.1 and 9.2.1 and TS 25.331 [36], clause 8.3.6.3.

UE in UTRA state U2, U3, U4 and U5 may send data on the established radio bearer and shall be accepted and handled.

### 10.1.2	Switch/Power off procedure

#### 10.1.2.1	Procedure

Table 10.1.2.1-1: Switch/Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 0A | The SS starts Timer_1 = 2 seconds |  |  |
| - | EXCEPTION: Steps 0B – 0Bb1 describe behaviour that depends on the UE implementation | - | - |
| 0Ba1 – 0Ba10 | The steps defined in Table 10.1.2.1-2 take place. Note The SS cancels the Timer_1 at step 0Ba1 |  |  |
| 0Bb1 | The SS waits for Timer_1 expiry |  |  |
| 1 | The UE is powered off or switched off, (see ICS) | - | - |
| - | EXCEPTION: Steps 1A to 7 specify the behaviour if UE supports pc_SwitchOnOff. | - | - |
| - | EXCEPTION: Step 1A depends on the UE implementation: if the steps 0Ba1 – 0Ba10 did not take place |  |  |
| 1A | the steps defined in Table 10.1.2.1-2 may take place |  |  |
| - | EXCEPTION: Steps 2 to 4 are used only when the UE is in UTRA idle end state (U1) and steps 0Ba1 – 0Ba10 or step 1A did not take place. |  |  |
| 2 | The UE transmits RRC CONNECTION REQUEST | --> | RRC CONNECTION REQUEST |
| 3 | The SS transmit a RRC CONNECTION SETUP | <-- | RRC CONNECTION SETUP |
| 4 | The UE transmits an RRC CONNECTION SETUP COMPLETE message | --> | RRC CONNECTION SETUP COMPLETE |
| 4Aa1 – 4Aa6 | Void | - | - |
| - | EXCEPTION: Step 4Aa7a1 to 4Aa7a2 specifies optional behaviour depending on the UE implementation. The UE may perform PDP context deactivation procedure before Detach. |  |  |
| 4Aa7a1 | The UE transmits DEACTIVATE PDP CONTEXT REQUEST | --> | DEACTIVATE PDP CONTEXT REQUEST |
| 4Aa7a2 | The SS transmits DEACTIVATE PDP CONTEXT ACCEPT | <-- | DEACTIVATE PDP CONTEXT ACCEPT |
| - | EXCEPTION: Step 5a1 specifies behaviour when the current UTRA cell is in NMO I and the UE is in condition: - C1 or - C3 | - | - |
| 5a1 | The UE transmits an UPLINK DIRECT TRANSFER message or INITIAL DIRECT TRANSFER message when the UE is in UTRA idle end state (U1). This message includes a DETACH REQUEST message with the detach type=‘power switched off, GPRS/IMSI combined detach' | --> | DETACH REQUEST |
| - | EXCEPTION: Step 5b1 specifies behaviour when the current UTRA cell is in (NMO I or NMO II) and the UE is in condition C4 | - | - |
| 5b1 | The UE transmits an UPLINK DIRECT TRANSFER message or INITIAL DIRECT TRANSFER message when the UE is in UTRA idle end state (U1). This message includes an IMSI DETACH INDICATION message | --> | IMSI DETACH INDICATION |
| - | EXCEPTION: Step 5c1 specifies behaviour when the current UTRA cell is in (NMO I or NMO II) and the UE is in condition C2 | - | - |
| 5c1 | The UE transmits an UPLINK DIRECT TRANSFER message or INITIAL DIRECT TRANSFER message when the UE is in UTRA idle end state (U1). This message includes a DETACH REQUEST message with detach type=‘power switched off, PS detach'' | --> | DETACH REQUEST |
| - | EXCEPTION: Steps 5d1 and 5d2 specify behaviour when the current UTRA cell is in NMO II and the UE is in condition: - C1 or - C3. Both detach messages (in steps 5d1 and 5d2) can be sent by UE in any order. | - | - |
| 5d1 | The UE transmits an UPLINK DIRECT TRANSFER message or INITIAL DIRECT TRANSFER message when the UE is in UTRA idle end state (U1) and this is the first message received. This message includes a DETACH REQUEST message with the detach type=‘power switched off, PS detach'' | --> | DETACH REQUEST |
| 5d2 | The UE transmits an UPLINK DIRECT TRANSFER message or INITIAL DIRECT TRANSFER message when the UE is in UTRA idle end state (U1) and this is the first message received. This message includes an IMSI DETACH INDICATION message | --> | IMSI DETACH INDICATION |
| 6 | The SS transmits an RRC CONNECTION RELEASE message | <-- | RRC CONNECTION RELEASE |
| 7 | The UE transmits a RRC CONNECTION RELEASE COMPLETE message | --> | RRC CONNECTION RELEASE COMPLETE |

Table 10.1.2.1-2: Optional IMS Deregistration

| - | EXCEPTION: Steps 1 to 3 are used only when the UE is in UTRA idle end state (U1). |  |  |
| --- | --- | --- | --- |
| 1 | The UE transmits RRC CONNECTION REQUEST | --> | RRC CONNECTION REQUEST |
| 2 | The SS transmit a RRC CONNECTION SETUP | <-- | RRC CONNECTION SETUP |
| 3 | The UE transmits an RRC CONNECTION SETUP COMPLETE message | --> | RRC CONNECTION SETUP COMPLETE |
| - | EXCEPTION: Step 4a1 to 4a6 specifies optional behaviour if the UE is registered to IMS services. UE may perform IMS-deregistration procedure. |  |  |
| 4a1 | The UE transmits UPLINK DIRECT TRANSFER message or INITIAL DIRECT TRANSFER message when the UE is in UTRA idle end state (U1). This message includes a GMM SERVICE REQUEST message with service type=Data. | --> | GMM SERVICE REQUEST |
| 4a2 | The SS transmits a SECURITY MODE COMMAND message for the ps domain. |  | SECURITY MODE COMMAND |
| 4a3 | The UE transmits a SECURITY MODE COMPLETE message. |  | SECURITY MODE COMPLETE |
| 4a4 | The SS transmits a RADIO BEARER SETUP message using the UTRA reference radio bearer parameters and combination “UTRA PS RB” according to TS 36.508 [3] clause 4.8.3 and Table 4.8.3-1. |  | RADIO BEARER SETUP |
| 4a5 | The UE transmits a RADIO BEARER SETUP COMPLETE |  | RADIO BEARER SETUP COMPLETE |
| 4a6 | IMS de-registration is performed using the generic procedure defined in TS 34.229-1 [40] Annex C.30. |  |  |

### 10.1.3	CC disconnect procedure

#### 10.1.3.1	Procedure

Table 10.1.3.1-1: CC disconnect procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a DISCONNECT message. | <-- | DISCONNECT |
| 2 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a RELEASE message. | --> | RELEASE |
| 3 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a RELEASE COMPLETE message. | <-- | RELEASE COMPLETE |
| 4 | Void |  |  |
| 5a | The SS transmits an RRC CONNECTION RELEASE message | <-- | RRC CONNECTION RELEASE |
| 5b | The UE transmits a RRC CONNECTION RELEASE COMPLETE message | --> | RRC CONNECTION RELEASE COMPLETE |
| 5c | The UE transmits RRC CONNECTION REQUEST | --> | RRC CONNECTION REQUEST |
| 5d | The SS transmit a RRC CONNECTION SETUP | <-- | RRC CONNECTION SETUP |
| 5e | The UE transmits an RRC CONNECTION SETUP COMPLETE message | --> | RRC CONNECTION SETUP COMPLETE |
| 6 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE REQUEST message with Update type ='Combined RA/LA Updated' | --> | ROUTING AREA UPDATE REQUEST |
| 7 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 8 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |

### 10.1.4	PS Routing Area Update procedure

#### 10.1.4.1	Procedure

Table 10.1.4.1-1: PS Routing Area Update procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: steps 1a1 to 1a5 specify the UE behaviour when the current UTRA cell is in NMO I and the UE is in condition: - C1 or - C3 and the UE is not registered to the LAC of the current UTRA cell | - | - |
| 1a1 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE REQUEST message with Update type ='Combined RA/LA Updated' | --> | ROUTING AREA UPDATE REQUEST |
| 1a2 | Void | - | - |
| 1a3 | Void | - | - |
| 1a4 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 1a5 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |
| - | EXCEPTION: steps 1b1 to 1b5 specify the UE behaviour when the current UTRA cell is in (NMO I or NMO II) and the UE is in condition: - C2 or - C3 and the UE is registered to the LAC of the current UTRA cell | - | - |
| 1b1 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE REQUEST message with Update type ='RA Update' | --> | ROUTING AREA UPDATE REQUEST |
| 1b2 | Void | - | - |
| 1b3 | Void | - | - |
| 1b4 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 1b5 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |
| - | EXCEPTION: steps 1c1 to 1c9 specify the UE behaviour when the current UTRA cell is in NMO II and the UE is in condition: - C1 or - C3 and the UE is not registered to the LAC of the current UTRA cell. The LOCATION UPDATE REQUEST message (step 1c6) can be received during the routing area updating procedure (steps 1c1 to 1c4). | - | - |
| 1c1 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE REQUEST message with Update type ='RA Update'. | --> | ROUTING AREA UPDATE REQUEST |
| 1c2 | Void | - | - |
| 1c3 | Void | - | - |
| 1c4 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 1c5 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |
| 1c6 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST |
| 1c7 | The SS transmits a SECURITY MODE COMMAND message. | <-- | SECURITY MODE COMMAND |
| 1c8 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE |
| 1c9 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a LOCATION UPDATING ACCEPT | <-- | LOCATION UPDATING ACCEPT |
| 1c10 | The EU transmits a UPLINK DIRECT TRANSFER message. This message includes a TMSI REALLOCATION COMPLETE | --> | TMSI REALLOCATION COMPLETE |

### 10.1.5	CS fallback procedure

#### 10.1.5.1	Procedure

Table 10.1.5.1-1: CS fallback procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: In parallel to the events described in step 1a1 to 2a5 the steps specified in table 10.1.5.1-2 takes place. | - | - |
| - | EXCEPTION: Steps 1a1 and 1a2 specify the MO call procedure and step 1b1 specifies the MT call procedure. | - | - |
| 1a1 | The UE transmits an INITIAL DIRECT TRANSFER message including a CM SERVICE REQUEST message. | --> | CM SERVICE REQUEST |
| 1a2 | The SS transmits an UPLINK DIRECT TRNASFER message including a CM SERVICE REJECT with the reject cause #32 (Service option not supported) | <-- | CM SERVICE REJECT |
| - | EXCEPTION: Step 1b1 specifies the MT call procedure. | - | - |
| 1b1 | The UE transmits an INITIAL DIRECT TRANSFER message including a PAGING RESPONSE message. | --> | PAGING RESPONSE |
| - | EXCEPTION: Steps 2a1 and 2a5 specify the location area update procedure when the current UTRA cell is in NMO II and the UE is in condition C3 and the UE is not registered to the LAC of the current UTRA cell. | - | - |
| 2a1 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST |
| 2a2 | The SS transmits a SECURITY MODE COMMAND message. | <-- | SECURITY MODE COMMAND |
| 2a3 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE |
| 2a4 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a LOCATION UPDATING ACCEPT | <-- | LOCATION UPDATING ACCEPT |
| 2a5 | The EU transmits a UPLINK DIRECT TRANSFER message. This message includes a TMSI REALLOCATION COMPLETE | --> | TMSI REALLOCATION COMPLETE |
| 3 | The SS transmits a SECURITY MODE COMMAND message. | <-- | SECURITY MODE COMMAND |
| 4 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE |
| 5 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 6 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |
| - | EXCEPTION: Steps 7a1 and 7a5 specify the combined routing updating procedure when the initial RAU procedure was for RA Only. This may occur either before, or after, steps 8 and 9 | - | - |
| 7a1 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST |
| 7a2 | The SS transmits a SECURITY MODE COMMAND message. | <-- | SECURITY MODE COMMAND |
| 7a3 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE |
| 7a4 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 7a5 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |
| - | EXCEPTION: Steps 7b1 and 7b4 specify the location updating procedure when the current UTRA cell is in network mode (NMO I or NMO II) and the UE is in condition C4 and the UE is not registered to the LAC of the current UTRA cell. | - | - |
| 7b1 | The UE transmits an UPLINK DIRECT TRANSFER message. This message includes a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST |
| 7b2 | The SS transmits a SECURITY MODE COMMAND message. | <-- | SECURITY MODE COMMAND |
| 7b3 | The UE transmits a SECURITY MODE COMPLETE message. | --> | SECURITY MODE COMPLETE |
| 7b4 | The SS transmits a DOWNLINK DIRECT TRANSFER message. This message includes a LOCATION UPDATING ACCEPT | <-- | LOCATION UPDATING ACCEPT |
| 7b5 | The EU transmits a UPLINK DIRECT TRANSFER message. This message includes a TMSI REALLOCATION COMPLETE | --> | TMSI REALLOCATION COMPLETE |
| 8 | The SS transmits an RRC CONNECTION RELEASE message. | <-- | RRC CONNECTION RELEASE |
| 9 | The UE transmits an RRC CONNECTION RELEASE COMPLETE message. | --> | RRC CONNECTION RELEASE COMPLETE |

Table 10.1.5.1-2: Parallel behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST | - | - |

## 10.2	Postambles for E-UTRAN to GERAN tests

This clause describes UE postamble procedures which are used at the end of inter-RAT test cases defined in TS 36.508 [3] so as to switch off the UE. UE LTE and GERAN postamble transitions are specified in table 10.2-1.

Table 10.2-1: UE postamble conditions

| LTE UE attach type | UE GERAN CS/PS domain | Postamble condition |  |
| --- | --- | --- | --- |
| attach | pc_GPRS | C1 |  |
| combined attach | pc_GPRS |  | C2 |
|  | NOT pc_GPRS |  | C3 |

### 10.2.1	UE postamble states and procedures for E-UTRA to GERAN test cases

In order to bring the UE to the switched/powered off state there are a number of procedures that need to be executed in a hierarchical sequence, according to the reference end state specified in each test procedure sequence. The sequences and the identified procedures are shown in figure 10.2.1-1.

Figure 10.2.1-1: UE postamble procedures for E-UTRA / GERAN test cases

NOTE 1:	Depending on the test case specifications the termination of a test case can be in any state of figure 10.2.1-1.

NOTE 2:	The security procedures for interworking to GERAN are according to TS 33.401 [24] clauses 10.2.1 and 10.3.1.

### 10.2.2	Switch/Power off procedure

#### 10.2.2.1	Procedure

Table 10.2.2.1-1: Switch/Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is powered off or switched off, (see ICS) | - | - |
| - | EXCEPTION: Steps 2a1 to 2c2 specify the behaviour if UE supports pc_SwitchOnOff. | - | - |
| - | EXCEPTION: Step 2a1 specifies behaviour when the GERAN cell is in (NMO I or NMO II) and UE is in condition C1 | - | - |
| 2a1 | The UE transmits a DETACH REQUEST message | --> | DETACH REQUEST |
| - | EXCEPTION: Step 2b1 specifies behaviour when the GERAN cell is in (NMO I or NMO II) and UE is in condition C3 | - | - |
| 2b1 | The UE transmits an IMSI DETACH INDICATION message | --> | IMSI DETACH INDICATION |
| - | EXCEPTION: Steps 2c1 – 2c1A3 and step 2c2 specify behaviour when the GERAN cell is in NMO II and UE is in condition C2. The messages can be sent in any order | - | - |
| 2c1 | The UE transmits an IMSI DETACH INDICATION message | --> | IMSI DETACH INDICATION |
| 2c1A1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE |
| - | EXCEPTION: Steps 2c1A2 and 2cA3 can occur in any order | - | - |
| 2c1A2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. |
| 2c1A3 | IF UE is still attached for GPRS (step 2c2 has not yet occurred) THEN the UE transmits a GPRS Suspension Request message | --> | GPRS SUSPENSION REQUEST |
| 2c2 | The UE transmits a DETACH REQUEST message | --> | DETACH REQUEST |

### 10.2.3	PS Handover procedure

#### 10.2.3.1	Procedure

Table 10.2.3.1-1: PS handover procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 and 1a3 specify the UE behaviour when GERAN cell is in NMO I and the UE is in condition C2 and the UE is not registered to the LAC of this cell. | - | - |
| 1a1 | The UE transmits a ROUTING AREA UPDATE REQUEST message with update type='Combined RA/LA Update'. | --> | ROUTING AREA UPDATE REQUEST |
| 1a2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 1a3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |
| - | EXCEPTION: Steps 1b1 and 1b3 specify the location updating procedure when GERAN cell is in (NMO I or NMO II) and the UE is in condition C2 and the UE is registered to the LAC of this cell. | - | - |
| 1b1 | The UE transmits a ROUTING AREA UPDATE REQUEST message with update type='RA Update'. | --> | ROUTING AREA UPDATE REQUEST |
| 1b2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 1b3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |
| - | EXCEPTION: Steps 1c1 and 1c6 specify the location updating procedure when GERAN cell is in NMO II and the UE is in condition C2 and the UE is not registered to the LAC of this cell. | - | - |
| 1c1 | The UE transmits a ROUTING AREA UPDATE REQUEST message with update type='RA Update'. | --> | ROUTING AREA UPDATE REQUEST |
| 1c2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 1c3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |
| 1c4 | The UE transmits a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST |
| 1c4A1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE |
| - | EXCEPTION: The next step describes behaviour that depends on UE capability. | - | - |
| 1c4A2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. |
| 1c5 | The SS transmits a LOCATION UPDATING ACCEPT | <-- | LOCATION UPDATING ACCEPT |
| 1c6 | The UE transmits a TMSI REALLOCATION COMPLETE |  | TMSI REALLOCATION COMPLETE |

### 10.2.4	CC disconnect procedure

#### 10.2.4.1	Procedure

Table 10.2.4.1-1: CC disconnect procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 0A | SS Starts 5s timer |  |  |
| 0Ba1 | The UE transmits a GPRS SUSPENSION REQUEST message | --> | GPRS SUSPENSION REQUEST |
| 0Ba2 | Stop 5s timer |  |  |
| 0Bb1 | The 5s timer expires |  |  |
| 1 | The SS transmits a DISCONNECT message. | <-- | DISCONNECT |
| 2 | The UE transmits a RELEASE message. | --> | RELEASE |
| 3 | The SS transmits a RELEASE COMPLETE message. | <-- | RELEASE COMPLETE |
| 4 | The SS transmits a CHANNEL RELEASE message. | <-- | CHANNEL RELEASE |

### 10.2.5	CS fallback procedure

#### 10.2.5.1	Procedure

Table 10.2.5.1-1: CS fallback procedure MO call

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: Steps 1a1 and 1a2 specify the MO call procedure. | - | - |
| 1a1 | The UE transmits a CM SERVICE REQUEST message. | --> | CM SERVICE REQUEST |
| 1a2 | The SS transmits a CM SERVICE REJECT with the reject cause #32 (Service option not supported) | <-- | CM SERVICE REJECT |
| - | EXCEPTION: Step 1b1 specifies the MT call procedure. | - | - |
| 1b1 | The UE transmits a PAGING RESPONSE message. | --> | PAGING RESPONSE |
| - | EXCEPTION: Steps 2a1 to 2a6 specify the procedure when GERAN cell is in NMO II and if the UE is in condition C2 and the UE is registered to the LAC of the current GERAN cell. | - | - |
| 2a1 | The UE transmits a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST |
| 2a1A1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE |
| - | EXCEPTION: The next step describes behaviour that depends on UE capability. | - | - |
| 2a1A2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. |
| 2a2 | The SS transmits a LOCATION UPDATING ACCEPT | <-- | LOCATION UPDATING ACCEPT |
| 2a3 | The UE transmits a TMSI REALLOCATION COMPLETE |  | TMSI REALLOCATION COMPLETE |
| 2a4 | The UE transmits a ROUTING AREA UPDATE REQUEST message. | --> | ROUTING AREA UPDATE REQUEST |
| 2a5 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 2a6 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |
| - | EXCEPTION: Steps 2b1 to 2b3 specify the location updating procedure when GERAN cell is in (NMO I or NMO II) and if the UE is in condition C3 and the UE is not registered to the LAC of the current GERAN cell | - | - |
| 2b1 | The UE transmits a LOCATION UPDATING REQUEST message. | --> | LOCATION UPDATING REQUEST |
| 2b1A1 | The UE transmits a Classmark Change message | --> | CLASSMARK CHANGE |
| - | EXCEPTION: The next step describes behaviour that depends on UE capability. | - | - |
| 2b1A2 | IF pc_UTRA THEN the UE transmits a Utran Classmark Change message. | --> | UTRAN CLASSMARK CHANGE. |
| 2b2 | The SS transmits a LOCATION UPDATING ACCEPT | <-- | LOCATION UPDATING ACCEPT |
| 2b3 | The UE transmits a TMSI REALLOCATION COMPLETE |  | TMSI REALLOCATION COMPLETE |
| - | EXCEPTION: Steps 2c1 to 2c3 specify the routing area updating procedure when the GERAN cell is in NMO I and the UE is in condition C2and the UE is not registered to the LAC of the current GERAN cell | - | - |
| 2c1 | The UE transmits a ROUTING AREA UPDATE REQUEST message with update type = ‘Combined RA/LA update'. | --> | ROUTING AREA UPDATE REQUEST |
| 2c2 | The SS transmits a ROUTING AREA UPDATE ACCEPT message. | <-- | ROUTING AREA UPDATE ACCEPT |
| 2c3 | The UE transmits a ROUTING AREA UPDATE COMPLETE message. | --> | ROUTING AREA UPDATE COMPLETE |

## 10.3	Postambles for E-UTRA test cases

This clause describes UE postamble states which can be used in the post condition of E-UTRA test cases defined in TS 36.523-1 [1]. The clause also specifies a set of procedures to bring the UE into these states.

### 10.3.1	UE postamble states and procedures for E-UTRA test cases

In order to bring the UE to switched/powered off state there are some procedures that need to be executed. The identified procedures are shown in figure 10.3.1-1.

Figure 10.3.1-1: UE postamble states and procedures for E-UTRA

### 10.3.2	Switch/Power off procedure in State E1

#### 10.3.2.1	Procedure

Table 10.3.2.1-1: Switch/Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is powered off or switched off, (see ICS) | - | - |
| - | EXCEPTION: Steps 2a1 to 2a4 specify behaviour if the UE supports pc_SwitchOnOff | - | - |
| 2a1 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 2a2 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| - | EXCEPTION: Steps 2a3Aa1 to 2a3Aa6 specify optional behaviour if the UE has previously performed IMS registration | - | - |
| 2a3Aa1 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 2a3Aa2 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 2a3Aa3 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 2a3Aa4 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
|  | EXCEPTION: In parallel to the event described in step 2a3Aa5 below, the behaviour in TS 34.229-1 [40] Annex C.30 may occur. (IMS de-registration) | - | - |
| 2a3Aa5 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 2a3Aa6 | The UE initiates the Detach procedure by sending DETACH REQUEST | --> | NAS: DETACH REQUEST |
|  | EXCEPTION : Step 2a3Ba1 below specifies the behaviour if the UE has not previously performed IMS registration |  |  |
| 2a3 Ba1 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Detach procedure by including the DETACH REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: DETACH REQUEST |
| 2a4 | The SS transmits an RRCConnectionRelease message | <-- | RRC: RRCConnectionRelease |

### 10.3.3	Switch/Power off procedure in State E2 and E3

#### 10.3.3.1	Procedure for E2 and E3

Table 10.3.3.1-1: Switch/Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| - | EXCEPTION: In parallel to the events described in steps 1 to 2a2, the steps described: - in Table 10.3.3.1-2 may take place if UE supports pc_eMBMS_SC and SIB15 is broadcasted in the cell, - in Table 10.3.3.1-3 may take place if UE supports pc_discSupportedBands and SIB19 is broadcasted in the cell or if UE supports pc_commSupportedBands and SIB18 is broadcasted in the cell. |  |  |
| 1 | The UE is powered off or switched off (see ICS) | - | - |
| - | EXCEPTION: Steps 2a1 to 2a2 specify behaviour if the UE supports pc_SwitchOnOff | - | - |
| - | EXCEPTION : Step 2a1Aa1 to 2a1Aa2 below specifies optional behaviour if the UE has previously performed IMS registration | - | - |
| 2a1Aa1 – 2a1Aa2 | The UE may perform the procedure described in TS 34.229-1 [40] Annex C.30 (IMS de-registration) | - | - |
| 2a1 | The UE transmits DETACH REQUEST | --> | DETACH REQUEST |
| 2a2 | The SS transmits an RRCConnectionRelease message | <-- | RRC: RRCConnectionRelease |

Table 10.3.3.1-2: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | UE transmits a MBMSInterestIndication message. | --> | MBMSInterestIndication |

Table 10.3.3.1-3: Parallel behaviour

| St | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | UE transmits a SidelinkUEInformation message. | --> | SidelinkUEInformation |

#### 10.3.3.2	Procedure for E2_T3440

Table 10.3.3.2-1: RRC release and switch/power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The SS transmits an RRCConnectionRelease message | <-- | RRC: RRCConnectionRelease |
| 2 | The SS waits for 5s to ensure that the UE goes to RRC_IDLE state. | - |  |
| 3 | The UE is powered off or switched off (see ICS) | - | - |
| - | EXCEPTION: Steps 4a1 to 4a4 specify behaviour if the UE supports pc_SwitchOnOff | - | - |
| 4a1 | UE transmits an RRCConnectionRequest message. | --> | RRC: RRCConnectionRequest |
| 4a2 | SS transmit an RRCConnectionSetup message. | <-- | RRC: RRCConnectionSetup |
| - | EXCEPTION: Steps 4a3Aa1 to 4a3Aa6 specify optional behaviour is the UE has previously performed IMS registration | - | - |
| 4a3Aa1 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the IMS signalling procedure by including the SERVICE REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: SERVICE REQUEST |
| 4a3Aa2 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | RRC: SecurityModeCommand |
| 4a3Aa3 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | RRC: SecurityModeComplete |
| 4a3Aa4 | The SS configures a new data radio bearer, associated with the default EPS bearer context. The RRCConnectionReconfiguration message is using condition SRB2-DRB(1, 0). The DRB associated with default EPS bearer context obtained during the attach procedure is established | <-- | RRC: RRCConnectionReconfiguration |
| - | EXCEPTION: In parallel to the event described in step 4a3Aa5 below, the behaviour in TS 34.229-1 [40] Annex C.30 may occur. (IMS de-registration) | - | - |
| 4a3Aa5 | The UE transmits an RRCConnectionReconfigurationComplete message to confirm the establishment of the new data radio bearer, associated with the default EPS bearer context. | --> | RRC: RRCConnectionReconfigurationComplete |
| 4a3Aa6 | The UE initiates the Detach procedure by sending DETACH REQUEST | --> | NAS: DETACH REQUEST |
| - | EXCEPTION : Step 4a3Ba1 below specifies the behaviour if the UE has not previously performed IMS registration | - |  |
| 4a3 Ba1 | The UE transmits an RRCConnectionSetupComplete message to confirm the successful completion of the connection establishment and to initiate the Detach procedure by including the DETACH REQUEST message. | --> | RRC: RRCConnectionSetupComplete NAS: DETACH REQUEST |
| 4a4 | The SS transmits an RRCConnectionRelease message | <-- | RRC: RRCConnectionRelease |

### 10.3.4	Switch/Power off procedure in State E4

#### 10.3.4.1	Procedure

Table 10.3.4.1-1: Switch/Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is powered off or switched off (see ICS) | - | - |

### 10.3.5	Automatic selection mode procedure in State E5 (current cell, neighbour cell)

#### 10.3.5.1	Procedure

Table 10.3.5.1-1: Automatic selection mode procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Set UE to Automatic PLMN selection mode. | - | - |
| 2 | The SS configures: - the current cell as a “Non-Suitable Off cell” - the neighbour cell as a “Serving cell” | - | - |
| 3 | Generic test procedure in TS 36.508 [3] clause 6.4.2.7-1 is performed on the neighbour cell | - | - |
| - | At the end of this test procedure sequence, the UE is in end state E-UTRA idle (E1) | - | - |

## 10.4	Postambles for E-UTRA to HRPD test cases

This clause describes UE postamble states which can be used in the post condition of E-UTRA test cases defined in TS 36.523-1 [1]. The clause also specifies a set of procedures to bring the UE into these states.

### 10.4.1	UE postamble procedures for E-UTRA to HRPD (No Pre-Registration)

#### 10.4.1.1	Registration on HRPD Cell

Table 10.4.1.1: Registration on HRPD Cell procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE transmits a UATIRequest message. | --> | UATIRequest |
| 2 | The SS transmits UATIAssignment message | <-- | UATIAssignment |
| 3 | The UE transmits UATIComplete message | --> | UATIComplete |
| 4 | The UE transmits ConnectionRequest message. | --> | ConnectionRequest |
| 5 | The SS transmits a TrafficChannelAssignment message. | <-- | TrafficChannelAssignment |
| 6 | The UE transmits TrafficChannelcomplete. | --> | TrafficChannelcomplete |
| 7 | The UE transmits ConfigurationRequest message for SCP configuration. | --> | SCP:ConfigurationRequest |
| 8 | The SS transmits a ConfigurationResponse message for SCP configuration. | <-- | SCP:ConfigurationResponse |
| 9 | The UE transmits ConfigurationRequest message for Stream protocol. | --> | Stream:ConfigurationRequest |
| 10 | The SS transmits a ConfigurationResponse message for Stream protocol accepting EMPA bound to service network. | <-- | Stream: ConfigurationResponse |
| 11 | The UE transmits EMPA ConfigurationRequest message. | --> | EMPA:ConfigurationRequest |
| 12 | The SS transmits an EMPA ConfigurationResponse message. | <-- | EMPA: ConfigurationResponse |
| 13 | The UE transmits ConfigurationComplete message. | --> | ConfigurationComplete |
| 14 | Optionally session negotiation initiated by the SS might take place | <--> | - |
| 15 | Optionally device level authentication may take place. | <--> | - |
| 16 | Optionally Location Update procedure may take place if the SS is configured to support it. | <--> | - |
| 17 | PPP LCP negotiation is performed between the UE and the SS. EAP-AKA is selected as the authentication protocol. | <--> | - |
| 18 | Tunnelled EAP-AKA is performed between the UE and the SS. | <--> | - |
| 19 | The UE transmits VSNCP Configure-Request message, including a PDN-ID, PDN Type, APN, PDN Address with empty content, Protocol Configuration Options, and Attach Type = "handover". The Address Allocation Preference option contained in the Protocol Configuration Options indicates whether the UE wants to perform the IP address allocation during the attach procedure or deferred IPv4 address allocation. PDN Type indicates the UE's IP capability (IPv4, IPv6 or IPv4/v6) | --> | VSNCP: Configure-Request |
| 20 | The SS transmits a VSNCP Configure-Ack message. | <-- | VSNCP: Configure-Ack |
| 21 | The SS transmits a VSNCP Configure-Request message including the PDN-ID configuration option. | <-- | VSNCP: Configure-Request |
| 22 | The UE transmits VSNCP Configure-Ack message. | --> | VSNCP :Configure-Ack |
| 23 | Optionally IPv4 address allocation by DHCPv4 may occur (depending on the Address Allocation Preference indicated by the UE at Step 19). | <--> | - |
| 24 | Optionally Link global IPv6 address configuration by ICMPv6 may occur (depending on the Address Allocation Preference indicated by the UE at Step 19).solicitation message. | <--> | - |

#### 10.4.1.2	Detach on HRPD Cell

Table 10.4.1.2: Detach on HRPD Cell procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE transmits PPP:LCP Terminate-Request | --> | LCP:Terminate-Request |
| 2 | The SS transmits PPP: LCP Terminate-Ack | <-- | LCP:Terminate-Ack |
| 3 | the UE and SS perform Session update to release the reservations | <--> | - |

## 10.5	Postambles for NB-IoT test cases

This clause describes UE postamble states which can be used in the post condition of NB-IoT test cases defined in TS 36.523-1 [1]. The clause also specifies a set of procedures to bring the UE into these states.

### 10.5.1	UE postamble states and procedures for NB-IoT test cases

In order to bring the UE to switched/powered off state there are some procedures that need to be executed. The identified procedures are shown in figure 10.5.1-1.

Figure 10.5.1-1: UE postamble states and procedures for NB-IoT

### 10.5.2	Switch/Power off procedure in State N1

#### 10.5.2.1	Procedure

Table 10.5.2.1-1: Switch/Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is powered off or switched off, (see ICS) | - | - |
| - | EXCEPTION: Steps 2a1 to 2a4 specify behaviour if the UE supports pc_SwitchOnOff | - | - |
| 2a1 | UE transmits an RRCConnectionRequest-NB message. | --> | RRC: RRCConnectionRequest-NB |
| 2a2 | SS transmit an RRCConnectionSetup-NB message. | <-- | RRC: RRCConnectionSetup-NB |
| 2a3 | The UE transmits an RRCConnectionSetupComplete-NB message to confirm the successful completion of the connection establishment and to initiate the Detach procedure by including the DETACH REQUEST message. | --> | RRC: RRCConnectionSetupComplete-NB NAS: DETACH REQUEST |
| 2a4 | The SS transmits an RRCConnectionRelease-NB message | <-- | RRCConnectionRelease-NB |

### 10.5.3	Switch/Power off procedure in State N2 and N3

#### 10.5.3.1	Procedure for N2 and N3

Table 10.5.3.1-1: Switch/Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is powered off or switched off (see ICS) | - | - |
| 2 | The UE transmits DETACH REQUEST | --> | DETACH REQUEST |
| 3 | The SS transmits an RRCConnectionRelease-NB message | <-- | RRCConnectionRelease-NB |

### 10.5.4	Switch/Power off procedure in State N4

#### 10.5.4.1	Procedure

Table 10.5.4.1-1: Switch/Power off procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is powered off or switched off (see ICS) | - | - |

### 10.5.5	Automatic selection mode procedure in State N5 (current cell, neighbour cell)

#### 10.5.5.1	Procedure

Table 10.5.5.1-1: Automatic selection mode procedure

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | Set UE to Automatic PLMN selection mode. | - | - |
| 2 | The SS configures: - the current cell as a “Non-Suitable Off cell” - the neighbour cell as a “Serving cell” | - | - |
| 3 | Generic test procedure in TS 36.508 [3] clause 8.1.5A.5 is performed on the neighbour cell | - | - |
| - | At the end of this test procedure sequence, the UE is in end state NB-IoT idle (N1) | - | - |

## 10.6	Postambles for WLAN test cases

This clause describes UE postamble states which can be used in the post condition of WLAN test cases defined in TS 36.523-1 [1] and TS 34.229-1 [40]. The clause specifies a set of procedures to bring the UE into these states.

### 10.6.1	UE postamble states and procedures for WLAN test cases

In order to bring the UE to switched/powered off state there are some procedures that need to be executed. The identified procedures are shown in figure 10.6.1-1.

Figure 10.6.1-1: UE postamble states and procedures for WLAN

### 10.6.2	IMS Deregistration

#### 10.6.2.1	Procedure

Table 10.6.2.1-1: IMS Deregistration

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE is powered off or switched off, (see ICS) | - | - |
| 2 | The SS starts Timer_1 = 2 seconds | - | - |
| - | EXCEPTION: Steps 3a1 – 3b1 describe behaviour that depends on the UE implementation | - | - |
| 3a1 | IMS de-registration is performed using the generic procedure defined in 34.229-1 [40] Annex C.30. Note The SS cancels the Timer_1 | - | - |
| 3b1 | The SS waits for Timer_1 expiry | - | - |

### 10.6.3	IPsec tunnel(s) release

#### 10.6.3.1	Procedure

Table 10.6.3.1-1: IPsec tunnel(s) release

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The generic procedure for IPsec Tunnel Disconnection according to TS 36.508 [3] clause 4.5A.23A takes place. | - | - |
| NOTE: There may be more than one IPsec tunnel to disconnect. |  |  |  |

### 10.6.4	WLAN AP disassociation

#### 10.6.4.1	Procedure

Table 10.6.4.1-1: WLAN AP disassociation

| Step | Procedure | Message Sequence |  |
| --- | --- | --- | --- |
|  |  | U - S | Message |
| 1 | The UE performs a disassociation from the WLAN AP as defined in IEEE 802.11-2012 [70] clause 4.5.3.5. | - | - |

# 11	Guidelines on test execution

This clause provides the guidelines on test executions.

The restriction on test case execution as listed in this clause is due to the restriction of bandwidth to accommodate the necessary number of radio frequencies for the specific operating Band as used by the test cases.

## 11.1	E-UTRA single technology

This clause provides the guidelines for the test cases to be executed on the pure E-UTRA test configuration.

A test case using more than one radio frequency, i.e. using the radio frequencies f2 or f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 13,

Band 18,

Band 31,

Band 72,

Band 73,

Band 87,

Band 88.

The list containing such test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.6b, 6.1.2.7, 6.1.2.8, 6.1.2.9, 6.1.2.11, 6.1.2.11a, 6.1.2.13, 6.1.2.15, 6.1.2.17, 6.1.2.18, 6.3.1, 6.3.5, 6.3.6, 6.3.9, 6.4.1, 6.4.2,

8.1.3.4, 8.1.3.5, 8.1.3.5a, 8.2.4.6, 8.2.4.26, 8.2.4.28, 8.3.1.3, 8.3.1.3a, 8.3.1.4, 8.3.1.6, 8.3.1.9, 8.3.1.10, 8.3.1.11, 8.3.1.26, 8.3.1.27, 8.3.1.31, 8.3.4.2, 8.3.4.3, 8.3.4.5, 8.6.2.2, 8.6.2.3a, 8.6.2.8, 8.6.2.10, 8.6.2.11, 8.6.2.12, 8.6.4.2, 8.6.4.3, 8.6.4.4, 8.6.4.8, 8.6.4.9, 8.6.4.10, 8.6.6.2, 8.6.6.5, 8.6.6.6, 8.6.6.7, 8.6.8.2, 8.6.8.6, 8.9.4a, 8.9.5a,

9.2.1.1.1a, 9.2.1.1.7, 9.2.1.1.13, 9.2.1.1.15, 9.2.1.1.16, 9.2.1.2.1, 9.2.1.2.10, 9.2.1.2.12, 9.2.1.2.14, 9.2.3.2.1, 9.2.3.2.12, 9.2.3.2.15, 9.2.3.2.16, 9.2.5.1,

11.2.6, 11.2.7,

13.4.1.2,

17.4.1, 17.4.2, 17.4.3,

21.3.1, 21.3.2, 21.3.3, 21.3.7,

24.1.3.

A test case using more than two radio frequencies, i.e. using the radio frequencies f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 24,

Band 30,

Band 38,

Band 39

Band 53

The list containing such test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.3, 6.1.1.6, 6.1.1.7, 6.1.1.8, 6.1.2.7, 6.1.2.8, 6.1.2.9, 6.1.2.11a, 6.1.2.15, 6.3.5

8.1.3.5, 8.1.3.5a, 8.3.1.4, 8.6.4.3, 8.9.4a, 8.9.5a,

9.2.1.1.1a, 9.2.1.1.7, 9.2.1.1.15,

11.2.7.

A test case using more than three radio frequencies, i.e. using the radio frequency f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 12,

Band 19,

Band 20,

Band 21,

Band 27,

Band 34,

Band 70,

Band 85.

The list containing such test cases is given below:

6.1.1.1, 6.1.1.2, 6.1.1.6, 6.1.1.8,

8.1.3.5a,

9.2.1.1.7.

### 11.1.1	Replacement of test case execution

In case of bandwidth limitation for accommodation of more frequencies, a number of test cases can be replaced with the corresponding mirror test cases without affecting the test coverage. The table 11.1.1-1 shows the possible replacements. Only one of the paired test cases is required for execution.

Table 11.1.1-1: Replacement of test cases

| Original test case | Replacing test case |
| --- | --- |
| 6.1.1.1 | 6.1.1.1b |
| 6.1.1.2 | 6.1.1.2a |
| 6.1.1.3 | 6.1.1.3b |
| 6.1.1.6 | 6.1.1.6a or 6.1.1.6b |
| 6.1.1.7 | 6.1.1.7a |
| 6.1.2.7 | 6.1.2.7a |
| 6.1.2.8 | 6.1.2.8a |
| 6.1.2.9 | 6.1.2.9a |
| 8.1.3.12 | 8.1.3.12b |
| 8.3.1.9 | 8.3.1.9a |
| 8.3.1.11 | 8.3.1.11a |
| 9.2.1.1.1a | 9.2.1.1.1b |
| 9.2.1.1.7 | 9.2.1.1.7a |
| 9.2.1.1.13 | 9.2.1.1.13a |
| 9.2.1.1.15 | 9.2.1.1.15a |
| 9.2.1.1.16 | 9.2.1.1.16a |
| 9.2.3.1.15 | 9.2.3.1.15a |
| 9.2.3.1.18 | 9.2.3.1.18a |
| 17.4.1 | 17.4.1a |

## 11.2	E-UTRA - UTRA - GERAN

This clause contains the guidelines for the E-UTRA interRAT test cases to be executed on the different test configurations: with only UTRA configured, with only GERAN configured or with UTRA-GERAN both configured. Whether or not an E-UTRA frequency band overlaps the UTRA band, the dependency will affect the restrictions of the test execution on this band.

NOTE:	The case where an E-UTRA band overlaps a GSM band is not considered.

### 11.2.1	UTRA configured – GERAN not configured

This clause provides the guidelines for the E-UTRA interRAT test cases where UTRA is configured, while GERAN is either not needed or not configured.

#### 11.2.1.1	E-UTRA band overlapping UTRA band

The restriction on test case execution as listed in this clause is due to the bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA or E-UTRA, UTRA radio frequencies if an E-UTRA band overlaps the UTRA Band. A test case using more than one radio frequency, on the same E-UTRA and UTRA band, shall avoid to be executed on operating

Band 13,

Band 18,

Band 31.

The list containing such test cases is given below:

6.2.1.2, 6.2.1.3, 6.2.2.1, 6.2.2.5, 6.2.2.8, 6.2.3.3, 6.2.3.3a, 6.2.3.4, 6.2.3.4a, 6.2.3.5, 6.2.3.5a, 6.2.3.6, 6.2.3.13, 6.2.3.31, 6.2.3.32, 6.2.3.33, 6.2.4.1, 6.2.4.2, 6.2.4.3, 6.2.4.4, 6.2.4.5, 6.2.4.6, 6.2.4.7, 6.3.3, 6.3.4, 6.3.7, 6.4.3, 6.4.4, 6.4.5, 6.4.6,

8.1.3.6, 8.1.3.6a, 8.1.3.7, 8.3.2.3, 8.3.2.3a, 8.3.2.4, 8.3.3.2, 8.3.4.4, 8.4.1.2, 8.4.1.4, 8.4.1.5, 8.4.2.2, 8.4.2.4, 8.5.2.1, 8.6.3.1, 8.6.3.4, 8.6.5.1, 8.6.5.1a, 8.6.7.1, 8.6.7.4, 8.6.9.1, 8.6.9.2, 8.6.10.1, 8.7.1,

9.2.1.1.11, 9.2.1.1.12, 9.2.1.2.1b, 9.2.1.2.1c, 9.2.1.2.1d, 9.2.1.2.5, 9.2.1.2.8, 9.2.1.2.9, 9.2.1.2.11, 9.2.1.2.13, 9.2.1.2.15, 9.2.2.1.3, 9.2.2.1.10, 9.2.3.1.6, 9.2.3.1.10, 9.2.3.1.11, 9.2.3.1.12, 9.2.3.1.15, 9.2.3.1.15a, 9.2.3.1.17, 9.2.3.1.18, 9.2.3.1.18a, 9.2.3.2.1a, 9.2.3.2.1b, 9.2.3.2.1c, 9.2.3.2.3, 9.2.3.2.5, 9.2.3.2.6, 9.2.3.2.7, 9.2.3.2.8, 9.2.3.2.9, 9.2.3.2.11, 9.2.3.2.13, 9.2.3.2.14, 9.2.3.3.1, 9.2.3.3.2, 9.2.3.3.3, 9.2.3.3.4, 9.2.3.3.5, 9.2.3.3.5a, 9.3.1.4, 9.3.1.5, 9.3.1.6,

11.2.10, 11.2.11, 11.3.3, 11.3.5, 11.3.7,

13.1.2, 13.1.2a, 13.1.3, 13.1.4, 13.1.5, 13.1.15, 13.1.16, 13.3.2.1, 13.4.2.1, 13.4.2.4, 13.4.3.1, 13.4.3.2, 13.4.3.4, 13.4.3.6, 13.4.3.7, 13.4.3.8, 13.4.3.9, 13.4.3.10, 13.4.3.11, 13.4.3.13, 13.4.3.14, 13.4.3.15, 13.4.3.16, 13.4.3.18, 13.4.3.19, 13.4.3.20.

A test case using more than two radio frequencies on the same E-UTRA and UTRA band shall avoid to be executed on operating:

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 38,

Band 39.

The list containing such test cases is given below:

6.2.1.2, 6.2.1.3,

8.6.3.4, 8.6.5.1a, 8.6.7.4,

9.2.1.2.9, 9.2.1.2.11, 9.2.1.2.13, 9.2.3.1.15, 9.2.3.1.18, 9.2.3.2.5, 9.2.3.2.6, 9.2.3.2.7, 9.2.3.2.8, 9.2.3.2.11, 9.2.3.2.13, 9.2.3.2.14.

A test case using more than three radio frequencies, on the same E-UTRA and UTRA band shall avoid to be executed on operating

Band 12,

Band 19,

Band 20,

Band 21,

Band 27,

Band 34,

Band 70,

Band 85.

The list containing such test cases is given below:

9.2.1.2.13, 9.2.3.2.13.

#### 11.2.1.2	E-UTRA band not overlapping UTRA band

The restriction on test case execution as listed in this clause is due to the bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA radio frequencies. A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31,

Band 72,

Band 73,

Band 87,

Band 88.

The list containing such test cases is given below:

6.2.1.2, 6.2.1.3,

8.6.3.4, 8.6.5.1a, 8.6.7.4,

9.2.1.2.9, 9.2.1.2.11, 9.2.1.2.13, 9.2.3.1.15, 9.2.3.1.18, 9.2.3.2.5, 9.2.3.2.6, 9.2.3.2.7, 9.2.3.2.8, 9.2.3.2.11, 9.2.3.2.13, 9.2.3.2.14,

A test case using more than two radio frequencies shall avoid to be executed on E-UTRA operating

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 24,

Band 30,

Band 38,

Band 39.

The list containing such test cases is given below:

9.2.1.2.13, 9.2.3.2.13.

### 11.2.2	GERAN configured - UTRA not configured

This clause provides the guidelines for the E-UTRA/GERAN test cases where UTRA is either not needed or not configured. The restriction on test case execution as listed in this clause is due to the restriction of bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA radio frequencies.

A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31,

Band 72,

Band 73,

Band 87,

Band 88.

The list containing such test cases is given below:

6.2.1.4, 6.2.3.17, 6.2.3.18,

8.3.2.2,

9.2.1.2.9, 9.2.1.2.11, 9.2.1.2.13, 9.2.3.1.15, 9.2.3.1.18, 9.2.3.2.5, 9.2.3.2.6, 9.2.3.2.7, 9.2.3.2.8, 9.2.3.2.11, 9.2.3.2.13, 9.2.3.2.14.

A test case using more than two radio frequencies, i.e. using the radio frequencies f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 24,

Band 30,

Band 38,

Band 39

The list containing such test cases is given below:

9.2.1.2.13, 9.2.3.2.13.

### 11.2.3	Neither UTRA nor GERAN configured

Certain EMM test cases can be executed as EUTRA_Only configuration despite of UTRA or GERAN test branches included in the test cases. The restriction on test case execution as listed in this clause is due to the bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA radio frequencies.

A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31,

Band 72,

Band 73,

Band 87,

Band 88.

The list containing such test cases is given below:

9.2.3.1.15, 9.2.3.1.18.

### 11.2.4	Both UTRA and GERAN configured

This clause provides the guidelines for the E-UTRA - UTRA - GERAN test cases where three RAT technologies are simultaneously configured.

#### 11.2.4.1	E-UTRA band overlapping UTRA band

The restriction on test case execution as listed in this clause is due to the bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA or E-UTRA, UTRA radio frequencies if an E-UTRA band overlaps the UTRA Band.

A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31.

The list containing such test cases is given below:

6.2.1.1, 8.3.2.5, 8.3.2.6, 13.1.21,

9.2.1.2.6, 9.2.1.2.7,

11.2.8

A test case using more than two radio frequencies, i.e. using the radio frequencies f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on operating:

Band 6,

Band 11,

Band 14,

Band 17,

Band 23 with 10MHz bandwidth,

Band 38

Band 39

The list containing such test cases is given below:

6.2.1.1.

#### 11.2.4.2	E-UTRA band not overlapping UTRA band

The restriction on test case execution as listed in this clause is due to the restriction of bandwidth of an E-UTRA Band accommodating the necessary number of E-UTRA radio frequencies.

A test case using more than one radio frequency shall avoid to be executed on E-UTRA operating

Band 13,

Band 18,

Band 31,

Band 72,

Band 73,

Band 87,

Band 88.

The list containing such test cases is given below:

6.2.1.1

### 11.2.5	Replacement of test case execution

In case of bandwidth limitation for accommodation of more frequencies, a number of test cases can be replaced with the corresponding mirror test cases without affecting the test coverage. The table 11.2.5-1 shows the possible replacements. Only one of the paired test cases is required for execution.

Table 11.2.5-1: Replacement of interRAT test cases

| Original test case | Replacing test case |
| --- | --- |
| 9.2.3.1.15 | 9.2.3.1.15a |
| 9.2.3.1.18 | 9.2.3.1.18a |

## 11.3	E-UTRA inter-band

The restriction on test case execution as listed in this clause is due to the restriction of bandwidth of an E-UTRA band accommodating the necessary number of E-UTRA radio frequencies. The inter-band test includes also E-UTRA FDD-TDD and inter-band carrier aggregation tests.

### 11.3.1	Primary operating band

A test case using more than one radio frequency on the first operating band, shall avoid to be executed on operating

Band 13,

Band 18,

Band 31,

Band 72,

Band 73,

Band 87,

Band 88.

The list containing such test cases is given below:

6.1.2.15a, 8.1.3.12.

### 11.3.2	Secondary operating band for inter-band cells

Test case using more than one radio frequency, on the second operating band, shall avoid to be executed on operating

Band 13,

Band 18,

Band 31,

Band 72,

Band 73,

Band 87,

Band 88.

The list containing such test cases is given below:

6.1.1.1a, 6.1.1.3a, 6.1.1.4a, 6.1.2.5c, 6.1.2.16, 8.1.3.11a, 8.1.3.12b, 8.2.4.13a, 8.2.4.14a, 8.3.1.12a, 8.3.1.14a

### 11.3.3	Replacement of test case execution

In case of bandwidth limitation for accommodation of more frequencies, a number of test cases can be replaced with the corresponding mirror test cases without affecting the test coverage. The table 11.3.3-1 shows the possible replacements. Only one of the paired test cases is required for execution.

Table 11.3.3-1: Replacement of test cases

| Original test case | Replacing test case |
| --- | --- |
| 8.1.3.12 | 8.1.3.12b |

### 11.3.4	Inter-band MBMS

MBMS service continuity test cases shall avoid to be executed on operating bands with different bandwidth.

The list containing such test cases is given below:

17.4.3a

## 11.4	E-UTRA CA

The restriction on CA test case execution as listed in this clause is due to the restriction of bandwidth of an E-UTRA CA band accommodating the necessary number of E-UTRA radio frequencies.

### 11.4.1	CA contiguous Intra-band operation

Test case using more than two radio frequencies, i.e. using the radio frequencies f3 or f4 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_5B,

CA_23B,

CA_38C,

CA_39C,

CA_70C.

The list containing such test cases is given below:

8.2.4.19.1, 8.2.4.21.1

The following test cases with UL transmission on f2 are not applicable to be run when executed in the band combinations CA_70C:

7.1.2.10.1,

7.1.2.11.1,

7.1.4.20.1,

8.2.2.7.1,

8.2.4.17.1,

8.2.4.20.1,

8.2.4.23.1,

8.3.1.17.1,

8.3.1.18.1

### 11.4.2	CA Inter-band operation

Test case using more than one radio frequency on the primary band, i.e. using the radio frequency f2 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_3A-11A,

CA_5A-29A,

CA_8A-11A,

CA_12A-25A,

CA_12A-30A,

CA_12A-66A,

CA_13A-66A,

CA_14A-30A,

CA_14A-66A,

CA_18A-28A,

CA_19A-21A,

CA_19A-28A,

CA_19A-42A,

CA_19A-42C,

CA_21A-42A,

CA_21A-42C,

CA_30A-66A,

CA_70A-71A,

CA_70C-71A

and on E-UTRA CA switched configuration

CA_1A-18A,

CA_1A-19A,

CA_1A-21A,

CA_2A-12A,

CA_2A-13A,

CA_2A-14A,

CA_2A-17A,

CA_2A-30A,

CA_3A-11A,

CA_3A-19A,

CA_4A-12A,

CA_4A-13A,

CA_4A-17A,

CA_4A-30A,

CA_5A-12A,

CA_5A-13A,

CA_5A-17A,

CA_5A-30A,

CA_7A-12A,

CA_8A-11A,

CA_11A-18A,

CA_12A-30A,

CA_14A-30A,

CA_19A-21A,

CA_20A-31A,

CA_29A-30A,

CA_29A-70C,

CA_46A-70A,

CA_46A-70A,

CA_66A-70A,

CA_66A-70C,

CA_66C-70A,

CA_66A-70C

The list containing such test cases is given below:

8.2.4.19.2

Test case using more than one radio frequency on the primary or secondary band, i.e. using the radio frequency f2 or f6 (depending upon the Cell being used) specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_3A-11A,

CA_8A-11A,

CA_19A-21A,

CA_12A-30A

CA_14A-30A,

and on E-UTRA CA switched configuration

CA_3A-11A,

CA_8A-11A,

CA_19A-21A,

CA_12A-30A,

CA_14A-30A,

CA_29A-30A

The list containing such test cases is given below:

8.2.4.21.2

These test cases can be run with switched allocation of PCell and SCell as specified in TS 36.508 [3] clause 6.2.3.2 (NOTE 3 and 4).

These test cases with switched allocation of PCell and SCell or with UL transmission on SCell shall avoid to be executed on E-UTRA CA Configuration

with a band having no UL frequency

CA_2A-29A,

CA_4A-29A,

CA_5A-29A,

CA_3A-32A,

CA_3A-69A,

CA_20A-32A,

CA_23A-29A,

CA_20A-67A

and on E-UTRA CA switched configuration

CA_29A-30A,

CA_29A-66A,

CA_29A-70A,

CA_29A-70C

with an LAA unlicenced band

CA_1A-46A,

CA_2A-46A,

CA_3A-46A,

CA_4A-46A,

CA_7A-46A,

CA_41A-46A,

CA_42A-46A

and with an LAA unlicenced band-switched configuration

CA_46A-66A,

CA_46C-66A,

CA_46A-70A,

8.2.4.17.2,

8.2.4.20.2,

8.2.4.23.2,

17.4.11.2,

21.3.12.2

The PICS for UL CA 36.523-2 table A.4.3.3.3-2 shall be set in accordance with the UL CA support in the Band combination under test.

The 3DL CA test cases can be run on 3CCs FDD-TDD band combinations, they are using more than one radio frequency on the primary band, i.e. using the radio frequency f2 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_19A-42C,

CA_21A-42A,

CA_21A-42C,

CA_70A-71A

The list containing such test cases is given below:

7.1.2.11.4

### 11.4.3	CA non-contiguous Intra-band operation

Test case using more than two radio frequencies, i.e. using the radio frequencies f3 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA CA Configuration:

CA_5A-5A,

CA_23A-23A

The list containing such test cases is given below:

8.2.4.19.3

8.2.4.21.3

## 11.5	E-UTRA MFBI

The following E-UTRA MFBI test cases shall be executed using the combinations specified in Table 11.5-1 for px_OverlappingNotSupportedFrequencyBand_MFBI and px_MFBI_FrequencyBand:

6.1.2.19, 6.1.2.20, 6.1.2.21, 6.1.2.22, 6.2.3.34, 6.2.3.35

Table 11.5-1: Operating and MFBI E-UTRA bands combinations

| px_OverlappingNotSupportedFrequencyBand_MFBI | px_MFBI_FrequencyBand (Note) |
| --- | --- |
| 2 | 25 |
| 3 | 9 |
| 4 | 10, 66 |
| 5 | 18, 19, 26 |
| 9 | 3 |
| 10 | 4, 66 |
| 12 | 17 |
| 17 | 12, 85 |
| 18 | 5, 26, 27 |
| 19 | 5, 26 |
| 25 | 2 |
| 26 | 5, 18, 19, 27 |
| 27 | 18, 26 |
| 33 | 39 |
| 38 | 41 |
| 39 | 33 |
| 41 | 38 |
| 66 | 4, 10 |
| 85 | 17 |
| Note: The UE supports one or more of the listed MFBI bands and does not support at least one overlapping band. If the UE supports all overlapping bands, these test cases are not applicable. Editor’s Note: MFBI E-UTRA bands combinations involving band 85 are not applicable for test case execution until the corresponding frequencies are defined in TS 36.508 [3], section 4.3.1.5. |  |

Test case 6.1.2.20 is not applicable to be run when executed in the band combination 5 & 18.

For test case 6.2.3.34 additionally the UTRA band combinations specified in Table 11.5-2 for px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI and px_MFBI_UTRAN_FrequencyBand shall also be applicable

Table 11.5-2: Operating and MFBI UTRA bands combinations

| px_UTRAN_OverlappingNotSupportedFrequencyBandMFBI | px_MFBI_UTRAN_FrequencyBand (Note) |
| --- | --- |
| 2 | 25 |
| 3 | 9 |
| 4 | 10 |
| 5 | 18, 19, 26 |
| 9 | 3 |
| 10 | 4 |
| 18 | 5, 26 |
| 19 | 5, 26 |
| 25 | 2 |
| 26 | 5, 18, 19 |
| Note: The UE supports one or more of the listed MFBI bands and does not support at least one overlapping band. If the UE supports all overlapping bands, these test cases are not applicable. |  |

The MFBI test case 6.1.2.23 shall be executed using the combinations specified in Table 11.5-3 for px_ePrimaryFrequencyBand, px_MFBI_FrequencyBand and px_OverlappingSupportedFrequencyBand_MFBI

Table 11.5-3: Operating and MFBI E-UTRA bands combinations

| px_ePrimaryFrequencyBand | px_MFBI_FrequencyBand | px_OverlappingSupportedFrequencyBand_MFBI |
| --- | --- | --- |
| 39 | 38 | 41 |

The following NB-IoT MFBI test cases shall be executed using the combinations specified in Table 11.5-4 for px_OverlappingNotSupportedFrequencyBand_MFBI and px_MFBI_FrequencyBand:

22.2.10

Table 11.5-4: Operating and MFBI E-UTRA bands combinations

| px_OverlappingNotSupportedFrequencyBand_MFBI | px_MFBI_FrequencyBand (Note) |
| --- | --- |
| 2 | 25 |
| 5 | 18, 19, 26 |
| 12 | 17 |
| 17 | 12, 85 |
| 19 | 5, 26 |
| 25 | 2 |
| 26 | 5, 18, 19 |
| 85 | 17 |
| Note: The UE supports one or more of the listed MFBI bands and does not support at least one overlapping band. If the UE supports all overlapping bands, these test cases are not applicable. Editor’s Note: MFBI E-UTRA bands combinations involving band 85 are not applicable for test case execution until the corresponding frequencies are defined in TS 36.508 [3], section 4.3.1.5. |  |

## 11.6	E-UTRA DC

The restriction on DC test case execution as listed in this clause is due to the restriction of bandwidth of an E-UTRA DC band accommodating the necessary number of E-UTRA radio frequencies.

Test case using more than one radio frequency on the primary band, i.e. using the radio frequency f2 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA DC inter-band configuration:

DC_19A-21A,

and on E-UTRA DC switched configuration

DC_1A-19A,

DC_1A-21A,

DC_2A-13A,

DC_3A-19A,

DC_4A-12A,

DC_4A-13A,

DC_4A-17A,

DC_5A-12A,

DC_19A-21A

The list containing such test cases is given below:

8.2.4.25.1, 8.2.4.25.2, 8.2.4.25.3, 8.2.4.25.4, 8.2.4.25.5, 8.2.4.25.6, 8.2.4.25.7

Test cases using more than one radio frequency on the secondary band, i.e. using the radio refuency f6 specified in TS 36.508 [3], shall avoid to be executed on E-UTRA DC inter-band configurations:

DC_1A-19A,

DC_1A-21A,

DC_2A-13A,

DC_3A-19A,

DC_4A-12A,

DC_4A-13A,

DC_4A-17A,

DC_5A-12A,

DC_19A-21A

and on E-UTRA DC switched configuration

DC_19A-21A

The list containing such test cases is given below:

8.5.1.8.1, 8.5.1.8.2