3GPP TSG-RAN5 Meeting #98	R5-231005

# Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 2267 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 4.5.2.6 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI15_Test, 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The SCell is never released during the test procedure |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a step 7a to release the SCell before releasing the EN-DC Added the message contents to release the SCell |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Readding an SCell that was never released could cause issues |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5.2.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### 4.5.2.6	EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC

4.5.2.6.1	Test purpose

The purpose of this test is to verify E-UTRAN PCell and NR PSCell interruptions during the measurement on the deactivated NR SCC, the UE missed ACK/NACK does not exceed the limits. This test will verify the missed ACK/NACK rate for E-UTRAN PCell and NR PSCell in EN-DC.

4.5.2.6.2	Test applicability

This test applies to all types of E-UTRA UE release 15 and forward supporting EN-DC and 2 DL CA in E-UTRA.

4.5.2.6.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 4.5.2.0.3.

The normative reference for this requirement is TS 38.133 [6] clause A.4.5.2.6.

4.5.2.6.4	Test description

4.5.2.6.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.5.2.6.4.1-1.

Table 4.5.2.6.4.1-1: Supported test configurations for EN-DC FR1 interruptions
during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC

| Configuration | Description |
| --- | --- |
|  | LTE PCell + NR PSCell Note 2 |
| 4.5.2.6-1 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.5.2.6-2 | LTE FDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.5.2.6-3 | LTE FDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| 4.5.2.6-4 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, FDD duplex mode |
| 4.5.2.6-5 | LTE TDD, NR 15 kHz SSB SCS, 10MHz bandwidth, TDD duplex mode |
| 4.5.2.6-6 | LTE TDD, NR 30 kHz SSB SCS, 40MHz bandwidth, TDD duplex mode |
| NOTE.1: The UE is only required to be tested in one of the supported test configurations. NOTE 2: The duplex mode of the LTE SCell is determined based on the band combination to be tested. |  |

Configure the test equipment and the DUT according to the parameters in Table 4.5.2.6.4.1-2.

Table 4.5.2.6.4.1-2: Initial conditions for EN-DC FR1 interruptions
during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.5.2.6.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in clause C.2.1. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 4.5.2.6.4.1-3.

2.	Message contents are defined in clause 4.5.2.6.4.3.

3.	There are two E-UTRAN carriers and one NR carrier and three cells specified in the test. Cell1 and Cell3 is E-UTRAN PCell and E-UTRAN deactivated SCell, Cell2 is NR FR1 PSCell. Cell 1 is the cell used for connection setup with the power level set according to Table A.6.1.1-1. Cell 3 shall be configured according to Table A.6.1.1-1 except for the RF channel number 3. Cell 2 shall be configured according to clauses C.1.1 and C.1.2.

Table 4.5.2.6.4.1-3: General test parameters for EN-DC FR1 interruptions
during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2, 3 | Two E-UTRAN RF channels and one NR RF channel |
| Active PCell |  | Cell1 | PCell on E-UTRAN RF channel number 1. |
| Active PSCell |  | Cell2 | PSCell on NR RF channel number 2. |
| Configured deactivated SCell |  | Cell3 | Deactivated SCell on E-UTRAN RF channel number 3. |
| CP length |  | Normal | Applicable to Cell1, Cell 2 and Cell3 |
| DRX |  | OFF |  |
| Measurement gap pattern Id |  | OFF |  |
| SCell measurement cycle (measCycleSCell) | ms | 640 |  |
| T1 | s | 10 |  |

4.5.2.6.4.2	Test procedure

The test consists of three cells: Cell1, Cell2 and Cell3. Cell1 and Cell3 is E-UTRAN PCell and E-UTRAN deactivated SCell, Cell2 is NR FR1 PSCell. The test consists of one time period, with duration of T1. Prior to the start of the time duration T1, the UE shall be connected to Cell1 and Cell2 and the RRC message including measCycleSCell or allowInterruptions for the deactivated NR SCells is received at the UE antenna connector. Cell1 shall be configured as E-UTRAN PCell, Cell2 shall be configured as NR PSCell and Cell3 shall be configured as E-UTRAN deactivated SCell. During T1 the UE shall be continuously scheduled on E-UTRAN PCell and NR PSCell.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG_and_SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

2.	Configure MCG according to TS 36.521-3 [26] clause C.0, C.1 and SCG according to clause C.1.1 and C.1.2 for all downlink physical channels.

3.	The SS shall configure SCell (Cell 3) on the SCC as per TS 36.508 [25] clause 5.2A.

4.	Set the parameters according to T1 in Table 4.5.2.6.5-1. Propagation conditions are set according to clause C.2.1. T1 starts.

5.	SS schedules on PCell and PSCell continuously and UE shall start sending ACK/NACK reports. The SS shall monitor DTX on PCell and ACK/NACK/DTX on PSCell.

6.	If more than 99.5% of uplink transmissions on PSCell are received by SS then count a success for the event "ACK/NACK". Otherwise count a fail for the event "ACK/NACK".

6a.	If no longer than X consecutive DTX on PCell is observed by the SS, then count a success for the event “PCell DTX”. Otherwise count a fail for the event “PCell DTX”. Where,

-	For test configuration 4.5.2.6-1, 4.5.2.6-2 and 4.5.2.6-3,

-	X = 1 if the PCell is not in the same band as the deactivated SCell, otherwise X = 9.

-	For test configuration 4.5.2.6-4, 4.5.2.6-5 and 4.5.2.6-6,

-	X = 1 if the PCell is not in the same band as the deactivated SCell, otherwise X = 5.

7.	If no longer than Xconsecutive DTX on PSCell is observed by the SS, then count a success for the event "PSCell DTX". Otherwise count a fail for the event "PSCell DTX". Where,

-	For test configuration 4.5.2.6-1 and 4.5.2.6-4,

-	Z = interruption length+k1 if k1 ≤ interruption length, otherwise Z = interruption length

-	For test configuration other than 4.5.2.6-1 and 4.5.2.6-4,

-	Z = interruption length.

-	Interruption length is given by Table 4.5.2.6.5-2 for inter-band case and Table 4.5.2.6.5-3 for intra-band case.

7a.	The SS shall transmit RRCConnectionReconfiguration message with condition SCell_Rel according to 4.5.2.6.4.3-4 to release E-UTRA SCell. The UE shall transmit RRCConnectionReconfigurationComplete message.

8.	The SS shall transmit RRCConnectionReconfiguration message with condition EN-DC_PSCell_Rel according to TS 36.508 [25] Table 4.6.1-8 to release NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message.

9.	The SS then shall transmit RRCConnectionReconfiguration message with condition MCG_and_SCG according to TS 36.508 [25] Table 4.6.1-8 to add NR cell (PSCell). The UE shall transmit RRCConnectionReconfigurationComplete message. If any the reconfiguration fails, switch off and on the UE and ensure the UE is in RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On according to TS 38.508-1 [14] clause 4.5

-	switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG_and_SCG, Connected without release On TS 38.508-1 [14] clause 4.5.

10.	Repeat step 2-9 until a test verdict has been achieved.

Each of the events "ACK/NACK" "PCell DTX" and "PSCell DTX" is evaluated independently for the statistic, resulting in an event verdict: pass or fail. Each event is evaluated only until the confidence level according to Table G.2.3-1 in Annex G.2 is achieved. Different events may require different times for a verdict.

If all events pass, the test passes. If one event fails, the test fails.

4.5.2.6.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions.

Table 4.5.2.6.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions |  |
| Specific message contents exceptions for Test Configuration 4.5.2.6-1, 4.5.2.6-2, 4.5.2.6-4 and 4.5.2.6-5 | Table H.3.1-3 with Condition INTRA-FREQ MO and SSB.1 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |
| Specific message contents exceptions for Test Configuration 4.5.2.6-3 and 4.5.2.6-6 | Table H.3.1-3 with Condition INTRA-FREQ MO and SSB.2 FR1 Table 7.3.1-3 in TS 38.508-1 [14] with condition SMTC.1 |

Table 4.5.2.6.4.3-2: MeasObjectEUTRA for E-UTRAN deactivated SCell

| Derivation Path: 36.508 Table 4.6.6-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasObjectEUTRA ::= SEQUENCE { |  |  |  |
| carrierFreq | Downlink EARFCN for E-UTRAN SCell |  |  |
| measCycleSCell-r10 | sf640 |  |  |
| } |  |  |  |

Table 4.5.2.6.4.3-3: RRCConnectionReconfiguration in step 3: SCell addition

Derivation Path: 36.508 Table 4.6.1-8, condition SCell_AddMod

Table 4.5.2.6.4.3-4: RRCConnectionReconfiguration in step 7a: SCell release

| Derivation Path: 36.508 Table 4.6.1-8 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |  |
| RRCConnectionReconfiguration ::= SEQUENCE { |  |  |  |  |
| sCellToReleaseList-r10 SEQUENCE (SIZE (1..maxSCell-r10) OF SEQUENCE { | 1 entry |  |  |  |
| sCellIndex-r10[1] | Cell Index of Cell 3 |  |  |  |
| } |  |  |  |  |

4.5.2.6.5	Test requirement

Table 4.5.2.6.5-1 defines the primary level settings including test tolerances for EN-DC FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC test configurations.

Table 4.5.2.6.5-1: NR cell specific test parameters for EN-DC FR1 interruptions
during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC

| Parameter |  |  | Unit | Cell 2 |
| --- | --- | --- | --- | --- |
| Frequency Range |  |  |  | FR1 |
| Duplex mode | Config 1,4 |  |  | FDD |
|  | Config 2,3,5,6 |  |  | TDD |
| TDD configuration | Config 1,4 |  |  | Not Applicable |
|  | Config 2,5 |  |  | TDDConf.1.1 |
|  | Config 3,6 |  |  | TDDConf.2.1 |
| BWchannel | Config 1,4 |  | MHz | 10: NRB,c = 52 |
|  | Config 2,5 |  |  | 10: NRB,c = 52 |
|  | Config 3,6 |  |  | 40: NRB,c = 106 |
| Initial DL BWP Configuration | Config 1,4 |  |  | DLBWP.0.1 |
|  | Config 2,5 |  |  | DLBWP.0.1 |
|  | Config 3,6 |  |  | DLBWP.0.1 |
| Dedicated DL BWP Configuration | Config 1,4 |  |  | DLBWP.1.1 |
|  | Config 2,5 |  |  | DLBWP.1.1 |
|  | Config 3,6 |  |  | DLBWP.1.1 |
| Initial UL BWP Configuration | Config 1,4 |  |  | ULBWP.0.1 |
|  | Config 2,5 |  |  | ULBWP.0.1 |
|  | Config 3,6 |  |  | ULBWP.0.1 |
| Dedicated UL BWP Configuration | Config 1,4 |  |  | ULBWP.1.1 |
|  | Config 2,5 |  |  | ULBWP.1.1 |
|  | Config 3,6 |  |  | ULBWP.1.1 |
| PDSCH Reference measurement channel | Config 1,4 |  |  | SR.1.1 FDD |
|  | Config 2,5 |  |  | SR.1.1 TDD |
|  | Config 3,6 |  |  | SR.2.1 TDD |
| RMSI CORESET parameters | Config 1,4 |  |  | CR.1.1 FDD |
|  | Config 2,5 |  |  | CR.1.1 TDD |
|  | Config 3,6 |  |  | CR.2.1 TDD |
| PDCCH CORESET parameters | Config 1,4 |  |  | CCR.1.1 FDD |
|  | Config 2,5 |  |  | CCR.1.1 TDD |
|  | Config 3,6 |  |  | CCR.2.1 TDD |
| TRS configuration | Config 1,4 |  |  | TRS.1.1 FDD |
|  | Config 2,5 |  |  | TRS.1.1 TDD |
|  | Config 3,6 |  |  | TRS.1.2 TDD |
| OCNG Patterns |  |  |  | OP.1 |
| SMTC Configuration |  |  |  | SMTC.1 |
| TCI state |  |  |  | TCI.State.0 |
| SSB Configuration | Config 1,2,4,5 |  |  | SSB.1 FR1 |
|  | Config 3,6 |  |  | SSB.2 FR1 |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| NocNote 2 |  |  | dBm/15 kHz | -104 |
| SS-RSRP Note 3 |  |  | dBm/15 kHz | -87 |
| Ês/Iot |  |  | dB | 17 |
| Ês/Noc |  |  | dB | 17 |
| IoNote3 | Config 1,2,4,5 |  | dBm/ 9.36MHz | -58.96 |
|  | Config 3,6 |  | dBm/ 38.16MHz | -52.86 |
| Time offset to Cell1Note 4 |  | Config 1,2,4,5 | s | 500 |  |
|  |  | Config 3,6 |  | 250 |  |
| Propagation Condition |  |  |  | AWGN |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Receive time difference of signals received between subframe timing boundary of E-UTRA PCell and slot timing boundary of PSCell at the UE antenna connector including time alignment error between the two cells. |  |  |  |  |

The UE shall be continuously scheduled in LTE PCell and NR PSCell during the entire length of T1. During the time duration T1 the UE shall transmit at least 99.5% of ACK/NACK on E-UTRAN PCell and NR PSCell. The UE is only allowed to cause one interruption on PCell and one interruption on PSCell . Each interruption on E-UTRAN PCell and NR PSCell shall not exceed the value defined in Table 4.5.2.6.5-2 and Table 4.5.2.6.5-3.

Table 4.5.2.6.5-2: Interruption duration if the NR PSCell is not
in the same band as the E-UTRAN deactivated SCell

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 2 |
| 1 | 0.5 | 2 |

Table 4.5.2.6.5-3: Interruption duration if the NR PSCell is
in the same band as the E-UTRAN deactivated SCell

|  | NR Slot length (ms) | Interruption length (slot) |
| --- | --- | --- |
| 0 | 1 | 2 + SMTC duration |
| 1 | 0.5 | 2 + SMTC duration |

Each interruption on E-UTRAN PCell shall not exceed 1 subframe if the PCell is not in the same band as the deactivated SCell, or 5 subframes if the PCell is in the same band as the deactivated SCell.

The rate of correct events observed during repeated tests shall be at least 90%.

< End of changes >