3GPP TSG-RAN5 Meeting #98 	R5-231848

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0644 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to test point 1-7 in 5.2.2.1.1_1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_HST-UEConTest |  |  |  |  |  | Date: |  |  | 2023-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test point 1-7 agreed in R5-216908 is not implemented in Table 5.2.2.1.1_1.4-1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test point 1-7 was added to Table 5.2.2.1.1_1.4-1 according to R5-216908. [ ] was removed from Test points 1-6 and 1-7 of Table 5.2.2.1.1.0-3 according to TS 38.101-4. Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5.2.2.1.1_1 test point 1-7 will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.1.0, 5.2.2.1.1_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-230983 and the outcome of 1 revisions to this document. r1: Invisible line was changed to solid line |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

5.2.2.1.1	2Rx FDD FR1 PDSCH mapping Type A performance

5.2.2.1.1.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.1.1.0-3 and Table 5.2.2.1.1.0-4, with the test parameters defined in table 5.2.2.1.1.0-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2.2.1.1.0-1.

Table 5.2.2.1.1.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers | 1-1, 1-2, 1-3, 1-5, 1-6, 1-7, 1-8, 2-1, 2-2 |
| Verify the PDSCH mapping Type A HARQ soft combining performance under 2 receive antenna conditions. | 1-4 |
| Verify the PDSCH mapping Type A performance requirements for Enhanced Receiver Type 1 under 2 receive antenna conditions. | 3-1 |

Table 5.2.2.1.1.0-2: Test Parameters for Testing

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
|  | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
| PDSCH configuration | PRB bundling size |  | 4 for Test 1-1 2 for other tests |
|  | Resource allocation type |  | Test 1-2: Type 1 with start RB = 23, LRBs = 6 Other tests: Type 0 |
|  | RBG size |  | Test 1-2: N/A Other tests: Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
|  | DMRS Type |  | Type 1 |
| PDSCH DMRS configuration | Number of additional DMRS |  | 2 for Tests 1-1, 1-5, 1-6, 1-7 1 for other tests |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | Test 1-5, 1-6, 1-7: 10 for CSI-RS resource 1,2,3,4. Other tests: Table 5.2-1. |
|  | CSI-RS offset | Slots | Test 1-5, 1-6, 1-7: 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4. Other tests: Table 5.2-1. |
| Number of HARQ Processes |  |  | 8 for Test 1-4 4 for other tests |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | 2 |

Table 5.2.2.1.1.0-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | 10 / 15 | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | 70 | -0.8 |
| 1-2 | R.PDSCH.1-1.2 FDD | 10 / 15 | QPSK, 0.30 | TDLC300-100 | 2x2, ULA Low | 70 | 0.2 |
| 1-3 | R.PDSCH.1-4.1 FDD | 10 / 15 | 256QAM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | 24.6 |
| 1-4 | R.PDSCH.1-2.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 30 | 1.1 |
| 1-5 | R.PDSCH.1-8.1 FDD | 10 / 15 | 16QAM, 0.48 | HST-750 | 1x2 | 70 | 6.2 |
| 1-6 | R.PDSCH.1-8.2 FDD | 10 / 15 | 64QAM, 0.43 | HST-972 | 1x2 | 70 | 9.9 |
| 1-7 | R.PDSCH.1-8.1 FDD | 10 / 15 | 16QAM, 0.48 | TDLC300-600 | 2x2 | 70 | 8.6 |
| 1-8 | R.PDSCH.1-17.1 FDD | 10 / 15 | 1024QAM, 0.79 | TDLD30-5 | 2x2, ULA Low | 70 | 29.5 |

Table 5.2.2.1.1.0-4: Minimum performance for Rank 2

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD | 10 / 15 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | 70 | 19.4 |
| 2-2 | R.PDSCH.2-1.1 FDD | 20 / 30 | 64QAM, 0.50 | TDLA30-10 | 2x2, ULA Low | 70 | 19.7 |

Table 5.2.2.1.1.0-5: Minimum performance for Rank 2 and Enhanced Receiver Type 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 3-1 | R.PDSCH.1-2.2 FDD | 10 / 15 | 16QAM, 0.48 | TDLA30-10 | 2x2, ULA Medium | 70 | 17.6 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.1.1.

5.2.2.1.1_1	2Rx FDD FR1 PDSCH mapping Type A performance - 2x2 MIMO with baseline receiver for both SA and NSA

Editor's Note: This test cases is incomplete in following aspects:

-	SNR in test requirements table is within square brackets for test point 1-6, 1-7.

5.2.2.1.1_1.1	Test purpose

To verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 1 and Rank 2 scenarios.

5.2.2.1.1_1.2	Test applicability

This test applies to all types of UE release 15 and forward supporting NR/5GC.

This test also applies to all types of UE release 15 and forward supporting EN-DC.

5.2.2.1.1_1.3	Test description

5.2.2.1.1_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Table 5.2.2.1.1.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for NR/5GC with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for EN-DC according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.1_1.3.3.

5.2.2.1.1_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

4.	Repeat steps from 1 to 3 for each subtest in Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 as appropriate.

5.2.2.1.1_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clauses 4.6.1 and 5.4.2.

5.2.2.1.1_1.3.3_1	Message exceptions for NR/5GC

Table 5.2.2.1.1_1.3.3_1-1: PDSCH-Config

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-26 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-Config ::= SEQUENCE { |  |  |  |
| prb-BundlingType CHOICE { |  |  |  |
| staticBundling SEQUENCE { |  |  |  |
| bundleSize | n4 | n4 for test 1-1 | test 1-1 |
|  | Not present | n2 will be used by default | test point other than test 1-1 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.1.1_1.3.3_1-2: DMRS-DownlinkConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-24 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-DownlinkConfig ::= SEQUENCE { |  |  |  |
| dmrs-AdditionalPosition | pos2 | For test 1-1, 1-5, 1-6 and 1-7 |  |
|  | pos1 | For other tests |  |
| } |  |  |  |

Table 5.2.2.1.1_1.3.3_1-3: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-25 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n8, n4 | n8 for Test 1-4 n4 for other tests |  |
| } |  |  |  |

Table 5.2.2.1.1_1.3.3_1-4: CSI-ResourcePeriodicityAndOffset for CSI Tracking

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots10 | 1 (for CSI-RS resources 1 and 2) 2 (for CSI-RS resources 3 and 4) | For test 1-5, 1-6, 1-7: CSI-RS offset: 1 for CSI-RS resources 1 and 2 2 for CSI-RS resources 3 and 4 CSI-RS periodicity: 10 slots |  |
| slots40 | 20 (for CSI-RS resources 1 and 2) 21 (for CSI-RS resources 3 and 4) | For test 2-2: CSI-RS offset: 20 for CSI-RS resources 1 and 2 21 for CSI-RS resources 3 and 4 CSI-RS periodicity: 40 slots |  |
| slots20 | 10 (for CSI-RS resources 1 and 2) 11 (for CSI-RS resources 3 and 4) | For other tests: CSI-RS offset: 10 for CSI-RS resources 1 and 2 11 for CSI-RS resources 3 and 4 CSI-RS periodicity: 20 slots |  |
| } |  |  |  |

5.2.2.1.1_1.3.3_2	Message exceptions for EN-DC

Same as 5.2.2.1.1_1.3.3_1

5.2.2.1.1_1.4	Test requirement

Tables 5.2.2.1.1_1.4-1 and 5.2.2.1.1_1.4-2 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A 3.2.1 for each throughput test shall meet or exceed the specified value in Table 5.2.2.1.1_1.4-1 and Table 5.2.2.1.1_1.4-2 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.1_1.4-1: Test Requirements for Rank 1

| Test num. | Reference channel | Modulation format | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 FDD | QPSK, 0.30 | TDLB100-400 | 2x2, ULA Low | 70 | 0.1 |
| 1-2 | R.PDSCH.1-1.2 FDD | QPSK, 0.30 | TDLC300-100 | 2x2, ULA Low | 70 | 1.1 |
| 1-3 | R.PDSCH.1-4.1 FDD | 256AM, 0.82 | TDLA30-10 | 2x2, ULA Low | 70 | 25.6 |
| 1-4 | R.PDSCH.1-2.1 FDD | 16QAM, 0.48 | TDLC300-100 | 2x2, ULA Low | 30 | 2 |
| 1-5 | R.PDSCH.1-8.1 FDD | 16QAM, 0.48 | HST-750 | 1x2 | 70 | 7.1 |
| 1-6 | R.PDSCH.1-8.2 FDD | 64QAM, 0.43 | HST-972 | 1x2 | 70 | [10.5] |
| 1-7 | R.PDSCH.1-8.1 FDD | 16QAM, 0.48 | TDLC300-600 | 2x2 | 70 | [9.5] |

Table 5.2.2.1.1_1.4-2: Test Requirements for Rank 2

| Test num. | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 2-1 | R.PDSCH.1-3.1 FDD | 64QAM, 0.51 | TDLA30-10 | 2x2, ULA Low | 70 | 20.4 |
| 2-2 | R.PDSCH.2-1.1 FDD | 64QAM, 0.51 | TDLA30-10 | 2x2, ULA Low | 70 | 20.7 |

<<End of change>>