3GPP TSG-RAN5 Meeting #98	R5-231698

# Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0645 | rev | 1 | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of HST DPS TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test, NR_HST-UEConTest |  |  |  |  |  | Date: |  |  | 2022-02-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The description of the HST DPS TC test procedures is unclear, which leads to ambiguity on how to implement the TCs. Therefor the test procedure needs to be updated to clarify open points discussed in previous meetings. The test procedure needs to be split for the different test points, since the requirements and procedures are different. Clarification is needed on which signals to send and how to activate the different TCI states. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update test procedures for all HST DPS test cases. Separate procedures for tests 1-1 and 1-2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HST-DPS TCs remain ambiguous. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.10_1.3.2, 5.2.2.2.10_1.3.2, 5.2.3.1.10_1.3.2, 5.2.3.2.10_1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections omitted >

##### 5.2.2.1.10	2Rx FDD FR1 HST DPS performance

5.2.2.1.10.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.1.10.0-3, with the test parameters defined in Table 5.2.2.1.10.0-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2.2.1.10.0-1.

Table 5.2.2.1.10.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-DPS scenario defined in B.3.3 | 1-1, 1-2 |

Table 5.2.2.1.10.0-2: Test Parameters for Testing

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #3 |
| NZP CSI-RS for CSI acquisition | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Note 1: SSB # (k mod 2) , CSI-RS (for tracking) resource set # ((k mod 2) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) are transmitted by kth RRH. For Test 1-1, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc+TfirstTRS+TTRS proc, 0] to slot# 2k+1n+THARQ+TMAC proc, PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc, 0] to slot# 2k+1n+THARQ+TMAC proc Where k=0, 1, 2… is the RRH number, n = 2520 is half of the number of slots between two RRH, THARQ = 2 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 3 is the number of slots for MAC CE processing, TfirstTRS = 6 is the number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE, TTRS proc = 2 is the number of slots for TRS processing. |  |  |  |  |

Table 5.2.2.1.10.0-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.4 FDD | 10 / 15 | 64QAM, 0.43 | HST-DPS | 1 | 2x2 | 70 | 13.4 |
| 1-2 | R.PDSCH.1-8.4 FDD | 10 / 15 | 64QAM, 0.43 | HST-DPS | 2 | 2x2 | 70 | 13.4 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.1.10.

###### 5.2.2.1.10_1	2Rx FDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.1.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.1.10_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.2.1.10_1.3	Test description

5.2.2.1.10_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Table 5.2.2.1.10.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.10_1.3.3.

5.2.2.1.10_1.3.2	Test procedure

Test 1-1:

1.		Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.10_1.4-1 as appropriate.

2.	SS is configured to transmit SSB and CSI-RS continuously and schedule PDSCH and PDCCH transmission according to Note 1 in 5.2.2.1.10_1.4-1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.10_1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

Note: All TCI states are known to the UE through configuration inside RrcReconfiguration. There is no need to configure additional L1-RSRP measurements.

3. 	Send MAC CE command “TCI State Indication for UE-specific PDCCH” according to the timing described in Note 1 of table 5.2.2.1.10_1.4-1 to switch from active TCI state 0 to 1 for PDCCH and vice versa periodically. PDSCH is automatically associated with TCI state 0 or 1 as tci-PresentInDCI is not present. TCI states 3 and 4 for SSBs are automatically activated through relation of QCL-Info in NZP CSI-RS.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

Test 1-2:

1.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.10_1.4-1 as appropriate.

2.	SS activates TCI state 0 and TCI 1 for PDSCH at the same time via MAC CE command “TCI States Activation/Deactivation for UE-specific PDSCH”.

3.	SS is configured to transmit SSB and CSI-RS continuously and schedule PDSCH and PDCCH transmission according to Note 1 in 5.2.2.1.10_1.4-1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.10_1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

Note: All TCI states are known to the UE through configuration inside RrcReconfiguration. There is no need to configure additional L1-RSRP measurements.

4. 	Send MAC CE command “TCI State Indication for UE-specific PDCCH” according to the timing described in Note 1 of table 5.2.2.1.10_1.4-1 to switch from active TCI state 0 to 1 for PDCCH and vice versa periodically. PDSCH is automatically associated with TCI state 0 or 1 as tci-PresentInDCI is not present. TCI states 3 and 4 for SSBs are automatically activated through relation of QCL-Info in NZP CSI-RS.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

#### < Unchanged sections omitted >

##### 5.2.2.2.10	2Rx TDD FR1 HST DPS performance

5.2.2.2.10.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.2.10.0-3, with the test parameters defined in Table 5.2.2.2.10.0-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.2.2.10.0-1.

Table 5.2.2.2.10.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-DPS scenario defined in B.3.3 | 1-1, 1-2 |

Table 5.2.2.2.10.0-2: Test Parameters for Testing

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
|  | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
| PDSCH configuration | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
|  | DMRS Type |  |  | Type 1 |
| PDSCH DMRS configuration | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 |
|  |  |  |  | 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #2 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 |
|  |  |  |  | l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 |
|  |  |  |  | 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #3 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: SSB # (k mod 2) , CSI-RS (for tracking) resource set # ((k mod 2) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) are transmitted by kth RRH. For Test 1-1, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc+TfirstTRS+TTRS proc, 0] to slot# 2k+1n+THARQ+TMAC proc, PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc, 0] to slot# 2k+1n+THARQ+TMAC proc Where k=0, 1, 2… is the RRH number, n = 5040 is half of the number of slots between two RRH, THARQ = 8 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 6 is the number of slots for MAC CE processing, TfirstTRS = 7 is the number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE, TTRS proc = 4 is the number of slots for TRS processing. |  |  |  |  |

Table 5.2.2.2.10.0-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-10.5 TDD | 40 / 30 | 64QAM, 0.43 | HST-DPS | 1 | 2x2 | 70 | 13.0 |
| 1-2 | R.PDSCH.2-10.5 TDD | 40 / 30 | 64QAM, 0.43 | HST-DPS | 2 | 2x2 | 70 | 13.0 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.2.2.10.

###### 5.2.2.2.10_1	2Rx TDD FR1 HST-DPS performance - 2x2 MIMO with baseline receiver for both SA and NSA

5.2.2.2.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.2.2.10_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.2.2.10_1.3	Test description

5.2.2.2.10_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.2 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Table 5.2.2.2.10.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.2.10_1.3.3.

5.2.2.2.10_1.3.2	Test procedure

Test 1-1:

1.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.10_1.4-1 as appropriate.

2.	SS is configured to transmit SSB and CSI-RS continuously and schedule PDSCH and PDCCH transmission according to Note 1 in 5.2.2.1.10_1.4-1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.10_1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

Note: All TCI states are known to the UE through configuration inside RrcReconfiguration. There is no need to configure additional L1-RSRP measurements.

3. 	Send MAC CE command “TCI State Indication for UE-specific PDCCH” according to the timing described in Note 1 of table 5.2.2.1.10_1.4-1 to switch from active TCI state 0 to 1 for PDCCH and vice versa periodically. PDSCH is automatically associated with TCI state 0 or 1 as tci-PresentInDCI is not present. TCI states 3 and 4 for SSBs are automatically activated through relation of QCL-Info in NZP CSI-RS.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

Test 1-2:

1.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.10_1.4-1 as appropriate.

2.	SS activates TCI state 0 and TCI 1 for PDSCH at the same time via MAC CE command “TCI States Activation/Deactivation for UE-specific PDSCH”.

3.	SS is configured to transmit SSB and CSI-RS continuously and schedule PDSCH and PDCCH transmission according to Note 1 in 5.2.2.1.10_1.4-1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.10_1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

Note: All TCI states are known to the UE through configuration inside RrcReconfiguration. There is no need to configure additional L1-RSRP measurements.

4. 	Send MAC CE command “TCI State Indication for UE-specific PDCCH” according to the timing described in Note 1 of table 5.2.2.1.10_1.4-1 to switch from active TCI state 0 to 1 for PDCCH and vice versa periodically. PDSCH is automatically associated with TCI state 0 or 1 as tci-PresentInDCI is not present. TCI states 3 and 4 for SSBs are automatically activated through relation of QCL-Info in NZP CSI-RS.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

#### < Unchanged sections omitted >

##### 5.2.3.1.10	4Rx FDD FR1 HST DPS performance

5.2.3.1.10.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.3.1.10.0-3, with the test parameters defined in Table 5.2.3.1.10.0-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2.3.1.10.0-1.

Table 5.2.3.1.10.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-DPS scenario defined in B.3.3 | 1-1, 1-2 |

Table 5.2.3.1.10.0-2: Test Parameters for Testing

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | FDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | 12 |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 1,2,3,4. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #2 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 10 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #3 |
| NZP CSI-RS for CSI acquisition | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 20 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 4 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | 2 |
| Note 1: SSB # (k mod 2) , CSI-RS (for tracking) resource set # ((k mod 2) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) are transmitted by kth RRH. For Test 1-1, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy . PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# maxâ¡[2k-1n+1+THARQ+TMAC proc+TfirstTRS+TTRS proc, 0 to slot# , PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy . PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# maxâ¡[2k-1n+1+THARQ+TMAC proc, 0] to slot# 2k+1n+THARQ+TMAC proc Where k=0, 1, 2… is the RRH number, n = 2520 is half of the number of slots between two RRH, = 2 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc TMAC proc = 3 is the number of slots for MAC CE processing, = 6 is the number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE, TTRS proc TTRS proc = 2 is the number of slots for TRS processing. |  |  |  |  |

Table 5.2.3.1.10.0-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-8.4 FDD | 10 / 15 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.6 |
| 1-2 | R.PDSCH.1-8.4 FDD | 10 / 15 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.6 |

The normative reference for this requirement is TS 38.101-4 [5], clause 5.2.3.1.10.

###### 5.2.3.1.10_1	4Rx FDD FR1 HST-DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA

5.2.3.1.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.3.1.10_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.3.1.10_1.3	Test description

5.2.3.1.10_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.4 for TE diagram and clause A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Table 5.2.3.1.10.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.3.1.10_1.3.3.

5.2.3.1.10_1.3.2	Test procedure

Test 1-1:

1.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.10_1.4-1 as appropriate.

2.	SS is configured to transmit SSB and CSI-RS continuously and schedule PDSCH and PDCCH transmission according to Note 1 in 5.2.2.1.10_1.4-1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.10_1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

Note: All TCI states are known to the UE through configuration inside RrcReconfiguration. There is no need to configure additional L1-RSRP measurements.

3. 	Send MAC CE command “TCI State Indication for UE-specific PDCCH” according to the timing described in Note 1 of table 5.2.2.1.10_1.4-1 to switch from active TCI state 0 to 1 for PDCCH and vice versa periodically. PDSCH is automatically associated with TCI state 0 or 1 as tci-PresentInDCI is not present. TCI states 3 and 4 for SSBs are automatically activated through relation of QCL-Info in NZP CSI-RS.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

Test 1-2:

1.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.10_1.4-1 as appropriate.

2.	SS activates TCI state 0 and TCI 1 for PDSCH at the same time via MAC CE command “TCI States Activation/Deactivation for UE-specific PDSCH”.

3.	SS is configured to transmit SSB and CSI-RS continuously and schedule PDSCH and PDCCH transmission according to Note 1 in 5.2.2.1.10_1.4-1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.10_1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

Note: All TCI states are known to the UE through configuration inside RrcReconfiguration. There is no need to configure additional L1-RSRP measurements.

4. 	Send MAC CE command “TCI State Indication for UE-specific PDCCH” according to the timing described in Note 1 of table 5.2.2.1.10_1.4-1 to switch from active TCI state 0 to 1 for PDCCH and vice versa periodically. PDSCH is automatically associated with TCI state 0 or 1 as tci-PresentInDCI is not present. TCI states 3 and 4 for SSBs are automatically activated through relation of QCL-Info in NZP CSI-RS.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

#### < Unchanged sections omitted >

##### 5.2.3.2.10	4Rx TDD FR1 HST DPS performance

5.2.3.2.10.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.3.2.10.0-3, with the addition of test parameters in Table 5.2.3.2.10.0-2 and the downlink physical channel setup according to Annex C.3.1.

The test purposes are specified in Table 5.2.3.2.10.0-1.

Table 5.2.3.2.10.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify UE performance in the HST-DPS scenario defined in B.3.3 | 1-1, 1-2 |

Table 5.2.3.2.10.0-2: Test parameters

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| Duplex mode |  |  |  | TDD |
| Active DL BWP index |  |  |  | 1 |
| PDCCH configuration | TCI state |  |  | Note 1 |
| PDSCH configuration | Mapping type |  |  | Type A |
|  | k0 |  |  | 0 |
|  | Starting symbol (S) |  |  | 2 |
|  | Length (L) |  |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  |  | 1 |
|  | PRB bundling type |  |  | Static |
|  | PRB bundling size |  |  | 2 |
|  | Resource allocation type |  |  | Type 0 |
|  | RBG size |  |  | Config2 |
|  | VRB-to-PRB mapping type |  |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  |  | N/A |
|  | TCI state |  |  | Note 1 |
| PDSCH DMRS configuration | DMRS Type |  |  | Type 1 |
|  | Number of additional DMRS |  |  | 2 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  |  | 1 |
| CSI-RS for tracking | Resource set #1 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 |
|  |  |  |  | l0 = 9 for CSI-RS resource 2 and 4 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 1,2,3,4 |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 1 and 2 |
|  |  |  |  | 2 for CSI-RS resource 3 and 4 |
|  |  | QCL info |  | TCI state #2 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
|  | Resource set #2 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 6 for CSI-RS resource 5 and 6 |
|  |  |  |  | l0 = 10 for CSI-RS resource 7 and 8 |
|  |  | CSI-RS periodicity | Slots | 20 for CSI-RS resource 5,6,7,8. |
|  |  | CSI-RS offset | Slots | 1 for CSI-RS resource 5 and 6 |
|  |  |  |  | 2 for CSI-RS resource 7 and 8 |
|  |  | QCL info |  | TCI state #3 |
|  |  | Frequency Occupation |  | Start PRB 0 |
|  |  |  |  | Number of PRB = 52 |
| NZP CSI-RS for CSI acquisition | Resource set #3 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 12 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #0 |
|  | Resource set #4 | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 13 |
|  |  | CSI-RS periodicity | Slots | 40 |
|  |  | CSI-RS offset | Slots | 0 |
|  |  | QCL info |  | TCI state #1 |
| TCI state #0 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 1 from 'CSI-RS for tracking Resource set #1' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #1 | Type 1 QCL information | CSI-RS resource |  | CSI-RS resource 5 from 'CSI-RS for tracking Resource set #2' configuration |
|  |  | QCL Type |  | Type A |
|  | Type 2 QCL information | CSI-RS resource |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #2 | Type 1 QCL information | SSB index |  | SSB #0 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| TCI state #3 | Type 1 QCL information | SSB index |  | SSB #1 |
|  |  | QCL Type |  | Type C |
|  | Type 2 QCL information | SSB index |  | N/A |
|  |  | QCL Type |  | N/A |
| Number of HARQ Processes |  |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: SSB # (k mod 2) , CSI-RS (for tracking) resource set # ((k mod 2) + 1) and CSI-RS (for CSI acquisition) resource set # ((k mod 2) + 3) are transmitted by kth RRH. For Test 1-1, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc+TfirstTRS+TTRS proc, 0] to slot# 2k+1n+THARQ+TMAC proc, PDCCH and PDSCH are DTXed in other slots in which throughput statistics are not considered. For Test 1-2, TCI state switching command scheduled by MAC CE with MCS 4 is transmitted in slot #i that satisfy modi,2n=n. PDCCH and PDSCH associated with TCI # (k mod 2) is transmitted by kth RRH from slot# max⁡[2k-1n+1+THARQ+TMAC proc, 0] to slot# 2k+1n+THARQ+TMAC proc Where k=0, 1, 2… is the RRH number, n = 5040 is half of the number of slots between two RRH, THARQ = 8 is the number of slots between PDSCH and corresponding HARQ-ACK information, TMAC proc = 6 is the number of slots for MAC CE processing, TfirstTRS = 7 is the number of slots to first TRS transmission occasion after MAC CE command is decoded by the UE, TTRS proc = 4 is the number of slots for TRS processing. |  |  |  |  |

Table 5.2.3.2.10.0-3: Minimum performance for HST-DPS

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Number of active PDSCH TCI states | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.2-10.5 TDD | 40 / 30 | 64QAM, 0.43 | HST-DPS | 1 | 2x4 | 70 | 10.2 |
| 1-2 | R.PDSCH.2-10.5 TDD | 40 / 30 | 64QAM, 0.43 | HST-DPS | 2 | 2x4 | 70 | 10.2 |

###### 5.2.3.2.10_1	4Rx TDD FR1 HST DPS performance - 2x4 MIMO with baseline receiver for both SA and NSA

5.2.3.2.10_1.1	Test purpose

To verify UE performance in the HST-DPS scenario defined in B.3.3 and with different channel models, MCSs and number of MIMO layers for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput and as well verify the HARQ soft combining with default baseline receiver configuration, for Rank 2 scenarios.

5.2.3.2.10_1.2	Test applicability

This test applies to all types of NR UE release 15 and forward.

This test also applies to all types of EUTRA UE release 15 and forward supporting EN-DC.

5.2.3.2.10_1.3	Test description

5.2.3.2.10_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 5.2.2.

For EN-DC within FR1 operation, setup the LTE link according to Annex D:

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.4 for TE diagram and clause A.3.2.5 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1 and Table 5.2.3.2.10.0-2 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without Release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.3.2.10_1.3.3.

5.2.3.2.10_1.3.2	Test procedure

Test 1-1:

1.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.10_1.4-1 as appropriate.

2.	SS is configured to transmit SSB and CSI-RS continuously and schedule PDSCH and PDCCH transmission according to Note 1 in 5.2.2.1.10_1.4-1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.10_1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

Note: All TCI states are known to the UE through configuration inside RrcReconfiguration. There is no need to configure additional L1-RSRP measurements.

3. 	Send MAC CE command “TCI State Indication for UE-specific PDCCH” according to the timing described in Note 1 of table 5.2.2.1.10_1.4-1 to switch from active TCI state 0 to 1 for PDCCH and vice versa periodically. PDSCH is automatically associated with TCI state 0 or 1 as tci-PresentInDCI is not present. TCI states 3 and 4 for SSBs are automatically activated through relation of QCL-Info in NZP CSI-RS.

4.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

Test 1-2:

1.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.1.10_1.4-1 as appropriate.

2.	SS activates TCI state 0 and TCI 1 for PDSCH at the same time via MAC CE command “TCI States Activation/Deactivation for UE-specific PDSCH”.

3.	SS is configured to transmit SSB and CSI-RS continuously and schedule PDSCH and PDCCH transmission according to Note 1 in 5.2.2.1.10_1.4-1. SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Tables 5.2.2.1.10_1.4-1. The SS sends downlink MAC padding bits on the DL RMC.

Note: All TCI states are known to the UE through configuration inside RrcReconfiguration. There is no need to configure additional L1-RSRP measurements.

4. 	Send MAC CE command “TCI State Indication for UE-specific PDCCH” according to the timing described in Note 1 of table 5.2.2.1.10_1.4-1 to switch from active TCI state 0 to 1 for PDCCH and vice versa periodically. PDSCH is automatically associated with TCI state 0 or 1 as tci-PresentInDCI is not present. TCI states 3 and 4 for SSBs are automatically activated through relation of QCL-Info in NZP CSI-RS.

5.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

#### < End of changes >