3GPP TSG RAN meeting #99										RP-230258

Rotterdam, Netherlands, March 20th – 23rd  2023

## Status Report to TSG

Agenda item:			9.3.2.6

| WI / SI Name | WI on NR Support for UAV (Uncrewed Aerial Vehicles) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_UAV |  |  |  |  |
| Unique ID | 941105 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223545 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 40% | Performance Part: | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Jedrzej Stanczak |
|  | Company | Nokia, Nokia Shanghai Bell |
|  | Email | jedrzej.stanczak@nokia.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

RAN1#112:

Agreement RAN1 only considers potential UE UAV beamforming capabilities for FR1 with directional antennas at UE side in Rel-18 if such capabilities don’t impact RAN4 requirements. Agreement If new UE UAV beamforming capabilities for FR1 with directional antennas at UE side are supported, Rel-17 unified TCI framework is considered as baseline. Agreement Multi-TRP beamforming is not supported for UAV UEs in FR1 in Rel-18. Agreement UE UAV beamforming for FR1 based on beam switching among fixed directional antennas is supported in Rel-18. - Note 1: new UE capabilities may not be necessary to support beam switching among fixed directional antennas - Note 2: no RAN4 specification impact is assumed - FFS: whether updating (e.g. extending to FR1) legacy UE capabilities is needed, and it is not precluded if it is needed - FFS: whether/how specification may be impacted

#### 2.1.2	Remaining Open issues

Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling:

- FR1 with directional antenna at UE side

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#121:

Agreements: 1. When event H1 or H2 triggers, the content of the measurement report is configurable by the network (i.e. it can contain UAV UEs height, location information and/or RSRP/RSRQ measurement results). FFS whether UAV UE’s height is mandatorily reported and which parameter/IE is used for height reporting. 2. Joint use of height-dependent condition and RSRP/RSRQ/SINR-based condition for measurement report triggering is supported in NR Rel-18 UAV. The combination of existing events will be used 3. Height-dependent parameter scaling is not supported as a part of Rel-18 NR 4. Do not extend the Number of triggering cells mechanism to apply to the inter-RAT scenario, i.e. event B1 and B2 triggering 5. Do not restrict the applicability of Number of triggering cells mechanism to FR1 only. In other words, the Number of triggering cells mechanism is applicable to FR1 and FR2 (up to network configuration). 6. The UE shall not ignore or bypass the Number of triggering cells mechanism, once configured. 7. Do not introduce the use of a “numberOfTriggeringBeams” mechanism. 8. Do not introduce an alternative mechanism to the Number of triggering cells mechanism. 9. Do not introduce an additional mechanism based on Number of changed cells. 10. For the purpose of interference control (i.e. for number of trigger cells), do not introduce a prohibit timer mechanism. 11. Report on leave is not triggered by a cell that was not previously included in the measurement report for the number of triggering cell.

Agreements: 1. Support configuring height-dependent more-than-one configurations targeting measurement and measurement reporting enhancement. UE applies corresponding configuration based on the UE height. The proposed solutions should aim at avoiding RAN4 impacts. FFS how this would be configured (i.e. different MO configurations or different parameters FFS Exact parameters and details.

Agreements: 1. The granularity of flightpath timestamp is 1s. 2. Timestamp in flightpath is encoded using AbsoluteTimeInfo-r16 IE

Agreements: - PC5-U is used to support BRID for UAV - Support both in-coverage and out-of-coverage scenarios - Mode 2 will be supported. FFS whether further mode 1 will be supported. - FFS whether separate pools are needed - FFS whether current configurations can support UAV requirements

#### 2.2.2	Remaining Open issues

Specify the following enhancements on measurement reports

- UE-triggered measurement report based on configured height thresholds

- Reporting of height, location and speed in measurement report

- Flight path reporting

- Measurement reporting based on a configured number of cells (i.e. larger than one) fulfilling the triggering criteria simultaneously

Study UE capability signaling to indicate UAV beamforming capabilities and, if necessary, RRC signaling:

- FR1 with directional antenna at UE side

Study and specify, if needed, enhancements to NR PC5 and LTE PC5 to support UAV identification broadcast.

Specify the signaling to support subscription-based aerial-UE identification.

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3#119:

Endorsed TDocs: - Stage 2 (TS 38.300) : draft CR in R3-230063 - Stage 3 (TS 38.413) : BL CR in R3-230065 - Stage 3 (TS 38.423) : BL CR in R3-230066

#### 2.3.2	Remaining Open issues

RAN3 has completed their UAV work in 99% (based on RAN3 assessment done at RAN3#119).

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SA2

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

TDocs submitted to RAN1:

- R1-2300132	UE capability and RRC signaling for UAV beamforming	Huawei, HiSilicon

- R1-2300482	Discussion on UE capability and RRC signaling for UAV beamforming	vivo

- R1-2300519	Discussion on UE capability and RRC signaling for UAV beamforming	Lenovo

- R1-2300564	Discussion on UE capability and RRC signaling for UAV beamforming	xiaomi

- R1-2300708	Discussion on UE capability for UAV beamforming	ZTE

- R1-2300713	Discussion on UE capability and RRC signaling for UAV beamforming	Nokia, Nokia Shanghai Bell

- R1-2301295	Discussion on UE capability and RRC signaling for UAV beamforming	Samsung

- R1-2301377	On UE capablity and RRC signaling for UAV beamforming	Apple

- R1-2301444	Discussion of UE capability and RRC signaling for UAV beamforming	Qualcomm Incorporated

- R1-2301522	Discussion on UE capability and RRC signaling for UAV beamforming	NTT DOCOMO, INC.

- R1-2301629	On UAV beamforming capabilities	Ericsson

- R1-2301953	Summary#1 of discussion on UAV capability and RRC signaling for UAV	Moderator (Nokia)

- R1-2301954	Summary#2 of discussion on UAV capability and RRC signaling for UAV	Moderator (Nokia)

TDocs submitted to RAN2:

- R2-2300006	OG0022_LS-MITRE-Engenuity Open Generation DAA input_PC5_DAA_RID_PRS OG0022 (contact: vivo)	MITRE Engenuity Open Generation 5G Consortium	LS in	NR_UAV-Core

- R2-2300061	LS response to ETSI TC LI on Location Services for Drones (RP-223555; contact: Ericsson)	RAN	LS in	Rel-18

- R2-2300080	LS on PC5 based Detect and Avoid mechanism (S2-2301854; contact: LGE)	SA2	LS in	Rel-18	FS_UAS_Ph2

- R2-2300478	Uncrewed Aerial Vehicles in Rel-18 - Updated Workplan	Nokia, Nokia Shanghai Bell	Work Plan	Rel-18	NR_UAV-Core

- R2-2301875	Discussion on the LS from SA2 for NR UAV	CATT	discussion	Rel-18	NR_UAV-Core

- R2-2300369	Interference control for UAV	Intel Corporation	discussion	Rel-18	NR_UAV-Core

- R2-2300371	mobility control for UAV	Intel Corporation	discussion	Rel-18	NR_UAV-Core

- R2-2300479	Report from [Post120][312][UAV] Mobility Control for UAVs (Nokia)	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2300583	Measurement and reporting enhancements	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

- R2-2300595	On Interference Reporting for UAV UEs	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2300746	Measurement reporting enhancement in UAV	Apple	discussion	Rel-18	NR_UAV

- R2-2300852	Discussion on Measurement Reports Enhancements	NEC Europe Ltd	discussion	Rel-18	NR_UAV-Core

- R2-2300897	UAV measurement reports 	Ericsson	discussion	Rel-18

- R2-2300941	Discussion on measurement reporting for NR UAV	Sharp	discussion

- R2-2300972	measurement enhancement for NR UAV	Lenovo	discussion	Rel-18

- R2-2300991	Measurement reporting for mobility and interference control	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

- R2-2300997	Interference Control for UAVs (Huawei)_Summary_v01_Rapporteur	Huawei Technologies R&D UK	discussion	Rel-18

- R2-2301095	Considerations about UAV mobility and user consent	Sony	discussion	Rel-18	NR_UAV

- R2-2301220	On UAV mobility and interference control	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

- R2-2301227	Measurement Reporting for NR UAV	CMCC	discussion	Rel-18	NR_UAV-Core

- R2-2301397	Discussion on measurement reporting for NR UAV	Xiaomi	discussion	Rel-18	NR_UAV-Core

- R2-2301592	On measurement reporting enhancements in NR UAV	Samsung Electronics Austria	discussion	Rel-18	NR_UAV-Core

- R2-2301677	Discussion on measurement reporting enhancement for NR UAV	vivo	discussion	Rel-18	NR_UAV-Core

- R2-2301765	Further discussion on NR support for UAV	NTT DOCOMO, INC.	discussion	Rel-18

- R2-2301771	Measurement Report Enhancement	LG Electronics	discussion	Rel-18

- R2-2301772	Number of changed cell related Enhancement	LG Electronics	discussion	Rel-18

- R2-2301807	Discussion on measurement reporting for NR UAV	China Telecom	discussion

- R2-2300368	Flight path update triggering for UAV	Intel Corporation	discussion	Rel-18	NR_UAV-Core

- R2-2300480	On Flight Path Plan (FPP) and Height-dependent Configurations	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2300584	Flight path reporting enhancements	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

- R2-2300747	Flight path reporting in UAV	Apple	discussion	Rel-18	NR_UAV

- R2-2300853	Discussion on Flight Path Reporting	NEC Europe Ltd	discussion	Rel-18	NR_UAV-Core

- R2-2300905	Flight path reporting 	Ericsson	discussion	Rel-18

- R2-2300942	Discussion on flight path reporting for NR UAV	Sharp	discussion

- R2-2300973	Remaining issues of flight path reporting for NR UAV	Lenovo	discussion	Rel-18

- R2-2300992	Flight path reporting	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

- R2-2301221	On flight path reporting	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

- R2-2301228	Flight path Reporting for NR UAV	CMCC	discussion	Rel-18	NR_UAV-Core

- R2-2301367	Flight path notification and reporting for UAV	InterDigital	discussion	Rel-18	NR_UAV-Core

- R2-2301387	Discussion on flight path reporting	Samsung	discussion	Rel-18	NR_UAV-Core

- R2-2301398	Discussion on flight path reporting for NR UAV	Xiaomi	discussion	Rel-18	NR_UAV-Core

- R2-2301676	Discussion on flight path reporting for NR UAV	vivo	discussion	Rel-18	NR_UAV-Core

- R2-2301810	Discussion on flight path reporting for NR UAV	China Telecom	discussion

- R2-2301876	Leftover Issues on Flight Path Reporting	CATT	discussion	Rel-18	NR_UAV-Core

- R2-2301883	Consideration on flight path reporting of NR support for UAV	DENSO CORPORATION	discussion

- R2-2300333	RAN2 aspects of PC5-based BRID and DAA support	Qualcomm Incorporated	discussion	Rel-18	NR_UAV-Core

- R2-2300481	RAN2 Impacts of PC5-based BRID and DAA for UAVs	Nokia, Nokia Shanghai Bell	discussion	Rel-18	NR_UAV-Core

- R2-2300538	On broadcasting UAV identification	Ericsson España S.A.	discussion	Rel-18

- R2-2300748	UAV identification broadcast over PC5	Apple	discussion	Rel-18	NR_UAV

- R2-2300854	Considerations on Enhancements for UAV identification broadcast	NEC Europe Ltd	discussion	Rel-18	NR_UAV-Core

- R2-2300974	Discussion on broadcasting remote id for UAV	Lenovo	discussion	Rel-18

- R2-2300993	Consideration on UAV remote identification broadcast	Huawei, HiSilicon	discussion	Rel-18	NR_UAV-Core

- R2-2301096	UAV identification broadcast	Sony	discussion	Rel-18	NR_UAV

- R2-2301169	Discussion on UAV identification broadcast	Samsung	discussion	Rel-18	NR_UAV-Core

- R2-2301222	On UAV identification broadcast	ZTE Corporation, Sanechips	discussion	Rel-18	NR_UAV-Core

- R2-2301229	UAV identification broadcast	CMCC	discussion	Rel-18	NR_UAV-Core

- R2-2301418	UAV Broadcast of Remote IDentification and Detect And Avoid	Beijing Xiaomi Mobile Software	discussion	Rel-18	NR_UAV-Core

- R2-2301678	Discussion on UAV identification broadcast	vivo	discussion	Rel-18	NR_UAV-Core

- R2-2301737	Draft Reply LS on PC5 based Detect and Avoid mechanism	LG Electronics France	LS out	Rel-18	FS_UAS_Ph2	To:SA2

- R2-2301877	RAN2 impact analysis to support UAV ID Broadcast	CATT	discussion	Rel-18	NR_UAV-Core

TDocs submitted to RAN3:

- R3-230063	Draft CR to 38.300 on NR support for UAV	Nokia, Nokia Shanghai Bell, Huawei, Intel Corporation, ZTE, Ericsson, Samsung, CATT, Qualcomm, Deutsche Telekom, NEC

- R3-230065	Introduction of Aerial authorization information	Ericsson, AT&T, NTT DOCOMO INC, Qualcomm Incorporated, Intel Corporation, Samsung, NEC, Nokia, Nokia Shanghai Bell, Huawei, ZTE, CATT

- R3-230066	NR support for UAV over Xn	Huawei, China Unicom, China Telecom, CATT, Intel Corporation, ZTE, Ericsson, Qualcomm, Nokia, Nokia Shanghai Bell, Samsung

- R3-230196	Introduction of Aerial authorization information	Ericsson, AT&T, NTT DOCOMO INC, Qualcomm Incorporated, Intel Corporation, Samsung, NEC, Nokia, Nokia Shanghai Bell, Huawei, ZTE, CATT

- R3-230291	Further discussion on NR support for UAV	Huawei

- R3-230448	Uncrewed Aerial Vehicles in Rel-18 - Updated Workplan	Nokia, Nokia Shanghai Bell

- R3-230511	NR support for UAV WI	Ericsson

- R3-230737	Discussion on UAV left issues with draft LS to RAN2 and SA2	ZTE

- R3-230788	LS on RAN dependency for UAS	SA2, LGE

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template