3GPP TSG RAN meeting #99								                       RP-230260

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item:	9.3.2.2 - Core part: Mobile Terminated-Small Data Transmission (MT-SDT) for NR

| WI / SI Name | Mobile Terminated-Small Data Transmission (MT-SDT) for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: - |
| Acronym | NR_MT_SDT |  |  |  |  |
| Unique ID | 941000 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222993 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: n/a | Core part: 09/2023 | Performance part: n/a | Testing part: n/a |  |
| Overall Completion level | Study Item: n/a | Core part: Overall: 37% RAN2: 50% RAN3: 25% | Performance Part: n/a | Testing part: n/a |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN2 |
| --- | --- | --- |
| Rapporteur | Name | Eswar Vutukuri |
|  | Company | ZTE Corporation |
|  | Email | eswar.vutukuri@zte.com.cn |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

RAN2#121

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

  - FFS 3a: Check for DVT (if UL data becomes available in UL)

  - 3b: Check for SDT RSRP threshold

  - 3c: Check for TA validation before selecting CG (if applicable)

  - 3d: Check for SSB level RSRP threshold for CG resource (if applicable)

- When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

- RBs configured for SDT are common for MO-SDT and MT-SDT

- If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

- To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

#### 2.2.2	Remaining Open issues

- Remaining Stage-2 level details including:

  - UL RA resources to be used during initial RACH procedure

  - Whether to check DVT in MAC or not

- Stage-3 details and CRs

  - and all Stage-3 aspects of MT-SDT triggering and finalizing the overall procedure for MT-SDT

- See WI description for further details

## 2.3	RAN3

#### 2.3.1	Agreements

RAN3#119

- BLCRs have been allocated.

- MT-SDT can be triggered by DL SDT user data and/or DL SDT signalling.

- Upon reception of DL SDT user data, the gNB-CU-UP may include the assistance information (e.g., Data size) in E1AP DL Data Notification message to gNB-CU-CP. FFS on MT-SDT indicator.

- When receiving DL SDT data, the anchor gNB may send MT-SDT information IE to the neighbour gNBs within the RNA, via XnAP RAN paging message. The encoding and the name of MT-SDT information IE is FFS.

- The gNB that receives MT-SDT information within the RNA takes into account this information received in the XnAP RAN PAGING message from the anchor gNB to decide whether to trigger MT-SDT Uu paging.

- Upon reception of MT-SDT information via XnAP RAN paging message from the anchor gNB-CU, the gNB-CU may send F1 MT-SDT information to the gNB-DU via F1AP Paging message. FFS on F1AP MT-SDT information

#### 2.3.2	Remaining Open issues

- Whether Data volume should also be provided from the CU to the DU in F1AP PAGING message.

- FFS: whether and how XnAP RTRV UE CTXT REQ message (that carries MT-SDT resume indication)

- Whether MT-SDT support indication in E1 Bearer Context procedure should be defined to enable the gNB-CU-UP to include the DL data size in the E1AP DL DATA NOTIFICATION message.

- Whether receiving gNB-CU or receving gNB-DU decides MT-SDT Uu paging, if we agree that it is the receving gNB to make the final decision on triggering MT-SDT Uu paging.

- FFS: MT-SDT assistance information sent from the anchor in the XnAP: RAN paging message and other alignment with RAN2 progress.

## 2.4	RAN4

#### 2.4.1	Agreements

#### 2.4.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### N/A

#### 3.1.1	Agreements with cross-TSG impacts

N/A

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

N/A

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

## 4.1	RAN1

## 4.2	RAN2

RAN2#121

- R2-2300182	Mobile Terminated Small Data Transmission Procedure in RRC_INACTIVE state	Samsung Electronics Co., Ltd

- R2-2300245	Discussion on Supporting MT-SDT	vivo Mobile Com. (Chongqing)

- R2-2300337	Discussion on paging triggered SDT	SHARP Corporation

- R2-2300386	Discussion on general pocedure of MT-SDT	OPPO

- R2-2300424	MT-SDT and quality measurements	Ericsson España S.A.

- R2-2300434	MT-SDT mechanism (including RB, paging, resume and capabilities)	Intel Corporation

- R2-2300497	Discussion on the MT-SDT procedure	Lenovo

- R2-2300559	MT-SDT procedure	ZTE Corporation, Sanechips

- R2-2300605	Discussion on MT-SDT procedure	Huawei, HiSilicon

- R2-2300651	Discussion on general procedure for MT-SDT	Spreadtrum Communications

- R2-2300738	Discussion on MT-SDT	Apple

- R2-2300777	Discussion on MT-SDT procedure	LG Electronics Inc.

- R2-2300805	Discussion on MT-SDT	NEC

- R2-2301102	DL SDT triggering and procedures	Sony

- R2-2301111	Remaining issues of MT SDT procedure	Xiaomi

- R2-2301245	Discussion on MT-SDT	CMCC

- R2-2301281	Discussion on MT-SDT	Qualcomm Incorporated

- R2-2301331	MT-SDT procedure	Nokia, Nokia Shanghai Bell

- R2-2301497	Mobile terminated SDT	InterDigital

- R2-2301544	Discussion on DL SPS for MT-SDT	ASUSTeK

- R2-2301804	Discussion on support of MT-SDT	CATT

- R2-2301813	Discussion on MT-SDT procedure	China Telecom

- R2-2302101	[AT120][306][R18 MT-SDT] summary of offline discussion (ZTE)	ZTE  Corporation (offline rapporteur)

## 4.3	RAN3

RAN3#119

- R3-230110 Work plan for the MT-SDT WI (ZTE (WI Rapporteur))

- R3-230111 Stage 2 issues on MT-SDT (ZTE)

- R3-230112 Stage 3 issues on MT-SDT (ZTE)

- R3-230082 Signaling enhancements to enable MT-SDT for RRC_INACTIVE UEs (Qualcomm Incorporated)

- R3-230104 (TPs to TS 38.300, 38.473, 37.483 BLCRs) Consideration on MT-SDT (Huawei)

- R3-230105 Introduction of MT-SDT (Huawei)

- R3-230150 (Draft CR for TS 38.300) Support of MT-SDT (CATT)

- R3-230151 Support of MT-SDT in XnAP (CATT)

- R3-230191 Discussion on MT-SDT (Xiaomi)

- R3-230222 (TP for TS 38.473, TS 38.423) Support of Paging Triggered NR MT-SDT (Nokia, Nokia Shanghai Bell)

- R3-230223 Introduction of NR MT-SDT (Nokia, Nokia Shanghai Bell, Orange)

- R3-230348 Support for Paging-Triggered SDT (Lenovo)

- R3-230538 Discussion on RAN3 impacts to support Paging-Triggered SDT and other aspects (Ericsson)

- R3-230539 Support of MT-SDT (Ericsson)

- R3-230562 Discussion on Support of MT-SDT (China Telecommunications)

- R3-230563 Text proposal on Support of MT-SDT in E1AP (China Telecommunications)

- R3-230676 (TP to TS 38.423, 38.473, 37.483 and 37.480) Support of MT-SDT (LG Electronics)

- R3-230677 (TP to TS 38.300 and 38.401) MT-SDT Support (LG Electronics)

- R3-230701 Discussions on MT-SDT impacts on RAN3 (including TP for TS 38.473) (Intel Corporation)

- R3-230702 Baseline CR for introducing Rel-18 NR MT SDT enhancement (Intel Corporation)

- R3-230727 Discussion on MT-SDT (Samsung)

- R3-230728 MT-SDT related IE in the XnAP RAN PAGING message (Samsung)

- R3-230913 (TP for MT-SDT BLCR to TS 38.423) Support of MT-SDT (Samsung, Ericsson, LG Electronics, Nokia, Nokia Shanghai Bell, CATT, Huawei, Xiaomi)

- R3-230914 (TP for MT-SDT BLCR to TS 38.473) Support of MT-SDT (LG Electronics Inc., ZTE, Ericsson, Nokia, Nokia Shanghai Bell, CATT, Huawei, Xiaomi, Intel Corporation)

- R3-230999 (TP for MT-SDT BLCR to TS 38.401) Support of MT-SDT (Xiaomi, Huawei, CATT, Ericsson, LG Electronics, Intel Corporation, Nokia, Nokia Shanghai Bell)

- R3-231001 Summary of Offline Discussion on CB: # 9_R18SDT (ZTE (moderator))

- R3-231008 (TP for MT-SDT BLCR to TS 37.483) Support of MT-SDT (China Telecom, Nokia, Nokia Shanghai Bell, Huawei, Ericsson, LG Electronics, Xiaomi, Intel Corporation)

## 4.1	RAN4

END