3GPP TSG RAN meeting #99									          RP-230265

Rotterdam, The Netherlands, March 20 - 23, 2023

## Status Report to TSG

Agenda item:			9.3.3.3

| WI / SI Name | Mobile IAB (Integrated Access and Backhaul) for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_mobile_IAB |  |  |  |  |
| Unique ID | 941009 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222671 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 12/2023 | Performance part: 06/2024 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 40% | Performance Part: 0% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Georg Hampel |
|  | Company | Qualcomm Incorporated |
|  | Email | ghampel@qti.qualcomm.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

R2#121

Mobility enhancements:

- Agreements on mobile-IAB-node to network indication:

- Postponed, AMF selection in the base-station is a Ran3 function, Ran2 expect RAN3 to ask for it if support for this is needed.

- Agreements on mobile-IAB-Node to UE indications and UE mobility enhancements:

- Working Assumption: support to have UE prioritization in cell reselection for mIAB cell(s), at least for inter-frequency cell-reselection.

- FFS if UE search and measure for mIAB cells on different frequencies is unspecified (autonomous search), FFS if such search can be done without assistance frequency information.

Other:

Agreements on BAP impact:

- For the upstream data handling at the BAP of mobile IAB MT, one common default BAP configuration to be used by both logical DUs is the baseline. RAN2 to further discuss the need of using logical-DU-specific default BAP configuration (e.g. when the two logical DUs use different donor-DUs).

- For the upstream data handling at the BAP of mobile IAB MT, RAN2 assume that the F1AP BAP configuration for each logical DU should be configured/controlled by the DU’s respective donor-CU via the corresponding F1AP connection (To be confirmed by RAN3).

- For the downstream data handling arriving at the mobile IAB node, RAN2 assume upper layers (e.g. IP layer) can differentiate the data to different logical DUs based on e.g. the IP address, i.e. no need to introduce logical-DU-specific BAP address. (To be confirmed by RAN3).

#### 2.2.2	Remaining Open issues

45% of the items defined in the RAN2 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.3	RAN3

#### 2.3.1	Agreements

R3#119

Agreed LS out to SA2, Cc: RAN, RAN2, RAN4, on FS_VMR solutions in R3-231011.

Agreements related to SA2 SI/WI on VMR

- With respect to mobile IAB, for issues concerning the control of UE access to MBSR using CAG function no enhancement is needed and no replies are foreseen from RAN3 on this matter.

- NGAP Initial Context Setup Request, UE Context Modification Request and HO Request to include an IE with code points “mobile-IAB authorized”, “mobile-IAB not-authorized”.

Agreements on DU migration procedure

- When triggering the F1 Setup procedure on the mIAB-node, the source logical mIAB-DU’s CU to include the information of target logical mIAB-DU’s CU (e.g. IP address, gNB-ID).

- The IAB-node can inform the source logical mIAB-DU’s CU via F1AP about the successful F1 Setup with the target logical mIAB-DU’s CU, and it can include the IDs of the cells activated by the target logical mIAB-DU’s CU.

- In case the target logical mIAB-DU’s CU is different from the mIAB-MT’s CU, the target logical mIAB-DU’s CU needs to be informed about the mIAB-MT’s CU ID and the mIAB-MT ID so that it can initiate the Xn TMM procedures towards mIAB-MT’s CU.

- The HO of UEs from the source logical mIAB-DU´s CU to the target logical mIAB-DU´s CU should happen after the completion of the F1 setup. When to trigger the HO is up to source logical mIAB-DU´s CU implementation.

- After all UEs have been handed over, the source logical mIAB-DU’s F1AP association can be released by the source logical mIAB-DU or by the source logical mIAB-DU’s CU.

- Target donor CU selection for mIAB-DU migration and triggering conditions for F1 setup can be up to source CU implementation (unless it is justified that this is not possible) or based on OAM configuration at the source CU.

- RAN3 not to work on solutions addressing use cases where inter donor IP connectivity is not available.

- For scenarios without Xn, RAN3 to investigate whether IAB-related Xn signaling for partial migration and DU migration can be carried via NG using a container to avoid the impact on the AMF.

Agreement on configuration of mIAB-DU:

- With respect to mIAB-DU migration and partial migration, RAN3 to discuss how the mobile IAB-DU’s parameters are (re-)configured.

Agreements on dynamic TAC

- Capture on stage 2 that the TAC/RANAC broadcast by the mobile IAB-DU can be changed in order to reflect the mIAB-node’s physical location. It needs to be further discussed how the mobile IAB-DU’s TAC/RANAC is changed and what Stage 3 impacts are (if any).

#### 2.3.2	Remaining Open issues

45% of the items defined in the RAN3 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

## 2.4	RAN4

#### 2.4.1	Agreements

R4#106

This was the first meeting to discuss this work item in RAN4, the RAN4 work plan was approved in R4-2300855.

RF Core Requirements:

The discussion was organized based on the moderator summary in R4-2303760

The agreements on RF requirements impact and co-existence study were all captured in the WF listed below:

WF on Mobile IAB RF requirements impact and co-existence study – R4-2302910

Issue 1-1: mIAB output power and power control

- Agreements

  - FFS if output power and power control requirements for Rel-18 mIAB could be the same as for UE.

Issue 1-2: mIAB FR1 bands

- Agreements

  - Consider developing mIAB for bands in addition to n41, n77, n78, n79 based primarily on operator request.

Issue 1-3: mIAB coexistence study

- Agreements

  - FFS the difference between Rel-18 mIAB and Rel-16 IAB coexistence study in terms of deployment, network layout and coexistence scenarios.

  - To list preliminary list of parameters (network layout, deployment, antenna parameters that might be different between Rel-18 mIAB and previous Rel-16 IAB coexistence study.

Issue 1-4: MT/DU/RAN carrier and frequency configurations

- Agreements

  - At least from co-existence study aspect, RAN4 focus on adjacent channel within same band.

Issue 1-5: mIAB-MT and DU TDM

- Agreements

  - For R18 mIAB, RAN4 only consider TDM for MT/DU operation.

Issue 1-5: Simulation assumptions

- Agreements

  - Network layout

    - Heterogeneous scenario (layout 1).

    - Homogeneous scenario (layout2).

    - Other layout options are not precluded.

  - FFS on the below listed simulation parameters:

    - test Inter-BS distance.

    - Minimum distance between moving IAB Node belonging to two cells.

    - Minimum distance between BS and UE.

    - Proposal for additional system parameters is shown in Table 2 in R4-2302910.

  - For the simulation parameters not listed in Annex in R4-2302910, Rel-16 IAB coexisting simulation can be utilized for preliminary studies.

  - Other parameters for coexisting are not precluded and new parameter values could be further discussed.

RRM Core Requirements:

The discussion was organized based on the moderator summary in R4-2302789

The agreements on RRM were all captured in the WF listed below:

WF on Mobile IAB RRM Requirements – R4-2303250

Issue 1-1: Scope of RRM Requirements

- Agreements

  - Do not define RRM requirements for IAB-DU

Issue 1-2: Baseline for RRM Requirements

- Agreement

  - UE RRM Requirements which are relevant to mobile IAB-MT are taken as baseline

Issue 1-3: Co-existence study impact

- Agreement

  - No RRM impact is expected from the adjacent channel co-existence study

Issue 1-4: Target RRM Requirements

- Agreement

  - Following requirements should be investigated and specified, if needed:

    - RRC_CONNECTED state mobility

      - Handover requirements

    - Signaling characteristics

      - TCI state switching delays

    - Measurement Procedures

      - Intra-frequency measurements for L3 mobility

      - L1-RSRP measurements for beam management

    - FFS on inter-frequency measurements

    - FFS on BWP switching delay

    - FFS whether to include additional requirements or adjust any of existing requirements

Issue 1-5: Timing requirements impact

- Agreement

  - Do not define additional timing requirements for mobile IAB.

    - Timing requirements include initial timing error, autonomous timing adjustment and timing advance

    - Note: the agreement applies for non-HST scenarios

  - FFS if HST scenario is in the scope of mobile IAB WI scope

Issue 1-7: Idle mode requirements for mobile IAB-MT

- Agreement

  - Do not introduce idle mode requirements

#### 2.4.2	Remaining Open issues

15% of items defined in the RAN4 WID objectives have been accomplished. All objectives from WID on enhancements to IAB for NR require further work.

.

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN2#121

| TDoc | Title | Source |
| --- | --- | --- |
| R2-2300034 | LS on static and dynamic TAC solutions for mobile IAB node (R3-226831; contact: ZTE) | RAN3 |
| R2-2300070 | Reply LS on FS_VMR solutions review (S2-2211437; contact: Qualcomm) | SA2 |
| R2-2300305 | Mobile IAB mobility-related issues and enhancements | Nokia, Nokia Shanghai Bell |
| R2-2300306 | Mobile IAB TAC/RANAC issues | Nokia, Nokia Shanghai Bell |
| R2-2300359 | Mobile IAB mobility enhancement and interference mitigation | Huawei, HiSilicon |
| R2-2300360 | BAP open issues due to the DU migration (two logical DUs) in mobile IAB | Huawei, Qualcomm Incorporate, LG Electronics, Intel Corporation, Lenovo, Apple, ZTE, vivo, Nokia, Nokia Shanghai Bell, Ericsson, Samsung |
| R2-2300437 | Mobility Enhancement of mobile IAB-node and served UEs | Intel Corporation |
| R2-2300438 | TAC/RANAC update of mIAB-node | Intel Corporation |
| R2-2300636 | Workplan for Rel-18 mobile IAB | Qualcomm Inc. (Rapporteur) |
| R2-2300637 | Enhancements for IAB-node mobility | Qualcomm Inc. |
| R2-2300638 | Configuration of Network-controlled Repeater | Qualcomm Inc. |
| R2-2300643 | Usage of “Supporting mobile-IAB” indication for cell reselection | SHARP Corporation |
| R2-2300668 | Impacts of PCI changes for IDLE/INACTIVE UE | SHARP Corporation |
| R2-2300710 | CONNECTED mobility enhancement in mobile IAB | Apple |
| R2-2300711 | Cell (re)selection and RNAU in mobile IAB | Apple |
| R2-2300807 | mIAB mobility enhancement aspects | Samsung Electronics Romania |
| R2-2300823 | Enhancements for mobility of IAB-node together with Ues | CATT |
| R2-2300824 | Other aspects for mobile IAB | CATT |
| R2-2300844 | Discussion on mobility enhancements for mobile IAB | NEC Corporation |
| R2-2300901 | Discussion on mIAB mobility Enhacement | vivo |
| R2-2300952 | Mobility enhancements for mobile IAB-node and its served UE | Lenovo |
| R2-2300953 | Discussion on BAP handling during mobile IAB-DU migration | Lenovo |
| R2-2301065 | On IAB node mobility state and associated UE behavior | InterDigital Inc. |
| R2-2301078 | Discussion on mobility enhancement | ZTE, Sanechips |
| R2-2301079 | Discussion on TAC and RNAC configuration of mobile IAB | ZTE, Sanechips |
| R2-2301100 | PCI collision in mobile IAB | Sony |
| R2-2301103 | Mobile IAB cell indication to UE behaviour | Sony |
| R2-2301194 | TAC handling for mIAB | Samsung R&D Institute UK |
| R2-2301301 | Discussion on mobility enhancements for mIAB node | Ericsson |
| R2-2301302 | Discussion on SA2 and migration aspects for mobile IAB | Ericsson |
| R2-2301419 | Mobile IAB cell type for UE cell selection and reselection | Beijing Xiaomi Mobile Software |
| R2-2301589 | Mobility enhancements for mobile IAB | Kyocera |
| R2-2301613 | Idle mode mobility for mIAB-MT and UE, and miscellaneous issue | LG Electronics Inc. |
| R2-2301614 | Connected mode mobility enhancements for onboard UEs | LG Electronics Inc. |
| R2-2301633 | Enhancements for IAB-node mobility and onboard Ues | AT&T |
| R2-2302067 | Reply LS on FS_VMR solutions review (R3-231011; contact: Qualcomm) | RAN3 |

RAN3#119

| TDoc | Title | Source |
| --- | --- | --- |
| R3-230032 | Reply LS on FS_VMR solutions review | SA2, Qualcomm |
| R3-230038 | LS on secured and trusted access to the serving PLMN OAM server by a MBSR | SA2, Nokia |
| R3-230136 | Discussion on the DU Migration of Mobile IAB-node | CANON Research Centre France |
| R3-230170 | Discussion on Consecutive partial migration and full migration | Nokia, Nokia Shanghai Bell |
| R3-230171 | Support for mobile IAB | Nokia, Nokia Shanghai Bell |
| R3-230172 | Discussion on mobility enhancements | Nokia, Nokia Shanghai Bell |
| R3-230173 | Mobile IAB interference mitigation | Nokia, Nokia Shanghai Bell |
| R3-230187 | Discussion on LS of FS_VMR solutions review | Xiaomi |
| R3-230188 | Discussion on IAB-node mobility | Xiaomi |
| R3-230189 | Discussion on mobility enhancement | Xiaomi |
| R3-230190 | Discussion on mitigation of interference | Xiaomi |
| R3-230226 | Workplan for Rel-18 mobile IAB | Qualcomm Inc. (Rapporteur) |
| R3-230227 | [Draft] Reply LS to SA2 on FS_VMR solutions review | Qualcomm Inc. |
| R3-230228 | Discussion on SA2 questions on FS_VMR enhancements | Qualcomm Inc. |
| R3-230229 | Topology adaptation for mobile IAB | Qualcomm Inc. |
| R3-230230 | Enhancements for mobility of IAB-node and its served UEs | Qualcomm Inc. |
| R3-230284 | Discussion on inter-donor migration in mobile IAB scenario | ZTE |
| R3-230285 | Discussion on enhancements to UE migration in mobile IAB scenario | ZTE |
| R3-230286 | Discussion on DU cell configuration for mobile IAB | ZTE |
| R3-230287 | Discussion on PCI collision avoidance for mobile IAB | ZTE |
| R3-230298 | Discussion on issues of VMR LS S2-2211437 | Huawei |
| R3-230299 | (TP for NR_mobile_IAB BL CR for TS 38.401): Support of mobility for mobile IAB | Huawei |
| R3-230300 | (TP for NR_mobile_IAB BL CR for TS 38.423): Mobility enhancement for mobile IAB | Huawei |
| R3-230301 | PCI collision for mobile IAB | Huawei |
| R3-230315 | Consideration on IAB-DU migration | CATT |
| R3-230316 | Enhancements for mobility of IAB-node together with Ues | CATT |
| R3-230338 | IAB-MT and IAB-DU migrate to different IAB-donors | Lenovo |
| R3-230339 | Discussion on mobile IAB-node inter-donor topology adaptation | Lenovo |
| R3-230340 | Mobility enhancements for mobile IAB-node and its served UE | Lenovo |
| R3-230341 | PCI collision mitigation of mobile IAB-node mobility | Lenovo |
| R3-230386 | Discussion on IAB-node consecutive partial migrations | Fujitsu |
| R3-230387 | Discussion on IAB full migration | Fujitsu |
| R3-230388 | Discussion on IAB-node mobility together with served UEs | Fujitsu |
| R3-230390 | Discussion of SA2 FS_VMR Solutions | Ericsson |
| R3-230391 | Migration Procedure for Mobile IAB-Nodes | Ericsson |
| R3-230392 | Enhancements of IAB-Node Mobility | Ericsson |
| R3-230393 | PCI Collision Avoidance for Mobile IAB-Nodes | Ericsson |
| R3-230493 | Dual-connected mobile IAB Architecture | MITRE Corporation |
| R3-230637 | Discussion on DU migration | Samsung |
| R3-230638 | Discussion on multiple partial migration | Samsung |
| R3-230639 | Discussion on mobility enhancements | Samsung |
| R3-230640 | Discussion on mitigation of interference | Samsung |
| R3-230859 | Summary of unofficial offline discussion on mobile IAB | Qualcomm |
| R3-230904 | Summary of offline discussion (CB: mIAB_1_LS by SA2) | Qualcomm |
| R3-230958 | [DRAFT] Reply LS on FS_VMR solutions review | Qualcomm |
| R3-230994 | Summary of offline discussion (CB: mIAB_1_LS by SA2) | Qualcomm |
| R3-231011 | Reply LS on FS_VMR solutions review | RAN3, Qualcomm |

RAN4#106

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2300294 | On the co-existence study for mobile IAB for NR | Qualcomm CDMA Technologies |
| R4-2300686 | General discussion on Mobile IAB | Nokia, Nokia Shanghai Bell |
| R4-2300687 | Discussion on Mobile IAB co-existence | Nokia, Nokia Shanghai Bell |
| R4-2300688 | Mobile IAB – RF core requirements | Nokia, Nokia Shanghai Bell |
| R4-2300854 | RRM Requirements for Mobile IAB | Qualcomm Incorporated |
| R4-2300855 | RAN4 Work Plan for Mobile IAB WI | Qualcomm Incorporated |
| R4-2300986 | Discussion on Mobile IAB co-ex study | Samsung |
| R4-2301013 | Discussion on Mobile IAB co-ex study | Samsung |
| R4-2301357 | Mobile IAB RRM considerations | Nokia, Nokia Shanghai Bell |
| R4-2301815 | Discussion on RRM impact on Rel-18 mobile IAB | Huawei, HiSilicon |
| R4-2302403 | Coexisting study simulation assumptions | Ericsson |
| R4-2302789 | Topic summary for [106][231] NR_mobile_IAB | Moderator (Qualcomm) |
| R4-2303760 | Topic summary for [106][315] NR_mobile_IAB_RF | Moderator - Qualcomm |

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template