3GPP TSG RAN Meeting #99									              RP-230293

Rotterdam, Netherlands, March 20-23, 2023

Status Report to TSG

Agenda item:			9.3.3.1

| WI / SI Name | # Further enhancement of data collection for SONMDT in NR standalone and MR-DC |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: No |  | Testing part: No |
| Acronym | NR_ENDC_SON_MDT_enh2-Core |  |  |  |  |
| Unique ID | 941007 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221825 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: mm/yyyy | Core part: 12/2023 | Performance part: mm/yyyy | Testing part: mm/yyyy |  |
| Overall Completion level | Study Item: xx% | Core part: 40% | Performance Part: xx% | Testing part: xx% |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN3 |
| --- | --- | --- |
| Rapporteur | Name | Liang LIU |
|  | Company | CMCC |
|  | Email | liuliang@chinamobile.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The change of the target completion date is due to postponement of Stage-3 functional completion to June 2022.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

2.1.2	Remaining Open issues

2.2	RAN2

2.2.1	Agreements

RAN2 #121 meeting

- MDT override

Solution 1: Override protection for logged MDT by simultaneous LTE and NR configuration for logged MDT.

Solution 2: Override protection by cross-RAT signaling but no cross-RAT reporting of LTE logged MDT report from NR to LTE.

Solution 3: Both solution 1 and 2 are supported for different UE implementation.

=>	Solution 2 is chosen for further specification work.

Agreements For solution 2:

1	Extend the LTE LoggedMeasurementConfiguration to include Logged MDT type indication information

2	NR signaling is needed to inform the gNB that signaling based MDT is configured by E-UTRA.

3	Try to reuse R17 NR signaling by the UE to inform gNB whether signaling based MDT is configured even when it is configured by E-UTRA.

=>	FFS  in RAN3 the details and the Need of differentiating the RAT type. Further discuss whether priority handling for signalling logged MDT configuration between different RAT types is needed or not.

- Inter-RAT SHR

Agreement:

1: For Q1 in the LS R2-2211160, RAN2 agrees to reduce/avoid the impact on LTE specification to support inter-RAT SHR.

2: For handover from NR to LTE,UE generates the NR SHR when SHR for inter-RAT mobility is triggered due to T310 or T312 trigger threshold is fulfilled.

3: For HO from NR to LTE, UE records the SHR for inter-RAT mobility in the VarSuccessHO-Report.

4: For inter-RAT SHR, below parameters is stored, reuse the existing IEs defined in Rel-17 for intra-NR SHR:

a.	Source NR cell information

c.	Measurement results for source, target and neighbours

d.	Cause to indicate which inter-RAT SHR triggering condition was met

e.	UE location Information

5:  A new EUTRA target cell CGI is introduced in inter-RAT SHR.

6: For HO from NR to LTE, the T310 and T312 threshold is provided to the UE by source gNB in the otherConfig.

7: For handover from NR to LTE, cross-RAT reporting is not supported, i.e., UE reports the SHR report to the network when it comes back to NR.

8: RAN2 further discuss if below content is needed for inter-RAT SHR when HO from NR to LTE:

a.	C-RNTI (FFS target or source)

c.	FFS: Time between report generating and fetching

Agreement

1: UE includes available location information in SPR .

2: UE stores SPR at most 48 hours after the last successful PSCell addition/PSCell change report is stored at UE if not fetched.

3:  At least the following options are needed for releasing  SPR report:

a. New SPR is initiated

b. Upon retrieval of SPR

c. Detach is initiated.

4:  In SPR, reuse CHO candidate cell flag to indicate whether a neighbor cell is CPAC candidate cell or not.

- SON for NR-U

=>	Postpone the discussion on the random-access attempt in NR-U to the next meeting as complexity analysis is required for each solution, when logging an RA attempt in the RA report.

Agreements:

1: 	Log the last successful RA procedure related information in the RA report. Only some information to be logged for multiple successive RA procedures failed due to LBT issue. FFS what information.

- RACH enhancement

Agreements:

1: To have “a list of SN RA report entries as a single NR container (i.e. NR RA-ReportList)”.

=> It is not supported in R18 that UE reports NR RACH Report to LTE cell when the UE is in standalone LTE.

=>	RAN2 assumes that the following two alternatives are feasible and would like to check RAN3’s views:

-	Alt 2b: Includes unique PSCell identities, i.e. if a PSCell occurs more than once in NR RA-ReportList, it is recorded only once in the list of PSCell identities

-	Alt 2c: Includes the last PSCell identity (in NR RA-ReportList)

2.2.2	Remaining Open issues

- Support of data collection for SON features, including, MRO for MR-DC SCG failure scenario, and MRO enhancement for inter-system handover voice fallback,

- Specification of the UE reporting necessary to enhance the mobility parameter tuning [RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces    
[RAN3]

- Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PScell change report

- Successful Handover Report (e.g. inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

- Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

2.3	RAN3

2.3.1	Agreements

RAN3 #119 meeting：

- BL CR

R3-230843, Endorsed unseen as BL CR

R3-230070, Endorsed as BL CR

R3-230071, Endorsed as BL CR

- SHR and SPR

2.1 	Inter-RAT SHR

RAN3 sees benefits to support inter-RAT SHR from LTE to NR for T304 trigger with no impact on LTE in Rel-18.

Take Option 3 (The receiving node forwards the inter-RAT SHR to corresponding node which generates the SHR trigger condition that triggers the inter-RAT SHR) as baseline for SHR forwarding mechanism in Rel-18.

WA: The content of inter-RAT SHR from LTE to NR includes at least Source LTE cell, Target NR cell .

2.2 	SPR

For classic addition/CPA, SN- and MN-initiated classic PSCell change/CPC, the target SN node decides the T304 trigger and performs root cause analysis.

For intra-SN classic PSCell change/CPC, the source SN decides SPR triggers of T310 and T312 and performs root cause analysis.

If the trigger is T312/310, the objective of SPR is to optimize lower layer issues of source PSCell and to optimize PSCell change configuration during mobility.

If the trigger is T312/310, for MN-initiated classic PSCell change/CPC, at least the MN who sent the SPR configuration to the UE is involved in SPR related optimizations.

- MRO

3.1	MRO for CPC

Naming for too early CPA execution? No need to rename.

Update the definition to wrong PSCell change/addition should be splitting to sub cases: 1) the wrong candidate cell comes from the cell in the list provided by the initiating node or 2) the wrong candidate cell is selected by the target node.

Information available in the network nodes should not be included in the SCGFailureInformation.

Reusing R17 signalling mechanism to report CPA/CPC failure/ related information over Xn from MN to source SN or last serving SN.

For MRO for CPC and CPA, if there are multiple events configured for CPA/CPC, the UE reports the first triggered CPAC event, and the time duration between the two triggered CPAC events.

3.2	MRO for the fast MCG recovery

Sub-Case b1/Sub-Case b2 would not be considered for MRO for fast MCG recovery failure.

It is beneficial for the UE to report at least PSCell where SCG failure happened, the cause of the fast MCG recovery failure (at least T316 expiry, SCG failure, SCG was deactivated or other cases that SCG is not available), and also if the problem is SCG failure, the SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

LS on MRO for CPC and CPA and fast MCG recovery to RAN2 in R3-230992 Agreed unseen

3.3	MRO for inter-system handover for voice fallback

TP to 38.300: R3-230993 Agreed unseen

- RACH enhancements

Using non-UE associated sigalling for RACH Indication over F1.

TP to 38.300: R3-231040 Agreed unseen

TP to 38.401: R3-230996  Agreed unseen

- SON/MDT Enhancements for Non-Public Networks

Wait for SA3 LS on MDT user consent for NPN.

TP to 38.413: R3-231022 Agreed unseen

- SON for NR-U

DU shall report the UL COT to CU in resource status report procedure when it is requested by CU, how to obtained this information by DU is up to implementation.

The granularity of UL COT reporting in F1 interface should be NR-U channel level.

TP to 38.473: R3-230980  Agreed unseen

2.3.2	Remaining Open issues

- Support of data collection for SON features, including, MRO for MR-DC SCG failure scenario, and MRO enhancement for inter-system handover voice fallback,

- Specification of the UE reporting necessary to enhance the mobility parameter tuning [RAN2]

- Specification of the inter-node information exchange, including possible enhancements to interfaces    
[RAN3]

- Support of SON/MDT enhancements for [RAN3, RAN2]:

- MR-DC CPAC

- Successful PScell change report

- Successful Handover Report (e.g. inter-RAT)

- NPN

- RACH report

- fast MCG recovery

- NR-U (MRO and UL MLB)

- Support of signaling based logged MDT override protection to address the scenario where the signaling based MDT is configured in E-UTRAN when [RAN2, RAN3]:

- UE reselects to NR while logged measurements are collected

- UE reselects to NR after logged measurements are collected and before uploading the logged MDT report.

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

4.	References

RAN2 #121:

R2-2300026	LS on user consent of Non-public Network (R3-226006; contact: ZTE)	RAN3

R2-2300031	Response LS on Possibility on LBT-FailureRecoveryConfig (R3-226809; contact: Nokia)	RAN3

R2-2300293	Consideration on Inter-RAT Signaling Based Logged MDT Override Protection	CATT

R2-2300294	Discussion on inter-RAT SHR and SPR	CATT

R2-2300295	SON Enhancement for NR-U	CATT

R2-2300296	RACH enhancement for SON	CATT

R2-2300297	Considerations on MRO about Fast MCG recovery Enhancement	CATT

R2-2300298	Discussion on SONMDT Enhancements for CPAC	CATT

R2-2300681	Remaining issues on SON for SPR	vivo

R2-2300682	Discussion on MRO for CPAC	vivo

R2-2300716	On MDT override protection	Apple

R2-2300717	Remaining issues of SON enhancements for RACH	Apple

R2-2300954	Successful Handover Report for inter-RAT HO	Lenovo

R2-2300955	SON enhancements for SPR	Lenovo

R2-2300956	Discussion on MRO for NR-U	Lenovo

R2-2300957	Enhancements for SN RACH report	Lenovo

R2-2300958	MRO for fast MCG link recovery	Lenovo

R2-2300959	SON enhancements for CPAC	Lenovo

R2-2301001	Signalling based logged MDT override protection	Nokia, Nokia Shanghai Bell

R2-2301002	SPR content enhancements	Nokia, Nokia Shanghai Bell

R2-2301003	SPR and SHR generation and reporting	Nokia, Nokia Shanghai Bell

R2-2301004	Discussion on storing LBT-FailureRecoveryConfig (Reply LS in R2-2300031)	Nokia, Nokia Shanghai Bell

R2-2301005	MRO enhancements for Fast MCG recovery and for MR-DC CPAC	Nokia, Nokia Shanghai Bell

R2-2301006	RA report retrieval	Nokia, Nokia Shanghai Bell

R2-2301044	Discussion on SHR and SPCR	Xiaomi

R2-2301045	Discussion on SON for NR-U	Xiaomi

R2-2301144	Consideration on MDT override issues	ZTE Corporation, Sanechips

R2-2301145	Consideration on SHR and SPR	ZTE Corporation, Sanechips

R2-2301146	Consideration on NR-U related SON	ZTE Corporation, Sanechips

R2-2301147	Consideration on RACH enhancements	ZTE Corporation, Sanechips

R2-2301148	Remaining issues on fast MCG recovery	ZTE Corporation, Sanechips

R2-2301149	Consideration on MRO for CPAC	ZTE Corporation, Sanechips

R2-2301192	Inter-RAT signaling based logged MDT override protection	Samsung

R2-2301193	SON/MDT enhancements for Fast MCG Recovery and CPAC MRO	Samsung

R2-2301195	SON/MDT enhancements for SHR and SPCR	Samsung

R2-2301213	SON/MDT enhancements for NR-U	Samsung

R2-2301214	SN RACH Report for EN-DC/NG-EN-DC	Samsung

R2-2301264	SONMDT enhancement for NR-U	CMCC

R2-2301265	SONMDT enhancement for NPN	CMCC

R2-2301274	MRO for inter-system handover for voice fallback	Ericsson

R2-2301275	MDT enhancements	Ericsson

R2-2301276	SPR and SHR enhancements	Ericsson

R2-2301277	Enhancements of SON reports for NR-U	Ericsson

R2-2301278	RA report enhancement	Ericsson

R2-2301279	MRO for SCG failure and fast MCG recovery optimization	Ericsson

R2-2301420	Signalling based logged MDT override protection	Qualcomm Incorporated

R2-2301421	Discussion on SHR for inter-RAT handover and successful PSCell change reporting	Qualcomm Incorporated

R2-2301422	Discussion on NR-U Related Enhancements	Qualcomm Incorporated

R2-2301424	On SgNB RACH report for MR-DC scenario	Qualcomm Incorporated

R2-2301425	Discussion on fast MCG Recovery Failure	Qualcomm Incorporated

R2-2301557	Discussion on successful PSCell change report	Sharp

R2-2301565	SON enhancement on fast MCG recovery	Sharp

R2-2301566	Discussion on failure information for CPAC	Sharp

R2-2301570	Discussion on the inter-system signalling based MDT override protection	Huawei, HiSilicon

R2-2301571	Discussion on SHR and SPR	Huawei, HiSilicon

R2-2301572	Discussion on SON for NR-U	Huawei, HiSilicon

R2-2301573	Discussion on RACH enhancement	Huawei, HiSilicon

R2-2301574	Discussion on Fast MCG recovery	Huawei, HiSilicon

R2-2301575	Discussion on MRO for CPAC	Huawei, HiSilicon

R2-2301582	Further Consideration on RACH Enhancement	CMCC

R2-2301583	Further considerations on fast MCG recovery	CMCC

R2-2301584	MHI Enhancement for SCG Activation/Deactivation	CMCC,Ericsson, CATT

R2-2301585	SON MDT enhancement for MR-DC CPAC	CMCC

R2-2301631	Considerations on the signaling based logged MDT override protection for E-UTRAN	Beijing Xiaomi Software Tech

R2-2301635	Discussion on the SON/MDT enhancement for RACH report	Beijing Xiaomi Software Tech

R2-2301763	Discussion on SPR	NTT DOCOMO, INC.

R2-2301775	Discussion on CPAC failure report	NTT DOCOMO, INC.

R2-2301811	SON on fast MCG recovery	OPPO

R2-2301812	Discussion on RACH enhancements	China Telecom

R2-2301833	Further steps on MRO for MR-DC SCGFailureInformation	Samsung, Lenovo

R2-2301856	Discussion of SON on MR-DC CPAC	OPPO

RAN3 #119:

R3-230057	BLCR to 38.300:enhancement of SON	CMCC

R3-230070	(BLCR to 38.423) Addition of SON features enhancement	Samsung

R3-230071	(BLCR to 38.473) Addition of SON features enhancement	Huawei

R3-230128	[TP to 38.423, SON] Configuration coordination for the successful PSCell change report	Nokia, Nokia Shanghai Bell

R3-230129	[TP to 37.340, SON] Further consideration on the CPC Execution to wrong PSCell	Nokia, Nokia Shanghai Bell

R3-230130	[TP to TS38423, SON] SN support for MRO for fast MCG recovery	Nokia, Nokia Shanghai Bell

R3-230131	The LBT waiting time for correct NR-U MRO analysis	Nokia, Nokia Shanghai Bell

R3-230234	(TP for SON BLCR for 37.340) Connection failure scenario for CPA	Intel Corporation

R3-230235	RA report enhancement for RACH partitioning	Intel Corporation

R3-230236	MDT enhancements for NPN	Intel Corporation

R3-230237	TP to TS 38.413 on area scope for NPN	Intel Corporation

R3-230238	NR-U enhancements for MRO and MLB	Intel Corporation

R3-230239	Inter-RAT SHR and SPR	Intel Corporation

R3-230276	Discussion on the support of MRO on fast MCG recovery	NEC

R3-230322	SON enhancements for inter-RAT SHR	Lenovo

R3-230323	SON enhancements for SPR	Lenovo

R3-230324	(TP for SON BLCR for 37.340) SON enhancements for CPAC	Lenovo

R3-230325	MRO for fast MCG link recovery	Lenovo

R3-230326	(TP for SON BLCR for 38.300) MRO for inter-system handover for voice fallback	Lenovo, CMCC

R3-230327	Discussion on MRO for NR-U	Lenovo

R3-230350	(TP for SON BL CR for TS 38.473) RACH Optimization enhancement	Ericsson

R3-230351	(TP for SON BL CR for TS 38.413) SON enhancements for Non-public networks	Ericsson

R3-230352	NR-U enhancements for MRO and MLB	Ericsson

R3-230353	(TP for SON BL CR for TS 38.423, TS 38.473) Radio Resource Status per NR-U Channel	Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Deutsche Telekom

R3-230354	(TP for SON BL CR for TS 38.423) Channel Occupancy of second tier neighbors	Ericsson, Qualcomm Incorporated, Deutsche Telekom

R3-230363	Successful Handover Report and Successful PSCell Change Report	Qualcomm Incorporated

R3-230364	MRO enhancements for CPAC, voice fallback and fast MCG recovery	Qualcomm Incorporated

R3-230365	RACH optimization enhancements	Qualcomm Incorporated

R3-230366	SON MDT enhancements for Non-Public networks	Qualcomm Incorporated

R3-230367	SON enhancements for NR-U	Qualcomm Incorporated

R3-230422	SHR	Huawei

R3-230423	SPR	Huawei

R3-230424	(TPs for SON BLCRs for TS 36.300): MRO related objectives	Huawei

R3-230494	(TP to SON BL CR to TS 38.423) Inter-RAT SHR and SPR discussion	Ericsson

R3-230495	(TP for SON BL CR for TS 38.423) UHI for CPAC	Ericsson

R3-230496	(TP for SON BL CR for TS 38.300) SON enhancements for Mobility Robustness	Ericsson

R3-230526	(TP for SON BL CR to TS 38.473) Further discussion on RACH optimisation	Nokia, Nokia Shanghai Bell

R3-230527	(TP for MDT BL CR to TS 38.413) Addition of SNPN identifiers in MDT area scope	Nokia, Nokia Shanghai Bell

R3-230572	(TPs for SON BLCR for TS 38.473): Remaining issues for RACH optimisation	Huawei

R3-230573	(TPs for SON BLCRs for TS 38.300): RACH report retrieval	Huawei, Orange, China Telecom, CMCC

R3-230574	(TPs for SON BLCRs for TS 38.401): RACH report retrieval	Huawei, Orange, China Telecom, CMCC

R3-230575	MDT support in SNPN	Huawei

R3-230576	(TP for MDT BL CR for TS 38.423) Addition of PNI-NPN in MDT Area Scope	Huawei, China Telecom, CMCC

R3-230577	(TPs for SON BLCR for TS 37.320): MDT support in SNPN	Huawei, Orange, China Telecom, CMCC

R3-230578	(TPs for SON BLCR for TS 38.423): SON for NR-U	Huawei

R3-230579	(TP for MDT BL CR for TS 38.413): Signalling based immediate MDT in NR-DC	Huawei, Deutsche Telekom, Orange, China Telecom, CMCC

R3-230586	Discussion on SON enhancement for SHR and SPR	CATT

R3-230587	Discussion on MRO for CPAC and MR-DC SCG failure	CATT

R3-230588	Discussion on MRO for fast MCG recovery and voice fallback	CATT

R3-230589	Discussion on RACH enhancement	CATT

R3-230590	Discussion on SONMDT enhancement for NPN	CATT

R3-230591	Discussion on SON enhancement for NR-U	CATT

R3-230603	(TP for SON BLCR for 38.423) SON enhancement for Successful Handover Report and Successful PSCell Change Report	Samsung

R3-230604	(TP for SON BLCR for 38.423) SON enhancements for CPAC and MCG failure recovery	Samsung

R3-230605	(TP for SON BLCR for 38.300 and 38.413) MRO for inter-system handover for voice fallback	Samsung

R3-230606	Discussion on SON for RACH	Samsung

R3-230607	Discussion on SON/MDT for NPN	Samsung

R3-230608	Discussion on SON for NR-U	Samsung

R3-230647	Update of work Plan for Enhancement of Data Collection for SON_MDT in NR standalone and MR-DC WI	CMCC

R3-230648	SHR for intra-system inter-RAT handover	CMCC

R3-230649	(TP for SON BLCR for 38.300) MRO enhancement for inter-system handover voice fallback	CMCC, CATT, Lenovo

R3-230652	Further considerations on fast MCG recovery	CMCC