3GPP TSG RAN Meeting #99									       RP-230308

Rotterdam, Netherlands, March 20-23, 2023

Status Report to TSG

Agenda item:			9.3.1.3

| WI / SI Name | Further NR coverage enhancements |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_cov_enh2 |  |  |  |  |
| Unique ID | 940095 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221858 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: NA | Core part: 12/2023 | Performance part: 06/2024 | Testing part: NA |  |
| Overall Completion level | Study Item: NA | Core part: 30% | Performance Part: 0% | Testing part: NA |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN WG1 |
| --- | --- | --- |
| Rapporteur | Name | Nanxi LI |
|  | Company | China Telecom |
|  | Email | linanxi@chinatelecom.cn |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

2.1	RAN1

2.1.1	Agreements

RAN1 #112

PRACH enhancements:

Agreement

For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

- If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported.

- FFS: details

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- Note: Separate RO means that the RO is separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Conclusion

For multiple PRACH transmissions within one RACH attempt, they are only transmitted over ROs associated with the same SSB/CSI-RS.

Note: This applies for multiple PRACH transmissions with same Tx beam, and also applies for multiple PRACH transmissions with different Tx beam (if supported).

Agreement

For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt.

Agreement

For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt.

  - FFS: the start position of the RAR window.

  - FFS: RA-RNTI.

Agreement

For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions.

Note 1: All ROs in one RO group is associated with the same SSB(s).

Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Note 3: whether/how to define “RO group” in specification will be discussed separately

Note 4: Valid RO(s) refers to what is defined in existing specification

FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH.

FFS: the time span of RO group.

FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions.

FFS: other details

Agreement

Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams.

Note: It is summarized by FL that for the same number of PRACH transmissions per source,

- 1 source [Ericsson] shows that: Multiple PRACH transmitted by beam sweeping, where a UE has no prior knowledge of channel and sweeps Tx beams across 360 degrees horizontally and 180 degrees vertically, outperforms multiple PRACH transmissions with the same Tx wide beam (omni direction) by at least 1 dB, provided gNB configures only one SSB and receives PRACH with a wide beam.

- 3 sources [ZTE, Nokia, vivo] show that: A gain from about 1~3 dB of beam sweeping is observed if a UE is able to direct at least one of its Tx beams in the right direction or to narrow down the azimuth and/or zenith range of 360 degrees and/or 180 degrees for beam sweeping compared with multiple PRACH transmissions with the same Tx wide beam.

- 1 source [Huawei] shows that: compared to the same wide beam for multiple PRACH transmission, if different Tx beams are finer beams, then 3.9~5 dB gains are observed assuming that only one PRACH occasion with the best detected SINR is selected at the gNB reception, where the beam gain of fine beam is 4 times that of wide beam.

- 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams far apart is 3 dB worse than PRACH repetition with single best beam

- 1 source [vivo] shows that: The performance of PRACH repetition with beam sweeping among beams in the directions close to the best Tx beam is 1dB worse than PRACH repetition with single best beam.

- 1 source [vivo] shows that: PRACH repetition via random beam directions performs 1 dB worse than PRACH repetition with omni beam.

Power domain enhancements:

Agreement

Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input).

  - FFS: details.

Agreement

If FDSS-SE is supported in Rel-18, RAN1 to further study the following approaches for DMRS, when the DMRS sequence length before extension of the sequence, if any, is larger than or equal to 30:

- Approach A – the DMRS sequence is extended: A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. Two sequence types can be considered:

  - A.1: The sequence is a Type 1 DMRS sequence.

  - A.2: The sequence is a Type 2 DMRS sequence.

FFS: how the sequence is extended.

- Approach B – the DMRS sequence is not extended: A DMRS sequence based on type 1 or type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension.

Note: if type 2 is used then both the number of PRBs in the inband and the number of PRBs in the inband+extension must be valid DFT sizes as per NR specification

Performance metrics considered for the study are PAPR, CM[, and OBO] for DMRS and 10% BLER SNR for data (to measure channel estimation accuracy).

Agreement

If FDSS-SE is supported in Rel-18, and RB allocations resulting in DMRS sequence length smaller than 30 before extension of the sequence, if any, are supported, RAN1 to study at least the following approaches:

- Approach A – the DMRS sequence is extended: A DMRS sequence is generated considering the number of PRBs in the inband (no extension). The sequence length depends on the number of PRBs in the inband. Two sequence types can be considered:

  - A.1: The sequence is obtained by DFT transformation of an existing DMRS sequence, e.g., Type 1 DMRS sequence.

  - A.2: The sequence is a Type 1 or Type 2 DMRS sequence.

FFS: how the sequence is extended.

- Approach B – the DMRS sequence is not extended: A DMRS sequence based on type 1 or type 2 DMRS sequence is generated considering the number of PRBs in the inband + extension. The sequence length depends on the number of PRBs in the inband + extension.

Note: if type 2 is used then both the number of PRBs in the inband and the number of PRBs in the inband+extension must be valid DFT sizes as per NR specification

Note:    Other sequences are not precluded for Approach A and Approach B.

Performance metrics considered for the study are PAPR, CM [, and OBO] for DMRS and 10% BLER SNR for data (to measure channel estimation accuracy).

Agreement

Include in the LS to RAN4 for reporting LLS results

Note: The excel file is used to collect the results.

Working Assumption

- The following set of configurations is for companies’ consideration for the comparsion of the performance of DMRS with FDSS-SE.

| No spectrum extension |  | With spectrum extension |  |  |  |
| --- | --- | --- | --- | --- | --- |
| #PRBs | MCS | #PRBs before extension | #PRBs after extension | MCS | Spectrum extension factor |
| 8 | 0 [only QPSK] | 6 | 8 | 1 [only QPSK] | ¼ |
| 8 | 6 | 6 | 8 | 8 | ¼ |
| 40 | 2 | 30 | 40 | 3 | ¼ |
| 40 | 6 | 30 | 40 | 8 | ¼ |
| [6 | 3 | 4 | 6 | 5 | 1/3] |
| [36 | 7 | 32 | 36 | 8 | 1/9] |

- FR1 4GHz Urban scenario is prioritized.

- The following filters are for companies’ consideration for the calibration of the performance of DMRS with FDSS-SE

- 3-tap (0.28 1 0.28)

- [Truncated RRC (0.5, 0.1667) or 2-tap (1 0.28)]

- Note1: Considered metrics are PAPR/CM, 10% BLER SNR of data for the considered DMRS configuration (for measuring impact of channel estimation accuracy)[, and OBO]

- Note2: companies are encouraged to consider a receiver which at least makes use of the extension for the decoding (e.g., MRC)

- Note3: The values above serve as a common basis, but any other configuration can be studied by companies.

Agreement

The Draft LS R1-2302080 is endorsed in principle.

Agreement

The Final LS R1-2302081 is endorsed.

Dynamic switching between DFT-S-OFDM and CP-OFDM:

Agreement

For single TB scheduled by single DCI, support new 1-bit field for dynamic waveform indication from UL scheduling DCI.

Note: no change of the current size alignment procedure between UL DCI and DL DCI.

Conclusion

There is no consensus to support “Dynamic waveform switching to PUSCH transmissions with a Type 2 configured grant” in R18.

Agreement

Dynamic waveform switching in R18 is not applicable to PUSCH transmissions with a Type 1 configured grant.

Conclusion

The dynamic waveform indication in a DCI containing a dynamic uplink grant applies only to PUSCH transmission(s) corresponding to the dynamic uplink grant.

2.1.2	Remaining Open issues

- PRACH coverage enhancements

  - Detailed mechanism(s) to support multiple PRACH transmissions with same beams for 4-step RACH procedure.

  - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure.

- Power domain enhancements

  - Study and if necessary specify enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations.

  - Study and if necessary specify enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation.

- Detailed mechanism(s) to support dynamic switching between DFT-S-OFDM and CP-OFDM.

2.2	RAN2

2.2.1	Agreements

2.2.2	Remaining Open issues

- Specify following PRACH coverage enhancements (RAN1, RAN2)

  - Multiple PRACH transmissions with same beams for 4-step RACH procedure

  - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure

  - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable.

  - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.

2.3	RAN3

2.3.1	Agreements

2.3.2	Remaining Open issues

2.4	RAN4

2.4.1	Agreements

RAN4 #106

The progress in RAN4 #106 meeting is summarized below:

- The topic summary for [106][143] NR_cov_enh2_part1 was provided in R4-2302836.

- The topic summary for [106][144] NR_cov_enh2_part2 was provided in R4-2302837.

- The Ad hoc minutes for [106][144] NR_cov_enh2_part2 was provided in R4-2303530.

- The way forward on coverage enhancement part 1 was approved in R4-2303560.

- The draft LS on enhancements to realize increasing UE power high limit for CA and DC was approved in R4-2303701.

- The way forward on enhancements to reduce MPR&PAR was approved in R4-2303559.

- The way forward on simulation assumptions for PAPR/MPR reduction was approved in R4-2303561.

- The way forward on RF simulation results template was approved in R4-230356.

2.4.2	Remaining Open issues

- Continue the discussion on enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations.

- Continue the discussion on enhancement to reduce MPR/PAR.

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN1 #112:

- R1-2300089	Discussion on PRACH coverage enhancements	Huawei, HiSilicon

- R1-2300090	Discussion on coverage enhancement in power domain	Huawei, HiSilicon

- R1-2300091	Discussion on dynamic waveform switching for coverage enhancement	Huawei, HiSilicon

- R1-2300244	Discussion on PRACH coverage enhancements	Spreadtrum Communications

- R1-2300245	Discussion on power domain enhancements	Spreadtrum Communications

- R1-2300246	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM	Spreadtrum Communications

- R1-2300276	PRACH coverage enhancements	OPPO

- R1-2300277	The study of power domain enhancements	OPPO

- R1-2300278	Considerations on dynamic switching between DFT-S-OFDM and CP-OFDM	OPPO

- R1-2300344	Discussion on PRACH coverage enhancements	ZTE

- R1-2300345	Discussion on power domain enhancements	ZTE

- R1-2300346	Discussion on dynamic waveform switching	ZTE

- R1-2300479	Discussions on PRACH coverage enhancements	vivo

- R1-2300480	Discussions on power domain enhancements	vivo

- R1-2300481	Discussions on dynamic waveform switching	vivo

- R1-2300495	Discussion on Resource Allocation for Multiple PRACH Transmission	FGI

- R1-2300561	Discussion on PRACH coverage enhancements	xiaomi

- R1-2300562	Discussion on power domain enhancements	xiaomi

- R1-2300563	Discussion on dynamic switching between DFT-s-OFDM and CP-OFDM	xiaomi

- R1-2300667	PRACH coverage enhancements	CATT

- R1-2300668	Discussion on MPR/PAR reduction enhancements	CATT

- R1-2300669	Dynamic switching between DFT-S-OFDM and CP-OFDM	CATT

- R1-2300728	Discussion on PRACH coverage enhancement	China Telecom

- R1-2300729	Discussion on power domain enhancements	China Telecom

- R1-2300730	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM	China Telecom

- R1-2300758	Discussion on PRACH coverage enhancements	Fujitsu

- R1-2300759	Discussion on Power domain enhancements	Fujitsu

- R1-2300796	Discussion on PRACH coverage enhancements	Panasonic

- R1-2300797	Discussion on power domain enhancements	Panasonic

- R1-2300828	Discussion on PRACH coverage enhancement	NEC

- R1-2300829	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM	NEC

- R1-2300838	PRACH coverage enhancements	TCL Communication Ltd.

- R1-2300856	Dynamic switching between DFT-S-OFDM and CP-OFDM	InterDigital, Inc.

- R1-2300860	PRACH coverage enhancements	Lenovo

- R1-2300861	Power domain enhancements	Lenovo

- R1-2300862	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM	Lenovo

- R1-2300864	Discussion on dynamic waveform switching	Panasonic

- R1-2300894	PRACH Coverage Enhancement using Multi PRACH Transmissions	Sony

- R1-2300895	Considerations on tone reservation for PAPR reduction	Sony

- R1-2300896	Further considerations on dynamic waveform switching for the UE UL	Sony

- R1-2300903	Discussion on solutions for NR dynamic switching between DFT-S-OFDM and CP-OFDM	Mavenir

- R1-2300904	Discussion on PRACH coverage enhancement	Mavenir

- R1-2300972	Discussions on PRACH coverage enhancement	Intel Corporation

- R1-2300973	Discussions on power domain enhancement	Intel Corporation

- R1-2300974	Dynamic switching between DFT-S-OFDM and CP-OFDM waveform	Intel Corporation

- R1-2301027	Discussion on PRACH coverage enhancements	CMCC

- R1-2301028	Discussion on power domain enhancements	CMCC

- R1-2301029	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM	CMCC

- R1-2301053	PRACH coverage enhancements	ETRI

- R1-2301054	Dynamic switching between DFT-S-OFDM and CP-OFDM	ETRI

- R1-2301074	Discussion on PRACH repeated transmission for NR coverage enhancement	LG Electronics

- R1-2301075	Discussion on dynamic waveform switching for NR coverage enhancement	LG Electronics

- R1-2301086	Discussion on dynamic waveform switching	DENSO CORPORATION

- R1-2301171	Discussion on PRACH coverage enhancements	InterDigital, Inc.

- R1-2301172	Discussion on power domain enhancements	InterDigital, Inc.

- R1-2301185	Discussion on PRACH coverage enhancement	Ericsson

- R1-2301186	Power Domain Enhancement Schemes and Performance	Ericsson

- R1-2301187	Discussion on Dynamic UL Waveform Switching	Ericsson

- R1-2301292	PRACH coverage enhancements	Samsung

- R1-2301293	Power domain enhancements	Samsung

- R1-2301294	Dynamic switching between DFT-S-OFDM and CP-OFDM	Samsung

- R1-2301312	Discussion of dynamic switching between DFT-S-OFDM and CP-OFDM	Transsion Holdings

- R1-2301374	Discussion on PRACH coverage enhancement	Apple

- R1-2301375	Discussion on power domain coverage enhancement	Apple

- R1-2301376	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM	Apple

- R1-2301441	PRACH Coverage Enhancements	Qualcomm Incorporated

- R1-2301442	Power-domain enhancements	Qualcomm Incorporated

- R1-2301443	Dynamic switching between DFT-S-OFDM and CP-OFDM	Qualcomm Incorporated

- R1-2301519	Discussion on PRACH coverage enhancements	NTT DOCOMO, INC.

- R1-2301520	Discussion on power domain enhancements	NTT DOCOMO, INC.

- R1-2301521	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM	NTT DOCOMO, INC.

- R1-2301540	Summary #1 on dynamic switching between DFT-S-OFDM and CP-OFDM	Moderator (InterDigital, Inc.)

- R1-2301541	Summary #2 on dynamic switching between DFT-S-OFDM and CP-OFDM	Moderator (InterDigital, Inc.)

- R1-2301550	Views on multiple PRACH transmission for coverage enhancement	Sharp

- R1-2301551	Dynamic switching between DFT-S-OFDM and CP-OFDM for Rel-18 CovEnh	Sharp

- R1-2301603	On PRACH coverage enhancements	MediaTek Inc.

- R1-2301604	Views on power domain enhancements	MediaTek Inc.

- R1-2301605	Dynamic switching between waveforms	MediaTek Inc.

- R1-2301635	DMRS design for power domain enhancements	Indian Institute of Tech (H)

- R1-2301654	PRACH coverage enhancements	Nokia, Nokia Shanghai Bell

- R1-2301655	RAN1 impacts for power domain enhancements	Nokia, Nokia Shanghai Bell

- R1-2301656	Dynamic switching between DFT-s-OFDM and CP-OFDM	Nokia, Nokia Shanghai Bell

- R1-2301770	Discussion on PRACH coverage enhancements	xiaomi

- R1-2301777	On PRACH coverage enhancements	MediaTek Inc.

- R1-2301778	Views on power domain enhancements	MediaTek Inc.

- R1-2301779	Dynamic switching between waveforms	MediaTek Inc.

- R1-2301848	FL Summary#1 on PRACH coverage enhancements	Moderator (China Telecom)

- R1-2301849	FL Summary#2 on PRACH coverage enhancements	Moderator (China Telecom)

- R1-2301850	FL Summary#3 on PRACH coverage enhancements	Moderator (China Telecom)

- R1-2301869	FL summary of power domain enhancements (AI 9.14.2)	Moderator (Nokia)

- R1-2301870	FL summary #2 of power domain enhancements (AI 9.14.2)	Moderator (Nokia)

- R1-2301871	FL summary #3 of power domain enhancements (AI 9.14.2)	Moderator (Nokia)

- R1-2301872	FL summary #4 of power domain enhancements (AI 9.14.2)	Moderator (Nokia)

- R1-2301873	Final FL summary of power domain enhancements (AI 9.14.2)	Moderator (Nokia)

- R1-2301995	DMRS design for power domain enhancements	Indian Institute of Tech (H)

- R1-2302069	Session notes for 9.14 (Further NR coverage enhancements)	Ad-Hoc Chair (CMCC)

- R1-2302080	[Draft] LS to RAN4 for results of the LLS performance evaluation of MPR/PAR reduction solutions	Moderator (Nokia)

- R1-2302081	LS to RAN4 for results of the LLS performance evaluation of MPR/PAR reduction solutions	RAN1, Nokia

- R1-2302124	Summary #3 on dynamic switching between DFT-S-OFDM and CP-OFDM	Moderator (InterDigital, Inc.)

- R1-2302222	Summary #4 on dynamic switching between DFT-S-OFDM and CP-OFDM	Moderator (InterDigital, Inc.)

RAN4 #106:

- R4-2300039	Necessity of reporting power class being used by a UE	Nokia, Nokia Shanghai Bell

- R4-2300162	Discussion on enhancement of increasing UE power high limit for CA and DC	Fujitsu Limited

- R4-2300341	Simulation results for transparent MPR reduction schemes	Apple

- R4-2300342	Simulation results for non-transparent MPR reduction schemes	Apple

- R4-2300710	On UL power enhancement from transparent techniques	Qualcomm Incorporated

- R4-2300711	On UE signaling to enhance ULCA and DC	Qualcomm Incorporated

- R4-2300753	Power-class fallback reporting in the PHR for improved scheduling and enhanced performance with and without the high-power limit	Ericsson

- R4-2301101	Discussion on enhancement of increasing UE maximum power high limit	Xiaomi

- R4-2301107	Views on enhancement for SAR issue mitigation in FR1	Samsung

- R4-2301178	R18 increase UE power high limit for CA and DC	OPPO

- R4-2301383	Draft LS on enhancements to realize increasing UE power high limit for CA and DC	NTT DOCOMO INC.

- R4-2301514	Discussion on RF simulation parameters for enhancement to reduce MPR	vivo

- R4-2301515	RF simulation results for transparent schemes for enhancement to reduce MPR	vivo

- R4-2301516	RF simulation results for non-transparent schemes for enhancement to reduce MPR	vivo

- R4-2301544	Further discussion on enhancement of increasing UE power high limit for CA and DC	vivo

- R4-2301545	Introduce new scenario for increase higher power limit for CA	vivo, Xiaomi, Huawei

- R4-2301546	Introduce new scenario for increase higher power limit for EN-DC	vivo, Xiaomi, Huawei

- R4-2301680	Scope of the work for MPR/PAR -objective	Nokia, Nokia Shanghai Bell

- R4-2301681	RF simulation parameters for MPR/PAR evaluations	Nokia, Nokia Shanghai Bell

- R4-2301682	RF simulation results for transparent schemes	Nokia, Nokia Shanghai Bell

- R4-2301683	RF simulation results for non-transparent schemes	Nokia, Nokia Shanghai Bell

- R4-2301684	RF specification impacts	Nokia, Nokia Shanghai Bell

- R4-2301761	On enhancements of increasing UE power high limit for CA and DC	Huawei, HiSilicon

- R4-2301762	On further enhancements to reduce MPR/PAR	Huawei, HiSilicon

- R4-2302420	RF spec impact for MPR reduction scheme	Ericsson

- R4-2302421	P-MPR for PRACH	Ericsson

- R4-2302422	simulation parameter discussion for transparent and non-transparent schemes	Ericsson

- R4-2302423	Simulation results for the non-transparent scheme	Ericsson

- R4-2302424	Simulation results for the transparent scheme	Ericsson

- R4-2302448	Addition for PC1.5 inter-band UL CA	T-Mobile USA

- R4-2302491	Discussion on MPR/PAR reduction for UL coverage enhancements	MediaTek (Chengdu) Inc.

- R4-2302648	On UL power enhancement from BWE techniques	Qualcomm Incorporated

- R4-2302740	Measurements for transparent and non-transparent schemes	Skyworks Solutions Inc.

- R4-2302836	Topic summary for [106][143] NR_cov_enh2_part1	Moderator (Huawei)

- R4-2302837	Topic summary for [106][144] NR_cov_enh2_part2	Moderator (Nokia)

- R4-2303560	WF on coverage enhancement part 1	Huawei

- R4-2303701	Draft LS on enhancements to realize increasing UE power high limit for CA and DC	NTT DOCOMO

- R4-2303530	Ad hoc minutes for [106][144] NR_cov_enh2_part2	Nokia

- R4-2303559	WF on enhancements to reduce MPR&PAR 	Nokia

- R4-2303561	WF on simulation assumptions for PAPR/MPR reduction 	Nokia

- R4-2303562	WF on RF simulation results template 	Ericsson, Nokia