3GPP TSG RAN Meeting #99									  RP-230332

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item:			9.3.4.6

| WI / SI Name |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | NR_RRM_enh3 |  |  |  |  |
| Unique ID | 950078 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-221696 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: Overall:65% RAN4: 65% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN4 |
| --- | --- | --- |
| Rapporteur | Name | Jie Cui |
|  | Company | Apple |
|  | Email | jie_cui@apple.com |
| Rapporteur | Name | Roy Hu |
|  | Company | OPPO |
|  | Email | hurongyi@oppo.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

RAN4#106 meeting

The following tdocs have been agreed in RAN4#104-bis-e meeting:

| Tdoc number | Title | Source |
| --- | --- | --- |
| R4-2303241 | Ad-hoc minutes for R18 RRM enhancements | Apple |
| R4-2303228 | WF on R18 RRM enhancement (part 1) | Apple |
| R4-2303304 | WF on R18 RRM enhancement (part 2) | OPPO |

RRM core requirements for FR2 SCell activation delay reduction

Issue 0-1: Work plan for FR2 SCell activation enhancement

Agreement:

Table 1. Summary for candidate solutions for FR2 unknown SCell activation enhancement

| Enhancement part | Issue index in topic summary (R4-2302769) | Enhancement solution |  | RAN4 decision on whether to continue discuss this enhancement solution in R18? |
| --- | --- | --- | --- | --- |
| L3 part | Issue 1-1-1, Issue 1-1-1a | New sub-states for known/unknown state for FR2 SCell |  | Discussed in CR stage |
|  | Issue 1-1-2/1-1-3/1-1-4/1-1-5/1-1-6 | New measurement status indication or report for FR2 SCell activation enhancement |  | Yes, continue discussion |
|  | Issue 1-2-1 | Beam sweeping factor enhancement in L3 part (not related with WI of FR2 multi-Rx chain DL reception) |  | Yes, continue discussion |
|  | Issue 1-3-1/1-3-2 | enhancement of “TFirstSSB_MAX + 15*TSMTC_MAX” part (sample reduction) |  | Deprioritized |
|  | Issue 1-3-3 | Enhancement of “8*Trs” part |  | Merged to issue 1-2-1/2-1-1 |
|  | Issue 1-4-2 | RS and QCL information related enhancement for FR2 unknown SCell activation |  | Deprioritized |
|  | Issue 1-4-1/Issue 1-4-3 | RS and QCL information related enhancement for FR1 unknown SCell activation |  | Yes, continue discussion |
|  | Issue 1-5-1/1-5-2 | Ending point of Scell activation of unknown case in FR2 |  | Deprioritized |
|  | Issue 1-6-1 | use SSB periodicity (e.g. 20ms) instead of SMTC periodicity for FR2 unknown SCell activation |  | Yes, continue discussion |
| L1 part | Issue 2-1-1 | Beam sweeping factor enhancement in L1-RSRP measurement (not related with WI of FR2 multi-Rx chain DL reception) |  | Yes, continue discussion |
|  | Issue 2-1-2 | Skip L1-RSRP measurement of FR2 unknown SCell activation |  | Yes, continue discussion |
|  | Issue 2-1-3 | Prioritization enhancement for L1-RSRP measurement | L1-RSRP measurement is performed in non-DRX mode even DRX is configured. | Yes, continue discussion |
|  |  |  | Whether prioritize L1 over L3 is under discussing | Deprioritized |
|  | Issue 2-2-1 | Skip fine timing tracking for SSB corresponding to the TCI state |  | Deprioritized |
|  | Issue 2-2-2 | TCI activation enhancement |  | Deprioritized |
|  | Issue 2-3-1 | Aperiodic RS for TFineTiming |  | Yes, continue discussion |
|  | Issue 2-3-2/ Issue 2-4-2 | Aperiodic RS for L1-RSRP measurement |  | Deprioritized |
| Others | Issue 3-1-1 | SCell without SSB for inter-band FR2 unknown SCell activation |  | Deprioritized |
|  | Issue 3-1-2/ Issue 3-1-3/Issue 3-1-4 | SCell without SSB for inter-band FR1 unknown SCell activation |  | Deprioritized |
|  | Issue 3-1-5 | known/unknown condition enhancement for FR2 SCell activation |  | Deprioritized |
|  | Issue 3-1-6 | Scheduling enhancement for FR2 unknown SCell activation |  | Yes, continue discussion |
|  | Issue 3-1-7 | CBRA for FR2 unknown SCell activation |  | Deprioritized |

Issue 1-2-1: Beam sweeping factor enhancement in L3 part of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

Issue 2-1-1: Beam sweeping factor enhancement in L1-RSRP measurement of FR2 unknown SCell activation (not related with WI of FR2 multi-Rx chain DL reception)

Agreement for issue 1-2-1 and 2-1-1:

    - For unknown FR2 SCell activation enhancement, introduce the UE capability to support Rx beam sweeping factor can be less than 8 for cell detection part of L3 and SSB based L1-RSRP measurement.

      - if UE has full set (N=8) of beam sweeping during AGC settling part in L3.

        - Beam sweeping factor capability of X1 for cell detection part (X1*Trs) of L3 and beam sweeping factor capability of X2 for SSB based L1-RSRP measurement

        - FFS on capability indication for X1 and X2

      - Note: above enhancement only applies for FR2 unknown SCell activation enhancement

Issue 1-3-3: enhancement of “8*Trs” part of current FR2 unknown SCell activation delay

Agreement:

- To keep “X1*Trs” part of current FR2 unknown SCell activation delay in the delay requirement for FR2 SCell activation enhancment.

  - X1 can be less than 8  in the beam sweeping factor reduction discussion for cell detection stage in L3 part based on UE capability.

RRM core requirements for FR1-FR1 NR-DC

Issue 1-1-1: TP/CR split for FR1+FR1 NR-DC

Agreement:

  - Agree on the work split of CRs for FR1+FR1 NR-DC RRM requirements based on the table as below.

| Potential RRM requirements for FR1+FR1 NR-DC | Impacted section/clause in TS 38.133 | Volunteer of draft CR |
| --- | --- | --- |
| Applicability rule | 3.6.2.4 | QC |
| PSCell addition/release delay and conditional PSCell addition delay | 8.2.4.1, 8.9.2 | vivo |
| Scheduling availability | 8.1.7, 8.5.7, 8.5.8, 9.5.6, | QC, OPPO, MTK, Nokia |
| CSSF | 9.1.5.1.4, 9.1.5.2.4 | OPPO |
| HO with PSCell | 6.1.5.1, 6.1.5.4 | Xiaomi |
| SCG activation/deactivation | 8.17.2 | Ericsson |

Issue 1-2-1: Scheduling availability for intra-frequency measurement without MG (of UE performing measurements in TDD bands on FR1)

Agreement:

  - For intra-frequency measurement without MG,

    - For UE who does not support simultaneousRxTxInterBandCA in ca-ParametersNR-ForDC, the existing scheduling availability requirement of UE performing measurements in TDD bands in FR1 inter-band CA case is used for FR1-FR1 NR-DC scenario.

    - FFS how to capture the agreements in the spec

Issue 1-2-2: Scheduling availability for intra-frequency measurement without MG (of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1)

Agreement:

  - No need to explicitly update the spec for scheduling availability requirement of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 for FR1-FR1 NR-DC scenario

Issue 1-2-3: How to capture scheduling availability for RLM/BFD/CBD/L1-RSRP measurements for FR1-FR1 NR-DC in TS 38.133

Agreement:

  - Specify the scheduling availability for RLM/BFD/CBD/L1-RSRP measurements for FR1-FR1 NR-DC in TS 38.133 in existing section of FR1.

Issue 1-3-1: Whether to define measurement restriction for the scenario that network configures mixed numerology on two CCs while UE doesn’t support simultaneous reception for two CCs?

Agreement:

  - Measurement restriction requirement is not applicable when network configures mixed numerology on two CCs, while UE doesn’t support simultaneous reception for two CCs.

    - no need to update spec for this issue since TS38.133 section 3.6.10 has already covered it.

  - Note: above conclusion means no measurement restriction requirement is applied for the case when network configures mixed numerology on two CCs while UE doesn’t support simultaneous reception for two CCs.

Issue 1-4-1: Tsearch for RACH-less SCG activation

Agreement:

  - For RACH-less based SCG activation for FR1+FR1 NR-DC,

    - [if TCI state is known, Tsearch = 0 ms if the target cell is a known FR1 PSCell]

    - FFS on requirements for additional scenarios including FR1 PSCell is unknown

Issue 1-4-2: Whether to discuss SCG activation with direct multiple SCell activation in this WI?

Agreement:

  - This issue will not be discussed in this R18 eFeRRM WI.

Issue 1-4-3: Whether to consider relax requirements on inter-frequency measurement during deactivated SCG state in R18

Agreement:

  - This issue will not be discussed in this R18 eFeRRM WI.

#### 2.4.2	Remaining Open issues

As summarized in approved way forward R4-2303228 and R4-2303304 (open options in the way forward).

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #106 meeting

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2300279 | Work plan for FR2 SCell activation enhancement | Apple, OPPO |
| R4-2300280 | On L3 part enhancement for FR2 SCell activation | Apple |
| R4-2300281 | On L1 part enhancement for FR2 SCell activation | Apple |
| R4-2300282 | On other potential enhancement for FR2 SCell activation | Apple |
| R4-2300283 | On RRM requirements for FR1-FR1 NR-DC | Apple |
| R4-2300438 | Unknown FR2 Scell activation enhancement | Qualcomm Incorporated |
| R4-2300439 | Remaining RRM requirements for FR1-FR1 NR-DC | Qualcomm Incorporated |
| R4-2300460 | Discussion on enhancement for FR2 SCell activation | Intel Corporation |
| R4-2300767 | L3 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2300768 | L1 part enhancement on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2300769 | Other enhancements on FR2 SCell activation delay reduction | Nokia, Nokia Shanghai Bell |
| R4-2300866 | Discussion on L1 part enhancement for FR2 SCell activation | CMCC |
| R4-2300867 | Discussion on L3 part enhancement for FR2 SCell activation | CMCC |
| R4-2300886 | Discussion on L3 part enhancement for FR2 SCell activation | Xiaomi |
| R4-2300887 | Discussion on L1 part enhancement for FR2 SCell activation | Xiaomi |
| R4-2300899 | Discussion on RRM core requirements for FR1-FR1 NR-DC | Xiaomi |
| R4-2300944 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | LG Electronics Inc. |
| R4-2300945 | Discussion on L1 enhancement for FR2 SCell activation delay reduction | LG Electronics Inc. |
| R4-2300981 | discussion on FR1-FR1 NR-DC | Nokia, Nokia Shanghai Bell |
| R4-2301000 | Discussion on L3 part enhancement for FR2 Scell activation | NTT DOCOMO, INC. |
| R4-2301001 | Discussion on L1 part enhancement for FR2 Scell activation | NTT DOCOMO, INC. |
| R4-2301008 | Discussion on L3 part enhancement for FR2 SCell activation | China Telecom |
| R4-2301009 | Discussion on L1 part enhancement for FR2 SCell activation | China Telecom |
| R4-2301010 | Discussion on other potential enhancement for SCell activation | China Telecom |
| R4-2301341 | Discussion on RRM core requirements for FR1-FR1 NR-DC | vivo |
| R4-2301402 | Discussion on other aspects of RRM requirements enhancement for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2301403 | Discussion on the L1 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2301404 | Discussion on the L3 part enhancement of RRM requirements for FR2 SCell activation delay reduction | ZTE Corporation |
| R4-2301654 | Discussion on L3 enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2301655 | Discussion on L1 enhancement for FR2 Scell activation delay reduction | OPPO |
| R4-2301656 | Discussion on RRM requirements for FR1+FR1 NR-DC | OPPO |
| R4-2301657 | Draft big CR to TS 38.133 on RRM requirements for FR1+FR1 NR-DC | OPPO |
| R4-2301701 | Discussion on L3 aspects of FR2 SCell activation enhancements | vivo |
| R4-2301702 | Discussion on L1 aspects of FR2 SCell activation enhancements | vivo |
| R4-2301703 | Discussion on other aspects of FR2 SCell activation enhancements | vivo |
| R4-2301808 | Discussion on L3 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2301809 | Discussion on L1 enhancement for FR2 SCell activation delay reduction | Huawei, HiSilicon |
| R4-2301810 | Discussion on FR2 SCell activation enhancement | Huawei, HiSilicon |
| R4-2301811 | Discussion on RRM requirements for FR1-FR1 NR-DC | Huawei, HiSilicon |
| R4-2302258 | Discussion on FR1-FR1 NR-DC RRM requirements | Ericsson |
| R4-2302360 | Discussion on L3 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2302361 | Discussion on L1 part enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2302362 | Discussion on other potential enhancement for FR2 SCell activation | MediaTek inc. |
| R4-2302363 | Discussion on R18 RRM for FR1-FR1 NR-DC | MediaTek inc. |
| R4-2302655 | On FR2 SCell activation delay reduction: L3 aspects | Ericsson |
| R4-2302656 | On FR2 SCell activation delay reduction: L1 RSRP | Ericsson |
| R4-2302657 | On FR2 SCell activation delay reduction: Others | Ericsson |
| R4-2302769 | Topic summary for [106][211] NR_RRM_enh3_part1 | Moderator (Apple) |
| R4-2302770 | Topic summary for [106][212] NR_RRM_enh3_part2 | Moderator (OPPO) |

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template