3GPP TSG RAN WG1 #112			R1-2302160

Athens, Greece, February 27th – March 3rd, 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | 0449 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the timeline of transmitting/receiving PSFCH with control information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (LG Electronics), ZTE, Sanechips, Spreadtrum Communications, vivo, Samsung, Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023-3-2 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to the following agreement of RAN1#107-e, when the time constraint T3 is used to derive the PSFCH occasion by a slot where expected/potential resource conflict occurs, T3 should be in unit of logical slot, i.e., slot in a resource pool. But in current specification, T3 is in unit of physical slots in this case. Agreement A resource pool level (pre-)configuration uses either of the following options - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3 - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs - FFS: How to account for processing timeline Note that it is possible not to configure either option1 or option 2.. - The transmitted SCI format 1-A/reserved resource and its associated PSFCH should be in the same resource pool. |
| Summary of change: |  | - Change 'T3 slots’ to ‘T3 slots of the resource pool’ if sl-PSFCH-Occasion = '1' - Change ‘a’ to ‘the’ |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The RAN1 agreement of RAN1#107-e will not be captured correctly in RAN1 specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 16.3.0, 16.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Unchanged parts omitted>

    - 16.3.0 UE procedure for transmitting PSFCH with control information

<Unchanged parts omitted>

If a UE transmits a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE transmits the PSFCH in the resource pool in a slot determined based on sl-PSFCH-Occasion

-	If sl-PSFCH-Occasion = '0', the UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH reception that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not transmit the PSFCH with conflict information.

-	If sl-PSFCH-Occasion = '1', the UE transmits the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots of the resource pool before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH reception that provides the SCI format 1-A; otherwise, the UE does not transmit the PSFCH with conflict information.

<Unchanged parts omitted>

    - 16.3.1	UE procedure for receiving PSFCH with control information

<Unchanged parts omitted>

A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by sl-InterUE-CoordinationScheme2, attempts to receive associated PSFCH with conflict information in the resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers

-	if sl-SlotLevelResourceExclusion is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A

-	if sl-SlotLevelResourceExclusion is provided, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A

If a UE receives a PSFCH with conflict information corresponding to a reserved resource indicated in an SCI format 1-A, the UE receives the PSFCH in the resource pool in a slot determined based on sl-PSFCHOccasion

-	If sl-PSFCHOccasion = '0', the UE receives the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a slot of a PSCCH transmission that provides the SCI format 1-A. The PSFCH resource is in a slot that is at least T3 slots [6, TS 38.214] before the resource associated with the conflict information; otherwise, the UE does not receive the PSFCH with conflict information

-	If sl-PSFCHOccasion = '1', the UE receives the PSFCH in a latest slot that includes PSFCH resources and is at least T3 slots of the resource pool before a slot of the resource associated with conflict information. The PSFCH resource is in a slot that is at least sl-MinTimeGapPSFCH slots after a slot of a PSCCH transmission that provides the SCI format 1-A; otherwise, the UE does not receive the PSFCH with conflict information

<Unchanged parts omitted>