3GPP TSG-RAN WG1 Meeting # 112	R1-2302202

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 0042 | rev | - | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to type 2 channel access for UL multi channel access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2023-03-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In 4.2.1.0.4, the use of Type 2 channel access procedure as described in 4.2.1.2 within the UL multi-channel access procedure is conditional upon Type 1 channel access procedure being indicated or intended for the scheduled or configured UL transmissions, respectively. While clause 4.2.1.2 specifies that an indication by an eNB of Type 2 procedure to an LAA UE is interpreted as Type 2A procedure, it is not clear for an NR-U UE which sub-type of Type 2 procedures under 4.2.1.2 is applicable. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In Clause 4.2.1.0.4, clarify that Type 2A channel access procedure is used in UL multi-channel access instead of Type 2 channel access procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguous UL multi-chanel access procedures for NR-U UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.0.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Unchanged text omitted ***

##### 4.2.1.0.4	Channel access procedures for UL multi-channel transmission(s)

If a UE

-	is scheduled to transmit on a set of channels C, and if the UL transmissions are scheduled to start transmissions at the same time on all channels in the set of channels C, or

-	intends to perform an uplink transmission on configured resources on the set of channels C, and if UL transmissions are configured to start transmissions at the same time on all channels in the set of channels C,

the following is applicable:

-	if Type 1 channel access procedure is indicated or intended for the scheduled or configured UL transmissions, respectively, to be transmitted on the set of channels C,

-	the UE may transmit on channel ci∈C using Type 2A channel access procedure as described in clause 4.2.1.2.1,

-	if the channel frequencies of the set of channels C is a subset of the sets of channel frequencies defined in clause 5.7.4 in [2], and

-	if Type 2A channel access procedure is performed on channel ci immediately before the UE transmission on channel cj∈C, i≠j, and

-	if the UE has accessed channel cj using Type 1 channel access procedure as described in clause 4.2.1.1,

-	where channel cj is selected by the UE uniformly randomly from the set of channels C before performing Type 1 channel access procedure on any channel in the set of channels C.

-	the UE may transmit on channel ci∈C using Type 1 channel access procedure as described in clause 4.2.1.1

-	the UE may not transmit on channel ci∈C within the bandwidth of a carrier, if the UE fails to access any of the channels, of the carrier bandwidth, on which the UE is scheduled or configured with UL resources.

-	the UE may not transmit on a channel within the bandwidth of a carrier if the UE is configured without intra-cell guard band(s) on an UL bandwidth part as described in clause 7 of [8], and the UE fails to access any of the channels of the UL bandwidth part.

*** Unchanged text omitted ***