3GPP TSG-RAN WG4 Meeting #106	 R4-2300089

# Athens, Greece, 27th Feburary - 3rd March 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | 0301 | rev | 1 | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for TS38.141-1 correction on the value of SNR in test requirements for PUSCH |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17 |  |  |  |  |  | Date: |  |  | 2023-2-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Regarding clause 8.2.1.5, from the specification v17.5.0, one of the SNR values for 1T8R in Table 8.2.1.5-2 has been missed. The value was “-8.1” dB until v17.4.0. The value blank is not found before Rel-17. |  |  |  |  |  |  |  |  |
| Summary of change: |  | One of the values of SNR in table 8.2.1.5-2 was modified as follow, “<blank>” to “-8.1”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The required SNR value remains missing in Table 8.2.1.5-2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of R4-2300037 |  |  |  |  |  |  |  |  |

--------------Start of text proposal-------------

8.2.1.5	Test Requirement

The throughput measured according to clause 8.2.1.4.2 shall not be below the limits for the SNR levels specified in table 8.2.1.5-1 to 8.2.1.5-18.

Table 8.2.1.5-1: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -1.7 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 10.7 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 12.9 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-1 | pos1 | 19.7 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -5.2 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 6.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 9.4 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-1 | pos1 | 16.1 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -8.1 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 3.6 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 6.2 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-1 | pos1 | 13.0 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | 1.8 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | 19.0 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | -1.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | 11.8 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | -4.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | 7.6 |

Table 8.2.1.5-2: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | -1.9 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | 10.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | 12.8 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-2 | pos1 | 20.1 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | -5.4 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | 6.9 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | 9.2 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-2 | pos1 | 16.5 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | -8.1 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | 3.7 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | 6.1 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-2 | pos1 | 13.2 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | 2.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | 19.1 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | -1.2 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | 12.0 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | -4.7 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | 7.6 |

Table 8.2.1.5-3: Test requirements for PUSCH with 70% of maximum throughput, Type A, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | -1.5 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | 10.6 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | 13.0 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | -4.9 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | 6.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | 9.2 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | -7.9 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | 3.6 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | 6.1 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | 2.9 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | 19.1 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | -1.0 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | 11.9 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | -4.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | 7.7 |

Table 8.2.1.5-4: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | -1.7 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | 10.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | 13.4 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-3 | pos1 | 19.9 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | -5.0 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | 7.0 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | 9.2 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-3 | pos1 | 16.2 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | -8.0 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | 3.9 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | 6.1 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-3 | pos1 | 13.2 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | 2.1 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | 19.2 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | -1.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | 12.0 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | -4.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | 7.8 |

Table 8.2.1.5-5: Test requirements for PUSCH with 70% of maximum throughput, Type A, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | -2.3 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | 10.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | 13.1 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | -5.4 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | 7.0 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | 9.2 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | -8.2 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | 3.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | 6.1 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | 2.1 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | 18.9 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | -1.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | 12.1 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | -4.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | 7.7 |

Table 8.2.1.5-6: Test requirements for PUSCH with 70% of maximum throughput, Type A, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | -1.9 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | 10.6 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | 13.0 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-4 | pos1 | 20.5 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | -5.2 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | 6.9 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | 9.1 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-4 | pos1 | 16.7 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | -8.1 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | 3.7 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | 6.0 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-4 | pos1 | 13.2 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | 2.1 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | 20.3 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | -1.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | 12.1 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | -4.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | 7.7 |

Table 8.2.1.5-7: Test requirements for PUSCH with 70% of maximum throughput, Type A, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | -2.2 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | 10.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | 13.6 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-5 | pos1 | 21.7 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | -5.2 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | 7.1 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | 9.6 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-5 | pos1 | 17.3 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | -8.1 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | 3.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | 6.4 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-5 | pos1 | 13.7 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | 2.2 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | 20.0 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | -1.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | 12.4 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | -4.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | 7.9 |

Table 8.2.1.5-8: Test requirements for PUSCH with 70% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -1.7 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 10.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 13.1 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-1 | pos1 | 19.7 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -5.1 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 6.9 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 9.5 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-1 | pos1 | 16.1 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-8 | pos1 | -8.1 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-8 | pos1 | 3.6 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-8 | pos1 | 6.3 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-1 | pos1 | 12.9 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | 2.3 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | 19.1 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | -1.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | 11.9 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-22 | pos1 | -4.6 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-22 | pos1 | 7.6 |

Table 8.2.1.5-9: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | -1.7 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | 11.1 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | 13.2 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-2 | pos1 | 20.1 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | -5.1 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | 7.1 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | 9.5 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-2 | pos1 | 16.5 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-9 | pos1 | -8.4 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-9 | pos1 | 3.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-9 | pos1 | 6.4 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-2 | pos1 | 13.1 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | 2.8 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | 19.5 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | -1.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | 12.1 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-23 | pos1 | -4.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-23 | pos1 | 7.8 |

Table 8.2.1.5-10: Test requirements for PUSCH with 70% of maximum throughput, Type B, 20 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | -1.5 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | 11.0 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | 12.9 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | -5.1 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | 6.9 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | 9.4 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-10 | pos1 | -7.9 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-10 | pos1 | 3.7 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-10 | pos1 | 6.3 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | 2.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | 18.9 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | -1.2 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | 12.0 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-24 | pos1 | -4.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-24 | pos1 | 7.7 |

Table 8.2.1.5-11: Test requirements for PUSCH with 70% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | -1.8 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | 10.7 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | 13.1 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-3 | pos1 | 19.8 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | -5.1 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | 7.0 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | 9.2 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-3 | pos1 | 16.3 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-11 | pos1 | -8.2 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-11 | pos1 | 3.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-11 | pos1 | 6.2 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-3 | pos1 | 13.0 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | 1.9 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | 19.3 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | -1.7 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | 12.1 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-25 | pos1 | -4.8 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-25 | pos1 | 7.8 |

Table 8.2.1.5-12: Test requirements for PUSCH with 70% of maximum throughput, Type B, 20 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | -2.3 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | 10.7 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | 13.1 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | -5.4 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | 6.9 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | 9.2 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-12 | pos1 | -8.4 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-12 | pos1 | 3.7 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-12 | pos1 | 6.2 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | 2.1 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | 19.0 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | -1.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | 12.0 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-26 | pos1 | -4.6 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-26 | pos1 | 7.8 |

Table 8.2.1.5-13: Test requirements for PUSCH with 70% of maximum throughput, Type B, 40 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | -1.9 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | 10.6 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | 13.1 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-4 | pos1 | 20.5 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | -5.2 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | 6.8 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | 9.3 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-4 | pos1 | 16.6 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-13 | pos1 | -8.2 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-13 | pos1 | 3.6 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-13 | pos1 | 6.1 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-4 | pos1 | 13.3 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | 2.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | 19.5 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | -1.3 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | 12.0 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-27 | pos1 | -4.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-27 | pos1 | 7.7 |

Table 8.2.1.5-14: Test requirements for PUSCH with 70% of maximum throughput, Type B, 100 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | -1.9 |
|  | 2 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | 10.7 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | 13.7 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-5 | pos1 | 21.7 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | -5.2 |
| 1 | 4 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | 6.9 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | 9.8 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-5 | pos1 | 17.5 |
|  |  | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-14 | pos1 | -8.1 |
|  | 8 | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-14 | pos1 | 3.7 |
|  |  | Normal | TDLA30-10 Low | 70 % | G-FR1-A5-14 | pos1 | 6.5 |
|  |  | Normal | TDLA30-10 Low | 70% | G-FR1-A8-5 | pos1 | 13.8 |
|  | 2 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | 2.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | 20.1 |
| 2 | 4 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | -1.4 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | 12.4 |
|  | 8 | Normal | TDLB100-400 Low | 70 % | G-FR1-A3-28 | pos1 | -4.5 |
|  |  | Normal | TDLC300-100 Low | 70 % | G-FR1-A4-28 | pos1 | 7.9 |

Table 8.2.1.5-15: Test requirements for PUSCH with 30% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 30 % | G-FR1-A4-8 | pos1 | 3.5 |

Table 8.2.1.5-16: Test requirements for PUSCH with 30% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 30 % | G-FR1-A4-11 | pos1 | 3.4 |

Table 8.2.1.5-17: Test requirements for PUSCH with 30% of maximum throughput, Type B, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 30 % | G-FR1-A4-8 | pos1 | 3.4 |

Table 8.2.1.5-18: Test requirements for PUSCH with 30% of maximum throughput, Type B, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | TDLC300-100 Low | 30 % | G-FR1-A4-11 | pos1 | 3.5 |

--------------End of text proposal-------------