3GPP TSG RAN Meeting #99												RP-230568

Rotterdam, Netherlands, March 20-23, 2023

Agenda Item:	9.3.2.7

Source:	Huawei, HiSilicon

Title:	Views on the coverage enhancements for non-terrestrial networks

Document for:	Discussion

Introduction

Currently uplink coverage enhancements on PUCCH of Msg4-ACK and PUSCH are discussed in RAN1. And according to the revised WID [1], the following remaining issue will be discussed in RAN#99:

The following sentence will be revisited in RAN#99 as part of the DL enhancements discussion: “The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density.” No work on this topic will take place in RAN WGs before the discussion on DL enhancements in RAN#99.

In this contribution, we provide our view on NTN downlink (or system-level) coverage enhancement.

Proposal on further study of NTN system-level coverage enhancement

Currently, RAN1 focuses on the coverage enhancement of the specific uplink PHY channels (i.e. PUSCH, PUCCH), which enhances the link-level coverage performance based on the simulation assumptions from TR 38.821 [2]. In RAN#98-e, some companies observed that due to unavoidable user behavior (i. e. user usually leaves smart phone in his pocket, briefcase or with certain blockage) and lower transmission power per beam, it is beneficial to consider downlink (DL) coverage enhancement for NR NTN [3]. In this contribution, the issues related to DL coverage are further studied and discussed.

Back to the current Rel-18 work item, the Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement [1]. Unlike the UL coverage enhancement, which can focus on the point-to-point link coverage performance, the discussion on DL coverage should further take the coverage area as well as the satellite capability into consideration.

For NTN, the potential serving area of a satellite is determined by the orbit altitude and the minimal UE elevation angle. Based on the simulation assumptions for NTN coverage enhancement, the edge/minimal UE elevation of LEO satellites’ serving area can be assumed to be 30⁰. Taking Set-1 LEO-600 satellite as an example, the beam diameter that corresponds to the Nadir point of the satellite can be assumed to be 50km as given in Table 6.1.1.1-1 of [2]. The satellite EIRP density and satellite Tx max Gain are assumed as 34 dBW/MHz and 30 dBi, respectively. Then, it can be obtained that the Tx power is 18.8 dBW with the system bandwidth assumption as 30 MHz (S-band).

As discussed in [3], the total transmission power on a satellite may need to be shared among multiple beams, considering there could be multiple UEs in the coverage of different beams simultaneously. Therefore, it is reasonable to take the limited satellite transmission power into consideration when discussing downlink coverage issue.

In order to get a better understanding of the impact due to satellite power limitation, we provide evaluations with LEO-600 and minimum UE elevation angle of 30⁰ as example. Evaluation results are illustrated in Figure 1. Note that the available maximum Tx power is determined by the actual design of the satellite, and here in the evaluation, we simply take the maximum Tx power of 18.8 dBW which is calculated based on the assumption in TR 38.821 as an example for analysis. In Figure 1, the curve of “LEO600 CNR” is obtained by assuming that the total power is shared among the beams transmitted simultaneously, therefore, the CNR at edge of serving area that can be provided by the satellite decreases when the number of active beams increases. The required SNRs of DL channels are illustrated by the horizontal red lines based on the simulation from [4]. By comparing the curves of CNR from link budget analysis and the required SNR, we see that the maximum number of active beams that can be simultaneously transmitted is 24. Meanwhile, the coverage percentage for LEO-600 is illustrated as the blue line, which represents the percentage of the simultaneously transmitting beams among the total beams of the satellite, assuming beam diameter of 50km. It can be further observed that, to satisfy the required SNR for different DL PHY channels/signals, the coverage percentage can only be up to 2% of the potential serving area of a satellite. 100% coverage cannot be achieved with just few dB link budget enhancement.

Figure 1 Link budget and coverage percentage against the number of active beams

Observation 1: With limited total transmission power and based on the required SNR of physical channels supported for NR NTN up to Rel-17, a satellite can only serve a small part of the potential coverage area at a time.

The evaluation in Figure 1 assumes the minimum elevation angle is 30 degree, which has impact on the total required beams per satellite. We also look into impact of minimum UE elevation angle and calculate how many beams and how many satellites are needed with 50km beam diameter assumption. Results are provided in Table 1. It can be observed that even with minimal UE elevation angle increased to 60 degree, 156 beams are required for continuous service within the potential serving area of a satellite, and under such condition 3306 satellites are needed for global coverage.

Table 1 Required beam number and satellite number for continue coverage

| Minimal UE Elevation Angle (degree) | Required Number of Beams per Satellite | Required Number of Satellites for Global Coverage |
| --- | --- | --- |
| 30 | 1143 | 462 |
| 40 | 610 | 855 |
| 50 | 321 | 1586 |
| 60 | 156 | 3306 |

With limited satellite capability, it is hard to support so many simultaneously transmitted beams while ensuring the link budget for DL coverage, especially in case when reflector antennas are used as assumed in TR 38.811 [5].

In conclusion, we need to solve the coverage issue not only from a link point of view but also from a system point of view. Also, it should be noticed that the current study and work for NTN assumes a small number of beams on a satellite. Therefore, it may be needed to investigate more suitable system parameter assumptions, e.g. satellite assumption for larger number of beams, and to study methods for improving the operation of satellite beams, etc. to resolve the coverage issue. If this investigation can’t be finished in Rel-18 we can continue the study in Rel-19.

Proposal 1: It is insufficient to consider only link-level coverage but not the system-level coverage, when DL coverage enhancement is discussed. Further investigation is needed for more suitable satellite parameter assumptions and system-level coverage enhancement solutions.

Conclusion

This contribution provides our analysis on NTN downlink (or system-level) coverage enhancement. Based on the analysis, we have below observation and proposal:

Observation 1: With limited total transmission power and based on the required SNR of physical channels supported for NR NTN up to Rel-17, a satellite can only serve a small part of the potential coverage area at a time.

Proposal 1: It is insufficient to consider only link-level coverage but not the system-level coverage, when DL coverage enhancement is discussed. Further investigation is needed for more suitable satellite parameter assumptions and system-level coverage enhancement solutions.

- References

RP-223534, Revised WID: NR NTN (Non-Terrestrial Networks) enhancements, Thales

TR 38.821, Solutions for NR to support non-terrestrial networks (NTN), 3GPP

RP-223205, On Rel-18 NR NTN Downlink Coverage Enhancement, Apple, Thales, Hughes/EchoStar, Lockheed Martin

R1-2205856, Discussion on coverage enhancement for NR NTN, Huawei, HiSilicon

TR 38.811, Study on New Radio (NR) to support non-terrestrial networks, 3GPP