3GPP TSG-RAN WG3 Meeting #119	R3-230311

Athens, GR, 27 Feb – 03 Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.483 | CR | 0049 | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on providing MBS Session Associated Information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, CBN, China Unicom, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Samsung, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2023-02-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MBS Session Associated Information IE provides the means to establish a MC MBS session level forwarding resource to support handover to a gNB not supporting NR MBS. And as defined in 9.3.1.134 and 9.3.1.135, this IE should be included in the MC Forwrading Resource Indication IE, but in asn.1 the IE is included in the MC Forwarding Resource Request IE instead. Details could refer to R3-225246. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In asn.1 part, move the mbsSessionAssociatedInformation from MCForwardingResourceRequest to MCForwardingResourceIndication. Impact Analysis: This CR has an isolated impact towards the previous version of the specification, and implemented in NBC way to avoid uncessary complexity. This CR only have impacts on the MBS multicast data forwarding function. This CR is non-backwards compatible with previous version of specification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors remain in specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.134, 9.3.1.135, 9.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------Start of the first change-------

#### 9.3.1.134	MC Forwarding Resource Request

This IE is used by the gNB-CU-CP for request from the gNB-CU-UP information from the peer node regarding a MC Forwarding Resource.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Forwarding Resource ID | M |  | 9.3.1.132 |  |
| MBS Area Session ID | O |  | 9.3.1.111 |  |
| MRB Forwarding Resource Request List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MRB Progress Information Type | O |  | 9.3.1.131 | Requests MRB Progress Information of the indicated type from the peer node |
| >MRB Forwarding Address Request | O |  | ENUMERATED (request, ...) |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofMRBs | Maximum no. of MRBs for one MBS Session. Value is 32. |

#### 9.3.1.135	MC Forwarding Resource Indication

This IE is used by the gNB-CU-CP for indicate to the gNB-CU-UP information from the peer node regarding MC Forwarding Resources.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| MC Forwarding Resource ID | M |  | 9.3.1.132 |  |
| MRB Forwarding Indication List |  | 0..<maxnoofMRBs> |  |  |
| >MRB ID | M |  | 9.3.1.16a |  |
| >MRB Progress Information | O |  | 9.3.1.130 | Provides MRB Progress Information from the peer node. |
| >MRB Forwarding Address | O |  | UP Transport Layer Information 9.3.2.1 |  |
| MBS Session Associated Information | O |  | 9.3.1.133 |  |

------Start of the second change-------

### 9.4.5	Information Element Definitions

-- ASN1START

-- **************************************************************

--

#### -- Information Element Definitions

--

-- **************************************************************

E1AP-IEs {

itu-t (0) identified-organization (4) etsi (0) mobileDomain (0)

ngran-access (22) modules (3) e1ap (5) version1 (1) e1ap-IEs (2) }

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

id-CommonNetworkInstance,

id-SNSSAI,

id-OldQoSFlowMap-ULendmarkerexpected,

//skip the unchanged part

-- MCForwardingResourceRequest

MCForwardingResourceRequest ::= SEQUENCE {

mcForwardingResourceID				MCForwardingResourceID,

mbsAreaSession-ID					MBSAreaSessionID					OPTIONAL,

mrbForwardingResourceRequestList	MRBForwardingResourceRequestList	OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCForwardingResourceRequest-ExtIEs} }	OPTIONAL,

...

}

MCForwardingResourceRequest-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRBForwardingResourceRequestList ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF MRBForwardingResourceRequest-Item

MRBForwardingResourceRequest-Item	::=	SEQUENCE {

mrb-ID								MRB-ID,

mrbProgressRequestType				MRB-ProgressInformationType	OPTIONAL,

mrbForwardingAddressRequest			ENUMERATED {request, ...}	OPTIONAL,

iE-Extensions							ProtocolExtensionContainer	{ {MRBForwardingResourceRequest-Item-ExtIEs} }	OPTIONAL,

...

}

MRBForwardingResourceRequest-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

-- MCForwardingResourceIndication

MCForwardingResourceIndication ::= SEQUENCE {

mcForwardingResourceID				MCForwardingResourceID,

mrbForwardingResourceIndicationList	MRBForwardingResourceIndicationList		OPTIONAL,

mbsSessionAssociatedInformation		MBSSessionAssociatedInformation			OPTIONAL,

iE-Extensions		ProtocolExtensionContainer { {MCForwardingResourceIndication-ExtIEs} }	OPTIONAL,

...

}

MCForwardingResourceIndication-ExtIEs E1AP-PROTOCOL-EXTENSION ::= {

...

}

MRBForwardingResourceIndicationList ::= SEQUENCE (SIZE(1.. maxnoofQoSFlows)) OF MRBForwardingResourceIndication-Item

MRBForwardingResourceIndication-Item	::=	SEQUENCE {

mrb-ID								MRB-ID,

mrb-ProgressInformation				MRB-ProgressInformation		OPTIONAL,

mrbForwardingAddress				UP-TNL-Information			OPTIONAL,

iE-Extensions						ProtocolExtensionContainer	{ {MRBForwardingResourceIndication-Item-ExtIEs} }	OPTIONAL,

...

}

MRBForwardingResourceIndication-Item-ExtIEs		E1AP-PROTOCOL-EXTENSION ::= {

...

}

------End of the changes-------