3GPP TSG-RAN WG3 #119	R3-230876

# Athens, Greece, 27th Feb – 3rd Mar, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.473 | CR | 1138 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TS 38.473 on Re-routing Enable Indicator |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Qualcomm, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R3 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IAB_enh-Core |  |  |  |  |  | Date: |  |  | 2023-02-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the description of BAP Mapping Configuration procedure, TS 38.340 is referred to on the IAB-DU behavior upon receipt of the Re-routing Enable Indicator IE as copied in the below: If the Re-routing Enable Indicator IE is included in the BAP MAPPING CONFIGURATION message, and the value is set as “false”, the gNB-DU shall, if supported, disable the inter-donor-DU re-routing, as specified in TS 38.340 [30]. If the Re-routing Enable Indicator IE is included in the BAP MAPPING CONFIGURATION message, and the value is set as “true”, the gNB-DU shall, if supported, enable the inter-donor-DU re-routing.. However, the disabling behavior in the above case (i.e. when the Re-routing Enable Indicator IE is included and the value is set as “false”) is not specified in TS 38.340. Instead, the enabling behavior when the Re-routing Enable Indicator IE is included and the value is set as “true” is captured in TS 38.340. So the description of the IAB-DU behavior upon receipt of the Re-routing Enable Indicator IE in TS 38.473 needs to be corrected and be aligned with TS 38.304. |
| Summary of change: |  | The description of the IAB-DU behavior upon receipt of the Re-routing Enable Indicator IE is corrected. The reference to TS 38.340 is removed in the case when the Re-routing Enable Indicator IE is included and the value is set as “false”. And the reference to TS 38.340 is added in the case when the Re-routing Enable Indicator IE is included and the value is set as “true”. Impact analysis Impact assessment towards the previous version of the specification (same release): This CR has isolated impact with the previous version of the specification (same release). This CR has impact under functional point of view. The impact can be considered isolated because the change affects only the IAB related procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The IAB-DU behavior upon receipt of the Re-routing Enable Indicator IE is not clear and not aligned with TS 38.340. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | - |  |  |  |  |  |  |  |  |

| This CR's revision history: | Rev 1: R3-230716 - Move as specified in TS 38.340 [30] to the end of the second sentence - Other changes are not needed |
| --- | --- |

-------------------------------------------Start of changes-------------------------------------------

#### 8.10.1.2	Successful Operation

Figure 8.10.1.2-1: BAP Mapping Configuration procedure: Successful Operation

The gNB-CU initiates the procedure by sending BAP MAPPING CONFIGURATION message to the gNB-DU. The gNB-DU replies to the gNB-CU with BAP MAPPING CONFIGURATION ACKNOWLEDGE.

If BH Routing Information Added List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, store the BH routing information from this IE and use it for DL/UL traffic forwarding as specified in TS 38.340 [30]. If BH Routing Information Added List IE contains information for an existing BAP Routing ID, the gNB-DU shall, if supported, replace the previously stored routing information for this BAP Routing ID with the corresponding information in the BH Routing Information Added List IE.

If BH Routing Information Removed List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, remove the BH routing information according to such IE.

If the Traffic Mapping Information IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, process the Traffic Mapping Information IE as follows:

-	if the IP to layer2 Traffic Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the IP to layer2 Traffic Mapping Info To Add IE, if present, and remove the previously stored mapping information as indicated by the IP to layer2 Traffic Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored for the mapping of IP traffic to layer 2, as specified in TS 38.340 [30].

-	if the BAP layer BH RLC channel Mapping Info IE is included, the gNB-DU shall store the mapping information contained in the BAP layer BH RLC channel Mapping Info To Add IE, if present, and remove the previously stored mapping information as indicated by the BAP layer BH RLC channel Mapping Info To Remove IE, if present. The gNB-DU shall use the mapping information stored when forwarding traffic on BAP sublayer, as specified in TS 38.340 [30].

If the Buffer Size Threshold IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, use it to determine the DL congestion based on the flow control feedback from child IAB-nodes as specified in TS 38.340 [30].

If BAP Header Rewriting Added List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, store the BAP header rewriting configuration from this IE, and use it as specified in TS 38.340 [30]. If BAP Header Rewriting Added List IE contains information for an existing ingress BAP Routing ID, the gNB-DU shall, if supported, replace the previously stored BAP header rewriting configuration for this ingress BAP Routing ID with the corresponding information in the BAP Header Rewriting Added List IE.

If BAP Header Rewriting Removed List IE is included in the BAP MAPPING CONFIGURATION message, the gNB-DU shall, if supported, remove the BAP header rewriting configuration according to such IE.

If the Re-routing Enable Indicator IE is included in the BAP MAPPING CONFIGURATION message, and the value is set as “false”, the gNB-DU shall, if supported, disable the inter-donor-DU re-routing. If the Re-routing Enable Indicator IE is included in the BAP MAPPING CONFIGURATION message, and the value is set as “true”, the gNB-DU shall, if supported, enable the inter-donor-DU re-routing, as specified in TS 38.340 [30].

-------------------------------------------End of change-------------------------------------------