3GPP TSG RAN meeting #99										RP-230662

Rotterdam, Netherlands, March 20-23, 2023

## Status Report to TSG

Agenda item:			9.5.1.1

| WI / SI Name | Extending current NR operation to 71GHz |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: No | Performance part: Yes |  | Testing part: No |
| Acronym | NR_ext_to_71GHz |  |  |  |  |
| Unique ID | 860041 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-222656 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: N/A | Core part: 09/2022 | Performance part: 03/2023 | Testing part: N/A |  |
| Overall Completion level | Study Item: N/A | Core part: 100% | Performance Part: 100% | Testing part: N/A |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Jing Sun (RAN1); Aida Vera Lopez (RAN4) |
|  | Company | Qualcomm Incorporated; Intel Corporation |
|  | Email | jingsun@qti.qualcomm.com; aida.l.vera.lopez@intel.com |

## 1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | No |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

## 2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

## 2.1	RAN1

#### 2.1.1	Agreements

#### 2.1.2	Remaining Open issues

RAN1 completed the work. Remaining details will be completed in maintenance phase.

## 2.2	RAN2

#### 2.2.1	Agreements

#### 2.2.2	Remaining Open issues

RAN2 completed the work. Remaining details will be completed in maintenance phase.

## 2.3	RAN3

#### 2.3.1	Agreements

#### 2.3.2	Remaining Open issues

## 2.4	RAN4

#### 2.4.1	Agreements

##### 2.4.1.1	BS RF conformance testing

RAN4 #106

Test Tolerance values and all OTA test requirements have been completed. Detailed agreements are captured below.

MU budget on Tx side (R4-2302994)

- Agreements:

  - C1-1 contributor (uncertainty of the RF power measurement equipment)

    - C1-1_PM = [0.98] dB, assuming appropriate description of the calibration procedure update will be captured in TR 37.941

  - C1-9 contributor (RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2)

    - C1-9 = [2.61] dB

  - C1-3 contributor (uncertainty of the network analyzer)

    - C1-3 = [0.85] dB

  - C1-4 contributor (uncertainty of the absolute gain of the reference antenna)

    - C1-4 = 0.3 dB for both in-band and out-of-band

  - A2-20 contributor (uncertainty of the mixer (FR2 only))

    - A2-20 = 2.25 dB for in-band and 2.35 dB for out-of-band

    - RAN4 should make sure the mixer number is not counted twice

  - RC

    - Re-use RC specific MU contributors based on R4-2302231

    - TRP summation error from FR2-1 will be reused: [1.2] dB

  - Use of power meter / power sensor as power measurement equipment for EIRP test case assumes the following conditions are met:

    - Measured signal power level needs to be strong enough to fall within the dynamic range of the power meter when considering the overall path loss in the OTA system

    - Measured signal needs to be sufficiently clean from spurious emissions like harmonics, images, LO-leakage, etc., to maintain expected MU

      - To ensure noise level does not impact the expected MU, the minimum SNR must be >10dB

    - Wideband noise level (including unfiltered components in point above) must be significantly lower than the measured carrier power level

      - To reduce wideband noise level, the use of band pass filter before power sensor is assumed

MU budget on Rx side (R4-2302916)

- Agreements:

  - Rx EIS measurement

    - Use power sensor as monitoring and controlling equipment of signal generator (with or without up converter) to reduce generator MU

  - Uncertainty due to use of PA

    - MUPA = 0 dB

  - B2-5 contributor (mismatch of transmit chain, i.e., between transmitting measurement antenna and BS)

    - B2-5 = 0.6 dB

  - MU excel sheets for Rx tests are included with approved WF (R4-2302916)

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2303749 | Topic summary for [106][304] NR_exto71GHz_ BSRF | Moderator (Huawei) | Noted |
| R4-2302871 | Ad-hoc minutes for FR2-2 MU | Huawei | Agreed |
| R4-2302916 | WF for MU budget on Receiver side | Keysight | Approved |
| R4-2302994 | WF for MU budget on Transmitter side | Ericsson, Keysight, Rohde & Schwarz, Huawei | Approved |
| R4-2301915 | Draft CR to 38.141-2: Correction on Rx intermodulation requirements for FR2-2 (Rel-17) | NEC | Endorsed |
| R4-2301916 | Draft CR to 38.141-2: Correction on Rx intermodulation requirements for FR2-2 (Rel-18) | NEC | Endorsed |
| R4-2302229 | Draft CR to TS 38.141-2: Addition of radiated transmit power requirement for FR2-2 in subclause 6.2 | Ericsson | Endorsed |
| R4-2302230 | Draft CR to TS 38.141-2: Addition of radiated transmit power requirement for FR2-2 in subclause 6.2 | Ericsson | Endorsed |
| R4-2302875 | Big CR to TS 38.141-2: FR2-2 BS RF test requirements introduction, Rel-17 | Huawei, HiSilicon | Agreed |
| R4-2302876 | Big CR to TS 38.141-2: FR2-2 BS RF test requirements introduction, Rel-18 | Huawei, HiSilicon | Agreed |

##### 2.4.1.2	Demodulation performance

RAN4 #106

UE demodulation (R4-2302972)

- Agreements:

  - PDCCH Configuration for PDSCH Requirements with PDSCH Allocation=20RBs

    - CORESET BW=18PRB, Duration = 2Sym, AL=4 (Baseline, Nokia)

  - Test configuration for single carrier and FR1 + FR2-2 CA test with reduced RB allocation (120kHz, MCS 17, 32RBs)

    - Do not introduce test with reduced RB allocation for single carrier or CA

  - Applicability rules stating FR2-2 UE demodulation and CSI requirements only apply to regions and countries where LBT is not mandatory

    - No applicable rules needed for both BS and UE performance requirements

  - PDCCH requirements table for FR1+FR2-2 CA

    - 100MHz/120KHz

      - PDCCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC)

      - RB allocation for FR2-2 CC is aligned with single CC requirements configuration

    - 400MHz/480KHz

      - PDCCH requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC)

      - RB allocation for FR2-2 CC is aligned with single CC requirements configuration

  - CQI configuration for high SNR test point

    - Use [7,8] dB SNR for CQI reporting requirements

  - CQI requirements table for FR1+FR2-2 CA

    - 100MHz/120KHz

      - CQI requirements: FR1(40MHz, single CC) + FR2-2(400MHz, single CC)

      - RB allocation for FR2-2 CC is 66 RBs

BS demodulation (R4-2302971)

- Agreements:

  - Rel-15 rule for performance requirements derivation

    - Rule can be used as starting point for the valid test cases

  - LOS component modelling for TDL-D

    - Adopt UE demodulation model

  - MCS and number of Tx/Rx branches for 16 QAM PUSCH requirements

    - For 1x2 Low, use TDLA30-650 for 100MHz CBW, TDLA10-650 for 400MHz CBW

    - For 2x2 Low, use TDLA30-200 for 100MHz CBW, TDLA10-200 for 400MHz CBW

      - Update requirements on CRs with

        - TDLA30-650 - 120 kHz and 100 MHz – add requirements in []

        - TDLA10-650 - 120 kHz and 400 MHz – add requirements in []

        - TDLA10-650 - 480 kHz and 400 MHz – add requirements in []

      - Simulation results for the following will be discussed in the next meeting

        - MCS16 (TDLA30-200 - 120 kHz and 100 MHz; TDLA10-200 - 120 kHz and 400 MHz; TDLA10-200 - 480 kHz and 400 MHz)

        - MCS12 (TDLA30-650 - 120 kHz and 100 MHz; TDLA10-650 - 120 kHz and 400 MHz; TDLA10-650 - 480 kHz and 400 MHz)

  - MCS and number of Tx/Rx branches for 64 QAM PUSCH requirements

    - For 1x2 Low, introduce MCS 20 for PUSCH requirements

    - There will be no requirements for MCS18 with 2T2R

Relevant documents

| T-doc number | Title | Source | Status |
| --- | --- | --- | --- |
| R4-2303766 | Topic summary for [106][321] NR_exto71GHz_ Demod_Part1 | Moderator (Nokia) | Noted |
| R4-2303767 | Topic summary for [106][322] NR_exto71GHz_ Demod_Part2 | Moderator (Qualcomm) | Noted |
| R4-2302971 | WF for FR2-2 BS demodulation requirements | Nokia | Approved |
| R4-2302972 | WF for FR2-2 UE demodulation and CSI requirements | Qualcomm Inc. | Approved |
| R4-2300515 | Draft CR 38.104: PUSCH requirements for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Endorsed |
| R4-2300517 | Draft CR 38.141-2: PUCCH requirements for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Endorsed |
| R4-2301022 | Draft CR for TS38.104 NR 71GHz FRC table (cat B) | Ericsson | Endorsed |
| R4-2301023 | Draft CR for TS38.104 NR 71GHz FRC table (cat A) | Ericsson | Endorsed |
| R4-2301024 | Draft CR for TS38.141-2 NR 71GHz FRC table (cat B) | Ericsson | Endorsed |
| R4-2301025 | Draft CR for TS38.141-2 NR 71GHz FRC table (cat A) | Ericsson | Endorsed |
| R4-2302846 | Draft CR 38.104: PUSCH requirements for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Endorsed |
| R4-2302847 | Draft CR: Introduction on FR2-2 PUSCH requirements in TS 38.141-2 | Huawei, HiSilicon | Endorsed |
| R4-2302848 | Draft CR: Introduction on FR2-2 PUSCH requirements in TS 38.141-2 | Huawei, HiSilicon | Endorsed |
| R4-2302851 | Draft CR 38.141-2: PUCCH requirements for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Endorsed |
| R4-2302852 | Draft CR: Introduction on FR2-2 PUCCH requirements in TS 38.104 | Huawei, HiSilicon | Endorsed |
| R4-2302853 | Draft CR: Introduction on FR2-2 PUCCH requirements in TS 38.104 | Huawei, HiSilicon | Endorsed |
| R4-2302854 | Draft CR Updated simplification step for FR2-2 channel model in 38.101-4 | Huawei, HiSilicon | Endorsed |
| R4-2302855 | draft CR on PDSCH requirements for 52.6 - 71 GHz band | Ericsson | Endorsed |
| R4-2302856 | Draft CR: Introduction on FR1+FR2-2 CA PDSCH requirements | Huawei, HiSilicon | Endorsed |
| R4-2302862 | DraftCR on PDCCH requirements for ext71GHz (rel-17 v17.7.0) | Nokia, Nokia Shanghai Bell | Endorsed |
| R4-2302928 | Draft CR for FRC and applicable rules of FR2-2 | Apple Inc. | Endorsed |
| R4-2302973 | Draft CR for general applicable rules of FR2-2 | Qualcomm Inc. | Endorsed |
| R4-2302974 | Draft CR for PDCCH and CQI requirements of CA case to FR2-2 | Apple Inc. | Endorsed |
| R4-2302999 | Big CR on FR2-2 UE demodulation performance requirements in TS 38.101-4 | Huawei | Agreed |
| R4-2300518 | Big CR to 38.104: demodulation requirements introduction for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Agreed |
| R4-2300519 | Big CR to 38.104: demodulation requirements introduction for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Agreed |
| R4-2300520 | Big CR to 38.141-2: demodulation requirements introduction for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Agreed |
| R4-2300521 | Big CR to 38.141-2: demodulation requirements introduction for FR2-2 | Nokia, Nokia Shanghai Bell, Intel Corporation | Agreed |

#### 2.4.2	Remaining Open issues

- BS RF conformance testing

  - Measurement uncertainties for spurious emission test are [FFS] and will be handled under Maintenance as they do not impact the test tolerances (and hence, the test requirements)

## 2.5	RAN5

#### 2.5.1	Agreements

#### 2.5.2	Remaining Open issues

#### 2.5.3	Remaining Open issues with cross-WG dependencies

## 2.6	RAN6

#### 2.6.1	Agreements

#### 2.6.2	Remaining Open issues

## 3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

NOTE: This section only needs to be filled in for WI/SIs where there is a corresponding relevant WI/SI in SA/CT.

## 3.1	SAx/CTs

#### 3.1.1	Agreements with cross-TSG impacts

#### 3.1.2	Remaining Open issues with cross-TSG impacts

NOTE: This section should also flag any critical dependencies that need TSG attention.

## 4.	References

NOTE:	This can be e.g. a list of all related Tdocs in the affected WGs since last TSG, references to LSs, produced TRs/TSs, the work/study item description or status reports of previous TSGs.

RAN4 #106

- R4-2300185, “CR to TS 38.104 on table references for OTA operating band unwanted emission limits,” Nokia, Nokia Shanghai Bell

- R4-2300186, “Proposals on measurement uncertainty of BS OTA transmitter requirements for extending current NR operation to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2300187, “Proposals on measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2300188, “Draft CR to TS 38.141-2 in clauses 6.3 to 6.5 for extending current NR operation to 71GHz,” Nokia, Nokia Shanghai Bell

- R4-2300253, “On PDSCH demod requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2300254, “Simulation results for PDCCH demod requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2300258, “Draft LS to RAN1 on L3-RSSI measurement for NR up to 71GHz,” Apple Inc.

- R4-2300259, “CR on L3-RSSI measurement for NR up to 71GHz,” Apple Inc.

- R4-2300260, “CR on L3-RSSI measurement for NR up to 71GHz,” Apple Inc.

- R4-2300493, “FR2-2 maintenance – power class aspects,” Intel Corporation

- R4-2300494, “CR to 38.101-2: FR2-2 power class content,” Intel Corporation

- R4-2300495, “CR for TS 38.101-2: FR2-2 power class content,” Intel Corporation

- R4-2300512, “Discussion on PUSCH demodulation requirements for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2300513, “PUSCH simulation results for the extension to 71 GHz,” Nokia, Nokia Shanghai Bell

- R4-2300514, “Draft CR 38.104: PUSCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2300515, “Draft CR 38.104: PUSCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2300516, “Draft CR 38.141-2: PUCCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2300517, “Draft CR 38.141-2: PUCCH requirements for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2300518, “Big CR to 38.104: demodulation requirements introduction for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2300519, “Big CR to 38.104: demodulation requirements introduction for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2300520, “Big CR to 38.141-2: demodulation requirements introduction for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2300521, “Big CR to 38.141-2: demodulation requirements introduction for FR2-2,” Nokia, Nokia Shanghai Bell, Intel Corporation

- R4-2300580, “Draft CR for 37.941, On FR2-2 RX directional requirements in clauses 10.1-10.5 and respective MU and TT summary,” CATT

- R4-2300581, “Discussion on MU for adjacent channel selectivity, in-band blocking, in-channel selectivity for FR2-2,” CATT

- R4-2300640, “Update to FR2-2 Test model,” ROHDE & SCHWARZ

- R4-2300697, “On PDSCH Requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2300698, “PDSCH simulation results for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2300699, “DraftCR on PDCCH requirements for ext71GHz (rel-17 v17.7.0),” Nokia, Nokia Shanghai Bell

- R4-2300700, “On CSI reporting requirements for ext71GHz,” Nokia, Nokia Shanghai Bell

- R4-2301021, “Discussion on NR 71GHz PUSCh demodulation requirement,” Ericsson

- R4-2301022, “Draft CR for TS38.104 NR 71GHz FRC table (cat B),” Ericsson

- R4-2301023, “Draft CR for TS38.104 NR 71GHz FRC table (cat A),” Ericsson

- R4-2301024, “Draft CR for TS38.141-2 NR 71GHz FRC table (cat B),” Ericsson

- R4-2301025, “Draft CR for TS38.141-2 NR 71GHz FRC table (cat A),” Ericsson

- R4-2301520, “Frequency range definition update for TS 38.101-3 (Rel-15),” vivo

- R4-2301522, “Frequency range definition update for TS 38.101-3 (Rel-17),” vivo

- R4-2301523, “Frequency range definition update for TS 38.101-3 (Rel-18),” vivo

- R4-2301524, “Frequency range definition update for TS 38.101-1 (Rel-18),” vivo

- R4-2301525, “Frequency range definition update for TS 38.101-1 (Rel-17),” vivo

- R4-2301530, “Discussion on frequency range definition update for TS 38.101-1/2/3,” vivo

- R4-2301675, “draftCR to include FR2-2 range to 38.101-1 (Rel-17),” Nokia, Nokia Shanghai Bell

- R4-2301791, “CR on RSSI measurement for FR2-2 R17,” Huawei, HiSilicon

- R4-2301792, “CR on RSSI measurement for FR2-2 R18,” Huawei, HiSilicon

- R4-2301793, “CR on CCA model for FR2-2 R17,” Huawei, HiSilicon

- R4-2301794, “CR on CCA model for FR2-2 R18,” Huawei, HiSilicon

- R4-2301795, “CR on test case maintenance for FR2-2 R17,” Huawei, HiSilicon

- R4-2301796, “CR on test case maintenance for FR2-2 R18,” Huawei, HiSilicon

- R4-2301886, “The remaining issues of the PDSCH requirements in 52.6 – 71 GHz band,” Ericsson

- R4-2301887, “Simulation results for PDSCH demodulation in 52.6 – 71 GHz band,” Ericsson

- R4-2301888, “The remaining issues for PDCCH requirements in FR2-2,” Ericsson

- R4-2301889, “Simulation results for PDCCH in FR2-2,” Ericsson

- R4-2301890, “CSI reporting requirements in 52.6 – 71 GHz band,” Ericsson

- R4-2301891, “Collection of Simulation Results for UE demodulation in FR2-2,” Ericsson

- R4-2301892, “draft CR on PDSCH requirements for 52.6 - 71 GHz band,” Ericsson

- R4-2301913, “Draft CR to 38.104: Correction on Rx intermodulation requirements for FR2-2 (Rel-17),” NEC

- R4-2301914, “Draft CR to 38.104: Correction on Rx intermodulation requirements for FR2-2 (Rel-18),” NEC

- R4-2301915, “Draft CR to 38.141-2: Correction on Rx intermodulation requirements for FR2-2 (Rel-17),” NEC

- R4-2301916, “Draft CR to 38.141-2: Correction on Rx intermodulation requirements for FR2-2 (Rel-18),” NEC

- R4-2302160, “Discussion on remaining issues for FR2-2 UE requirements,” Huawei, HiSilicon

- R4-2302161, “Simulation results on FR2-2 PDCCH and PBCH requirements,” Huawei, HiSilicon

- R4-2302162, “Simulation and discussions on FR2-2 CQI requirements,” Huawei, HiSilicon

- R4-2302163, “Summary of simulation results for FR2-2 BS requirements,” Huawei, HiSilicon

- R4-2302164, “Discussion on remaining issues on FR2-2 PUSCH demodulation requirements,” Huawei, HiSilicon

- R4-2302165, “Updated simulation results for FR2-2 PUCCH demodulation requirements,” Huawei, HiSilicon

- R4-2302166, “Draft CR: Introduction on FR1+FR2-2 CA PDSCH requirements,” Huawei, HiSilicon

- R4-2302167, “Draft CR: Introduction on FR2-2 PUSCH requirements in TS 38.141-2,” Huawei, HiSilicon

- R4-2302168, “Draft CR: Introduction on FR2-2 PUCCH requirements in TS 38.104,” Huawei, HiSilicon

- R4-2302226, “Draft CR to TR 37.941: Addition of aspects related to EIRP measurement in CATR relevant for FR2-2 in sub-clause 7.3, 8.3, 9.2.3, 9.2.7, 9.2.8, 9.3.3, 9.3.4 and 9.3.5,” Ericsson

- R4-2302227, “Draft CR to TR 37.941: Addition of aspects related to EIS measurement in CATR relevant for FR2-2 in sub-clause 10.2.3 and sub-clause 10.2.7,” Ericsson

- R4-2302228, “Draft CR to TR 37.941: Addition of aspects related to TRP measurement in RC relevant for FR2-2 in subclause 11.2.5, 11.3.5, 11.4.5 and 12.2.4,” Ericsson

- R4-2302229, “Draft CR to TS 38.141-2: Addition of radiated transmit power requirement for FR2-2 in subclause 6.2,” Ericsson

- R4-2302230, “Draft CR to TS 38.141-2: Addition of radiated transmit power requirement for FR2-2 in subclause 6.2,” Ericsson

- R4-2302231, “On aspects related to FR2-2 transmitter conformance testing,” Ericsson

- R4-2302232, “On aspects related to FR2-2 receiver conformance testing,” Ericsson

- R4-2302233, “On general aspects relevant for FR2-2 conformance testing,” Ericsson

- R4-2302235, “Draft CR to TR 37.941: Addition of technical background information related to test of receiver out-of-band blocking in a general chamber in subclause 14.2,” Ericsson

- R4-2302260, “CR to TS 38.104 on table references for OTA operating band unwanted emission limits,” Nokia, Nokia Shanghai Bell

- R4-2302265, “FR2-2 Test equipment MU update for BS conformance testing,” Keysight Technologies UK Ltd

- R4-2302266, “FR2-2 BS conformance test system, Test system consideration,” Keysight Technologies UK Ltd

- R4-2302267, “draftCR to 38.141-2: 71 GHz Extension BS EVM Clause 6.6.3 (6.6.3),” Keysight Technologies UK Ltd

- R4-2302268, “DraftCR to 37.941: 71 GHz Extension TE MU update,” Keysight Technologies UK Ltd

- R4-2302328, “Draft CR Updated simplification step for FR2-2 channel model in 38.101-4,” Huawei, HiSilicon

- R4-2302461, “CR to TS 38.104: NR-ARFCN table reference correction for band n263, Rel-17,” Huawei, HiSilicon

- R4-2302462, “CR to TS 38.104: NR-ARFCN table reference correction for band n263, Rel-18,” Huawei, HiSilicon

- R4-2302463, “Big CR to TS 38.141-2: FR2-2 BS RF test requirements introduction, Rel-17,” Huawei, HiSilicon

- R4-2302484, “Discussion on General FR2-2 UE Demodulation,” Qualcomm Inc.

- R4-2302485, “Simulation Results for FR2-2 UE Demodulation PDSCH,” Qualcomm Inc.

- R4-2302486, “draftCR for corrections to R4-R4-2219514 BigCR for FR2-2 UE Demodulation and CQI Requirements,” Qualcomm Inc.

- R4-2302606, “Discussion and simulation results on PUSCH demodulation requirements for Rel-17 71GHz,” Samsung

- R4-2302681, “CR on TC - L1 RSRP measurement for beam reporting,” Qualcomm Inc.

- R4-2302682, “CR on TC - L1 RSRP measurement for beam reporting,” Qualcomm Inc.

- R4-2302736, “Draft CR on PDSCH demod requirements for 52.6 - 71 GHz,” Apple Inc.

- R4-2302763, “Topic summary for [106][205] NR_ext_to_71GHz,” Moderator (Qualcomm)

- R4-2302872, “CR to 38.104: Correction on Rx intermodulation requirements for FR2-2 (Rel-17),” NEC

- R4-2302873, “CR to 38.104: Correction on Rx intermodulation requirements for FR2-2 (Rel-18),” NEC

- R4-2302875, “Big CR to TS 38.141-2: FR2-2 BS RF test requirements introduction, Rel-17,” Huawei, HiSilicon

- R4-2302876, “Big CR to TS 38.141-2: FR2-2 BS RF test requirements introduction, Rel-18,” Huawei, HiSilicon

- R4-2302999, “Big CR on FR2-2 UE demodulation performance requirements in TS 38.101-4,” Huawei

- R4-2303256, “CR on TC - L1 RSRP measurement for beam reporting,” Qualcomm Inc.

- R4-2303749, “Topic summary for [106][304] NR_exto71GHz_BSRF,” Moderator (Huawei)

- R4-2303766, “Topic summary for [106][321] NR_exto71GHz_Demod_Part1,” Moderator (Nokia)

- R4-2303767, “Topic summary for [106][322] NR_exto71GHz_Demod_Part2,” Moderator (Qualcomm)

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template