3GPP TSG RAN Meeting #99	RP-230676

Rotterdam, Netherlands, 20-23 March, 2023

Source:	Huawei, HiSilicon

Title:	pCR for TR 38.848: Study on Ambient IoT (Internet of Things) in RAN

Agenda item:	9.2.3

Document for:	Agreement

- Introduction

This pCR proposes to incorporate into TR 38.848 the changes which would arise from [2] – [6]. We would intend to captured from [7] after further discussions.

This pCR must be read in conjunction with that in [1], which shows an implementation of agreements reached in RAN#98-e. Where practicable, the present pCR does not duplicate those changes, and hence the text shown in [1] should also be understood as being present. Where ease of understanding requires, we have nevertheless included some pCR text from [1], marked with a different change-tracking editor name. We rely on the TR editor to manage the change-on-change.

- References

- RP-230571, “pCR for TR 38.848 on RAN#98e agreements for Ambient IoT”, Huawei, HiSilicon, RAN#99, Rotterdam, Netherlands, March 2023.

- RP-230670, “Use case grouping for Ambient IoT”, Huawei, HiSilicon, RAN#99, Rotterdam, Netherlands, March 2023.

- RP-230671, “Ambient IoT deployment scenarios and their characteristics”, Huawei, HiSilicon, RAN#99, Rotterdam, Netherlands, March 2023.

- RP-230672, “Connectivity topologies for Ambient IoT”, Huawei, HiSilicon, RAN#99, Rotterdam, Netherlands, March 2023.

- RP-230673, “Ambient IoT device characteristics and categorization”, Huawei, HiSilicon, RAN#99, Rotterdam, Netherlands, March 2023.

- RP-230674, “RAN design targets for Ambient IoT”, Huawei, HiSilicon, RAN#99, Rotterdam, Netherlands, March 2023.

- RP-230675, “Feasibility assessment and required functionalities for Ambient IoT”, Huawei, HiSilicon, RAN#99, Rotterdam, Netherlands, March 2023.

- Proposed changes

# <Unchanged text is omitted>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

IoT	Internet of Things

LPWA	Low-power, wide-area

LTE-MTC	Long Term Evolution – Machine Type Communication

NB-IoT	Narrowband IoT

rUC	Representative use case

RFID	Radio-frequency identification

UC	Use case

# 4	Deployment scenarios, use cases, services

## 4.1	Use cases/services

The use cases described above are formed into six representative use cases for the purpose of identifying RAN-based characteristics that allow them to be matched to deployment scenarios in Clause 4.2.

### 4.1.1	Representative use case 1: Indoor large service area with continuous coverage

In rUC1, Ambient IoT service is requested over large service area varying from several thousand to several hundred thousand square meters. The service area size is assumed to be far beyond the coverage (e.g., around or less than 1000 m2) of a single indoor node. In addition, continuous coverage over the service area is usually necessary or recommended.

### 4.1.2	Representative use case 2: Indoor small service area with continuous coverage

In rUC2, one or only a few nodes are assumed to be sufficient to cover a small service area of a few thousand square meters or less, such as a basestation machine room or cattle stable. Both basestation and other types of node (e.g., relay nodes) can be considered to support Ambient IoT communication. Continuous coverage is expected.

### 4.1.3	Representative use case 3: Indoor home or personal area

In rUC3, Ambient IoT service is requested over a personal area or at home. Indoor basestation presence is not assumed. In case of insufficient link budget for outdoor-to-indoor coverage of outdoor basestations, the Ambient IoT device can communicate with e.g. a personal cell phone for UCs such as ranging at home and museum guiding, or an intermediate node for UCs such as smart homes and smart laundry. Whether the coverage is outdoor-to-indoor or indoor-to-indoor can vary.

### 4.1.4	Representative use case 4: Outdoor large service area with continuous coverage

In rUC4, Ambient IoT service is requested over large local area or wide area. In some use cases, hundreds or thousands of Ambient IoT devices can be widely dispersed in a local area from several ten thousand square meters to several square kilometres.

### 4.1.5	Representative use case 5: Outdoor small service area

In rUC5, Ambient IoT service is only required at some particular locations, e.g. tollgates along roads. In this case, the Ambient IoT device may communicate with nearby basestations or independently deployed nodes (e.g., relay node or UE type node) at the particular locations.

### 4.1.6	Representative use case 6: Outdoor personal area

In rUC 6, a command will be sent to an Ambient IoT device to indicate the operation of some personal equipment. The application can be satisfied by either personal area coverage from a cell phone or wide area coverage from basestation in outdoor scenarios.

## 4.2	Deployment scenarios

### 4.2.1	Connectivity topologies

The following connectivity topologies for Ambient IoT networks and devices are defined for the purposes of the study. In all these topologies, the Ambient IoT device may be provided with a carrier wave from other node(s) either inside or outside the topology. The links in each topology may be birdirectional or unidirectional.

Combination between different ones among the following connectivity topologies can be regarded as a network planning issue. It is recommended to target for same Ambient IoT air interface for different connectivity topologies. The backhaul or configuration link(s) between other nodes are transparent to Ambient IoT device.

Joint transmitting or receiving from or to multiple nodes is an implementation option which is expected to be helpful for the coverage performance optimization of Ambient IoT. Note that the limited capability of Ambient IoT devices may mean that there is, e.g. possibly no CSI measurement.

#### 4.2.1.1	Topology 1: BS ↔ Ambient IoT device

Figure 4.2.1.1-1: Topology 1

This topology includes the possibility that the BS transmitting to the Ambient IoT device is different from the BS receiving from the Ambient IoT device. In Topology 1, the Ambient IoT device directly and bidirectionally communicates with a basestation. The communication between the basestation and the ambient IoT device includes Ambient IoT data and/or signalling. For device A and B, the Ambient IoT device may be provided with a carrier wave from another node(s) either inside or outside the topology. Topology 1 can be regarded as the baseline for deployment scenarios demanding continuous coverage over a large service area.

#### 4.2.1.2	Topology 2: BS ↔ intermediate node ↔ Ambient IoT device

Figure 4.2.1.2-1: Topology 2

In Topology 2, the Ambient IoT device communicates bidirectionally with an intermediate node between the device and basestation. In this topology, the intermediate node can be a relay, IAB node, UE, repeater, etc. which is capable of Ambient IoT. The intermediate node transfers the information between BS and the Ambient IoT device.  For device A and B, the Ambient IoT device may be provided with a carrier wave from another node(s) either inside or outside the topology. Topology 2 can be regarded as extending the coverage of the basestation in Topology 1.

#### 4.2.1.3	Topology 3: BS ↔ assisting UE ↔ Ambient IoT device ↔ BS

Figure 4.2.1.3-1: Topology 3 with downlink assistance

Figure 4.2.1.3-2: Topology 3 with uplink assistance

In Topology 3, the Ambient IoT device transmits data/signalling to a basestation, and receives data/signalling from the assisting node; or the Ambient IoT device receives data/signalling from a basestation and transmits data/signalling to the assisting node. In this topology, the assisting node can be a relay, IAB, UE, repeater, etc. which is capable of ambient IoT. For device A and B, the Ambient IoT device may be provided with carrier wave from another node(s) either inside or outside the topology. Topology 3 can help when the link budgets between downlink and uplink are balanced in Topology 1.

#### 4.2.1.4	Topology 4: UE ↔ Ambient IoT device

Figure 4.2.1.4-1: Topology 4

In Topology 4, the Ambient IoT device communicates bidirectionally with a UE. The communication between UE and the ambient IoT device includes Ambient IoT data and/or signalling. For device A and B, the Ambient IoT device may be provided with carrier wave from another node(s) either inside or outside the topology. This topology can be applied for some personal or home-oriented applications.

### 4.2.2	Deployment scenario tables

#### 4.2.2.1	Deployment scenario 1: Indoor device, indoor BS

| Applicable representative use cases | Characteristic | Description |
| --- | --- | --- |
| rUC1: indoor large service area rUC2: indoor small service area | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Micro- or pico-cell BS |
|  | Connectivity topology | Topology (1) |
|  | Spectrum | Licensed FDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site deployment or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B |

#### 4.2.2.2	Deployment scenario 2: Indoor device, outdoor BS

| Applicable representative use cases | Characteristic | Description |
| --- | --- | --- |
| rUC2: indoor small service area | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or micro-cell BS |
|  | Connectivity topology | Topology (1) |
|  | Spectrum | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site deployment or new site |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device C |

#### 4.2.2.3	Deployment scenario 3: Indoor device, intermediate node, outdoor BS

| Applicable representative use cases | Characteristic | Description |
| --- | --- | --- |
| rUC2: indoor small service area rUC3: indoor home or personal area | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | Macro- or micro-cell BS |
|  | Connectivity topology | Topology (2) |
|  | Spectrum | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site or new site, new UE, legacy UE |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or C |

#### 4.2.2.4	Deployment scenario 4: Indoor device, UE node as reader

| Applicable representative use cases | Characteristic | Description |
| --- | --- | --- |
| rUC2: indoor small service area rUC3: indoor home or personal area | Environment (of device) | Indoor |
|  | Basestation characteristic (if any) | None |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | legacy UE or new UE |
|  | Traffic assumption | Device terminated and originated |
|  | Device characteristic | Device A or B or C |

#### 4.2.2.5	Deployment scenario 5: Outdoor device, outdoor BS

| Applicable representative use cases | Characteristic | Description |
| --- | --- | --- |
| rUC4: outdoor large service area | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or micro-cell BS |
|  | Connectivity topology | Topology (1) |
|  | Spectrum | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site deployment |
|  | Traffic assumption | Device terminated or originated |
|  | Device characteristic | Device C |

#### 4.2.2.6	Deployment scenario 6: Outdoor device, intermediate node, outdoor BS

| Applicable representative use cases | Characteristic | Description |
| --- | --- | --- |
| rUC5: outdoor small service area rUC6: outdoor personal area | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Macro- or micro-cell BS |
|  | Connectivity topology | Topology (2) |
|  | Spectrum | Licensed FDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | Co-site, new site, new UE, legacy UE |
|  | Traffic assumption | Device terminated |
|  | Device characteristic | Device A or B or C |

#### 4.2.2.7	Deployment scenario 7: Outdoor device, outdoor BS with assisting node

| Applicable representative use cases | Characteristic | Description |
| --- | --- | --- |
| rUC5: outdoor small service area rUC6: outdoor personal area | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | Outdoor macro- or micro-cell BS |
|  | Connectivity topology | Topology (3) |
|  | Spectrum | Licensed FDD |
|  | Coexistence with existing 3GPP technologies | Co-site, new site, new UE, legacy UE |
|  | Traffic assumption | Device terminated |
|  | Device characteristic | Device C |

#### 4.2.2.8	Deployment scenario 8: Outdoor device, UE as reader

| Applicable representative use cases | Characteristic | Description |
| --- | --- | --- |
| rUC5: outdoor small service area rUC6: outdoor personal area | Environment (of device) | Outdoor |
|  | Basestation characteristic (if any) | None |
|  | Device characteristic | Device A or B or C |
|  | Connectivity topology | Topology (4) |
|  | Spectrum | Licensed FDD or Unlicensed |
|  | Coexistence with existing 3GPP technologies | new UE, legacy UE |
|  | Traffic assumption | Device terminated |

## 4.3	Device categorization

Temporary note: Please refer to the pCR implementing RAN98-e agreements for full context of the following changes.

The energy storage used to accumulate energy for reference Device B based on integration of reference Device A with a capacitor to work continuously during an inventory lasting for 1 second or 10 second level is assumed to be at 1 μJ-level or 10 μJ-level, respectively.

Storage capacity 2 is primarily applicable to the reference Device B described above, for which an energy storage of 100 μJ is taken as a reference upper bound for E1.

The energy storage used to persistently supply energy to reference Device C for a duration of hour or day level, needing to support frequent periodic (e.g., 15 minutes period) reporting is assumed to be at 10 mJ level or 100 mJ level, respectively. For such a reference device C, an energy storage of 1 J is taken as a reference upper bound for E2.

# 5	RAN design targets

## 5.1	Power consumption

For Device A, the power consumption of transmitting and of receiving should be approximately 1 μW.

For Device C, the power consumption of transmitting and of receiving should not exceed 1 mW, and strives to be hundreds of μW.

For Device B, the power consumption of transmitting and of receiving should be between Device A and C.

## 5.2 	Complexity

For Device A, the device complexity should be comparable to UHF RFID passive tag.

For Device C, the device complexity should be significantly (e.g., X times) lower than NB-IoT device.

For Device B, the device complexity should be between Device A and C.

## 5.3	Coverage

The target is to support typical 3GPP inter-site distances (ISDs) of indoor base station for all Devices A/B/C, and of outdoor base station for Device C.

RAN1 is expected to decide which channel model(s), ISD(s), etc. are most relevant for WG-level study and evaluation of solutions versus this overall design target.

-	E.g.: indoor coverage target is ISD ≈ 30 m, and outdoor coverage target is ISD ≈ 500 m or 1732 m for urban and rural area, respectively.

Deployment scenarios without basestations have the same target as indoor coverage, regardless of environment.

New-site deployments share the same targets for coverage performance.

## 5.4	Data rate

Minimum supported data rate of 100 bps on both the uplink and downlink.

Maximum data rate no lower than 5 kbps.

## 5.5	Positioning accuracy

3 meters at 90% for indoor location by cellular network.

1-3 meters at 90% for indoor ranging by UE-type nodes.

10 meters at 90% for outdoor location by cellular network.

## 5.6	Connection density

Maximum connection density of 150 devices per 100 m2 for indoor scenarios.

Maximum connection density of 10 devices per 100 m2 for outdoor scenarios.

## 5.7	Latency

Maximum latency of 10 seconds.

Minimum latency no longer than 1 second.

# <Unchanged text is omitted>