3GPP TSG RAN meeting #99										RP-230714

Rotterdam, Netherlands, March 20-23, 2023

Status Report to TSG

Agenda item:			9.3.1.8

| WI / SI Name | Network energy savings for NR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| included in this status report | Study Item: No | Core part: Yes | Performance part: Yes |  | Testing part: No |
| Acronym | Netw_Energy_NR |  |  |  |  |
| Unique ID | 981037 |  |  |  |  |
| TSG Tdoc of latest approved WI/SI description (if any) | RP-223540 |  |  |  |  |
| Target Completion Date (indicate if changed) | Study Item: | Core part: 12/2023 | Performance part: 06/2024 | Testing part: |  |
| Overall Completion level | Study Item: | Core part: 15% | Performance Part: 0% | Testing part: |  |

Note: Overall completion level percentage numbers should use one of the colors below:

- xx%: Normal progress, no RAN plenary action needed

- xx%: Progress behind schedule, may need RAN plenary intervention. If so, SR should clearly define requested action

- xx%: Progress critically behind, RAN plenary shall intervene. SR should define requested action

Source:

| Leading WG |  | RAN1 |
| --- | --- | --- |
| Rapporteur | Name | Yi Wang |
|  | Company | Huawei |
|  | Email | wangyi6@huawei.com |

1	Work plan related evaluation

| Do you want to modify the time budget for this WI/SI compared to what was endorsed at the last RAN meeting? | Yes |
| --- | --- |

If you answered No:	Then please remove the Excel file from the zip file of this status report.

If you answered Yes:	Then please fill out the attached Excel template to request a modification of the time 		budgets for your WI /SI. The Excel table has to be filled out for all affected RAN WGs and 		up to the target date of the WI/SI. The basis are the endorsed time budgets of the last 		RAN meeting. Please highlight all changes of the values.
		One time unit (TU) corresponds to ~ 2 hours in the meeting.
		If this status report covers a WI with Core and Performance part, then please have one 		line for each in the attached Excel table.
		Note: If no Excel table is attached, then this means no time budget change.

Additional explanations/motivations for the time budget changes in the attached Excel table:

The WI includes a core part and a performance part. The TU for performance part is missing. Half of the TUs for RAN4 in Q4 2023 is split for performance part, and TUs for performance part starting from Q1 to Q2 2024 are added.

2.	Detailed progress in RAN WGs since last TSG meeting (for all involved WGs)

NOTE: Agreements and Open issues impacted cross-TSG aspects shall be explicitly highlighted

2.1	RAN1

2.1.1	Agreements

##### RAN1#112

The update of TR 38.864 is agreed in R1-2302234.

R1-2301964	FL summary#1 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases

- Type 1: all antenna elements associated to a logical antenna port is disabled/enabled

- Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

R1-2301965	FL summary#2 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

For spatial element adaptation, further study the following

- A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern

  - FFS: Details on how the association is done

- A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns

  - FFS: Details on how the association is done

- FFS: Details on the definition of “spatial adaptation patterns”

Agreement

For spatial element adaptation, further study the following

- A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern

- A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern

  - FFS: Details of sub-configuration

R1-2301966	FL summary#3 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g.

- FFS: gNB indicates to UE which CSI(s) the UE shall report

- FFS: the UE selects which CSI(s) are reported

- FFS: multiple CSI(s) are reported in a joint CSI report

- FFS: Overhead reduction for multiple CSI(s)

Note: UE complexity needs to be taken into account.

Agreement

For adaptation of power offset values between PDSCH and CSI-RS, further study the following

- Where/how to configure multiple power offset values

  - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception

  - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS

  - Whether other UE report content can be included

Agreement

For spatial and power domain adaptation, solution(s) based on adaptation within an active BWP is considered as baseline

Agreement

Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that

- Whether there is a need for transition time per adaptation (for UE)

- Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.

R1-2301815	Discussion Summary #1 for enhancements on cell DTX/DRX mechanism	Moderator (Intel Corporation)

Agreement

- RAN1 continues discussion on the at least following physical layer related aspects of cell DTX/DRX aspects

  - physical layer signals/channels and procedures expected to be impacted during non-active periods of cell DTX/DRX

    - consider impact to at least KPIs from the SI when physical layers/signals/channels are impacted by cell DTX/DRX

- Further discussions on other aspects are not precluded

R1-2302131	Discussion Summary #2 for enhancements on cell DTX/DRX mechanism	Moderator (Intel Corporation)

Agreement

At least the following candidate signals/channels for connected mode UEs, which the UE may be expected to not transmit or receive during non-active periods of cell DTX/DRX, are considered from RAN1 perspective for further discussion. The exact set of signals/channels that the UE may be expected to not transmit or receive is FFS.

- DL

  - Periodic/Semi-persistent CSI-RS (including TRS)

  - PRS

  - PDCCH scrambled with UE specific RNTI

  - PDCCH in Type-3 CSS

  - SPS-PDSCH

- UL

  - SR

  - Periodic/Semi-persistent CSI report

  - Periodic/Semi-persistent SRS

  - CG-PUSCH

Other signals/channels are not precluded

2.1.2	Remaining Open issues

Remaining issues for techniques in spatial and power domains.

Remaining issues for enhancement on cell DTX/DRX mechanism.

2.2	RAN2

2.2.1	Agreements

##### RAN2#121

Cell DTX/DRX:

- There will be no impact to RACH, paging, and SIBs in idle/inactive for both gNB and Rel-18 and legacy UEs

- Rel-18 NES capable CONNECTED UE(s) can perform RACH and receive SIBs in non-active duration of cell DTX and/or DRX (i.e., same behavior for cell DTX and cell DRX).  No further enhancements for CBRA and CFRA will be pursued.

- Pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.   FFS whether we have DTX UE specific inactivity timer.  FFS on configuration signaling and stage 3.

- Confirm study item agreement that we can have separate DTX and DRX configuration.   We will focus on designing DTX/DRX for at least single configuration.  FFS whether multiple configuration of cell DTX or DRX will be supported.

Cell barring and camping for legacy and NES UEs:

- RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible

Connected mode mobility:

- Study whether CHO enhancements are needed for the purpose of turning off the cell

- Continue discussing CHO in the context of different NES techniques.

2.2.2	Remaining Open issues

Remaining issues for cell DTX/DRX (e.g. configuration, activation/deactivation, UE alignment).

Remaining issues for SSB-less and fast SCell activation (e.g. possible RAN2 impact).

Remaining issues for cell barring and cell selection/reselection.

Remaining issues for connected mode mobility.

2.3	RAN3

2.3.1	Agreements

##### RAN3#119

Inter-node beam activation

- Support beam level activation over Xn and F1.

2.3.2	Remaining Open issues

Remaining issues for Inter-node beam activation (e.g. the contained message, the exact IE design etc).

Remaining issues for Paging enhancements (e.g. starting from stationary UE? applicable for both inactive and idle RRC state, or inactive RRC state only?).

Potential issues for the Cell DTX/DRX.

2.4	RAN4

2.4.1	Agreements

2.4.2	Remaining Open issues

2.5	RAN5

2.5.1	Agreements

2.5.2	Remaining Open issues

2.5.3	Remaining Open issues with cross-WG dependencies

2.6	RAN6

2.6.1	Agreements

2.6.2	Remaining Open issues

3.	Detailed progress in SA/CT WGs since last TSG meeting (for all involved WGs)

3.1	SAx/CTs

3.1.1	Agreements with cross-TSG impacts

3.1.2	Remaining Open issues with cross-TSG impacts

4.	References

##### RAN1#112

R1-2300133	WI Work plan for R18 network energy savings	Rapporteur (Huawei)

R1-2301717	Addition of abbreviations and symbols to TR 38.864	Huawei

R1-2301964	FL summary#1 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

R1-2301965	FL summary#2 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

R1-2301966	FL summary#3 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

R1-2302179	Final FL summary for spatial and power domain techniques for R18 NES	Moderator (Huawei)

R1-2301815	Discussion Summary #1 for enhancements on cell DTX/DRX mechanism	Moderator (Intel Corporation)

R1-2302131	Discussion Summary #2 for enhancements on cell DTX/DRX mechanism	Moderator (Intel Corporation)

R1-2300405	Considerations on Network energy savings for NR	KT Corp.

R1-2300065	Spatial and Power Adaptations for Network Energy Savings	FUTUREWEI

R1-2300073	Power saving techniques in spatial and power domains	Huawei, HiSilicon

R1-2300143	Techniques in spatial and power domains	Nokia, Nokia Shanghai Bell

R1-2300230	Discussion on NES techniques in spatial and power domains	Spreadtrum Communications

R1-2300264	Discussion on techniques in spatial and power domains	OPPO

R1-2300360	Spatial and power domain adaptation for network energy saving	Panasonic

R1-2300372	Discussion on NES techniques in spatial and power domains	ZTE, Sanechips

R1-2300403	Network Energy Saving in Spatial and Power Domain	Google

R1-2300465	Discussions on NES techniques in spatial and power domain	vivo

R1-2300587	Discussion on techniques in spatial and power domains	Xiaomi

R1-2300692	Network Energy Saving techniques in spatial and power domain	CATT

R1-2300722	Discussion on NES techniques in spatial and power domains	China Telecom

R1-2300752	Discussion on NW energy saving techniques in spatial and power domains	Fujitsu

R1-2300768	Discussion on network energy saving techniques in spatial and power domains	NEC

R1-2300784	Discussion on techniques in spatial and power domains	InterDigital Communications

R1-2300960	Discussion on NWES techniques in spatial and power domain	Intel Corporation

R1-2301014	Discussion on network energy saving techniques in spatial and power domains	CMCC

R1-2301047	Network energy saving techniques in spatial domain	ETRI

R1-2301107	Discussion on NES techniques in spatial and power domains	LG Electronics

R1-2301163	Spatial Domain Adaptation for NES	Fraunhofer IIS, Fraunhofer HHI

R1-2301206	Network energy saving techniques in spatial and power domains	Lenovo

R1-2301222	Network energy savings techniques in spatial and power domains	AT&T

R1-2301276	Techniques in spatial and power domains	Samsung

R1-2301310	Discussion of NES techniques in spatial domain and power domain	Transsion Holdings

R1-2301319	Discussion on techniques in spatial and power domains	ITRI

R1-2301358	Discussion on spatial and power domain enhancements to support network energy saving	Apple

R1-2301425	Techniques in spatial and power domains	Qualcomm Incorporated

R1-2301505	Discussion on spatial and power domain enhancements for NW energy savings	NTT DOCOMO, INC.

R1-2301554	NW energy saving techniques in spatial and power domains	Ericsson

R1-2301599	On NW energy saving techniques in spatial and power domains	MediaTek Inc.

R1-2301698	Discussion on spatial and power adaptations for network energy saving	CEWiT, Reliance Jio

R1-2300066	Enhancements on cell DTXDRX for NES	FUTUREWEI

R1-2300074	Cell DTX/DRX mechanism for network energy saving	Huawei, HiSilicon

R1-2300144	Enhancements on cell DTX/DRX mechanism	Nokia, Nokia Shanghai Bell

R1-2300231	Discussion on enhancements on cell DTX/DRX mechanism	Spreadtrum Communications

R1-2300265	Discussion on enhancements on cell DTX/DRX mechanism	OPPO

R1-2300361	Cell DTX/DRX enhancement for network energy saving	Panasonic

R1-2300373	Discussion on cell DTX/DRX	ZTE, Sanechips

R1-2300404	Network Energy Saving on Cell DTX and DRX	Google

R1-2300466	Discussions on enhancements on cell DTX/DRX mechanism	vivo

R1-2300588	Discussions on cell DTX-DRX for network energy saving	xiaomi

R1-2300693	DTX/DRX for network Energy Saving	CATT

R1-2300723	Discussion on cell DTX/DRX mechanism	China Telecom

R1-2300753	Discussion on cell DTX/DRX mechanism	Fujitsu

R1-2300764	Cell DTX/DRX Configuration for Network Energy Saving	NEC

R1-2300785	Discussion on enhancements on cell DTX/DRX mechanism	InterDigital Communications

R1-2300961	Discussion on enhancements on cell DTX/DRX mechanism	Intel Corporation

R1-2301015	Discussion on cell DTX/DRX enhancements	CMCC

R1-2301048	Enhancements on cell DTX/DRX mechanism	ETRI

R1-2301108	Discussion on cell DTX/DRX mechanism	LG Electronics

R1-2301162	Discussion on cell DTX/DRX mechanism	Rakuten Mobile, Inc

R1-2301164	RAN1 Considerations for Cell DTX and DRX	Fraunhofer IIS, Fraunhofer HHI

R1-2301277	Enhancements on cell DTX/DRX mechanism	Samsung

R1-2301311	Discussion on Enhancement on cell DTX DRX mechanism	Transsion Holdings

R1-2301320	Discussion on potential enhancements on cell DTX/DRX mechanism for NR	ITRI

R1-2301359	Discussion on UE behavior for cell DRX/DTX	Apple

R1-2301426	Enhancements on cell DTX and DRX mechanism	Qualcomm Incorporated

R1-2301506	Discussion on enhancements on Cell DTX/DRX mechanism	NTT DOCOMO, INC.

R1-2301555	RAN1 aspects of cell DTX/DRX	Ericsson

R1-2301600	On NW energy saving enhancements for cell DTX/DRX mechanism	MediaTek Inc.

R1-2301699	Discussion on cell DTX/DRX mechanism for network energy saving	CEWiT, Reliance Jio

##### RAN2#121

R2-2300038	LS on network energy saving techniques (R3-226898; contact: Huawei)	RAN3

R2-2300228	Discussion on SSB-less SCell operation	Huawei, HiSilicon

R2-2300229	Discussion on CHO enhancement for NES	Huawei, HiSilicon

R2-2300230	Discussion on cell DTX/DRX	Huawei, HiSilicon

R2-2300231	Discussion on cell selection/reselection for NES	Huawei, HiSilicon

R2-2300247	Cell DTX and DRX support	NEC

R2-2300248	CHO procedure enhancement to support NES mode	NEC

R2-2300255	On remaining issues of SBB/SIB-less NES solutions	Dell Technologies

R2-2300279	Work plan for NR network energy savings	Huawei, HiSilicon

R2-2300377	Considerations on Cell selection/re-selection	KDDI Corporation

R2-2300378	Considerations on Cell DTX/DRX	KDDI Corporation

R2-2300444	Initial discussion on DTX-DRX mechanism	vivo

R2-2300445	RAN2 impact from inter-band SSB-less Scell operation	vivo

R2-2300446	Discussion on cell selection/re-selection	vivo

R2-2300447	Conditional handover enhancement for network energy saving	vivo

R2-2300456	Discussion on DTX DRX mechanism	OPPO

R2-2300457	Discussion on cell selection reselection	OPPO

R2-2300458	Discussion on connected mode mobility	OPPO

R2-2300491	Alignment to Cell DRX	Lenovo

R2-2300492	Alignment to Cell DTX	Lenovo

R2-2300539	Cell DTX-DRX Mechanism	Qualcomm Incorporated

R2-2300540	SSB-less Scell Operation	Qualcomm Incorporated

R2-2300541	NES Cell Selection Mechanism	Qualcomm Incorporated

R2-2300542	NES Connected mode mobility	Qualcomm Incorporated

R2-2300608	CHO procedure enhancement for NES	Intel Corporation

R2-2300609	Cell (re)selection for handling legacy UEs in NES	Intel Corporation

R2-2300610	SSB-less SCell operation for inter-band CA	Intel Corporation

R2-2300611	Considerations of Cell DTX and DRX	Intel Corporation

R2-2300632	Cell DTX/DRX mechanism	InterDigital

R2-2300633	Cell selection and resection for NES	InterDigital

R2-2300634	NES mobility aspects	InterDigital

R2-2300635	SSB-less Scell operation	InterDigital

R2-2300701	Discussion on Cell DTX / DRX	Apple

R2-2300702	Discussion on CHO enhancement in NES	Apple

R2-2300703	Mechanism of legacy UE barring in NES cell	Apple

R2-2300704	Discussion on RAN2 work of inter-band SSB-less CA	Apple

R2-2300819	Discussion on Cell DTX/DRX	CATT

R2-2300820	Enhancements on SCell activation procedures	CATT

R2-2300821	Consideration on Cell selection/re-selection on NES cells	CATT

R2-2300822	Consideration on CHO enhancement for NES	CATT

R2-2300871	Cell selection and reselection handling for legacy Ues	Nokia, Nokia Shanghai Bell

R2-2300872	CHO on NES	Nokia, Nokia Shanghai Bell

R2-2300893	CHO for NES	Ericsson

R2-2300940	Discussion on CHO enhancements for NES	Sharp

R2-2300977	Cell selection/re-selection in NES	Lenovo

R2-2300978	NES impact to UE mobility	Lenovo

R2-2301063	Preventing legacy UEs camping on NES cell	ZTE Corporation, Sanechips

R2-2301064	Discussion on cell DTX and DRX mechanism for NES	ZTE Corporation, Sanechips

R2-2301066	Discussion on CHO enhancements for NES	ZTE Corporation, Sanechips

R2-2301068	Discussion on SSB-less SCell operation for NES	ZTE Corporation, Sanechips

R2-2301088	Handover enhancement for NES	Sony

R2-2301230	Discussion on network DTX/DRX	CMCC

R2-2301231	Discussion on SSB-less SCell operation	CMCC

R2-2301232	Discussion on cell barring and reselection for NES	CMCC

R2-2301233	Discussion on CHO enhancements for NES	CMCC

R2-2301343	SSB-less Scell operation	Nokia, Nokia Shanghai Bell

R2-2301399	Further aspects on cell DTX/DRX	Ericsson

R2-2301400	SSB-less SCell operation on inter-band CA for FR1	Ericsson

R2-2301401	NES Cell selection/reselection	Ericsson

R2-2301463	RRC Inactive/Idle UE handling for NES	ETRI

R2-2301503	Discussion on Connected mode mobility for network energy savings	Fujitsu Limited

R2-2301515	Further details on Cell DTX/DRX	Nokia, Nokia Shanghai Bell

R2-2301521	Enhancements on SSB-less activation	NEC Telecom MODUS Ltd.

R2-2301522	Procedure for legacy UEs camping on NES cells	NEC Telecom MODUS Ltd.

R2-2301550	Discussion on DTX/DRX for NES	Samsung

R2-2301551	Discussion on SSB/SIB-less Solutions for NES	Samsung

R2-2301552	Discussion on Cell Selection and Reselection for NES	Samsung

R2-2301553	Discussion on Connected mode mobility for NES	Samsung

R2-2301616	Access restriction enhancement for NES	LG Electronics Inc.

R2-2301733	Discussion on DTX/DRX mechanism	LG Electronics Inc.

R2-2301768	Connected Mode Mobility	LG Electronics

R2-2301776	Discussion for Cell DTX/DRX	NTT DOCOMO, INC.

R2-2301777	Discussion for Cell selection	NTT DOCOMO, INC.

R2-2301854	Further discussion on Cell DTX/DRX	MediaTek Inc.

R2-2301857	Legacy UEs and NES UEs accessing to NES cells	BT plc, NTT DOCOMO, Turkcell

R2-2301873	Cell Selection and Re-Selection for NES	Fraunhofer IIS, Fraunhofer HHI

R2-2301882	Cell DTX and DRX	Fraunhofer IIS

##### RAN3#119

R3-230303	WI Work plan for R18 network energy savings	Huawei

R3-230304	Network energy saving techniques 	Huawei

R3-230302	TP for Network energy saving techniques 	Huawei

R3-230528	Initial discussion on cell DTX/DRX for network energy saving 	Nokia, Nokia Shanghai Bell

R3-230529	Inter-node beam activation and enhancements on restricting paging in a limited area 	Nokia, Nokia Shanghai Bell

R3-230232	(TP for NES WI for 38.473) Considerations on network energy savings 	Intel Corporation

R3-230233	Baseline CR to 38.423 on NES 	Intel Corporation

R3-230317	Disccussion on support of network energy saving	 CATT

R3-230318	TP for 38.423/38.473 on network energy saving 	CATT

R3-230349	Discussion on NES state 	Lenovo

R3-230374	Paging enhancements and Inter-node beam activation 	Qualcomm Incorporated

R3-230512	Introduction of Network Energy Saving 	Ericsson

R3-230520	Introduction of Network Energy Saving 	Ericsson

R3-230615 	Discussion on network energy saving 	Samsung

R3-230616	CR to 38.473 for paging enhancements on NES 	Samsung

R3-230663	Discussion on Cell DTX&DRX 	CMCC

R3-230779	(TPs to BL CRs of 38.423/38.473/38.300) Inter-node beam activation 	ZTE

R3-230780	(TP to BL CR of 38.473) Paging enhancement 	ZTE

01.02.2023		minor adaptations for RAN #99

27.10.2022		minor adaptations for RAN #98e

01.08.2022		minor adaptations for RAN #97e

21.05.2022		minor adaptations for RAN #96

10.01.2022		minor adaptations for RAN #95e

04.10.2021		minor adaptations for RAN #94e

08.08.2021		minor adaptations for RAN #93e

17.05.2021		minor adaptations for RAN #92e

28.01.2021		minor adaptations for RAN #91e

09.11.2020		minor adaptations for RAN #90e

31.08.2020		minor adaptations for RAN #89e

20.04.2020		minor adaptations for RAN #88e

18.02.2020		minor adaptations for RAN #87e

14.11.2019		minor adaptations for RAN #86

18.08.2019		minor adaptations for RAN #85

12.05.2019		minor adaptations for RAN #84

27.02.2019		minor adaptations for RAN #83

21.11.2018		completion levels with colours added (for RAN #82)

v04.81	31.07.2018		simplification of template and addition of cross-TSG aspects (for RAN #81)

v04.80	21.05.2018		minor adaptations for RAN #80

v04.79	26.02.2018		minor adaptations for RAN #79

v04.78	18.11.2017		minor adaptations for RAN #78

v04.77	06.08.2017		minor adaptations for RAN #77

v04.76	15.05.2017		minor adaptations for RAN #76

v04.75	31.01.2017		minor adaptations for RAN #75

v04.74	28.10.2016		minor adaptations for RAN #74

v04.73	01.09.2016		adaptations for RAN #73 (time units in extra Excel table, RAN6 reporting included)

v04.72	26.05.2016		adaptations for RAN #72 (introduction of NR & GERAN TUs)

v04.71	10.02.2016		minor adaptations for RAN #71

v04.70	30.10.2015		minor adaptations for RAN #70

v04.69	12.08.2015		minor adaptations for RAN #69

v04.68	21.05.2015		minor adaptations for RAN #68

v04.67	01.02.2015		minor adaptations for RAN #67

v04.66	16.11.2014		minor adaptations for RAN #66

v04.65	16.08.2014		minor adaptations for RAN #65

v04.64	22.05.2014		minor adaptations for RAN #64

v04.63	24.01.2014		restructuring for RAN #63 to cover Core & Perf. in one doc file

v03.62	11.11.2013		section 1.2.3 adapted for RAN #62

v03	11.08.2013		section 1.2.3 added on time budget

v02	07.05.2010		history added, some spelling corrections

v01	13.11.2009		First version of the template