3GPP TSG RAN Meeting #99						             RP-230777

Rotterdam, Netherlands, March 20-23, 2023

Title:	Proposal for PR1 in eRedCap

Agenda item:	9.3.1.7

Source:	Moderator (CMCC)

Document for:	Discussion

Proposal

- PR1 is supported by default by R18 eRedCap UE

- Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, e.g., 10Mbps

Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively.

Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz.

Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f   in order to have the same peak rate.

Note 4: No introducing specific mechanism to identify PR1 during initial access procedure

Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by either of the following:

- Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1, or

- Same as “Rel-17 RedCap” , e.g., by assuming that the network (capable of supporting PR1) can limit the peak data rate

Background

- Rule 2: Peak data rate as what is defined in LTE

  - UE type 1(Rule 2): Peak data rate defined for Rel-17 RedCap

    - Rel-17 RedCap UE: UE capable of 20MHz

  - UE type 2(Rule 2): Peak data rate define for Rel-18 eRedCap, e.g., 10Mbps

    - Rel-18 eRedCap: UE capable of 20MHz + PR1

    - Rel-18 eRedCap: UE capable of BW3/PR3 + PR1

Finally, it means PR1 should be supported by default by R18 eRedCap UE

Example for addressing transparency of PR1 to Network in initial access

whether Rel-18 PR1 is supported by the network can be indicated by network in SIBx, i,e.,

- if network only supports UE capable of 20MHz without upgrading to support Rel-18 PR1, no indication from the network, the UE capable of 20MHz + PR1 will be barred;

- if network supports UE capable of 20MHz + PR1 by upgrading to support Rel-18 PR1, the UE capable of 20MHz + PR1 can access on receiving the indication.

Note: whether Rel-18 PR1 is supported by the network can be indicated by network in SIBx, i,e.,

      - if network supports Rel-18 PR1, no indication from the network, the UE capable of BW3/PR3 + PR1 will be barred;

      - if network supports Rel-18 PR1 with indication, the UE capable of BW3/PR3 + PR1 can access on receiving the indication.

- Rule 1: Bandwidth capability

  - UE type 1(Rule 1): below two are of same type and share the same early indication

    - Rel-17 RedCap UE: UE capable of 20MHz

    - Rel-18 eRedCap UE: UE capable of 20MHz + PR1

  - UE type 2(Rule 1): below two are of same type and share the same early indication

    - Rel-18 eRedCap UE: UE capable of BW3/PR3

    - Rel-18 eRedCap UE: UE capable of BW3/PR3 + PR1

Finally, it means standalone PR1 is supported

- Rule 3: Unified implementation/feature combination (from cost/complexity reduction point of view)

    - UE type 1(Rule 3): UE capable of 20MHz

    - UE type 2(Rule 3): either BW3/PR3+PR1 or BW3/PR3

Finally, it means PR1 will not be supported or PR1 can’t be standalone