3GPP TSG RAN Rel-18 workshop	RWS-210155

Electronic Meeting, June 28 – July 2, 2021	(revision of RP-XXXXXX)

# Source:	China Telecom

# Title:	Draft WID on Further Enhancement of Private Network Support for NG-RAN

# Document for:	Information

Agenda Item:	4.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Work Item on Further Enhancement of Private Network Support for NG-RAN

## Acronym: FS_FeNPN_RAN

## Unique identifier: 	{A number to be provided by MCC at the plenary}

NOTE:	For new WIs/SIs leave the Unique identifier empty and make a proposal for an Acronym.

For a revised WI/SI: Take Unique identifier and acronym as shown in 3GPP workplan.

If this is a RAN WID including Core and Perf. part, then Title, Acronym and Unique identifier refer to the feature WI.

Please tick (X) the applicable box(es) in the table below:

Either:

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part |  |

or:

| This WID includes a Testing part |  |  |
| --- | --- | --- |
| and it addresses the following 3GPP work area: | Radio Access |  |
|  | Core Network |  |
|  | Services |  |

Potential target Release: Rel-18.

Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan. In case of contradiction with the target dates of clause 5, clause 5 determines the target release.

## 1	Impacts 	{ For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study.}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No | X |  |  |  | X |
| Don't know |  |  |  |  |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a … {Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN }

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

NOTE:	Normally, Core/Perf./Testing parts in RAN WIDs are Building Blocks. Only if they are under an SA or CT umbrella, they are defined as work tasks. If you are in doubt, please contact MCC.

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

NOTE:	RAN agreed some time ago, that it describes the feature WI + Core/Perf. part WI or Testing part WI in one 	WID. Therefore the table above should just include the feature WI data (In case the feature covers Core and 	Perf. part, please list under Working Group the leading WG of the Core part).

### 2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as preceding SI or a preceding WI (e.g. if further enhancing a feature).}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 830081 | Private Network Support for NG-RAN | Rel.16 RAN NPN Support WI |
| 840024 | Study on enhanced support of Non-Public Networks | The SI handles further architectural aspects of NPN, and SNPN/CAG support where inter-dependencies with RAN exist. |
| 890049 | Enhancement of Private Network Support for NG-RAN | The Rel-17 WI is to support enhanced non-public network (eNPN) for NG-RAN resulting from SA2 study on enhanced support of NPN. |

NOTE:	Also related or dependent WIs/SIs in other TSGs should be indicated.

Dependency on non-3GPP (draft) specification:

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency.}

## 3	Justification

Private Network support (PRN) for NG-RAN was introduced in Rel-16 to support the basic functionality of two types of NPN, i.e. Standalone NPN and Public Network Integrated NPN(PNI-NPN), then its functionality was enhanced in Rel-17 to enable wider cooperation between different networks/different entities such as providing authentication for the third party users and onboarding provisioning, as well as IMS voice and emergency service related use cases.

In rel-18, in order to better support the development of vertical industrial, the NPN function needs to be further enhanced. From SA aspects, according to TR 23.700-07, there are two key issues which were not included in the scope of Rel-17 SA eNPN WID may need be addressed in Rel-18, namely:

-	Support for equivalent SNPNs

-	Support of non-3GPP access for SNPN services

In addition, due to time constraints, there may be some unresolved issues left in Rel-17 eNPN, which need to be further addressed in Rel-18.

For example, a PDU session anchored at the SNPN cannot be maintained during mobility when the UE accesses the SNPN using credentials owned by a Credentials Holder separate from the original SNPN, which implies a lack of service continuity. Another example is that control plane provisioning enables 3GPP network to have control of the full onboarding process, without relying on mechanism outside 3GPP and making the deployment of onboarding function more efficient.

So, future investigations and studies may be required to complete the aspects that cannot be solved within the Rel-17 time frame.

Meanwhile, SA1 is progressing on new use cases and requirements which can be related to NPN e.g. in the study item 5G networks Providing Access to Localized Services (FS_PALS). Normative SA1 work may generate new stage 1 requirements on NPN. Once the SA1 work stabilizes, the SA2 work should be aligned with the outcome of the SA1 work (referring to requirements from e.g. TS 22.261), e.g. to enable NPN as the hosting network providing the localized services.

As discussion above, RAN could take the work of the possible SA2 FeNPN study item in Rel-18 and its corresponding RAN impact into consideration and specify the exact RAN functionality if any.

From RAN points of view, there are also several aspects which need further work in Rel-18 as follows:

- The NB-IoT/eMTC connected to 5GC has been supported. However, the NPN support for NB-IoT/eMTC is not. As many operators have deployed the NB-IoT networks and China Telecom has operated the largest NB-IoT network in the world, it is envisioned that the NPN for NB-IoT/eMTC can provide operators more comprehensive support of IoT use cases.

- So far, NG-RAN node is not aware of the UE serving (selected) CAG ID in case of initial access and handover. However, RAN awareness of the serving CAG ID usage is much beneficial for operator’s network optimization and showing the efficiency of PNI-NPN to the vertical and/or enterprise customers who are willing to pay for the private network. Also, the operators need to have full knowledge of CAG cell usage condition and determine whether any adjustment for CAG cell setting (such as the coverage, direction and location of cells) is needed.

## 4	Objective

### 4.1	Objective of Core part WI

The objective of this work item is to support possible further enhanced non-public network (FeNPN) for NG-RAN resulting from the SA2 potential study on further enhanced support of NPN as well as the leftover RAN functionality in Rel-17 ePRN WI.

1)  Check the RAN impact of SA2 study on further enhanced support of NPN, and specify the corresponding RAN functionality where necessary:

- Support for enhanced mobility when involving SNPN:

- Equivalent SNPNs.

- Service/Session continuity with PDU Session anchored at Credentials Holder's network.

- Support for non-3GPP access for SNPN;

- Support for enhanced onboarding functionality (It is TBD which of the following aspects to be added, based on the progress of the Rel-17 eNPN normative work):

- Control Plane based remote provisioning.

NOTE: Whether to include CP provisioning for SNPN in scope of the work is dependent on SA3.

- Standardized way to provide Onboarding SNPN with the information of provisioning server for Control Plane provisioning.

- Onboarding SNPN (ON-SNPN) to support multiple provisioning servers and subscription owner SNPN for selection when using CP provisioning.

- Enable more efficient SNPN support for voice services;

- Avoid the need for configuration of IMC in the UE e.g. credentials and IMPI/IMPU.

- Support for IMS services in scenarios with separate serving network and subscribed network roaming.

- Address new SA1 requirements (e.g. from PALS work) related to NPN.

2）Specify the RAN functionality of the following Further enhanced NPN in NG-RAN:

- The support of SNPN and PNI-NPN for eMTC/NB-IoT connected to 5GC;

- For PNI-NPN, support RAN awareness of serving (selected) CAG ID usage including initial access and handover, if justified;

### 4.2	Objective of Performance part WI

NOTE:	Leave empty if the WI proposal does not contain a RAN performance part.

### 4.3	RAN time budget request (not applicable to RAN5 WIs/SIs)

NOTE:	For all new RAN related WIs/SIs which are not led by RAN WG5 the WI/SI rapporteur has to fill out the attached Excel table to request time budgets for corresponding RAN WG meetings.
The Excel table has to be filled out for all affected RAN WGs and up to the target date of the WI/SI.
One time unit (TU) corresponds to ~ 2 hours in the meeting.
If no TU is needed, then leave the field empty otherwise enter a number >0 in the field.

For revisions of already approved WI/SI descriptions: Please remove the Excel table from the WID/SID's zip file. The time budgets are already recorded. If you want to modify them, then this has to be done via the status report and not via a revised WID/SID.

If this WID is covering Core and Performance part, then please fill out one line for each part in the attached Excel table.

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |

{Note 1: Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
By default a new specs can only be new for one of both parts.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.300 | To support further enhanced non-public network (eNPN) for NG-RAN | XX | Core part |
| 38.304 | To support further enhanced non-public network (eNPN) for NG-RAN | XX | Core part |
| 38.331 | To support further enhanced non-public network (eNPN) for NG-RAN | XX | Core part |
| 38.401 | To support further enhanced non-public network (eNPN) for NG-RAN | XX | Core part |
| 38.410 | To support further enhanced non-public network (eNPN) for NG-RAN | XX | Core part |
| 38.413 | To support further enhanced non-public network (eNPN) for NG-RAN | XX | Core part |
| 38.423 | To support further enhanced non-public network (eNPN) for NG-RAN | XX | Core part |
| 38.463 | To support further enhanced non-public network (eNPN) for NG-RAN | XX | Core part |
| 38.473 | To support further enhanced non-public network (eNPN) for NG-RAN | XX | Core part |

NOTE:	If this is a RAN WI including Core and Perf. part, then all new Core part specs have to be listed first and then all new Perf. part specs. Indicate "Core part" or "Perf. part" under Remarks for each spec.
If an existing spec is affected by both (Core part and Perf. part), then it has to be listed twice with appropriate approval dates.

## 6	Work item Rapporteur(s)

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>.}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s).}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. Secondary Rapporteur(s) are possible for specific secondary task(s)}.

## 7	Work item leadership

Primary: RAN3

Secondary: RAN2

## 8	Aspects that involve other WGs

NOTE:	For RAN WIs: Section 8 applies only toWGs outside of TSG RAN because RAN WG aspects have to be covered in section 4.

## 9	Supporting Individual Members

Supporting IM name

China Telecom