3GPP TSG RAN Rel-18 workshop	RWS-210314

Electronic Meeting, June 28 – July 2, 2021

# Source:	Ericsson

# Title:	New WID on enhanced support of reduced capability NR devices

# Document for:	Discussion

Agenda Item:	4.2

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

# Title: 	Enhanced support of reduced capability NR devices

## Acronym: NR_eredcap

## Unique identifier: 	 xxxxxx

| This WID includes a Core part | X |
| --- | --- |
| This WID includes a Performance part | X |

Potential target Release: Rel-18

## 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X |  |  |
| No | X |  |  |  | X |
| Don't know |  |  |  | X |  |

## 2	Classification of the Work Item and linked work items

### 2.1	Primary classification

This work item is a

| X | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
|  | Study Item |

### 2.2	Parent Work Item

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |

### 2.3	Other related Work Items and dependencies

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860035 | Study on support of reduced capability NR devices |  |
| 860036 | Study on NR coverage enhancements |  |
| 860047 | UE power saving enhancements for NR |  |
| 900061 | NR coverage enhancements |  |
| 900062 | Support of reduced capability NR devices |  |

## 3	Justification

5G aims to accelerate industrial transformation and digitalization, which improve flexibility, enhance productivity and efficiency, reduce maintenance cost, and improve operational safety. Industrial sensors play an important role for realizing such a vision. Not only widely used in industrial automation and digitalization use cases, industrial sensors are also widely used in the general environmental monitoring use cases such as monitoring of critical infrastructure (e.g., buildings, bridges, water dams, etc.) or monitoring for natural disasters (e.g., wild fire, flood, tsunami, earthquake, etc.).

Another emerging new class of new 5G use cases is the smart city vertical, which covers data collection and processing to more efficiently monitor and control city resources, and to provide services to city residents. Especially, the deployment of surveillance cameras is an essential part of the smart city but also of factories and industries.

Furthermore, there have been increasing interests in wearables use cases such as smart watches, eHealth related devices, and medical monitoring devices. These use cases call for different design considerations and have different requirements in terms of form factor, UE complexity and energy efficiency, compared to eMBB devices.

The support of industrial sensors, video surveillance, and wearables were the motivations behind Rel-17 RedCap. Through the Rel-17 NR RedCap work item, 3GPP has established a framework for enabling reduced capability NR devices suitable for a range of use cases, including the industrial sensors, video surveillance, and wearables use cases mentioned above, with requirements on low UE complexity and sometimes also on low UE power consumption.

Now when the foundation has been laid in Rel-17, some enhancements can be considered to enable as efficient support as possible for the mentioned use cases and to widen the range of use cases that can be addressed without fundamental changes to the basic RedCap UE type defined in Rel-17.

Many industrial sensors use cases require a deployment of a massive number of sensors. Replacing the battery of each of these sensors might be prohibitively difficult or undesirable. In certain use cases, it might be difficult to access or even exactly locate the sensors after they have been deployed. Thus, for these use cases, a key enabler is to allow the sensors to sustain decades of operation without ever needing battery replacement. Furthermore, many of the sensor use cases operate in environments where it is possible to harvest ambient energy for operation. The harvested ambient energy may be, for example, vibrational energy, photovoltaic energy, thermal-electric generated energy.

Some of these considerations are also applicable to video surveillance and medical wearable use cases. For example, a video surveillance camera deployed outdoors may harvest solar energy. A medical wearable device may be able to harvest energy through vibration and it may be desirable that the patients do not need to replace battery themselves (i.e., battery lasts between office visits).

Positioning function is also expected to support the applications in some scenarios for industrial sensors, video surveillance and wearables, for example in asset tracking, logistics and warehouse, factories and construction site, safety, and security management, and so on.

To further expand the RedCap use cases, the following enhancements can be considered:

- Improved UE energy efficiency: An improved UE energy efficiency will help improve battery lives and may even enable battery-less operation in some cases, which may be a highly attractive property, e.g., for many sensor applications.

  - Enable energy efficient UE wake-up radio: A very energy efficient UE wake-up radio may be highly attractive for use cases where a very long battery life is important, but the downlink reachability requirement is relaxed. A wake-up signal for waking up a low-power wake-up radio at least in RRC_IDLE would need to be specified to allow an extremely low power consumption in sleep mode, and possibly some extensions to the paging protocol to fully realize the potential gains.

  - Enable sensors to operate on harvested energy: In scenarios where the surrounding environment provides an opportunity to harvest energy, e.g. from vibrations, it may be possible to enable battery-less operation, e.g. for use cases where battery-driven operation is not feasible. To facilitate this, definition of one or more lower UE power classes can be considered, and possibly some optimizations of the RRC protocol to consider potential variations of the amount of harvested energy and traffic.

- Improved support for mobility and for positioning: In the first RedCap release (Rel-17), the focus is on basic functionality. There are many RedCap use cases that could benefit from full standard support including RAN4 requirements for inter-RAT mobility as well as for positioning support.

  - Inter-RAT mobility to/from LTE: Many RedCap devices can be expected to also support LTE. Specifying full standard support including RAN4 requirements for inter-RAT mobility in Rel-18 (unless it is specified already as part of Rel-17) helps enabling efficient transition without interruption between NR and LTE for RedCap devices.

  - Positioning support: For many applications, accurate positioning is beneficial or even critical. The RAN4 requirements for existing NR positioning methods (NR E-CID, NR DL-TDOA, NR DL-AoD, NR Multi-RTT) should be updated to be applicable to RedCap UEs with a reduced number of Rx branches.

These enhancements can be introduced while maintaining the integrity of RedCap ecosystem and maximize the benefit of economies of scale. These enhancements can be introduced by having new firmware/software running on the same baseband platform as Rel-17 RedCap or enabling a companion hardware (e.g., a WUR) working with the same hardware platform as Rel-17 RedCap.

This WI aims to specify enhancements applicable to the RedCap UE type and framework defined in Rel-17.

## 4	Objective

### 4.1	Objective of SI or Core part WI or Testing part WI

The objective is to specify the following set of improvements for RedCap UEs:

- Specify support for the following to support devices operating on harvested energy:

  - Lower UE power class(es) [RAN4, RAN2]

  - L2/L3 protocol optimizations to consider potential variations of amount of harvested energy and traffic [RAN2]

- Specify support for the following to enable energy efficient UE wake-up radio:

  - Wake-up signal for low-power wake-up radio at least in RRC_IDLE [RAN1, RAN2, RAN4]

  - Related extensions to the paging protocol [RAN2, RAN3]

- Specify support for the following mobility enhancement [RAN4, RAN2]

  - Inter-RAT mobility to/from LTE

- Specify support for the following positioning methods [RAN4, RAN2, RAN3]

  - NR E-CID

  - NR DL-TDOA

  - NR DL-AoD

  - NR Multi-RTT

Notes:

- Rel-15 SSB bandwidth and the NR CP-OFDM waveform are reused.

- The work defined as part of this WI is not to overlap with LPWA use cases.

- How the devices harvest and store energy is outside the scope of this work item.

- No new mechanisms for coverage compensation for UEs of lower power class(es) are specified.

- Coexistence with non-RedCap UEs and Rel-17 RedCap UEs is to be ensured.

- This WI focuses on SA mode and single connectivity with operation in a single band at a time.

- This WI considers all frequency ranges and all applicable duplex modes unless otherwise specified.

### 4.2	Objective of Performance part WI

Specify necessary performance requirements, measurement accuracy requirements and test cases related to the above-mentioned enhancements and core requirements [RAN4].

### 4.3	RAN time budget request

additional comments to the time budget request in the attached Excel table:

## 5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Title | For info at TSG# | For approval at TSG# | Remarks |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.202 | NR; Services provided by the physical layer | RAN#X | Core part |
| 38.211 | NR; Physical channels and modulation | RAN#X | Core part |
| 38.212 | NR; Multiplexing and channel coding | RAN#X | Core part |
| 38.213 | NR; Physical layer procedures for control | RAN#X | Core part |
| 38.214 | NR; Physical layer procedures for data | RAN#X | Core part |
| 38.300 | NR; NR and NG-RAN Overall description; Stage-2 | RAN#X+1 | Core part |
| 38.304 | NR; User Equipment (UE) procedures in idle mode and in RRC Inactive state | RAN#X+1 | Core part |
| 38.306 | NR; User Equipment (UE) radio access capabilities | RAN#X+1 | Core part |
| 38.321 | NR; Medium Access Control (MAC) protocol specification | RAN#X+1 | Core part |
| 38.331 | NR; Radio Resource Control (RRC) protocol specification | RAN#X+1 | Core part |
| 38.413 | NG-RAN; NG Application Protocol (NGAP) | RAN#X+1 | Core part |
| 38.455 | NR Positioning Protocol A (NRPPa) | RAN#X+1 | Core part |
| 37.355 | LTE Positioning Protocol (LPP) | RAN#X+1 | Core part |
| 38.101-1 | NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone | RAN#X+1 | Core part |
| 38.101-2 | NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone | RAN#X+1 | Core part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#X+1 | Core part |
| 36.133 | Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management | RAN#X+1 | Core part |
| 38.101-4 | NR; User Equipment (UE) radio transmission and reception; Part 4: Performance requirements | RAN#X+3 | Perf. part |
| 38.104 | NR; Base Station (BS) radio transmission and reception | RAN#X+3 | Perf. part |
| 38.133 | NR; Requirements for support of radio resource management | RAN#X+3 | Perf. part |
| 36.133 | Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management | RAN#X+3 | Perf. part |
| 38.141-1 | NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing | RAN#X+3 | Perf. part |
| 38.141-2 | NR; Base Station (BS) conformance testing Part 2: Radiated conformance testing | RAN#X+3 | Perf. part |

## 6	Work item Rapporteur(s)

Johan Bergman, Ericsson, johan.bergman@ericsson.com

## 7	Work item leadership

Primary:	RAN WG1

Secondary:  RAN WG2, RAN WG4, RAN WG3

## 8	Aspects that involve other WGs

## 9	Supporting Individual Members

Supporting IM name