3GPP TSG RAN WG1 #100-E		       R1- 2000430

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.4.2.2

Source:	TCL Communication

Title:	Resource allocation for NR sidelink - Mode 2

Document for:	Discussion and decision

# Introduction

The objective of the Work Item [1] is to specify radio solutions that are necessary for NR to support sidelink and enable several aspects of advanced V2X services. This WI is based on the results of the SI on NR V2X [2].

## Rel.16 WID scope for resource allocation

The objective of the Release 16 work item [1] was to specify radio solutions that are necessary for NR to support advanced V2X services (except the remote driving use case which was studied in TR 38.824) and in particular, for the NR sidelink:

Specify NR sidelink solutions necessary to support sidelink unicast, sidelink groupcast, and sidelink broadcast for V2X services, considering in-network coverage, out-of-network coverage, and partial network coverage.

- Resource allocation [RAN1, RAN2]

  - Mode 1

    - NR sidelink scheduling by NR Uu and LTE Uu as per the study outcome

  - Mode 2

    - Sensing and resource selection procedures based on sidelink pre-configuration and configuration by NR Uu and LTE Uu as per the study outcome

  - Support for simultaneous configuration of Mode 1 and Mode 2 for a UE

    - Transmitter UE operation in this configuration is to be discussed after the design of mode 1 only and mode 2 only.

    - Receiver UE can receive the transmissions without knowing the resource allocation mode used by the transmitter UE.

  - UE relaying resource pool configuration or resource configuration is not supported in this work in Rel-16.

Several aspects of the Mode 2 resource allocation issues are listed in the TR 38.885 v2.0.0 [2]

## V2X Task list

The RAN#86 plenary meeting noted a list of open issues in the document “Task list for 5G V2X in RAN1#100”[3] of remaining open issues to be considered in this (e-)meeting. Although not endorsed nor agreed as a whole, several aspects are necessary to be discussed.

Regarding the topic of Resource Allocation Mode 2, the following topics have been identified:

| Category |  | Remaining issue in RAN1 | RAN2 impact |
| --- | --- | --- | --- |
| Mode 2 Resource Allocation | C1 | Details of T1, T2, T3 in the sensing and resource (re-)selection window | No |
|  | C2 | Details on resource selection from the identified candidate resources within the selection window for blind and HARQ feedback-based (re)transmission | No |
|  | C3 | Remaining details of pre-emption | Depends on the outcome of RAN1 discussion |
|  | C4 | Remaining details of re-evaluation of Step 1 and Step 2 | No |

# Remaining details of pre-emption (C3)

Pre-emption in Mode 2 was agreed in RAN1 #98b but several aspects remain not detailed. This also corresponds to the “C3” issue of the task-list document [3].

Agreements: [RAN1 #98b]

- Support a resource pre-emption mechanism for Mode-2

  - A UE triggers reselection of already signaled resource(s) as a resource reservation in case of overlap with resource(s) of a higher priority reservation from a different UE and, SL-RSRP measurement associated with the resource reserved by that different UE is larger than an associated SL-RSRP threshold

    - Only the overlapped resource(s) is/are reselected

    - FFS

      - the timeline for reselection

      - other details

    - FFS whether or not to support other potential UE behaviour (e.g, power boosting/reduction)

  - This mechanism can be enabled or disabled, per resource pool

    - FFS details

For sidelink transmissions, if a sidelink Tx has reserved a resource in future, and later it detects another reservation of higher priority overlapping with its reserved resource, according to the current agreement, the former Tx with lower priority transmission will not transmit over the overlapping resource. This non-transmission, or pre-emption event, is conditioned upon the estimated signal power of the higher priority reservation being larger than a power threshold. Precisely, SL-RSRP measurement associated with the resource reserved should be larger than the associated SL-RSRP threshold.

## Transmit power reduction

Dropping the overlapping yet reserved and signaled transmission disrupts the system operation. The UE dropping its overlapping transmission needs to do the reselection again. On one hand, more retransmissions may be required to have the successful decoding of such packets. On the other hand, such events result in exceeding the packet delay budget, eventually resulting in reduction of QoS.

We believe that a better system operation can be achieved by having the lower priority UE transmit over its signaled resource with reduced power, even if it overlaps with a reservation made for a high priority transmission. The power reduction for the lower priority UE can be computed based upon the RSRP threshold, the detected power of the higher priority UE and the nominal power with which this UE was intending to transmit had it detected any overlap.

Proposal 1: Support a reduced transmit power transmission for the user that detects a situation of overlap of an already reserved resource by a reservation from a higher priority transmission.

FFS on how to precisely compute the power reduction factor.

## Self-preemption

When receiving a new data packet to be scheduled with a higher priority or with a stringent deadline, a user that already transmitted a reservation signal can decide to change the reservation and repurpose the reserved resource to transmit another TB.

The self-pre-emption can benefit several use cases, such as:

- Internal queueing and priority management towards a given user

- Priority management and pre-emption towards another user

- Repurposing reserved resource for a transmission that has been successfully received

The self-preemption can be considered to be limited to the same resource as reserved, or using resource that overlaps the already reserved ones. Already reserved resources do not need a whole (re)selection mechanism, while non-reserved resource require to fulfil (re)selection procedures.

Proposal 2: A user that already transmitted a reservation signal can reuse that reserved resource for different TB than the one originally planned. The transmitted TB can be destined to the same user as originally intended or another user.

The change of transmission compared to the original transmission is transparent to unrelated users and by properly adapting the fields of the SCIs that will be decoded by all nearby users anyway, no confusion shall happen. SCI fields that may need updates for self pre-emption may include: destination ID, HARQ ID, NDI, Priority, MCS, …

Self pre-emption of reserved resources without a dedicated full resource (re)selection procedure shall only be done for transmissions with identical or higher priority level, so that the priority of the actual transmission is not lowered and be perceived as not fairly reserved by other users.

## Configurability

To avoid user to constantly monitor resource reservation after their own reservation is sent out, pre-emption mechanisms can also be disabled or enabled depending on how busy the channel is. The rationale being that a low usage of resource will likely not lead to a pre-emption need from other users and thus there is no need to overhear resource reservations to save energy.

Proposal 3: Pre-emption mechanisms can be enabled or disabled per resource pool. It can also be (de)activated based on ongoing traffic and CBR.

Note that although the preemption can be enabled or disabled per resource pool, it is also possible that pre-emption mechanisms are supported or not by users and the UE feature capability can be exchanged. Users making pre-emption decisions should consider the pre-empted UE capability to do so.

Proposal 4: When enabled in a resource pool, how a user react to pre-emption can vary depending on UE feature capabilities and these are exchanged between users. User should take other users’ capability into the decision process of pre-empting resources.

# Details on the re-evaluation of step 1 and step 2 (C4)

## SL-RSRP thresholds to trigger reselection

The SL-RSRP thresholds to be considered a pre-emption (i.e. to trigger a reselection) of already reserved resource shall be tightly related to the thresholds used for resource selection, to maintain consistency over the network.

When a user performs the resource selection, it shall compare already received reservation SL-RSRP with a threshold depending on both transmissions’ priority. Thus, if that resource is considered available, the priority is high enough (and/or the RSRP is low enough) that it is considered beneficial to use it that way.

Assuming that the users and channels are not significantly changing between the two reservation, SL-RSRP should be similar at both TX users, and the user that performed the original reservation should understand, through comparing with the threshold, that its transmission priority is lower or the RSRP too low.

The cases where this can change are: the channel significantly changed, transmit power are different between users, one of the users has low resource availability (i.e. applied the +3dB threshold increase until reaching the 20pct resource available)

Proposal 5: Reuse the SL-RSRP threshold values considered for resource selection in Step-1 as baseline. The thresholds can be adapted to take into account a transmit power variation between the reservation transmission and subsequent transmissions.

# Other Release 16 leftovers

It is planed that Release 17 scope would be reconsidered and potentially include some more left-overs. Here is a selection of left-over we think are important to be considered for Sidelink.

## Location-based resource pool configuration

RAN1#98b meeting did not reach a consensus on the question of location-based resource pool configuration.

Proposals:

- RAN1 to conclude on one of the following options with respect to location-based Mode-2 resource allocation

  - Option 1: Support LTE Rel.14 mechanism for location-based Mode-2 resource allocation, i.e. resource pool may be autonomously selected using TX UE coordinates

  - Option 2: Do not introduce mechanisms for location-based Mode-2 resource allocation

After several discussions, no consensus emerged to support or not location-based resource pool configuration. We support Option 1, i.e. the use of a location indication for resource selection, at least in Mode 2, and in order to reduce standardization efforts and possibly allow some inter-operability with LTE, we recommended to use Zone ID in a manner close to LTE Rel.14 but with few improvements. Although for the sake of completion of Rel.16, we are understanding that this could be pushed in the Release 17 left-overs.

It is understood that excessive location-based resource sharing can lead to performance degradation in saturation cases, and zoning-based resource pool configurations do not prevent a network to configure a resource pool for all zones (or all zones in a given area). However, simulation results provided by other companies are focusing on scenarios where the space is split with different resource pool, thus the resource for a give usage more fragmented.

On another approach, we would rather see a resource pool covering a given area with specific usage (e.g. highway, urban, stadium, hotspots etc.) and its specific physical layer configuration (e.g. based on traffic/density/QoS expectations, users’ mobility, co-existence between modes/RATs, etc.). Such area can possibly be defined using several zones or zone patterns, not necessarily a single zone, so that all users of a common area of interest can easily communicate with each other and avoid splitting the resource availability.

Proposal 6: Further study mechanisms for location-based Mode-2 resource allocation, including zone granularity, definition of zone patterns and resource pool configured for several zones.

## Handling of reserved but unused resources

In the case of feedback-based or blind retransmissions, the issue of wasted resource versus sensing efficiency is raised. To cope with the potential waste of resource, several approaches are possible, that do not require a new signalling.

In a first approach, one can optimize the reservation scheme. Taking advantage of the statistical gains, we propose that the resources reserved for retransmissions can, in some cases, be also reserved by other users. Retransmissions are not events that are likely to happen, especially in the case of several retransmissions. Thus, performing an "overbooking" of resource for retransmissions is possible without significant system performance loss.

A resource is considered a candidate resource if the received SL-RSRP of a reservation SCI is below a certain threshold. This way allows the resource selection procedure to overbook resources based, at least, on both transmissions' priority to avoid blocking from distant enough transmissions during the sensing procedure.

The determination of the status of candidate can further be extended to include the retransmission index when several retransmissions with HARQ-based feedback are reserved, to cope with the open issue of resource waste when the transmission was successful. Upon decoding the SCI of reservation, a user can determine the retransmissions planned, and associate a different SL-RSRP threshold for each reserved resource. The rationale being that when a transmission reserves several retransmissions, they will likely not be used and thus, from a system point of view, it can be beneficial to reuse these potentially unused resources.

Proposal 7: In the case of multiple retransmission booking, the SL-RSRP thresholds used to consider a resource as candidate are also defined based on the retransmission index and the type of retransmission.

In a second approach, one can optimize the reservation release. Resource allocations and reservations are broadcasted and thus users can also listen to the feedback of other users to update their reservation table once they receive confirmation that a TB was well received, and thus the resources reserved for potential retransmissions will not be used.

Proposal 8: NR V2X Mode-2 HARQ-feedback based retransmissions supports, at least in some cases, monitoring the feedback of other users.

Proposal 9: NR V2X Mode-2 HARQ-feedback based retransmissions supports that retransmissions resources that are not used due to successful reception are released and available for other users.

# Conclusions

The following proposals have been made in this document.

Proposal 1: When a sidelink Tx receives a reservation of higher priority transmission over the resource overlapping with its signaled reserved resources, and SL-RSRP measurement associated with the resource reserved by that different UE is larger than an associated SL-RSRP threshold, the sidelink Tx with lower priority may transmit over the overlapping resource with reduced power. FFS on how to precisely compute the power reduction factor.

Proposal 2: A user that already transmitted a reservation signal can reuse that reserved resource for another TB than the one originally planned. The transmitted TB can be destined to the same user as originally intended or another user.

Proposal 3: Pre-emption mechanisms can be enabled or disabled per resource pool. It can also be (de)activated based on ongoing traffic and CBR.

Proposal 4: When enabled in a resource pool, how a user react to pre-emption can vary depending on UE feature capabilities and these are exchanged between users. User should take other users’ capability into the decision process of pre-empting resources.

Proposal 5: Reuse the SL-RSRP threshold values considered for resource selection in Step-1 as baseline. The thresholds can be adapted to take into account a transmit power variation between the reservation transmission and subsequent transmissions.

Proposal 6: Further study mechanisms for location-based Mode-2 resource allocation, including zone granularity, definition of zone patterns and resource pool configured for several zones.

Proposal 7: In the case of multiple retransmission booking, the SL-RSRP thresholds used to consider a resource as candidate are also defined based on the retransmission index and the type of retransmission.

Proposal 8: NR V2X Mode-2 HARQ-feedback based retransmissions supports, at least in some cases, monitoring the feedback of other users.

Proposal 9: NR V2X Mode-2 HARQ-feedback based retransmissions supports that retransmissions resources that are not used due to successful reception are released and available for other users.

# References

- RP-191723 “Revised WID on 5G V2X with NR sidelink”; Sep. 2019

- Study on NR Vehicle-to-Everything (V2X)," 3GPP TR 38.885 V2.0.0, Mar. 2019.

- RP-193198 “Task list for 5G V2X in RAN1#100”; Dec. 2019