3GPP TSG RAN WG1 Meeting #100                                      R1-2000534

e-Meeting, February 24th – March 6th, 2020

Source:	CATT

Title:	Remaining issues on CSI enhancement for MU-MIMO

Agenda Item:	7.2.6.1

Document for:	Discussion and Decision

- Introduction

In RAN1 #99 meeting, majority of issues on CSI enhancement for MU-MIMO have been discussed and corresponding agreements were reached. But there are issues with TS 38.214. In this contribution, some test proposals are given to solve those issues.

- TP for Enhanced Type II Codebook

For Rel-16 enhanced Type II codebook, Cx,y is used to indicate the FD basis selected by UE and defined in Table 5.2.2.2.5-4. While Cx,y defined in 5.2.2.2.3 is used to indicate the SD basis selected by UE. The range of Cx,y defined in 5.2.2.2.3 is smaller that of Cx,y in Table 5.2.2.2.5-4. Cx,y defined in 5.2.2.2.3 cannot be applied to the algorithm for calculating selected FD basis index. Hence, Cx,y defined in 5.2.2.2.3should be replaced by Cx,y defined in Table 5.2.2.2.5-4 for calculating selected FD basis index, and the following test proposal is proposed.

-------------------------------------------------- Start of text proposal -----------------------------------------------------------

- 5.2.2.2.5	Enhanced Type II Codebook

*** Unchanged text is omitted ***

For all values of N3, n3,l(0)=0 for l=1,…,υ. The nonzero elements of n3,l, identified by  n3,l(1), …, n3,lMυ-1, are found from i1,6,l (l=1,…,υ), for N3≤19, and from i1,6,l (l=1,…,υ) and Minitial, for N3>19, using  Cx,y as defined in  Table 5.2.2.2.5-4 and the algorithm:

s0=0

for f=1,…,Mυ-1

Find the largest x*∈Mυ-1-f,…,N3-1-f in Table 5.2.2.2.5-4 such that

i1,6,l -sf-1≥Cx*,Mυ-f

ef=Cx*,Mυ-f

sf=sf-1+ef

if N3≤19

n3,l(f)=N3-1-x*

else

nl(f)=2Mυ-1-x*

if nl(f)≤Minitial+2Mυ-1

n3,l(f)=nl(f)

else

n3,l(f)=nl(f)+(N3-2Mυ)

end if

end if

-------------------------------------------------End of text proposal ------------------------------------------------------

Proposal 1: Cx,y defined in Table 5.2.2.2.5-4 should be applied to the algorithm for calculating selected FD basis index.

- TP for Enhanced Type II Port Selection Codebook

In clause 5.2.2.2.5 of TS 38.214, yt,l are given to define the selected FD basis of the l-th layer. For the enhanced Type II Port Selection Codebook, the selected FD bases of different layers are different. If yt is adapted to indicate FD basis, it could not be used to denote the selected FD bases for different layers. In order to overcome this issue, yt should be replaced by yt,l defined as in clause 5.2.2.2.5 of TS 38.214 as well for enhanced Type II port selection codebook. The following test proposal is provided.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

- 5.2.2.2.6	 Enhanced Type II Port Selection Codebook

*** Unchanged text is omitted ***

The codebooks for 1-4 layers are given in Table 5.2.2.2.6-2, where vm is a PCSI-RS/2 -element column vector containing a value of 1 in element m mod PCSI-RS/2  and zeros elsewhere (where the first element is element 0), and the quantities φl,i,f and yt  yt,l are defined as in clause 5.2.2.2.5.

-------------------------------------------------End of text proposal ------------------------------------------------------

Proposal 2: yt,l should be used instead of yt in clause 5.2.2.2.6.

- TP for CSI reporting using PUSCH

In Table 5.2.3-1, priority reporting levels for Part 2 CSI of Rel-15 and Rel16 are described. There is no additional table, such as Table 5.2.3-2 to describe priority reporting levels for Part 2 CSI. Therefore, the following test proposal is given.

-------------------------------------------------- Start of text proposal ------------------------------------------------------

- 5.2.3	CSI reporting using PUSCH

*** Unchanged text is omitted ***

When CSI reporting on PUSCH comprises two parts, the UE may omit a portion of the Part 2 CSI. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where  is the number of CSI reports configured to be carried on the PUSCH. Priority 0 is the highest priority and priority  is the lowest priority and the CSI report n corresponds to the CSI report with the nth smallest Prii,CSI(y,k,c,s) value among the  CSI reports as defined in Clause 5.2.5. The subbands for a given CSI report n indicated by the higher layer parameter csi-ReportingBand are numbered continuously in increasing order with the lowest subband of csi-ReportingBand as subband 0. When omitting Part 2 CSI information for a particular priority level, the UE shall omit all of the information at that priority level.

-	For Enhanced Type II reports, for a given CSI report n, each reported element of indices  i2,4,l i2,5,l and i1,7,l, indexed by l,i and f, is associated with a priority value Pril,i,f=2⋅L⋅υ⋅πf+υ⋅i+l, with π(f)=min⁡(2⋅n3,lf,2⋅N3-n3,l(f)-1) with l=1,2,…,υ, i=0,1,…,2L-1, and f=0,1,…,Mυ-1, and where n3,l(f) is defined in Clause 5.2.2.2.5. The element with the highest priority has the lowest associated value Pril,i,f. Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1, where

-	Group 0 includes indices i1,1, i1,2 and i1,8,l (l=1,…,υ).

-	Group 1 includes indices i1,5 (if reported), i1,6,l, the υ2LMυ-KNZ/2 highest priority elements of i1,7,l,  i2,3,l, the  KNZ/2-υ highest priority elements of i2,4,l and the  KNZ/2-υ highest priority elements of i2,5,l (l=1,…,υ).

-	Group 2 includes the KNZ/2 lowest priority elements of i1,7,l, the KNZ/2 lowest priority elements of i2,4,l and the KNZ/2 lowest priority elements of i2,5,l (l=1,…,υ).

Table 5.2.3-1: Priority reporting levels for Part 2 CSI

Priority 0: For CSI reports 1 to NRep, Group 0 CSI for CSI reports configured as ‘typeII-r16’ or ‘typeII-PortSelection-r16’; Part 2 wideband CSI for CSI reports configured otherwise

Priority 1: Group 1 CSI for CSI report 1, if configured as ‘typeII-r16’ or ‘typeII-PortSelection-r16’; Part 2 subband CSI of even subbands for CSI report 1, if configured otherwise

Priority 2: Group 2 CSI for CSI report 1, if configured as ‘typeII-r16’ or ‘typeII-PortSelection-r16’; Part 2 subband CSI of odd subbands for CSI report 1, if configured otherwise

Priority 3: Group 1 CSI for CSI report 2, if configured as ‘typeII-r16’ or ‘typeII-PortSelection-r16’; Part 2 subband CSI of even subbands for CSI report 2, if configured otherwise

Priority 4: Group 2 CSI for CSI report 2, if configured as ‘typeII-r16’ or ‘typeII-PortSelection-r16’. Part 2 subband CSI of odd subbands for CSI report 2, if configured otherwise

⁞

Priority 2NRep-1: Group 1 CSI for CSI report NRep, if configured as ‘typeII-r16’ or ‘typeII-PortSelection-r16’; Part 2 subband CSI of even subbands for CSI report NRep, if configured otherwise

Priority 2NRep: Group 2 CSI for CSI report NRep, if configured as ‘typeII-r16’ or ‘typeII-PortSelection-r16’; Part 2 subband CSI of odd subbands for CSI report NRep, if configured otherwise

-------------------------------------------------- End of text proposal ------------------------------------------------------

Proposal 3: Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1.

- Conclusion

In this contribution, we provided the following test proposals for TS 38.214:

Proposal 1: Cx,y defined in Table 5.2.2.2.5-4 should be applied to the algorithm for calculating selected FD basis index.

Proposal 2: yt,l should be used instead of yt in clause 5.2.2.2.6.

Proposal 3: Omission of Part 2 CSI is according to the priority order shown in Table 5.2.3-1.