3GPP TSG RAN WG1 #100-e		                                             R1-2000608

e-Meeting, February 24th – March 6th, 2020

Agenda item:	7.2.2.1.2

Source: 	Samsung

Title:	DL signals and channels for NR-U

Document for:	Discussion and Decision

Introduction

This contribution discusses remaining issues related to the DL signals and channels for NR-U:

- Remaining issue on search space set switching

- Clarification on average CSI-RS measurement

- Clarification on DL reception outside of COT duration

Remaining issue on search space set switching

In order to provide flexible downlink transmission starting positions while minimizing UE power consumption, search space set switching feature has been adopted for NR-U. One remaining issue on search space set switching is to define required UE processing time (i.e., P1 and P2 value) when UE is required to switch search space set. For example, if a UE is provided by SearchSpaceSwitchTrigger-r16 and if the search space set switching field indicates search space set switching, the UE will start monitoring PDCCH at a first slot that is at least P1 symbols after the slot where UE detects DCI format 2_0. Similarly, if a UE is not provided by SearchSpaceSwitchTrigger-r16 and if the UE detects a DCI format by monitoring PDCCH with a search space set, the UE will start monitoring PDCCH with the other search space set at a first slot that is at least P2 symbols after the slot where UE detects the DCI format.

In Release 15, a processing time for UE required to provide HARQ-ACK information in response to a SPS PDSCH release is defined in TS38.213 as follows.

A UE is expected to provide HARQ-ACK information in response to a SPS PDSCH release after symbols from the last symbol of a PDCCH providing the SPS PDSCH release. If processingType2Enabled of PDSCH-ServingCellConfig is set to enable for the serving cell with the PDCCH providing the SPS PDSCH release, for , for , and for , otherwise, for , for , for , and for , wherein corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH providing the SPS PDSCH release and the SCS configuration of a PUCCH carrying the HARQ-ACK information in response to a SPS PDSCH release.

Since the overall UE processing time required to perform search space set switching is similar to the above case, the same processing time can be applied to search space set switching for both P1 and P2.

Proposal 1: Adopt following TP for TS38.213 to define processing time for search space set switching.

- 10.4 Search space set switching A UE can be provided a group index for a respective search space set by searchSpaceGroupIdList-r16 for PDCCH monitoring on a serving cell indicated by searchSpaceSwitchingGroup-r16. If the UE is not provided searchSpaceGroupIdList-r16 for a search space set, or for PDCCH monitoring on a serving cell that is not indicated by searchSpaceSwitchingGroup-r16, the following procedures are not applicable for PDCCH monitoring according to the search space set. A UE can be provided, by searchSpaceSwitchingTimer-r16, a timer value. The UE decrements the timer value by one after each slot in the active DL BWP of the serving cell where the UE monitors PDCCH for detection of DCI format 2_0. If a UE is provided by SearchSpaceSwitchTrigger-r16 a location of a search space set switching field for a serving cell in a DCI format 2_0, as described in Clause 11.1.1, and detects DCI format 2_0 in a slot - if the UE is not monitoring PDCCH according to search space sets with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at a first slot that is at least N symbols after the slot in the active DL BWP of the serving cell, where N is defined in Clause 10.2, if a value of the search space set switching field is 0 - if the UE is not monitoring PDCCH according to search space sets with group index 1, the UE monitors PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least N symbols after the slot in the active DL BWP of the serving cell, where N is defined in Clause 10.2, and the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16, if a value of the search space set switching field is 1 - if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least N symbols after a slot where the timer expires, where N is defined in Clause 10.2 or after a last slot of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0 If a UE is not provided SearchSpaceSwitchTrigger-r16 for a serving cell, - if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0 in a slot, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least N symbols after the slot in the active DL BWP of a serving cell, where N is defined in Clause 10.2, the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16 if the UE detects a DCI format by monitoring PDCCH in any search space set - if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least N symbols after a slot where the timer expires, where N is defined in Clause 10.2 or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last slot of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

Clarification on average CSI-RS measurement

In RAN1 #99 meeting, the following agreement regarding CSI-RS power assumption was made.

Agreement: A UE shall not average CSI-RS measurements for channel estimation across different transmission bursts from the UE's perspective. - FFS: Potential issues due to AGC

The agreement is captured in 38.214 as below.

5.2.1.1 Report settings ----omitted---- For operation with shared spectrum channel access, the UE should not average CSI-RS measurements for channel estimation from occasions of an NZP CSI-RS (defined in [4, TS 38.211]) located in different DL transmissions burst (defined in [X, TS 37.213]).

However, based on TS 37.213, the DL transmission burst definition is specified from gNB point of view, and this definition is not aligned with the agreement where the transmission bursts should be defined from the UE’s perspective.

A UE may not know the exact duration of a DL transmission burst, e.g., DCI format 2_0 is not configured and/or a DL transmission burst from gNB includes DL transmissions for other UEs, the current 38.214 description incurs ambiguity in a UE behaviour especially when the UE does not know about gNB’s DL transmission burst.

To be more aligned with the agreement, following TP is proposed.

Proposal 2: Adopt following TP for TS38.214 to be more aligned with the agreement.

----------------------------------------Text Proposal for 38.214---------------------------------------

5.2.1.1 Report settings ----omitted---- For operation with shared spectrum channel access, the UE should not average CSI-RS measurements for channel estimation from occasions of an NZP CSI-RS (defined in [4, TS 38.211]) located in discontinuous DL transmissions whose gap is greater than 16us.

Clarification on DL reception outside of COT duration

According to current specification, a UE should assume existence of higher layer configured DL signal/channel (e.g., periodic CSI-RS, SPS PDSCH, etc.) outside of gNB’s channel occupancy. If the UE assume the existence for TRS or periodic CSI-RS for channel measurement, the UE’s tracking performance and CSI reporting quality can be severely affected. Similarly, if the UE assumes the existence for SPS PDSCH outside of gNB’s channel occupancy, the UE would waste power for decoding SPS PDSCH unnecessarily. Moreover, the UE will transmit HARQ-ACK, which is waste resource in UE perspective as well as in system perspective. To address this issue, available information related to gNB’s CO should be utilized to validate the reception of such DL signals and channels. In particular, the UE assumes those DL signals/channels are transmitted if the occasions are fully overlapped with a CO duration indicated by a DCI 2_0, if configured, or a DL transmission burst identified from the UE’s perspective, or with a duration when the timer introduced for search space set switching is running.

Proposal 3: A UE assumes periodic CSI-RS and/or SPS PDSCH are transmitted if the occasions for those signals/channels are fully overlapped with at least one of followings

- Remaining CO duration indicated by DCI format 2_0

- DL transmission burst from UE’s perspective

- Within the timer introduced for search space set switching is running

Conclusion

The proposals made in this contribution are summarized below:

Proposal 1: Adopt following TP for TS38.213 to define processing time for search space set switching.

- 10.4 Search space set switching A UE can be provided a group index for a respective search space set by searchSpaceGroupIdList-r16 for PDCCH monitoring on a serving cell indicated by searchSpaceSwitchingGroup-r16. If the UE is not provided searchSpaceGroupIdList-r16 for a search space set, or for PDCCH monitoring on a serving cell that is not indicated by searchSpaceSwitchingGroup-r16, the following procedures are not applicable for PDCCH monitoring according to the search space set. A UE can be provided, by searchSpaceSwitchingTimer-r16, a timer value. The UE decrements the timer value by one after each slot in the active DL BWP of the serving cell where the UE monitors PDCCH for detection of DCI format 2_0. If a UE is provided by SearchSpaceSwitchTrigger-r16 a location of a search space set switching field for a serving cell in a DCI format 2_0, as described in Clause 11.1.1, and detects DCI format 2_0 in a slot - if the UE is not monitoring PDCCH according to search space sets with group index 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at a first slot that is at least N symbols after the slot in the active DL BWP of the serving cell, where N is defined in Clause 10.2, if a value of the search space set switching field is 0 - if the UE is not monitoring PDCCH according to search space sets with group index 1, the UE monitors PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least N symbols after the slot in the active DL BWP of the serving cell, where N is defined in Clause 10.2, and the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16, if a value of the search space set switching field is 1 - if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least N symbols after a slot where the timer expires, where N is defined in Clause 10.2 or after a last slot of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0 If a UE is not provided SearchSpaceSwitchTrigger-r16 for a serving cell, - if the UE detects a DCI format by monitoring PDCCH according to a search space set with group index 0 in a slot, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, on the serving cell at a first slot that is at least N symbols after the slot in the active DL BWP of a serving cell, where N is defined in Clause 10.2, the UE sets the timer value to the value provided by searchSpaceSwitchingTimer-r16 if the UE detects a DCI format by monitoring PDCCH in any search space set - if the UE monitors PDCCH on a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH on the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, on the serving cell at the beginning of the first slot that is at least N symbols after a slot where the timer expires, where N is defined in Clause 10.2 or, if the UE is provided a search space set to monitor PDCCH for detecting a DCI format 2_0, after a last slot of a remaining channel occupancy duration for the serving cell that is indicated by DCI format 2_0

Proposal 2: Adopt following TP for TS38.214 to be more aligned with the agreement.

----------------------------------------Text Proposal for 38.214---------------------------------------

5.2.1.1 Report settings ----omitted---- For operation with shared spectrum channel access, the UE should not average CSI-RS measurements for channel estimation from occasions of an NZP CSI-RS (defined in [4, TS 38.211]) located in discontinuous DL transmissions whose gap is greater than 16us.

Proposal 3: A UE assumes periodic CSI-RS and/or SPS PDSCH are transmitted if the occasions for those signals/channels are fully overlapped with at least one of followings

- Remaining CO duration indicated by DCI format 2_0

- DL transmission burst from UE’s perspective

- Within the timer introduced for search space set switching is running