3GPP TSG RAN WG1 #100									    R1-2000836

e-Meeting, February 24th – March 6th, 2020

Agenda Item:	7.2.4.2.2

Source:	InterDigital, Inc.

Title:	Remaining Issues on NR Sidelink Mode 2 Resource Allocation

Document for:	Discussion and Decision

# Introduction

In RAN #86, the following issues were discussed and identified as remaining open issues for Mode 2 Resource allocation to be addressed [1] in RAN1 #100:

| Category |  | Remaining issue in RAN1 | RAN2 impact |
| --- | --- | --- | --- |
| Mode 2 Resource Allocation | C1 | Details of T1, T2, T3 in the sensing and resource (re-)selection window | No |
|  | C2 | Details on resource selection from the identified candidate resources within the selection window for blind and HARQ feedback-based (re)transmission | No |
|  | C3 | Remaining details of pre-emption | Depends on the outcome of RAN1 discussion |
|  | C4 | Remaining details of re-evaluation of Step 1 and Step 2 | No |

In this contribution, we propose the remaining details of NR sidelink resource allocation schemes for mode 2.

# Discussion

## Blind and HARQ-based retransmission

It has been agreed to support blind retransmission and HARQ-based retransmission. Each of retransmission scheme seems to be beneficial in different scenario although both can be used for higher reliability transmission. For example, a blind retransmission is more appropriate for a packet with stringent PDB since it allows multiple retransmissions without HARQ feedback delay. On the other hand, HARQ-based retransmission is adequate for a packet with more relaxed PDB since it can support more retransmissions and/or it can reduce resource waste as it will reuse reserved resource for another TB transmission after reception of positive HARQ feedback.

A mixture of these two retransmission schemes has been proposed. However, there seems to no clear benefit to support the mixture of two retransmission schemes while it increases standards efforts and UE implementation complexity.

Proposal 1: A mixture of blind and HARQ-based retransmission is not supported for Mode 2.

## Resource allocation for HARQ based retransmission

NR V2X supports HARQ-based retransmission and blind retransmission. The timing of a HARQ-based retransmission resource should be after the HARQ feedback. Therefore, to help the UE in selecting resources for initial transmission and HARQ-based retransmission of the same TB, the resource selection windows for each transmission should be specified. Specifically, a HARQ based retransmission resource should be selected to allow the Tx UE having time to receive HARQ feedback from the Rx UE and prepare the retransmission TB.

Proposal 2: Resource selection window for HARQ-based retransmission resource of the same TB considers HARQ feedback timing and PSFCH processing time.

## Resource reservation for retransmissions

It was agreed [2] that resource reservation for HARQ feedback-based PSSCH retransmission is supported. However, in most cases (e.g., 90% HARQ ACK is received) the reserved resource is not used by the Tx UE which may result in resource wastage. Therefore, it is necessary to restrict the usage of resource reservation for HARQ feedback-based PSSCH in a certain scenario and to allow HARQ feedback-based PSSCH retransmission without resource reservation in other scenarios.

Proposal 3: HARQ feedback-based PSSCH retransmissions is also supported without resource reservation from the prior transmission of the same TB.

If HARQ feedback-based PSSCH transmission without reservation from the prior transmission of the same TB is supported, the resources for retransmission are selected only after HARQ NACK is received. Since there is no reserved resource for retransmission which is beneficial when a UE received HARQ ACK, the resources for retransmission can be selected as similar to that of initial transmission.

Proposal 4: When HARQ feedback-based PSSCH retransmission without resource reservation is used, the resource selection for retransmission after receiving HARQ NACK is the same as that for the initial transmission.

When the Tx UE reserves HARQ feedback-based PSSCH retransmission, the reserved HARQ re-transmission resource can be used by Tx, Rx, or other UEs when HARQ ACK is received. If the reserved resource is used by the Tx or Rx UE only, the resource can be wasted, since in most cases, e.g., 90%, HARQ ACK is received and the Tx or Rx UEs may have empty buffer so that there is no data to send in the released resource. Therefore, to increase resource utilization of the released resource efficiently, other UEs in the coverage of the Rx UE should be allowed to reuse the released resource if it detects HARQ ACK from the Rx UE.

In the following, system level simulations were conducted to evaluate the system performance based on the usage of the released resource by other UEs. Sidelink unicast is assumed in the simulation and details of simulation parameters are described in Table 1 in Appendix. The following three HARQ transmission schemes were evaluated:

- No HARQ scheme: Initial transmission followed by blind retransmission. Initial transmission resource based on long term sensing.

- HARQ without reuse: Initial transmission followed by HARQ (NACK) based re-transmission. Both initial transmission resource and HARQ retransmission resource are selected based on long term sensing. Retransmission on HARQ retransmission resource occurs if NACK is received by transmitter. In case of ACK, there is no retransmission, and the reserved HARQ resource goes unused.

- HARQ with reuse: Initial transmission followed by HARQ (NACK) based re-transmission: Both initial transmission resource and HARQ retransmission resource are selected based on long term sensing. Retransmission on HARQ retransmission resource if NACK is received by the transmitter. In case of ACK, there is no retransmission, and the reserved HARQ resource is freed up and available for use by other UEs who received the resource reservation message and HARQ ACK

Figure 1: Performance of HARQ transmission schemes where the periodic of the traffic is 30ms.

Figure 2: Performance of HARQ transmission schemes where the periodic of the traffic is 30ms.

Figure 3: Performance of HARQ transmission schemes where the periodic of the traffic is 30ms.

Figures 1, 2, and 3 show the performance of three HARQ transmission schemes in the Urban scenario, when the periodic of the traffic are 30, 20, and 10ms, respectively. It can be observed from the figures that, the HARQ with reuse scheme performs the best and the performance gap between the HARQ reuse scheme and other schemes increases as the distance index increases. Moreover, HARQ with reuse can achieve up to 5% PRR gain compare to the HARQ without reuse scheme and 20% compare to the no HARQ scheme. It is because allowing the other UEs to reuse the reserved HARQ resource can increase the spectrum efficiency of the system and reduce the collision.

Observation 1: Significant PRR gain is obtained when reserved HARQ resource is reused by other UEs.

Proposal 5: The reserved HARQ retransmission resource can be reused by other UEs based on HARQ-ACK detection.

## Resource (re-)selection procedure

In the last meeting, RAN1 agreed that when the candidate resource is smaller than X% of the total resource, the RSRP thresholds are increased by 3dB and the remaining issue is whether to support the stop of RSRP threshold increment. From our perspective, it is necessary to restrict the RSRP threshold increment. It is because when the RSRP threshold is high enough, it illustrates that the excluded resources are occupied by the high priority transmissions or the nearby UEs. Not restricting the RSRP threshold increment may result in the high possibility of selecting the resources reserved by nearby UEs or the high priority transmission as a consequence it may result in a high collision probability.

To alleviate the issue, a maximum number of RSRP threshold increment should be configured and if the maximum RSRP threshold increment is reached, the UE can select the transmission resource from the candidate ones even the set of candidate resources is smaller than X% of the total resource.

Proposal 6: Support the maximum number of RSRP threshold increment.

Proposal 7: The UE selects the transmission resource from the candidate resources when RSRP threshold increment is greater than a configured value.

## Resource (re-)evaluation

In a re-evaluation procedure, the UE reports the set of candidate resources SA to the MAC layer to perform resource selection. We first discuss how to change the resource(s) from the previous iteration to the current iteration. There are two possible options

- Option 1: Change the collided resource(s) only

- Option 2: Change all resource(s).

Option 1 requires a complicated resource selection procedure and it limits the possible resources to select. Specifically, for both blind and HARQ feedback based retransmissions the newly selected resource(s) should be within the window W of the non-collided resource(s) from the previous iteration. For HARQ feedback based retransmission, the newly selected resource(s) needs to guarantee the HARQ feedback timing of the non-collided resource(s) from the previous iteration. Moreover, due to the limited possible resources selection, there may be no available resource to replace the collided resource(s), further UE behaviours may need to be specified.

Option 2 may result in the transmission delay since the UE sometimes replaces an early resource to a later one in time event it is not a collided one. However, this option is simpler to implement and does not require additional specification impact. For example, the initial resource selection procedure can be reused for the resource selection procedure of any re-evaluation iteration.

We now discuss on the condition to change the resource(s) from the previous iteration to the resource(s) from current iteration. If one of the selected resource(s) in the last iteration is not in the set SA, that resource needs to be reselected. Therefore, the UE changes the resource(s) from the previous iteration to the resource(s) from current iteration if at least one of the resource(s) from the previous iteration is not in the set SA of current iteration.

Proposal 8: The UE changes all the resource(s) from the previous iteration to the resource(s) from current iteration if at least one of the resource(s) from the previous iteration is not in set SA of the current iteration.

It was agreed that the UE is required to evaluate a selected resource at least slot ‘m-T3’ before its reservation is signaled at slot ‘m’. However, if the time gap between slot ‘m’ and the maximum delay slot is small, the UE may not be able to select resource(s) for transmission. In this case, it is necessary to allow UE not to trigger re-evaluation. The maximum delay slot is the last possible transmission slot the UE can make for the TB to guarantee that TB reception of the TB is within the packet delay budget.

Proposal 9: The UE is not required to perform re-evaluation for resource reservation signaled at slot ‘m’ if the time gap between slot ‘m’ and the maximum delay slot is small.

# Conclusion

In this contribution, we discussed on the remaining issues for Mode 2 resource allocation for NR V2X. Based on the discussions, we propose the following:

Proposal 1: A mixture of blind and HARQ-based retransmission is not supported for Mode 2.

Proposal 2: Resource selection window for HARQ-based retransmission resource of the same TB considers HARQ feedback timing and PSFCH processing time.

Proposal 3: HARQ feedback-based PSSCH retransmissions is also supported without resource reservation from the prior transmission of the same TB.

Proposal 4: When HARQ feedback-based PSSCH retransmission without resource reservation is used, the resource selection for retransmission after receiving HARQ NACK is the same as that for the initial transmission.

Proposal 5: The reserved HARQ retransmission resource can be reused by other UEs based on HARQ-ACK detection.

Proposal 6: Support the maximum number of RSRP threshold increment.

Proposal 7: The UE selects the transmission resource from the candidate resources when RSRP threshold increment is greater than a configured value.

Proposal 8: The UE changes all the resource(s) from the previous iteration to resource(s) from current iteration if at least one of the resource(s) from the previous iteration is not in set SA of the current iteration.

Proposal 9: The UE is not required to perform re-evaluation for resource reservation signaled at slot ‘m’ if the time gap between slot ‘m’ and the maximum delay slot is small.

# References

- RP-193198, “Task list for 5G V2X in RAN1 #100”, LG Electronics

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #98bis Chongqing, China, Oct. 2019.

- Chairman’s Notes, 3GPP TSG RAN1 WG1 #97 Reno, US, May 2019.

# Appendix

Table 1: Simulation assumption for HARQ feedback for unicast

| Parameter | Value |
| --- | --- |
| Carrier frequency | 6 GHz |
| Bandwidth | 20 MHz |
| Sub-channel size | 32 RBs |
| Sub-carrier spacing | 15 kHz |
| TX/Rx Antennas | 2 Tx/2 Rx |
| Scenario | Urban |
| Traffic model | Periodic: Medium intensity (Model 2), 10/20/30 ms inter-packet arrival, 50% vehicles generate packets. |
| SCI/Data resource multiplexing | Same slot SCI and Data transmission with Option 3 multiplexing scheme |
| SCI/Data frequency resource allocation | PSCCH: 5 PRB PSSCH: 32 PRB |
| SCI/Data time resource allocation | PSCCH: 2 Symbols PSSCH: 10 Symbols (no PSFCH in slot) : 9 symbols (PSFCH in same slot) |
| Overhead (for both PSSCH and PSCCH) | 4/14 (GP, DMRS, AGC) |
| Sensing and resource selection | Mode 2 based on long term sensing procedure (as specified for LTE mode 4) |
| Sensing distance (dsensing) | 700 m |
| Unicast session distance | 500 m for Highway 150m for Urban Grid |
| Data packet Tx parameters | Periodic: 800 bytes 16-QAM (CR 0.42/0.48) 1200 bytes: 16 QAM (CR 0.64) / 64-QAM (CR 0.5) |
| Fixed or maximum number of retransmissions (without/with HARQ feedback, respectively) | 1 |