3GPP TSG RAN WG1 #100	R1-2000873

e-Meeting, February 24th – March 6th, 2020

Source:	Sharp

Title:	Remaining issues and corrections on channel access procedure for NR-U

Agenda Item:	7.2.2.2.1

Document for:	Discussion and Decision

Background

The following agreement was achieved at RAN1#99 meeting [1].

Agreement: - For RAR, we use the same LBT type and CP extension tables and CAPC selection mechanism as used for the Fallback DCI UL grant. - This requires 2 bits to be signaled in RAR (the PDSCH) - If the UE multiplexes user plane data with PUSCH - The UE assumes CAPC=4 was used by the gNB to acquire the CO, - For UE initiated COTs (Cat4 case) the UE may select the CAPC by itself. - Note: The mapping between priority classes and traffic classes follows the same mechanism as defined for UL CG transmissions. - The frequency domain resource allocation field can be reduced to accommodate 2 bits in the RAR - Include this agreement in an LS to RAN2

Remaining details for channel access procedure is discussed.

Discussion

The agreement above is implemented in TS38.213V16.0.0 [2]. Since the frequency domain resource allocation field in RAR UL grant is truncated two bits for operation with shared spectrum channel access, the corresponding interpretation on the field should be also updated.

Proposal: The following Text proposal is adopted.

Text proposal --------- beginning of text proposal for TS 38.213 - 8.3 PUSCH scheduled by RAR UL grant An active UL BWP, as described in Clause 12 and in [4, TS 38.211], for a PUSCH transmission scheduled by a RAR UL grant is indicated by higher layers. For determining the frequency domain resource allocation for the PUSCH transmission within the active UL BWP - if the active UL BWP and the initial UL BWP have same SCS and same CP length and the active UL BWP includes all RBs of the initial UL BWP, or the active UL BWP is the initial UL BWP, the initial UL BWP is used - else, the RB numbering starts from the first RB of the active UL BWP and the maximum number of RBs for frequency domain resource allocation equals the number of RBs in the initial UL BWP The frequency domain resource allocation is by uplink resource allocation type 1 if useInterlacePUSCH-Common is not provided and by uplink resource allocation type 2 if useInterlacePUSCH-Common is provided [6, TS 38.214]. For an initial UL BWP size of RBs, if useInterlacePUSCH-Common is not provided, a UE processes the frequency domain resource assignment field as follows - if - truncate the frequency domain resource assignment field to its least significant bits and interpret the truncated frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212] - else - insert for operation without shared spectrum channel access and log2NBWPsize⋅NBWPsize+1/2-12 for operation with shared spectrum channel access, most significant bits with value set to '0' after the bits to the frequency domain resource assignment field, where if the frequency hopping flag is set to '0' and is provided in Table 8.3-1 if the hopping flag bit is set to '1', and interpret the expanded frequency resource assignment field as for the frequency resource assignment field in DCI format 0_0 as described in [5, TS 38.212] - end if -------- Unchanged contents are omitted --------- end of text proposal

Conclusion

In this contribution, we have the following proposals:

Proposal: The following Text proposal is adopted.

References

- Chairman’s note at RAN1#99

- 3GPP TS38.213V16.0.0