3GPP TSG RAN WG1 Meeting #100-e	R1-2000984

February 24th – March 6th, 2020

Agenda item:	7.2.12

Source: 			Qualcomm Incorporated

Title: 			Discussion

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide text change proposals for the Rel-16 NR RAN1 UE feature list.

| Features | Index | Feature group | Components | Prerequisite feature groups |  | Need for the gNB to know if the feature is supported |  | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | - RACH type selection based on RSRP threshold - Fall back from 2-step RACH to 4-step RACH - Separate RO with 4-step RO configuration - Shared RO with 4-step RO configuration - msgA PUSCH occasions configuration - Intra-slot frequency hopping for msgA PUSCH - Up to two msgA PUSCH configurations in an UL BWP from UE perspective - Periodic validation of msgA PRACH and PUSCH occasion per SSB to RO association period - Periodic mapping between preamble and PUSCH occasion with DM-RS resource per SSB to RO association period - msgA PUSCH scrambling with ID set extension - Multiple DMRS sequences for msgA PUSCH - msgA PRACH power control - msgA PUSCH power control - msgB DCI format with CRC scrambled by msgB-RNTI and indication of 2 LSBs ofSFN - Monitoring of msgB - Support PUCCH transmission for HARQ-ACK feedback to a MsgB that contains successRAR, and the PUCCH is configured with the same numerology, TX spatial filter and UL BWP as the last msgA PUSCH transmission/retransmission - For MsgA PRACH support all preamble formats applicable to the band - For MsgA PRACH, support the same set of numerologies as for 4-step RACH in the same band - For MsgA PUSCH, support the same set of numerologies as for Msg3 in the same band - For MsgA PUSCH waveform, support both CP-OFDM and DFT-s-OFDM - Support DMRS ports with configuration type 1. - Minimum TX gap between PRACH and PUSCH (for both TDD and FDD, FR1 and FR2, single CC and intra-band CA) - Minimum TX gap between last DL SSB reception symbol and PRACH (TDD. FR1 and FR2, single CC and intra-band CA) - MsgA PRACH and PUSCH in different slots - Support 2-step RACH on PCell or PSCell after UL carrier selection - Minimum TX gap between the last symbol of msgB PDSCH and the first symbol of PUCCH carrying HARQ-ACK to msgB PDSCH - Support msgA-prach-ConfigurationIndexNew in FR1: INTEGER (256.. 262). - Support RV=0 for msgA PUSCH initial transmission, re-attempt and fall-back - msgB PDSCH multiplexing different types of RARs and BI, including 12-bit MAC CE for timing advance command, 3-bit PDSCH-to-HARQ FB timing indicator, 4-bit PUCCH resource indicator and new MAC subheader |  |  | Yes |  | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 9-2 | 2-step RACH for NR-unlicensed | - Separate LBT for msgA PRACH and PUSCH - Interlaced msgA PUSCH | 9-1 |  | Yes |  | N/A | UE cannot initiate a 2-step RACH process for NR-U | per band | N/A | N/A | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 9-3 | Parallel MsgA and PRACH Msg1/SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel MsgA and PRACH Msg1/SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA | 9-1, 2-52, 4-1, 2-12, 6-6 |  | Yes |  | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | Per band pair per band combination | N/A | N/A | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 9-4 | MsgA operation in a band combination including SUL | MsgA operations in a band combination including SUL | 9-1, 6-16 |  | Yes |  | N/A | UE does not support msgA operations in a band combination including SUL | per band combination |  |  | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 9-5 | 2-step RACH on SCell in mTAG |  | 9-1 |  | Yes |  | N/A |  | per band | N/A | N/A | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
|  | 9-6 | 2-step RACH on SCell in sTAG |  | 9-1, 9-5 |  | Yes |  | N/A |  | per band | N/A | N/A | support mixture of FDD/TDD and/or FR1/FR2 |  | Optional with capability signalling |
| 10. NR-unlicensed | 10-1 | LBE channel access mechanism | 1. Cat 1 LBT 2. Cat 2 16us LBT 3. Cat 2 25us LBT 4. Cat 4 LBT 5. Reading COT frequency location info |  |  |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for LBE operation, including Cat 4 LBT, Cat 2 LBT, Cat 1 LBT, contention window adjustment. UE supports either 16 us or 25 us or both 16 and 25 us for Cat-2 LBT. | TBD |
|  | 10-2 | FBE channel access mechanism FFS: if UE as initiating device is supported | 1. Cat 1 LBT 2. Cat 2 LBT 3. 5ms and 10ms FFP |  |  |  |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of channel access mechanism for FBE operation, including fixed frame period, Cat 2 LBT, Cat 1 LBT | TBD |
|  | 10-2a | Shorter than 5ms FFP | Support of Fixed Frame Period less than 5ms | 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-2b | Reading COT frequency localization info with FBE | Reading COT frequency localization info | 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-3 | PRB interlace PUCCH and PUSCH | 1) PRB interlace PUCCH (EPF0, EPF1, EPF2, EPF3) 2) PRB interlace PUSCH | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | This can be a basic feature group for operating in unlicensed band. Support of PRB interlace PUCCH and PUSCH | TBD |
|  | 10-4 | 16us Cat 2 LBT support |  | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | If 16us Cat 2 LBT is supported | Optional with capability signalling |
|  | 10-5 | 25us Cat 2 LBT support |  | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | If 25us Cat 2 LBT is supported | Optional with capability signalling |
|  | 10-6 | Cat 4 LBT support with LBE |  | 10-1 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | If Cat4 LBT with contention window adjustment is supported | Optional with capability signalling |
|  | 10-7 | LBT bandwidth size | 1. 20MHz 2. 10MHz [Can be transferred to RAN4 responsibility] | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  |  |
|  | 10-8 | Type B PDSCH length | Length 3, 5, 6, 8, 9, 10, 11, 12, 13 FFS the capability are separate for each length or some groups are formed to signal the capability together | 10-1 or 10-2 |  | Yes |  | N/A |  | FFS: Per UE or per band |  |  |  | Support of additional length (other than 2/4/7) for type B PDSCH | Optional with capability signalling |
|  | 10-9 | PDCCH Case 2 operation withsearch space set group switching with LBE | Two groups of search space sets, the first group according to PDCCH Case 2 and the second group according to PDCC Case 1-1. Any grant received in the first group switches the UE to monitor the second group. | 10-1 |  | Yes |  | N/A |  | FFS: Per UE or per band |  |  |  | Being configured with two groups of search spaces (FFS more than two), and switch between them. Some search space sets can be configured in both groups. | Optional with capability signalling |
|  | 10-10 | RSSI measurement | RSSI measurement | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band |  |  |  | RSSI measurement for channel occupancy. | Optional with capability signalling [Already agreed in RAN2] |
|  | 10-11 | SRS starting position |  | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Support transmitting SRS starting in all symbols (0,…,13) of a slot | Optional with capability signalling |
|  | 10-12 | PRB interlace PF2 PF3 OCC FFS if need this capability | 1. No OCC 2. OCC2 3. OCC4 | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | UE OCC capability for EPF2/EFP3 | Optional with capability signalling |
|  | 10-13 | Extended CP range for PUSCH/PUCCH with LBE FFS if need this capability | Maximum length of CP extension a UE can generate before the first symbol | 10-1 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | How long a UE can generate the CP extension | Optional with capability signalling |
|  | 10-14 | Non-numerical PDSCH to HARQ-ACK timing | Non-numerical value for dl-DataToUL-ACK | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | If non-numerical K1 value is supported | Optional with capability signalling |
|  | 10-15 | Enhanced dynamic HARQ codebook | Support of bit fields signalling PDSCH HARQ group index and NFI in DCI 1_1 | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Enhanced dynamic HARQ codebook supporting grouping of HARQ ACK and triggering the retransmission of HARQ ACK in each groups | Optional with capability signalling |
|  | 10-16 | One-shot HARQ ACK feedback | HARQ-ACK codebook containing all configured HARQ processes for all configured CCs | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Upon triggering, UE reports A/N for all HARQ processes and all CCs in a PUCCH group. | Optional with capability signalling |
|  | 10-17 | Multi-PUSCH UL grant | Support of scheduling more than one PUSCH with one UL grant | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-18a | Configured grant with retransmission in CG resources with LBE | Configured grant with retransmission in CG resources with LBE | 10-1 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
|  | 10-18b | Configured grant with retransmission in CG resources with FBE | Configured grant with retransmission in CG resources with FBE | 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Support configured grant with retransmission in configured grant resource | Optional with capability signalling |
|  | 10-19 | Number of LBT bandwidth | Number of ED measurements the UE is able to perform simultaneously Whether the UE in WB operating mode can support nx20MHz measurement | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | This is the number of LBT bandwidth a UE can perform separate ED check on simultaneously Note: Do not intend to imply fewer LBT than number of UL CCs in CA | Optional with capability signalling |
|  | 10-20 | Support search space set association with frequency offsets | Only for wideband Search space is replicated across CORESETs but total BD limit is not increased Frequency offset between replica CORESETS don’t follow 6RB raster Enhanced CORESET frequency granularity | 10-1 or 10-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 10-21a | COT indication in same CC with LBE | Reading COT duration information in scheduled CC | 10-1 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Does not imply dynamic SFI capability | Optional with capability signalling |
|  | 10-21b | COT indication in same band with LBE | Reading COT duration information in another CC in the band of the scheduled CC | 10-1 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Does not imply dynamic SFI capability | Optional with capability signalling |
|  | 10-21c | COT indication in licensed for LBE | Reading COT duration information in licensed band | 10-1 |  | Yes |  | N/A |  | Per band | Yes | Yes | differentiation is from the perspective of cell with DCI | Does not imply dynamic SFI capability | Optional with capability signalling |
|  | 10-22 | 2-step RACH transmission with no gap | Back-to-back transmission of MsgA PRACH and msgA PUSCH with no gap in between | 9-1, 10-1 |  | Yes |  | N/A |  | per band | N/A | N/A | support mixture of FDD/TDD and/or FR1/FR2 | Applicable to NR-U only | Optional with capability signalling |
| 11. NR_L1enh_URLLC | 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 | 1) Supports monitoring DCI format 1_2 for DL scheduling 2) Supports monitoring DCI format 0_2 for UL scheduling |  |  | Yes |  | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the scheduling cell |  | Optional with capability signalling |
|  | 11-1a | DCI format 1_2 with DCI format 1_1 in the same search space | Monitoring DCI format 1_2 together with DCI format 1_1 in the same search space | 11-1 |  | Yes |  | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the scheduling cell |  | Optional with capability signalling |
|  | 11-1b | DCI format 0_2 with DCI format 0_1 in the same search space | Monitoring DCI format 0_2 together with DCI format 0_1 in the same search space | 11-1 |  | Yes |  | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the scheduling cell |  | Optional with capability signalling |
|  | 11-1c | DL priority indication in DCI with mixed DCI formats | When both DCI format 1_1 and DCI format 1_2 are configured to be monitored per BWP, a DCI format (from the formats 1_1/1_2) can be used to schedule PDSCH with different HARQ-ACK priorities. | 11-1 |  | Yes |  | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the scheduling cell |  | Optional with capability signalling |
|  | 11-1d | UL priority indication in DCI with mixed DCI formats | When both DCI format 0_1 and DCI format 0_2 are configured to be monitored per BWP, a DCI format (from the formats 0_1/0_2) can be used to schedule PUSCH with different priorities. | 11-1 |  | Yes |  | N/A |  | Per UE | Yes | Yes | The differentiation is from the perspective of the scheduling cell |  | Optional with capability signalling |
|  | 11-2 | Increased PDCCH monitoring capability | 1) Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span and the limit M on the maximum number of BDs for combination (X, Y, ) 2) Supported combinations of (X, Y, ) 3) If UE reports the support of more than one combination of (X, Y) for a given SCS, and if multiple combinations of (X, Y) are valid for the span pattern, the span pattern with the maximum value of C and M from the valid combinations is applied | 3-5b |  | Yes |  | N/A |  | FSPC | N/A | N/A |  | This capability is necessary for SCS 15kHz and 30 kHz. Component-2 candidate value set: (X, Y) = {(7, 3), (4, 3), (2, 2)} | Optional with capability signalling |
|  | 11-2a | Mix of increased PDCCH monitoring capability and FG3-1 PDCCH monitoring capability in the same slot in the same CC | 1) Supports PDCCH monitoring operation according to FG3-1 2) In addition to 1), supports PDCCH monitoring with limit C on the maximum number of additional non-overlapped CCEs for channel estimation per PDCCH monitoring span and with limit M on the maximum number of additional BDs, for a combination (X, Y, ) 3) Supported combinations of (X, Y, ) 4) If UE reports the support of more than one combination of (X, Y) for a given SCS, and if multiple combinations of (X, Y) are valid for the span pattern, the span pattern with the maximum value of C and M from the valid combinations is applied | 3-5b |  | Yes |  | N/A |  | FSPC | N/A | N/A |  | This capability is necessary for SCS 15kHz and 30 kHz. Component-3 candidate value set: (X, Y) = {(7, 3), (4, 3), (2, 2)} | Optional with capability signalling |
|  | 11-2b | Mix of increased PDCCH monitoring capability and FG3-2 PDCCH monitoring capability in the same slot in the same CC | 1) Supports PDCCH monitoring operation according to FG3-2 2) In addition to 1), supports PDCCH monitoring with limit C on the maximum number of additional non-overlapped CCEs for channel estimation per PDCCH monitoring span and with limit M on the maximum number of additional BDs, for a combination (X, Y, ) 3) Supported combinations of (X, Y, ) 4) If UE reports the support of more than one combination of (X, Y) for a given SCS, and if multiple combinations of (X, Y) are valid for the span pattern, the span pattern with the maximum value of C and M from the valid combinations is applied | 3-2, 3-5b |  | Yes |  | N/A |  | FSPC | N/A | N/A |  | This capability is necessary for SCS 15kHz and 30 kHz. Component-3 candidate value set: (X, Y) = {(7, 3), (4, 3), (2, 2)} | Optional with capability signalling |
|  | 11-2c | Mix of increased PDCCH monitoring capability and FG3-5b PDCCH monitoring capability in the same slot in the same CC | 1) Supports PDCCH monitoring operation according to FG3-5b for combination (X1, Y1, ) 2) In addition to 1), supports PDCCH monitoring with limit C on the maximum number of additional non-overlapped CCEs for channel estimation per PDCCH monitoring span and with limit M on the maximum number of additional BDs, for a combination (X, Y, ) 3) Supported combinations of (X1, Y1, ) 4) Supported combinations of (X2, Y2, ) 5) If UE reports the support of more than one combination of (X2, Y2) for a given SCS, and if multiple combinations of (X2, Y2) are valid for the span pattern, the span pattern with the maximum value of C and M from the valid combinations is applied | 3-5b |  | Yes |  | N/A |  | FSPC | N/A | N/A |  | This capability is necessary for SCS 15kHz and 30 kHz. Component-3 candidate value set: (X1, Y1) = {(7, 3), (4, 3) and (7, 3), (2, 2) and (4, 3) and (7, 3)} Component-4 candidate value set: (X2, Y2) = {(7, 3), (4, 3), (2, 2)} | Optional with capability signalling |
|  | 11-2b | Relative TDRA for DL | The time domain resource allocation for PDSCH is a function of the PDCCH symbol index within the slot | 11-2 |  | Yes |  | N/A |  | FS | N/A | N/A |  |  | Optional with capability signalling |
|  | 5-11c | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 - PDSCH(s) for Msg. 4 is included |  |  | Yes |  | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
|  | 5-12c | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  |  | Yes |  | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS | Optional with capability signalling |
|  | 5-13d | Up to 3 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 3 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a or 5-5b |  | Yes |  | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |
|  | 5-13g | Up to 3 unicast PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 3 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology • When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 3 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c |  | Yes |  | N/A |  | FS | N/A | N/A |  | This capability is necessary for each SCS More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signalling |
|  | 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | 1) Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot. • At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. - Supported combinations of (X, Y), where X is the minimum gap between sub-slots containing actual PUCCH transmissions measured from beginning to beginning of the sub-slots, including across slots, and Y is the sub-slot duration, with both X and Y in units of symbols |  |  | Yes |  | N/A |  | Perband | N/A | N/A |  | Component-2 candidate value set: (X, Y) = {(7, 7), (4, 2) and (7, 7), (2, 2) and (4, 2) and (7, 7)} | Optional with capability signalling |
|  | 11-3a | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-5a or 5-5b |  | Yes |  | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
|  | 11-3b | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13 |  | Yes |  | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
|  | 11-3c | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13a |  | Yes |  | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
|  | 11-3d | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13c |  | Yes |  | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
|  | 11-3e | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | CBG based transmission for UL with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time Capability 2 | 5-13d |  | Yes |  | N/A |  | Per UE | No | FR1 only |  | [Modification of Rel-15 capability] | Optional with capability signalling |
|  | 11-4a | Up to two HARQ-ACK codebooks constructed for supporting different service types for a UE with restriction | 1) Supports up to two HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4) Only one of the HARQ ACK codebooks can have sub-slot based PUCCH configuration 5) If both processing time capability 1 CC(s) and processing time capability 2 CC(s) are configured, and both slot-based and sub-slot based PUCCH are configured, then HARQ-ACK feedback for a processing time capability 1 CC can only take place in the slot-based PUCCH and HARQ-ACK feedback for a processing time capability 2 CC can only take place in the sub-slot based PUCCH |  |  | Yes |  | N/A |  | FS | N/A | N/A | The differentiation is from the perspective of the PUCCH cell |  | Optional with capability signalling |
|  | 11-4b | Up to two HARQ-ACK codebooks constructed for supporting different service types for a UE without restriction | 1) Supports up to two HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. | 11-4a |  | Yes |  | N/A |  | FS | N/A | N/A | The differentiation is from the perspective of the PUCCH cell |  | Optional with capability signalling |
|  | 11-5a | PUSCH repetition Type B with self-scheduling with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions |  |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5b | PUSCH repetition Type B with self-scheduling with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions | 5-12 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5c | PUSCH repetition Type B with self-scheduling with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions | 5-12a |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5d | PUSCH repetition Type B with self-scheduling with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions | 5-12b |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5e | PUSCH repetition Type B with self-scheduling with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 1. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions | 5-12c |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5f | PUSCH repetition Type B with self-scheduling with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with 1 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions | 5-5c |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5g | PUSCH repetition Type B with self-scheduling with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with up to 2 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions | 5-13d |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5h | PUSCH repetition Type B with self-scheduling with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with up to 7 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions | 5-13e |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5i | PUSCH repetition Type B with self-scheduling with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with up to 4 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions | 5-13f |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5j | PUSCH repetition Type B with self-scheduling with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2 | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with self-scheduling with up to 3 unicast PUSCHs per slot per CC for different TBs with UE processing time capability 2. • The number of the repetitions signalled by gNB represents the "nominal" number of repetitions. The actual number of repetitions can be larger than the nominal number. • The time domain resource assignment (TDRA) field in the DCI or the TDRA parameter in the type 1 configured grant indicates the resource for the first "nominal" repetition. • The time domain resources for the remaining repetitions are derived based at least on the resources for the first repetition and the UL/DL direction of the symbols • If a "nominal" repetition goes across the slot boundary or DL/UL switching point, this "nominal" repetition is split into multiple PUSCH repetitions, with one PUSCH repetition in each UL period in a slot. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. • Jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. • The maximum TDRA table size is increased to 64. 3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field. 4) PUSCH repetition Type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) Maximum number of actual PUSCH repetitions | 5-13g |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Componenet-5 candidate value set: {0, 1, 2, 4, 7, 16} | Optional with capability signalling |
|  | 11-5k | PUSCH repetition Type B with cross-carrier scheduling with UE processing time capability 1 | For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with cross- carrier scheduling with UE processing time capability 1. | 6-10 or 6-10a, and 11-5a, 11-5b, 115c, 11-5d or 11-5e |  | Yes |  | N/A |  | Per band | N/A | N/A |  | The maximum number of actual PUSCH repetitions is the same as in Component-5 of 11-5a, 11-5b, 115c, 11-5d or 11-5e | Optional with capability signalling |
|  | 11-5l | PUSCH repetition Type B with cross-carrier scheduling with UE processing time capability 2 | For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots with cross- carrier scheduling with UE processing time capability 2. | 6-10 or 6-10a, and 5-5c and 11-5f, 11-5g, 115h, 11-5i or 11-5j |  | Yes |  | N/A |  | Per band | N/A | N/A |  | The maximum number of actual PUSCH repetitions is the same as in Component-5 of 11-5f, 11-5g, 115h, 11-5i or 11-5j | Optional with capability signalling |
|  | 11-6 | PUSCH repetition Type A | PUSCH repetition Type A. 1) With slot aggregation, the number of repetitions can be dynamically indicated 2) Maximum number of PUSCH repetitions | 2-12, 2-13, 2-14, 2-15 |  | Yes |  | N/A |  | Perband | N/A | N/A |  | Componenet-1 candidate value set: {‘semi-static only’, ‘both semi-static and dynamic’} Componenet-2 candidate value set: {8, 12, 16} | Optional with capability signalling |
|  | 11-7a | UL cancelation scheme on same CC | 1) Supports group common DCI (i.e. DCI format 2_4) for cancelation indication on the same CC as PUSCH or SRS 2) UL cancelation for PUSCH 3) UL cancelation for SRS |  |  | Yes |  | N/A |  | FS | N/A | N/A |  |  | Optional with capability signalling |
|  | 11-7b | More than one monitoring occasion for DCI 2_4 per slot on same CC | Monitoring occasions per slot for DCI 2_4 reception on the same CC as PUSCH or SRS Supported combinations of (X, Y), where X is the minimum gap between monitoring occasions measured from beginning to beginning of the monitoring occasions, including across slots, and Y is the duration of the monitoring occasion, with both X and Y in units of symbols | 11-7a |  | Yes |  | N/A |  | FS | N/A | N/A |  | Candidate value set: (X, Y) = {(7, 7), (4, 2) and (7, 7), (2, 2) and (4, 2) and (7, 7)} | Optional with capability signalling |
|  | 11-7c | UL cancelation scheme on another CC | 1) Supports group common DCI (i.e. DCI format 2_4) for cancelation indication on a different CC from PUSCH or SRS 2) UL cancelation for PUSCH 3) UL cancelation for SRS |  |  | Yes |  | N/A |  | FS | Yes | Yes | The differentiation is from the perspective of the cell with DCI |  | Optional with capability signalling |
|  | 11-7d | More than one monitoring occasion for DCI 2_4 per slot on another CC | Monitoring occasions per slot for DCI 2_4 reception on a different CC from PUSCH or SRS Supported combinations of (X, Y), where X is the minimum gap between monitoring occasions measured from beginning to beginning of the monitoring occasions, including across slots, and Y is the duration of the monitoring occasion, with both X and Y in units of symbols | 11-7c |  | Yes |  | N/A |  | FS | N/A | N/A |  | Candidate value set: (X, Y) = {(7, 7), (4, 2) and (7, 7), (2, 2) and (4, 2) and (7, 7)} | Optional with capability signalling |
|  | 11-7e | TB CRC for re-transmission of an interrupted PUSCH | Support of including valid (not all-zero) TB CRC for a re-transmission of an interrupted PUSCH where the re-transmission doesn’t include all CBGs of the interrupted transmission. | 5-25, 11-7a or 11-7c |  | Yes |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 11-8 | Enhanced UL power control scheme | For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set |  |  | Yes |  | N/A |  | Per UE | Yes | Yes | If UE supports 11-8 and also supports cross-carrier scheduling, then the differentiation is from the perspective of the scheduled cell |  | Optional with capability signalling |
|  | 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | 1) Supports up to 12 configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations - Max number of configured grant configurations in a BWP of a serving cell - Max number of configured grant configurations across all BWPs and serving cells |  |  | Yes |  | N/A |  | Per UE | No | Yes |  | Component-2, candidate value set is {1, 2, …, 12} Component-3, candidate value set is {1, 2, …, [24]} Total number in FR1 is not greater than X value reported for FR1. Total number in FR2 is not greater than X value reported for FR2. Total number across FR1 and FR2 is not greater than the larger of the FR1 and FR2 values | Optional with capability signalling |
|  | 11-9a | Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication | 11-9 |  | Yes |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of channel/signals with two priority levels | Support intra-UE multiplexing/prioritization of UL channels/signals with two priority levels - Prioritization between UL channels/signals with different priority levels - Multiplexing/prioritization between UL channels/signals with the same priority level |  |  | Yes |  | N/A |  | FS | N/A | N/A |  | Component-1 candidate value set: {‘supported’, ‘not supported’} Component-2 candidate value set: {‘supported’, ‘not supported’} | Optional with capability signaling |
|  | 12-2 | Multiple SPS configurations | - Support of multiple SPS configurations, including separate activation/release and joint release - The max number of SPS configurations in a BWP of a serving cell - The max number of SPS configurations across all serving cells - The related HARQ-ACK enhancements to support multiple SPS configurations | 5-18 |  | Yes |  | N/A |  | Per UE | Yes | Yes | differentiation is from the perspective of cell applying activation/release | Component-2, candidate value set is {1, 2, …, 8} Component-3, candidate value set is {1, 2, …, [16]} Total number in FR1 is not greater than X value reported for FR1. Total number in FR2 is not greater than X value reported for FR2. Total number across FR1 and FR2 is not greater than the larger of the FR1 and FR2 values | Optional with capability signaling |
|  | 12-3 | Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, where the association between each state indicated by the indication and the SPS configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple SPS configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the SPS configuration index indicated by the indication | 12-2 |  | Yes |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
| 13. NR Positioning | 13-1 | Concurrent support of DL-TDOA, AoD, multi-RTT Positioning Methods | Support of concurrent configuration of DL-TDOA, AoD, multi-RTT Positioning methods. |  |  | Yes |  | N/A |  | Per UE | N/A | N/A |  | Component-1 candidate value set: Up to 8 strings with values {000, …, 111} A 3-bit string in which each bit corresponds to the concurrent support of the DL-TDOA, AoD, multi-RTT respectively, where 0 means “no support” and 1 means “support” | Optional with capability signalling |
|  | 13-2 | support of DL-TDOA, AoD, multi-RTT Positioning Methods per supported CA Band Combination | For each supported CA Band Combination in the BandCombinationList, indication of concurrent configuration of a list of the DL-TDOA, AoD, multi-RTT Positioning methods |  |  | Yes |  | N/A |  | Per band combination | N/A | N/A |  | Component-1 candidate value set: Up to 8 strings with values {000, …, 111} A 3-bit string in which each bit corresponds to the concurrent support of the DL-TDOA, AoD, multi-RTT respectively, where 0 means “no support” and 1 means “support” | Optional with capability signalling |
|  | 13-3 | DL PRS processing across all positioning methods per measurement window when measurement gaps are configured | - Max # of frequency layers (X1) that can be supported by the UE across all positioning methods - A limit on the maximum number N1 of DL PRS resources the UE is expected to measure across all TRPs and frequency layers within a measurement window of T1 msec reported as a list of tuples {N1, T1} when measurement gaps are configured - A limit on the maximum number of symbols N2 containing PRS resources of maximum bandwidth the UE is expected to measure within a measurement window of T2 msec reported as a list of tuples {N2, T2} when measurement gaps are configured The values T1, T2 in msec of the two measurement windows |  |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {1,4} Component-2 candidate value set: N1 = FFS T1 = {0.125,0.25,0.5,1,40,80,160,320, 640,1280} Component-3 candidate value set: N2 = FFS T2 = {0.125,0.25,0.5,1,40,80,160,320, 640,1280} | Optional with capability signalling |
|  | 13-4 | DL PRS processing across all positioning methods per measurement window when measurement gaps are not configured | - Max # of frequency layers (X1) that can be supported by the UE across all positioning methods - A limit on the maximum number N3 of DL PRS resources the UE is expected to measure across all TRPs and frequency layers within a measurement window of T3 msec reported as a list of tuples {N3, T3} when measurement gaps are not configured - A limit on the maximum number of symbols N4 containing PRS resources of maximum bandwidth the UE is expected to measure within a measurement window of T4 msec reported as a list of tuples {N4, T4} when measurement gaps are not configured The values T3, T4 in msec of the two measurement windows |  |  | Yes |  | N/A | When the UE does not support this feature, the UE does not support DL PRS processing outside measurement gaps | Per band | N/A | N/A |  | Component-1 candidate value set: {1,4} Component-2 candidate value set: N3 = FFS T3 = {0.125,0.25,0.5,1,40,80,160,320, 640,1280} Component-3 candidate value set: N4 = FFS T4 = {0.125,0.25,0.5,1,40,80,160,320, 640,1280} | Optional with capability signalling |
| 13a. DL-TDOA positioning | 13a-1 | DL PRS RSTD Measurement | 1. Max # of frequency layers (X1) that can be configured 2. Max number of TRPs per frequency layer a UE can be configured (X2) 3. Max # of DL PRS resource sets per TRP per frequency layer supported by UE (X3) 4. Max number of Resources per PRS resource set (X4) that can be configured 5. Max number of DL PRS Resources that UE can be configured (across all frequency layers, TRPs and resource sets) (X5) 6. Max number of TRPs for all frequency layers that can be configured per UE (X6) 7. Max number of Resources that can be configured per frequency layer (X7) 8. Max number of DL PRS resources per TRP a UE can be configured 9. Support of inter-frequency measurement 10. Support of measurement quality metric 11. Support of SSB from neighbour cell as QCL Type C source of a DL PRS resource 12. Support of SSB from neighbour cell as QCL Type D source of a DL PRS resource 13. Support of a DL PRS resource as QCL Type D source of another DL PRS resource | 13-1,13-3,13-4 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {0, 1, 4} Component-2 is 64 Component-3 candidate value set: {1, 2} Component-4 candidate value set: {4, 8, 64} Component-5 candidate value set: {64, 128, 192, 256, 512, 1024, 2048} Component-6 candidate value set: {64, 128, 256} Component-7 candidate value set: {24, 64, 128, 256, 512, 1024} Component-8 candidate value set: {4, 8, 16, 32, 64, 128, 256, 512} | Optional with capability signalling |
|  | 13a-2 | DL PRS RSRP Measurement | 1. Max # of frequency layers (X1) that can be configured 2. Max number of TRPs per frequency layer a UE can be configured (X2) 3. Max # of DL PRS resource sets per TRP per frequency layer supported by UE (X3) 4. Max number of Resources per PRS resource set (X4) that can be configured 5. Max number of DL PRS Resources that UE can be configured (across all frequency layers, TRPs and resource sets) (X5) 6. Max number of TRPs for all frequency layers per UE (X6) that can be configured 7. Max number of Resources that can be configured per frequency layer (X7) 8. Max number of DL PRS resources per TRP a UE can be configured 9. Support of inter-frequency measurement 10. Support of measurement quality metric 11. Support of SSB from neighbor cell as QCL Type C source of a DL PRS resource 12. Support of SSB from neighbor cell as QCL Type D source of a DL PRS resource 13. Support of a DL PRS resource as QCL Type D source of another DL PRS resource 14. Support of RSRP measurement with fixed Rx beam & reporting | 13-1,13-3,13-4 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {0, 1, 4} Component-2 is 64 Component-3 candidate value set: {1, 2} Component-4 candidate value set: {4, 8, 64} Component-5 candidate value set: {64, 128, 192, 256, 512, 1024, 2048} Component-6 candidate value set: {64, 128, 256} Component-7 candidate value set: {24, 64, 128, 256, 512, 1024} Component-8 candidate value set: {4, 8, 16, 32, 64, 128, 256, 512} | Optional with capability signaling |
|  | 13a-3 | DL PRS measurement reporting for UE assisted positioning | - Number M of RSTDs the UE can report on different DL PRS resources of the same pair of TRPs. Note: All the RSTD measurements in a single report should have a single reference timing Number N of RSRPs the UE can report on different DL PRS resources of the same pair of TRPs. | 13a-1, 13a-2 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {1, 2, 4} Component-2 candidate value set: {1, 2, 4, 8} | Optional with capability signalling |
|  | 13a-4 | UE Based Positioning | TBD specific content in RAN2 WG - Estimation of location coordinate at UE side and support of corresponding signaling for DL PRS configuration / measurements | 13a-1, 13-1,13-3,13-4 |  | Yes |  | N/A |  | per UE | Yes | Yes | Differentiation is from the perspective of frequency layer of measurements |  | Optional with capability signalling |
| 13b. NR DL-AoD positioning | 13b-1 | DL PRS RSRP Measurement | 1. Max # of frequency layers (X1) that can be configured 2. Max number of TRPs per frequency layer a UE can be configured (X2) 3. Max # of DL PRS resource sets per TRP per frequency layer supported by UE (X3) 4. Max number of Resources per PRS resource set (X4) that can be configured 5. Max number of DL PRS Resources that UE can be configured (across all frequency layers, TRPs and resource sets) (X5) 6. Max number of TRPs for all frequency layers per UE (X6) that can be configured 7. Max number of Resources that can be configured per frequency layer (X7) 8. Max number of DL PRS resources per TRP a UE can be configured 9. Support of inter-frequency measurement 10. Support of measurement quality metric 11. Support of SSB from neighbor cell as QCL Type C source of a DL PRS resource 12. Support of SSB from neighbor cell as QCL Type D source of a DL PRS resource 13. Support of a DL PRS resource as QCL Type D source of another DL PRS resource 14. Support of RSRP measurement with fixed Rx beam & reporting | 13-1,13-3,13-4 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {0, 1, 4} Component-2 is 64 Component-3 candidate value set: {1, 2} Component-4 candidate value set: {4, 8, 64} Component-5 candidate value set: {64, 128, 192, 256, 512, 1024, 2048} Component-6 candidate value set: {64, 128, 256} Component-7 candidate value set: {24, 64, 128, 256, 512, 1024} Component-8 candidate value set: {4, 8, 16, 32, 64, 128, 256, 512} | Optional with capability signaling |
|  | 13b-2 | DL PRS measurement reporting for UE assisted positioning | Number N of RSRPs the UE can report on different DL PRS resources of the same pair of TRPs. | 13b-1 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {1, 2, 4, 8} | Optional with capability signalling |
|  | 13b-3 | UE Based Positioning | TBD specific content in RAN2 WG - Estimation of location coordinate at UE side and support of corresponding signaling for DL PRS configuration / measurements | 13b-1, 13-1,13-3,13-4 |  | Yes |  | N/A |  | Per UE | Yes | Yes | Differentiation is from the perspective of frequency layer of measurements |  | Optional with capability signalling |
| 13c NR E-CID positioning | 13c-1 | DL SSB RRM measurements for NR Positioning | Configuration and reporting of SSB RRM measurements (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 | support of SSB measurements for RRM (SS-RSRP, SS-RSRQ) based on Rel.15 for NR Positioning in Rel.16 |  | Yes |  | N/A |  | Per UE | Yes | Yes |  |  | Optional with capability signalling |
|  | 13c-2 | CSI-RS RRM measurements for NR Positioning | Configuration and reporting of CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) for NR Positioning in Rel.16 | support CSI-RS for RRM (CSI-RSRP, CSI-RSRQ) based on Rel.15 for NR Positioning in Rel.16 |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 13d UL-TDOA and AoA positioning | 13d-1 | Transmission of SRS for NR positioning | 1. Max # of SRS Resource Sets for positioning per BWP 2a. Max # of aperiodic SRS Resources for positioning across all SRS Resource Sets per BWP 2b. Max # of aperiodic SRS Resources for positioning across all SRS Resource Sets per BWP per slot 3a. Max # of semi-persistent SRS Resources for positioning all SRS Resource Sets per BWP 3b. Max # of semi-persistent SRS Resources for across all SRS Resource Sets per BWP per slot 4a. Max # of periodic SRS Resources for positioning across all SRS Resource Sets per BWP 4b. Max # of periodic SRS Resources for positioning across all SRS Resource Sets per BWP per slot 5. Support of SSB for neighbour cell DL pathloss estimation and SRS for positioning OLPC 6. Support of DL PRS for neighbour cell DL pathloss estimation and SRS for positioning OLPC 7. Spatial Relation of SRS Resources for positioning with DL PRS / SSB 8. Spatial Relation of SRS Resources for positioning with SRS resources for positioning 9. Number of additional pathloss estimates across all SRS resource sets for positioning that the UE is expected to simultaneously maintain, in addition to the up to four pathloss estimates per serving cell specified for PUSCH/PUCCH/SRS transmissions |  |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {0, 1, 2, 4, 8, 16} Component 2a,3a,4a candidate value set: {1, 2, 4, 8, 16} Component 2b,3b,4b candidate value set: {1, 2, 3, 4, 5, 6, 8, 12, 14} Component-9 candidate value set: {0, 4, 8, 16} | Optional with capability signalling |
| 13e. NR multi-RTT positioning | 13e-1 | UE Rx-Tx Time Difference Measurement and Reporting | 1. Max # of frequency layers (X1) that can be configured 2. Max number of TRPs per frequency layer a UE can be configured (X2) 3. Max # of DL PRS resource sets per TRP per frequency layer supported by UE (X3) 4. Max number of Resources per PRS resource set (X4) that can be configured 5. Max number of DL PRS Resources that UE can be configured (across all frequency layers, TRPs and resource sets) (X5) 6. Max number of TRPs for all frequency layers per UE (X6) that can be configured 7. Max number of Resources that can be configured per frequency layer (X7) 8. Max number of DL PRS resources per TRP a UE can be configured 9. Support of inter-frequency measurement 10. Support of measurement quality metric 11. Support of SSB from neighbor cell as QCL Type C source of a DL PRS resource 12. Support of SSB from neighbor cell as QCL Type D source of a DL PRS resource 13. Support of a DL PRS resource as QCL Type D source of another DL PRS resource 14. Number of Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL PRS resource/resource set of the same cell. Note: The DL PRS resource/resource sets can be in different positioning frequency layers if the UE supports inter-frequency UE Rx-Tx time difference measurements. | 13-1,13-3,13-4, 13e-3 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {0, 1, 4} Component-2 is 64 Component-3 candidate value set: {1, 2} Component-4 candidate value set: {4, 8, 64} Component-5 candidate value set: {64, 128, 192, 256, 512, 1024, 2048} Component-6 candidate value set: {64, 128, 256} Component-7 candidate value set: {24, 64, 128, 256, 512, 1024} Component-8 candidate value set: {4, 8, 16, 32, 64, 128, 256, 512} Component-14 candidate value set: {1, 2, 4} | Optional with capability signalling |
|  | 13e-2 | DL PRS RSRP Measurement | 1. Max # of frequency layers (X1) that can be configured 2. Max number of TRPs per frequency layer a UE can be configured (X2) 3. Max # of DL PRS resource sets per TRP per frequency layer supported by UE (X3) 4. Max number of Resources per PRS resource set (X4) that can be configured 5. Max number of DL PRS Resources that UE can be configured (across all frequency layers, TRPs and resource sets) (X5) 6. Max number of TRPs for all frequency layers per UE (X6) that can be configured 7. Max number of Resources that can be configured per frequency layer (X7) 8. Max number of DL PRS resources per TRP a UE can be configured 9. Support of inter-frequency measurement 10. Support of measurement quality metric 11. Support of SSB from neighbor cell as QCL Type C source of a DL PRS resource 12. Support of SSB from neighbor cell as QCL Type D source of a DL PRS resource 13. Support of a DL PRS resource as QCL Type D source of another DL PRS resource 14. Support of RSRP measurement with fixed Rx beam & reporting 15. Number N of RSRPs the UE can report on different DL PRS resources of the same pair of TRPs. |  |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {0, 1, 4} Component-2 is 64 Component-3 candidate value set: {1, 2} Component-4 candidate value set: {4, 8, 64} Component-5 candidate value set: {64, 128, 192, 256, 512, 1024, 2048} Component-6 candidate value set: {64, 128, 256} Component-7 candidate value set: {24, 64, 128, 256, 512, 1024} Component-8 candidate value set: {4, 8, 16, 32, 64, 128, 256, 512} Component-15 candidate value set: {1, 2, 4, 8} | Optional with capability signalling |
|  | 13e-3 | Transmission of SRS for NR positioning | 1. Max # of SRS Resource Sets for positioning per BWP 2a. Max # of aperiodic SRS Resources for positioning across all SRS Resource Sets per BWP 2b. Max # of aperiodic SRS Resources for positioning across all SRS Resource Sets per BWP per slot 3a. Max # of semi-persistent SRS Resources for positioning all SRS Resource Sets per BWP 3b. Max # of semi-persistent SRS Resources for across all SRS Resource Sets per BWP per slot 4a. Max # of periodic SRS Resources for positioning across all SRS Resource Sets per BWP 4b. Max # of periodic SRS Resources for positioning across all SRS Resource Sets per BWP per slot 5. Support of SSB for neighbour cell DL pathloss estimation and SRS for positioning OLPC 6. Support of DL PRS for neighbour cell DL pathloss estimation and SRS for positioning OLPC 7. Spatial Relation of SRS Resources for positioning with DL PRS / SSB 8. Spatial Relation of SRS Resources for positioning with SRS resources for positioning 9. Number of additional pathloss estimates across all SRS resource sets for positioning that the UE is expected to simultaneously maintain, in addition to the up to four pathloss estimates per serving cell specified for PUSCH/PUCCH/SRS transmissions |  |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {0, 1, 2, 4, 8, 16} Component 2a,3a,4a candidate value set: {1, 2, 4, 8, 16} Component 2b,3b,4b candidate value set: {1, 2, 3, 4, 5, 6, 8, 12, 14} Component-9 candidate value set: {0, 4, 8, 16} | Optional with capability signalling |
| 13f. NR RAT Independent Positioning | 13f-1 | TBD in RAN2 WG | [TBD in RAN2 WG] |  |  | Yes |  | N/A |  | Per UE | N/A | N/A |  |  | Optional with capability signalling |
| 14. NR TEI | 14-1 | Multiple LTE-CRS rate matching patterns | - Maximum number of LTE-CRS rate matching patterns in total within a NR carrier | 5-28 (Rate-matching around LTE CRS) |  | Yes |  | N/A |  | Per band | N/A | N/A |  | For DSS | Optional with capability signaling {2, 3, 4, 5, 6} |
|  | 14-2 | Multiple LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier | - Up to two LTE-CRS overlapping rate matching patterns within a part of NR carrier overlapping with a LTE carrier | 14-1 (Multiple LTE-CRS rate matching patterns), x-x (mTRP support) |  | Yes |  | N/A |  | [Per band] | N/A | N/A |  | For DSS | Optional with capability signaling |
|  | 14-3 | PDSCH Type B mapping of length 9 and 10 OFDM symbols | - PDSCH Type B mapping of length 9 OFDM symbols PDSCH Type B mapping of length 10 OFDM symbols | 5-6a (PDSCH mapping type B) |  | Yes |  | N/A |  | Perband | N/A | N/A |  | For DSS | Optional with capability signaling |
|  | 14-4 | One-slot periodic TRS | One-slot periodic TRS configuration for FR1 TDD |  |  | Yes |  | N/A |  | Per UE | TDD only | FR1 only |  | Applicable only when no two consecutive slots are indicated as downlink slots by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigDedicated | Optional with capability signaling |
|  | 14-5 | Rel-16 configuration of SRS for antenna switching | 1) Support SRS Tx port switch, 2) Report whether the uplink Tx switching impact to downlink receiving in a band 3) Report whether the UL Tx is switched together with UL Tx in another band | 2-55 |  | Yes |  | N/A |  | FSPC | N/A | N/A |  | Component-1 is a list of TxRx pairs, candidates are {“Not supported”, {t1r1, t1r2}, {t1r1, t1r2, t1r4}, {t1r1, t1r2, t2r2, t2r4}, {t1r1, t2r2}, {t1r1, t2r2, t4r4}, {t1r1, t1r2, t2r2, t1r4, t2r4} ] Component-2: Candidate value set: {yes, no} Component-3: Candidate value set: {yes, no} | Optional with capability signaling |
|  | 14-6 | Duplex direction collision handling with same SCS | Support for directional collision handling between reference and other cell(s) with the same SCS |  |  | Yes |  | N/A |  | Per UE | TDD only | Yes |  | Applicable to TDD intra-band CA and to TDD CCs in inter-band CA with half-duplex constraint | Optional with capability signaling |
|  | 14-7 | Duplex direction collision handling with different SCS | Support for directional collision handling between reference and other cell(s) with the different SCS |  |  | Yes |  | N/A |  | Per UE | TDD only | Yes |  | Applicable to TDD CCs in inter-band CA with half-duplex constraint | Optional with capability signaling |
| 15. 5G_V2X_NRSL | 15-1 | Receiving NR sidelink based on pre-configured parameters | 1) UE can receive PSCCH/PSSCH transmitted using NR sidelink. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot counting both PSCCH and PSSCH. 4) UE supports normal 64QAM MCS table. 5) UE supports reception of PSFCH. 6) UE supports 2 receive antennas |  |  |  |  | No |  | Per band | N/A | N/A |  | This is the basic FG for sidelink Component-2 candidate value set: {value1, value2, …} Component-3 candidate value set: {value1, value2, …} | Optional with capability signaling. For UE supports NR sidelink, UE must indicate this FG is supported |
|  | 15-1a | Receiving NR sidelink configured by NR Uu | 1) UE can receive PSCCH/PSSCH transmitted using NR sidelink 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot counting both PSCCH and PSSCH. 4) UE supports normal 64QAM MCS table. 5) UE supports reception of PSFCH 6) UE supports 2 receive antennas | 15-1 |  |  |  |  |  | Per band | N/A | N/A |  | Component-2 and Component-3 are the same as in FG15-1 and are not reported separately | Optional with capability signalling |
|  | 15-2 | Transmitting NR sidelink mode 2 based on pre-configured parameters | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 based on pre-configured parameters. 2) UE supports normal 64QAM MCS table. 3) Transmission of PSFCH. 4) Rank 1 PSSCH transmission | 15-1 |  |  |  | No |  | Per band | N/A | N/A |  | Note: Random selection in the exceptional pool is supported | Optional with capability signalling |
|  | 15-3 | Transmitting NR sidelink mode 1 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using dynamic scheduling and configured grant type 1 and 2 in NR sidelink mode 1 scheduled by NR Uu. 2) UE supports normal 64QAM MCS table. 3) Transmission of PSFCH. 4) Rank 1 PSSCH transmission | 15-1 |  | Yes |  | No |  | Per band | N/A | N/A |  | Note: Random selection in the exceptional pool is supported | Optional with capability signalling |
|  | 15-3a | Transmitting NR sidelink mode 2 configured by NR Uu | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu 2) UE supports normal 64QAM MCS table. 3) Transmission of PSFCH 4) Rank 1 PSSCH transmission | 15-1, 15-2 |  |  |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-4 | S-SSB for NR sidelink based on pre-configured parameters for GNSS based synchronisation in case of P0, P1, P2 and P6 priority sources | - UE can transmit and receive S-SSB in NR sidelink - UE supports GNSS based synchronisation based on P0, P1, P2 and P6 priority order of synchronisation sources | 15-1, 15-2 |  |  |  | No |  | Per band | N/A | N/A |  |  | Mandatory without capability signaling |
|  | 15-4a | S-SSB for NR sidelink based on pre-configured parameters for GNSS based synchronisation in case of P3, P4 and P5 priority sources | - UE additionally supports GNSS based synchronisation based on P3, P4 and P5 priority order of synchronisation sources | 15-4 |  |  |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-4b | S-SSB for NR sidelink based on pre-configured parameters for eNB/gNB based synchronization | - UE support gNB/eNB based synchronisation based on P0’ to P6’ priority order of synchronisation source | 15-4a |  |  |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-4c | S-SSB for NR sidelink configured by NR Uu | - UE can transmit and receive S-SSB in NR sidelink.configured by NR Uu | 15-4b |  |  |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-5 | Sidelink congestion control | 1) UE can report CBR measurement to gNB. 2) UE can adjust its radio parameters based on CBR measurement and CRlimit. |  |  | Yes |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-6 | Short-term time-scale TDM for in-device coexistence | 1) Support prioritization between LTE sidelink transmission/reception and NR sidelink transmission/reception | At least one of 15-1, 15-2 |  |  |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-7 | Transmitting LTE sidelink mode 3 scheduled by NR Uu | 1) UE can transmit PSCCH/PSSCH using SPS in LTE sidelink mode 3 scheduled by NR Uu. |  |  | Yes |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-8 | UE inter-RAT sidelink control processing capability | 1) UE reports a value ‘X’ for the minimum value it supports for the additional time indicated in the NR DCI scheduling LTE sidelink mode 3. | 15-7 |  | Yes |  | No |  | Per band | N/A | N/A |  | Component-1 candidate value set: {value1, value2, …} | Optional with capability signalling |
|  | 15-9 | Transmitting LTE sidelink mode 4 configured by NR Uu | 1) UE can transmit PSCCH/PSSCH using LTE sidelink mode 4 configured by NR Uu and preconfiguration. |  |  | Yes |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-10 | Transmitting NR sidelink mode 1 scheduled by LTE Uu | UE can transmit PSCCH/PSSCH using configured grant Type -1 in NR sidelink mode 1 scheduled by LTE Uu. |  |  | Yes |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-11 | Transmitting NR sidelink mode 2 configured by LTE Uu | UE can transmit PSCCH/PSSCH using in NR sidelink mode 2 configured by LTE Uu. |  |  | Yes |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-12 | 256QAM sidelink transmission | 1) UE can transmit PSSCH with 256QAM in NR sidelink | 15-2 |  |  |  | Yes | UE supports QPSK, 16QAM, and 64 QAM for transmission. | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-13 | 256QAM sidelink reception | 1) UE can receive PSSCH with 256QAM in NR sidelink | 15-1 |  |  |  | Yes |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-14 | Low-spectral efficiency 64QAM MCS table | 1) UE additionally supports transmission/reception with low-spectral efficiency 64QAM MCS table. | At least one of 15-1, 15-2 |  |  |  | Yes | UE supports normal 64QAM MCS table. | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-15 | PT-RS | 1) UE supports NR sidelink transmission and reception using PT-RS. | At least one of 15-1, 15-2 |  |  |  | Yes |  | Per UE | N/A | FR2 only |  |  | in Mandatory without capability signalling in FR2. Not supported in FR1 |
|  | 15-16 | 60kHz SCS for FR1 | UE additionally supports 60KHz SCS for FR1 for PSCCH/PSSCH/PSFCH/S-SSB | 15-1 |  |  |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-17 | 15kHz SCS for FR1 | UE additionally supports 15KHz SCS for FR1 for PSCCH/PSSCH/PSFCH/S-SSB | 15-1, |  |  |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-18 | 4Rx antenna | UE supports 4 receive antennas | 15-1 |  |  |  | No |  | FS | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-19 | Support of SL slot less than 14 consecutive symbols | UE additionally support transmission/reception of SL slot less than 14 consecutive symbols | 15-1 |  |  |  | No |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-20 | Support of Rank 2 transmission | UE additionally supports Rank 2 transmission for unicast | 15-22 and at least one of 15-2, 15-3 |  |  |  | Yes |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-21 | Support of Rank 2 reception | UE additionally supports Rank 2 reception for unicast | 15-1 and 15-23 |  |  |  | Yes |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-22 | Support of CSI RS transmission | UE support transmission of CSI RS | At least one of 15-2, 15-3 |  |  |  | Yes |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-23 | Support of CSI RS reception | - UE support CSI RS reception - UE support CSI reporting |  |  |  |  | Yes |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 15-24 | Support of open loop SL power control and RSRP report | Support open loop power control and RSRP report in case of unicast | At least one of 15-2, 15-3 |  |  |  | Yes |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
| 16. NR_eMIMO |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 16-1a | L1-SINR measurement | - L1-SINR based on CMR without dedicated IMR - L1-SINR based on CMR and dedicated ZP IMR - L1-SINR based on CMR and dedicated NZP IMR | 2-21 or 2-22 or 2-23 or 2-23a |  | Yes |  | N/A |  | Per Band | N/A | N/A |  | Component-1 candidate value set: {‘supported’, ‘not supported’} Component-2 candidate value set: {‘supported’, ‘not supported’} Component-3 candidate value set: {‘supported’, ‘not supported’} | Optional with capability signalling |
|  | 16-1b | Simultaneous TCI state activation across multiple CCs | - Simultaneous TCI state activation across multiple CCs: PDCCH, PDSCH - Max number of RRC configured CC lists | 2-4 |  | Yes |  | N/A |  | FS | N/A | N/A |  | Component-2: candidate value set: {0, 1, 2} | Optional with capability signalling |
|  | 16-1c | Simultaneous spatial relation update across multiple CCs | - Simultaneous spatial relation update across multiple CCs: AP-SRS, SP-SRS - Max number of RRC configured CC lists | 2-53, 2-59, 2-60 |  | Yes |  | N/A |  | FS | TDD only | FR2 only |  | Component-2: candidate value set: {0, 1, 2} Note: Applicable to FR2 only | Optional with capability signalling |
|  | 16-1d | Simultaneous spatial relation update for multiple PUCCH resources in a CC | - Simultaneous spatial relation update per PUCCH resource group sharing same spatial relation per BWP in a CC - Max number of RRC configured PUCCH resource groups sharing same spatial relation per BWP | 2-59, 2-60 |  | Yes |  | N/A |  | Per UE | TDD only | FR2 only |  | Component-2: candidate value set: {0, 1, 2, 3, 4} Note: Applicable to FR2 only | Optional with capability signalling |
|  | 16-1e | Default spatial relation and pathloss RS for dedicated PUCCH / SRS / PUSCH | Default spatial relation and pathloss RS for dedicated PUCCH / SRS / PUSCH | 2-59, 2-60, 8-2 |  | Yes |  | N/A |  | Per UE | TDD only | FR2 only |  |  | Optional with capability signalling |
|  | 16-1f | Spatial relation update for AP-SRS via MAC CE | Spatial relation update for AP-SRS via MAC CE | 2-53, 2-59, 2-60 |  | Yes |  | N/A |  | Per UE | TDD only | FR2 only |  |  | Optional with capability signalling |
|  | 16-1g | Pathloss reference RS update for PUSCH / SRS via MAC CE | - Pathloss reference RS update for PUSCH / SRS via MAC CE - Max total number of pathloss RS for power control of PUSCH / PUCCH / SRS per BWP | 8-2 |  | Yes |  | N/A |  | Per UE | No | No |  | Component-1 candidate value set: {‘supported’, ‘not supported’} Component-2: candidate value set: {4, 8, 16, 32, 64} | Optional with capability signalling |
|  | 16-1h | SCell beam failure recovery | - SCell BFR - Max number of SCells configured for BFR - Send BFR request in PUCCH | 2-31 |  | Yes |  | N/A |  | Per UE | No | No |  | Component-2: candidate value set: {0, 1, 2, 4, 8, 10, 12, 15} Component-3 candidate value set: {‘supported’, ‘not supported’} | Optional with capability signalling |
|  | 16-1i | Configured UL spatial relations | Maximum number of configured spatial relations per CC for PUCCH and SRS | 2-59 |  | Yes |  | N/A |  | Per UE | No | No |  | Candidate value set: {4, 8, 16, 32, 64, 96, 128, 256, 320} Note: extension of 2-59 Max value is increased from 64+8x4=96 to 64+64x4=320 to account for max number of PUCCH spatial relation increase from 8 to 64 | Optional with capability signalling |
|  | 16-1j | Max number of candidate beams for BFR | Max number of different CSI-RS and/or SSB resources configured across all CCs for new beam identification | 2-31 |  | Yes |  | N/A |  | Per UE | No need | No need |  | Candidate value set is: {1, …, 1024} Note: extension of 2-31, component 3 Max value is increased from 256 to 1024 to account for max candidate beam number increase from 16 to 64 | Optional with capability signalling |
|  | 16-1k | Band pair sharing same DL QCL | Indication of band pairs or band groups sharing the same DL QCL |  |  | Yes |  | N/A |  | FS | N/A | N/A |  | List of member bands for each QCL band group per band combination | Optional with capability signalling |
|  | 16-1l | Band pair sharing same UL QCL | Indication of band pairs or band groups sharing the same UL QCL |  |  | Yes |  | N/A |  | FS | TDD only | FR2 only |  | List of member bands for each QCL band group per band combination Note: Applicable to FR2 only | Optional with capability signalling |
|  | 16-1m | Max number of NZP CSI-RS across CCs per slot for beam management | Max # of NZP CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP and L1-SINR within a slot shall not exceed MB_x | 2-21, 2-22, 2-23, 2-23a, 16-1a |  | Yes |  | N/A |  | Per UE | No | Yes |  | Candidate value set for MB_x is {0, 4, 8, 16, 32, 64} Total number in FR1 is not greater than value reported for FR1. Total number in FR2 is not greater than value reported for FR2. Total number across FR1 and FR2 is not greater than the larger of the FR1 and FR2 values | Optional with capability signalling |
|  | 16-1n | Max number of CSI-RS across CCs per slot for BFD | Max # of total CSI-RS resources across all CCs in a slot for CSI-RS based BFD | 2-31, 16-1h, 1-14 |  | Yes |  | N/A |  | Per UE | No | No |  | Note: The all CCs include SPCell and SCell on both FR1 and FR2 Candidate value set: {2, 4, 6, 8} | Optional with capability signalling |
|  | 16-2a | SDM Scheme | Support of SDM scheme for single-DCI based single-PDSCH Single-DCI based single-PDSCH with 2 TCI states for 2 sets of antenna ports belonging to 2 CDM groups |  |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 16-2b | SDM PTRS | Support 2 ports for DL PTRS for SDM scheme | 16-2a |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 16-2c | FDM Scheme A | Support of FDM scheme A for single-DCI based single-PDSCH |  |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 16-2d | FDM Scheme B | Support of FDM scheme B for single-DCI based single-PDSCH |  |  | Yes |  | N/A |  | FSPC | N/A | N/A |  |  | Optional with capability signalling |
|  | 16-2e | FDM Scheme B with soft combining | Support of soft combining of the two codewords in FDM scheme B | 16-2d |  | Yes |  | N/A |  | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 16-2f | TDM Scheme A | Support of TDM scheme A for single-DCI based single-PDSCH - Maximum TB size supported for TDM scheme A in units of CBs |  |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {0, 10} CBs | Optional with capability signalling |
|  | 16-2g | TDM Scheme B | Support of TDM scheme B for single-DCI based single-PDSCH TDM scheme 4 with different transmissions occasions in different slots with up to 2 TCI states - Maximum TB size supported for TDM scheme B in units of CBs - Maximum number of repetitions (RepNum) |  |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Component-1 candidate value set: {0, 10} CBs Component-2 candidate value set: {8, 16} | Optional with capability signalling |
|  | 16-2h | Two default beams for s-DCI | Two default beams for single-DCI based single-PDSCH, determined as the TCI-states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states |  |  | Yes |  | N/A |  | Per band | TDD only | FR2 only |  | Note: Applicable to FR2 only | Optional with capability signalling |
|  | 16-2i | m-DCI based m-PDSCH | Support of multi-DCI based multi-PDSCH - Maximum number of CORESETs per BWP for multi-DCI based multi-PDSCH (other than CORESET 0) - Maximum number of CORESETs per BWP per CORESETPoolIndex value for multi-DCI based multi-PDSCH (other than CORESET 0) - The value of R for BD/CCE limit |  |  | Yes |  | N/A |  | FSPC | N/A | N/A |  | Component 1 candidate values: {2, 3, 4} Component 2 candidate values: {1, 2} Component 3 candidate values: {1, 2} | Optional with capability signalling |
|  | 16-2j | m-DCI Feedback | Indicates which feedback mode is supported for m-DCI based m-PDSCH | 16-2i |  | Yes |  | N/A |  | Per UE | No | No |  | Candidate values: {separate; joint, both} If value is omitted, inter-slot feedback is assumed (based on Rel-15). Note: ‘separate’ means non-overlapping PUCCH within the slot | Optional with capability signalling |
|  | 16-2k | Two simultaneous beams for m-DCI | Indicates whether UE supports receiving time-overlapping PDSCHs/PDCCHs with different beams | 16-2i |  | Yes |  | N/A |  | Per band | TDD | FR2 only |  | Note: Applicable to FR2 only | Optional with capability signalling |
|  | 2-4b | 16 TCI states for PDSCH | Support 16 active TCI states per BWP per CC, including control and data | 2-1 |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Note: UE is required to track only the active TCI states Note: If a UE reports 16 active TCI state(s), it is not expected that more than 16 active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell becomes active for the UE. | Optional with capability signalling |
|  | 16-2l | Active TCI per-TRP | Supported maximum number of active TCI states per BWP per CC per value of CORESETPoolIndex (including both data and control) | 16-2i |  | Yes |  | N/A |  | Per band | N/A | N/A |  | Candidate values: (1, 2, 4, 8} | Optional with capability signalling |
|  | 16-2m | PDSCH rate matching per CORESETPoolIndex | Support of separate rate matching per CORESETPoolIndex value for CRS patterns associated with a value of CORESETPoolIndex (if configured) applicable to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex | 16-2i |  | Yes |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 16-2n | Two default beams for m-DCI | Two default beams for multi-DCI based multi-PDSCH (lowest CORESET-ID per CORESETPoolIndex) | 16-2i, 16-2k |  | Yes |  | N/A |  | Per band | TDD only | FR2 only |  | Note: Applicable to FR2 only | Optional with capability signalling |
|  | 16-2o | Out-of-order for m-DCI | Out of order operation for multi-DCI based multi-PDSCH | 16-2i |  | Yes |  | N/A |  | Per UE | No | No |  |  | Optional with capability signalling |
|  | 16-3 | eType-II codebook | Regular eType-II - 8 parameter combinations - 2 PMI FD resolutions (R=1 and 2) - Rank 1 to 4 - CBSR restriction - List of {R, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} | TBD |  | Yes |  | N/A |  | Per band and separately per BC | N/A | N/A |  | Component-1 candidate value set: {(1, 2, 3, 4, 5, 6), (1, 2, 3, …, 7, 8)} Component-2 candidate value set: {(R=1), (R=1, 2)} Component-3 candidate value set: {(RI=1, 2), (RI=1, 2, 3, 4)} Component-4 candidate value set: {‘no soft amplitude restriction’, ‘support soft amplitude restriction’} | Optional with capability signalling |
|  | 16-4 | eType-II codebook with port selection | Port selection eType-II - 6 parameter combinations (combos with L=6 don’t apply) - 2 PMI FD resolutions (R=1 and 2) - Rank 1 to 4 - List of {R, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} | TBD |  | Yes |  | N/A |  | Per band and separately per BC | N/A | N/A |  | Component-2 candidate value set: {(R=1), (R=1, 2)} Component-3 candidate value set: {(RI=1, 2), (RI=1, 2, 3, 4)} | Optional with capability signalling |
|  | 16-4a | Concurrent codebook operation | Indicates joint capability when multiple codebook types are configured - List of {List of Codebook types, R, Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} |  |  | Yes |  | N/A |  | Per band and separately per BC | N/A | N/A |  |  | Optional with capability signalling |
|  | 16-5 | Power-efficient DL transmission | Low PAPR DMRS for PDSCH |  |  | Yes |  | N/A |  | Per UE | No | Yes |  |  | Optional with capability signalling |
|  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |
|  | 16-6a | Low PAPR DM-RS for PUSCH | Low PAPR DMRS for PUSCH - PUSCH without transform precoding - PUSCH with transform precoding and with pi/2 BPSK modulation |  |  | Yes |  | N/A | Yes | Per band | N/A | N/A |  | Component-1 candidate value set: {‘supported’, ‘not supported’} Component-2 candidate value set: {‘supported’, ‘not supported’} | Optional with capability signalling |
|  | 16-6b | Low PAPR DM-RS for PUCCH | Low PAPR DMRS for PUCCH - PUCCH with transform precoding and with pi/2 BPSK modulation |  |  | Yes |  | N/A | Yes | Per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 16-7 | UL full power transmission mode 0 | - UL full power transmission mode 0 for 2Tx - UL full power transmission mode 0 for 4Tx | 2-13, 2-14 |  | Yes |  | N/A | Yes | FS | N/A | N/A |  | Component-1 candidate value set: {‘supported’, ‘not supported’} Component-2 candidate value set: {‘supported’, ‘not supported’} | Optional with capability signalling |
|  | 16-8 | UL full power transmission mode 1 | - UL full power transmission mode 1 for 2Tx - New UL codebook set(s) for 2Tx - UL full power transmission mode 1 for 4Tx - New UL codebook set(s) for 4Tx | 2-13, 2-14 |  | Yes |  | N/A | Yes | FS | N/A | N/A |  | Component-1 candidate value set: {‘supported’, ‘not supported’} Component-2 candidate value set: {‘supported’, ‘not supported’} | Optional with capability signalling |
|  | 16-9 | UL full power transmission mode 2 | - UL full power transmission mode 2 for 2Tx - UL full power transmission mode 2 for 4Tx | 2-13, 2-14 |  | Yes |  | N/A | Yes | FS | N/A | N/A |  | Component-1 candidate value set: {‘supported’, ‘not supported’} Component-2 candidate value set: {‘supported’, ‘not supported’} | Optional with capability signalling |
|  | 16-10 | Indication of TPMIs with full power capability | UE indication of TPMI(s) per BWP which delivers full power - TPMIs for 2Tx - TPMIs for 4Tx non-coherent - TPMIs for 4Tx partially coherent | 16-9 |  | Yes |  | N/A | Yes | FS | N/A | N/A |  | Component-1: 2-bit bitmap, b0, b1 Component-2: 2-bit: {G0, G1, …, G3} Component-3: 4-bit: {G0, G1, …, G6} | Optional with capability signalling |
|  | 16-11 | Multiple SRS resources in set with different number of ports for full power Tx | - Support multiple SRS resources in SRS resource set with different number of ports for 2Tx - Maximum number of SRS resources in an SRS resource set for 2Tx - Support multiple SRS resources in set with different number of ports for 4Tx - Maximum number of SRS resources in an SRS resource set for 4Tx | 16-9 |  | Yes |  | N/A | Yes | FS | N/A | N/A |  | Component-1 candidate value set: {‘supported’, ‘not supported’} Component-2 candidate value set: {2, 3, 4} Component-3 candidate value set: {‘supported’, ‘not supported’} Component-4 candidate value set: {2, 3, 4} | Optional with capability signalling |
| 17. NR_CLI_RIM | 17-1 | CLI measurement SRS-RSRP | CLI measurement SRS-RSRP - Same SCS as active BWP - The max number of SSB/CSI-RS (1Tx) (sum of aperiodic/periodic/semi-persistent) and CLI SRS-RSRP resources across all CCs configured to measure L1-RSRP 'CRI/RSRP', 'SSBRI/RSRP' and ‘CLI SRS-RSRP’ within a slot shall not exceed MB_1 - The max number of SRS ports per SRS resource for CLI measurement |  |  | Yes |  | N/A |  | Per band | TDD only | N/A |  | Component-2 candidate value set for MB_1 is {0, 8, 16, 32, 64} Component-3 candidate value set: {1, 2, 4} [Note: Already agreed in RAN2, except limit on number on SRS and ports] | Optional with capability signalling |
|  | 17-2 | CLI measurement CLI-RSSI | CLI measurement CLI-RSSI - RSSI SCS is same as active BWP SCS - The max number of resources (sum of aperiodic/periodic) across all CCs configured to measure RSSI simultaneously shall not exceed MC_1 |  |  | Yes |  | N/A |  | Per band | TDD only | N/A |  | Component-2, candidate value set for MC_1 is {0, 1, … 4} [Note: Already agreed in RAN2] | Optional with capability signalling |
|  | 17-3 | Simultaneous reception of PDSCH and CLI-RSSI measurement resource | [simultaneousRxPDSCHCLIRSSIMeasResource] | 17-1 |  | Yes |  | N/A |  | Per UE | TDD only | Yes |  | [UE can assume that PDSCH is FDMed with CLI-RSSI measurement resource(s)] [Note: Already agreed in RAN2] | Optional with capability signalling |
|  | 17-4 | Simultaneous reception of PDSCH and SRS-RSRP measurement resource | [simultaneousRxPDSCHSRSRSRPMeasResource] | 17-1 |  | Yes |  | N/A |  | Per UE | TDD only | Yes |  | [UE can assume that PDSCH is FDMed with SRS RSRP measurement resource(s).] [Note: Already agreed in RAN2] | Optional with capability signalling |
| 18. MR-DC/CA enhancement | 18-1 | UL power sharing for DC | UL power sharing modes supported by the UE - Supported modes |  |  | Yes |  | N/A |  | Per band combination | No | Yes |  | Absence means DC not supported. UE supporting DC must indicate support for at least 1 UL power sharing scheme. Indication for support of any combination of the schemes needs to be possible. Component-1: 4-bit bitmap = { b0: semi-static without duplex direction check in other CG, b1: semi-static with duplex direction check in other CG, b2: dynamic without look-ahead, b3: dynamic with look-ahead} | Optional with capability signalling |
|  | 18-1a | Non-SFN synchronous NR DC operation | Operation with non-zero slot offset between MCG and SCG |  |  | Yes |  | N/A |  | Per band combination | N/A | N/A |  |  | Optional with capability signalling |
|  | 18-1b | Asynchronous NR DC operation | Operation with no slot alignment between MCG and SCG |  |  | Yes |  | N/A |  | Per band combination | N/A | N/A |  |  | Optional with capability signalling |
|  | 18-2a | 1Tx operation for TDD intra-band EN-DC | 1Tx operation for TDD intra-band EN-DC LTE PRACH transmission outside of the restricted TDM subframes |  |  | Yes |  | N/A |  | Per band combination | Applicable to TDD intra-band EN-DC only | N/A |  | Extension of the R15 capability singleUL-Transmission to intra-band TDD EN-DC | Optional with capability signalling |
|  | 18-2b | HARQ subframe offset for 1Tx operation for TDD intra-band EN-DC | HARQ subframe offset for 1Tx operation for TDD intra-band EN-DC | 18-12a |  | Yes |  | N/A |  | Per UE | Applicable to TDD intra-band EN-DC only | N/A |  | Extension of the R15 capability singleUL-Transmission to intra-band TDD EN-DC | Optional with capability signalling |
|  | 18-3a | TDM restriction to an FD-LTE cell in EN-DC (TDM pattern for 2Tx UE) | TDM restriction to an FD-LTE cell in EN-DC Rel-16 TDM pattern HARQ subframe offset Simultaneous LTE and NR transmission LTE PRACH transmission outside of the TDM restricted subframes LTE PUSCH transmission outside of the TDM restricted subframes |  |  | Yes |  | N/A |  | Per band combination | Applicable in EN-DC with LTE FDD only | N/A |  | Extension of the R15 capability tdm-Pattern to a 2Tx UE: Indicates that the UE supports restricting the TD-LTE uplink transmission occasions with a TDM pattern. | Optional with capability signalling |
|  | 18-3b | PRACH transmission without restriction with 1Tx operation in LTE FDD | LTE PRACH transmission outside of the TDM restricted subframes with LTE FDD Rel-16 TDM pattern | 6-13 |  | Yes |  | N/A |  | Per band combination | Applicable in EN-DC with LTE FDD only | N/A |  | Extension of the R15 TDM-Pattern | Optional with capability signalling |
|  | 18-3c | PUSCH transmission without restriction with 1Tx operation in LTE FDD | LTE PUSCH transmission outside of the TDM restricted subframes in LTE FDD Rel-16 TDM pattern | 6-13 |  | Yes |  | N/A |  | Per band combination | Applicable in EN-DC with LTE FDD only | N/A |  | Extension of the R15 TDM-Pattern | Optional with capability signalling |
|  | 18-3d | PUSCH transmission without restriction with 1Tx operation in LTE TDD | LTE PUSCH transmission outside of the TDM restricted subframes in LTE TDD | 18-12a |  | Yes |  | N/A |  | Per band combination | Applicable in EN-DC with LTE TDD only | N/A |  | Extension of the R15 TDM-Pattern | Optional with capability signalling |
|  | 18-3e | Transmission of semi-statically configured signals without restriction in LTE FDD | Transmission of semi-statically configured signals, other than PRACH, outside of the TDM restricted subframes in LTE FDD Rel-16 TDM pattern | 6-13 |  | Yes |  | N/A |  | Per band combination | Applicable in EN-DC with LTE FDD only | N/A |  | Extension of the R15 TDM-Pattern | Optional with capability signalling |
|  | 18-3f | Transmission of semi-statically configured signals without restriction in LTE TDD | Transmission of semi-statically configured signals, other than PRACH, outside of the TDM restricted subframes in LTE TDD | 18-12a |  | Yes |  | N/A |  | Per band combination | Applicable in EN-DC with LTE TDD only | N/A |  | Extension of the R15 TDM-Pattern | Optional with capability signalling |
|  | 18-4 | SCell dormancy with L1 indication | SCell dormancy with L1 indication |  |  | Yes |  | N/A |  | Per band combination | N/A | N/A |  | Support L1-based mechanism to indicate dormancy/non-dormancy status for group of Scells | Optional with capability signalling |
|  | 18-5 | Cross-carrier scheduling with different SCS | Cross-carrier scheduling with different SCS |  |  | Yes |  | N/A |  | Per band pair per band combination | N/A | N/A |  | crossCarrierScheduling-OtherSCS Indicates whether the UE supports cross carrier scheduling for the different numerologies with carrier indicator field (CIF) in DL carrier aggregation where numerologies for the scheduling cell and scheduled cell are different. {Scheduling cell of lower SCS and scheduled cell of higher SCS, Scheduling cell of higher SCS and scheduled cell of lower SCS, both} Note: this capability exists in Rel-15 38.306, but the functionality was not completed in Rel-15. | Optional with capability signalling |
|  | 18-6a | Cross-carrier A-CSI RS triggering from lower SCS | Cross-carrier A-CSI RS triggering from lower to higher SCS |  |  | Yes |  | N/A |  | Per band combination | No | No |  | Indicates whether the UE supports triggering of aperiodic CSI-RS where the CSI-RS is on a carrier with one sub-carrier spacing and the triggering PDCCH is on another carrier with a different sub-carrier spacing compared to the carrier with CSI-RS. | Optional with capability signalling |
|  | 18-6b | Cross-carrier A-CSI RS triggering from higher SCS | Cross-carrier A-CSI RS triggering higher to lower SCS |  |  | Yes |  | N/A |  | Per band combination | No | No |  | Indicates whether the UE supports triggering of aperiodic CSI-RS where the CSI-RS is on a carrier with one sub-carrier spacing and the triggering PDCCH is on another carrier with a different sub-carrier spacing compared to the carrier with CSI-RS. | Optional with capability signalling |
|  | 18-7 | CA with non-aligned frame boundaries | CA with non-aligned frame boundaries |  |  | Yes |  | N/A |  | Per band pair per bandcombination | N/A | N/A |  | Defines whether the UE supports carrier aggregation operation where the frame boundaries of the PCell and the SCell are not aligned, while the slot boundaries are. | Optional with capability signalling |
|  | 18-8 | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group | HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group |  |  | Yes |  | N/A |  | Per UE | No | No |  | Support HARQ-ACK codebook type and HARQ-ACK spatial bundling configuration per PUCCH group. Rel-15 had this per cell group | Optional with capability signalling |
|  | 18-9 | Default QCL assumption for cross-carrier scheduling with same SCS | Default QCL assumption for cross-carrier scheduling with same SCS Indicates whether the UE supports the case that TCI-state is absent in the cross-carrier scheduling DCI or the case that QCL-TypeD is contained by any configured TCI-state and the time offset between the cross-carrier scheduling PDCCH and scheduled PDSCH is smaller than the beam switching timing threshold. |  |  | Yes |  | N/A |  | FS | N/A | N/A |  |  | Optional with capability signalling |
|  | 18-10 | Default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS | Default QCL assumption for cross-carrier A-CSI-RS triggering with same SCS Indicates whether the UE supports the case that CORESET is not configured in the active BWP of cell if the triggering time offset is smaller than the beam switching timing threshold.and there is no any other DL signal with an indicated TCI state in the same symbols as the CSI-RS |  |  | Yes |  | N/A |  | Per band combination | No | No |  |  | Optional with capability signalling |
| 19.UE Power Saving | 19-1 | PDCCH-based power saving channel | - PS_offset for the detection of DCI format 2_6 with CRC scrambling by PS-RNTI before DRX ON duration - DCI format 2_6 - Indication of UE wakeup or not - Configured behaviour for non-detected PS-RNTI DCI - Indication of UE dormancy-like behavior - The minimum time gap between the end of the slot of last DCI format 2_6 monitoring occasion and the start of the DRX ON period is Xi slots where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | Support of dormancy is pre-requisite for Component-5 |  | Yes |  | N/A |  | Per band combination | No | Yes |  | PS_offset is determined based on UE capability Component-6, candidate value set is Xi = {1, 2, 3, ... [z]} slots | Optional with capability signalling |
|  | 19-2 | Cross Slot Scheduling | - Enhancement of DCI format 0_1 and 1_1 (2) DCI Indication of minimum scheduling offset |  |  |  | Yes | N/A |  | FS | N/A | N/A | Differentiation from the perspective of scheduled cell |  | Optional with capability signalling |
|  | 19-3a | Maximum MIMO Layer Adaptation DL | - Maximum DL MIMO layer per DL BWP |  |  |  | Yes | N/A |  | Per band | N/A | N/A | Differentiation from the perspective of PDSCH cell |  | Optional with capability signalling |
|  | 19-3b | Maximum MIMO Layer Adaptation UL | - Maximum UL MIMO layer per UL BWP |  | 2-13 or 2-14 |  | Yes | N/A |  | Per band | N/A | N/A | Differentiation from the perspective of PUSCH cell |  | Optional with capability signalling |
|  | 19-4 | UE assistance information | - Preferred c-DRX configuration - Preferred aggregated bandwidth - Preferred SCell configuration - Preferred MIMO configuration - Release Request - Preferred minimum K0 - Preferred minimum K2 |  |  |  | Yes | N/A |  | Per band | N/A | N/A |  | [Can be transferred to RAN2] Component-6, candidate value set is K0= {1, 2, 4, 6} slots for 15kHz {1, 2, 4, 6} slots for 30kHz {2, 4, 8, 12} slots for 60Hz {2, 4, 8, 12} slots for 120kHz Component-7, candidate value set is K2= {1, 2, 4, 6} slots for 15kHz {1, 2, 4, 6} slots for 30kHz {2, 4, 8, 12} slots for 60Hz {2, 4, 8, 12} slots for 120kHz | Optional with capability signalling |
|  | 19-5 | RRM measurement relaxation | - Relaxation of intra and inter-frequency RRM measurements of neighboring cells in RRC_IDLE/Inactive |  |  |  | [Yes] | N/A |  | [Per UE] | No | Yes | Differentiation from the perspective of frequency layer measured | [Can be transferred to RAN2] | Optional with capability signalling |
| 20. NR_IAB | 20-1 | Inter-IAB-node discovery and measurements: SSB transmission Configuration | Support up to 4 STCs configured for an IAB node DU per cell per SSB frequency location, including IAB-specific SSB transmission periodicities |  |  |  | Yes | N/A | Separate configuration of SSB transmissions for access UE and IAB node measurements is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Optional with capability signalling. |
|  | 20-2 | Inter-IAB-node discovery and measurements: SSB reception configuration | Support up to 4 SMTCs configured for an IAB node MT per SSB frequency location, including IAB-specific SMTC window periodicities |  |  |  | Yes | N/A | Separate configuration of SMTC windows for Inter-IAB node discovery and measurement is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
|  | 20-3 | Extension of RACH occasions and periodicities for backhaul RACH resources | Support RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  |  |  | Yes | N/A | Separate configuration of RACH transmissions/receptions for access UEs and IAB nodes is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
|  | 20-3b | Extension of RACH occasions and periodicities for backhaul RACH resources | Support reception of RACH configuration separately from the RACH configuration for UE access, including new IAB-specific offset and scaling factors |  |  |  | Yes | N/A | Separate configuration of RACH transmissions/receptions for access UEs and IAB nodes is not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Optional with capability signalling. |
|  | 20-5a | MT UL-Flexible-DL slot formats | MT support of semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources |  |  |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
|  | 20-5b | DU UL-Flexible-DL slot formats | DU support of semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources |  |  |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Optional with capability signalling |
|  | 20-6a | MT dynamic indication of soft resource availability | MT support of DCI Format 2_5 based indication of soft resource availability to an IAB node |  |  |  | Yes | N/A | Explicit indication of soft resource availability is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling. |
|  | 20-6b | DU dynamic indication of soft resource availability | DU support of DCI Format 2_5 based indication of soft resource availability to an IAB node |  |  |  | Yes | N/A | Explicit indication of soft resource availability is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Optional with capability signalling |
|  | 20-7 | Case 1 OTA timing alignment | Support IAB-DU DL timing alignment with parent node based on NTA+NTA,offset⋅Tc/2+Tdelta |  |  |  | Yes | N/A | Case-1 OTA timing alignment is not supported | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling. |
|  | 20-7a | Indication of T_delta | Support transmission of T_delta to children IAB nodes. |  |  |  | Yes | N/A | Case-1 OTA timing synchronization at children nodes not possible | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Mandatory |
|  | 20-8 | DU resource configuration | DU support of resource configuration (Hard/Soft/Unavailable and D/U/F). |  |  |  | Yes | N/A | IAB-DU cannot operate | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Mandatory |
|  | 20-9 | DU configuration of IAB child node’s DU resource configuration information | Support for DU configuration of the IAB child node’s DU resource configuration information |  |  |  | Yes | N/A | IAB-DU cannot optimize scheduling and proactively address half-duplex conflicts at child nodes | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Optional with capability signalling |
|  | 20-10 | MT support for guard symbols for MT to DU and DU to MT transitions | MT support for request of guard symbols and for indication of provided guard symbols for MT to DU and DU to MT transitions |  |  |  | Yes | N/A | IAB-node may not receive optimum guard symbols allocation by parent node | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling |
|  | 20-11 | DU support for guard symbols for MT to DU and DU to MT transitions | DU support for request of guard symbols and for indication of provided guard symbols for MT to DU and DU to MT transitions |  |  |  | Yes | N/A | IAB-node may not be able to help child IAB-nodes in dealing with symbol resource conflicts for MT to DU and DU to MT transitions | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact, IAB-MT impact | Optional with capability signalling |
|  | 20-12 | DU configuration of IAB child node’s cell specific signals/channels configuration information | Support for DU configuration of IAB child node’s cell specific signals/channels configuration information |  |  |  | Yes | N/A | IAB-DU cannot proactively address half-duplex conflicts at child nodes in presence of unavailable DU resources used by the DU because they correspond to cell specific signals/channels. | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-DU impact | Optional with capability signalling |
| [21. Mobility Enhancement] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

# Appendix: Layer-1 UE feature list in TR38.822 v1.1.0 [9]

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 [2] | Parent IE in TS 38.331 [2] | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0. Waveform, modulation, subcarrier spacings, and CP | 0-1 | CP-OFDM waveform for DL and UL | 1) CP-OFDM for DL 2) CP -OFDM for UL |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-2 | DFT-S-OFDM waveform for UL | Transform precoding for single-layer PUSCH |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-3 | DL modulation scheme | 1) QPSK modulation 2) 16QAM modulation 3) 64QAM modulation for FR1 |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-4 | UL modulation scheme | 1) QPSK modulation 2) 16QAM modulation |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 0-5 | Extended CP | Extended CP | 1-1 in Table 4.3-1 | extendedCP | BandNR | n/a | n/a |  | Optional with capability signalling |
| 1. Initial access and mobility | 1-1 | Basic initial access channels and procedures | 1) RACH preamble format 2) SS block based RRM measurement 3) Broadcast SIB reception including RMSI/OSI and paging |  | n/a | n/a | No | No | Broadcast SIB reception including RMSI/OSI and paging are components of basic initial access channels and procedures for NR standalone and NE-DC | Mandatory without capability signalling |
|  | 1-2 | SS block based SINR measurement (SS-SINR) | SS-SINR measurement | 1-1 | ss-SINR-Meas | MeasAndMobParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 1-3 | SS block based RLM | SS block based RLM | 1-1 | ssb-RLM | MeasAndMobParametersCommon | No | No |  | Mandatory with capability signalling which shall be set to ‘1’ |
|  | 1-4 | CSI-RS based RRM measurement with associated SS-block | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithSSB | MeasAndMobParametersFRX-Diff | No | Yes | This does not discourage RAN4 to complete their work. There is expectation that RAN4 will complete the corresponding RRM measurement | Optional with capability signalling |
|  | 1-5 | CSI-RS based RRM measurement without associated SS-block | 1) CSI-RSRP measurement 2) CSI-RSRQ measurement 3) There is SS-block in the target frequency on which the RRM measurement is performed | 1-1, CSI-RS | csi-RSRP-AndRSRQ-MeasWithoutSSB | MeasAndMobParametersFRX-Diff | No | Yes | This does not discourage RAN4 to complete their work. There is expectation that RAN4 will complete the corresponding RRM measurement | Optional with capability signalling |
|  | 1-6 | CSI-RS based RS-SINR measurement | CSI-SINR measurements | 1-1, 1-4 | csi-SINR-Meas | MeasAndMobParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 1-7 | CSI-RS based RLM | CSI-RS based RLM | 1-1, CSI-RS | csi-RS-RLM | MeasAndMobParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 1-8 | RLM based on a mix of SS block and CSI-RS signals within active BWP | RLM based on a mix of SS block and CSI-RS signals within active BWP | 1-3 and 1-7 | ssb-AndCSI-RS-RLM | MeasAndMobParametersCommon | No | No |  | Optional with capability signalling |
|  | 1-9 | CSI-RS based contention free RA for HO | CSI-RS based contention free RA for HO | 1-1, CSI-RS, 1-4 or 1-5 | csi-RS-CFRA-ForHO | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 1-10 | Support of SCell without SS/PBCH block | Support SCell without SS/PBCH block | 1-1 | scellWithoutSSB | FeatureSetDownlink | n/a | n/a | Component 1) Whether or not UE is able to use SS/PBCH block from other Cells for time/frequency synchronization of SCell without SS/PBCH block | Mandatory with capability signalling for intra-band CA This feature is not supported for inter band CA |
|  | 1-11 | Support of CSI-RS RRM measurement for SCell without SS/PBCH block |  | 1-10 | csi-RS-MeasSCellWithoutSSB | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 1-12 | E-UTRA RS-SINR measurement |  |  | rs-SINR-MeasEUTRA | EUTRA-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 1-13 | Maximal number of CSI-RS resources for RRM and RS-SINR measurement across all measurement frequencies per slot |  | 1-4 or 1-5 or 1-6 | maxNumberCSI-RS-RRM-RS-SINR | MeasAndMobParametersCommon | No | No | If UE supports any of 1-5, 1-5a, and 1-6, UE shall report this capability 1-13 | Candidate value set: {4,8,16,32,64, 96} |
|  | 1-14 | Maximal number of CSI-RS resources within a slot per PCell/PSCell for CSI-RS based RLM |  | 1-7 or 1-8 | maxNumberResource-CSI-RS-RLM | MeasAndMobParametersFRX-Diff | No | Yes | If UE supports any of 1-7 and 1-8, UE shall report this capability 1-14 | Candidate value set: {2,4, 6, 8} |
| 2. MIMO | 2-1 | Basic PDSCH reception | 1) Data RE mapping 2) Single layer transmission 3) Support one TCI state |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 2-2 | PDSCH beam switching | 1) Time duration (definition follows section 5.1.5 in TS 38.214), Xi, to determine and apply spatial QCL information for corresponding PDSCH reception. Time duration is defined counting from end of last symbol of PDCCH to beginning of the first symbol of PDSCH. Xi is the number of OFDM symbols, i is the index of SCS, l=1,2, corresponding to 60,120 kHz SCS. | 2-1 | timeDurationForQCL | FeatureSetDownlink | No | Applicable only to FR2 |  | Mandatory with capability signalling for FR2 Candidate value set for X1 is {7, 14, 28}, Candidate value set for X2, {14, 28} |
|  | 2-3 | PDSCH MIMO layers | Supported maximal number of MIMO layers | 2-1 | maxNumberMIMO-LayersPDSCH | FeatureSetDownlinkPerCC | n/a | n/a |  | For single CC standalone NR, it is mandatory with capability signalling to support at least 4 MIMO layers in the bands where 4Rx is specified as mandatory for the given UE and at least 2 MIMO layers in FR2. Some relaxations to this requirement may be applicable in the future (including in Rel-15). Mandatory in all cases means mandatory with capability signalling. It is not expected that there is a signalling change (i.e. signalling remains to be defined as {1, 2, 4, 8} in every band and every band combination, including FR1 and FR2 in all cases. |
|  | 2-4 | TCI states for PDSCH | 1) Support number of active TCI states per BWP per CC, including control and data 2) Maximum number of configured TCI states per CC for PDSCH | 2-1 | tci-StatePDSCH { 1. maxNumberActiveTCI-PerBWP 2. maxNumberConfiguredTCIstatesPerCC } | MIMO-ParametersPerBand | n/a | n/a | UE is required to track only the active TCI states For component 1 of FG2-4, if a UE reports X active TCI state(s), it is not expected that more than X active QCL type D assumption(s) for any PDSCH and any CORESETs for a given BWP of a serving cell become active for the UE. | Mandatory with capability signalling Component-1: Candidate value set: {1, 2, 4, 8} Component-2: candidate value set: {4, 8, 16, 32, 64, 128} UE is mandated to signal 64 for FR2. For FR1, UE is mandated to report at least the max number of allowed SSB in the band. |
|  | 2-4a | Additional active TCI state for PDCCH | Support one additional active TCI state for control in addition to the supported number of active TCI states for PDSCH | 2-1 | additionalActiveTCI-StatePDCCH | MIMO-ParametersPerBand | n/a | n/a | Only applicable if Component-1 of 2-4 is set to 1 | Mandatory with capability signalling |
|  | 2-5 | Basic downlink DMRS for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols for at least one port. | 2-1 | n/a | n/a | n/a | n/a | conditioned to whether PDSCH scheduling type A is supported | Mandatory without capability signalling (condition to scheduling capability) |
|  | 2-6 | Basic downlink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  | n/a | n/a | n/a | n/a | conditioned to whether PDSCH scheduling type B is supported | Mandatory without capability signalling (condition to scheduling capability) |
|  | 2-6a | Support 1+2 DMRS (downlink) | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port | 2-5 | oneFL-DMRS-TwoAdditionalDMRS-DL | FeatureSetDownlink-v1540 | No | Yes |  | Mandatory with capability signalling |
|  | 2-6b | Support alternative additional DMRS location | Support alternative additional DMRS position for co-existence with LTE CRS | 2-5 and 5-28 | additionalDMRS-DL-Alt | FeatureSetDownlink-v1540 | No | n/a | This FG applies to FR1 only and 15kHz SCS. This applies to one additional DMRS case only | Optional with capability signalling |
|  | 2-7 | Supported 2 symbols front-loaded DMRS (downlink) | Support 2 symbols FL-DMRS | 2-5 | twoFL-DMRS (MSB) | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-8 | Supported 2 symbols front-loaded +2 symbols additional DMRS (downlink) | Support 2-symbol FL DMRS + one additional 2-symbols DMRS | 2-5 | twoFL-DMRS-TwoAdditionalDMRS-DL | FeatureSetDownlink-v1540 | No | Yes |  | Optional with capability signalling |
|  | 2-9 | Support 1+3 DMRS symbols(downlink) | Support 1 symbol FL DMRS and 3 additional DMRS symbols | 2-5 | oneFL-DMRS-ThreeAdditionalDMRS-DL | FeatureSetDownlink-v1540 | No | Yes |  | Optional with capability signalling |
|  | 2-10 | Support DMRS type (downlink) | Support DMRS {type 1, both type 1 and type 2} |  | supportedDMRS-TypeDL | Phy-ParametersFRX-Diff | No | Yes |  | Type 1 is mandatory with capability signalling. Type 2 is optional with capability signalling |
|  | 2-11 | Downlink dynamic PRB bundling (downlink) | Support dynamic PRB bundling indication via DCI | 2-1 | dynamicPRB-BundlingDL | Phy-ParametersCommon | No | No | Support of semi-static PRB bundling is mandatory | Optional with capability signalling |
|  | 2-12 | Basic PUSCH transmission | Data RE mapping Single layer (single Tx) transmission Single port, single resource SRS transmission (SRS set use is configured as for codebook) |  | n/a | n/a | n/a | n/a | Support of SRS set usage configured as for codebook does not imply UE support of codebook based PUSCH MIMO transmission. | Mandatory without capability signalling |
|  | 2-13 | PUSCH codebook coherency subset | Supported codebook coherency subset type | 2-12 | pusch-TransCoherence | MIMO-ParametersPerBand | n/a | n/a |  | Optional with UE capability Candidate value set: {non-coherent, partial/non-coherent, full/partial/non-coherent} |
|  | 2-14 | Codebook based PUSCH MIMO transmission | 1) Supported codebook based PUSCH MIMO with maximal number of supported layers 2) Supported max number of SRS resource per set (SRS set use is configured as for codebook). | 2-13 | mimo-CB-PUSCH { 1. maxNumberMIMO-LayersCB-PUSCH 2. maxNumberSRS-ResourcePerSet } | FeatureSetUplinkPerCC | n/a | n/a | For SUL, uplink MIMO is not supported. | Optional with UE capability Component-1: Candidate value: {no-codebook based MIMO, 1, 2, 4} Component-2 Candidate value: {1, 2} |
|  | 2-15 | non-codebook based PUSCH transmission | 1) Maximal number of supported layers (non-codebook transmission scheme) 2) Supported max number of SRS resource per set (SRS set use is configured as for non-codebook transmission). 3) Maximum number of simultaneous transmitted SRS resources at one symbol | 2-12 | 1. maxNumberMIMO-LayersNonCB-PUSCH mimo-NonCB-PUSCH { 2. maxNumberSRS-ResourcePerSet 3. maxNumberSimultaneousSRS-ResourceTx } | 1. FeatureSetUplinkPerCC 2, 3. FeatureSetUplinkPerCC-v1540 | n/a | n/a | For SUL, uplink MIMO is not supported | Optional with UE capability Component-1 candidate values: {1, 2, 4} Component-2 Candidate value: {1,2,3,4} Component-3 Candidate value: {1,2,3,4} |
|  | 2-15a | Association between CSI-RS and SRS | 1) Support association between NZP-CSI-RS and SRS resource set via RRC parameter “SRSresoureset” 2) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. | 2-15 | srs-AssocCSI-RS SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 2.1. maxNumberTxPortsPerResource 2.2. maxNumberResourcesPerBand 2.3. totalNumberTxPortsPerBand } | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling Component-2: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports is: {from 2 to 256} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 2.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 2.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-15b | CSI-RS processing framework for SRS | 1) Maximum number of periodic SRS resources associated with CSI-RS per BWP 2) Maximum number of aperiodic SRS resources associated with CSI-RS per BWP 3) Maximum number of semi-persistent SRS resources associated with CSI-RS per BWP 4) UE can process Y SRS resources associated with CSI-RS resources simultaneously in a CC. Includes P/SP/A SRS. 5) UE can process X SRS resources associated with CSI-RS resources simultaneously across all CCs. Includes P/SP/A SRS. | 2-15a | csi-RS-ProcFrameworkForSRS { 1. maxNumberPeriodicSRS-AssocCSI-RS-PerBWP 2. maxNumberAperiodicSRS-AssocCSI-RS-PerBWP 3. maxNumberSP-SRS-AssocCSI-RS-PerBWP 4. simultaneousSRS-AssocCSI-RS-PerCC } | MIMO-ParametersPerBand Phy-ParametersFRX-Diff (for FR1 + FR2 band combination) | n/a | n/a | Other MIMO capabilities than component 5 may further restrict (reduce) the number of SRS associated with CSI-RS that the UE has to simultaneously derive. | Optional with capability signalling Component-1 candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-3 candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |
|  |  |  |  |  | 5. simultaneousSRS-AssocCSI-RS-AllCC | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-16 | Basic uplink DMRS (uplink) for scheduling type A | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbols 3) Support 1 symbol FL DMRS and 2 additional DMRS symbols |  | n/a | n/a | n/a | n/a | Conditioned to whether PUSCH scheduling type A is supported | Mandatory without capability signalling |
|  | 2-16a | Basic uplink DMRS for scheduling type B | 1) Support 1 symbol FL DMRS without additional symbol(s) 2) Support 1 symbol FL DMRS and 1 additional DMRS symbol |  | n/a | n/a | n/a | n/a | conditioned to whether PUSCH scheduling type B is supported | Mandatory without capability signalling |
|  | 2-16b | Support 1+2 DMRS (uplink) | Support 1 symbol FL DMRS and 2 additional DMRS symbols for more than one port | 2-16a and 2-16 | oneFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-17 | Support DMRS type (uplink) | Support DMRS {type 1, both type 1 and type 2} | 2-16 | supportedDMRS-TypeUL | Phy-ParametersFRX-Diff | No | Yes |  | Support both type 1 and type 2 are mandatory with capability signalling |
|  | 2-18 | Supported 2 symbols front-loaded DMRS (uplink) | Support 2 symbols FL-DMRS | 2-16 | twoFL-DMRS (LSB) | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-18a | Supported 2 symbols front-loaded +2 symbols additional DMRS (uplink) | Support 2-symbol FL DMRS + one additional 2-symbols DMRS | 2-16 | twoFL-DMRS-TwoAdditionalDMRS-UL | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-19 | Support 1+3 uplink DMRS symbols(uplink) | Support 1 symbol FL DMRS and 3 additional DMRS symbols | 2-16 | oneFL-DMRS-ThreeAdditionalDMRS-UL | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-20 | Beam correspondence | Support Beam correspondence |  | beamCorrespondenceWithoutUL-BeamSweeping | MIMO-ParametersPerBand | No | Applicable only to FR2 | Beam correspondence means each Tx port can be beamformed in a desirable direction but does not imply setting phase across ports. | Mandatory with capability signalling - UE that fulfils the beam correspondence requirement without the uplink beam sweeping shall set the bit to 1 - UE that fulfils the beam correspondence requirement with the uplink beam sweeping shall set the bit to 0 |
|  | 2-21 | Periodic beam report | 1) Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot 2) Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot |  | periodicBeamReport | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling for both FR1 and FR2 |
|  | 2-22 | Aperiodic beam report | Support aperiodic report on PUSCH |  | aperiodicBeamReport | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling for both FR1 and FR2 |
|  | 2-23 | Semi-persistent beam report on PUCCH | 1) Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) 2) Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) |  | sp-BeamReportPUCCH | MIMO-ParametersPerBand | n/a | Yes |  | Optional with capability signalling |
|  | 2-23a | Semi-persistent beam report on PUSCH | Support semi-persistent report on PUSCH |  | sp-BeamReportPUSCH | MIMO-ParametersPerBand | n/a | Yes |  | Optional with capability signalling |
|  | 2-24 | SSB/CSI-RS for beam measurement | 1) The max number of SSB/CSI-RS (1Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP within a slot shall not exceed MB_1 2) The max number of CSI-RS resources (sum of aperiodic/periodic/semi-persistent) across all CCs configured to measure L1-RSRP shall not exceed MC_1 3) The max number of CSI-RS (2Tx) resources (sum of aperiodic/periodic/semi-persistent) across all CCs to measure L1-RSRP within a slot shall not exceed MB_2 4) Supported density of CSI-RS 5) The max number of aperiodic CSI-RS resources across all CCs configured to measure L1-RSRP shall not exceed MD_1 | 2-21, 2-22 or 2-23, 2-23a | beamManagementSSB-CSI-RS { 1. maxNumberSSB-CSI-RS-ResourceOneTx 2. maxNumberCSI-RS-Resource 3. maxNumberCSI-RS-ResourceTwoTx 4. supportedCSI-RS-Density 5. maxNumberAperiodicCSI-RS-Resource } | MIMO-ParametersPerBand | No | Yes |  | Mandatory with capability signalling Component-1, candidate value set for MB_1 is {0, 8, 16, 32, 64} On FR2, UE is mandated to signal MB_1 >=8 On FR1, MB_1 >=8 is supported mandatory with capability signalling. Component-2, candidate value set for MC_1 is {0, 4, 8, 16, 32, 64} For FR1, UE is mandated to report at least 8. Component-3, candidate value set for MB_2 is {0, 4, 8, 16, 32, 64} Component-4: candidate value set: {“not supported”, “1 only”, “3 only”, “both 1 and 3”} On FR2, UE is mandated to signal either “3 only” or “both 1 and 3” On FR1, either “3 only” or “both 1 and 3” is mandatory with UE capability signalling. Component-5, candidate value set for MD_2 is {0, 1, 4, 8, 16, 32, 64} For both FR1 and FR2, UE is mandated to report at least 4 |
|  | 2-25 | Beam reporting timing | The number of symbols, Xi, between the last symbol of SSB/CSI-RS and the first symbol of the transmission channel containing beam report is at least RBi, where i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-24 | beamReportTiming | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability Candidate value sets: X1 is {2, 4, 8} X2 is {4, 8, 14, 28} X3 is {8,14, 28} X4 is{14,28, 56} |
|  | 2-26 | Receiving beam selection using CSI-RS resource repetition "ON" | 1. Support Rx beam switching procedure using CSI-RS resource repetition "ON" 2. Recommended CSI-RS resource repetition number per resource set, |  | maxNumberRxBeam | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with UE capability at least for FR2 Componet-2: candidate value set {2, 3, 4, 5, 6, 7, 8} |
|  | 2-27 | Beam switching | Maximum number of Tx + Rx beam changes a UE can conduct during a slot across the whole band CC B_(B_Total,). This number is defined as per SCS | 2-24 | maxNumberRxTxBeamSwitchDL | MIMO-ParametersPerBand | n/a | Applicable only to FR2 | It is assumed that spec enable the possibility to restrict the same beam across intra-band CCs | Optional with capability signalling Candidate value set: {4, 7, 14} |
|  | 2-28 | A-CSI-RS beam switching timing | Minimum time between the DCI triggering of AP-CSI-RS and aperiodic CSI-RS transmission shall be at least KBi symbols. (Symbols measured from last symbol containing the indication to first symbol of CSI-RS), where i is the index of SCS, l=1,2 corresponding to 60,120 kHz SCS. |  | beamSwitchTiming | MIMO-ParametersPerBand | n/a | Applicable only to FR2 |  | Optional with capability signalling Candidate values: {14, 28, 48, 224, 336} |
|  | 2-29 | Non-group based beam reporting | Support of non-group based RSRP reporting with N_max RSRP values reported |  | maxNumberNonGroupBeamReporting | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability for both FR1 and FR2 candidate value set is {1, 2, 4} |
|  | 2-29a | Group based beam reporting | Support of beam group RSRP reporting for group of 2 beams |  | groupBeamReporting | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling |
|  | 2-30 | Uplink beam management | 1) Support of SRS based beam management 2) Supported max number of SRS resource per set (SRS set use is configured as for beam management). 3) Supported max number of SRS resource sets (SRS set use is configured as for beam management). |  | uplinkBeamManagement { 2. maxNumberSRS-ResourcePerSet-BM 3. maxNumberSRS-ResourceSet } | MIMO-ParametersPerBand | n/a | n/a | Component-3 also impose additional constraint on the maximum number of SRS resource sets per supported time domain behaviour (periodic/semi-persistent/aperiodic) as {1,1,1,2,2,2,4,4} corresponding to reported values {from 1 to 8} | Optional with capability signalling - Capability signalling shall be set to 1 if 2-20 is set to 0 - For the UE meeting the minimum peak EIRP and spherical coverage requirements without the uplink beam sweeping, this feature is optional Component-2, candidate value set is {2, 4, 8, 16} Component-3, candidate value set is {from 1 to 8} |
|  | 2-31 | Beam failure recovery | 1) Maximal number of CSI-RS resources across all CCs for UE to monitor PDCCH quality 2) Maximal number of different SSBs across all CCs for UE to monitor PDCCH quality 3) Maximal number of different CSI-RS and/or SSB resources across all CCs for new beam identifications. | 1-7 for CSI-RS based BFD/BFR | 1. maxNumberCSI-RS-BFD 2. maxNumberSSB-BFD 3. maxNumberCSI-RS-SSB-CBD | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 Component-1 candidate value set: {from 1 to 16} Component-2 candidate: {from 1 to 16} Component-3: Candidate value set is: {from 1 to 128} UE is mandated to support at least 32 for FR2 |
|  | 2-32 | Basic CSI feedback | 1) Type I single panel codebook based PMI (further discuss which mode or both to be supported as mandatory) 2) 2Tx codebook for FR1 and FR2 3) 4Tx codebook for FR1 4) 8Tx codebook for FR1 when configured as wideband CSI report 5) p-CSI on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) 6) p-CSI report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) 7) a-CSI on PUSCH (at least Z value >= 14 symbols, detail processing time to be discussed separately) further check a-CSI on p-CSI-RS and/or SP-CSI-RS from component-7 |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 2-32a | Semi-persistent CSI report on PUCCH | 1) Support report on PUCCH formats over 1 – 2 OFDM symbols once per slot (or piggybacked on a PUSCH) s 2) Support report on PUCCH formats over 4 – 14 OFDM symbols once per slot (or piggybacked on a PUSCH) |  | sp-CSI-ReportPUCCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-32b | Semi-persistent CSI report on PUSCH | Support semi-persistent CSI report on PUSCH |  | sp-CSI-ReportPUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-32c | New CQI table | CQI table with target BLER of 10^-5 |  | cqi-TableAlt | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-33 | CSI-RS and CSI-IM reception for CSI feedback | 1) Supported max # of configured NZP-CSI-RS resources per CC, 2) Supported max # of ports across all configured NZP-CSI-RS resources per CC 3) Supported max # of configured CSI-IM resources per CC 4) Supported max # simultaneous NZP-CSI-RS resources in active BWPs across all CCs 5) Supported max # simultaneous NZP-CSI-RS resources per CC 6) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs 7) Supported max total # of CSI-RS ports in simultaneous NZP-CSI-RS resources per CC | 2-32 | csi-RS-IM-ReceptionForFeedback { 1. maxConfigNumberNZP-CSI-RS-PerCC 2. maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC 3. maxConfigNumberCSI-IM-PerCC 5. maxNumberSimultaneousNZP-CSI-RS-PerCC 7. totalNumberPortsSimultaneousNZP-CSI-RS-PerCC } | MIMO-ParametersPerBand Phy-ParametersFRX-Diff (for FR1 + FR2 band combination) | n/a | n/a | All the candidate values are the range of capability signalling which doesn’t determine whether UE is mandatory to support all the signalling values. | Mandatory with capability signalling Component-1 candidate values: {from 1 to 32} Component-2 candidate values: {2, 4, 8, 12, 16, 24, 32, 40, 48 … ,256} Component-3: candidate values: {1,2,4,8,16,32} Component-4: candidate values {5, 6, 7, 8, 9, 10, 12, 14, 16, …, 62, 64} (includes all even numbers between 16 and 64) Component-5: candidate values {1, 2, 3 … 32} Component-6: candidate values {8, 16, 24, …, 248, 256} Component-7: candidate values {8, 16, 24, … 128 } |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 4. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 6. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-33a | Supported PDSCH RE-mapping patterns | 1) Supported max # of RE mapping patterns, each pattern can be described as a resource (including NZP/ZP CSI-RS and CRS, CORESET and SSB and bitmap configured in 5-26/27) Note: patterns are counted as per symbol per CC 2) Supported max # of RE mapping patterns, each pattern can be described as a resource (including NZP/ZP CSI-RS and CRS, CORESET and SSB and bitmap configured in 5-26/27/27a) Note: patterns are counted as per slot per CC |  | 1. pdsch-RE-MappingFR1-PerSymbol 2. pdsch-RE-MappingFR1-PerSlot | Phy-ParametersFR1 | No | Yes |  | Mandatory with capability signalling candidate values: {10, 20} for FR1 {6, 20} for FR2 Compponent-2 candidate values: {from 16: 16: 256} for FR1 {16: 16: 256} for FR2 |
|  |  |  |  |  | 1. pdsch-RE-MappingFR2-PerSymbol 2. pdsch-RE-MappingFR2-PerSlot | Phy-ParametersFR2 |  |  |  |  |
|  | 2-33b | SP CSI-RS | Support SP CSI-RS | 2-1 | sp-CSI-RS | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 2-33c | SP CSI-IM | Support SP CSI-IM | 2-1 | sp-CSI-IM | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-34 | NZP-CSI-RS based interference measurement | Support NZP-CSI-RS based interference measurement | 2-33 | nzp-CSI-RS-IntefMgmt | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-35 | CSI report framework | 1) Maximum number of periodic CSI report setting per BWP for CSI report 2) Maximum number of periodic CSI report setting per BWP for beam report 3) Maximum number of aperiodic CSI report setting per BWP for CSI report 4) Maximum number of aperiodic CSI report setting per BWP for beam report 5) Maximum number of configured aperiodic CSI triggering states in CSI-AperiodicTriggerStateList per CC, 6) Maximum number of semi-persistent CSI report setting per BWP for CSI report 7) Maximum number of semi-persistent CSI report setting per BWP for beam report 8) UE can process Y CSI report(s) simultaneously in a CC. CSI reports can be P/SP/A CSI and any latency class and codebook type. 9) UE can process X CSI report(s) simultaneously across all CCs. CSI reports can be P/SP/A CSI and any latency class and codebook type. | 2-32 | csi-ReportFramework { 1. maxNumberPeriodicCSI-PerBWP-ForCSI-Report 2. maxNumberAperiodicCSI-PerBWP-ForCSI-Report 3. maxNumberSemiPersistentCSI-PerBWP-ForCSI-Report 4. maxNumberPeriodicCSI-PerBWP-ForBeamReport 5. maxNumberAperiodicCSI-PerBWP-ForBeamReport 6. maxNumberAperiodicCSI-triggeringStatePerCC 7. maxNumberSemiPersistentCSI-PerBWP-ForBeamReport 8. simultaneousCSI-ReportsPerCC } | MIMO-ParametersPerBand Phy-ParametersFRX-Diff (for FR1 + FR2 band combination) | n/a | n/a | Other MIMO capabilities than component 5 may further restrict (reduce) the number of simultaneously CSI report that UE is required to update The CSI report in component 4 and 5 includes the beam report and CSI report Each component is independent CSI report setting are counted in the CC indicated by the parameter carrier in CSI-ResourceConfig. | Mandatory with capability signaling Component-1 candidate values: {1, 2, 3, 4} Component-1a candidate values: {1, 2, 3, 4} Component-2 candidate values {1, 2, 3, 4} Component-2a candidate values {1, 2, 3, 4} Component-2b candidate values {3, 7, 15, 31, 63, 128} Component-3 candidate values: {0, 1, 2, 3, 4} Component-3a candidate values: {0, 1, 2, 3, 4} Component-4 candidate values: {from 1 to 8} Component-5: candidate values: {from 5 to 32} |
|  |  |  |  |  | 9. simultaneousCSI-ReportsAllCC | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-36 | Type I single panel codebook | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Supported Codebook Mode(s) 3) Max # of CSI-RS resource in a resource set | 2-35 | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. modes 3. maxNumberCSI-RS-PerResourceSet | CodebookParameters | No | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Mandatory with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {2, 4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: {“Mode-1 only”, “Mode-1 and Mode-2”}. Component-3 Candidate values set: {1:8} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-37 | Support Semi-open loop CSI | Support Semi-open loop CSI report | 2-35 | semiOpenLoopCSI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-38 | CSI report without PMI | Support CSI report without PMI | 2-35 | csi-ReportWithoutPMI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-39a | CSI report without CQI | Support CSI report without CQI | 2-35 | csi-ReportWithoutCQI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 2-40 | Type I multi-panel codebook | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Supported Codebook Mode(s): 3) Supported number of panels, Ng 4) Max # of CSI-RS resource in a resource set | 2-35 | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. modes 3. nrofPanels 4. maxNumberCSI-RS-PerResourceSet | CodebookParameters | n/a | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Optional with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {8, 16, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2 candidate values: {Mode-1, Mode-2, both} Component-3: Candidate value: {2,4} Component-4: candidate value set is {1:8} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-41 | Type II codebook | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Parameter “Lx” (number of beams) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3) Support amplitude scaling type 4) Support amplitude subset restriction level | 2-35 | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. parameterLx 3. amplitudeScalingType 4. amplitudeSubsetRestriction | CodebookParameters | n/a | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Optional with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4, candidate value set: {“no amplitude subset restriction”, “support amplitude subset restriction”} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-42 | Support Type II SP-CSI feedback on long PUCCH | Support type II SP-CSI feedback part-1 on PUCCH formats over 4 – 14 OFDM symbols once per slot | 2-41 | type2-SP-CSI-Feedback-LongPUCCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 2-43 | Type II codebook with port selection | 1) A list of supported combinations, each combination is {Max # of Tx ports in one resource, Max # of resources and total # of Tx ports} across all CCs simultaneously. Note: the above list doesn’t differentiate the latency class and feedback type. 2) Parameter “Lx” (number of selected ports) in codebook generation, where x is index of Tx ports, corresponding to 4,8,12,16,24 and 32 ports. 3) Support amplitude scaling type |  | 1. supportedCSI-RS-ResourceList SEQUENCE (SIZE (1..maxNrofCSI-RS-Resources)) OF { 1.1. maxNumberTxPortsPerResource 1.2. maxNumberResourcesPerBand 1.3. totalNumberTxPortsPerBand } 2. parameterLx 3. amplitudeScalingType | CodebookParameters | n/a | n/a | Simultaneously doesn’t mean in the same slot For the purpose of component-1 calculation: CSI-RS resources and CSI-RS ports within one CSI-RS resource are counted N times if the CSI-RS resource is referred by N report settings. | Optional with capability signalling Component-1: Maximum size of the list is 16. the candidate values for the max # of Tx port in one resource is {4, 8, 12, 16, 24, 32} The candidate value set of the max # of resources is: {from 1 to 64} The candidate value set of total # of ports (including both channel and NZP-CSI-RS based interference measurement) is: {from 2 to 256} Component-2, candidate values set for “Lx” is {2,3,4} Component-3, candidate values set: {wideband, wideband/subband} Component-4: candidate value set is {1:8} |
|  |  |  |  |  | csi-RS-IM-ReceptionForFeedbackPerBandComb { 1.2. maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC 1.3. totalNumberPortsSimultaneousNZP-CSI-RS-ActBWP-AllCC } | CA-ParametersNR-v1540 |  |  |  |  |
|  | 2-44 | Basic DL PTRS | Support 1 port of DL PTRS |  | onePortsPTRS (MSB) | Phy-ParametersFRX-Diff | n/a | Yes |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 |
|  | 2-46 | Downlink PTRS density recommendation | Preferred threshold sets, TSi for determine PTRS density, candidate value range is the same as that of downlink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-44 | ptrs-DensityRecommendationSetDL { 1. frequencyDensity1 2. frequencyDensity2 3. timeDensity1 4. timeDensity2 5. timeDensity3 } | MIMO-ParametersPerBand | n/a | n/a | For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density | Optional with capability signalling |
|  | 2-47 | Basic UL PTRS | Support 1 port of UL PTRS |  | onePortsPTRS (LSB) | Phy-ParametersFRX-Diff | n/a | Yes |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 |
|  | 2-48 | Uplink PTRS | Supported 2 ports of PTRS | 2-47 | twoPortsPTRS-UL | MIMO-ParametersPerBand | n/a | n/a |  | Optional with capability signalling |
|  | 2-49 | Uplink PTRS density recommendation | Preferred threshold sets, TSi, for determine PTRS density, candidate value range is the same as that of uplink PTRS RRC configuration. i is the index of SCS, i=1,2,3,4 corresponding to 15,30,60,120 kHz SCS. | 2-47 | ptrs-DensityRecommendationSetUL { 1. frequencyDensity1 2. frequencyDensity2 3. timeDensity1 4. timeDensity2 5. timeDensity3 6. sampleDensity1 7. sampleDensity2 8. sampleDensity3 9. sampleDensity4 10. sampleDensity5 } | MIMO-ParametersPerBand | n/a | n/a | For each TSi, it composes of two values each selected from {1..276} for frequency density, and three values each selected from {0..29} for time density, and five values each selected from {1..276} for sample density | Optional with capability signalling |
|  | 2-50 | Basic TRS | 1) Support of TRS (mandatory) 2) All the periodicity are supported. 3) Support TRS bandwidth configuration as both “BWP” and “min(52, BWP)” |  | n/a | n/a | n/a | n/a | TRS bandwidth configuration does not imply UE processing bandwidth | Mandatory without capability signalling |
|  | 2-51 | TRS (CSI-RS for tracking) | 1) TRS burst length (X), 2) Max # of TRS resource sets (per CC) UE is able to track simultaneously 3) Max # of TRS resource sets configured to UE per CC 4) Max # of TRS resource sets configured to UE across CCs | 2-50 | csi-RS-ForTracking { 1. maxBurstLength 2. maxSimultaneousResourceSetsPerCC 3. maxConfiguredResourceSetsPerCC 4. maxConfiguredResourceSetsAllCC } | MIMO-ParametersPerBand | n/a | n/a |  | Mandatory with capability signalling Component-1: candidate values {1, ”both 1 and 2”}. UE is mandated to report “both 1 and 2” Component-2: Candidate value set: {1 to 8} Component-3: Candidate value set: {1 to 64} UE is mandated to report at least 8 for FR1 and 16 for FR2. Component-4: Candidate value set: {1 to 256} UE is mandated to report at least 16 for FR1 and 32 for FR2. |
|  | 2-51a | Aperiodic TRS | DCI triggering Aperiodic TRS associated with periodic TRS | 2-50 | aperiodicTRS | MIMO-ParametersPerBand | n/a | Yes |  | Optional with capability signalling |
|  | 2-52 | Basic SRS | 1) Support 1 port SRS transmission 2) Support periodic/aperiodic SRS transmission 3) Support SRS Frequency intra/inter-slot hopping within BWP 4) At least one SRS resource per CC for aperiodic and periodic separately |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 2-53 | SRS resources | 1) Maximum number of aperiodic SRS resources (configured to UE) per BWP 2) Maximum number of aperiodic SRS resources (configured to UE) per BWP per slot 3) Maximum number of periodic SRS resources (configured to UE) per BWP 4) Maximum number of periodic SRS resources (configured to UE) per BWP per slot 5) Maximum number of semi-persistent SRS resources (configured to UE) per BWP 6) Maximum number of semi-persistent SRS resources (configured to UE) per BWP per slot 7) Maximum number of SRS port per resource | 2-52 | supportedSRS-Resources { 1. maxNumberAperiodicSRS-PerBWP 2. maxNumberAperiodicSRS-PerBWP-PerSlot 3. maxNumberPeriodicSRS-PerBWP 4. maxNumberPeriodicSRS-PerBWP-PerSlot 5. maxNumberSemiPersitentSRS-PerBWP 6. maxNumberSP-SRS-PerBWP-PerSlot 7. maxNumberSRS-Ports-PerResource } | FeatureSetUplink | n/a | n/a |  | Mandatory with capability signalling Component-1: candidate value: {from 1, 2, 4, 8, 16} Component-2 candidate value: {1,2,3,4,5,6} Component-3: candidate value: {from 1, 2, 4, 8, 16} Component-4 candidate value: {1,2,3,4,5, 6} Component-5: candidate value: {from 1, 2, 4, 8, 16} } Component-6 candidate value: {1, 2,3,4,5, 6} Component-7 candidate values: {1, 2, 4} Support SP-SRS is mandatory with capability |
|  | 2-55 | SRS Tx switch | 1) Support SRS Tx port switch, 2) Report whether the uplink TX switching impact to downlink receiving in a band, 3) Report whether the UL Tx is switched together with UL Tx in another band | 2-53 | srs-TxSwitch { 1. supportedSRS-TxPortSwitch 2. txSwitchImpactToRx 3. txSwitchWithAnotherBand } | BandParameters-v1540 | n/a | n/a | Component-2 is per band pair per band combination Component-3 is per band pair per band combination The band pair in Component-2 and Component-3 can be an LTE band and an NR band 2T4R is 2 pairs of antennas “R” refers to a subset/set of receive antennas for PDSCH; “T” refers to the SRS antennas used for DL CSI acquisition | Mandatory with capability signalling Component-1 is a list of TRx pairs, candidates are {“Not supported”, “1T2R”, “1T4R”, “2T4R”, “1T4R/2T4R”, “1T=1R”, “2T=2R”, “4T=4R”} Component-2: Candidate value set: {yes, no} Component-3: Candidate value set: {yes, no} |
|  | 2-56 | SRS carrier switch | Report inter-cell switching time capability | 2-53 | srs-CarrierSwitch { srs-SwitchingTimesListNR, or srs-SwitchingTimesListEUTRA } | BandParameters-v1540 | No | n/a | RAN4 reply LS, R1-1805817, includes candidate value sets | Optional with capability signalling |
|  | 2-58 | For SRS for CB PUSCH and antenna switching on FR1, zero slot offset for aperiodic SRS transmission | For SRS for CB PUSCH and antenna switching on FR1, support of zero slot offset between aperiodic SRS triggering and transmission | 2-53 | zeroSlotOffsetAperiodicSRS | FeatureSetUplink-v1540 | n/a | n/a |  | Optional with capability signalling |
|  | 2-59 | Configured spatial relations | Maximum number of configured spatial relations per CC for PUCCH and SRS |  | spatialRelations { 2-59. maxNumberConfiguredSpatialRelations 2-60. maxNumberActiveSpatialRelations 2-61. additionalActiveSpatialRelationPUCCH 2-62. maxNumberDL-RS-QCL-TypeD } | MIMO-ParametersPerBand | n/a | Only applicable to FR2 |  | Candidate value set: {4, 8, 16, 32, 64, 96} UE is mandated to report 16 or higher values. |
|  | 2-60 | Active spatial relations | Maximum total number of {unique DL RS (except for aperiodic NZP CSI-RS) and SRS without spatial relation configured, and, TCI states available for DCI triggering of aperiodic NZP CSI-RS}, for indicating spatial domain transmit filter for PUCCH and SRS for PUSCH, per BWP per CC | 2-59 |  |  | n/a | Only applicable to FR2 | “Unique” means RS identity. An SSB and a CSI-RS are always counted as different. Two CSI-RSs are different if they have different CSI-RS resource IDs. | Mandatory with capability signalling Candidate value set: {1, 2, 4, 8, 14} |
|  | 2-61 | Additional active spatial relation for PUCCH | Support one additional active spatial relation for PUCCH | 2-60 |  |  | n/a | n/a | Only applicable if 2-60 is set to 1. | Mandatory with capability signalling |
|  | 2-62 | Max number of downlink RS resources used for QCL type-D in the active TCI states and active spatial relation info | Max number of downlink RS resources in the active TCI states and active spatial relation info per CC | 2-4, 2-4a and 2-60 |  |  | n/a | n/a | Reference relationship follows 2-4/2-60 | Optional with capability signalling Candidate value set: {1,2,4,8, 14} |
| 3. DL control channel and procedure | 3-1 | Basic DL control channel | 1) One configured CORESET per BWP per cell in addition to CORESET0 - CORESET resource allocation of 6RB bit-map and duration of 1 – 3 OFDM symbols for FR1 - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSSs, CORESET resource allocation of 6RB bit-map and duration 1-3 OFDM symbols for FR2 - For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 1-2 OFDM symbols for FR2 - REG-bundle sizes of 2/3 RBs or 6 RBs - Interleaved and non-interleaved CCE-to-REG mapping - Precoder-granularity of REG-bundle size - PDCCH DMRS scrambling determination - TCI state(s) for a CORESET configuration 2) CSS and UE-SS configurations for unicast PDCCH transmission per BWP per cell - PDCCH aggregation levels 1, 2, 4, 8, 16 - UP to 3 search space sets in a slot for a scheduled SCell per BWP This search space limit is before applying all dropping rules. - For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot 3) Monitoring DCI formats 0_0, 1_0, 0_1, 1_1 4) Number of PDCCH blind decodes per slot with a given SCS follows Case 1-1 table 5) Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot per scheduled CC for FDD 6) Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot per scheduled CC for TDD |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 3-1’ | For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2 | For type 1 CSS with dedicated RRC configuration and for type 3 CSS, UE specific SS, CORESET resource allocation of 6RB bit-map and duration 3 OFDM symbols for FR2 |  | type1-3-CSS | FeatureSetDownlink | n/a | n/a |  | Mandatory with capability signalling |
|  | 3-2 | PDCCH monitoring on any span of up to 3 consecutive OFDM symbols of a slot | For a given UE, all search space configurations are within the same span of 3 consecutive OFDM symbols in the slot |  | pdcchMonitoringSingleOccasion | Phy-ParametersFR1 | No | Applicable only to FR1 |  | Optional with capability signalling |
|  | 3-3 | More than one CORESET configurations per BWP in addition to CORESET0 | More than one CORESET configurations per BWP in addition to CORESET0 |  | multipleCORESET | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling for FR1 Mandatory with capability signalling for FR2 |
|  | 3-4 | More than one TCI state configurations per CORESET | More than one TCI state configurations per CORESET |  | multipleTCI | BandNR | n/a | n/a | UE is only required to track one active TCI state per CORESET UE is required to support minimum between 64 and number of configured TCI states in 2-4, component 2). | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 3-5 | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 |  | pdcch-MonitoringAnyOccasions { 3-5. withoutDCI-Gap 3-5a. withDCI-Gap } | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 3-5a | For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a DCI gap | For type 1 CSS with dedicated RRC configuration, type 3 CSS and UE-SS, monitoring occasion can be any OFDM symbol(s) of a slot for Case 2, with minimum time separation (including the cross-slot boundary case) between two DL unicast DCIs, between two UL unicast DCIs, or between a DL and an UL unicast DCI in different monitoring occasions where at least one of them is not the monitoring occasions of FG-3-1, for a same UE as - 2OFDM symbols for 15kHz - 4OFDM symbols for 30kHz - 7OFDM symbols for 60kHz with NCP - 11OFDM symbols for 120kHz Up to one unicast DL DCI and up to one unicast UL DCI in a monitoring occasion except for the monitoring occasions of FG 3-1. In addition for TDD the minimum separation between the first two UL unicast DCIs within the first 3 OFDM symbols of a slot can be zero OFDM symbols. |  |  |  | n/a | n/a |  | Optional with capability signalling |
|  | 3-5b | All PDCCH monitoring occasion can be any OFDM symbol(s) of a slot for Case 2 with a span gap | PDCCH monitoring occasions of FG-3-1, plus additional PDCCH monitoring occasion(s) can be any OFDM symbol(s) of a slot for Case 2, and for any two PDCCH monitoring occasions belonging to different spans, where at least one of them is not the monitoring occasions of FG-3-1, in same or different search spaces, there is a minimum time separation of X OFDM symbols (including the cross-slot boundary case) between the start of two spans, where each span is of length up to Y consecutive OFDM symbols of a slot. Spans do not overlap. Every span is contained in a single slot. The same span pattern repeats in every slot. The separation between consecutive spans within and across slots may be unequal but the same (X, Y) limit must be satisfied by all spans. Every monitoring occasion is fully contained in one span. In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max{maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration. A particular PDCCH monitoring configuration meets the UE capability limitation if the span arrangement satisfies the gap separation for at least one (X, Y) in the UE reported candidate value set in every slot, including cross slot boundary. For the set of monitoring occasions which are within the same span: • Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for FDD • Processing one unicast DCI scheduling DL and two unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD • Processing two unicast DCI scheduling DL and one unicast DCI scheduling UL per scheduled CC across this set of monitoring occasions for TDD The number of different start symbol indices of spans for all PDCCH monitoring occasions per slot, including PDCCH monitoring occasions of FG-3-1, is no more than floor(14/X) (X is minimum among values reported by UE). The number of different start symbol indices of PDCCH monitoring occasions per slot including PDCCH monitoring occasions of FG-3-1, is no more than 7. The number of different start symbol indices of PDCCH monitoring occasions per half-slot including PDCCH monitoring occasions of FG-3-1 is no more than 4 in SCell. |  | pdcch-MonitoringAnyOccasionsWithSpanGap (X, Y): set1 = (7, 3); set2 = (4, 3) and (7, 3); set3 = (2, 2) and (4, 3) and (7, 3). | FeatureSetDownlink-v1540 | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3) and (7, 3), (2, 2) and (4, 3) and (7, 3)} |
|  | 3-6 | Dynamic SFI monitoring | Adjust periodic and semi-persistent signal reception and transmission in response to detected dynamic UL/DL configuration |  | dynamicSFI | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Optional with capability signalling |
|  | 3-7 | Precoder-granularity of CORESET size | Precoder-granularity of CORESET size |  | precoderGranularityCORESET | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 3-8 | Up to 10 search spaces in a SCell | Up to 10 search spaces in a slot in an SCell per BWP |  | maxNumberSearchSpaces | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
| 4. UL control channel and procedure | 4-1 | Basic UL control channel | 1) PUCCH format 0 over 1 OFDM symbols once per slot 2) PUCCH format 0 over 2 OFDM symbols once per slot with frequency hopping as “enabled” 3) PUCCH format 1 over 4 – 14 OFDM symbols once per slot with intra-slot frequency hopping as “enabled” 5) One SR configuration per PUCCH group 6) HARQ-ACK transmission once per slot with its resource/timing determined by using the DCI 7) SR/HARQ multiplexing once per slot using a PUCCH when SR/HARQ-ACK are supposed to be sent by overlapping PUCCH resources with the same starting symbols in a slot 8) HARQ-ACK piggyback on PUSCH with/without aperiodic CSI once per slot when the starting OFDM symbol of the PUSCH is the same as the starting OFDM symbols of the PUCCH resource that HARQ-ACK would have been transmitted on 9) Semi-static beta-offset configuration for HARQ-ACK 10) Single group of overlapping PUCCH/PUCCH and overlapping PUCCH/PUSCH s per slot per PUCCH cell group for control multiplexing |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 4-2 | 2 PUCCH of format 0 or 2 in consecutive symbols | 1) 2 PUCCH format 0/2 in different symbols and once per slot for HARQ-ACK, 2) 2 PUCCH format 0 in different symbols and once per slot for SR 3) 2 PUCCH format 2 in different symbols and once per slot for CSI over two consecutive OFDM symbols |  | twoPUCCH-F0-2-ConsecSymbols | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Optional with capability signalling |
|  | 4-3 | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as “enabled” | PUCCH format 2 over 1 – 2 OFDM symbols once per slot with frequency hopping as “enabled” |  | pucch-F2-WithFH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling which shall be set to ‘1’ |
|  | 4-4 | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” | PUCCH format 3 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  | pucch-F3-WithFH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling which shall be set to ‘1’ |
|  | 4-5 | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” | PUCCH format 4 over 4 – 14 OFDM symbols once per slot with frequency hopping as “enabled” |  | pucch-F4-WithFH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 4-6 | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as “disabled” | Non-frequency hopping for PUCCH formats 0 and 2 with frequency hopping as “disabled” |  | freqHoppingPUCCH-F0-2 | Phy-ParametersFRX-Diff | No | Yes | The value indicated by this field is “notSupported”. | Mandatory with capability signalling |
|  | 4-7 | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as “disabled” | Non-frequency hopping for PUCCH format 1, 3, and 4 with frequency hopping as “disabled” |  | freqHoppingPUCCH-F1-3-4 | Phy-ParametersFRX-Diff | No | Yes | The value indicated by this field is “notSupported”. | Mandatory with capability signalling |
|  | 4-10 | Dynamic HARQ-ACK codebook | Dynamic HARQ-ACK codebook |  | dynamicHARQ-ACK-Codebook | Phy-ParametersCommon | No | No |  | Mandatory with capability signaling which shall be set to ‘1’ |
|  | 4-11 | Semi-static HARQ-ACK codebook | Semi-static HARQ-ACK codebook |  | semiStaticHARQ-ACK-Codebook | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 4-12 | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group | HARQ-ACK spatial bundling for PUCCH or PUSCH per PUCCH group |  | spatialBundlingHARQ-ACK | Phy-ParametersCommon | No | No | Applicable to UE supporting more than 4 layers | Mandatory with capability signalling |
|  | 4-13 | More than one SR configurations per PUCCH group | More than one SR configurations per PUCCH group |  | multipleSR-Configurations | MAC-ParametersXDD-Diff | Yes | No |  | Optional with capability signalling |
|  | 4-19 | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same starting symbol on the PUCCH resources in a slot | SR/HARQ-ACK/CSI multiplexing once per slot, where overlapping PUCCH resources have the same starting symbols on the PUCCH resources in a slot while precluding the case of SR/HARQ-ACK by overlapping PUCCH resources with the same starting symbols on the PUCCH resources in a slot |  | sameSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff | No | Yes | If FG4-28 is not included or not supported, HARQ-ACK/CSI piggyback on PUSCH once per slot when the starting OFDM symbol of the PUSCH is the same as the starting OFDM symbols of the PUCCH resource(s) that would have been transmitted on If FG4-28 is included and supported, HARQ-ACK/CSI piggyback on PUSCH once per slot for which case the starting OFDM symbol of the PUSCH is the different from the starting OFDM symbols of the PUCCH resource(s) that would have been transmitted on | Mandatory with capability signalling |
|  | 4-19a | SR/HARQ-ACK multiplexing once per slot using a PUCCH (or HARQ-ACK piggybacked on a PUSCH) when SR/HARQ-ACK are supposed to be sent with different starting symbols in a slot | Overlapping PUCCH resources have different starting symbols in a slot | 4-19 | mux-SR-HARQ-ACK-PUCCH | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-19b | SR/HARQ-ACK/CSI multiplexing more than once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with the same or different starting symbol in a slot | Overlapping PUCCH resources have same or different starting symbols in a slot | 4-19c | mux-SR-HARQ-ACK-CSI-PUCCH-MultiPerSlot | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-19c | SR/HARQ-ACK/CSI multiplexing once per slot using a PUCCH (or HARQ-ACK/CSI piggybacked on a PUSCH) when SR/HARQ-ACK/CSI are supposed to be sent with different starting symbols in a slot | Overlapping PUCCH resources have different starting symbols in a slot | 4-19a | diffSymbol in mux-SR-HARQ-ACK-CSI-PUCCH-OncePerSlot | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-20 | UCI code-block segmentation | UCI code-block segmentation |  | uci-CodeBlockSegmentation | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 4-21 | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI | Dynamic beta-offset configuration and indication for HARQ-ACK and/or CSI |  | dynamicBetaOffsetInd-HARQ-ACK-CSI | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 4-22 | 1 PUCCH format 0 or 2 and 1 PUCCH format 1, 3, or 4 in the same slot | 1 PUCCH format 0 or 2and 1 PUCCH format 1, 3, and 4 in the same slot |  | onePUCCH-LongAndShortFormat | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-22a | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 | 2 PUCCH transmissions in the same slot which are not covered by 4-22 and 4-2 |  | twoPUCCH-AnyOthersInSlot | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 4-23 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 | Repetitions for PUCCH format 1, 3, and 4 over multiple slots with K = 2, 4, 8 |  | pucch-Repetition-F1-3-4 | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 4-24 | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource | PUCCH-spatialrelationinfo indication by a MAC CE per PUCCH resource |  | pucch-SpatialRelInfoMAC-CE | BandNR | n/a | n/a |  | Mandatory with capability signalling for FR2 Optional with capability signalling for FR1 |
|  | 4-25 | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA | Parallel SRS and PUCCH/PUSCH transmission across CCs in inter-band CA | 2-52, 4-1, 2-12, 6-6 | parallelTxSRS-PUCCH-PUSCH | CA-ParametersNR | n/a | n/a | This feature is supported only in inter-band CA. | Optional with capability signalling |
|  | 4-26 | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel PRACH and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | 1-1, 2-52, 4-1, 2-12, 6-6 | parallelTxPRACH-SRS-PUCCH-PUSCH | CA-ParametersNR | n/a | n/a | This feature is supported only in inter-band CA. | Optional with capability signalling |
|  | 4-27 | More than one group of overlapping channels for control multiplexing | More than one group of overlapping PUCCHs and PUSCHs per slot per PUCCH cell group for control multiplexing |  | mux-MultipleGroupCtrlCH-Overlap | Phy-ParametersFRX-Diff | No | Yes | If a UE does not indicate supporting any of 4-2, 4-22, and 4-22a, the UE is not expected to be scheduled with more than one group of overlapping PUCCHs without PUSCH in each of the groups | Optional with capability signalling |
|  | 4-28 | HARQ-ACK multiplexing on PUSCH with different PUCCH/PUSCH starting OFDM symbols | HARQ-ACK piggyback on a PUSCH with/without aperiodic CSI once per slot when the starting OFDM symbol of the PUSCH is different from the starting OFDM symbols of the PUCCH resource that HARQ-ACK would have been transmitted on | 4-1 | mux-HARQ-ACK-PUSCH-DiffSymbol | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
| 5. Scheduling/HARQ operation | 5-1 | Basic scheduling/HARQ operation | 1) Frequency-domain resource allocation - RA Type 0 only and Type 1 only for PDSCH without interleaving - RA Type 1 for PUSCH without interleaving 2) Time-domain resource allocation - 1-14 OFDM symbols for PUSCH once per slot - One unicast PDSCH per slot - Starting symbol, and duration are determined by using the DCI - PDSCH mapping type A with 7-14 OFDM symbols - PUSCH mapping type A and type B - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, PDSCH mapping type A with {4-14} OFDM symbols and type B with {2, 4, 7} OFDM symbols 3) TBS determination 4) Nominal UE processing time for N1 and N2 (Capability #1) 5) HARQ process operation with configurable number of DL HARQ processes of up to 16 6) Cell specific RRC configured UL/DL assignment for TDD 7) Dynamic UL/DL determination based on L1 scheduling DCI with/without cell specific RRC configured UL/DL assignment 8) Intra-slot frequency-hopping for PUSCH scheduled by Type 1 CSS before RRC connection 9) In TDD support at most one switch point per slot for actual DL/UL transmission(s) 10) DL scheduling slot offset K0=0 11) DL scheduling slot offset K0=1 for type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS 12) UL scheduling slot offset K2<=12 For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, interleaving for VRB-to-PRB mapping for PDSCH |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
|  | 5-1a | UE specific RRC configure UL/DL assignment | Dynamic UL/DL determination based on L1 scheduling DCI with cell-specific and UE specific RRC configured UL/DL assignment |  | ue-SpecificUL-DL-Assignment | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 5-1b | More than one DL/UL switch point in a slot | In TDD support more than one switch points in a slot for actual DL/UL transmission(s) |  | tdd-MultiDL-UL-SwitchPerSlot | Phy-ParametersFRX-Diff | TDD only | Yes |  | Optional with capability signalling |
|  | 5-2 | RA Type 0 for PUSCH | RA Type 0 for PUSCH |  | ra-Type0-PUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-3 | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH | Dynamic switching between RA Type 0 and RA Type 1 for PDSCH |  | dynamicSwitchRA-Type0-1-PDSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-4 | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH | Dynamic switching between RA Type 0 and RA Type 1 for PUSCH | 5-2 | dynamicSwitchRA-Type0-1-PUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-5a | UE PDSCH processing capability #2 | UE can report values ‘X’ and ‘Fallback’, and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 1 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled, otherwise - If Fallback = ‘SC’, UE supports Capability #2 processing time on lowest cell index among the configured carriers in the band where the value is reported - If Fallback = ‘Cap1-only’, UE supports only Capability #1, in the band where the value is reported 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz |  | pdsch-ProcessingType2 | FeatureSetDownlink-v1540 | n/a | Applicable to FR1 only | This capability is necessary for each SCS (15kHz, 30kHz, 60kHz) More than one set of per SCS per band reports can be signaled for a given band combination | ptional with capability signaling Candidate values for Component 1: X in {1, ..., 16}, Fallback {‘SC’,’Cap1-only’} |
|  | 5-5b | UE PDSCH processing capability #2 with scheduling limitation for 30kHz-SCS | Capability #2 supported only if 1 carrier configured in the band (independent of #carriers configured in other bands) 2) Max PDSCH BW of 136 PRBs on the configured serving cell which processingType2Enabled is configured and set to enabled 3) N1 based on Table 5.3-2 of TS 38.214 for 30 kHz SCS 4) UE reports the number of unicast PDSCH per slot for different TBs |  | pdsch-ProcessingType2-Limited | FeatureSetDownlink-v1540 | n/a | Applicable to FR1 only | This capability is applicable to 30kHz-SCS only | Optional with capability signaling Component 4) the value ranges {1, 2, 4, 7} |
|  | 5-5c | UE PUSCH processing capability #2 | UE can report values ‘X’ and ‘Fallback’, and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 1 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled, otherwise - If Fallback = ‘SC’, UE supports Capability #2 processing time on lowest cell index among the configured carriers in the band where the value is reported - If Fallback = ‘Cap1-only’, UE supports only Capability #1, in the band where the value is reported 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz |  | pusch-ProcessingType2 | FeatureSetUplink-v1540 | n/a | Applicable to FR1 only | This capability is necessary for each SCS (15kHz, 30kHz, 60kHz) More than one set of per SCS per band reports can be signaled for a given band combination | Optional with capability signaling Candidate values for Component 1: X in {1, …, 16}, Fallback {‘SC’,’Cap1-only’} |
|  | 5-6 | PDSCH mapping type A with less than 7 OFDM symbols | or type 1 CSS with dedicated RRC configuration, for type 3 CSS and UE-SS, PDSCH mapping type A with less than 7 OFDM symbols |  | pdsch-MappingTypeA | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling which shall be set to “1” |
|  | 5-6a | PDSCH mapping type B | PDSCH mapping type B |  | pdsch-MappingTypeB | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-7 | Interleaving for VRB-to-PRB mapping for PDSCH | Interleaving for VRB-to-PRB mapping for PDSCH |  | interleavingVRB-ToPRB-PDSCH | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-9 | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 CSS before RRC connection | Intra-slot frequency-hopping for PUSCH except for PUSCH scheduled by Type 1 CSS before RRC connection |  | intraSlotFreqHopping-PUSCH | Phy-ParametersFRX-Diff | No | Yes |  | Mandatory with capability signalling |
|  | 5-10 | Inter-slot frequency hopping for PUSCH | Inter-slot frequency hopping for PUSCH |  | interSlotFreqHopping-PUSCH | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-11 | Up to 2 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 2 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  | pdsch-ProcessingType1-DifferentTB-PerSlot | FeatureSetDownlink | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-11a | Up to 7 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 7 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-11b | Up to 4 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 4 unicast PDSCHs per slot per CC only in TDM is supported for Capability 1 1) PDSCH(s) for Msg. 4 is included |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-12 | Up to 2 PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 2 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  | pusch-ProcessingType1-DifferentTB-PerSlot | FeatureSetUplink | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-12a | Up to 7 PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 7 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-12b | Up to 4 PUSCHs per slot per CC for different TBs for UE processing time Capability 1 | Up to 4 unicast PUSCHs per slot per CC only in TDM is supported for Capability 1 |  |  |  | n/a | n/a | This capability is necessary for each SCS. | Optional with capability signalling |
|  | 5-13 | Up to 2 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 2 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 2 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a | pdsch-ProcessingType2 | FeatureSetDownlink | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13a | Up to 7 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 7 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 7 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a |  |  | n/a | n//a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13c | Up to 4 unicast PDSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 4 unicast PDSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X DL CCs, the UE may expect to be scheduled with up to 4 PDSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) No scheduling limitation 3) N1 based on Table 5.3-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5a |  |  | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13d | Up to 2 PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 2 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 2 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c | pusch-ProcessingType2 | FeatureSetUplink | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13e | Up to 7 PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 7 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 7 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c |  |  | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-13f | Up to 4 PUSCHs per slot per CC for different TBs for UE processing time Capability 2 | Up to 4 unicast PUSCHs per slot per CC only in TDM is supported for Capability 2 UE can report values ‘X’ and supports the following operation, only when all carriers are self-scheduled and all Capability #2 carriers in a band are of the same numerology - When configured with less than or equal to X UL CCs, the UE may expect to be scheduled with up to 4 PUSCHs per slot with Capability #2 on all of the configured serving cells for which processingType2Enabled is configured and set to enabled 2) N2 based on Table 6.4-2 of TS 38.214 for given SCS from {15, 30, 60} kHz | 5-5c |  |  | n/a | n/a | This capability is necessary for each SCS More than one set of per SCS per band reports can be signalled for a given band combination | Optional with capability signalling Candidate values for Component 1: X in {1, …, 16}, |
|  | 5-14 | Type 1 configured PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions with RV sequences |  | type1-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-16 | Type 2 configured PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions with RV sequences |  | type2-PUSCH-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-17 | PUSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions |  | pusch-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-17a | PDSCH repetitions over multiple slots | K = 2, 4, 8 times repetitions |  | pdsch-RepetitionMultiSlots | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-18 | DL SPS | DL SPS |  | downlinkSPS | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-19 | Type 1 Configured UL grant | K = 1 |  | configuredUL-GrantType1 | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-20 | Type 2 Configured UL grant | K = 1 |  | configuredUL-GrantType2 | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-21 | Pre-emption indication for DL | Pre-emption indication for DL |  | pre-EmptIndication-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-22 | CBG-based re-transmission for DL using CBGTI | CBG-based re-transmission for DL using CBGTI |  | cbg-TransIndication-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-23 | CBGFI for CBG-based re-transmission for DL | CBGFI for CBG-based re-transmission for DL | 5-22 | cbg-FlushIndication-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-24 | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL | Dynamic HARQ-ACK codebook using sub-codebooks for CBG-based re-transmission for DL |  | dynamicHARQ-ACK-CodeB-CBG-Retx-DL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-25 | CBG-based re-transmission for UL using CBGTI | CBG-based re-transmission for UL using CBGTI |  | cbg-TransIndication-UL | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-26 | Semi-static rate-matching resource set configuration for DL | 1) Bitmap 1/2/3 2) controlResourceSet |  | rateMatchingResrcSetSemi-Static | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-27 | Dynamic rate-matching resource set configuration for DL | Bitmap 1/2/3 |  | rateMatchingResrcSetDynamic | Phy-ParametersCommon | No | No |  | Optional with capability signalling |
|  | 5-27a | Dynamic rate-matching control resource set for DL | Dynamic rate-matching control resource set for DL |  | rateMatchingCtrlResrcSetDynamic | Phy-ParametersCommon | No | No |  | Mandatory with capability signalling |
|  | 5-28 | Rate-matching around LTE CRS | Rate-matching around LTE CRS |  | rateMatchingLTE-CRS | BandNR | n/a | n/a |  | Mandatory with capability signalling |
|  | 5-29 | LBRM for PUSCH | Limited buffer rate matching in UL |  | pusch-LBRM | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 5-30 | DL scheduling slot offset greater than zero for PDSCH mapping type A | Support of DL scheduling slot offset (K0) greater than zero for PDSCH mapping type A |  | dl-SchedulingOffset-PDSCH-TypeA | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 5-30a | DL scheduling slot offset greater than zero for PDSCH mapping type B | Support of DL scheduling slot offset (K0) greater than zero for PDSCH mapping type B |  | dl-SchedulingOffset-PDSCH-TypeB | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 5-31 | UL scheduling slot offset greater than 12 | Support of UL scheduling slot offset (K2) greater than 12 |  | ul-SchedulingOffset | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 5-32 | Separation of two unicast PDSCHs with a gap | For any two consecutive slots n and n+1, if there are more than 1 unicast PDSCH in either slot, the minimum time separation between starting time of any two unicast PDSCHs within the duration of these slots is 4 OFDM symbol for 30kHz and 7 OFDM symbol for 60kHz | 5-11, 5-11b, 5-13, or 5-13c | pdsch-SeparationWithGap | FeatureSetDownlink-v1540 | No | No | This feature only applies to SCS 30kHz and 60kHz | Optional with capability signalling |
|  | 5-33 | Separation of two unicast PUSCHs with a gap | For any two consecutive slots n and n+1, if there are more than 1 unicast PUSCH in either slot, the minimum time separation between starting time of any two unicast PUSCHs within the duration of these slots is 2OFDM symbols for 15kHz, 4 OFDM symbols for 30kHz and 7 OFDM symbols for 60kHz | 5-12, 5-12b, 5-13d, or 5-13f | pusch-SeparationWithGap | FeatureSetUplink-v1540 | No | No | This feature only applies to SCS 15kHz, 30kHz and 60kHz | Optional with capability signalling |
|  | 5-34 | New 64QAM MCS table for PDSCH | New 64QAM MCS table for PDSCH |  | dl-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 5-34a | New 64QAM MCS table for PUSCH | New 64QAM MCS tables for PUSCH with and without transform precoding respectively |  | ul-64QAM-MCS-TableAlt | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 5-34b | Dynamic indication of MCS table with MCS-C-RNTI for PDSCH | Dynamic indication of MCS table using MCS-C-RNTI for PDSCH | 5-34 | dl-MCS-TableAlt-DynamicIndication | FeatureSetDownlink-v1540 | n/a | n/a |  | Optional with capability signalling |
|  | 5-34c | Dynamic indication of MCS tables with MCS-C-RNTI for PUSCH | Dynamic indication of MCS tables using MCS-C-RNTI for PUSCH | 5-34a | ul-MCS-TableAlt-DynamicIndication | FeatureSetUplink-v1540 | n/a | n/a |  | Optional with capability signalling |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |  | n/a | n/a | n/a | n/a | This feature should be mandatory without capability signalling for at least BWPs which is the same as the set of specified channel BW UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP | Mandatory without capability signalling |
|  | 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell | 6-1, 6-2, 6-3, or 6-4 | bwp-WithoutRestriction | BandNR | n/a | n/a | 6-1a is applicable to 6-1, 6-2, 6-3, or 6-4. | Optional with capability signalling |
|  | 6-2 | Type A BWP adaptation with same numerology | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto2 in bwp-SameNumerology | BandNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-3 | Type B BWP adaptation with same numerology | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-SameNumerology | BandNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-4 | BWP adaptation with different numerologies | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-DiffNumerology | BandNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-5 | Basic DL NR-NR CA operation | 1) Up to16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |  | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of DL CA band combination(s). | Optional with capability signalling |
|  | 6-5a | PDCCH blind detection capability for CA | 1) More than 4 DL CCs 2) Reporting value is one of integer from 4 to 16 | 6-5 | pdcch-BlindDetectionCA | Phy-ParametersFRX-Diff | No | Yes | If UE supports CA with more than 4 DL CCs, UE should report this capability | {4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16} |
|  | 6-6 | Basic UL NR-NR CA operation | 1) Up to16 UL carriers 2) Same numerology across carrier for data/control channel at a given time 3) One PUCCH group 4) Single TAG | 6-5 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of UL CA band combination(s). The terminology ‘carrier’ in the components in this FG does not refer to ‘SUL’. | Optional with capability signalling |
|  | 6-7 | Two NR PUCCH group with same numerology | 1) For NR CA UE, same numerology across NR carriers for data/control channel at a given time 2) For EN-DC UE, same numerology across NR carriers for data/control channel at a given time, wherein an NR PUCCH group is configured in FR1 and another NR PUCCH group is configured in FR2 | 6-5, 6-6 | twoPUCCH-Group | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |
|  | 6-8 | Different numerology across NR PUCCH groups | For both NR CA UE and EN-DC UE, different numerology between two NR PUCCH groups for data/control channel at a given time | 6-5, 6-7 | diffNumerologyAcrossPUCCH-Group | CA-ParametersNR | n/a | n/a |  | Optional with capability signalling |
|  | 6-9 | Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of smaller SCS | 1) For both NR CA UE, EN-DC/NE-DC UE and NR-DC UEs, same numerology between DL and UL per carrier for data/control channel at a given time 2) For both NR CA UE and EN-DC/NE-DC UE with one NR PUCCH group, different numerologies across NR carriers within the same NR PUCCH groups up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time 3-1) For NR CA UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time 3-2) For EN-DC/NE-DC UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within an NR PUCCH group in FR1 wherein NR PUCCH is sent on the carrier with smaller SCS, and same numerology across NR carriers within another NR PUCCH group in FR2 for data/control channel at a given time 4) For NR DC UE, different numerologies across NR carriers within the same NR PUCCH group in MCG (in FR1) and up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with smaller SCS for data/control channel at a given time; and same numerology across NR carriers in SCG (in FR2). | 6-5 | diffNumerologyWithinPUCCH-GroupSmallerSCS | CA-ParametersNR | n/a | n/a | The terminologies ‘UL’ and ‘carrier’ in this FG do not refer to ‘SUL’. NR PUCCH is sent on a carrier with SCS not larger than SCS of any DL carriers corresponding to the NR PUCCH group. The case with PUCCH on UL carrier with different numerologies within SCG is not supported for NR-DC. | Optional with capability signalling |
|  | 6-9a | Different numerologies across NR carriers within the same NR PUCCH group, with PUCCH on a carrier of larger SCS | 1) For both NR CA UE, EN-DC/NE-DC UE and NR DC UEs, same numerology between DL and UL per carrier for data/control channel at a given time 2) For both NR CA UE and EN-DC/NE-DC UE with one NR PUCCH group, different numerologies across NR carriers within the same NR PUCCH groups up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is on the carrier with larger SCS for data/control channel at a given time 3-1) For NR CA UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with larger SCS for data/control channel at a given time 3-2) For EN-DC/NE-DC UE with two NR PUCCH groups, different numerologies across NR carriers up to two different numerologies within an NR PUCCH group in FR1 wherein NR PUCCH is sent on the carrier with larger SCS, and same numerology across NR carriers within another NR PUCCH group in FR2 for data/control channel at a given time 4) For NR DC UE, different numerologies across NR carriers within the same NR PUCCH group in MCG (in FR1) and up to two different numerologies within the same NR PUCCH group wherein NR PUCCH is sent on the carrier with larger SCS for data/control channel at a given time; and same numerology across NR carriers in SCG (in FR2). | 6-5 | diffNumerologyWithinPUCCH-GroupLargerSCS | CA-ParametersNR-v1560 | n/a | n/a | The terminologies ‘UL’ and ‘carrier’ in this FG do not refer to ‘SUL’. NR PUCCH is sent on a carrier with SCS not smaller than SCS of any DL carriers corresponding to the NR PUCCH group. The case with PUCCH on UL carrier with different numerologies within SCG is not supported for NR-DC. | Optional with capability signalling |
|  | 6-10 | Cross carrier scheduling for the same numerology | Cross carrier scheduling for the same numerology with CIF where numerologies for scheduling cell and scheduled cell are same | 6-5, 6-6 | crossCarrierScheduling-SameSCS | BandNR | n/a | n/a |  |  |
|  | 6-10a | Cross carrier scheduling for different numerologies | Cross carrier scheduling for the different numerologies with CIF where numerologies for scheduling cell and scheduled cell are different | 6-10 | crossCarrierScheduling-OtherSCS | FeatureSetDownlink FeatureSetUplink | n/a | n/a | This is not supported in Rel-15 |  |
|  | 6-11 | Number of supported TAGs | Need of multiple capability question about the resolution here |  | supportedNumberTAG | CA-ParametersNR | n/a | n/a | This feature group is applied to NR-NR CA and EN-DC. For EN-DC, the feature group indicates number of TAGs only for NR CG. The number of TAGs for the LTE MCG is signalled by existing LTE TAG capability signalling | {1, 2, 3, 4} |
|  | 6-12 | Support 2 simultaneous UL transmissions for problematic cases | Support 2 simultaneous UL transmissions for problematic cases |  | singleUL-Transmission | MRDC-Parameters | n/a | n/a | This is a UE feature for LTE for a LTE/NR dual connectivity UE | Optional with capability signalling |
|  | 6-13 | Case 1 Single Tx UL LTE-NR DC | 1) Case 1: DL-reference UL/DL configuration defined for LTE-FDD-SCell in LTE-TDD-FDD CA with LTE-TDD-PCell 2) HARQ subframe offset |  | tdm-Pattern NOTE: This capability bit also indicates support of the feature 8-2, i.e. Operation A with single UL Tx case 1. | MRDC-Parameters | Yes | Yes | This is a UE feature for LTE for a LTE/NR dual connectivity UE | Mandatory with capability signalling conditional on the UE not supporting simultaneous dual-Tx operation in the band combination; optional if the UE supports simultaneous dual-Tx operation in the band combination |
|  | 6-16 | Supplemental uplink | 1) RACH, PUSCH, PUCCH, SRS operations in a band combination including SUL 2) Supplemental uplink with same numerology between SUL and non SUL carriers | 6-15 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of SUL band combination(s). | Optional with capability signalling |
|  | 6-17 | Supplemental uplink with different numerologies between SUL and non SUL carriers | Different numerologies between SUL and non SUL | 6-16 | supportedBandCombinationList | RF-Parameters | n/a | n/a | This is conditioned on the support of SUL band combination(s). | Mandatory with capability signalling |
|  | 6-18 | Supplemental uplink with dynamic switch | DCI based selection of PUSCH carrier | 6-16 | dynamicSwitchSUL | FeatureSetUplink | n/a | n/a | his is conditioned on the support of SUL band combination(s). | Optional with capability signalling |
|  | 6-19 | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS on the other UL carrier in the same cell | Simultaneous transmission of SRS on an SUL/non-SUL carrier and PUSCH/PUCCH/SRS on the other UL carrier in the same cell | 6-16 | simultaneousTxSUL-NonSUL | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |
|  | 6-21 | DL search space sharing for CA | DL search space sharing for CA | 6-10 or 6-10a | searchSpaceSharingCA-DL | FeatureSetDownlink | n/a | n/a |  | Optional with capability signalling |
|  | 6-22 | UL search space sharing for CA | UL search space sharing for CA | 6-10 or 6-10a | searchSpaceSharingCA-UL | FeatureSetUplink | n/a | n/a |  | Optional with capability signalling |
|  | 6-23 | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP | Incapability motivated by impacts of PA phase discontinuity with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers for intra-band EN-DC, intra-band CA, and FDM based ULSUP |  | pa-PhaseDiscontinuityImpacts | FeatureSetUplink-v1540 | n/a | n/a | See LS (R1-1809992) | Optional with capability signalling |
|  | 6-24 | Applying the same UL timing between NR and LTE | Applying the same UL timing between NR and LTE for dynamic power sharing capable UE operating in intra-band contiguous synchronous EN-DC | 8-1 | ul-TimingAlignmentEUTRA-NR | MRDC-Parameters | n/a | n/a | UEs that set this bit to 0 should be able to operate with a timing difference up to applicable MTTD requirements when operating in a synchronous intra-band contiguous EN-DC network. | Optional with capability signalling |
|  | 6-25 | Support of synchronous NR-NR DC operation only wherein MCG is only in FR1 and SCG is only in FR2 | Support of synchronous NR-NR DC operation only wherein MCG is only in FR1 and SCG is only in FR2 |  | ca-ParametersNRDC | BandCombination-v1560 | n/a | n/a | This is conditioned on the support of DC band combination(s). UE reports a set of supported band partitionings corresponding to MCG in FR1 and to SCG in FR2. | Optional with capability signalling |
|  | 6-25a | PDCCH blind detection capability for MCG and for SCG in synchronous NR-NR DC | RRC parameters pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE for optional new UE capability signalling that informs the maximum values for pdcch-BlindDetectionMCG and pdcch-BlindDetectionSCG, respectively | 6-5, 6-25 | pdcch-BlindDetectionMCG-UE pdcch-BlindDetectionSCG-UE | Phy-ParametersFRX-Diff | No | Yes | pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE are per UE capability signalling. The value range of pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE is - [1, …, pdcch-BlindDetectionCA-1] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= pdcch-BlindDetectionCA if the UE reports pdcch-BlindDetectionCA, and - [1, 2, 3] and pdcch-BlindDetectionMCG-UE + pdcch-BlindDetectionSCG-UE >= the maximum number of DL serving cells over CGs that UE can support if the UE does not report pdcch-BlindDetectionCA. If the UE does not report pdcch-BlindDetectionMCG-UE and pdcch-BlindDetectionSCG-UE, - pdcch-BlindDetectionCA for NR-CA is re-used as the UE capability signalling for NR-DC to determine BD/CCE limit across serving cells over CGs if the UE reports pdcch-BlindDetectionCA, and - the number of configured DL serving cells over CGs is used to determine BD/CCE limit across serving cells over CGs if the UE does not report pdcch-BlindDetectionCA. If the UE reports pdcch-BlindDetectionMCG-UE or pdcch-BlindDetectionSCG-UE, both of them are reported (i.e., not either of them). | Optional with capability signalling |
| 7. Channel coding | 7-1 | Channel coding | 1) LDPC encoding and associated functions for data on DL and UL 2) Polar encoding and associated functions for PBCH, DCI, and UCI 3) Coding for very small blocks |  | n/a | n/a | n/a | n/a |  | Mandatory without capability signalling |
| 8. UL TPC | 8-1 | Dynamic power sharing for LTE-NR DC | When total transmission power exceeds Pcmax, UE scales NR transmission power. | EN-DC | dynamicPowerSharing | MRDC-Parameters | n/a | n/a | RP-172833 | Mandatory with capability signalling |
|  | 8-2 | Operation A with single UL Tx case 1 | Operation A with single UL Tx case 1 | EN-DC | tdm-Pattern NOTE: This capability bit also indicates support of the feature 6-13, i.e. Case 1 Single Tx UL LTE-NR DC. | MRDC-Parameters | Yes | Yes | RP-172833 | Mandatory with capability signalling conditioned that UE does not support dynamic power sharing, i.e., UE indicate “0” as non-support for 8-1, optional for UEs supporting dynamic power sharing |
|  | 8-3 | Basic power control operation | 1) Accumulated power control mode for closed loop 2) 1 TPC command loop for PUSCH, PUCCH respectively 3) One or multiple DL RS configured for pathloss estimation 4) One or multiple p0-alpha values configured for open loop PC 5) PUSCH power control 6) PUCCH power control 7) PRACH power control 8) SRS power control 9) PHR |  | n/a | n/a | No | No |  | Mandatory without capability signalling |
|  | 8-4 | TPC-PUSCH-RNTI | Specific group DCI message for TPC commands for PUSCH |  | tpc-PUSCH-RNTI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-5 | TPC-PUCCH-RNTI | Specific group DCI message for TPC commands for PUCCH |  | tpc-PUCCH-RNTI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-6 | TPC-SRS-RNTI | Specific group DCI message for TPC commands for SRS |  | tpc-SRS-RNTI | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-7 | Absolute TPC command mode | Absolute TPC command mode |  | absoluteTPC-Command | Phy-ParametersFRX-Diff | No | Yes |  | Optional with capability signalling |
|  | 8-8 | UL power control with 2 PUSCH closed loops | Two different TPC loops |  | twoDifferentTPC-Loop-PUSCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |
|  | 8-9 | UL power control with 2 PUCCH closed loops | Two different TPC loops |  | twoDifferentTPC-Loop-PUCCH | Phy-ParametersXDD-Diff Phy-ParametersFRX-Diff | Yes | Yes |  | Mandatory with capability signalling |