3GPP TSG RAN WG1 Meeting #100	R1-2001120

E-meeting, February 24th – March 06th 2020

Agenda Item: 	7.2.6.3

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	Enhancements on UE multi-beam operation

Document for:	Decision

# Introduction

In this paper, a remaining issue regarding multi-beam operations in Rel. 16 is discussed.

# MAC-CE-based pathloss reference RS update for PUSCH

In RAN1#99 [1], the following agreements were made with respect to the update of pathloss reference for PUSCH via MAC-CE.

Agreement (RRC impact)

For the feature of MAC CE based pathloss RS updates for PUSCH/SRS in Rel-16,

- Introduce one new RRC parameter to enable the feature of MAC CE based pathloss RS updates for PUSCH/SRS in Rel-16, i.e.,

  - enablePLRSupdateForPUSCHSRS

Agreement

When enablePLRSupdateForPUSCHSRS is configured, if a grant-based or grant-free PUSCH transmission is scheduled/activated by DCI format 0_1 that does not include a SRI field, the RS resource index qd corresponding to the PUSCH-PathlossReferenceRS-Id mapped with sri-PUSCH-PowerControlId = 0 is used for path-loss measurement of PUSCH transmission. In this case, UE expects to be configured with sri-PUSCH-PowerControl

From the above agreements, the pathloss reference RS update for the PUSCH is possible via a MAC-CE message when the PUSCH is scheduled via DCI format 0_0 or 0_1 (with or without an SRI field). However, when the UE is scheduled via a higher layer grant (PUSCH configured with a ConfiguredGrantConfig that includes rrc-ConfiguredGrantConfig), the pathloss reference indication is performed using the RRC parameter pathlossReferenceIndex. Hence, the update of pathloss reference RS for the PUSCH, in this case, may be performed only via RRC reconfiguration. To exploit the MAC-CE based update of the pathloss reference RS for the PUSCH scheduled via higher layer grant, the following proposal is made.

Proposal 1: For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, and if the UE is provided with enablePLRSupdateForPUSCHSRS, the UE determines the pathloss reference RS from the PUSCH-PathlossReferenceRS that is mapped to the sri-PUSCH-PowerControlId indicated by the srs-ResourceIndicator value in the rrc-ConfiguredUplinkGrant.

With the above UE behavior, the pathloss reference RS update for a PUSCH scheduled via the higher layer grant may be performed using the MAC-CE message that updates the mapping between an SRI-PUSCH-PowerControlId and PUSCH-PathlossReferenceRs-Id. The following text change in TS 38.213 [2] is provided to support the above proposal (text to be updated is shown in red).

<----------------------------------------- Begin: Text proposal – TS 38.213 ---------------------------------------->

## 7.1	Physical uplink shared channel

7.1.1	UE behaviour

<----------------------------------------- Unchanged parts omitted ---------------------------------------->

For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig,

- if the UE is provided enablePLRSupdateForPUSCHSRS, the UE determines a RS resource index  from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the sri-PUSCH-PowerControlId indicated by the srs-ResourceIndicator value included in rrc-ConfiguredUplinkGrant

- if the UE is not provided enablePLRSupdateForPUSCHSRS, a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

<----------------------------------------- Unchanged parts omitted ---------------------------------------->

<----------------------------------------- End: Text proposal – TS 38.213 ---------------------------------------->

# Conclusion

The following proposal is made from the above discussion in the Tdoc.

Proposal 1: For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig, and if the UE is provided with enablePLRSupdateForPUSCHSRS, the UE determines the pathloss reference RS from the PUSCH-PathlossReferenceRS that is mapped to the sri-PUSCH-PowerControlId indicated by the srs-ResourceIndicator value in the rrc-ConfiguredUplinkGrant.

<----------------------------------------- Begin: Text proposal – TS 38.213 ---------------------------------------->

## 7.1	Physical uplink shared channel

7.1.1	UE behaviour

<----------------------------------------- Unchanged parts omitted ---------------------------------------->

For a PUSCH transmission configured by ConfiguredGrantConfig, if rrc-ConfiguredUplinkGrant is included in ConfiguredGrantConfig,

- if the UE is provided enablePLRSupdateForPUSCHSRS, the UE determines a RS resource index  from the value of PUSCH-PathlossReferenceRS-Id that is mapped to the srs-ResourceIndicator value included in rrc-ConfiguredUplinkGrant

- if the UE is not provided enablePLRSupdateForPUSCHSRS, a RS resource index  is provided by a value of pathlossReferenceIndex included in rrc-ConfiguredUplinkGrant where the RS resource is either on serving cell  or, if provided, on a serving cell indicated by a value of pathlossReferenceLinking

<----------------------------------------- Unchanged parts omitted ---------------------------------------->

<----------------------------------------- End: Text proposal – TS 38.213 ---------------------------------------->

# References

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #99, Reno, USA, November 18th – 22nd, 2019.

- TS 38.213, NR; Physical layer procedures for control, v16.0.0, December 2019.