3GPP TSG RAN WG1 #100		R1-2001197

e-Meeting, February 24th – March 6th, 2020

Agenda item:	6.2.4

Source: 	Qualcomm Incorporated

Title: 	Updated summary of issues for LTE-based 5G terrestrial broadcast

Document for:	Discussion and Decision

# Summary of contributions

Four tdocs were submitted under 6.2.4 and sub-agenda items.

| Tdoc | Agenda Item | Summary |
| --- | --- | --- |
| R1-2000713 (Qualcomm) | 6.2.4 | TR for approval (need to be approved in next RAN) |
| R1-2000213 (Huawei, HiSilicon) | 6.2.4.1 | Issue 1: Stagger of MBSFN-RS Issue 2: Scrambling initialization based on slot instead of subframe. Issue 3: TBS determination |
| R1-2000711 (Qualcomm) | 6.2.4.1 | Issue 1: Stagger of MBSFN-RS (same as issue 1 in x0213) Issue 2: Warm-up subframes |
| R1-2000712 (Qualcomm) | 6.2.4.2 | Issue 1: Change “shall” to “may” for CFI in MIB. Issue 2: Clarify that AL16 applies to all non-MBSFN subframes |

# Issue #1: TR update

FL assessed priority: High

Since the TR is due for approval in next RAN plenary, RAN1 should endorse a new version. Feature lead proposes to start an email discussion to discuss the updates in x0713.

# Issue #2: Stagger of MBSFN-RS

This issue was already discussed during the approval of the CR for TS 36.211, and the two text proposals submitted to this meeting have exactly the same change (some minor difference in mathematical notation). The proposed change is to make the stagger depend on ns=nf4×13+ns'.

# Issue #3: TBS determination

FL assessed priority: Medium

It is proposed to remove entry 33B from the TBS determination table, since this entry cannot be used for scheduling PMCH, as follows:

[…]

then round half-way towards positive infinity to the closest valid transport block size in the union of Table 7.1.7.2.1-1 and Table 7.1.7.2.4-1 by including, in the rounding procedure, only the TBS entries in Table 7.1.7.2.1-1 with ITBS≤33 and the entries in 7.1.7.2.4-1 for which the TBS_L1 is present in Table 7.1.7.2.1-1 with ITBS≤33 if the UE is configured by higher layers to decode the PMCH based on QPSK, 16QAM, 64QAM, and 256QAM

[…]

# Issue #4: Scrambling initialization

FL assessed priority: High

It is proposed that the scrambling generator initialization is initialized at the start of each slot (instead of subframe). This should be easy to agree, it should be discussed how to capture in the spec.

[…]

-	In clause 6.3.1, for Δf = 1.25 kHz and Δf ≈ 0.37 kHz, the scrambling generator shall be initialised at the start of each slot.

[…]

# Issue #5: Warm-up subframes

FL assessed priority: High

The issue of causality of channel estimation was discussed during the WID, but no consensus was reached. It is proposed to allow the UE to assume that, if slot n has PMCH with 0.37kHz, then the previous slots also have MBSFN-RS as follows:

[…]

For a PMCH transmitted with 0.37 kHz subcarrier spacing in slot n,

-	for MBSFN reference signal pattern type 1, the UE may assume that MBSFN reference signals associated with the same NIDMBSFN are present in the three preceding slots to slot n.

-	for MBSFN reference signal pattern type 2, the UE may assume that MBSFN reference signals associated with the same NIDMBSFN are present in the preceding slot to slot n.

[…]

# Issue #6: “Shall” vs “may” for CFI in MIB

FL assessed priority: Medium

It is proposed to change a “shall” to “may” in the applicability of MIB-based CFI as follows:

[…]

For a MBMS-dedicated cell, if a UE is configured with higher layer parameter semistaticCFI included in MasterInformationBlock-MBMS, the UE may assume the CFI is equal to the value of the higher layer parameter semistaticCFI for non-MBSFN subframes.

[…]

# Issue #7: Applicability of AL16 to all non-MBSFN subframes

FL assessed priority: High

It is proposed to clarify that Al16 is present in all non-MBSFN subframes for an MBMS dedicated carrier (not only for the CAS), as follows:

[…]

The CCEs available in the system are numbered from 0 to, where . The PDCCH supports multiple formats as listed in Table 6.8.1-1 where PDCCH format 4 is supported only for non-MBSFN subframes in an MBMS-dedicated cell. A PDCCH consisting of  consecutive CCEs may only start on a CCE fulfilling, where  is the CCE number.

[…]

# Prioritization of topics

In view of the points above, we propose to have the following email discussions:

Proposal: The prioritization of topics for discussion is as follows:

| Agenda Item | Issue | Priority |
| --- | --- | --- |
| 6.2.4 | Endorse new TR version | High |
| 6.2.4.1 | Staggering of MBSFN-RS | High |
| 6.2.4.1 | Scrambling initialization based on slot | High |
| 6.2.4.1 | Warm-up subframes | High |
| 6.2.4.1 | Removal of TBS 33B from procedural text | Medium |
| 6.2.4.2 | “Shall” vs “may” for CFI in MIB | Medium |
| 6.2.4.2 | AL16 applicability to all non-MBSFN subframes | High |

In view of the discussion in the preparation phase, it is proposed to have the following email discussion:

- Email discussion #1: Endorse new TR version

- Email discussion #2: Issues for 300us numerology:

  - Staggering of MBSFN-RS

  - Scrambling initialization based on slot

  - Warm-up subframes

  - (Lower priority) Removal of TBS 33B from procedural text

- Email discussion #3: Issues for CAS

  - AL16 PDCCH applicability to all non-MBSFN subframes

  - (Lower priority) “Shall” vs “may” for CFI in MIB

The TP in R1-2001111 is postponed to the next meeting.