3GPP TSG RAN WG1 Meeting #100                                                	                    R1-2001363

e-Meeting, February 24th – March 6th, 2020

Source:	CATT

Title:	              Feature lead summary on AI 7.2.4.3 #3 Sidelink synchronization mechanism – Email discussion outcomes

Agenda Item:	7.2.4.3

Document for:	Discussion and Decision

Introduction

In RAN1#100e meeting, 3 email discussions were taken place.

- [100e-NR-5G_V2X_NRSL-SYNC-01]

Email discussion/approval on the content of PSBCH:

- indication of TDD configuration

by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2 – Teng (CATT)

- [100e-NR-5G_V2X_NRSL-SYNC-02]

Email discussion/approval on the following issues:

- DM-RS sequence initialization for PSBCH: How to generate Cinit

- Scrambling of PSBCH

- S-PSS/S-SSS frequency position in S-SSB

by 2/28; if there is a spec impact, endorsing the corresponding TP by 3/3 – Teng (CATT)

- [100e-NR-5G_V2X_NRSL-SYNC-03]

Email discussion/approval on the tx power of S-SSB:

- S-SSB power control

- Power difference for S-PSS, S-SSS, PSBCH DM-RS, and PSBCH data

by 2/27; if there is a spec impact, endorsing the corresponding TP by 3/2 – Teng (CATT)

After long time discussion, we achieved some agreements and endorsed several TPs, which are captured in Section 2 in this document.

Agreements and endorsed TPs

[100e-NR-5G_V2X_NRSL-SYNC-01]

Agreements:

- Slot-level indication is supported in TDD configuration indication.

Agreements:

- The TDD configuration indication is done as follows:

  - X bits to indicate patterns + Y bits to indicate periodicity + Z bits to indicate UL slots.

    - FFS the values of X, Y and Z.

    - Total Z bits to indicate UL slots in pattern 1 and pattern 2 respectively if two patterns are configured.

[100e-NR-5G_V2X_NRSL-SYNC-02]

Agreements:

- SL SSID is used for DM-RS sequence initialization in PSBCH.

- The DM-RS sequence initialization for PSBCH is to be down-selected one from the following Alts:

  - Alt 1: cinit=211iS-SSB+1NIDSL+1+26iS-SSB+1, where iS-SSB is 3 LSBs of S-SSB index.

  - Alt 2: cinit=NIDSL

Agreements:

- Scrambling for PSBCH is processed after rate matching.

- NR PSBCH scrambling sequence is initialized with cinit=NIDSL at the start of each PSBCH block.

Agreements:

- To agree the following TP on S-PSS/S-SSS frequency position in S-SSB.

------------------------------------------------------ Start of Draft TP of 38. 211----------------------------------------------

<Unchanged parts omitted>

Table 8.4.3.1-1: Resources within an S-SS/PSBCH block for S-PSS, S-SSS, PSBCH, and DM-RS.

| Channel or signal | OFDM symbol number l relative to the start of an S-SS/PSBCH block | Subcarrier number k relative to the start of an S-SS/PSBCH block |
| --- | --- | --- |
| S-PSS | 1, 2 | 2-128 |
| S-SSS | 3, 4 | 2-128 |
| Set to 0 | 1,2,3,4 | 0,1, 129,130,131 |
| PSBCH | 0, 5,6, …,NsymbS-SSB-1 | 0,1,…,131 |
| DM-RS for PSBCH | 0, 5,6, …,NsymbS-SSB-1 | 0, 4, 8, …., 128 |

---------------------------------------------------------- End of Draft TP -------------------------------------------------------

TP-1 (endorsed)

--------------------------------------------- Start of Draft TP for 8.3.3.1 of TS 38. 211------------------------------------

8.3.3.1 Scrambling

The block of bits b0, …, b(Mbit-1), where Mbit is the number of bits transmitted on the physical sidelink broadcast channel, shall be scrambled prior to modulation, resulting in a block of scrambled bits  b0, …, b(Mbit-1) according to

bi=bi+c(i) mod 2

where the scrambling sequence ci is given by clause 5.2.1. The scrambling sequence generator shall be initialized with cinit=NIDSL at the start of each S-SS/PSBCH block.

---------------------------------------------- End of Draft TP for 8.3.3.1 of TS 38. 211 -----------------------------------

TP-2 (endorsed)

--------------------------------------------- Start of Draft TP for 8.4.3.1 of TS 38. 211------------------------------------

<Unchanged parts omitted>

Table 8.4.3.1-1: Resources within an S-SS/PSBCH block for S-PSS, S-SSS, PSBCH, and DM-RS.

| Channel or signal | OFDM symbol number l relative to the start of an S-SS/PSBCH block | Subcarrier number k relative to the start of an S-SS/PSBCH block |
| --- | --- | --- |
| S-PSS | 1, 2 | 2-128 |
| S-SSS | 3, 4 | 2-128 |
| Set to 0 | 1,2,3,4 | 0,1, 129,130,131 |
| PSBCH | 0, 5,6, …,NsymbS-SSB-1 | 0,1,…,131 |
| DM-RS for PSBCH | 0, 5,6, …,NsymbS-SSB-1 | 0, 4, 8, …., 128 |

---------------------------------------------- End of Draft TP for 8.4.3.1 of TS 38. 211 -----------------------------------

[100e-NR-5G_V2X_NRSL-SYNC-03]

Agreements:

- Downlink pathloss based S-SSB open-loop power control is supported.

- The transmission power is same for S-PSS, S-SSS, and PSBCH symbol.

TP-1 (endorsed)

------------------------------ Start of Draft TP for Section 16.1 of TS 38.213 --------------------------------

16.1 Synchronization procedures

---------------------------------------------- <Unchanged parts omitted> -----------------------------------------

For reception of a S-SS/PSBCH block, a UE assumes a frequency location corresponding to the subcarrier with index 66 in the S-SS/PSBCH block [4, TS 38.211], is provided by absoluteFrequencySSB-SL. The UE assumes that S-PSS symbol, S-SSS symbol, and PSBCH symbol have same transmission power. The UE assumes a same numerology of the S-SS/PSBCH as for a SL BWP of the S-SS/PSBCH block reception, and that a bandwidth of the S-SS/PSBCH is within a bandwidth of the SL BWP. The UE assumes the subcarrier with index 0 in the S-SS/PBCH block is aligned with a subcarrier with index 0 in the SL BWP.

---------------------------------------------- <Unchanged parts omitted> -----------------------------------------

---------------------------------- End of Draft TP for Section 16.1 of TS 38.213-----------------------------

TP-2 (endorsed)

------------------------------ Start of Draft TP for Section 16.2 of TS 38.213 --------------------------------

16.2 Power control

A UE transmits S-SS/PSBCH blocks, or a PSSCH, or a PSCCH, or a PSFCH with a same non-zero power in all corresponding symbols.

---------------------------------------------- <Unchanged parts omitted> -----------------------------------------

16.2.x S-SS/PSBCH blocks

A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i as

PS-SSB(i)=minPCMAX,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm]

where

-	PCMAX is defined in [8-1, TS 38.101-1]

-	PO,S-SSB is a value of p0-DL-S-SSB if provided; else, PS-SSB(i)=PCMAX

-	αS-SSB is a value of alpha-DL-S-SSB, if provided; else,αS-SSB=1

-	PL=PLb,f,c(qd) as described in Subclause 7.1.1

-     MRBS-SSB=11 is the number of resource blocks for the S-SS/PSBCH block transmission and μ is a SCS configuration

---------------------------------------------- <Unchanged parts omitted> -----------------------------------------

---------------------------------- End of Draft TP for Section 16.2 of TS 38.213------------------------------