3GPP TSG RAN WG1 #100-e	R1-2000168

e-Meeting, February 24th – March 6th, 2020

3GPP TSG RAN WG1 #100	R1-2000020

Athens, Greece, February 24th – 28th, 2020

3GPP TSG-RAN2 Meeting #108	R2- 1916645

Reno, USA, 18th - 22th Nov 2019

Title:	LS on clarification of CLI resource configuration

Response to:

Release:	Rel-16

Work Item:	NR_CLI_RIM-Core

Source:	RAN2

To:	RAN1

Cc:

Contact Person:

#### Name:	Sangwon Kim

E-mail Address:	sangwon7.kim@lge.com

Attachments:	-

1. Overall Description:

The existing SRS resource frequency domain configuration is associated with the BWP configuration or common resource block #0. However, for CLI measurements, the SRS resource transmitted by aggressor UE is configured without any association with the BWP configuration or serving cell information of victim UE, and freqDomainPosition and freqDomainShift are the only information to indicate the frequency position of the SRS resource. Therefore RAN2 thinks additional information is needed to indicate the frequency position of the SRS resource. RAN2 discussed the potential solutions but have not reached consensus, therefore, RAN2 would like to ask RAN1 to make decision on the additional information and inform RAN2.

Question1: What the additional information is needed to indicate the frequency position of the SRS resource other than freqDomainPosition and freqDomainShift??

Similarly for CLI RSSI resource configuration, currently only startPRB and nrofPRBs are indicated for frequency position. Considering RAN2 agreed CLI measurement is applicable to CA scenario, the question is whether additional information is also needed for CLI RSSI resource.

Question2: Is additional information needed for CLI RSSI resource?

The description provided by RAN1 for CLI-RSSI Resources indicates that the CLI-RSSI Resources are configured with reference SCS. Depending on active BWP, the UE will calculate the number of symbols by scaling factor based on ratio between active SCS and reference SCS. As per this description, network can define single CLI-RSSI resource across all the active BWPs and UE is assumed to measure on the overlapping part of resources when operating in any of the active BWPs. Hence RAN2 request RAN1 to clarify whether single CLI resource configuration across all the BWPs is to be supported or not.

Question3: whether a CLI RSSI resource can be configured across multiple BWPs.

2. Actions:

To RAN1

RAN2 kindly requests RAN1 to give responses to RAN2 with regard to above questions.

3. Date of Next RAN2 Meetings:

TSG-RAN2 Meeting #109	24 - 28 February 2020	Athens, Greece

TSG-RAN2 Meeting #109bis	20 - 24 April 2020	Japan