3GPP TSG RAN WG1 Meeting #100bis-e 	R1-2001603

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.8.4

Source:	ZTE

Title:	Maintenance of physical-layer procedures to support UE/gNB measurements

Document for:	Discussion and Decision

Introduction

In this contribution, we present our view on the maintenance of physical-layer procedures to support UE/gNB measurements.

UE procedures for determining and reporting the reference DL PRS

The following agreement was agreed in RAN1# 96b for network to indicate a reference time, and also for UE to determine  and report a reference time for the RSTD measurement[1].

Agreements:

The network can indicate one or more of the following for the UE to use to determine a reference (reference time based on the DL PRS Resource ID(s)) for DL RSTD measurements.

    - A DL PRS Resource ID

    - A subset of DL PRS Resource IDs from a single DL PRS Resource set

    - A DL PRS Resource set

- The UE may use different DL PRS Resource ID(s) (with the condition that the multiple DL PRS Resource IDs belong to a single DL PRS Resource set) or a different DL PRS Resource set for determining the reference for the RSTD measurement, and if it chooses to do so, it should report the DL PRS Resource ID(s) and/or the information on the DL PRS Resource set used to determine the reference.

Meanwhile, it was agreed that UE can report an [ID] different from the configured [ID] provided by DL-PRS-RstdReferenceInfo in last meeting [2]. Therefore, UE can choose one of the following options to determine a reference time,

- Option 1: A DL PRS resource where the DL PRS resource ID is provided by DL-PRS-RstdReferenceInfo.

- Option 2: One of the  DL PRS resources within the the list of DL PRS resource IDs provided by DL-PRS-RstdReferenceInfo.

- Option 3: The DL PRS resource set provided by DL-PRS-RstdReferenceInfo.

- Option 4: One of the DL PRS resources within the DL PRS resource set provided by DL-PRS-RstdReferenceInfo.

- Option 5: A DL PRS resource set which can be any DL PRS resource set associated with the [ID] provided by DL-PRS-RstdReferenceInfo.

- Option 6: A DL PRS resource set associated with an [ID] other than the [ID] provided by DL-PRS-RstdReferenceInfo

- Option 7: A DL PRS resource associated with an [ID] other than the [ID] provided by DL-PRS-RstdReferenceInfo

The current spec has not yet captured all above options. So we have the following proposal.

Proposal 1: Adopt the following text changes to allow all the possible options for a UE to determine a reference time.

==================TS 38.214 clause 5.1.6.4 unchanged part omitted===============

The UE may be indicated by the network that a DL PRS resource can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter DL-PRS-RstdReferenceInfo. The reference time indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters DL-PRS-expectedRSTD and DL-PRS-expectedRSTD-uncertainty. The UE expects the reference time to be indicated whenever it is expected to receive the DL PRS. This reference time provided by DL-PRS-RstdReferenceInfo may include an [ID], a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of PRS resource IDs from a single DL PRS resource set. The UE may use different DL PRS resources within a single DL PRS resource set provided by DL-PRS-RstdReferenceInfo or a different DL PRS resource set which can be any DL PRS resource set associated with the [ID] provided by DL-PRS-RstdReferenceInfo or different DL PRS resources associated with an [ID] other than the [ID] provided by DL-PRS-RstdReferenceInfo or different DL PRS resource sets associated with an [ID] other than the [ID] provided by DL-PRS-RstdReferenceInfo to determine the reference time for the RSTD measurement. If the UE chooses to use a different reference time than indicated by the network, then it is expected to report the [ID], the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference.

========================== Unchanged part omitted =========================

Conclusion

In this contribution, we discussed the maintenance of physical-layer procedures to support UE/gNB measurements.

Proposal 1: Adopt the following text changes to allow all the possible options for a UE to determine a reference time.

==================TS 38.214 clause 5.1.6.4 unchanged part omitted===============

The UE may be indicated by the network that a DL PRS resource can be used as the reference for the DL RSTD, DL PRS-RSRP, and UE Rx-Tx time difference measurements in a higher layer parameter DL-PRS-RstdReferenceInfo. The reference time indicated by the network to the UE can also be used by the UE to determine how to apply higher layer parameters DL-PRS-expectedRSTD and DL-PRS-expectedRSTD-uncertainty. The UE expects the reference time to be indicated whenever it is expected to receive the DL PRS. This reference time provided by DL-PRS-RstdReferenceInfo may include an [ID], a DL PRS resource set ID, and optionally a single DL PRS resource ID or a list of PRS resource IDs from a single DL PRS resource set. The UE may use different DL PRS resources within a single DL PRS resource set provided by DL-PRS-RstdReferenceInfo or a different DL PRS resource set which can be any DL PRS resource set associated with the [ID] provided by DL-PRS-RstdReferenceInfo or different DL PRS resources associated with a [ID] other than the [ID] provided by DL-PRS-RstdReferenceInfo or different DL PRS resource sets associated with a [ID] other than the [ID] provided by DL-PRS-RstdReferenceInfo to determine the reference time for the RSTD measurement. If the UE chooses to use a different reference time than indicated by the network, then it is expected to report the [ID], the DL PRS resource ID(s) or the DL PRS resource set ID used to determine the reference.

========================== Unchanged part omitted =========================

References

- Final Report of 3GPP TSG RAN WG1 #98bis v2.0.0(Chongqing, China, 14th–20th October 2019)

- Draft Report of 3GPP TSG RAN WG1 #100-e v0.2.0(Online meeting, 24th February -6th March 2020)

- 3GPP TS 38.214 V16.1.0, “Physical layer procedures for data (Release 16)”.