3GPP TSG RAN WG1 #100bis	R1- 2001813

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.5.1

Source: 	Quectel

Title: 	Remaining Issues on PDCCH Enhancements for URLLC

Document for:	Discussion and Decision

1	Introduction

In this document, the remaining issues on PDCCH enhancements, in particular the calculations of  MPDCCHtotal,(X,Y),μand CPDCCHtotal,(X,Y),μare addressed for Release 16 URLLC. Corresponding text proposals are also provided.

2         Interactions between URLLC and Cross-Carrier Scheduling

At the last e-meeting, the following working assumptions were made in URLLC email thread [100e-NR-L1enh_URLLC_PDCCH-03] for aligned span cases :

Working assumption:

If a UE is configured with Ncells,r16DL,(X,Y),μ downlink cells with Rel-16 PDCCH monitoring capability with an associated combination (X, Y) and SCS configuration µ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, the UE is not required to monitor more than CPDCCHtotal,(X,Y),μ non-overlapping CCEs per span on the active DL BWP(s) of scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells if the spans on all downlink cells from the Ncells,r16DL,(X,Y),μ downlink cells are aligned, where

CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j

  - is the number serving cells configured with Rel-16 PDCCH monitoring capability with SCS configuration j.

  - If a UE is configured with multiple carriers with a mix of Rel-15 and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by.

  - The associated combination (X, Y) is the combination (X, Y) associated with largest maximum number of CPDCCHmax,X,Y,μ , if the UE indicates a capability to monitor PDCCH according to multiple (X, Y) combinations and a configuration of search space sets to the UE results in a span pattern with a separation of any two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for two or more of the (X, Y) combinations.

If a UE is configured with Ncells,r16DL,(X,Y),μ downlink cells with Rel-16 PDCCH monitoring capability with an associated combination (X, Y) and SCS configuration µ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ PDCCH candidates per span on the active DL BWP(s) of scheduling cell(s) from the Ncells,r16DL,(X,Y),μ serving cells if the spans on all downlink cells from the Ncells,r16DL,(X,Y),μ downlink cells are aligned, where

MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j

  - is the number serving cells configured with Rel-16 PDCCH monitoring capability with SCS configuration j.

  - If a UE is configured with multiple carriers with a mix of Rel-15 and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by.

The associated combination (X, Y) is the combination (X, Y) associated with largest maximum number of MPDCCHmax,(X,Y),μ, if the UE indicates a capability to monitor PDCCH according to multiple (X, Y) combinations and a configuration of search space sets to the UE results in a span pattern with a separation of any two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for two or more of the (X, Y) combinations

In light of the working assumptions, the PDCCH candidates and non-overlapped CCEs are distributed according to the serving cell numerologies and associated combinations for the span-aligned case. The number of allocated PDCCH candidates and the number of non-overlapped CCEs are both proportionate to the number of serving cells using the same numerology and the same combination. This kind of partition makes much sense for multiple carrier operations with self-scheduling where a scheduled cell and its corresponding scheduling cell need not be differentiated. However, the descriptions in the working assumptions may not be well suited for multicarrier operations with cross-carrier scheduling, as shown by two examples respectively in Figure 1 and Figure 2.

Figure 1. Cross-carrier scheduling with serving cells provided different PDCCH monitoring capabilities

Figure 2. Cross-carrier scheduling with serving cells provided different combinations

As illustrated in Figure 1, the PDCCH monitoring capability provided for a scheduled cell may not be consistent with the monitoring capability provided for its scheduling cell. Although a scheduled cell may not assume to be configured with a PDCCH monitoring capability, similar ambiguity still exists when span-based PDCCH monitoring capability is configured for its scheduling cell, because by default a serving cell not provided PDCCHMonitoringCapabilityConfig shall assume slot-based monitoring. Moreover, as illustrated in Figure 2, the set of combinations provided for a scheduled cell may be different from the set of combinations provided for its scheduling cell. As the numbers of monitored PDCCH candidates and non-overlapped CCEs are expected to be shared among the scheduling cells with same PDCCH monitoring capability and same set of combinations, the inconsistency shown above may lead to mismatch between the allocated portion of PDCCH candidates and non-overlapped CCEs for a set of serving cells and the actual demand. Therefore, modifications and clarifications are needed for the abovementioned working assumptions. Two alternative ways could be considered to address this issue:

- Alt-1: Both the scheduling cell and scheduled cell for cross-carrier scheduling are restricted to be configured the same PDCCH monitoring capability (i.e., Rel-15 PDCCH monitoring capability or Rel-16 PDCCH monitoring capability) , and provided the same combination when Rel-16 PDCCH monitoring capability is configured,

- Alt-2: The calculations of MPDCCHtotal,(X,Y),μand CPDCCHtotal,(X,Y),μare based on the number of scheduled cells whose scheduling cells are configured with the same PDCCH monitoring capability and provided the same combination (X, Y) when Rel-16 PDCCH monitoring capability is configured.

As cross-carrier scheduling is well compatible with URLLC and is likely to benefit URLLC, Alt-1 will largely limit the use of cross-carrier scheduling for URLLC, in which ensuring scheduling flexibility is quite essential. In contrast, Alt-2 does not prohibit the use of cross-carrier scheduling with different PDCCH monitoring capabilities or with different combinations and maintains the scheduling flexibility with limited specification efforts. Therefore, Alt- 2 is preferred.

Proposal 1: The calculations of MPDCCHtotal,(X,Y),μand CPDCCHtotal,(X,Y),μare based on the number of scheduled cells whose scheduling cells are configured with the same PDCCH monitoring capability and provided the same combination (X, Y) when Rel-16 PDCCH monitoring capability is configured.

Accordingly, the proposed text changes are as follows:

******************************************* Start of TP  *******************************************

10.1	UE procedure for determining physical downlink control channel assignment

< Unchanged parts are omitted >

If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells with associated PDCCH candidates monitored using Rel-15 PDCCH monitoring capability in the active DL BWPs of the scheduling cell(s) using SCS configuration , where μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j  PDCCH candidates or more than CPDCCHtotal,slot,μ=Ncellscap⋅CPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) for the Ncells,0DL+Ncells,1DL downlink cells. If a UE is configured with downlink cells with scheduling cell(s) using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability,  Ncellscap is replaced by Ncells,r15cap-r16.

< Unchanged parts are omitted >

If a UE is configured only with Ncells,r16DL,μ downlink cells with associated PDCCH candidates monitored using Rel-16 PDCCH monitoring capability in the active DL BWPs in the scheduling cell(s) using SCS configuration μ, and with Ncells,r16DL,(X,Y),μ of the Ncells,r16DL,μ downlink cells with associated PDCCH candidates monitored in the scheduling cell(s) where combination X,Y is used and the spans are alignedμ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor more than MPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅MPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j  PDCCH candidates or more than CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j non-overlapped CCEs per span on the active DL BWP(s) of scheduling cell(s) for the Ncells,r16DL,(X,Y),μ downlink cellsNcells,r16DL,(X,Y),μNcells,r16DL,jj. If a UE is configured with downlink cells with scheduling cell(s) using both Rel-15 PDCCH monitoring capability and Rel-16 PDCCH monitoring capability,  Ncellscap-r16 is replaced by Ncells,r16cap-r16.

< Unchanged parts are omitted >

******************************************** End of TP  *******************************************

3         Interactions between URLLC and eMIMO

In order to support multiple DCIs scheduling multiple PDSCHs, a blind decoding factor, namely BDFactor, is introduced by eMIMO. The maximum number of blind decodings and non-overlapped CCEs can be scaled by setting the BDFactor other than 1. The serving cells are grouped as well according to the number of provided CORESET resource pools for the derivation of Ncellscap so as the scaling operation can be absorbed. One remaining open issue is whether to support span based PDCCH monitoring and multiple DCIs scheduling multiple PDSCHs in a serving cell simultaneously. Increasing spectrum efficiency is a major motivation to support multiple DCIs scheduling multiple PDSCH, while lower latency and lower PDCCH blocking rate are targeted by the span based PDCCH design. It is obvious that the motivations for these two designs deviate a lot. On the other hand, the UE PDCCH monitoring capability may be required to be largely increased in order to support either of multiple DCIs scheduling multiple PDSCHs or span based monitoring. It is expected to bring overwhelming burden to UE implementations if both of them are supported in a serving cell simultaneously, but with few foreseeable use cases.

Proposal 2: Multiple DCIs scheduling multiple PDSCHs and Span based PDCCH monitoring are not supported simultaneously in a serving cell.

Corresponding proposed text changes are as follow:

******************************************* Start of TP  *******************************************

10	UE procedure for receiving control information

< Unchanged parts are omitted >

A UE is not expected to be provided CORESETPoolIndex with a value 0 for a first CORESET and with a value 1 for a second CORESET on any DL BWP of a serving cell using Rel-16 PDCCH monitoring capability.

< Unchanged parts are omitted >

******************************************** End of TP  *******************************************

For a serving cell using Rel-15 PDCCH monitoring capability, whether to support multiple DCIs scheduling multiple PDSCHs should be up to UE capability and network configuration. The serving cell grouping according to the number of CORESET resource pool indices could be reused when Rel-15 PDCCH monitoring capability is configured. Following the similar logics presented in Section 2, serving cells are grouped according to the number of CORESET resource pool indices configured for the scheduling cell(s) rather than the serving cells themselves when Rel-15 PDCCH monitoring capability is configured for the scheduling cell(s). Separate scaling factor reports (i.e., R and Rr15 could be different) or a same scaling factor report (i.e., a same R) for Rel-15 PDCCH monitoring capability could be considered.

Proposal 3: Whether multiple DCIs scheduling multiple PDSCHs is applied for a serving cell using Rel-15 PDCCH monitoring capability is up to UE capability and network configuration.

The following text changes reflect the proposal.

******************************************* Start of TP  *******************************************

10	UE procedure for receiving control information

< Unchanged parts are omitted >

If a UE can support

-	a first set of Ncells,0DL serving cells with scheduling cell(s) where the UE is either not provided CORESETPoolIndex or is provided CORESETPoolIndex with a single value for all CORESETs on all DL BWPs of each scheduling of the first set of serving cells, and

-	a second set of Ncells,1DL serving cells with scheduling cell(s) where the UE is provided CORESETPoolIndex with a value 0 for a first CORESET and with a value 1 for a second CORESET on any DL BWP of each scheduling cell of the second set of serving cells

the UE determines, for the purpose of reporting pdcch-BlindDetectionCA, a number of serving cells as Ncells,0DL+R⋅Ncells,1DL where R is either a value reported by the UE or R=TBD if the UE does not report a value of R.

If a UE indicates in UE-NR-Capability a carrier aggregation capability larger than 4 serving cells and the UE is not provided PDCCHMonitoringCapabilityConfig for any downlink cell or if the UE is provided PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability for all downlink cells where the UE monitors PDCCH, the UE includes in UE-NR-Capability an indication for a maximum number of PDCCH candidates and for a maximum number of non-overlapped CCEs the UE can monitor per slot when the UE is configured for carrier aggregation operation over more than 4 cells. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot that corresponds to  downlink cells, where

-	 is Ncells,0DL+R⋅Ncells,1DL if the UE does not provide pdcch-BlindDetectionCA where Ncells,0DL+Ncells,1DL is the number of configured downlink serving cells

-	otherwise,  is the value of pdcch-BlindDetectionCA

< Unchanged parts are omitted >

If a UE indicates in UE-NR-Capability-r16 a carrier aggregation capability larger than X downlink cells, the UE includes in UE-NR-Capability-r16 an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs that the UE can monitor per span when the UE is configured for carrier aggregation operation over more than X downlink cells. When a UE is not configured for NR-DC operation and the UE is provided PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability for all downlink cell where the UE monitors PDCCH, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per span that corresponds to Ncellscap-r16 downlink cells, where

-	Ncellscap-r16 is the number of configured downlink cells if the UE does not provide pdcch-BlindDetectionCA-r16

-	otherwise, Ncellscap-r16 is the value of pdcch-BlindDetectionCA-r16

If a UE indicates in UE-NR-Capability-r15 or in UE-NR-Capability-r16 a carrier aggregation capability larger than Y downlink cells or larger than Z downlink cells, respectively, the UE includes in UE-NR-Capability-r15 or in UE-NR-Capability-r16 an indication for a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs the UE can monitor for downlink cells with PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability or for downlink cells with PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability when the UE is configured for carrier aggregation operation over more than Y downlink cells or over more than Z downlink cells, respectively, and with at least one downlink cells from the Y downlink cells and at least one downlink cell from the Z downlink cells. When a UE is not configured for NR-DC operation, the UE determines a capability to monitor a maximum number of PDCCH candidates and a maximum number of non-overlapped CCEs per slot or per span that corresponds to Ncells,r15cap-r16 downlink cells or to Ncells,r16cap-r16 downlink cells, respectively, where

-	Ncells,r15cap-r16 is Ncells,r15,0DL+R⋅Ncells,r15,1DL if the UE does not provide pdcch-BlindDetectionCA-r15 where Ncells,r15,0DL+Ncells,r15,1DL is the number of configured downlink serving cells with scheduling cell(s) using PDCCHMonitoringCapabilityConfig = R15 PDCCH monitoring capability

-	otherwise, Ncells,r15cap-r16 is the value of pdcch-BlindDetectionCA-r15

and

-	Ncells,r16cap-r16 is the number of configured downlink cells with scheduling cell(s) using PDCCHMonitoringCapabilityConfig = R16 PDCCH monitoring capability if the UE does not provide pdcch-BlindDetectionCA-r16

-	otherwise, Ncells,r16cap-r16 is the value of pdcch-BlindDetectionCA-r16

the UE determines, for the purpose of reporting pdcch-BlindDetectionCA-r15, a number of serving cells as Ncells,r15,0DL+R⋅Ncells,r15,1DL.

< Unchanged parts are omitted >

******************************************** End of TP  *******************************************

4         PDCCH monitoring for non-aligned span

For PDCCH candidate and non-overlapped CCE allocation in non-aligned span cases, a proposal gathered supports from a number of companies at the last RAN1 e-meeting was discussed, but eventually not agreed due to the concerns from some other companies on the implementation complexity. The proposal is copied below:

If a UE is configured with Ncells,r16DL,(X,Y),μ downlink cells with Rel-16 PDCCH monitoring capability with an associated combination (X, Y) and SCS configuration µ, where μ=01Ncells,r16DL,μ>Ncellscap-r16, the UE is not required to monitor more than CPDCCHtotal,(X,Y),μ non-overlapping CCEs for any set of spans across the active DL BWP(s) of scheduling cell(s) from the Ncells,r16DL,(X,Y),μ downlink cells if the spans on different downlink cells from the Ncells,r16DL,(X,Y),μ downlink cells are not aligned, with at most one span per scheduling cell for each set, where

CPDCCHtotal,(X,Y),μ=Ncellscap-r16⋅CPDCCHmax,(X,Y),μ⋅Ncells,r16DL,(X,Y),μj=01Ncells,r16DL,j

  - is the number serving cells configured with Rel-16 PDCCH monitoring capability with SCS configuration j.

  - If a UE is configured with multiple carriers with a mix of Rel-15 and Rel-16 PDCCH monitoring capability, Ncellscap-r16 is replaced by.

  - The associated combination (X, Y) is the combination (X, Y) associated with largest maximum number of CPDCCHmax,X,Y,μ , if the UE indicates a capability to monitor PDCCH according to multiple (X, Y) combinations and a configuration of search space sets to the UE results in a span pattern with a separation of any two consecutive PDCCH monitoring spans that is equal to or larger than the value of X for two or more of the (X, Y) combinations.

The restriction “for any set of spans” is strong and conservative which can indeed guarantee the allocated PDCCH candidates and non-overlapped CCEs not overflowing the UE capability. On the other hand, due to increased number of possible span combinations, it may be too complicated to attain an optimal or sub-optimal configuration even at the gNB side. In the sourcing company’s opinion, the non-aligned spans are similar to the unaligned slots due to different numerologies. As hard partitioning is applied between serving cells with different numerologies for MPDCCHtotal,(X,Y),μand CPDCCHtotal,(X,Y),μcalculations, i.e., no sharing between two cells with different numerologies, further optimization could not be pursued either for the non-aligned span cases. In this sense，it is proposed to consider a similar method as different numerologies for the non-aligned spans, e.g., the number of PDCCH candidates and the number of non-overlapped CCEs are not shared between two scheduling cells where the spans are not-aligned.

Proposal 4: The sharing between any two scheduling cells where the spans are not aligned, for the number of PDCCH candidates and the number of non-overlapped CCEs, is not supported.

The main change to the specification according to Proposal 4 is to modify the definition of Ncells,r16DL,(X,Y),μ to the number of scheduled cells with scheduling cell(s) where the combination (X,Y) is used and the spans are aligned, for the calculations of MPDCCHtotal,(X,Y),μand CPDCCHtotal,(X,Y),μ, as shown by the TP in Section 2. A serving cell without span alignment with any other serving cell means Ncells,r16DL,(X,Y),μ is equal to 1.

5         Conclusions

In this contribution, the partitioning of PDCCH candidates and non-overlapped CCEs across serving cells are discussed and the followings are proposed:

Proposal 1: The calculations of MPDCCHtotal,(X,Y),μand CPDCCHtotal,(X,Y),μare based on the number of scheduled cells whose scheduling cells are configured with the same PDCCH monitoring capability and provided the same combination (X, Y) when Rel-16 PDCCH monitoring capability is configured.

Proposal 2: Multiple DCIs scheduling multiple PDSCHs and Span based PDCCH monitoring are not supported simultaneously in a serving cell.

Proposal 3: Whether multiple DCIs scheduling multiple PDSCHs is applied for a serving cell using Rel-15 PDCCH monitoring capability is up to UE capability and network configuration.

Proposal 4: The sharing between any two scheduling cells where the spans are not aligned, for the number of PDCCH candidates and the number of non-overlapped CCEs, is not supported.

The corresponding proposed texts are also included in this contribution.

6         References

[1]    R1-2001409,	Summary of email discussion [100e-NR-L1enh_URLLC_PDCCH-03] on remaining issue on enhanced PDCCH monitoring capability, Huawei