3GPP TSG RAN WG1 Meeting #100bis-e	R1-2002049

e-Meeting, April 20th-30th, 2020

Agenda Item:	7.2.8.4

Source:	Futurewei

Title:	Remaining Details on Measurement Procedures

Document for:	Discussion and decision

# Introduction

# At RAN1#99, RAN1 completed the standardization work for NR Positioning Support. During the RAN1#100-e meeting, numerous CRs proposals were discussed. In this contribution, we revisit and provide our views on one of the remaining proposals that were left out from previous discussion on the Procedures for Positioning [1].

# Discussions

In Section 5 of [1], RAN1 discussed the terminology nr-DL-PRS-RxBeamIndex-r16 that was used in TS37.355 (LTE Positioning Protocol (LPP)) by RAN2. The related CR was approved in [2].

## nr-DL-PRS-RxBeamIndex

For the support of DL-AoD positioning, the following measurements report are considered in RAN2 [2]:

# NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

trp-ID-r16							TRP-ID-r16			OPTIONAL,

nr-DL-PRS-ResourceId-r16			NR-DL-PRS-ResourceId-r16	 OPTIONAL,

nr-DL-PRS-ResourceSetId-r16			NR-DL-PRS-ResourceSetId-r16 OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

nr-PRS-RSRP-Result-r16				INTEGER (FFS)			OPTIONAL, -- Need RAN4 inputs on value range

nr-DL-PRS-RxBeamIndex-r16			INTEGER (1..8),

nr-TimingMeasQuality-r16				NR-TimingMeasQuality-r16,

nr-DL-Aod-AdditionalMeasurements-r16		NR-DL-AoD-AdditionalMeasurements-r16,

...

}

The parameter nr-DL-PRS-RxBeamIndex, with a size of 8, it limits the maximum receive beam to 8. However, the concept of DL PRS-RxBeamIndex has not been discussed and is not defined in any RAN1 or RAN2 specifications. Some questions and clarifications that needed would be:

- What does it correspond to i.e. physical beam or certain reference signal?

- Is there a one-to-one mapping of the physical beam to the beam index?

- Can the number of physical receive beams larger than the max number beam index of 8?

In [3], it was proposed that the nr-DL-PRS-RxBeamIndex be interpreted based on the following proposal:

Proposal 5: It is RAN1 understanding, nr-DL-PRS-RxBeamIndex should be interpreted as the group ID of the RSRP measurements, where the RSRP measurements in a group are received with the same Rx beam.

If there are different understanding and answers to the questions above, other interpretation may exist in RAN1. It is important that RAN1 discuss and define this parameter to ensure correct use of this parameter.

Proposal: RAN1 should discuss and decide on the definition of nr-DL-PRS-RxBeamIndex.

# Conclusions

# In this contribution, we propose the following:

Proposal: RAN1 should discuss and decide on the definition of nr-DL-PRS-RxBeamIndex.

# References

R1-2001168, “Summary for 7.2.8.4 - PHY procedures for positioning measurements,” Qualcomm.

RP-200345, “RAN2 CRs to NR positioning support,” RAN2.

R1-2000193, "Maintenance of physical layer procedures to support positioning measurements", Huawei, HiSilicon.