3GPP TSG RAN WG1 Meeting #100bis                                   R1-2002069

e-Meeting, April 20th – 30th, 2020

Source:	CATT

Title:	Discussion of UE features for NR V2X

Agenda Item:	7.2.11.4

Document for:	Discussion and Decision

- Introduction

NR Rel-16 UE features were under email discussion. In this contribution, we provide our views on UE features for 5G_V2X_NRSL based on the latest version[1].

- Discussion

FG 15-1: Receiving NR sidelink [by pre-configuration]

[FG 15-1a]: Receiving NR sidelink configured by NR Uu

Regarding the FG 15-1 and 15-1a, since NR V2X should be at least deployed in ITS dedicated carrier, and the UE should support working in OOC scenario at least. Therefore, it is preferred to separate FGs between pre-configuration and configured by NR Uu. And both FG 15-1 and FG 15-1a shall share same component definitions.

Regarding to the supported SCS candidate set (component-8), it is better to follow NR Uu design principle, i.e., {15KHz, 30KHz} shall be mandatory for FR1, and {60KHz, 120KHz} shall be mandatory for FR2.

Proposal 1: FG 15-1 and FG 15-1a shall be separated FGs, and share the same components.

Proposal 2: Regarding the component-8 in FG 15-1 and 15-1a, the supported SCS candidate value set is shown as following:

- FR1: {15KHz, 30KHz}, or {15KHz, 30KHz, 60KHz }

- FR2: {60KHz, 120KHz}

FG 15-3: Transmitting NR sidelink mode 2 configured by [NR Uu or] preconfiguration

Similar as that in FG 15-1 and FG 15-1a, it is also better to separate the FGs between pre-configuration and configured by NR Uu.

Proposal 3: FG 15-3 shall be separated into FG 15-3 and 15-3a for “by pre-configuration”and “by NR Uu” respectively.

FG 15-5: Sidelink congestion control

Regarding to the mandatory/optional, since sidelink congestion control is a resource pool specific configuration, if congestion control is enabled for a resource pool, and UEs supporting congestion control and UEs not supporting congestion control share this pool, it is unfair to the UEs supporting congestion control. It could either be marked as a basis feature group, or moved it into 15-3 as components.

CBR measurement report to gNB is related to Uu behaviour, it is not necessary to include it into this FG.

Proposal 4: FG 15-5(sidelink congestion control) shall be a basic FG.

Proposal 5: CMR measurement report to gNB(component-1) is removed from FG 15-5.

FG 15-6: Short-term time-scale TDM for in-device coexistence

Regarding the FFS part on whether define a set of candidate processing time value for inter-RAT collision resolution. We think it is better for UE to report the processing time, otherwise it would be difficult to test the UE’s behaviour.

Proposal 6: In FG15-6, a new component shall be introduced for reporting UE processing time

FG 15-9: Transmitting LTE sidelink mode 4 configured by NR Uu or preconfiguration

Remove “or pre-configuration” in feature group description. Pre-configuration has been supported in LTE mode 4.

Proposal 7: FG 15-9 shall remove “or pre-configuration” in feature group description

FG 15-11: PSFCH format 0

Since the component-4(Sidelink HARQ-ACK forwarding to gNB) is only related to mode 1 resource allocation, then it is better to move it into 15-2.

Proposal 8: The component-4(Sidelink HARQ-ACK forwarding to gNB) in FG 15-11 shall be moved to FG15-2.

- Conclusion

In this contribution, the UE features for 5G_V2X_NRSL are discussed. Particularly, we have following proposals:

Proposal 1: FG 15-1 and FG 15-1a shall be separated FGs, and share the same components.

Proposal 2: Regarding the component-8 in FG 15-1 and 15-1a, the supported SCS candidate value set is shown as following:

- FR1: {15KHz, 30KHz}, or {15KHz, 30KHz, 60KHz }

- FR2: {60KHz, 120KHz}

Proposal 3: FG 15-3 shall be separated into FG 15-3 and 15-3a for “by pre-configuration”and “by NR Uu” respectively.

Proposal 4: FG 15-5(sidelink congestion control) shall be a basic FG.

Proposal 5: CMR measurement report to gNB(component-1) is removed from FG 15-5.

Proposal 6: In FG15-6, a new component shall be introduced for reporting UE processing time

Proposal 7: FG 15-9 shall remove “or pre-configuration” in feature group description

Proposal 8: The component-4(Sidelink HARQ-ACK forwarding to gNB) in FG 15-11 shall be moved to FG15-2.

- References

- R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, AT&T, NTT DOCOMO