3GPP TSG-RAN WG1 Meeting #100bis	R1- 2002372

e-Meeting, April 20th – 30th 2020

Agenda Item:	7.2.11.1

Source:	Ericsson

Title:	Two-step RACH UE features

Document for:	Decision

# 1	Introduction

In this contribution, we discuss the UE features for two step RACH.  In [1], the outcome of the email discussion for the UE features for Release-16 is captured. Here we discuss updates to [1], in particular the level of detail needed in the basic capability 9-1.

# 2	Discussion

Regarding the basic two-step RACH capability 9-1, alternative 1 is rather more detailed than we see needed.  While we appreciate the effort to fully document the feature, we are concerned that an exhaustive listing like this may be counter productive, since there can be differences in interpretation, and there is the danger that some aspect in the specs not listed will then be interpreted as not needed for the feature.  We also observe that other features do not seem to be following this approach, and it is essential that features have a common method of describing features for the feature lists to be interpreted correctly. Therefore, we support the rapporteur’s suggestion to go with the majority view and to select Alt 2.

Regarding the specifics of Alt-2, the overall content seems fine. We suggest to clarify component 2), presuming that HARQ-ACK feedback is intended:

2)	MsgB monitoring, reception, and HARQ-ACK feedback

Our understanding is that the Alt-1 components are intended to differentiate over functionality that already exists in Rel-15.  However, a number of these may need further clarification if we go into this level of detail by pursuing Alt-1.  For example, how MsgA PUSCH transmission is different than Rel-15 PUSCH, how receiving MsgB PDSCH is different from Rel-15 PDSCH, how MsgA PRACH transmission configuration is different than Rel-15 PRACH configurations, etc. We will defer discussion of these details, addressing them later if Alt-1 remains a candidate.

Alt-1 components require further clarification, including how they differ from Rel-15 behavior.

Update two step RACH basic UE capability 9-1 to be terse and to clarify MsgB feedback as proposed in Table 1.

Table 1: Proposed updates for UE features related two step RACH.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | Alt 2 simplified basic feature group: - MsgA PRACH and PUSCH transmission - MsgB monitoring, reception, and HARQ-ACK feedback - Power control for MsgA PRACH, MsgA PUSCH, and PUCCH for HARQ-ACK feedback to a MsgB FFS: if any of the components need to be more specific, i.e. the following components in Alt. 1 can be further discussed. [Alt 1 detailed feature group]: - [RACH type selection based on a SSB-based RSRP threshold] - [Separately configured ROs not applicable to 4-step RO configuration] - [Same ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration] - [Maximum two MsgA PUSCH configurations in an UL BWP from UE perspective] - [Validation of MsgA PRACH and PUSCH] - [Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH] - [MsgA PUSCH transmission including scrambling, DMRS sequences and ports, RV, etc] - [MsgA open loop power control] - [Monitoring of MsgB with PDCCH addressed to msgB-RNTI or C-RNTI, and receiving MsgB including SuccessRAR, fallbackRAR, and backoff indication] - [Support PUCCH transmission for HARQ-ACK feedback to a MsgB] - [MsgA PRACH configuration and preamble formats] - [PUCCH power control for HARQ-ACK feedback to a MsgB] - [Minimum TX gap between PRACH and PUSCH (for both TDD and FDD, FR1 and FR2, single CC and intra-band CA) as specified in Rel-15] - [Minimum TX gap between last DL SSB reception symbol and PRACH (TDD. FR1 and FR2, single CC and intra-band CA) as specified in Rel-15] - [MsgA PRACH and PUSCH transmissions in different PRACH and PUSCH slots] - [Minimum TX gap between the last symbol of MsgB PDSCH and the first symbol of PUCCH carrying HARQ-ACK to MsgB PDSCH] - [2-step RACH operation in RRC_IDLE/INACTIVE/CONNECTED state] - [Intra-slot frequency hopping for MsgA PUSCH] |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-2 | Supported 2 symbols DMRS for msgA PUSCH | Supported 2 symbols DMRS for msgA PUSCH (‘len2’) | 9-1, 2-18 | Yes | N/A | If UE does not support ‘len2’, and if msgA-maxLength is configured as ‘len2’, the UE cannot use 2-step RACH resources | per UE | N/A | Yes |  | Shall be aligned with 2-18 | Conditionallymandatory for UE supporting both 9-1 and 2-18 |
|  | 9-3 | Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel MsgA and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell | 9-1 | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | per band combination | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-4 | MsgA operation in a band combination including SUL | MsgA operations in a band combination including SUL | 9-1, 6-16 | Yes | N/A | UE does not support msgA operations in a band combination including SUL | per band combination | N/A | N/A |  |  | Optional with capability signalling |

# Conclusion

In this contribution, we have discussed updates to the outcome of the RAN1#100-e email discussion on UE features for two step RACH in [1], in particular the level of detail needed in the basic capability 9-1.  In summary, we made the following observation and proposal:

Observation 1	Alt-1 components require further clarification, including how they differ from Rel-15 behavior.

Proposal 1	Update two step RACH basic UE capability 9-1 to be terse and to clarify MsgB feedback as proposed in Table 1.

# References

R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, AT&T, NTT DOCOMO INC., RAN1 #100-e, March 2020