3GPP TSG RAN WG1 #100bis-e		                                                 R1-2002482

e-Meeting, April 20th – 30th, 2020

Agenda item: 	7.2.11.5

Source: 	Nokia, Nokia Shanghai Bell

Title:	On UE features for URLLC/IIOT

Document for:	Discussion and Decision

# Introduction

The Rel-16 UE feature e-mail discussions have gone through two phases in RAN1 e-mail reflector, with the current proposals available in [1]. In this contribution we present our further views regarding the UE features for URLLC/IIOT.

# Discussion

The detailed comments are provided for the URLLC feature list (i.e. 11-X) below following the numbering of features in [1], and which can be found in the Annex for reference:

- 11-1a: We do not see a need to split capability of UL grants and DL assignments.

- 11-2: We could be fine with merging components 1 & 3, and components 2 & 4

- 11-2b: As noted earlier, we think that the UE should be able report more than one valid combination of R15 & R16 carriers to be able to operate the UE efficiently (e.g. 2 or 3 combinations allowed)

- 11-3: We see no need for CBG-related restrictions, and hence we are fine with removing component 3..

- 11-4:

  - 11-4 / 12-1 Merge: These feature groups are strongly related. One cannot operate 11-4 (having PUSCH & 2 CBs of different HARQ-Ack priorities) without the related multiplexing / prioritization which is part of 12-1. Propose to combine 11-4 and 12-1 into a single feature group.

  - Rapporteur question: Keep component 4 - no need for separate capability here, same applies to the related FFS for 11-4a.

  - 11-4 / 11-4X (Rapporteur question): No need to have separate capability of slot or sub-slot based CB – i.e. we don’t see a need to split 11-4X from 11-4. Please note that for subslot HARQ-ACK we have the independent capability 11-3 already. From our understanding, a UE supporting 11-3 and 11-4 should support slot or subslot based codebook for either of the two codebooks.

  - No need identified for separate capability 6.

- 11-5:

  - Rapporteur Question a): We still see no need for capabilities on PUSCH per TB per slot, as the Rel-15 capabilities are already in place and can be applied directly (with the understanding of one TB per Rel-15 PUSCH).

  - Rapporteur Question b): See no need for it, we can use the Rel-15 indication of UE proc cap 1 & 2.

  - Rapporteur Question c): We do not see a difference between CG and DG here, therefore no need for separate capability. Of course the UE would need to indicate CG support in general (i.e. at least Rel-15 CG, i.e. 5-19 / 5-20).

  - Rapporteur Question d): No need for separate capability for SFI handling. Naturally the support of dynamic SFI would be subject to the support of SFI handling (i.e. Rel-15 capability 3-6).

  - Rapporteur Question e): we don’t see a need for separate capability here. Specifically as for PUSCH repetition Type B there is any now support for intra-repetition hopping, but the two defined hopping schemes follow the Rel-15 inter-slot hopping in a way, that each repetition is having a different hop. So basically a UE supporting inter-slot FH support (i.e. 5-10), should also be able to support the FH schemes for PUSCH repetition Type B.

  - Rapporteur Question f): we are fine with rapporteur proposal.

- 11-6: We do not see a need for separate capability on the number of supported repetitions. A similar approach has been followed in Rel-15 already.

- 11-7:

  - Rapporteur proposal on the monitoring: We are fine to have a note there. Anyhow, it should be clear that if UE does not support more than one PDCCH occasion per slot, then this would equally apply for UL CI monitoring.

  - Rapporteur Question 2: We don’t see a need for separate capability for cross-carrier UL CI indication.

  - Addition of component 4 could be fine.

  - 11-7a: we have not identified the need for separate capability here for pa-PhaseDiscontinuityImpacts. A UE indicating 11-7 and 6-23, should automatically support also 11-7a. If needed, this could be spelled out as a new component in 11-7 if needed, but clearly no independent reporting is seen as needed.

- 11-9: We are fine with having UE capability for the new components 2 & 3, but we think the UE capability signaling should be on the number of ACTIVE CG configurations (and not the configured ones). Especially when operating with several UE BWPs, this will make a clear difference in operation.

- 11-9a: We could be fine to request a UE supporting 11-9a to also need to indicate 11-10/11-11.

- 11-10/11-11: Propose to merge in a single feature group.

The detailed comments are provided for the IIoT feaure list (i.e. 12-X) below following the numbering of features in [1], and which can be found in the Annex for reference:

- 12-1 /11-4 Merge: These feature groups are strongly related. One cannot operate 11-4 (having PUSCH & 2 CBs of different HARQ-Ack priorities) without the related multiplexing / prioritization which is part of 12-1 and vice versa. Propose to combine 11-4 and 12-1 into a single feature group.

# Summary

In this contribution we presented our proposals on UE features for URLLC/IIOT.

# References

[1] R1-2001484, “RAN1 UE features list for Rel-16 NR after RAN1#100-E”, AT&T, NTT DOCOMO, April 2020

# Annex

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 11. NR_L1enh_URLLC | 11-1 | Monitoring DCI format 1_2 and DCI format 0_2 | 1) Supports monitoring DCI format 1_2 for DL scheduling 2) Supports monitoring DCI format 0_2 for UL scheduling |  | Yes | N/A |  | Per UE | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] |  | Optional with capability signalling |
|  | 11-1a | Monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 1) Supports monitoring both DCI format 0_1/1_1 and DCI format 0_2/1_2 in the same search space | 11-1 | Yes | N/A |  | Per UE | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2 ] | FFS: Whether to split 11-1a into two rows as below: 11-1a: DCI format 1_2 with DCI format 1_1 in the same search space 11-1b: DCI format 0_2 with DCI format 0_1 in the same search space | Optional with capability signalling |
|  | 11-2 | Rel-16 PDCCH monitoring capability | 1) Supports the limit C on the maximum number of non-overlapped CCEs for channel estimation per PDCCH monitoring span for combination (X, Y, ) 2) If UE reports the support of more than one combination of C(X, Y) for a given SCS, and if multiple combinations of C(X, Y) are valid for the span pattern, the maximum value of C of the valid combinations is applied 3) Supports the limit M on the maximum number of monitored PDCCH candidates per PDCCH monitoring span for combination (X, Y, ) 4) If UE reports the support of more than one combination of M(X, Y) for a given SCS, and if multiple combinations of M(X, Y) are valid for the span pattern, the maximum value of M of the valid combinations is applied 5) Capability on the number of CCs with Rel-16 PDCCH monitoring capability on all the serving cells. | 3-5b | Yes | N/A |  | [FSPC] FFS: Compoent 5) reported per UE | [N/A] | [N/A] |  | This capability is necessary for SCS 15 kHz and 30 kHz. FFS: Adding a component for “supported combination(s) (X, Y, ), which may depend on how to report C, M and (X, Y, ) A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #1; A list of separate UE capabilities C(X, Y, ), M(X, Y, ) for processing capability #2; For component 5), if UE supports carrier aggregation with more than [x] DL carriers with Rel-16 PDCCH monitoring capability on all the carriers, UE should report this capability. Value of x (can be < 4) is TBD. FFS: Whether to merge component 1) and 3), and accordingly merge component 2) and 4) FFS：Whether to add a capability for supporting 3 unicast PDSCH/PUSCH per slot separately for each minimum processing capability to match the number of spans for (4,3) pair | Optional with capability signalling Candidate value set for (X, Y): {(7, 3), (4, 3), (2, 2)} The value of C for combination (7, 3) for 15 kHz and 30 kHz is 56 FFS the value of C for combination (4, 3) and (2, 2) FFS the value of M for combination (7, 3), (4, 3) and (2, 2) Candidate value for component 5): { x, x+1, …, 16} |
|  | 11-2b | Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells | [Support Rel-15 monitoring capability and Rel-16 monitoring capability on different serving cells] - Capability on the number of CCs with Rel-15 PDCCH monitoring capability - Capability on the number of CCs with Rel-16 PDCCH monitoring capability | 11-2 | Yes | N/A |  | [Per UE] | [No] | TBD |  | Capability on the number of CCs with Rel-15 PDCCH monitoring capability can be smaller than 4 CCs; Capability on the number of CCs with Rel-16 PDCCH monitoring capability can be smaller than 4 CCs; The summation of the minimum of the capability on the number of CCs with Rel-15 PDCCH monitoring capability and the minimum of the capability on the number of CCs with Rel-16 PDCCH monitoring capability is not larger than 4 [Rel-15 monitoring capability here is subjected to the capability of FG 3-1, FG 3-2 and FG 3-5b.] | Optional with capability signalling |
|  | 11-3 | More than one PUCCH for HARQ-ACK transmission within a slot | 1) Supports sub-slot based HARQ-ACK feedback procedure. • A UL slot consists of a number of sub-slots. No more than one transmitted PUCCH carrying HARQ-ACKs starts in a sub-slot.• At least one sub-slot configuration for PUCCH can be UE specifically configured to a UE. • Supports a single configuration for PUCCH resource for all sub-slots in a slot. The starting symbol of a PUCCH resource is defined with respect to the first symbol of sub-slot. Any sub-slot PUCCH resource is not across sub-slot boundaries. 2) Supported sub-slot configuration [3) Supported combinations of (A, B), where A is the minimum gap between sub-slots containing actual PUCCH transmissions measured from beginning to beginning of the sub-slots, including across slots, and B is the sub-slot duration, with both A and B in units of symbols] |  | Yes | N/A |  | [Per UE] | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | Candidate value set for component 2): { 7-symbol*2, 2-symbol*7 and 7-symbol*2} [Candidate value set for component 3): (A, B) = {(7, 7), (4, 2) and (7, 7), (2, 2) and (7, 7)}] FFS: Whether to keep component 3) and accordingly the above note for component 3) FFS: Any relationship between FG 11-3 and CBG-based PUSCH with minimum processing time capability #2? | Optional with capability signalling |
|  | 11-4 | Two HARQ-ACK codebooks [with up to one sub-slot based HARQ-ACK codebook] simultaneously constructed for supporting PDSCH reception with different priorities at a UE | 1) Supports two HARQ-ACK codebooks with different priorities to be simultaneously constructed [with the restriction up to one sub-slot based HARQ-ACK codebook]. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. [4) Supports a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured per BWP] 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. [6) Supported maximum number of actual PUCCH transmissions for HARQ-ACK within a slot] |  | Yes | N/A |  | [Per UE] FFS: FS | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: Whether and how to combine FG 11-4 and FG 12-1 FFS: For component 4), whether to separate DL priority and UL priority, and whether to separate DCI format 0_1/1_1 and DCI format 0_2/1_2 | Optional with capability signalling |
|  | [11-4x] | [Two sub-slot based HARQ-ACK codebooks simultaneously constructed for supporting PDSCH reception with different priorities at a UE]. | 1) Supports two sub-slot based HARQ-ACK codebooks with different priorities to be simultaneously constructed. 2) Supports separate PUCCH configuration for different HARQ-ACK codebooks 3) Supports 2-level priority of HARQ-ACK for dynamically scheduled PDSCH and SPS PDSCH. 4) Supports a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when only DCI format 0_1/1_1 is configured or only DCI format 0_2/1_2 is configured in USS per BWP 5) Supports separate configuration of parameters PDSCH-HARQ-ACK-Codebook, UCI-OnPUSCH and ‘codeBlockGroupTransmission” for different HARQ-ACK codebooks. | 11-3 | Yes | N/A |  | [Per UE] | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: whether to add this FG and the contents of this FG | Optional with capability signalling |
|  | 11-4a | Monitoring a DCI format (from the formats 0_1/1_1/0_2/1_2) scheduling PDSCH with different HARQ-ACK priorities or PUSCH with different priorities when both DCI format 0_1/1_1 and DCI format 0_2/1_2 are configured to be monitored per BWP |  | 11-1a, 11-4 | Yes | N/A | FFS | Per UE | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: Whether to split 11-4a into two rows as below: 11-4x: DL priority indication in DCI with mixed DCI formats 11-4y: UL priority indication in DCI with mixed DCI formats | Optional with capability signalling |
|  | 11-5 | PUSCH repetition type B | 1) For a transport block, one dynamic UL grant or one configured grant schedules two or more PUSCH repetitions that can be in one slot, or across slot boundary in consecutive available slots. 2) Dynamic indication of the nominal number of repetitions in the DCI scheduling dynamic PUSCH. [3) The time window within which valid symbols are used for transmission is L*K, starting from the first symbol indicated by the SLIV in TDRA field.] 4) PUSCH repetition type B is supported for DCI format 0_1 and DCI format 0_2 (for DG and type 2 CG). 5) S and L are separately indicated (4-bit for S and 4-bit for L). L <= 14. [6) TBS is determined based on L indicated in TDRA table entry reusing Rel-15 mechanism.] [7) Handling of interaction with DL/UL directions depending on whether dynamic SFI is configured or not, including both cases with and without higher layer parameter InvalidSymbolPattern configured] [8) Supported maximum number of actual repetitions within a slot] [9) Supported PUSCH hopping scheme] |  | Yes | N/A |  | [Per UE] FFS: Per band | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | Candidate value for component 8): {2, 3, 4, 7, [8], [12]} FFS: Whether to add new feature groups for the total number of unicast PUSCHs for different TBs per slot per CC, or just add some note here with an example below: [The total number of unicast PUSCHs for different TBs per slot per CC is subjected to the capability reported by FG 5-12, 5-12a, 5-12b, 5-13d, 5-13e and 5-13f] FFS: Whether to set separate UE capabilities for dynamic grant and configured grant. Can we just add some note here with an example below for compromise? [PUSCH repetition type B with configured grant is applied only if UE reports the support of FG 5-19 or FG 5-20, and subjected to the capability of FG 5-19 and FG 5-20]. FFS: Whether to set separate UE capabilities for the case that dynamic SFI is configured and InvalidSymbolPattern is configured. Can we just add some note here with an example below for compromise? [The case that both dynamic SFI and InvalidSymbolPattern are configured is applied only if UE reports the support of FG3-6.] FFS: Whether to set separate UE capabilities for DCI format 0_1 and DCI format 0_2 for PUSCH repetition type B. Can we go majority view that no separate UE capability? | Optional with capability signalling |
|  | 11-6 | PUSCH repetition Type A | 1) PUSCH transmission with Rel-15 behavior with or without slot aggregation. • With slot aggregation, the number of repetitions can be [either semi-statically configured (as in Rel-15) or] dynamically indicated (as agreed for Rel-16). • When dynamically indicated, the number of repetitions is jointly coded with SLIV in TDRA table, by adding an additional column for the number of repetitions in the TDRA table. | 2-12, 2-13, 2-14, 2-15 | Yes | N/A |  | [Per UE] | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: Whether to add a component for the supported maximum number of PUSCH repetitions | Optional with capability signalling |
|  | 11-7 | UL cancelation scheme | 1) Supports group common DCI (i.e. DCI format 2_4) for cancelation indication 2) UL cancelation for PUSCH • Cancellation is applied to each PUSCH repetition individually in case of PUSCH repetitions 3) UL cancelation for SRS symbols that overlap with the cancelled symbols [4) For the serving cell, the UE determines the first symbol of the TCI symbols to be the first symbol that is after Tproc,2+d from the end of a PDCCH reception where the UE detects the DCI format 2_4, where d is provided by higher layer.] |  | Yes | N/A |  | [Per UE] FFS: FS | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: Whether to split this FG 11-7 into one feature group for the case of UL CI on the same CC and another feature group for the case of UL CI on another CC FFS: Whether to add new FG with FG11-7 as prerequisite for the support of more than one monitoring occasion for DCI 2_4 per slot? Can we just add the following note to address the concern? [More than one monitoring occasion for DCI format 2_4 per slot is applied only if the UE reports to support FG 3-5 or FG 3-5a or FG 3-5b] | Optional with capability signalling |
|  | [11-7a] | Cancellation of the overlapping PUSCHs in an intra-band UL CA without indication in the DCI format 2-4 | 1) For a UE indicating the capability of pa-PhaseDiscontinuityImpacts, and if the PUSCH on at least one serving cell is cancelled, the UE may cancel the (repetition of the) PUSCHs transmission on all other intra-band serving cell(s). The cancellation of the (repetition of the) PUSCH transmission on a the set of intra-band serving cell(s) includes all symbols from the earliest symbol that is overlapping with the first cancelled symbol of the PUSCH on the serving cell for which the DCI format 2_4 is applicable to. | 6-23, 11-7 | Yes | N/A |  | [PerBand] | [N/A] | [N/A] | TBD | FFS: Whether to add this FG and the content for each column if added | Optional with capability signaling |
|  | 11-8 | Enhanced UL power control scheme | For DG-PUSCH, one bit (separately from SRI) in UL grant is used to indicate the open loop power control parameter set if SRI is present in the UL grant, and 1 or 2 bits is used to indicate the P0 value if SRI is not present in the UL grant |  | Yes | N/A |  | Per UE | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] |  | Optional with capability signalling |
|  | 11-9 | Multiple active configured grant configurations for a BWP of a serving cell | 1) Supports up to 12 configured/active configured grant configurations in a BWP of a serving cell. • Separate RRC parameters for different configured grant configurations • Separate activation for different configured grant Type 2 configurations • Separate release for different configured grant Type 2 configurations [2) Supported maximum number of configured grant configurations in a BWP of a serving cell] [3) Supported maximum number of configured grant configurations across all serving cells] |  | Yes | N/A |  | [Per UE] FFS: FSPC | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] |  | Optional with capability signalling |
|  | 11-9a | Joint release in a DCI for two or more configured grant Type 2 configurations for a given BWP of a serving cell | M<=4 bits indication in the Release DCI is used for indicating which CG configuration(s) is/are released, where the association between each state indicated by the indication and the CG configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple CG configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the CG configuration index indicated by the indication | 11-9 | Yes | N/A |  | Per UE | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] | FFS: A UE supporting this feature shall also support 11-10 (Type 2 configured grant release by DCI format 0_1). A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 11-11 (Type 2 configured grant release by DCI format 0_2). | Optional with capability signalling |
|  | 11-10 | Type 2 configured grant release by DCI format 0_1 | Support of type 2 configured grant release by DCI format 0_1 |  | Yes | N/A |  | Per UE | [No] | [No] | [support mixture of FDD/TDD and/or FR1/FR2] FFS: The capability interpretation is from the perspective of a carrier on which the release DCI is received | [A UE supporting this feature and 11-1 (DCI format 0_2/1_2) shall also support 11-11 (Type 2 configured grant release by DCI format 0_2).] FFS: Whether to merge with FG 11-11 | Optional with capability signalling |
|  | 11-11 | Type 2 configured grant release by DCI format 0_2 | Support of type 2 configured grant release by DCI format 0_2 | 11-1 | Yes | N/A |  | Per UE | No | No | [support mixture of FDD/TDD and/or FR1/FR2 ] | [A UE supporting this feature shall also support 11-10 (Type 2 configured grant release by DCI format 0_1).] FFS: Whether to merge with FG 11-10 | Optional with capability signalling |

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 12. NR_IIOT | 12-1 | UL intra-UE multiplexing/prioritization of overlapping channel/signals with two priority levels in physical layer | Support intra-UE multiplexing/prioritization of UL overlapping channels/signals with two priority levels in physical layer (PHY) - Configuration of PHY priority level for CG PUSCH and SR, and dynamic indication of priority level for dynamic PUSCH - Multiplexing/prioritization between UL channels/signals with the same PHY priority level - Prioritization between UL channels/signals with different PHY priority levels - Additional number of symbols (d1) needed beyond the PUSCH preparation time for cancelling a low priority UL transmission. - Additional number of symbols (d2) needed beyond the PUSCH preparation time for scheduling a high priority UL transmission that cancels a low priority UL transmission |  | Yes | N/A |  | Per UE | No | No | [support mixture of FDD/TDD and/or FR1/FR2 ] | A UE supporting this feature shall also support the LCP restriction based on DCI priority indication ([lch-ToGrantPriorityRestriction-r16]) and intra-UE prioritization in MAC ([lch-PriorityBasedPrioritization-r16]). The relationship between this feature and the feature of up to two HARQ-ACK codebooks of 11-4[x] including merging these features should be further discussed. | Optional with capability signaling Candidate value set for component 4: {0, 1, 2} Candidate value set for component 5: {0, 1, 2} |
|  | 12-2 | Multiple SPS configurations | - Support of up to 8 configured SPS configurations in a BWP of a serving cell and up to 32 configured SPS configurations in a cell group, including separate RRC parameters and separate activation/release for different SPS configurations - The max number of active SPS configurations in a BWP of a serving cell - The max number of active SPS configurations across all serving cells - The related HARQ-ACK enhancements to support multiple active SPS configurations | downlinkSPS | Yes | N/A |  | Per UE | No | No | [support mixture of FDD/TDD and/or FR1/FR2 ] |  | Optional with capability signaling Component-2, candidate value set is {1, 2, …, 8} Component-3, candidate value set is {2, …, [32]} |
|  | 12-2a | Joint release in a DCI for two or more SPS configurations for a given BWP of a serving cell | - M<=4 bits indication in the Release DCI is used for indicating which SPS configuration(s) is/are released, where the association between each state indicated by the indication and the SPS configuration(s) is • Up to 2^M states are higher layer configurable, where each of the state can be mapped to a single or multiple SPS configurations to be released • In case of no higher layer configured state(s), separate release is used where the release corresponds to the SPS configuration index indicated by the indication - The related HARQ-ACK enhancements to support joint release | 12-2 | Yes | N/A |  | Per UE | No | No | [support mixture of FDD/TDD and/or FR1/FR2 ] |  | Optional with capability signaling |
|  | 12-3 | SPS release by DCI format 1_1 and 1_2 | - Support of SPS release by DCI format 1_1 - Support of SPS release by DCI format 1_2 | downlinkSPS | Yes | N/A |  | Per UE | No | No | [support mixture of FDD/TDD and/or FR1/FR2 ] | A UE supporting component 1 and 11-1 (DCI format 0_2/1_2) shall also support component 2 (SPS release by DCI format 1_2). | Optional with capability signaling |
|  | 12-5 | Configuration of aggregation factor per SPS configuration | Support of configurable PDSCH aggregation factor ({1, 2, 4, 8}) per DL SPS configuration | downlinkSPS | Yes | N/A |  | Per UE | No | No | [support mixture of FDD/TDD and/or FR1/FR2 ] |  | Optional with capability signaling |