3GPP TSG RAN WG1 #100-Bis-E			R1-2002562

April 20th – 30th , 2020

Agenda item:	7.2.11.1

Source: 	Qualcomm Incorporated

Title: 	Discussion on Two-Step RACH UE Features

Document for:	Discussion/Decision

- Introduction

In this contribution, we share our views on UE feature group drafted for NR Rel-16 two-step RACH [1]. Proposed changes for the UE feature group are based on the agreements in [4-5]. The remaining issues for the channel structure and procedures for NR Rel-16 two-step RACH can be found in our companion papers [2-3].

- Discussion on Two-Step RACH UE Features for NR Rel-16

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 9. NR_2step_RACH | 9-1 | Basic channel structure and procedure of 2-step RACH | Alt 2 simplified basic feature group: - MsgA PRACH and PUSCH transmission - MsgB monitoring, reception, and feedback - Power control for MsgA PRACH, MsgA PUSCH, and PUCCH for HARQ-ACK feedback to a MsgB FFS: if any of the components need to be more specific, i.e. the following components in Alt. 1 can be further discussed. [Alt 1 detailed feature group]: - [RACH type selection based on a SSB-based RSRP threshold] - [Separately configured ROs not applicable to 4-step RO configuration] - [Same ROs but different preamble sequences partitioning with 4-step RO preamble sequences configuration] - [Maximum two MsgA PUSCH configurations in an UL BWP from UE perspective] - [Validation of MsgA PRACH and PUSCH] - [Mapping between preamble of MsgA PRACH and PUSCH occasion with DMRS resource of MsgA PUSCH] - [MsgA PUSCH transmission including scrambling, DMRS sequences and ports, RV, etc] - [MsgA open loop power control] - [Monitoring of MsgB with PDCCH addressed to msgB-RNTI or C-RNTI, and receiving MsgB including SuccessRAR, fallbackRAR, and backoff indication] - [Support PUCCH transmission for HARQ-ACK feedback to a MsgB] - [MsgA PRACH configuration and preamble formats] - [PUCCH power control for HARQ-ACK feedback to a MsgB] - [Minimum TX gap between PRACH and PUSCH (for both TDD and FDD, FR1 and FR2, single CC and intra-band CA) as specified in Rel-15] - [Minimum TX gap between last DL SSB reception symbol and PRACH (TDD. FR1 and FR2, single CC and intra-band CA) as specified in Rel-15] - [MsgA PRACH and PUSCH transmissions in different PRACH and PUSCH slots] - [Minimum TX gap between the last symbol of MsgB PDSCH and the first symbol of PUCCH carrying HARQ-ACK to MsgB PDSCH] - [2-step RACH operation in RRC_IDLE/INACTIVE/CONNECTED state] - [Intra-slot frequency hopping for MsgA PUSCH] |  | Yes | N/A | UE cannot initiate a 2-step RACH process, and thus would not be expected understand the 2-step RACH configurations | per band | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-2 | Supported 2 symbols DMRS for msgA PUSCH | Supported 2 symbols DMRS for msgA PUSCH (‘len2’) | 9-1, 2-18 | Yes | N/A | If UE does not support ‘len2’, and if msgA-maxLength is configured as ‘len2’, the UE cannot use 2-step RACH resources | per UE | N/A | Yes |  | Shall be aligned with 2-18 | Conditionallymandatory for UE supporting both 9-1 and 2-18 |
|  | 9-3 | Parallel MsgA and SRS/PUCCH/PUSCH transmissions across CCs in inter-band CA | Parallel MsgA and SRS./PUCCH/PUSCH transmissions across CCs in inter-band CA with msgA in PCell/PScell | 9-1 | Yes | N/A | UE cannot transmit an MsgA and other UL transmissions in parallel across CCs in inter-band CA | per band combination | N/A | N/A |  |  | Optional with capability signalling |
|  | 9-4 | MsgA operation in a band combination including SUL | MsgA operations in a band combination including SUL | 9-1, 6-16 | Yes | N/A | UE does not support msgA operations in a band combination including SUL | per band combination | N/A | N/A |  |  | Optional with capability signalling |

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | UE feature group 9-1 - Alt 2 is not acceptable in its current form, since it is an over-simplification and leads to potential ambiguity/confusion for UE implementation. - Alt 1 requires the following changes: - Item 11: Need to be more specific in msgA PRACH configuration and formats by capturing the following details: - the new PRACH configuration indexes introduced in Rel-16 are used for NR licensed UE only - UE without shared spectrum channel access supports PRACH formats with LRA=139 or 839 only - UE with shared spectrum channel access supports PRACH formats with LRA=139, 571 or 1171 only - the PRACH format, starting PRB and FDM pattern of msgA PRACH is configured separately on dedicated RO - Item 12: Including implementation details for the transmission of HARQ feedback (including ACK and NACK) on PUCCH - Item 16: changing ACK to “feedback” - Item 18: adding the condition to support intra-slot frequency hopping - Item 19: adding the waveform support for CP-OFDM and DFT-s-OFDM waveform - Item 20: adding the details for the DMRS type supported in msgA PUSCH transmission - Item 21: adding the details for msgA PUSCH numerology in FR1 (15 or 30 kHz) and FR2 (60 or 120 kHz) UE feature group 9-5 - Adding 60 kHz SCS for msgA PUSCH as an optional UE feature in FR1 To summarize, we propose the following UE features for NR Rel-16 2-step RACH: |

# References

- R1-2002184, “RAN1 UE Feature List for Rel-16 NR after RAN1#100-E,” 3GPP TSG RAN1#100-E.

- R1-2002526, “Remaining Issues and Clarification on Channel Structure for Two-Step RACH,” Qualcomm, 3GPP TSG RAN1#100bis-E.

- R1-2002527, “Remaining Issues on Procedures for Two-Step RACH,” Qualcomm, 3GPP TSG RAN1#100bis-E.

- R1-1913598, “RAN1 Agreements for Rel-16 2-step RACH,” Reno, USA, November 2019.

- R1-1913599, “Summary of NR-U agreements till RAN1 #99,” Reno, USA, November 2019.