3GPP TSG RAN WG1 #100bis						R1-2003009

e-Meeting, April 20th – 30th, 2020

______________________________________________________________________Agenda item: 7.2.4.5

Source: Moderator (LG Electronics)

Title: Text proposal from Email discussion thread #3 for AI 7.2.4.5 Physical layer procedures for sidelink

Document for: Discussion and decision

- Introduction and proposal

RAN1 made the agreements copied in Appendix in [100b-e-NR-5G_V2X_NRSL-PHY-Procedure-03] Email discussion/approval regarding indicating SL HARQ feedback related information. This contribution includes the text proposal endorsed. It is proposed to adopt the text proposal in Section 2 for the following reasons

- Reason for change: RAN1 made the agreements for the SCI format details and the relation to the sidelink HARQ operations. The text proposal is to implement these agreements.

- Summary of change: Two 2nd SCI formats are defined and the HARQ related information delivered in each format is specified.

- Consequences if not approved: SCI cannot indicate the information necessary for the HARQ operation in use.

- Text proposal

  - Text proposal for TS 38.212

### 8.4.1	2nd-stage SCI formats

The fields defined in each of the 2nd-stage SCI formats below are mapped to the information bits  a0 to aA-1 as follows:

Each field is mapped in the order in which it appears in the description, with the first field mapped to the lowest order information bit a0 and each successive field mapped to higher order information bits. The most significant bit of each field is mapped to the lowest order information bit for that field, e.g. the most significant bit of the first field is mapped to a0.

#### 8.4.1.1	SCI format 0-2

SCI format 0-2 is used for the decoding of PSSCH.

The following information is transmitted by means of the SCI format 0-2:

-	HARQ Process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213].

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213].

-	Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214].

-	CSI request – 1 bit as defined in clause 8.2.1 of [6, TS 38.214].

If the 2nd-stage SCI format field in the corresponding SCI format 0-1 indicates type 1 groupcast as defined in clause x.x.x of [6, TS 38.214], the following fields are present:

-	Zone ID – 12 bits as defined in clause x.x.x of [9, TS 38.331].

-	Communication range requirement –4 bits as defined in clause x.x.x of [9, TS 38.331]

#### 8.4.1.2	SCI format 0-3

SCI format 0-3 is used for the decoding of PSSCH.

The following information is transmitted by means of the SCI format 0-3:

-	HARQ Process ID – [x] bits as defined in clause 16.4 of [5, TS 38.213].

-	New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213].

-	Redundancy version – 2 bits as defined in clause x.x.x of [6, TS 38.214].

-	Source ID – 8 bits as defined in clause x.x.x of [6, TS 38.214].

-	Destination ID – 16 bits as defined in clause x.x.x of [6, TS 38.214].

-	Zone ID – 12 bits as defined in clause x.x.x of [9, TS 38.331].

-  Communication range requirement –4 bits as defined in clause x.x.x of [9, TS 38.331]

Appendix: Agreements made in the email discussion [100b-e-NR-5G_V2X_NRSL-PHY-Procedure-03]

Agreements: One SCI format (referred to as 2nd SCI format A) is defined as follows:

- This format includes Zone ID and Communication range requirement.

- This format is used when the following HARQ operations are in use

  - HARQ-ACK information includes only NACK

  - FFS: No HARQ feedback

Agreements: One SCI format (referred to as 2nd SCI format B) is defined as follows:

- This format does not include Zone ID or Communication range requirement.

- This format is used when the following HARQ operations are in use

  - No HARQ feedback

  - HARQ-ACK information includes ACK or NACK

    - FFS: how to determine M_ID in the equation for the PSFCH resource index

      - Option 1: Based on L1 ID(s)

      - Option 2: An explicit indication in SCI

  - FFS: HARQ-ACK information includes only NACK

Agreements:: Down-select one out of the following for the indication of HARQ feedback enable/disable:

- Option 1: This indication is conveyed in the 1st SCI.

- Option 2: This indication is conveyed in the 2nd SCI.

  - Option 2-1: This indication is present both in 2nd SCI format A and B.

  - Option 2-2: This indication is present in 2nd SCI format B but not in 2nd SCI format A.

Agreements: Send an LS to RAN2 regarding HARQ operations

- RAN1 informs RAN2 that RAN1 discussed whether to support mixing blind and feedback-based HARQ retransmissions of a TB and RAN1 agreed that this is an issue RAN2 needs to make decision.