3GPP TSG RAN WG1 Meeting #100b-e	R1-2003xxx

e-Meeting, April 20th – 30th, 2020

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | xxxx | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Additional MTC Enhancements for LTE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC5-Core |  |  |  |  |  | Date: |  |  | 2020-05-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Corrections needed on PUR transmission, multi-TB scheduling, coexistence, non-BL UEs improvement and use of LTE control channel region features of Rel-16 Additional MTC Enhancements for LTE. - The start of the pre-configured UL resource search space window is incorrect. Priority of paging search space vs transmissions using pre-configured UL resources is not specified. Corrections to specify how a UE is configured to transmit PUSCH using preconfigured uplink resources, how retransmissions of transport blocks transmitted using preconfigured uplink resource are scheduled, how to initialize scrambling for transmissions using preconfigured uplink resource, how a BL/CE UE determines MCS for transmissions using preconfigured uplink resources. - The definition of PO_UE_PUSCH,c(3) is unclear for transmissions using pre-configured UL resources. The specification is unclear that the closed-loop component fc(i) is zero for transmissions using pre-configured UL resources. - Update the corresponding text in 36.213 based on RAN1#100e agreement on on HARQ/NDI/RV/FH encoding for 36.212. - Capture the RAN1#100bis-e agreement for RV cycling, sub-PRB interleaving. - Capture the RAN1#100bis-e agreement regarding the HARQ-ACK bundling size, bundling patterns and DCI field mapping. - Clarify that SRS transmission that falls into a reserved UL resource should be dropped, MPDCCH-less SPS transmissions should follow the LTE-MTC resource reservation in the same way as MPDCCH-scheduled transmissions. - Capture how the second PMI is calculated in Submode 1 of the periodic CSI reporting Mode 1-1 for CSI-RS based CSI feedback, computing CQI/PMI based on CSI-RS and using non-zero transmission power CSI-RS when the BL/CE UE is configured with the higher layer parameter ce-CSI-RS-Feedback. - It is unclear what type of MPDCCH transmission (distributed or localized) is used for UE-specific search space configured by PUR C-RNTI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - [100b-e-LTE-eMTC5-PUR-01] eMTC alignment to NB-IOT, include the endorsed text proposal for section 8.0, 8.6, 9.1.5. The start of the pre-configured UL resource search space window is changed from “n+5 subframe” to “subframe n+4” in section 9.1.5 Specify that the priority of the paging search space is lower than transmissions using pre-configured UL resources in section 9.1.5. Added text for a UE may transmit PUSCH using preconfigured uplink resources as configured by higher layers. Retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a MPDCCH with DCI format 6-0A/6-0B. The scrambling initialization of PUSCH transmission in preconfigured uplink resource is by PUR C-RNTI - in section 8.0; a BL/CE UE determines MCS for transmissions using preconfigured uplink resources using the higher layer parameter mcs-r16 in PUR-Config in section 8.6 - [100b-e-LTE-eMTC5-PUR-02] Power control correction, include the endorsed text proposal for section 5.1.1.1. Added definition where PO_UE_PUSCH,c(3) = p0-UE-PUSCH in PUR-Config Added text to indicate that closed-loop component fc(i) is zero for transmissions using pre-configured UL resources. - [100b-e-LTE-eMTC5-Multi-TB-01] TBS/MCS/RV determination, include the endorsed text proposal for section 7.1.7, 7.1.7.1, 7.1.7.2, 8.0, 8.6.1. The 36.213 text for determination of MCS, RV and HARQ ID is updated to take the HARQ/NDI/RV/FH encoding in 36.212 into account in section 7.1.7, 7.1.7.1, 7.1.7.2, 8.0, 8.6.1 - [100b-e-LTE-eMTC5-Multi-TB-02] on RV cycling, TB interleaving and frequency hopping, include the endorsed text proposal for section 7.1.7.1, 8.0, 8.6.1. Captured the following RAN1#100bis-e agreement for RV cycling, sub-PRB interleaving in section 7.1.7.1, 8.0, 8.6.1: i0 is initialized at the beginning of each TB, and the corresponding includes all the subframes belonging to the TB and excludes those associated with other TBs scheduled by the DCI. For the sub-PRB interleaving case, the granularity can be similar with NB-IOT which is based on the TB repetition. - [100b-e-LTE-eMTC5-Multi-TB-03] HARQ-ACK bundling, include the endorsed text proposal for section 7.3. Captured the RAN1#100bis-e agreement regarding the HARQ-ACK bundling size, bundling patterns and DCI field mapping in section 7.3 - [100b-e-LTE-eMTC5-Coex-NR-01] on SRS and SPS, include the endorsed text proposal for section 7.1, 8.0, 8.2. It is clarified that SRS transmission falling into a reserved UL resource is dropped in section 8.2. It is clarified that MPDCCH-less SPS transmission follows the resource reservation in the same way as MPDCCH-scheduled transmissions in section 7.1, and 8.0. - [100b-e-LTE-eMTC5-non-BL-UEs-01] on the CSI-RS based CSI feedback, include the endorsed text proposal for section 7.2.2, 7.2.3, 7.2.4, 7.2.5. Added text on how the second PMI is calculated in Submode 1 of the periodic CSI reporting Mode 1-1 in section 7.2.2 Added text for computing CQI/PMI based on CSI-RS and using non-zero transmission power CSI-RS when the BL/CE UE is configured with the higher layer parameter ce-CSI-RS-Feedback in section 7.2.3, 7.2.4, 7.2.5. - [100b-e-LTE-eMTC5-Use-of-LTE-control-channel-region-01] on support PUR, include the endorsed text proposal for section 9.1.5. Specified that distributed MPDCCH transmission is used for UE-specific search space configured by PUR C-RNTI in section 9.1.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect functionality of Rel-16 Additional MTC Enhancements for LTE features. - The start of the PUR SS window will be 1 SF later than agreed by RAN1. Unspecified UE behaviour when the paging search space and transmissions using pre-configured UL resources collide. Missing configuration, retransmission, and scrambling initialization procedures for transmissions using preconfigured uplink resources which will lead to unspecified UE behaviour. Unspecified UE behaviour of how a BL/CE UE determines MCS for transmissions using preconfigured uplink resources. - Unclear definition of PO_UE_PUSCH,c(3) for transmissions using pre-configured UL resources. Unclear specification that the closed-loop component fc(i) is zero for transmissions using pre-configured UL resources. - Determination of MCS, RV and HARQ ID for multi-TB scheduling may be incorrectly implemented. - RV cycling and sub-PRB interleaving may be incorrectly implemented. - HARQ-ACK bundling may be incorrectly implemented. - Resource reservation may be incorrectly implemented for SRS and MPDCCH-less SPS transmissions, which may cause collision with other transmissions. - Undefined UE behaviour of how the second PMI is calculated in Submode 1 of the periodic CSI reporting Mode 1-1. No introduction of the CSI-RS based CSI feedback for non-BL UEs in CE mode A. - Unclear what type of MPDCCH transmission is used for UE-specific search space configured by PUR C-RNTI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.1, 7.1, 7.1.7, 7.1.7.1, 7.1.7.2, 7.2.2, 7.2.3, 7.2.4, 7.2.5, 7.3, 8.0, 8.2, 8.6, 8.6,1, 9.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 36.211 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.1.1.1	UE behaviour

The setting of the UE Transmit power for a Physical Uplink Shared Channel (PUSCH) transmission is defined as follows.

<Unchanged parts are omitted>

Otherwise

-	is a parameter composed of the sum of a component  provided from higher layers for j=0, 1 and 3 and a component  provided by higher layers for j=0, 1 and 3 for serving cell . For PUSCH (re)transmissions corresponding to a semi-persistent grant then j=0 , for PUSCH (re)transmissions corresponding to a dynamic scheduled grant then j=1, for PUSCH (re)transmissions corresponding to the random access response grant then j=2 and for BL/CE UE PUSCH (re)transmission using preconfigured uplink resource then j=3 and PO_UE_PUSCH,c3 = p0-UE-PUSCH in PUR-Config. and , where the parameter preambleInitialReceivedTargetPower [8] () and  are signalled from higher layers for serving cell .

<Unchanged parts are omitted>

-	For both types of  (accumulation or current absolute) the first value is set as follows:

-	If  value is changed by higher layers and serving cell  is the primary cell or, if  value is received by higher layers and serving cell  is a Secondary cell or, if a BL/CE UE transmits using preconfigured uplink resources,

-

Note: fci = 0 for i = 0,1,… up to the last subframe of the PUSCH transmission using preconfigured uplink resource.

-	For serving cell , when the UE is configured with higher layer parameter shortTTI or when there is a change in configuration corresponding to the higher layer parameter shortTTI,  for the first following PUSCH transmission in a slot or subslot in a given subframe is set to the value of  associated with PUSCH of the previous uplink subframe.

<Unchanged parts are omitted>

## 7.1	UE procedure for receiving the physical downlink shared channel

<Unchanged parts are omitted>

For BL/CE UEs, the set of BL/CE DL subframes is indicated as follows

-	If higher layer parameter ce-reserved-resource-DL-freq or ce-reserved-resource-DL-time is configured,

-	for PDSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PDSCH transmission without a corresponding MPDCCH,

-	if the Resource reservation field in the DCI is set to 0, then the set of BL/CE DL subframes corresponds to all downlink subframes during the PDSCH transmission;

-	if the Resource reservation field in the DCI is set to 1, then the set of BL/CE DL subframes corresponds to all downlink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols of all PRBs of the PDSCH transmission are reserved in the subframe);

-	for MPDCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space,

-	the set of BL/CE DL subframes corresponds to all downlink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all OFDM symbols of all PRBs of the MPDCCH transmission are reserved in the subframe).

-	In all other cases, the set of BL/CE DL subframes is indicated by the higher layers according to fdd-DownlinkOrTddSubframeBitmapBR [11].

<Unchanged parts are omitted>

### 7.1.7	Modulation order and transport block size determination

To determine the modulation order and transport block size(s) in the physical downlink shared channel, the UE shall first

-	if the UE is a BL/CE UE

-	if PDSCH is assigned by MPDCCH DCI format 6-1A

-	if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config-r15 and the DCI is mapped onto the UE specific search space and the repetition number field in the DCI indicates PDSCH repetition level 1

-  if “Scheduling TBs for Unicast” field in DCI format 6-1A is present and either 4 or 6 TBs are scheduled by the corresponding DCI,

-  read the 4-bit “modulation and coding scheme ()" field in the DCI

-  the UE is not expected to receive a DCI format 6-1A indicating

-	otherwise, 	read the 5-bit extended "modulation and coding scheme ()" field in the DCI

-	otherwise

-	read the 4-bit "modulation and coding scheme ()" field in the DCI

-	The UE is not expected to receive a DCI format 6-1A indicating

<Unchanged parts are omitted>

#### 7.1.7.1	Modulation order and redundancy version determination

<Unchanged parts are omitted>

For BL/CE UEs, for a PDSCH not carrying SystemInformationBlockType1-BR or SI message, the same redundancy version is applied to PDSCH associated with a TB that is transmitted in a given block of  consecutive subframes associated with the TB, including subframes that are not BL/CE DL subframes. The subframe number of the first subframe in each block of consecutive subframes, denoted as , satisfies , where  for FDD and  for TDD. Denote  as the subframe number of the first downlink subframe intended for PDSCH associated with a TB, as defined in Subclause 7.1.11. The PDSCH transmission associated with the TB spans Nabs,TBPDSCH consecutive subframes associated with the TB, including subframes that are not BL/CE DL subframes where the PDSCH transmission is postponed and excluding subframes associated with other TBs scheduled by the DCI, if any. For the  block of consecutive subframes within the set of Nabs,TBPDSCH subframes associated with the TB as described above, the redundancy version (rvidx) associated with the TB is determined according to Table 7.1.7.1-2 using , where , and JPDSCH=Nabs,TBPDSCH +i0-δ mod NaccNacc. The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured with CEModeB, or a BL/CE UE receiving PDSCH associated with P-RNTI,  for FDD and  for TDD, and . For a BL/CE UE configured in CEModeA, . For a BL/CE UE configured in CEModeA and not configured with the higher layer parameter multi-TB-DL-config,  for the TB is determined by the 'Redundancy version' field in DCI format 6-1A. For a BL/CE UE configured in CEModeA and configured with higher layer parameter multi-TB-DL-config,

-	if NTB=1, for the TB is determined by the 'Redundancy version' in the ‘Scheduling TBs for Unicast’ field in DCI format 6-1A

-	else if NTB=2 and the HARQ process IDs for each of the scheduled TBs are h1 and h2 (h1<h2),  of the scheduled TB with HARQ process ID h1 is determined by the ‘Redundancy version for TB 1’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-1A, and  of the scheduled TB with HARQ process ID h2 is determined as follows:

-	If the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config and the repetition number field in the DCI indicates no PDSCH repetition, it is given by the‘Redundancy version for TB 1’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-1A

-	else if the UE is configured with higher layer parameter mpdcch-pdsch-HoppingConfig set to ‘on’ and the repetition number field in the DCI indicates PDSCH repetition, it is given by the ‘Redundancy version for TB 1’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-1A

-	else it is given by the ‘Redundancy version for TB 2’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-1A

-	else if NTB = 4 or 6 is indicated by the corresponding DCI,  for all scheduled TBs

-	else

-	if the UE is configured with higher layer parameter ce-PDSCH-64QAM-Config and the repetition number field in the DCI indicates no PDSCH repetition,  for all TBs

-	else if the UE is configured with higher layer parameter mpdcch-pdsch-HoppingConfig set to 'on' and the repetition number field in the DCI indicates PDSCH repetition,  for all TBs

-	else  of all TBs is determined by the ‘Redundancy version for all TBs’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-1A.

Table 7.1.7.1-2: Redundancy version

| Redundancy version Index | rvidx |
| --- | --- |
| 0 | 0 |
| 1 | 2 |
| 2 | 3 |
| 3 | 1 |

<Unchanged parts are omitted>

#### 7.1.7.2	Transport block size determination

<Unchanged parts are omitted>

For a BL/CE UE, if the UE is configured with higher layer parameter multi-TB-DL-config and multiple TB, , are scheduled in the corresponding DCI with CRC scrambled by C-RNTI, the HARQ process ID for each of the scheduled  TBs are determined from the value of the ‘HARQ index with offset’ in the ‘Scheduling TBs for Unicast’ field for CEmodeA or the HARQ index in the ‘Scheduling TBs for Unicast’ field for CEmodeB in the corresponding DCI which is a combinatorial index r defined as +roffset, where

-	the set , () contains the  sorted HARQ process IDs and  is the extended binomial coefficient, resulting in unique label +roffset,

-  roffset is the offset value as defined in 5.3.3.1.12 of [4] for CE mode A, and roffset=0 for CEmodeB,

-	 is the number of scheduled TB, and

-	 if UE is configured with CEModeA, and  if UE is configured with CEModeB,

-	 if UE is configured with CEModeA, and ‘Multi-TB HARQ processes group’ field is present and set to '1' in the corresponding DCI,  otherwise.

For a BL/CE UE, if the UE is configured with higher layer parameter multi-TB-DL-config and NTB,max TBs are scheduled in the corresponding DCI with CRC scrambled by C-RNTI, the HARQ process IDs for each scheduled TB are hi=i+kNTB,max  , i=0,1,…,NTB,max-1, where

-	 if UE is configured with CEModeA, and  if UE is configured with CEModeB,

-	if UE is configured with CEModeA, and ‘Multi-TB HARQ processes group’ field is present and set to '1' in the corresponding DCI,  otherwise.

The NDI and HARQ process ID, as signalled on PDCCH/EPDCCH/MPDCCH/SPDCCH, and the TBS, as determined above, shall be delivered to higher layers.

<Unchanged parts are omitted>

### 7.2.2	Periodic CSI Reporting using PUCCH

<Unchanged parts are omitted>

For a BL/CE UE, the periodic CSI reporting modes are described as following:

- Wideband feedback

  - Mode 1-0 description:

    - In the subframe where CQI is reported:

      - A UE shall report a type 4 report consisting of one wideband CQI value which is calculated assuming transmission on all narrowband(s) in the CSI reference resource. The wideband CQI is calculated conditioned on transmission rank 1.

  - Mode 1-1 description:

    - In the subframe where a first PMI is reported for transmission modes 9 configured with submode 1 and 8 CSI-RS ports, the UE shall report a type 2a report consisting of a first PMI corresponding to a set of precoding matrices selected from the codebook subset assuming transmission on all narrowband(s) in the CSI reference resource. The wideband first PMI value is calculated conditioned on transmission rank 1.

    - In the subframe where wideband CQI/second PMI is reported for transmission modes 9 with 8 CSI-RS ports and submode 1:

      - A single precoding matrix is selected from the codebook subset assuming transmission on all narrowband(s) in the CSI reference resource.

      - A UE shall report a type 2b report consisting of

        - A single wideband CQI value which is calculated assuming the use of the single precoding matrix in all narrowband(s) in the CSI reference resource and transmission on all narrowband(s) in the CSI reference resource.

        - The wideband second PMI corresponding to the selected single precoding matrix.

      - The wideband second PMI is calculated conditioned on transmission rank 1 and the last reported wideband first PMI. The wideband CQI is calculated conditioned on the selected precoding matrix and transmission rank 1.

    - In the subframe where wideband CQI/first PMI/second PMI is reported for transmission modes 9 with submode 2 and 8 CSI-RS ports configured:

      - A single precoding matrix is selected from the codebook subset assuming transmission on all narrowband(s) in the CSI reference resource.

      - A UE shall report a type 2c report consisting of

        - A single wideband CQI value which is calculated assuming the use of the single precoding matrix in all narrowband(s) in the CSI reference resource and transmission on all narrowband(s) in the CSI reference resource.

        - The wideband first PMI and the wideband second PMI corresponding to the selected single precoding matrix as defined in Subclause 7.2.4.

      - The wideband first PMI, the wideband second PMI and the wideband CQI are calculated conditioned on transmission rank 1.

    - In the subframe where CQI/PMI is reported:

      - A single precoding matrix is selected from the codebook subset assuming transmission on all narrowband(s) in the CSI reference resource. The PMI is calculated conditioned on transmission rank 1.

      - A UE shall report a type 2 report consisting of

        - A single wideband CQI value which is calculated assuming the use of a single precoding matrix in all narrowband(s) in the CSI reference resource and transmission on all narrowband(s) in the CSI reference resource. The wideband CQI is calculated conditioned on transmission rank 1.

        - The selected single PMI (wideband PMI).

<Unchanged parts are omitted>

### 7.2.3	Channel Quality Indicator (CQI) definition

<Unchanged parts are omitted>

For a UE in transmission mode 9 when parameter pmi-RI-Report is configured by higher layers and parameter eMIMO-Type is not configured by higher layers, the UE shall derive the channel measurements for computing the CQI value reported in uplink subframe/slot/subslot n based on only the Channel-State Information (CSI) reference signals (CSI-RS) defined in [3] for which the UE is configured to assume non-zero power for the CSI-RS. For a non-BL/CE UE in transmission mode 9 when the parameter pmi-RI-Report is not configured by higher layers or in transmission modes 1-8 the UE shall derive the channel measurements for computing CQI based on CRS. For a BL/CE UE the UE shall derive the channel measurements for computing CQI based on CSI-RS if configured with the higher layer parameter ce-CSI-RS-Feedback for transmission mode 9 and the number of CSI-RS ports=8, otherwise based on CRS.

For a UE in transmission mode 10, when parameter eMIMO-Type is not configured by higher layers, the UE shall derive the channel measurements for computing the CQI value reported in uplink subframe/slot/subslot n and corresponding to a CSI process, based on only the non-zero power CSI-RS (defined in [3]) within a configured CSI-RS resource associated with the CSI process.

<Unchanged parts are omitted>

### 7.2.4	Precoding Matrix Indicator (PMI) definition

For transmission modes 4, 5 and 6, precoding feedback is used for channel dependent codebook based precoding and relies on UEs reporting precoding matrix indicator (PMI). For transmission mode 8, the UE shall report PMI if configured with PMI/RI reporting. For transmission modes 9 and 10, the non-BL/CE UE shall report PMI if configured with PMI/RI reporting and the number of CSI-RS ports is larger than 1. For transmission modes 9, the BL/CE UE shall report PMI based on CSI-RS if configured with the higher layer parameter ce-CSI-RS-Feedback and the number of CSI-RS ports=8, otherwise based on CRS. A UE shall report PMI based on the feedback modes described in 7.2.1 and 7.2.2. For other transmission modes, PMI reporting is not supported.

<Unchanged parts are omitted>

### 7.2.5	Channel-State Information – Reference Signal (CSI-RS) definition

<Unchanged parts are omitted>

A UE configured in transmission mode 10, and configured with higher layer parameter eMIMO-Type and eMIMO-Type2, and with quasi co-location type B, is not expected to receive CSI-RS resource configurations for eMIMO-Type and eMIMO-Type2 of the CSI process that have different values of the higher layer parameter qcl-CRS-Info-r11.

A BL/CE UE configured with CEModeA or CEModeB is not expected to be configured with non-zero transmission power CSI-RS, except when the BL/CE UE is configured with the higher layer parameter ce-CSI-RS-Feedback.

A UE configured in transmission mode 9 or 10, and configured with higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS A', and more than one CSI-RS configurations for a CSI-RS resource, is not expected to receive CSI-RS configurations for the CSI-RS resource that have different values of frequencyDensity.

<Unchanged parts are omitted>

## 7.3	UE procedure for reporting HARQ-ACK

<Unchanged parts are omitted>

For a BL/CE UE, if the UE is configured with CEModeA, and if the UE is configured with higher layer parameter multi-TB-DL-HARQ-bundling and multiple TB are scheduled in the corresponding DCI format 6-1A with CRC scrambled by C-RNTI,

-	for HARQ-ACK transmission associated with the corresponding DCI, the UE shall generate M HARQ-ACK bits by performing a logical AND operation of HARQ-ACKs across all TBs in each TB bundle Ab where b = 1, …, M;

-	the set of TBs that belong to TB bundle Ab and the number of TB bundles M are given by Table 7.3-1;

-	the value of is the number of scheduled TB determined in the corresponding DCI.

Table 7.3-1:  Value of Ab and M for different values of DCI field ‘Multi-TB HARQ-ACK bundling size’ and for different values of number of scheduled transport blocks NTB

| DCI field ‘Multi-TB HARQ-ACK bundling size’ | NTB=1 | NTB=2 | NTB=4 | NTB=6 | NTB=8 |
| --- | --- | --- | --- | --- | --- |
| 00 | A1={TB0} | A1=TB0 A2=TB1 | A1=TB0 A2=TB1 A3=TB2 A4=TB3 | A1=TB0 A2=TB1 A3=TB2 A4=TB3 A5=TB4 A6=TB5 | A1=TB0 A2=TB1 A3=TB2 A4=TB3 A5=TB4 A6=TB5 A7=TB6 A8=TB7 |
| 01 | - | A1=TB0, TB1 | A1=TB0, TB1 A2=TB2, TB3 | A1=TB0, TB1 A2=TB2, TB3 A3=TB4, TB5 | A1=TB0, TB1 A2=TB2, TB3 A3=TB4, TB5 A4=TB6, TB7 |
| 10 | - | - | A1=TB0, TB1 A2=TB2 A3=TB3 | A1=TB0, TB1,TB2 A2=TB3, TB4,TB5 | A1=TB0, TB1,TB2 A2=TB3, TB4,TB5 A3=TB6, TB7 |
| 11 | - | - | A1=TB0, TB1,TB2, TB3 | A1=TB0, TB1,TB2,TB3 A2=TB4, TB5 | A1=TB0, TB1,TB2,TB3 A2=TB4, TB5,TB6,TB7 |

<Unchanged parts are omitted>

## 8.0	UE procedure for transmitting the physical uplink shared channel

<Unchanged parts are omitted>

For BL/CE UEs, the set of BL/CE UL subframes is indicated as follows

-	If higher layer parameter ce-reserved-resource-UL-time is configured,

-	for PUSCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PUSCH transmission without a corresponding MPDCCH,

-	if the Resource reservation field in the DCI is set to 0, then the set of BL/CE UL subframes corresponds to all uplink subframes during the PUSCH transmission;

-	if the Resource reservation field in the DCI is set to 1, then the set of BL/CE UL subframes corresponds to all uplink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols of the PUSCH transmission are reserved in the subframe);

-	for PUCCH transmission associated with C-RNTI or SPS C-RNTI using UE-specific MPDCCH search space including PUCCH transmission without a corresponding MPDCCH,

-	the set of BL/CE UL subframes corresponds to all uplink subframes that are not fully reserved according to higher layer parameters (a subframe is considered fully reserved if and only if all SC-FDMA symbols of the PUCCH transmission are reserved in the subframe).

-	In all other cases, the set of BL/CE UL subframes is indicated by the higher layers according to fdd-DownlinkOrTddSubframeBitmapBR and fdd-UplinkSubframeBitmapBR [11].

For BL/CE UEs, PUSCH transmission can be scheduled by a MPDCCH with DCI format 6-0A/6-0B, or the transmission can correspond to using preconfigured uplink resource configured by higher layers. Transmission using preconfigured uplink resource is initiated by higher layers as specified in [14], while retransmission of transport blocks transmitted using preconfigured uplink resource are scheduled by a MPDCCH with DCI format 6-0A/6-0B.

For a PUSCH transmission using preconfigured uplink resource, the UE shall use the repetition number determined by the repetition adjustment field according to Table 8-2b and Table 8-2c from the most recent MPDCCH DCI format 6-0A/6-0B with CRC scrambled by PUR C-RNTI for PUR ACK feedback indication (as defined in [4]) if detected, configured by higher layers otherwise.

A BL/CE UE shall upon detection on a given serving cell of an MPDCCH with DCI format 6-0A/6-0B scheduling PUSCH intended for the UE, perform a corresponding PUSCH transmission in subframe(s) ni = n+ki if a transport block(s) corresponding to the HARQ process(es) of the PUSCH transmission is generated as described in [8] with i = 0, 1, …, NTBN-1 according to the MPDCCH, where

-	subframe n is the last subframe in which the MPDCCH is transmitted;

-	the value of is the number of scheduled TB determined by the corresponding DCI if present,  otherwise;

-	 and the value of  is determined by the repetition number field in the corresponding DCI, where

-	if the UE is configured with higher layer parameter ce-pdsch-puschEnhancement-config with value 'On' are given by {1,2,4,8,12,16,24,32}

-	otherwise, are given in Table 8-2b and Table 8-2c; and

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment in the corresponding DCI is using uplink resource allocation type 5,  where N ≤ 32 for CE Mode A and N ≤ 2048 for CE Mode B,  is defined in [3] and  is determined according to procedure in subclause 8.1.6,  otherwise

-	in case N>1, subframe(s) n+ki with i=0,1,…, NTBN-1 are NTBN consecutive BL/CE UL subframe(s) starting with subframe n+x, and in case N=1, k0=x;

-	for ,

-	if the UE is configured with higher layer parameter multi-TB-UL-Unicast-Interleaving-config, and PUSCH corresponding to a MPDCCH with DCI CRC scrambled by C-RNTI and

-	where C=MslotsUL2∙MRU if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15 and the PUSCH resource assignment is using uplink resource allocation type 5, otherwise  for UE configured with CEModeA, and  for UE configured with CEModeB,

-	BL/CE UL subframes  with  are associated with TBr+1 ,

-	otherwise,

-	BL/CE UL subframes  with  are associated with TBr+1 ,

-	the HARQ process ID for each of the scheduled  TBs are determined from the value of the ‘HARQ index with offset’ in the ‘Scheduling TBs for Unicast’ field for CEmodeA or the ‘HARQ index’ in the ‘Scheduling TBs for Unicast’ field for CEmodeB in the corresponding DCI which is a combinatorial index r defined as +roffset, where

-	the set , () contains the sorted HARQ process IDs and  is the extended binomial coefficient, resulting in unique label +roffset,

-  roffset is the offset value as defined in 5.3.3.1.10 of [4] for CE mode A, and roffset=0 for CE mode B,

-	 if UE is configured with CEModeA, and  if UE is configured with CEModeB.

-	for FDD, x = 4;

<Unchanged parts are omitted>

If a UE is configured by higher layers to decode PDCCHs/EPDCCHs with the CRC scrambled by the AUL C-RNTI, the UE shall decode the PDCCH/EPDCCH according to the combination defined in Table 8-9.

Table 8-9: PDCCH/EPDCCH configured by AUL C-RNTI

| Autonomous uplink Transmission mode | DCI format | Search Space | Transmission scheme of corresponding autonomous PUSCH |
| --- | --- | --- | --- |
| Mode 1 | DCI format 0A | UE specific by C-RNTI | Single-antenna port, port 10 (see Subclause 8.0.1) |
| Mode 2 | DCI format 4A | UE specific by C-RNTI | Closed-loop spatial multiplexing (see Subclause 8.0.2) |

A UE may transmit PUSCH on preconfigured uplink resources as configured by higher layers. The scrambling initialization of PUSCH transmission using preconfigured uplink resource is by PUR C-RNTI.

If a UE is configured by higher layers to decode MPDCCHs with the CRC scrambled by the PUR C-RNTI, the UE shall decode the MPDCCH according to the combination defined in Table 8-10 and in case the indication in the DCI corresponds to the retransmission of a transport block transmitted using preconfigured uplink resource, transmit a corresponding PUSCH. The scrambling initialization of this PUSCH corresponding to these MPDCCHs and the PUSCH retransmission for the same transport block is by PUR C-RNTI.

<Unchanged parts are omitted>

## 8.2	UE sounding procedure

A BL/CE UE not configured with the higher layer parameter srs-UpPtsAdd shall not transmit SRS in UpPTS if SRS frequency location is different from DwPTS reception narrowband in the same special subframe.

For a BL/CE UE, the SRS transmission that falls into the reserved symbol of a BL/CE UL subframe is dropped.

For a TDD serving cell, , not configured for PUSCH/PUCCH transmission, the UE is not expected to be configured with SRS resource(s) such that the SRS transmission (including any interruption due to uplink or downlink RF retuning time [10]) may overlap in time with PDCCH monitoring in subframes 0 or 5 on serving cell , if the UE is not capable of simultaneous transmission and reception on serving cell  and serving cell .

<Unchanged parts are omitted>

## 8.6	Modulation order, redundancy version and transport block size determination

To determine the modulation order, redundancy version and transport block size for the physical uplink shared channel, the UE shall first

-	for a cell that is

-	a LAA SCell or,

-	configured with higher layer parameter shortProcessingTime and the PDCCH with CRC scrambled by C-RNTI corresponding to the PUSCH is in the UE-specific search space, or

-	configured with higher layer parameter shortTTI and the associated DCI is of format 7-0A/7-0B,

-		read the "modulation and coding scheme" field () and "redundancy version" field (),

-	otherwise

-	if the UE is a non-BL/CE UE,

- 	read the "modulation and coding scheme and redundancy version" field (),

-	elseif the UE is a BL/CE UE,

- 	for transmission using preconfigured uplink resources, read the higher layer parameter mcs-r16 in PUR-Config

- 	otherwise, read the "modulation and coding scheme" field ()

and

-	check the "CSI request" bit field, and

-	compute the total number of allocated PRBs () based on the procedure defined in Subclause 8.1, and

-	compute the number of coded symbols for control information.

<Unchanged parts are omitted>

### 8.6.1	Modulation order and redundancy version determination

<Unchanged parts are omitted>

For a BL/CE UE or for UEs configured with higher layer parameter PUSCH-EnhancementsConfig,

-	if the UE is configured with higher layer parameter ce-PUSCH-SubPRB-Config-r15, and the PUSCH resource assignment is using uplink resource allocation type 5, the redundancy version (rvidx) to use for the i-th BL/CE UL subframe associated with a TB in the physical uplink shared channel is determined according to Table 7.1.7.1-2 using  where i=0, 1, …, N-1, and N is the number of BL/CE UL subframes associated with the TB for the PUSCH transmission as determined in subclause 8.0. For a BL/CE UE configured in CEModeA, is determined by the 'Redundancy version' field in DCI format 6-0A, if present. For a BL/CE UE configured in CEModeA, if the UE is configured with higher layer parameter multi-TB-UL-config and multiple TB are scheduled in the corresponding DCI, and the 'Redundancy version' field for a scheduled TB is not present in the corresponding DCI,  for all TBs scheduled by the DCI. For a BL/CE UE configured with CEModeB, . For a BL/CE UE,  for a PUSCH transmission using preconfigured uplink resource.

-	otherwise, the same redundancy version is applied to PUSCH associated with a TB that is transmitted in a given block of  consecutive subframes associated with a TB, including subframes that are not BL/CE UL subframes. The subframe number of the first subframe in each block of  such consecutive subframes, denoted as , satisfies . Denote  as the subframe number of the first uplink subframe intended for PUSCH associated with a TB. For BL/CE UEs, the PUSCH transmission associated with a TB spans Nabs,TBPUSCH consecutive subframes associated with the TB, including subframes that are not BL/CE UL subframes where the PUSCH transmission is postponed and excluding subframes associated with other TBs scheduled by the DCI, if any. For the  block of consecutive subframes within the set of Nabs,TBPUSCH subframes associated with the TB as described above, the redundancy version (rvidx) associated with the TB is determined according to Table 7.1.7.1-2 using , where , andJPUSCH=Nabs,TBPUSCH +i0 mod NaccNacc. The  blocks of subframes are sequential in time, starting with  to which subframe belongs. For a BL/CE UE configured with CEModeB,  for FDD and  for TDD, and . For a UE configured with higher layer parameter PUSCH-EnhancementsConfig,  and  is determined by the 'Redundancy version' field in DCI format 0C. For UEs configured with higher layer parameter PUSCH-EnhancementsConfig,NabsPUSCH=NrepPUSCH. Nabs,TBPUSCH=NrepPUSCH. For a BL/CE UE configured in CEModeA, . For a BL/CE UE configured in CEModeA, and not configured with the higher layer parameter multi-TB-DL-config,  for a TB is determined by the 'Redundancy version' field in DCI format 6-0A.

-	if NTB=1 is indicated by the corresponding DCI,  for the TB is determined by the 'Redundancy version' in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A

-	else if NTB=2 is indicated by the corresponding DCI, and the HARQ process IDs for each of the scheduled TBs are h1 and h2 (h1<h2),  of the scheduled TB with HARQ process ID h1 is determined by the ‘Redundancy version for TB 1’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A, and  of the scheduled TB with HARQ process ID h2 is determined by

-	if the UE is configured with higher layer parameter pusch-HoppingConfig set to’on’ and the repetition number field in the DCI indicates PUSCH repetition, the ‘Redundancy version for TB 1’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A

-	otherwise the ‘Redundancy version for TB 2’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A

-	if NTB = 4 or 6,  for all schedueld TBs

-	else

-	if the UE is configured with higher layer parameter pusch-HoppingConfig set to 'on' and the repetition number field in the DCI indicates PUSCH repetition,  for all TBs

-	otherwise  of all TBs is determined by the ‘Redundancy version for all TBs’ in the ‘Scheduling TBs for Unicast’ field in DCI format 6-0A.

<Unchanged parts are omitted>

### 9.1.5	MPDCCH assignment procedure

<Unchanged parts are omitted>

The BL/CE UE is not required to monitor Type2A-MPDCCH common search space if the set of subframes comprising the search space include any subframes in which it monitors Type1A-MPDCCH common search space or any subframes in which the UE receives PDSCH assigned by MPDCCH with DCI CRC scrambled by SC-RNTI.

A BL/CE UE is not required to monitor Type1-MPDCCH common search space if the set of subframes comprising the search space include any subframes in which the UE has initiated a PUSCH transmission using preconfigured uplink resource on a given serving cell.

A BL/CE UE is not expected to monitor an MPDCCH candidate, if an ECCE corresponding to that MPDCCH candidate is mapped to a PRB pair that overlaps with a transmission of PDSCH scheduled previously in the same subframe.

<Unchanged parts are omitted>

For Type1-MPDCCH common search space, Type1A-MPDCCH common search space, Type2-MPDCCH common search space and Type2A-MPDCCH common search space, distributed MPDCCH transmission is used.

For MPDCCH UE-specific search space configured by PUR C-RNTI, distributed MPDCCH transmission is used.

<Unchanged parts are omitted>

If the UE has initiated a PUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the MPDCCH UE-specific search space in a search space window starting in subframe n+4 with duration given by higher layer parameter pur-MPDCCH-SS-window-duration. Upon detection of a MPDCCH with DCI format 6-0A/6-0B with CRC scrambled by PUR C-RNTI intended for the UE within the search space window and the corresponding DCI is for PUR ACK/fallback indication (as defined in [4]), the UE is not required to monitor the MPDCCH UE-specific search space for the remaining search space window duration.

<Unchanged parts are omitted>