3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	6.2.5.3

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-LTE-UEFeatures-MIMO-02]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-LTE-UEFeatures-MIMO-02] during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-LTE-UEFeatures-MIMO-02] Email discussion/approval of “Proposal 2 (3-1B)” in R1-2001864 till 4/24 – Ralf (ATT) - Note: “FL Proposal 4 (3-3)“ in R1-2001860 can be discussed within the scope of this email discussion

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-LTE-UEFeatures-MIMO-02] “Email discussion/approval of “Proposal 2 (3-1B)” in R1-2001864” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-LTE-UEFeatures-MIMO-02]

The following is the proposal in Error: Reference source not found for approval in this email discussion:

FL Proposal 2 (3-1B):

| 3-1B | Additional SRS symbols within normal UL subframes with antenna switching | With antenna switching | 3-1 | Yes | N/A | Network cannot utilize antenna switching for additional SRS symbols within normal UL subframes | ALT 1-1) Per BoBC ALT 1-2) Per UE | ALT 1-1) N/A ALT 1-2) TDD only | N/A | Alt. 2-1: inherited from legacy capability signalling ue-TxAntennaSelection-SRS-1T4R-r15, ue-TxAntennaSelection-SRS-2T4R-2Pairs-r15, ue-TxAntennaSelection-SRS-2T4R-3Pairs-r15 or one of {1T2R, 1T4R, 2T4R_2pairs, 2T4R_3pairs} is reported as a R16 capability. Alt. 2-2: UE reports, per BoBC, whether it supports 1T2R, 1T4R, 2T4R (2/3 pairs), or no switching. UE reports whether it needs gaps or not, for the case of repetition and no repetition | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- Down-select between Alt. 1-1 and Alt. 1-2

- Down-select between Alt. 2-1 and Alt. 2-2

FL Proposal 4 (3-3) in [1] can also be discussed within the scope of this email discussion:

FL Proposal 4 (3-3):

Alt. 1: Leave the discussion up to RAN4

Alt. 2: Introduce a new row for a new FG 3-3

| 3-3A | Gap symbol with antenna switching | Support of gap symbol with antenna switching | 3-1B | Yes | N/A | eNB has no information to enable or disable gap symbol | Per BoBC | N/A | N/A | N/A | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3-3B | Gap symbol with frequency hopping | Support of gap symbol with frequency hopping | 3-1A | Yes | N/A | eNB has no information to enable or disable gap symbol | Per BoBC | N/A | N/A | N/A | Optional with capability signalling |

- Alt. 1 can be an outcome of the discussions on FL Proposals 1 and 2, Alt. 2 can be an alternative solution for FFS points in FL Proposals 1 and 2

Companies are asked to provide their views and comments in the following table.

Regarding the down-selection between Alt. 1-1 and Alt. 1-2:

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | - As an extra note, we would like to point out that legacy AS capability is per BoBC. |

Regarding the down-selection between Alt. 2-1 and Alt. 2-2:

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | 2-2 |

Regarding FL Proposal 4 (3-3):

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Qualcomm | We would be fine with either option, but there has to be a note in the feature list (i.e., if we conclude Alt.1, we would add a note in the corresponding FG that RAN4 has to resolve). |

…

# Conclusions

…

# References

- R1-2001864, Summary on UE features for DL MIMO efficiency enhancements for LTE, Moderator (AT&T)