3GPP TSG RAN WG1 Meeting #100bis 	R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda item:	7.2.10.2

Source:	Apple Inc.

Title:	FL summary of remaining issues on enhancements for single Tx transmission in EN-DC

Document for:	Decision

# Introduction and Proposals

Per chairman’s guidance, I’d like to kick off the email discussion on following issue:

[100b-e-NR- LTE_NR_DC_CA-SingleTx-01] Email discussion/approval regarding the following issues:

- Issue 1 on type 1 UE hailing of colliding UL in single Tx operation, as described in section 2 of R1-2002348

- Issue 2 on type 1 UE capability with semi-static UL transmission, as described in section 3 of R1-2002348

  - Discussion of the TP is delayed until the corresponding UE feature discussions finish.

- Issue 3a and 3b on clarification of Type 2 UE behavior, as described in sections 4.1 and 4.2 of R1-2002348, respectively.

till 4/23, with potential TPs for approval till 4/28 – (Apple, Wei)

# Type 1 UE handling of colliding UL in 1Tx operation

Background: It was discussed in RAN1 #100e that the current description in 38.213 does not capture type 1 UE behavior for ‘single-tx’ case, as the corresponding text is under the condition “If a UE is configured with …”. For single-tx case, the UE cannot transmit simultaneously on LTE and NR regardless of  ,   settings. So, the behavior of dropping NR should be specified outside the    restriction.

This issue was brought up and discussed in RAN1 #100e, and two alternative TP’s were proposed. However no consensus was reached. For this meeting, a 3rd TP was also proposed by Qualcomm [6]. These 3 proposed TP are listed below:

TP option 1 ---------------------------- start TP1 to sub clause 7.6.1 of 38.213v16.1.0 -------------------------------------- 7.6.1 EN-DC If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power for transmissions on the MCG by p-MaxEUTRA and a maximum power for transmissions in FR1 on the SCG by p-NR-FR1. The UE determines a transmission power for the MCG as described in [13, TS 36.213] using as the maximum transmission power. The UE determines transmission power for the SCG in FR1 as described Clauses 7.1 through 7.5 using as the maximum transmission power. The UE determines transmission power for the SCG in FR2 as described Clauses 7.1 through 7.5. A UE does not expect to be configured for operation with shortened TTI and/or processing time [13, TS 36.213] on a cell that is included in an EN-DC configuration. If the UE indicates capability for dynamic power sharing between E-UTRA and NR for EN-DC, and if the UE is configured with tdm-PatternConfig-r16, then for the band combinations for which the UE indicates using singleUL-Transmission that it does not support simultaneous UL transmissions (as defined in TS 38.101-3 [8-3]), the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG. If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows. - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and - if UE transmission(s) in subframe of the MCG overlap in time with UE transmission(s) in slot of the SCG in FR1, and

TP option 2 ---------------------------- start TP2 to sub clause 7.6.1 of 38.213v16.1.0 -------------------------------------- 7.6.1 EN-DC If a UE is configured with a MCG using E-UTRA radio access and with a SCG using NR radio access, the UE is configured a maximum power for transmissions on the MCG by p-MaxEUTRA and a maximum power for transmissions in FR1 on the SCG by p-NR-FR1. The UE determines a transmission power for the MCG as described in [13, TS 36.213] using as the maximum transmission power. The UE determines transmission power for the SCG in FR1 as described Clauses 7.1 through 7.5 using as the maximum transmission power. The UE determines transmission power for the SCG in FR2 as described Clauses 7.1 through 7.5. A UE does not expect to be configured for operation with shortened TTI and/or processing time [13, TS 36.213] on a cell that is included in an EN-DC configuration. If the UE indicates capability for dynamic power sharing between E-UTRA and NR for EN-DC and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG. If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows. - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and - if UE transmission(s) in subframe of the MCG overlap in time with UE transmission(s) in slot of the SCG in FR1, and

TP option 3 ---------------------------- start TP3 to sub clause 7.6.1 of 38.213v16.1.0 -------------------------------------- If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, for the band combinations for which the UE indicates singleUL-Transmission, if the conditions in TS 38.101-3 [8-3] are fullfilled, - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r16 in [13, TS 36.213]), - the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on an UL-offset subframe (as specified in [13, TS 36.213]) on the MCG - the UE does not expect to be configured or indicated to transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe other than the UL-offset subframes (as specified in [13, TS 36.213]) on the MCG - Otherwise, - the UE does not expect to be configured or indicated to transmit on the SCG in FR1 when the UE has overlapped transmission on any subframe on the MCG. If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows. - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC and - if UE transmission(s) in subframe of the MCG overlap in time with UE transmission(s) in slot of the SCG in FR1, and - if in any portion of slot of the SCG, the UE reduces transmission power in any portion of slot of the SCG so that in any portion of slot , where and are the linear values of the total UE transmission powers in subframe of the MCG and in slot of the SCG in FR1, respectively. The UE is not required to transmit in any portion of slot of the SCG if would need to be reduced by more than the value provided by XSCALE in order for in any portion of slot of the SCG. The UE is required to transmit in slot of the SCG if would not need to be reduced by more than the value provided by XSCALE in order for in all portions of slot . - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE expects to be configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213]).

It seems to me that main difference between option 1 vs option 2 is

I’d like companies to provide input to the following questions:

- Q1-1: In addition to the following cases, is there any other case where network can configure a UE can be configured with case 1 Reference HARQ timing (i.e. tdm-PatternConfig-r16)? (If yes, please also point us to the RAN1 agreement)

  - Case 1): type 1 UE indicates singleUL-Transmission for the difficult BC listed in 38.101

  - Case 2): type 1 UE indicates tdm-pattern-dualTx for BC with potential DL de-sense due to harmonics

  - Case 3): type 2 UE configured with P_lte + P_nr > P_total

| Company | Comments |
| --- | --- |
| ZTE | We feel like the current spec doesn’t preclude configuring tdm-PatternConfig for other cases in addition to the above three cases. The key issue is, for a UE indicates neither singleUL-Transmission or tdm-pattern-dualTx, it is not clear whether it should be regarded as a “single Tx” UE or a “dual Tx” UE. tdm-Pattern Indicates whether the UE supports the tdm-PatternConfig for single UL-transmission associated functionality, as specified in TS 36.331 [17]. Support is conditionally mandatory in (NG)EN-DC for UEs that do not support dynamicPowerSharingENDC and for UEs that indicate single UL transmission for any (NG)EN-DC BC. Support is conditionally mandatory in NE-DC for UEs that do not support dynamicPowerSharingNEDC and for UEs that indicate single UL transmission for any NE-DC BC. The feature is optional otherwise. |
| Apple | For type 1 UE, supporting TDM pattern is optional feature, so only after UE indicated singleUL-Transimssion or tdm-pattern-dual Tx, then gNB can configured t tdm-PatternConfig-16. For type 2 UE, supporting TDM pattern is mandatory feature, so network can configure the tdm-PatternConfig/ tdm-PatternConfig-16 without UE reporting capability. And for the case of P_LTE + P_NR <= Pcmax, the network can configure the TMD pattern as well, this was agreed in RAN1#91 meeting, but it was not captured in the spec. Operation B with Case1: P_LTE + P_NR <= Pcmax, in which case NR UL can be scheduled in any UL subframe/slot (while the UE behaviour in case of being simultaneously scheduled on LTE and NR uplinks is not specified) As the issue#1 is only related to type1 UE, so network can only configure UE with TMD pattern for case 1 and case 2. |
| MTK | As quoted by ZTE about the definition of tdm-Pattern, it is mandatory for Type 2 UE and single UL UE, and still can be configured for other scenarios. It seems to us the current spec does not preclude configuring tdm-PatternConfig-r16 to UE for other cases in addition to the three cases mentioned in Q1-1. We also think whether the network can configure tdm-PatternConfig-r16 is not related to Type1/Type2 UE or P_lte + P_nr > P_total. The expected UE behavior in our opinion is listed as below: |
| Huawei, HiSilicon | As MediaTek pointed out, there are three UE capabilities in Rel-16, and the first one singleUL-Transmission is for IMD issue, a UE reporting support of tdm-pattern can be a UE without IMD issue and capable of dynamic power sharing, which is proved by the sentence “The feature is optional otherwise.” in 38.306 highlighted by ZTE. Therefore, the current UE behaviors for three capabilities are as the table below, UE capability : RRC configuration; tdm-PatternConfig-r16 configured; tdm-PatternConfig-r16 NOT configured. tdm-pattern; Single UL Tx; N/A. tdm-pattern-dual-Tx; Dual UL Tx; N/A. singleUL-Transmission; Single UL Tx; Single UL Tx. Regarding ZTE’s question whether the UE is regarded as “dual Tx” or “single Tx”, it has been answered by the first agreement below, i.e. the reason to introduce tdm-pattern-dual-Tx was, as the highlight in the table above, tdm-pattern only indicates Single UL Tx under Rel-16 TDM pattern configuration, but no restriction if TDM pattern is not configured. If tdm-pattern were indicating Dual UL Tx in Rel-16, we would not have introduced a new UE capability tdm-pattern-Dual-Tx. Please note that the two agreements following the agreement of dual-Tx capability was to discuss the UE behavior for a UE configured with TDM pattern, instead of a UE suffering IMD issue or not. Under a TDM pattern configuration, the dual-Tx case refers to a UE capable of the new tdm-pattern-dual-Tx instead of a UE not reporting IMD issue. Similarly, the single-Tx case is not bound to IMD issue. In short, only Option 2 is in line with the agreements. Agreements: - Introduce a new UE capability of supporting DL reference configuration for dual-Tx UL (e.g., to handle DL de-sensing from harmonics) Agreements: - For the single-Tx case, for FDD LTE Pcell, - All uplink subframes can be scheduled for LTE for type 1 UEs - In which case, NR transmission is dropped for when the LTE and NR transmissions collide - Note: there is no change of UL scheduling timing for LTE Agreements: - For the dual-Tx case, for FDD LTE Pcell, - All uplink subframes can be scheduled for LTE at least for type 1 UEs - Note: there is no change of UL scheduling timing for LTE |

- Q1-2: Which of the 3 proposed TPs are accept

| Company | Comments |
| --- | --- |
| ZTE | We prefer Option1. For a UE indicates neither singleUL-Transmission or tdm-pattern-dualTx, our understanding is that this UE should be regarded as a “dual Tx” UE. Seems like two issues are mixed up in Option3. We prefer to separate the discussion of “not expect to be configured or indicated to transmit”. |
| Apple | TP1 is preferred. It’s clearer and no confusion. |
| MTK | To our understanding, “UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG (LTE)” only holds when UE indicates singleUL-Transmission. Hence, we prefer Option 1 or Option 3. |
| Huawei, HiSilicon | Only Option 2 is in line with agreements. |

# Type 1 UE capability with semi-static UL transmission

In RAN1#99, the following agreements were reached [7]. According to the agreement, whether the semi-static configured UL transmissions are allowed in all UL subframes is subjected to UE capability.  Two text proposals are provided by [2] [6].

Agreements: For a UE configured with DL-reference DL/UL configuration in Rel-16 (including single Tx with LTE TDD PCell or LTE FDD PCell, and dual Tx cases): - For type 2 UE (i.e., UE without dynamic power sharing capability), any LTE UL transmissions should take place only in UL subframes designated for HARQ-ACK feedback. - For type 1 UE (i.e., UE with dynamic power sharing capability), - Confirm that any LTE UL transmissions scheduled/triggered by DCI can take place in UL subframes not designated for HARQ-ACK feedback. - FFS UE is not expected to transmit semi-statically configured LTE UL transmissions in the UL subframes other than those designated as UL in the DL-reference configuration if such transmission collide with NR UL transmissions. Agreements For the FFS part in the agreement above, - semi-statically configured LTE UL transmissions are allowed in all UL subframes. - Note: In case of collision, LTE transmission is prioritized - Note: this configuration is subject to UE capability

Proposal 2: Discussion further the detailed TP.

| ---------------------------- start ZTE TP to sub clause 7.6.1 of 38.213v16.1.0 -------------------------------------- If a UE is configured with , where is the linear value of , is the linear value of , and is the linear value of a configured maximum transmission power for EN-DC operation as defined in [8-3, TS 38.101-3] for FR1, the UE determines a transmission power for the SCG as follows. - If the UE is configured with reference TDD configuration for E-UTRA (by tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 in [13, TS 36.213]) - If the UE does not indicate a capability for dynamic power sharing between E-UTRA and NR for EN-DC, the UE does not transmit in a slot on the SCG in FR1 when a corresponding subframe on the MCG is an UL subframe in the reference TDD configuration. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and does not indicate a capability tdm-Pattern-dualTx in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission on a subframe on the MCG. - If the UE indicates a capability for dynamic power sharing between E-UTRA and NR for EN-DC, and indicates a capability tdm-Pattern-dualTx and a capability semi-staticULTransInAllSubframe in [16, TS 38.306], and is configured with tdm-PatternConfig-r16, the UE does not transmit on the SCG in FR1 when the UE has overlapped transmission that is not associated with a DCI on a subframe on the MCG. |  |
| --- | --- |

---------------------------- start ZTE TP to sub clause 5.1 of 36.213v16.1.0 -------------------------------------- For a UE configured with EN-DC, if the UE does not indicate a capability for dynamic power sharing (as specified in [17]) and if the UE is configured with subframeAssignment-r16 for a serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r16. For a UE configured with EN-DC, if the UE indicates a capability for dynamic power sharing and does not indicate a capability semi-staticULTransInAllSubframe (as specified in [17]) and if the UE is configured with subframeAssignment-r16 for a serving cell, the UE is not expected to transmit any uplink physical channel or signal without associated DCI in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r16.

---------------------------- start Qualcomm TP to clause 6 or 8 of 36.213v16.1.0 -------------------------------------- For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r15 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r15. For a UE configured with EN-DC, if the UE does not indicate a capability for dynamic power sharing (as specified in [17]) and if the UE is configured with subframeAssignment-r16 for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r16. For a UE configured with EN-DC, if the UE indicates a capability for dynamic power sharing (as specified in [17]) and if the UE is configured with subframeAssignment-r16 for the serving cell, the UE is not expected to be configured or indicated to transmit any uplink physical channel or signal, in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r16, if the UE does not indicate the UE capability [FG18-2a or FG18-3a] (as specified in [17]).

As the exact definition of the corresponding UE capability will be discussed in this meeting, and there are different proposals, it might be desirable to wait until the corresponding UE feature discussed is concluded.

FL proposal 2: The related TP discussion could be delayed until the corresponding UE feature discussion is completed.

# Clarifications on Type 2 UE behavior

## Clarification on Type 2 UE behavior

Background: The type 2 UE behavior is not fully specified in the specification TS38.213 for the case of P_LTE+P_NR <= P_total. Specifically, the following proposal on clarification of type 2 UE behavior is made in [3] as follows:

Proposal : Clarify the whether the following UE behaviour is matching the Rel-16 agreement for UL collision handling:

- For Type 2 UE, NR UL can collide with LTE UL under “P_LTE+P_NR <= P_total” and the corresponding transmission behavior is up to UE implementation

- since all slots can be scheduled for NR UL (no restriction) when tdm-PatternConfig-r15 or tdm-PatternConfig-r15 is configured according to current spec of 38.213 Rel-16 spec 7.6.1 EN-DC.

FL summary: In general, type 2 UE (i.e., without fast communication between LTE and NR modems) has been quite clear from existing agreements:

- LTE UL is only transmitted

- Colliding UL Tx is considered as error case, and it is up to UE implementation.

However, it seems that existing 38.213 does not explicitly capture this for type 2 UE in the case of P_LTE+P_NR <= P_total.

- Q2-1: Do we need a TP to explicitly clarify in 38.213 type 2 UE assumption/behavior when P_lte + P_nr <= P_total and the UE is configured with Reference HARQ timing? (e.g. up to UE implementation)

| Company | Comments |
| --- | --- |
| ZTE | Our view is that we may first discuss what’s the intended UE behavior in case of P_lte + P_nr <= P_total for type2 UE. And then we could discuss whether a TP is needed or not. Our preliminary understanding is the UE behavior in case of P_lte + P_nr <= P_total for type2 UE could be the same as that in case of P_lte + P_nr > P_total. We are open to further discuss the intended UE behavior. |
| Apple | Copy/paste our above comments. And for the case of P_LTE + P_NR <= Pcmax, the network can configure the TMD pattern as well, this was agreed in RAN1#91 meeting, but it was not captured in the spec. Operation B with Case1: P_LTE + P_NR <= Pcmax, in which case NR UL can be scheduled in any UL subframe/slot (while the UE behaviour in case of being simultaneously scheduled on LTE and NR uplinks is not specified) We are open to clarify this in the spec. |
| MTK | We agree with FL’s summary that for type 2 UE in the case of P_LTE+P_NR <= P_total, - LTE UL is only transmitted in designated UL subframes in tdm-pattern - Colliding UL Tx is considered as error case, and it is up to UE implementation. Constructing a TP to clarify these behaviors seems like a good idea. |
| Huawei, HiSilicon | The target case has been covered by the TP Option2 in issue#1. If it is agreed, no additional text change is needed. |

## Type 2 UE’s assumption on “other than offset-UL subframes”

In Rel.15, the following agreements made in RAN1#90bis.

Agreements:

- In Case 1, LTE TDD UL HARQ timing is supported and the UE is allowed to transmit only in the subframes designated as UL in the reference TDD configuration. Additionally, a UE-specific HARQ subframe offset can be configured.

The agreement was captured in three places, i.e., section 5.1, section 6 and section 8 of TS36.213, with the same contents.

For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r15 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r15.

In Rel. 16, the similar agreements were reached in EN-DC enhancement, as shown below. The agreements were captured in both section 6 and section 8 of TS36.213, but not in section 5.1. To avoid confusion between Rel.15 and Rel.16, It was proposed to capture the same contents in section 5.1 as these in section 6 and section 8 by [4]. But according to [1], this alignment is not necessary.

Agreements:

For a UE configured with DL-reference DL/UL configuration in Rel-16 (including single Tx with LTE TDD PCell or LTE FDD PCell, and dual Tx cases):

- For type 2 UE (i.e., UE without dynamic power sharing capability), any LTE UL transmissions should take place only in UL subframes designated for HARQ-ACK feedback.

Proposal 3: Discuss further whether adopt the proposed text proposal on uplink power control section in TS36.213.

5.1 Uplink power control ### < Unchanged parts are omitted > For a UE configured with EN-DC/NE-DC and serving cell frame structure type 1, if the UE is configured with subframeAssignment-r15 for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r15 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r15. For a UE configured with EN-DC, if the UE does not indicate a capability for dynamic power sharing (as specified in [17]) and if the UE is configured with subframeAssignment-r16 for the serving cell, the UE is not expected to transmit any uplink physical channel or signal in the serving cell on subframes other than offset-UL subframes, where the offset-UL subframes are determined by applying an offset value given by harq-Offset-r16 to the subframes denoted as uplink in the UL/DL configuration subframeAssignment-r16.

- Q2-2: Do we need to copy the same text on R16 UE behavior from section 6 & 8 to section 5.1 as well in 36.213? (Note that we indeed have the same text in section 6 & 8 & 5.1 on R15 UE behavior)

| Company | Comments |
| --- | --- |
| ZTE | Copying the same text seems fine to us. |
| Apple | We prefer to capture the TP in the spec, otherwise it could cause confusion why it is different from Rel.15. |
| MTK | We think it is better to copy the same text to section 5.1. |
| Huawei, HiSilicon | It is not necessary. Because such UE behavior has been captured at multiple sections for all PHY UL channels, the specification 36.213 is clear, no need to duplicate them into the section of power control. Or please clarify which PHY UL channel is missing. |

# References

- R1-2001529, “Enhancements for single UL operation for EN-DC”, Huawei, HiSilicon, 3GPP RAN1#100 bis e-meeting, April 20th – 30th, 2020

- R1-2001619, “Remaining Issues on Single Tx Switched Uplink Solution for EN-DC”, ZTE, 3GPP RAN1#100 bis e-meeting, April 20th – 30th, 2020

- R1-2001835, “Remaining issues on single Tx switched UL solution for EN-DC”, MediaTek Inc., 3GPP RAN1#100 bis e-meeting, April 20th – 30th, 2020

- R1-2002347, “Remaining issues on single Tx operation for EN-DC”, Apple, 3GPP RAN1#100 bis e-meeting, April 20th – 30th, 2020

- R1-2002419, “Remaining issues for single Tx UL enhancements”, Ericsson, 3GPP RAN1#100 bis e-meeting, April 20th – 30th, 2020

- R1-2002559, “Remaining issues for EN-DC Single-Tx TDM Operation”, Qualcomm Incorporated, 3GPP RAN1#100 bis e-meeting, April 20th – 30th, 2020

- “Chairman Notes, RAN1 #99”, 3GPP RAN1 #99, Reno, USA, November 18th – 22nd, 2019.