3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.3

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-IAB-02]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-IAB-02] during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-IAB-02] Email discussion/approval of proposal 3 in R1-2001866 till 4/24 – Ralf (ATT)

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-IAB-02] “Email discussion/approval of Proposal 3 in R1-2001866” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-NR-UEFeatures-IAB-02]

The following is the proposal in Error: Reference source not found for approval in this email discussion:

FL Proposal 3 (high priority): Down-select between Alt. 1 and Alt. 2

Alt. 1:

| 20-5 | UL-Flexible-DL slot formats | Support semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt.2:

| 20-5a | UL-Flexible-DL slot formats | Support semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | [per IAB node] | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20-5b | UL-Flexible-DL slot formats | Support dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources |  | Yes | N/A | Dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |

Companies are asked to provide their views and comments in the following table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Huawei | We support Alt.2. It may be necessary to clarify first what dynamic indication means here. Our understanding is that it means SFI based DCI forma 2_0. In Rel-15 UE feature list, UE specific RRC configure UL/DL assignment (5-1a) and Dynamic SFI (3-6) are defined as separate features for Rel-15 UE hence the same principle can be applied for IAB-MT. |
| Ericsson | We prefer Alternative 2. In case Alt. 2 is favored, should 20-5a and 20-5b be separate components of 20-5 or separate feature groups? |
| Intel | We prefer Alt.2. We agree with Huawei that further clarification may be needed: - 20-5a corresponds to Rel-15 UE feature list 5-1a for UE specific RRC configure UL/DL assignment and dynamic UL/DL determination based on L1 scheduling DCI; - 20-5b corresponds to Rel-15 UE feature list 3-6 for SFI RRC configure and dynamic UL/DL determination based on DCI format 2_0 scheduling. We are also okay with either 20-5a/20-5b separate components in the same FG or separate FGs. |
| ZTE, Sanechips | Alt2 is slightly preferred. |
| AT&T | We agree there is a direct link between 20-5 and 5-1a and 3-6. Perhaps a simple solution is to go with Alt 1, but remove the description of “semi-static” and “dynamic” configuration/indication and just make the FG about support of the new slot formats: “Support semi-static and dynamic configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources” This way, if the IAB-MT supports only 5-1a then it will only support RRC configuration of the new slot formats. If the IAB-MT also supports 3-6, then both RRC and DCI 2_0 indications may include the UL-Flexible-D formats. It does not seem justified for an IAB-MT to support 20-5b but not 3-6 for example. |
| Nokia, NSB | In case we go for Alternative 1 then two separate capability indications are needed between semi-static and dynamic operation. Hence, Alt.2 would be a cleaner option. |
| NTT DOCOMO | We prefer Alt2, to align with Rel-15 UE feature list (the features of semi-static and dynamic configuration are listed separately) |
| vivo | Alt. 2 is prefered |

Looking at the company inputs provided so far there seems to be a clear preference towards Alt. 2. I have thus updated the original proposal as follows:

Revised FL Proposal 3 (high priority):

- Replace FG 20-5 by a new FG 20-5a given below

- Introduce a new FG 20-5a given below

| 20-5a | UL-Flexible-DL slot formats | Support semi-static configuration/indication of UL-Flexible-DL slot formats for IAB-DU and IAB-MT resources |  | Yes | N/A | Only Rel-15 slot formats can be configured for backhaul links | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 20-5b | UL-Flexible-DL slot formats | Support dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources |  | Yes | N/A | Dynamic indication of UL-Flexible-DL slot formats for IAB-MT resources is not supported | per IAB node | No | No | support mixture of FDD/TDD and/or FR1/FR2 | IAB-MT impact | Optional with capability signalling |

Several companies mentioned that further clarifications may be necessary. Please provide in much detail (what clarification, where …) what it takes for you to agree the proposal.

| Company | Can we agree the Revised FL Proposal 3? Please answer yes or no. | Comments/Questions/Suggestions |
| --- | --- | --- |
| Samsung | Yes | We don’t think an indication of 20-5a and 20-5b is meaningful without an indication of 5-1a and 3-6, respectively. Also, to address the issue commented by AT&T, 5-1a can be a prerequisite feature group for 20-5a and 3-6 can be a prerequisite feature group for 20-5b. |
| AT&T | Yes, if 5-1a and 3-6 are prerequisite FGs for 20-5a and 20-5b respectively, otherwise we do not agree with the proposal | We think the suggestion from Samsung solves the issue raised previously and makes Alt 2 an acceptable solution |
| LG | Yes, if 5-1a and 3-6 are prerequisite FGs for 20-5a and 20-5b respectively. | Same view with Samsung and AT&T. One clarification on new FL proposal3: the second bullet needs to be revised as: - Replace FG 20-5 by a new FG 20-5a given below - Introduce a new FG 20-5ab given below |
| Nokia | Yes |  |
| Intel | Yes, if 5-1a and 3-6 are prerequisite FGs for 20-5a and 20-5b respectively. | Same view with Samsung and AT&T. We are okay with either 20-5a/20-5b separate components in the same FG or separate FGs. |

# Conclusions

# References

# R1-2001866, Summary on UE features for IAB, Moderator (AT&T)

…