3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.4

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-V2X-03]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-V2X-03] during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-V2X-03] Email discussion/approval of Proposals {1, 3, 5, 6, 10, 11, 13, 14, 16, 17} (stabilizing the overall number of rows and separate FGs) – focusing on the high priority parts - in R1-2001867 by 4/24 – Ralf (ATT)

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-V2X-03] “Email discussion/approval of Proposals {1, 3, 5, 6, 10, 11, 13, 14, 16, 17} (stabilizing the overall number of rows and separate FGs) – focusing on the high priority parts -  in R1-2001867” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-NR-UEFeatures-V2X-03]

## Feature groups potentially not transparent to pre-configuration versus Uu configuration

The following are the proposals in Error: Reference source not found for approval in this email discussion:

## FL Proposal 1 (basic VUE feature groups)

The basic feature groups for vehicle UEs (VUEs) to include:

- High priority:

  - Subcarrier spacing

    - Alt. 1: for a given carrier, the numerologies that are mandatory for NR UL are also mandator for NR SL

    - Alt. 2: [At least] one subcarrier spacing (30 kHz for FR1 [FFS: FR2])

    - Note: This impacts signalling design (signal as candidate value for components or introduce separate FG per subcarrier spacing)

  - 15-1 [including mandatory DMRS patterns and PSSCH symbols]

  - [15-2] [including mandatory DMRS patterns and PSSCH symbols]

  - 15-3 [including mandatory DMRS patterns and PSSCH symbols] [except (4) sensing]

Companies can provide their views and comments in the following table. However, it may be hard to agree or even discuss FL Proposal 1 in isolation. The main reason FL Proposal 1 on basic VUE feature groups is included in this email discussion is that at the very least, when companies comment on FGs {15-1, 15-3, 15-4, 15-10a, 15-11, 15-18, 15-19, 15-23, 15-24} below they should also clarify the consequence of their preference, i.e., how does deleting or adding a feature group, or moving a component from one FG to another impact FL Proposal 1. Hopefully, or ideally, the answer to FL Proposal 1 is a natural outcome of the following exercise. Nevertheless, companies are strongly encouraged to make comments that can resolve FL Proposal 1, despite the interconnect with the following discussion. Looking at the summary in [1], it appears at least discussing how subcarrier spacing is handled could be worthwhile at this early stage. There have been proposals to make this implicit (see Alt. 1) or to signal it either via dedicated FG(s) or simply as candidate component values. Since this directly impacts the overall number of rows/FGs, it is important we address this as soon as possible. But as said before, this may be hard in isolation and could better be accomplished as part of the subsequent discussion on  FGs {15-1, 15-3, 15-4, 15-10a, 15-11, 15-18, 15-19, 15-23, 15-24}. Similar comments apply to mandatory DMRS patterns and PSSCH symbols.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | It is unclear whether Alt 1 can work for the ITS band case where no Uu is defined. Our preference is to indicate the supported subcarrier spacing either as candidate value for components or separate FG. We are okay with having a default numerology as in Alt 2. |
| Ericsson | We support Alt.1. In other words, it is important that the same numerologies are mandated as in UL. Note that we have a RAN1 agreement that a UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time. Not supporting the same numerologies on SL as for Uu, it will limit the flexibility of different SL operations, especially when SL operates in licensed carriers.. Basic feature group: We think that 15-15/15-15a should also be considered as part of basic feature group, at least for licensed carrier operation. In addition, in our view, at least 15-1/15.1a, 15-2 , 15-3/15-3a, 15-4 (merged with 15-15/15-15a) and 15-11 should be the basic feature groups. |
| Nokia | We prefer Alt.1. It is important to ensure NR SL can coexist well with Uu when they operate in the same carriers. While the considerations for ITS are valid, the RAN1 specifications are typically band-agnostic, and hence it is not our preference to make strong assumptions in RAN1 feature list that would be valid only for V2X UEs operating in ITS band. |
| Futurewei | Agree with 15-1, 15-2 and 15-3 (except 4) as part of the basic FG. In addition to the proposals by the FL for the FG, we think that 15-4, 15-5, and 15-11 should be part of the basic FG |
| NTT DOCOMO | Support Alt. 1. We share views with Ericsson/Nokia. If the supported SCS for SL is different from Uu, the possible operations are quite limited. At least the same SCS should be supported. |
| vivo | Alt 1 for SCS is preferred for licensed band. For ITS band all the SCS defined by RAN4 should be supported. Regarding the DMRS pattern and PSSCH symbol, the number of PSSCH symbol can be defined in the FG, but all the DMRS pattern associated with that PSSCH symbol should be supported (i.e., no need to define/report in the FG) |
| Qualcomm | We share LG’s view that Alt1 won’t work for the ITS or a dedicated band. Alt 2 with 30 kHz for FR1 [15-2] (Mode 1) should be removed from the basic feature group 15-3 sensing should be included, RAN1 has not defined a resource selection procedure for Mode 2 without sensing. All DMRS patterns should be mandatory |
| Panasonic | Our understanding is this UE feature discussion for V2X is UE feature expression. For ITS dedicated band of out of coverage, no signaling among UEs without PC5-RRC but it would be useful to proper expression of UE feature. For such purpose, preconfiguration of mode 2 of both Tx and Rx (i.e. 15-1 and 15-3) would be the basic feature group. In this case, the subcarrier spacing should be list of the candidate values. If this UE feature discussion for V2X is only for the signaling to be reported for the network (or PC5-RRC), the basic feature group would be also 15-1, 15-2 and 15-3 as it is only for in-coverage. In this case, the subcarrier spacing should be same as supported in Uu. |
| Samsung | In order to resolve the concern from LGE and QC, numerology needs to be defined when NR SL is operate in ITS band (ex, SCS=30kHz for 5.9 GHz band) but the same numerologies for NR Uu are mandated when NR SL is operate in a band defined for Uu. |
| Huawei, HiSilicon | For SCS, we prefer Alt2 for dedicated band, e.g. ITS band and Alt1 for shared licensed band. We think subcarrier spacing should be defined/signaled per band. For dedicated band we agree to define a default numerology (30kHz for FR1) as in Alt 2 for ITS band since UE need to communicate with each other in OOC case. But for shared carrier, there’s an agreement “UE is not expected to use different numerology in the configured SL BWP and active UL BWP in the same carrier at a given time”, in this case the numerologies that are mandatory for NR UL are also should also be mandatory for NR SL to have the friendly co-existed with Uu interface. We agree 15-1, [15-2], 15-3 should be basic feature groups. For the DMRS pattern and the PSSCH symbols, we agree to introduce a mandatory capability for the NR SL UE that operates with 14-symbol slot in ITS spectrum. NOTE: We understand that anything agreed here does not have any effect whatsoever on agreeing more basic FGs in a later stage. We suggest keeping the list of basic FGs open before agreement and populating it with others until it is ready to be agreed as an overall package. |
| OPPO | We have the same view with Ericsson and Nokia; therefore, we support Alt. 1 when UL and SL are operating in a same carrier. In case of partial coverage, our understanding is that the network will ensure the numerology used for in-coverage SL and UL will be same as that for out-of-coverage (pre-configuration). On the other hand, when considering V2X operation in the ITS band, similar view to LGE, we are OK to have a default numerology as in Alt. 2. Regarding BFGs, we have very similar view as Ericsson. Specifically, the followings should be made as BFGs. - 15-1/15-1a should be merged and make the merged FG (e.g. merged 15-1) as a BFG - 15-3/15-3a should be merged and make the merged FG (e.g. merged 15-3) as a BFG - 15-2, 15-4, 15-11, 15-15 and 15-15a are also made as BFGs |
| CATT | Regarding to subcarrier spacing issue, we think it is better to follow NR Uu design, some default SCSs shall be mandatory for FR1 and FR2 respectively, e.g. {15KHz, 30KHz} for FR1, and {60KHz, 120KHz} for FR2. |
| Intel | For SL operation, we prefer Alt.2. We do not see issue with Uu link, since if UE supports other numerologies it can declare it over Uu signaling and can be properly configured. Otherwise its operation can be limited to carriers with 30kHz support only. Additionally, we can discuss whether UE supports shared carrier scenario first and corresponding numerologies. In our view, synchronization as a basic FG should precede discussion on basic communication FGs. As minimum, we would like to discuss framework of FGs to support basic Mode-2 only UEs and basic Mode-1 only UEs. It is also preferred to separately discuss TX and RX for each mode. |
| Apple | For SCS, we prefer Alt. 1. |
| ZTE,Sanechips | Alt 2 is preferred. |

The following are the high priority aspects of FGs {15-1, 15-3, 15-4, 15-10a, 15-11, 15-18, 15-19, 15-23, 15-24}. To the extend possible, we should refrain from discussing detailed component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites. As for whether the feature is mandatory or optional the notes on FL Proposal 1 apply.  As captured in the Chairman’s Notes, the primary goal of this email discussion is to stabilize the overall number of rows and separate FGs. To that end, the following summarizes all cases in [1] where companies’ proposals would impact the total number of rows or FGs. Since the primary goal is to stabilize the overall number of rows and separate FGs, for any proposed down-selection in FL Proposals {3, 5, 6, 10, 11, 13, 14, 16, 17} we are not agreeing on detailed component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites. We simply need to reach consensus on how many rows/FGs we need as new reference for a second round already approved by the RAN1 Chairman.

[100b-e-NR-UEFeatures-V2X-04] Email discussion/approval of Proposals {3, 4, 5, 8, 11, 12, 13, 14, 15, 16, 17}– focusing on the medium priority parts - in R1-2001867, dates TBD – Ralf (ATT)

With affixed structure in place, we can then address FL Proposal 1 and subsequently finalize component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites.

## FL Proposal 3 (15-1 receiving NR sidelink)

Alt. 1:

| 15-1 | Receiving NR sidelink [by preconfiguration] | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table in FR1]. 5) UE supports PT-RS reception in FR2. FFS: 6) The UE can receive [Z] total number of soft channel bits in a slot. 8) UE can receive using the subcarrier spacing it reports. FFS: 9) CP length 10) Supports 14-symbol SL slot with DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [11] UE can receive PSSCH with 256QAM in NR sidelink] | None | FFS | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink Component-2 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 Component-3 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 FFS: Component-6 candidate value set: {value1, value2, …} Component-8 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} FFS: whether to mandate an SCS. | Optional with capability signaling. For UE supports NR sidelink, UE must indicate this FG is supported. Candidate values for A are {value1, value2 …} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [15-1a] | FFS: Receiving NR sidelink configured by NR Uu | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table]. 5) UE supports PT-RS reception in FR2. 6) The UE can receive [Z] total number of soft channel bits in a slot. 7) UE supports 2 receive antennas. [8] 30 kHz SCS is supported for FR1.] | 15-1 |  |  |  |  |  |  |  | [Component-2 and Component-3 are the same as in FG15-1 and are not reported separately] | Optional with capability signalling |

Alt. 2:

| 15-1 | Receiving NR sidelink [by preconfiguration] | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table in FR1]. 5) UE supports PT-RS reception in FR2. FFS: 6) The UE can receive [Z] total number of soft channel bits in a slot. 8) UE can receive using the subcarrier spacing it reports. FFS: 9) CP length 10) Supports 14-symbol SL slot with DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH [11] UE can receive PSSCH with 256QAM in NR sidelink] | None | FFS | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink Component-2 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 Component-3 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 FFS: Component-6 candidate value set: {value1, value2, …} Component-8 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} FFS: whether to mandate an SCS. | Optional with capability signaling. For UE supports NR sidelink, UE must indicate this FG is supported. Candidate values for A are {value1, value2 …} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [15-1a] | FFS: Receiving NR sidelink configured by NR Uu | 1) UE can receive NR PSCCH/PSSCH. Up to [A] sidelink processes are supported. 2) UE can receive [X] PSCCH in a slot. 3) UE can decode [Y] RBs per slot (FFS: counting both PSCCH and PSSCH). 4) UE supports reception based on the normal 64QAM MCS table [and 256QAM MCS table]. 5) UE supports PT-RS reception in FR2. FFS: 6) The UE can receive [Z] total number of soft channel bits in a slot. 7) UE supports 2 receive antennas. [8] 30 kHz SCS is supported for FR1.] [9] UE can receive PSSCH with 256QAM in NR sidelink] 10) UE can receive using the subcarrier spacing it reports. | 15-1 |  |  |  | Per band |  |  |  | [Component-2 and Component-3 are the same as in FG15-1 and are not reported separately] Or This is the basic FG for sidelink Component-2 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 Component-3 candidate value set: {value1, value2, …} FFS: whether to report different value for each SCS indicated in component-8 FFS: Component-6 candidate value set: {value1, value2, …} Component-8 candidate value set in FR1: {{15 kHz}, {30 kHz}, {60 kHz}, {15, 30 kHz}, {30, 60 kHz}, {15, 60 kHz}, {15, 30, 60 kHz}} Component-8 candidate value set in FR2: {{60 kHz}, {120 kHz}, {60, 120 kHz}} FFS: whether to mandate an SCS. | Optional with capability signalling |

- High priority

  - Down-select between Alt. 1 and Alt. 2

Can we agree the down-selection with minor changes to component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites? If so, please indicate the preferred alternative. If not, please see next table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | Alt 1 is preferred. Concerns on the impact on the non-Uu-capable UEs can be addressed by clarifying that, for the sidelink operation in a frequency band, the UE supports the pre-configuration and it also supports the network configuration via Uu in a frequency band where the concurrent Uu operation is supported. Note that the non-Uu-capable UEs will report no concurrent Uu operation capability. |
| Ericsson | Alt 1 should be agreed. |
| Nokia | Alt 1. Besides the reasons raised by LGE above, it is not up to RAN1 to decide on potential support of non-Uu-capable UEs in the first place. |
| NTT DOCOMO | Alt 1. |
| Vivo | Alt 1 is preferred. Provision from network with proper configurations according to the location/environment is essential for NR V2X. Although it is possible for a UE supporting only PC5 operation in ITS band, the UE should be able to receive updated pre-configurations from network, which implies that Uu operation is anyway required in other band. |
| Qualcomm | Alt 2. Out-coverage and operation in a dedicate carrier without any Uu connection needs to be supported. |
| Panasonic | If this UE feature discussion for V2X is UE feature expression, whether the configuration by Uu is mandated for ITS dedicated carrier of out-of-coverage only UE is up to RAN2 discussion as its complexity and testing feasibility is RAN2 expertise. Therefore, RAN2 should decide the configuration of Uu is mandatory or not. If this UE feature discussion for V2X is only for the signaling to be reported for the network(or PC5-RRC), alt 1 is reasonable. |
| Samsung | Alt 1 |
| Huawei, HiSilicon | We prefer Alt. 2. It provides much better flexibility for NR sidelink UE design. Not all NR SL UEs should be able to receive NR SL through gNB configuration. For example, one NR SL UE that only operates in ITS band based on preconfiguration. If NR SL UE is forced to have Uu capability, the chipset cost incurred by the Uu protocol stack will be rather high. For the proposal from LG: There does not seem to be any “concurrent Uu” capability under consideration, so the outcome would be to force all UEs to have both PC5 and Uu interaces. If the intention is to add such signaling, it would amount to the same as defining 15-1a (and 15-2a) but in a more complex overall structure. To Panasonic: issues of UE hardware impact are not typically handed over to RAN2 first. We’d like to remove ‘FFS’ in component 6), i.e. keep 6) and remove ‘in a slot’ as following: FFS: 6) The UE can receive [Z] total number of soft channel bits in a slot. We think the total number of soft channel bits are necessary to show how many TBs can be decoded by the receiver totally if the initial transmission is not successfully decoded, such that the UE has to buffer re-transmissions. Note that this part is not captured by the current (proposed) formulation in terms of the number of sidelink processes and number of PSSCHs in a slot |
| OPPO | Alt 1 is preferred as a V2X UE for safety reason should be capable to operate in all operating scenarios (i.e. IC, OOC, PC). In addition, - components 7) should be updated to “UE supports rank 2 PSSCH receptions” - [ ] brackets around 256QAM MCS table should be removed. We don’t understand why 256QAM MCS table is limited to FR1 as we understand this is a baseband processing capability. We suggest to remove “in FR1”. |
| CATT | From our understanding, the sidelink operation without Uu connection shall be ensured as a basic feature, since the V2X UE may only operate in ITS band by pre-configuration. Therefore, our first preference is alt 2. If majority companies want to use alt 1, we think it should be add one component to indicate the UE capability to obtain the sidelink configuration. The candidate value for the new component can be {Pre-configuration, Pre-configuration and Uu} |
| Intel | Alt.1 NR sidelink is designed in a way that from UE reception perspective it is not important how configuration settings were provided. We assume that Alt.1 is transparent to pre-configuration or Uu configuration option. |
| Apple | Alt. 1 is preferred. |
| ZTE, Sanechips | We prefer Alt 2. We think it's better to have a FG defined clearly for non Uu capable UE. |

## FL Proposal 5 (15-3 transmitting NR sidelink mode 2)

Alt. 1:

| 15-3 | Transmitting NR sidelink mode 2 configured by [NR Uu or] preconfiguration Or Transmitting NR sidelink mode 2 configured by [NR Uu or] preconfiguration | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by [NR Uu or] preconfiguration. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform sensing and resource allocation operations. 6) UE can transmit using the subcarrier spacing it reports for FG 15-1. FFS: 7) CP length 8) Supports 14-symbol SL slot with DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH | 15-1 | FFS | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. This is the basic FG for sidelink in ITS spectrum where gNB is not defined and optional FG for licensed spectrum where gNB is defined | Optional with capability signalling For UE supports NR sidelink, for UE supports NR sidelink in ITS spectrum where gNB is not defined, UE must indicate this FG is supported, UE must indicate this FG is supported. Candidate values for B are {value1, value2 …} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-3a | FFS: Transmitting NR sidelink mode 2 configured by NR Uu | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table and 256QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform sensing and resource allocation operations. [5] UE supports rank 1 PSSCH transmissions.] [6] 30 kHz SCS is supported for FR1.] | 15-1, 15-2 |  |  |  |  |  |  |  | Note: Random selection in the exceptional pool is supported. | Optional with capability signalling |

Alt. 2:

| 15-3 | Transmitting NR sidelink mode 2 configured by [NR Uu or] preconfiguration | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by [NR Uu or] preconfiguration. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform sensing and resource allocation operations. 6) UE can transmit using the subcarrier spacing it reports for FG 15-1. FFS: 7) CP length 8) Supports 14-symbol SL slot with DMRS patterns corresponding to {#PSSCH symbols, #DMRS symbols} = {12, 4}, {9, 3} for slots w/wo PFSCH | 15-1 | FFS | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. This is the basic FG for sidelink in ITS spectrum where gNB is not defined and optional FG for licensed spectrum where gNB is defined | Optional with capability signalling For UE supports NR sidelink, for UE supports NR sidelink in ITS spectrum where gNB is not defined, UE must indicate this FG is supported, UE must indicate this FG is supported. Candidate values for B are {value1, value2 …} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-3a | FFS: Transmitting NR sidelink mode 2 configured by NR Uu | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table and 256QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform sensing and resource allocation operations. [5] UE supports rank 1 PSSCH transmissions.] [6] 30 kHz SCS is supported for FR1.] 7) UE can transmit using the subcarrier spacing it reports for FG 15-1. | 15-1, 15-2 |  |  |  | Per band |  |  |  | Note: Random selection in the exceptional pool is supported. | Optional with capability signalling |

Alt. 3:

| 15-3 | Transmitting NR sidelink mode 2 configured by [NR Uu or] preconfiguration | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by [NR Uu or] preconfiguration. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform sensing and resource allocation operations. 6) UE can transmit using the subcarrier spacing it reports for FG 15-1. FFS: 7) CP length | 15-1 | FFS | No |  | Per band | N.A. | N.A. | N.A. | Note: Random selection in the exceptional pool is supported. This is the basic FG for sidelink. | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-3a | FFS: Transmitting NR sidelink mode 2 configured by NR Uu | 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 configured by NR Uu. Up to [B] sidelink processes are supported. 2) UE supports transmission based on the normal 64QAM MCS table and 256QAM MCS table. 3) UE supports PT-RS transmission in FR2. 4) UE can perform sensing and resource allocation operations. [5] UE supports rank 1 PSSCH transmissions.] [6] 30 kHz SCS is supported for FR1.] 7) Support sidelink pathloss based open loop power control and RSRP report in case of unicast 8) UE can transmit using the subcarrier spacing it reports for FG 15-1. | 15-1, 15-2 |  |  |  | Per band |  |  |  | Note: Random selection in the exceptional pool is supported. | Optional with capability signalling |
| 15-3x | Sensing | 1) UE is capable of performing sensing | At least one of 15-3, 15-3a | Yes | No |  | Per band | N.A. | N.A | N.A |  | Optional with capability signaling |

- High priority

  - Down-select between Alt. 1, Alt. 2 and Alt. 3

Can we agree the down-selection with minor changes to component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites? If so, please indicate the preferred alternative. If not, please see next table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | Alt 1 is preferred, and we propose to make the same clarification proposed for 15-1 here. |
| Ericsson | We support Alt.1 similar to 15-1. |
| Nokia | Alt. 1, similarly to 15-1. |
| NTT DOCOMO | Alt. 1. |
| Vivo | Alt. 1 is preferred with a separate sensing capability (in Alt.3) is defined together. This is beneficial for supporting PUE in next release. |
| Qualcomm | Alt 2. Same as 15-1: need to support out-of-coverage UEs and UEs operating only in a dedicated carrier without Uu. All DMRS patterns should be supported “in ITS spectrum where gNB is not defined and optional FG for licensed spectrum where gNB is defined” should be removed. Mode 2 operation is not limited to ITS spectrum. |
| Panasonic | If this UE feature discussion for V2X is UE feature expression, whether the configuration by Uu is mandated for ITS dedicated carrier of out-of-coverage only UE is up to RAN2 discussion as its complexity and testing feasibility is RAN2 expertise. Therefore, RAN2 should decide the configuration of Uu is mandatory or not. If this UE feature discussion for V2X is only for the signaling to be reported for the network(or PC5-RRC), alt 1 is reasonable. |
| Samsung | Alt. 1 is preferred with a separate sensing capability (same view with vivo) |
| Huawei, HiSilicon | We prefer Alt. 2. The reason is the same as that for FL proposal 1, i.e., it provides much better flexibility for NR sidelink UE design. It is proposed FG 15-23 should be a component of FG 15-2, 15-3, and 15-3a, which is similar to component 7) in Alt 3. 15-3a. |
| OPPO | We support Alt. 1 for the same reason that we supported Alt. 1 to merge 15-1/15-1a. Alt 3 should not even be here. The scope of this WI does not include V2P use cases that is not listed in WI justification section. For the approved Rel-17 eSL WI, power saving mechanisms for PUE and PS use cases with reduced or no sensing are the main focus. |
| CATT | Same as for 15-1/15-1a, our first preference is alt 2. If majority companies want to use alt 1, we think it should be add one component to indicate the UE capability to obtain the sidelink configuration. The candidate value for the new component can be {Pre-configuration, Pre-configuration and Uu} |
| Intel | Alt.2 is closer to our preference. However, we propose to split feature of transmission and configuration. We prefer to split support of different type of SL configuration support (pre-configuration, eNB Uu based configuration and gNB Uu based configuration) |
| Apple | Alt 3 is preferred. For pedestrian UE, sensing is not always required. |

## FL Proposal 6 (15-4 synchronization source)

Alt. 1:

| 15-4 | GNSS and S-SSB for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-15a | gNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an gNB eNB. 2) If UE supports 15-4, UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4, UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

Alt. 2:

| 15-4 | gNB, GNSS and S-SSB for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. 4) UE can transmit or receive NR sidelink based on the synchronization to an gNB 5) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 6) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-15a | gNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an eNB. 2) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

Alt. 3:

| 15-4 | GNSS and S-SSB for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-15a | gNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an gNB eNB. 2) If UE supports 15-4, UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4, UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |
| 15-15b | GNSS synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to GNSS. 2) If UE supports 15-4, UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS. 3) If UE supports 15-4, UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. 4) If UE supports 15-4, UE additionally supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

Alt. 4:

| 15-4 | GNSS as the synchronization reference and S-SSB for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports only GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. | Optional with capability signalling For UE supports NR sidelink, UE must indicate this FG is supported. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15-4a | S-SSB transmission/reception for NR sidelink | 1) UE can receive S-SSB in NR sidelink if it supports 15-1. 2) UE can transmit S-SSB in NR sidelink if it supports 15-2 or 15-3. 3) UE supports GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to false. | At least one of 15-1, 15-2, 15-3, 15-4. | Yes | No |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. | Optional with capability signalling |
| 15-15a | gNB type synchronization source for NR sidelink | 1) UE can transmit or receive NR sidelink based on the synchronization to an gNB eNB. 2) If UE supports 15-4 and 15-4a, UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to gnbEnb. 3) If UE supports 15-4 and 15-4a, UE additionally supports gNB, GNSS and SyncRef UE as the synchronization reference according to the synchronization procedure with sl-SyncPriority set to GNSS and sl-NbAsSync set to true. | At least one of 15-1, 15-2, 15-3 | Yes | No |  | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling. |

- High priority

  - Down-select between Alt. 1, Alt. 2, Alt. 3, and Alt. 4

Can we agree the down-selection with minor changes to component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites? If so, please indicate the preferred alternative. If not, please see next table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | Alt 1 is preferred. Alt 2 is okay if gNB-based synchronization is supported only when the UE supports concurrent operation of Uu and SL (similarly to what we proposed for 15-1 and 15-3). |
| Ericsson | We support Alt. 2. In our view, we should try to limit the alternatives. The only discussion that should take place is to whether to have eNB/gNB as part of basic feature group for synchronization or not (i.e. Alt. 1 vs Alt. 2) |
| Nokia | Alt 2 |
| NTT DOCOMO | Alt 2. |
| vivo | We slightly prefer Alt.2. |
| Qualcomm | Alt 1. Same as earlier comment, need to support UEs without a Uu connection |
| Panasonic | If this UE feature discussion for V2X is UE feature expression, whether the configuration by Uu is mandated for ITS dedicated carrier of out-of-coverage only UE is up to RAN2 discussion as its complexity and testing feasibility is RAN2 expertise. Therefore, RAN2 should decide the configuration of Uu is mandatory or not. If this UE feature discussion for V2X is only for the signaling to be reported for the network(or PC5-RRC), alt 1 is reasonable. |
| Huawei, HiSilicon | Alt 1 is preferred. Alt2 is undesirable since it mixes gNB and GNSS/SyncRefUE together. In our view, whether to support gNB sync source should be a separate UE capability. For Alt3 and Alt4, they are similar with Alt 1, but we do not need to separate GNSS and UE type synchronization source since both of them are important for ITS application to derive sidelink timing under pre-configuration. To Panasonic: issues of UE hardware impact are not typically handed over to RAN2 first. |
| OPPO | We support Alt. 2 for the same reason that we support to merge 15-1/15-1a and 15-3/15-3a. |
| CATT | Our preference is alt 1, and whether supporting gNB/eNB sync source is an optional feature |
| Intel Corporation | At first, we believe SL sync should be pre-requisite of any SL communication type (and not vice versa as it is implemented now) Alt.4 or Alt.3 is closer to our understanding (we need to clarify difference) SLSS TX and RX should be different UE feature groups Reference sources GNSS, gNB, eNB, SLSS should be different feature groups |
| Apple | We prefer Alt.3. In Alt. 3, FG 15-4 is the only one in basic FG for sidelink. We think that is enough for a UE to operate on sidelink. |
| ZTE, Sanechips | Alt.3. We think the same principle of 15-1 and 15-1a split should apply here. A UE could function with either GNSS or gNB based synchronization. We don't see the reason why GNSS is grouped together with SSB as basic feature group. |

## FL Proposal 11 (15-11 PSFCH format 0)

| 15-11 | PSFCH format 0 | 1) UE can transmit and receive NR PSFCH format 0 2) UE can receive [N] PSFCH(s) in a slot. 3) UE can transmit [M] PSFCH(s) in a slot. 4) UE can report sidelink HARQ-ACK to gNB via PUCCH and PUSCH when it is operating in NR sidelink mode 1. [FFS: move to 15-2 or to new FG 15-11a] | At least one of 15-1, 15-2, 15-3 | Yes | Yes |  | Per band | N.A. | N.A. | N.A. | This is the basic FG for sidelink. | Optional with capability signalling FFS: For UE supports NR sidelink, UE must indicate this FG is supported. ALT 1) Candidate values for N are {5, [10,] 15, [20,] 25, [30,] 35, [40,] 45, 50 } ALT 2) Candidate values for N are {32, 64} Candidate values for M are {1, 4, 8, 16} |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- High priority

  - Resolve FFS in component (4)

How to resolve the FFS for component 4 (new FG, move components to other FG, relation to FL Proposal 1 …)?

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | We think component 4 should remain in 15-11. If it is moved to 15-2, does it mean that a UE not supporting PSFCH should be able to report HARQ-ACK to gNB? |
| Ericsson | In our view, (4) should be part of 15-11 FG as it is now. |
| NTT DOCOMO | Component 4 should be in this FG. The current specification does not define UE behavior that transmits/receives PSFCH but does not support HARQ-ACK to gNB. |
| vivo | We prefer to have component 4 in 15-11. |
| Panasonic | The component 4 should be 15-2 and it should be described as "component 4 is mandatory when UE support 15-11". |
| Samsung | Since Component 4) is not about PSFCH format, it is not proper this component is included this FG. This can be moved into new FG (for example, 15-11a) |
| Huawei, HiSilicon | Support FG 15-11 as a basic FG for NR SL, after moving component 4) to FG 15-2, since DG, CG type 1 and CG type 2 all provide the PSFCH-to-HARQ timing indicator. It means component 4) is essential for mode 1 sidelink transmission scheduled by Uu. It is also okay to add a new FG 15-11a, which considers FG 15-2 as prerequisite FG. For the candidate value set, we suggest {1, 4, 8, 16} for M and Alt. 2 for N. |
| OPPO | Component 4) should be moved to 15-2 as this is only useful for mode 1 operation in a licensed band. As to LGE’s question, once 15-11 is confirmed as a BFG, PSFCH will be supported by V2X UE. |
| CATT | Component-4 should be removed from 15-11, since the V2X UE may only operate in ITS band without Uu connection. We are open to either move to 15-2 or a new FG(e.g. 15-11a)for i. |
| Intel | Component-4 should be moved out (i.e. separate feature). Desirable to split on TX and RX support of PSFCH. |
| Apple | Component-4 should be moved to a new FG 15-11a. |
| ZTE, Sanechips | We prefer to move it to a new FG 15-11a |

## Feature groups transparent to pre-configuration versus Uu configuration

## FL Proposal 10 (15-10a 256QAM)

Alt. 1: delete 15-10a

Alt. 2:

| 15-10a | FFS: 256QAM sidelink reception | 1) UE can receive PSSCH with 256QAM in NR sidelink | 15-1 | Yes | Yes | UE supports QPSK, 16QAM, and 64 QAM for reception only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- High priority

  - Down-select between Alt. 1 and Alt. 2

Can we agree the down-selection with minor changes to component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites? If so, please indicate the preferred alternative. If not, please see next table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | Alt 2 is preferred. |
| Ericsson | We support Alt. 1. i.e. 256QAM should be mandatory from reception point of view. |
| Nokia | Alt. 1, as otherwise it is not feasible to utilize the feature in most scenarios. |
| NTT DOCOMO | Both are OK for us. If alt. 1 is supported, 256QAM TX can be (pre-)configured and can be used for broadcast/groupcast. If alt. 2 is adopted, 256QAM table can be used only in unicast. This is our understanding. |
| vivo | Alt.2 is preferred, and it can be defined as a basic FG for VUE. Reception of 256QAM can be optional for PUE. |
| Qualcomm | Using 256-QAM without a CQI report would lead to reliability issues. Since CSI reporting in only supported in unicast, this would be primary use case for 256-QAM. Alt 2 will not limit deployment. |
| Panasonic | Alt 2 is our preference. |
| Samsung | Alt 2 is preferred. |
| Huawei, HiSilicon | Alt 1 should be agreed. We think 256QAM reception should be mandatory for NR SL. If 256QAM reception is optional and needs capability signaling, the gNB needs the capabilities of both the TX UE (256QAM transmission), and would benefit from knowing the RX UE so that it can set the MCS ranges properly for the TX UE. Furthermore, if 256QAM is set as an optional feature, the TX UE can never use 256QAM in groupcast or broadcast scenarios, even though there is no consequence on UE complexity. As such, we believe FG 15-10a should be removed and act as a component in FG 15-1 and FG 15-1a. |
| OPPO | Alt. 1 is preferred for exactly same reason as we did with 64QAM reception in Rel-15 for LTE-V2X. As such, the [ ] brackets around 256QAM MCS table in 15-1 should be removed as also commented earlier. |
| Intel | Alt.2 Should be supported only for unicast communication based on UE-UE indication |
| Apple | Alt 2 is preferred. |
| ZTE,Sanechips | Alt 2 is preferred. We think 256QAM may not necessarily be used in broadcast/groupcast. |

## FL Proposal 13 (15-18 rank 2 transmission)

Alt. 1: delete 15-18

Alt. 2:

| 15-18 | Support of rank 2 transmission | 1) UE additionally supports rank 2 PSSCH transmission | At least one of 15-2 and 15-3 | Yes | No | UE supports rank 1 PSSCH transmission only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- High priority

  - Down-select between Alt. 1 and Alt. 2

Can we agree the down-selection with minor changes to component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites? If so, please indicate the preferred alternative. If not, please see next table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | We prefer Alt 2. |
| Ericsson | We prefer Alt. 2. |
| Nokia | Alt 2 |
| NTT DOCOMO | Alt 2 |
| vivo | Alt. 2 is preferred. |
| Qualcomm | Alt 2 |
| Panasonic | Alt 2. |
| Samsung | Alt 2. |
| Huawei, HiSilicon | We prefer Alt. 2 |
| OPPO | We prefer Alt. 2. In addition, this needs to be reported to the other UE in unicast for RI and CQI reporting. |
| CATT | Alt 2 |
| Intel | Alt.2 |
| Apple | Alt 2 |

The following working assumption was agreed during the conference call on April 21, 2020.

Working assumption: Agree on FG 15-18 as shown below:

| 15-18 | Support of rank 2 transmission | 1) UE additionally supports rank 2 PSSCH transmission | [At least one of 15-2 and 15-3] | FFS | FFS | UE supports rank 1 PSSCH transmission only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

## FL Proposal 14 (15-19 rank 2 reception)

Alt. 1: delete FG 15-19

Alt. 2:

| 15-19 | FFS: Support of rank 2 reception | 1) UE additionally supports rank 2 PSSCH reception | 15-1 | FFS | Yes | UE supports rank 1 PSSCH reception only. | Per band | N.A. | N.A. | N.A. |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- High priority

  - Down-select between Alt. 1 and Alt. 2

Can we agree the down-selection with minor changes to component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites? If so, please indicate the preferred alternative. If not, please see next table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | We prefer Alt 2. |
| NTT DOCOMO | Alt 2. |
| vivo | Alt.2 is preferred, and it can be defined as a basic FG for VUE. Rank2 reception can be optional for PUE. |
| Qualcomm | Alt 2: Similar to 256-QAM, CSI reporting (RI) is needed to reliably use 2-layer transmissions. |
| Panasonic | Alt 2 |
| Samsung | Alt 2 |
| Huawei, HiSilicon | We think Alt. 1 is more reasonable. Our reason is similar to that for FL proposal 10, i.e., rank-2 reception should be mandatory for NR SL to ensure a clear step forward compared to LTE. |
| OPPO | Alt. 1, same reason as 256QAM reception. |
| Intel | Alt 2 support is subject to UE-to-UE signaling |
| Apple | Alt 2 |

## FL Proposal 16 (15-23 RSRP report)

Alt. 1: Delete FG 15-23 and add new component to FG 15-2a, FG 15-3 and FG 15-3a as “Support sidelink pathloss based open loop power control and RSRP report in case of unicast”

Alt. 2:

| 15-23 | FFS: Support of open loop SL power control and RSRP report | [1) Support sidelink pathloss based open loop power control and RSRP report in case of unicast] | FFS | FFS | Yes |  | Per band | N.A. | N/A | N.A. |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- High priority

  - Down-select between Alt. 1 and Alt. 2

Can we agree the down-selection with minor changes to component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites? If so, please indicate the preferred alternative. If not, please see next table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | We prefer Alt 2. And we prefer keeping the original text “open loop SL power control and” to also include the capability of adjusting TX power based on the RSRP report. |
| NTT DOCOMO | We support Alt. 2 without removal of OLPC and with clarification. FG 15-23 describes only SL pathloss-based OLPC as ‘Components’ explains, not DL pathloss-based OLPC. Meanwhile, the name of FG is ‘Support of open loop SL power control and RSRP report’, which can be interpreted as that DL pathloss-based OLPC is included as well. To avoid the misunderstanding, the name should be updated. |
| vivo | Alt.1 is preferred. |
| Qualcomm | Alt 2, keeping “open loop SL power control and” |
| Panasonic | Alt 2 |
| Samsung | Same view with QC. |
| Huawei, HiSilicon | We think open loop power control is essential for NR SL to suppress the interference. Therefore, Alt. 1 is preferred. |
| OPPO | Alt. 1 is preferred as open loop power control and RSRP reporting is essential to limit the interference. |
| Intel | Alt.2. Prefer to split on report and control. |
| Apple | Alt. 2 |

## FL Proposal 17 (15-24 multiple synchronization references)

Alt. 1: Delete 15-24

Alt. 2:

| 15-24 | FFS: Support of multiple synchronization references | [1) UE can support sidelink reception using up to A synchronziaion references in a carrier/BWP.] | At least one of 15-1, 15-2, 15-3 | Yes | No | UE supports only a single synchronization reference in a carrier/BWP. | Per band | N.A. | N.A. | N.A. | Component-1 candidate value set: {1, 2, 3, 4} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- High priority

  - Down-select between Alt. 1 and Alt. 2

Can we agree the down-selection with minor changes to component descriptions and candidate values, as well as type, xDD/FRx differentiation, whether the gNB needs to know if the feature is supported, whether capability exchange between UEs is applicable, notes, consequences if a feature is not supported by a UE, and prerequisites? If so, please indicate the preferred alternative. If not, please see next table.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| LGE | Alt 1 is preferred. Now it is decided not to treat the issue of multiple synchronization references in this e-meeting, and we don’t see a value of defining the corresponding FG. |
| Ericsson | We support Alt. 2. |
| Qualcomm | Alt 1. RAN1 has not agreed to support this feature nor defined a procedure on how to use it. |
| Panasonic | Alt 2 as the meaning only 1 synchronization source. Not to define it means only 1 is also ok. |
| Huawei, HiSilicon | Alt 2 is preferred. What we would like to understand is the impact if this FG is not defined. E.g., each RX resource pool can be configured with a sync reference, and there can be up to 16 RX resource pools. How many different references is the system (network or pre-configuration) allowed to use if this FG is not defined? Does it imply that all such resource pools have the same source, even if some are under asynchronous gNBs, or some under gNB and some under eNB, or GNSS, etc.? Note that LTE-V2X defines a similar capability, and not as a part of D2D. If RAN1 takes the direction that all UEs have a single reference for synchronization for all purposes (i.e. all resource pools, all network sources, all GNSS are synchronous by assumption), this has impacts on other WGs which they need to be informed of. To us, it seems practical to define this feature, and it implies little RAN1 effort. Candidate values of component-1 set: {1, 2, 3, 4}. |
| OPPO | We support Alt. 2. |
| Intel | Definition of feature is unclear. Need more discussion whether it is about SLSS, Uu, etc. |
| Apple | Alt. 2 |

…

# Conclusions

…

# References

- R1-2001867, Summary on UE features for 5G V2X, Moderator (AT&T)