3GPP TSG RAN WG1 #100bis			                                   					       												        R1-200xxxx

e-Meeting, April 20th – 30th, 2020

Agenda Item:	7.2.11.6

Source:	Moderator (AT&T)

Title:	Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-02]

Document for:	Discussion/Decision

# Introduction

This document presents the summary of email approval [100b-e-NR-UEFeatures-eMIMO-02]  during RAN1 #100bis-e. According to the Chairman’s Notes:

[100b-e-NR-UEFeatures-eMIMO-02] Email discussion/approval till 4/24 – Ralf (ATT), (16-2 family) - Decide how many new FGs to add as a result of separating basic from optional components in 16-2a “Multi-DCI based multi-TRP” - Decide how many new FGs to add as a result of separating basic from optional components in 16-2b “Single-DCI based multi-TRP”

The following was discussed and agreed during RAN1 #100bis-e within the scope of [100b-e-NR-UEFeatures-eMIMO-02]  “Email discussion/approval” Error: Reference source not found.

The following will be removed from the final document, however, in the meantime, please take note of this guidance of the RAN1 MCC technical officer:

W.r.t the naming convention, the following suggestion […] may be helpful to keep the previous company’s name (only the most recent one) in the filename, so that we can easily tell which previous version this is based on, and may solve the issue when there are crossing emails. e.g. something like the following: 5_Incoming_Liaison_Statements/Summary-1_v1-LG 5_Incoming_Liaison_Statements/Summary-1_v2-LG-CATT 5_Incoming_Liaison_Statements/Summary-1_v2-LG-vivo 5_Incoming_Liaison_Statements/Summary-1_v3-CATT-HWHiSi

# Summary of Email Approval [100b-e-NR-UEFeatures-eMIMO-02]

The following is the proposal in Error: Reference source not found for discussion in this email approval:

FL Proposal 2: (16-2 family)

- Decide how many new FGs to add as a result of separating basic from optional components in 16-2a “Multi-DCI based multi-TRP”

- Decide how many new FGs to add as a result of separating basic from optional components in 16-2b “Single-DCI based multi-TRP”

## Multi-DCI based multi-TRP

The following are the alternatives in Error: Reference source not found for discussion in this email approval:

Alt. 1:

| 16-2a | Multi-DCI based multi-TRP | Basic components: - The maximum number of CORESETs configured per “PDCCH-Config” (other than CORESET 0) - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” (other than CORESET 0) - The value of R=[1,2] for BD/CCE - Support of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) - Support of out-of-order operation for PDCCH to PDSCH (FFS whether to be a basic component) - Support of out-of-order operation for PDSCH to HARQ-ACK (FFS whether to be a basic component) - Support of out-of-order operation for PDCCH to PUSCH (FFS whether to be a basic component) - FFS: The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - FFS: the maximum number of CCs supporting multi-DCI based multi-TRP Optional components: - Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex - FFS: Support of two PDSCH scrambling sequences per serving cell - Support of default QCL assumption per CORESETPoolIndex - Support of separate HARQ-ACK - Support of joint HARQ-ACK - Support of two TDMed long PUCCHs in a slot | TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2a-2 | Different PDSCH scrambling per TRP | FFS: Support of two PDSCH scrambling sequences per serving cell | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| 16-2a-3 | Default QCL enhancement fr multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| 16-2a-4 | HARQ-ACK for multi-DCI based multi-TRP | - Support of separate HARQ-ACK - Support of joint HARQ-ACK - Support of two TDMed long PUCCHs in a slot (two TDMed long PUCCHs within a slot, TDMed short PUCCH and long PUCCH within a slot, TDMed short PUCCH and short PUCCH within a slot) | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |

Alt. 2:

| 16-2a | Multi-DCI based multi-TRP | Basic components: - The maximum number of CORESETs configured per “PDCCH-Config” (other than CORESET 0) - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” (other than CORESET 0) - The value of R=[1,2] for BD/CCE - Support of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) - Support of out-of-order operation for PDCCH to PDSCH (FFS whether to be a basic component) - Support of out-of-order operation for PDSCH to HARQ-ACK (FFS whether to be a basic component) - Support of out-of-order operation for PDCCH to PUSCH (FFS whether to be a basic component) - FFS: The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - FFS: the maximum number of CCs supporting multi-DCI based multi-TRP - Support of two PDSCH scrambling sequences per serving cell - Support of separate HARQ-ACK Optional components: - Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex - FFS: Support of two PDSCH scrambling sequences per serving cell - Support of default QCL assumption per CORESETPoolIndex - Support of separate HARQ-ACK - Support of joint HARQ-ACK - Support of two TDMed long PUCCHs in a slot | TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2a-3 | Default QCL enhancement fr multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| 16-2a-4 | Joint HARQ-ACK for multi-DCI based multi-TRP | Support of joint HARQ-ACK | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| 16-2a-5 | Two TDMed long PUCCHs in a slot | Support of two TDMed long PUCCHs in a slot (two TDMed long PUCCHs within a slot, TDMed short PUCCH and long PUCCH within a slot, TDMed short PUCCH and short PUCCH within a slot) | 16-2a |  |  |  |  |  |  |  |  |  |

Alt. 3:

| 16-2a | Multi-DCI based multi-TRP | Basic components: - The maximum number of CORESETs configured per “PDCCH-Config” (other than CORESET 0) - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” (other than CORESET 0) - The value of R=[1,2] for BD/CCE - Support of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) - Support of out-of-order operation for PDCCH to PDSCH (FFS whether to be a basic component) - Support of out-of-order operation for PDSCH to HARQ-ACK (FFS whether to be a basic component) - Support of out-of-order operation for PDCCH to PUSCH (FFS whether to be a basic component) - FFS: The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - FFS: the maximum number of CCs supporting multi-DCI based multi-TRP - Indication of supporting type(s) of separate/joint HARQ-ACK (candidate values {separate, joint, both}) - Support of default QCL assumption per CORESETPoolIndex - PDSCH processing capability for a CC configured with multi-DCI - Maximum number of unicast PDSCHs per CORESETPoolIndex per slot Optional components: - Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex - FFS: Support of two PDSCH scrambling sequences per serving cell - Support of default QCL assumption per CORESETPoolIndex - Support of separate HARQ-ACK - Support of joint HARQ-ACK - Support of two TDMed long PUCCHs in a slot | TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  | Component 1 candidate values: {2, 3, 4} Component 2 candidate values: {1, 2} Component 3 candidate values: {1, 2} Component 13 candidate values: {Capability 1, Capability 2 with scheduling limitation, Capability 2} Component 14 candidate values: {1, 2, 4, 7} | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2a-2 | Different PDSCH scrambling per TRP | FFS: Support of two PDSCH scrambling sequences per serving cell | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| 16-2a-3 | Out-of-order operation for PDCCH to PDSCH | Support of out-of-order operation for PDCCH to PDSCH (FFS whether to be a basic component) | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| 16-2a-4 | Out-of-order operation for PDSCH to HARQ-ACK | Support of out-of-order operation for PDSCH to HARQ-ACK (FFS whether to be a basic component) | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| 16-2a-5 | Out-of-order operation for PDCCH to PUSCH | Support of out-of-order operation for PDCCH to PUSCH (FFS whether to be a basic component) | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| 16-2a-6 | Simultaneous reception for PDCCH/PDSCH | Support of simultaneous reception for PDCCH/PDSCH associated with two QCL-Type-D RSs | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| 16-2a-7 | PDSCHs overlapping types | Indication of supported “PDSCHs overlapping types”, with bitmap signalling for {fully, partial, non-overlapped TDM, non-overlapped FDM}.” | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |

Alt. 4:

| 16-2a | Multi-DCI based multi-TRP | Basic components: - The maximum number of CORESETs configured per “PDCCH-Config” (other than CORESET 0) - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” (other than CORESET 0) - The value of R=[1,2] for BD/CCE - Support of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) - PDSCH processing capability for a CC configured with multi-DCI - Maximum number of unicast PDSCHs per CORESETPoolIndex per slot - Support of out-of-order operation for PDCCH to PDSCH (FFS whether to be a basic component) - Support of out-of-order operation for PDSCH to HARQ-ACK (FFS whether to be a basic component) - Support of out-of-order operation for PDCCH to PUSCH (FFS whether to be a basic component) - FFS: The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - FFS: the maximum number of CCs supporting multi-DCI based multi-TRP Optional components: - Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex - FFS: Support of two PDSCH scrambling sequences per serving cell - Support of default QCL assumption per CORESETPoolIndex - Support of separate HARQ-ACK - Support of joint HARQ-ACK Support of two TDMed long PUCCHs in a slot | TBD |  | N/A |  | per FSPC | N | TBD |  | Component 1 candidate values: {2, 3, 4} Component 2 candidate values: {1, 2} Component 3 candidate values: {1, 2} Component 4 candidate values: {Capability 1, Capability 2 with scheduling limitation, Capability 2} Component 5 candidate values: {1, 2, 4, 7} | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2a-1 | Out-of-order for multi-DCI based multi-TRP | - Support of out-of-order operation for PDCCH to PDSCH (FFS whether to be a basic component) - Support of out-of-order operation for PDSCH to HARQ-ACK (FFS whether to be a basic component) - Support of out-of-order operation for PDCCH to PUSCH (FFS whether to be a basic component) | 16-2a |  |  |  | Per UE |  |  |  |  |  |
| 16-2a-2 | Maximum number of active TCI states for multi-DCI based multi-TRP | - Maximum number of active TCI states per BWP per CC per value of CORESETPoolIndex (including both data and control) - Support 16 active TCI states per BWP per CC, including control and data | 16-2a |  |  |  | Per band |  |  |  | Component 1 candidate values: {1, 2, 4, 8} |  |
| 16-2a-3 | CRS rate matching for multi-DCI based multi-TRP | Support of rate match around configured CRS patterns which is associated with CORESETPoolIndex and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex | 16-2a |  |  |  | Per UE |  |  |  |  |  |
| 16-2a-4 | Multi-beam for multi-DCI based multi-TRP | Indicates whether UE supports receiving time-overlapping PDSCHs/PDCCHs with different beams | 16-2a |  |  |  | Per band | TDD only | FR2 only |  |  |  |
| 16-2a-5 | Two default beams for multi-DCI based multi-TRP | Support of default QCL assumption per CORESETPoolIndex | 16-2a, 16-2a-5 |  |  |  | Per band | TDD only | FR2 only |  |  |  |
| 16-2a-6 | Feedback for multi-DCI based multi-TRP | - Support of separate HARQ-ACK - Support of joint HARQ-ACK - Support of two TDMed long PUCCHs in a slot | 16-2a |  |  |  | Per UE |  |  |  |  |  |
| 16-2a-7 | PDSCH scrambling for multi-DCI based multi-TRP | FFS: Support of two PDSCH scrambling sequence per serving cell | 16-2a |  |  |  | Per UE |  |  |  |  |  |

Note there were different options for “separate CRS rate matching” in [1], however, both require one row/FG.

Alt. 1:

| 16-2a-1 | Separate CRS rate matching | Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Alt. 2:

| 16-2a-1 | Separate CRS rate matching | Support of CRS rate matching for Multi-DCI based Multi-TRP operation. “Joint” is mandatory for UE that supports Multi-DCI based Multi-TRP CRS rate matching, but “Separate” is optional - Joint: UE rate match around the union of CRS from both TRPs - Separate: UE rate match around configured CRS patterns which is associated with CORESETPoolIndex (if not configured, CORESETPoolIndex=0) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex | 16-2a, TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

According to the feedback in [1], two companies preferred Alt. 2, four companies Alt. 2, two companies Alt. 3, and  four companies Alt. 4. Since both Alt. 1 and Alt. 2 require four rows/FGs, half the companies think four FGs are enough for multi-DCI based multi-TRP. However, there is much difference in the details between Alt. 1 and Alt. 2. Alt. 3 and Alt. 4 have in common that basic components {6,7,8} are removed as basic and become three (Alt. 3) or one (Alt. 4) new row/FG.

How many rows/FGs are needed for out-of-order for multi-DCI based multi-TRP? None (basic), one or three?

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We prefer to have 3 FG, one for each OOO type, i.e. PUSCH/PDSCH/HARQ-ACK |
| MediaTek | We can decide how many FGs are needed later after the basic/optional components are clear. (e.g., the OOO ones.) |

This is one of the most unstable sections of the eMIMO NR UE feature list. Please make constructive comments or suggestions how to finalize the number of rows/FGs for muti-DCI based multi-TRP.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | We are flexible as long as it provides UE with flexible capability signalling. In addition to the basic feature group, we prefer additional FG(s) to cover the following - Different PDSCH scrambling per TRP - Out-of-order operation for PDCCH to PDSCH - Out-of-order operation for PDSCH to HARQ-ACK - Out-of-order operation for PDCCH to PUSCH - Simultaneous reception for PDCCH/PDSCH - CRS rate matching - HARQ-ACK feedback mode - Inter-slot, Intra-slot separate, joint - Intra-slot separate (1) Long + Long (2) Long + Short (3) Short + Short (Rel-15 does not support 2 HARQ-ACK PUCCH in a slot) - PDSCHs overlapping types - Support of default QCL assumption per CORESETPoolIndex - The maximum number of CCs with Multi-DCI MTRP operation. Further discussion is need to consider the joint configuration of the number of CCs with single TRP - Support of common closed-loop power control process for PUCCH/PUSCH associated with different CORESETPoolIndex |
| MediaTek | We suggest to clarify RAN1’s understanding on scenarios supporting M-DCI operation to facilitate discussion. - Can we support M-DCI operation if a UE supports only one Rx beam? - Can we support M-DCI operation in ideal backhaul case? - Can we support M-DCI operation with only non-overlapping PDSCHs (i.e., no full/partial overlapping)? If all answers are YES, we think optional capability signaling for OOO, overlapping type, and simultaneous reception for PDCCH/PDSCH (i.e., 2, 3, 4, 5, 8 in Apple’s summary and they probably the most controversial ones) are all necessary. |

The following highlighted parts were briefly discussed during the first RAN1 #100bis-e eMIMO conference call on Monday, April 20, 2020.

| Multi-DCI based multi-TRP | Basic components: - The maximum number of CORESETs configured per “PDCCH-Config” - The maximum number of CORESETs configured per CORESETPoolIndex ( if CORESETPoolIndex is not configured, it is assumed CORESETPoolIndex = 0) per “PDCCH-Config” - The value of R=[1,2] for BD/CCE - Support of fully/partially time/frequency overlapped PDSCH reception (PDSCHs overlapping types in time and frequency domain) (requires 5/6/7 to schedule) - Support of out-of-order operation for PDCCH to PDSCH (FFS whether to be a basic component) - Support of out-of-order operation for PDSCH to HARQ-ACK (FFS whether to be a basic component) - Support of out-of-order operation for PDCCH to PUSCH (FFS whether to be a basic component) - FFS: The maximum number of activated TCI states - FFS: The maximum number of MIMO layers of scheduled PDSCHs - FFS: the maximum number of CCs supporting multi-DCI based multi-TRP Optional components: - Whether the UE shall rate match around configured CRS patterns which is associated with CORESETPoolIndex (if configured) and are applied to the PDSCH scheduled with a DCI detected on a CORESET with the same value of CORESETPoolIndex - FFS: Support of two PDSCH scrambling sequences per serving cell - Support of default QCL assumption per CORESETPoolIndex - Support of separate HARQ-ACK - Support of joint HARQ-ACK - Support of two TDMed long PUCCHs in a slot | TBD |  | N/A |  | TBD [per band / per FSPC] | N | TBD |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Some companies argued that it was already agreed to make out-of-order operation optional. However, other companies argued that components #5, #6, #7 above cannot be considered in isolation since component #4 requires them as prerequisite. It is suggested to focus on this issue as well as it seems highly controversial.  Please comment on the relationship between component #4 with components #5, #6, #7 and how to move forward respecting past agreements. For now, please do not focus on whether one or three (or N) new FGs are required for out-of-order operation. Let us first decide whether out-of-order operation is a basic feature as proposed by some and if not how to address basic component #4. Once we conclude on that, we can discuss the number of separate new FGs for out-of-order operation, if any. There seems to be at least some degree of flexibility among those companies that prefer out-of-order operation to be optional and not a basic feature.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | There is no agreement that UE has to support overlapping PDSCH if UE supports Multi-DCI based Multi-TRP However, there is explicit agreement that OOO PDSCH/PUSCH/HARQ-ACK is optional for UE that supports Multi-DCI based Multi-TRP. We do not prefer the excuse to revert agreement especially when we already clarified this issue during the MTRP online discussion in front of Chairman Younsun The proposal is also made up without careful thinking, why overlapping PDSCH has anything to do with OOO PUSCH or even OOO PDSCH? Also if you check the 214 description of OOO PDSCH, it allows even non-overlapping PDSCH OOO, i.e. complete OOO. It simply makes no sense to completely tight the support of overlapping PDSCH to the support of OOO PDSCH. |
| vivo | We support to put Component 4 together with Component 5/6/7 as a new optional component. Overlapping PDSCH should be optional for multi-DCI multi-TRP operation since for FR2 this capability needs UE support of group based report and may also be dependent on per CORESET default beam, which are all optional. |
| MediaTek | OOO had been agreed to be optional. M-DCI still can work without allowing OOO across TRPs by semi-static scheduling restriction or in ideal backhaul scenarios. Similarly, supporting of overlapping PDSCHs is not a must-have for M-DCI operation. Component #4 should be decoupled with components #5, #6, #7. At least one FG is needed for the three OOO functions. |

## Single-DCI based multi-TRP

The following are the alternatives in Error: Reference source not found for discussion in this email approval:

Alt. 1:

| 16-2b | Single-DCI based multi-TRP | Basic component(s): - FFS: Support of MAC CE to activate multiple two TCI states for a TCI codepoint - FFS: Number of CCs supporting single-DCI based multi-TRP operation Optional components: - Support of default QCL assumption with two TCI states | TBD |  | N/A |  |  | N | TBD |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2b-0 | Two default beams for single-DCI based multi-TRP | Support of default QCL assumption with two TCI states | 16-2b |  | N/A |  | Per band | TDD only | FR2 only |  |  | TBD |
| 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Whether supporting Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
| 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
| 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | TBD [per FSPC] | N | TBD |  |  | TBD |
| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 2 candidate values {10 CBs, TBD} | TBD |
| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, TBD} | TBD |

Alt. 2:

| 16-2b | Single-DCI based multi-TRP | Basic component(s): - FFS: Support of MAC CE to activate multiple two TCI states for a TCI codepoint - Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Support of DMRS entry {0, 2, 3} - FFS: Number of CCs supporting single-DCI based multi-TRP operation Optional components: - Support of default QCL assumption with two TCI states | TBD |  | N/A |  |  | N | TBD |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2b-0 | Single-DCI based multi-TRP optional components | Support of default QCL assumption with two TCI states | TBD |  | N/A |  |  | N | TBD |  |  | TBD |
| 16-2b-1 | Two PTRS ports for single-DCI based multi-TRP Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Whether supporting Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
| 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
| 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | TBD [per FSPC] | N | TBD |  |  | TBD |
| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 2 candidate values {10 CBs, TBD} | TBD |
| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, TBD} | TBD |

Alt. 3:

| 16-2b | Single-DCI based multi-TRP | Basic component(s): - FFS: Support of MAC CE to activate multiple two TCI states for a TCI codepoint - FFS: Number of CCs supporting single-DCI based multi-TRP operation Optional components: - Support of default QCL assumption with two TCI states | TBD |  | N/A |  |  | N | TBD |  |  | TBD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Whether supporting Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} - Support of default QCL assumption with two TCI states | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
| 16-2b-2 | Single-DCI based FDMSchemeA | Support of FDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  |  | TBD |
| 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining | 16-2b, TBD |  | N/A |  | TBD [per FSPC] | N | TBD |  |  | TBD |
| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 2 candidate values {10 CBs, TBD} | TBD |
| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  | Per band | N | TBD |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, TBD} | TBD |

Alt. 4:

| 16-2b | Single-DCI based multi-TRP | Basic component(s): - FFS: Support of MAC CE to activate multiple TCI states for a TCI codepoint - FFS: Number of CCs supporting single-DCI based multi-TRP operation Optional components: - Support of default QCL assumption with two TCI states | TBD |  | N/A |  |  | N | Y |  |  | Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 16-2b-1 | Single-DCI based SDM scheme | - FFS: Support of DCI indication of of 2 TCI states by a codepoint and DMRS ports within two CDM groups - Whether supporting Support of two PTRS ports - FFS Support of DMRS entry {0, 2, 3} - Maximum number of QCL Type D RSs indicated in a codepoint of a DCI | 16-2b, TBD |  | N/A |  | Per Band | N | Y |  |  | Optional |
| 16-2b-2 | Single-DCI based FDMSchemeA | - Support of FDMSchemeA - Maximum number of QCL Type D RSs indicated in a codepoint of a DCI | 16-2b, TBD |  | N/A |  | Per Band | N | Y |  |  | Optional |
| 16-2b-3 | Single-DCI based FDMSchemeB | - Support of FDMSchemeB - For FDMSchemeB, whether the UE can support CW soft combining - Maximum number of QCL Type D RSs indicated in a codepoint of a DCI | 16-2b, TBD |  | N/A |  | Per Band] | N | Y |  |  | Optional |
| 16-2b-4 | Single-DCI based TDMSchemeA | - Support of TDMSchemeA - Supported maximum TBS size for TDMSchemeA | 16-2b, TBD |  | N/A |  | Per band | N | N |  | Component 2 candidate values {10 CBs, TBD} | Optional |
| 16-2b-5 | Single-DCI based inter-slot TDM | - Support of RepNumR16 in PDSCH-TimeDomainResourceAllocation and the maximum value of RepNumR16 - Supported maximum TBS size according to RepNumR16 in PDSCH-TimeDomainResourceAllocation - FFS: TCI state mapping to PDSCH transmission occasions (Cyclical mapping or Sequential mapping) | 16-2b, TBD |  | N/A |  | Per band | N | N |  | Component 1 candidate values: {8,16} Component 2 candidate values {10 CBs, TBD} | Optional |

According to the feedback in [1], eight out of eleven companies prefers Alt. 1.

Can we agree Alt. 1 as new baseline/reference for single-DCI based multi-TRP? Further details such as component description, candidate values, type, xDD/FRx differentiation can be part of a second round email discussion.

| Company | Comments/Questions/Suggestions |
| --- | --- |
| Apple | Yes, we are fine to agree on Alt.1 as starting point with further clarification of the components |
| vivo | We are ok with Alt 1, with the following comments: - The rationale for reporting the number of CCs supporting single DCI based multi-TRP operation should be clarified. We don’t see the necessity to support this component. |
| MediaTek | We agree that Alt. 1 can be the baseline for further discussion. Regarding other companies’ view on the necessity of “Maximum number of QCL Type D RSs indicated in a codepoint of a DCI”, we would like to know whether S-DCI M-TRP can be supported in FR2 for SDM/FDM with only one Rx beam. If the answer is YES, then this component should be included, instead of just saying SDM/FDM is supported or not per band or per FR. |

# Conclusions

…

# References

- R1-2001868, Summary on UE features for eMIMO, Moderator (AT&T)